Appropriate Assessment Study Zonal Urban Plan Wind Energy Park 48 (46) Wind Power Plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads loc. Cerchezu, Constanta county

Beneficiaries:

SC South Wind SRL

Report Title:

Appropriate Assessment Study

Date

April 2025

Document control sheet

| Project | "Zonal Urban Plan - Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads" |
| --- | --- |
| Contracting Entity | SC South Wind SRL |
| Provider | SC Ramboll South East Europe SRL |
| Versi a | [Final version] |
| Dat a | April 2025 |
| Document Title | APPROPRIATE ASSESSMENT STUDY |

| Prepared by: GIS Expert Fieldwork and special contributions: | Vlad Dinu Ruxandra Florentina Buga Oana Falup Dan George Constantin Stelian Valentin STĂNESCU |
| --- | --- |
| Verified by: | Gabriela Mușat |
| Approved by | Ileana Fălcescu |

Contents

1. Description and analysis of the PP submitted for approval	7

1.1 PP Presentation	7

1.1.1 General information on the PP: name, holder, purpose and objectives	7

1.1.2 Geographical and administrative localization	9

1.1.3 Justification of the need for PP	11

1.1.4 Description of the life cycle of the Plan	12

1.1.5 Natural resources needed to implement the PP	18

1.1.6 Information on the production being carried out, information on raw materials, chemical substances or preparations used	19

1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities	20

1.1.8 Waste generated by PP and how to manage it	35

1.1.9 Land use requirements necessary for PP implementation	37

1.1.10 Additional services required for PP implementation	40

1.1.11 Activities generated as a result of PP implementation	40

1.1.12 Description of PP technological processes	43

1.1.13 Characteristics of existing, proposed or approved PPs that may generate cumulative impact with the PP that is under assessment and that may affect ANPIC	43

1.1.14 Other information requested by ACPM	48

1.1.15 Summary of effects generated by the implementation of the PP	49

1.1.16 Summary maps of all interventions potentially affecting ANPIC	65

1.2 Effects of PP interventions	68

1.3 Other PPs with which the analyzed PP can generate cumulative impact	79

2. Information on protected natural areas of Community interest affected by the implementation of the PP	108

2.1 Data on protected natural areas of Community interest	109

2.1.1 ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake	111

2.1.2 ROSPA0036 Dumbrăveni	114

2.1.3 ROSPA0094 Hagieni Forest	116

2.1.4 ROSAC0157 Hagieni - Cotul Văii Forest	118

2.1.5 ROSPA0166 Plopeni-Chirnogeni	120

2.1.1 BG0000569 Kardam	121

2.1.2 BG0000570 Izvorovo - Kraishte	122

2.2 Data on habitats/species in ANPIC possibly affected by PP	124

2.3 Structural and functional relationships that create and maintain the integrity of ANPIC	185

2.3.1 Green infrastructure	186

2.3.2 Watercourses	189

2.3.3 Migration routes	191

2.3.4 Functional relationships	204

2.4 ANPIC's conservation objectives	270

2.5 Analysis of the conservation measures in the management plan/ ANPIC regulation that may limit/ influence the interventions and activities proposed by the PP	270

2.6 Other information relevant to the conservation of ANPIC, including possible changes in its natural evolution	271

3. Presenting the results of field activities	272

3.1 Plants and habitats	276

3.1.1 Description of the working method	276

3.1.2 Results achieved	276

3.1.3 Distribution map	285

3.1.4 Identified pressures, other important issues	286

3.2 Mammals	287

3.2.1 Description of the working method	287

3.2.2 Results achieved	288

3.2.3 Distribution map	288

3.2.4 Identified pressures, other important issues	293

3.3 Amphibians and reptiles	293

3.3.1 Description of the working method	293

3.3.2 Results achieved	293

3.3.1 Distribution map	294

3.3.2 Identified pressures, other important issues	297

3.4 Avifauna	298

3.4.1 Description of the working method	298

3.4.2 Results achieved	298

3.4.1 Distribution map	302

3.4.2 Identified pressures, other important issues	310

3.5 Chiropteran	310

3.5.1 Description of the working method	310

3.5.2 Results achieved	312

3.5.3 Distribution map	319

3.5.4 Identified pressures, other important issues	325

3.6 Natura 2000 sites in Bulgaria	326

4. Analysis of pressures and threats	336

5. Impact assessment	398

5.1 Identifying and quantifying impacts	398

5.1.1 Identification of potentially significant impacts based on the Specific Conservation Objectives	398

5.1.2 Quantifying impacts	409

5.2 Assessing the significance of impacts	443

5.2.1 ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake	443

5.2.2 ROSCI0157/ROSAC157 Hagieni - Cotul Văii Forest	444

5.2.3 ROSPA0166 Plopeni - Chirnogeni	444

5.2.4 ROSPA0036 Dumbrăveni	445

5.2.5 ROSPA0094 Hagieni Forest	446

5.2.6 BG0000569 Kardam	447

5.2.7 BG0000570 Izvorovo - Kraishte	448

6. Prevention, avoidance and mitigation measures	450

6.1 Measures proposed to avoid and reduce impacts	450

6.2 Timetable for implementation and monitoring	457

7. Monitoring prevention, avoidance and mitigation measures	461

8. Residual impact assessment	465

9. Alternative solutions	466

10. Compensatory measures	466

11. Methods used to collect information on affected species and/or habitats of Community interest	466

11.1 Field data collection for plant habitats/communities and flora species	466

11.2 Field data collection for inver invertebrates	467

11.3 Field data collection for amphibians and reptiles	467

11.4 Field data collection for avifauna	468

11.5 Field data collection for mammals (including chiroptera)	468

12. Appropriate evaluation conclusions	470

Selective bibliography	472

INTRODUCTION

The environmental assessment is an integral part of the procedure for adopting plans and programs that may have significant effects on the environment, the procedure for its realization being regulated by HG no. 1076/2004 on establishing the procedure for carrying out the environmental assessment for plans and programs, with subsequent amendments. This decision transposes into national legislation the provisions of Directive 2001/42/EC of the European Parliament and of the Council of 27.06.2001 on the assessment of the effects of certain plans and programs on the environment (hereinafter referred to as "SEA Directive").

The environmental report shall identify, describe and assess the potential significant environmental effects of the implementation of the Plan, as well as its alternatives, taking into account the objectives and geographical coverage, as well as the specific objectives of the species, habitats and conservation objectives of the protected natural areas in the Plan area

The Environmental Report was prepared in accordance with the provisions of Annex no. 2 of GD no. 1076/2004 with subsequent amendments, the recommendations of the Generic Guide for the Strategic Environmental Assessment (SEA) procedure developed by the Ministry of Environment and Water Management in collaboration with the National Agency for Environmental Protection and the Manual for the application of the procedure for carrying out the environmental assessment for plans and programs, approved by Order of the Minister of Environment and Water Management no. 117/2006.

The Adequate Assessment Study was carried out in accordance with the provisions of the Methodological Guidelines on the adequate assessment of the potential effects of plans or projects on protected natural areas of Community interest approved by Order no. 1682/2023, subsequently amended and supplemented by Order no. 2.452/2023 and in compliance with the provisions of the Specific Guide (approved by Order no. 1679/2023 for the approval of the Specific Methodological Guide on the appropriate assessment of the potential effects of plans/projects in the areas of interest) taking into account that the Plan in question is included in the field of "Energy Production".

The conclusions of this Appropriate Assessment Study are presented and integrated within the Environmental Report.

The Environmental Report and the Adequate Assessment Study were carried out by SC Ramboll South East Europe SRL, a company certified by the Commission for the Attestation of Natural and Legal Persons that prepare environmental studies, appointed by the Ministry of Environment, Water and Forests and registered in the Register of certified experts for the preparation of environmental studies, for the following fields and studies: Environmental Impact Assessment Report (RIM-2, RIM-3, RIM-7, RIM-11a, RIM-11b, RIM-11c); Site Report (RA-7; RM-3, RM-11a), Environmental Report (RM-11b, RM-11c, RM-13b); Safety Report (RS-2, RS-11c); Environmental Balance (BM-2, BM-11b, BM-11c); Adequacy Assessment Study (EA); Climate Change Study (EGSC) - Certificate no. RGX 333/1.08.2022 (see Annex Documents).

# Description and analysis of the PP submitted for approval

## PP presentation

### General information on the PP: name, holder, aim and objectives

Plan name

"Plan Urbanistic Zonal Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads" located in the Cerchezu locality, Constanța County, falls under the scope of art.28, of OUG 57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna, approved by Law no.49/2011. Thus, for the continuation of the environmental procedure, the Agency for Environmental Protection Constanta (hereinafter referred to as ,,APM Constanta) decided, after going through the framing stage, by the initial framing Decision No. 32/12.04.2024, that for this plan it is necessary to go through the environmental assessment procedure, the appropriate assessment procedure and the environmental assessment procedure in the cross-functional context, considering the framing of the PUZ in the provisions of art.5, paragraph 2 of GD no. 1076/2004, the results of the assessment of the criteria set out in Annex no. 1 of GD no. 1076/2004 and the possible effects it could have on protected natural areas. Thus, the PUZ will be submitted to the adoption procedure with environmental opinion.

To go through the environmental procedure, it was necessary to prepare the Environmental Report and the Adequate Assessment Study, as well as the environmental assessment in a transboundary context taking into account the fact that the plan is located at a distance of about 410 m from the protected natural area BG 0000569 Kardam (Bulgaria) and at a distance of about 3.64 km from the protected natural area BG0000570 Izvorovo - Kraiste, in order to comply with the requirements of the initial classification Decision no. 32/12.04.2024, issued by APM Constanta, namely ,, the Environmental Report and Adequate Assessment Study must contain a detailed analysis of the potential impact on the conservation objectives of the natural areas on the territory of the Bulgarian side." Thus, for the analysis of the potential impact on the conservation objectives of the natural areas BG 0000569 Kardam and BG 0000570 Izvorovo - Kraiste, correspondence was conducted with the Ministry of Environment and Water of Bulgaria in order to identify all the data necessary for the detailed assessment of the potential impact, namely address no. 99-00-243-1 dated 30.05.2024 and address no. 99-00-243-3 dated 13.06.2024 (see Annex Documents)

For the Adequate Assessment Study, the provisions of the Methodological Guide on the adequate assessment of the potential effects of plans or projects on protected natural areas of community interest, approved by Order no. 1682/2023, subsequently amended and supplemented by Order no. 2.452/2023 and the provisions of the specific Guide (approved by Order no. 1679/2023 for the approval of the specific Methodological Guide on the adequate assessment of the potential effects of plans/projects in the areas of interest) were respected, taking into account that the Plan in question is included in the field of Energy Production.

The conclusions of the Appropriate Assessment Study are presented and integrated within the Environmental Report.

Plan holder

The holder of the plan "Plan Urbanistic Zonal Plan Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads" is SOUTH WIND S.R.L., with registered office in Bucharest, Sector 1, Turda Street, No. 122, Bl. 39, Sc. B, Et. 2, Ap. 54.

General Designer ARHICO CONSULTING S.R.L., with registered office in Constanța, Bld. Mamaia, Nr.68, Bl.Bi4, Sc. B, Et. 2, Ap. 35, Constanța County.

After obtaining the TRANSGAZ SA approval no. 34618/948/10.05.2023 for the plan in question, it was imposed to give up 2 sites, namely the wind power plants T17 and T23, which did not comply with the legal distances from the identified pipelines and thus to comply with the conditions imposed by the Transgaz approval the number of wind power plants is 46.

Wind energy park 48 (46) wind power plants approx. 316,8MW (303,6MW) will play an important role in decarbonizing the national energy system, will contribute to the replacement of polluting capacities and to reaching the Renewable Sources of Energy in the Electricity Sector (RES-E) target.

Objectives of the zonal urban plan

The Zonal Urban Development Plan was initiated in order to establish the conditions of location, sizing, technological compliance and urban planning regulation for the "Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads".

The studied land is located in a favorable area for the development of the wind energy power generation function, both in terms of wind potential, topography and infrastructure - electricity grids, as well as in terms of the availability of the local community and investors.

The only development potential with significant economic impact for the area is the almost permanent presence of winds. In addition to this potential, the area can be used for agriculture.

According to HG no. 1535/2003 on the approval of the Strategy for the utilization of renewable energy sources, five distinct wind energy zones have been identified in Romania, according to the existing energy potential, environmental and topographical conditions. Within these regions, favorable locations for the siting of wind turbines are those aiming at "energy exploitation of the flow over the hilltop effect or the channelling effect of air currents".

The Dobrogean plateau which is the beneficiary of a "mild" climate is part of one of the wind areas with high energy potential.

The proposed plan to capitalize on the natural potential of the area is an opportunity that must be approached with the utmost professional exigency, with critical discernment, so that the present effects of such an objective do not produce urban dysfunctions, irreparable degradation of environmental conditions, changes to ecosystems present in the area.

### Geographical and administrative location

"Wind energy park 48 (46) wind power plants approx. 316.8MW (303.6MW), transformer stations, electrical connection networks, construction and modernization of communication and access roads" will be located in the extravilan, in the western and eastern part of the administrative territory of Cerchezu commune, at the border of Chirnogeni, Independența, Dumbrăveni and Negru Voda communes.

The location of the energy park is in the immediate vicinity of the border line with Bulgaria.

From a geographical point of view, the area of implementation of the PUZ is located in the southern extremity of the Dobrogei Plateau, a podiș relatively rigid, with a gentle, gently undulating relief and relatively low altitudes (200-300m). The Dobrogean Plateau which is the beneficiary of a "mild" climate is part of one of the wind areas with high energy potential.

According to the Certificate of Urbanism no. 129/28.11.2022, the land related to the buildings are registered in the land registers no. 100259 (A439/2/15), 100395 (A411/1/10), 100998 (A439/1/16), 100592 (A 439/3/3), 100933 Cerchezu (CAD 134-A208/2/3), 100725 (A430/2/15), 100950 (A98/2/4/2), 100277 (430/1/17), 100243 (A98/1/8), 100147 Cerchezu (CAD 59-A207/7), 102382 (A 195/25), 100229 (A208/2/21), 100937 (A179/10/1), 101098 (A174/11), 100187 (A208/1/20), 100511 (A208/1/13), 100671 (A78/22), 100677 (A78/23), 100664 (A177/23), 100317 Cerchezu (CAD 202-A177/10), 101927 (A169/27), 101191 (A359/12), 102366 (A359/3), 101246 (A359/25), 100192 (A363/2/9), 102419 (A363/1/23), 100705 (A72/4), 101832 (A359/12), 102366 (A359/3), 101246 (A359/25), 100192 (A363/2/9), 102419 (A363/1/23), 100705 (A72/4), 101823 (A90/9), A102098 (A90/8a), 100370 (A363/2/30), 100372 (A367/49), 100438 (A72/34), 100443 (A367/12/2), 100426 (A9/12 lot 1), 100430 (A9/12 lot 2), 100428 (A9/12 lot 3), 100666 (A78/10), 100093 (A1/20/1), 100094 (A1/20/2), 100762 (A277/2/11), 100754 (A277/2/3), 100894 (A124/11), 100614 (A250/26), 100508 (CAD138/1-A251/28/1), 100035 (CAD 218-A250/10), 100942 (A338/1/21), 101156 (A379/7/5), 100948 (A338/2/6), 102302 (A338/3/5), 100860 (A335/1/3), 100111 (A314/1/18/1) 100112 (A314/1/18/2), 101101 (A314/1/12), 100256 Cerchezu (CAD 151-A411/2/4), 100693 (A5/28), 100378 (A251/1/31), 100506 (A251/1/26), 101279 (A163/18/2), 100318 (A207/11).

After obtaining the TRANSGAZ SA approval no. 34618/948/10.05.2023 for the PUZ, it was necessary to give up two turbines T17 and T23, which correspond to the land maps no: A102098 (A90/8a) - related to turbine T17 and 100318 (A207/11) related to turbine T23.

Figure 1-1 Administrative location of the PUZ

The site's neighborhoods are:

- North: Independența TAU boundary - privately owned agricultural land;

- South: border with Bulgaria;

- East: boundary between the administrative units of Chirnogeni and Negru Voda - private agricultural land;

- West: boundary between Independența TAU and Dumbrăveni TAU - private agricultural land.

The Stereo70 coordinates for the wind turbine sites proposed by this PUZ are presented in tabular form as an appendix to this study.

The distances to the nearest settlements to the study area are:

- 1 km from Cerchezu T5;

- 1 km from the village of Măgura T35;

- 1.1 km from Viroaga T32;

- 1.3 km from the village of Căscioarele T27;

- 1.56 km from the Bulgarian town of Iovkov T3;

- 2.1 km from the Bulgarian town of Kraiste T47;

- 3.98 km from Negru Voda T9;

- 5.5 km from Independence T32;

- 5 km from Olteni T32.

The following criteria were taken into account in the selection of the proposed sites for the turbines envisaged by the PUZ:

- the existence of wind potential that can be harnessed to ensure investment efficiency

- the existence of electricity transmission networks in the area, allowing connection in optimal conditions to the SEN.

In the area, in the vicinity of wind power plants T1, T4, T12, T26, T27 and T28 there are irrigation canals, which are not operational.

The main function of the studied area is agriculture, the agricultural land with the current arable function has an area of 2,908.24 ha, representing 96.05% of the total area studied. Land use categories are presented in detail in chapter 1.1.9 Land use requirements.

In the DTAC phase, it is proposed to permanently take out of agricultural use an area of 30 ha related to the base of the pillars, turbine access roads and maintenance platforms and temporarily an area of 70 ha related to the storage platforms for component parts and crane assembly.

### Why PP is needed

The necessity and timeliness of the PUZ had as a starting point the conclusions of the PUG regarding the organization of the administrative territory of Cerchezu commune, namely:

- Stimulating the complex evolution of the commune by elaborating and implementing spatial, sustainable and integral development strategies in the short, medium and long term;

- implementation in the territory of new types of investments contributing to the economic and social development of the commune;

- the organization of road and pedestrian circulation at the level of traffic growth in the new areas introduced in the built-up area and its links with the other functional areas of the localities;

- integration and harmonization of new constructions and developments with the specific locality;

- the enhancement of the natural setting, the relief and its harmonization with the proposed sites;

- Ensuring the infrastructure and the technical and public utilities equipment in the new areas introduced in the urbanization.

With the realization of this investment, new jobs will be created, necessary for the economic and social growth of the localities of Cerchezu commune.

The implementation of the plan for the wind energy park totaling 316.8MW (303.6MW) in the studied territory is based on the national and international conventions on energy and climate change, based on which Romania has developed the National Integrated Energy and Climate Change Plan 2020 - 2030, as well as the Romanian Energy Strategy 2022-2030, with a 2050 perspective and the Romanian Long-term Strategy for GHG reduction Romania Neutral in 2050.

The proposed plan responds to the general objective of Romania's Energy Strategy, namely the growth of the energy sector under sustainable conditions, respecting national, European and global benchmarks, the realization of this plan contributing to the development of the national energy system and also to the compliance with Romania's international obligations (Paris Agreement and subsequent strategies).

The continued increase in the use of renewable resources has been identified as the main solution to reduce the environmental pressures on the energy sector from fossil fuel exploitation.

In addition, building a "Resilient Energy Union with a forward-looking climate change policy" is a major priority of the European Commission. The Energy Union Strategy, through its five dimensions: energy security, an integrated internal energy market, energy efficiency, decarbonization of the economy and research and innovation, provides a framework for achieving this priority.

As part of the "Clean Energy for All Europeans" package, the EU also revised the EU's energy targets for 2030, with at least a 32% share of renewable energy, which was further revised as part of the "Ready for 55" package to at least 40% by 2030.

Encouraging projects that enable the use of renewable energy provides the decarbonization dimension of the economy. This dimension affirms that "an ambitious climate change policy is an integral part of our energy union" and aims to make the EU the world leader in renewable energy.

The Plan is not directly related to protected nature areas and is not necessary in the process of conservation of species and habitats of Community interest within protected nature areas. However, ensuring the use of energy from renewable sources will reduce greenhouse gas emissions, improve environmental quality and have a positive impact on biodiversity.

It is assessed that the proposed PUZ represents a major investment in the area, which will generate viable direct and indirect opportunities for long-term improvement of the socio-economic situation of the community, without creating significant effects on environmental factors.

### Description of the life cycle of the Plan

The life cycle of the Plan involves: planning and design, construction, operation and maintenance and finally decommissioning. The activities involved in each stage of the life cycle of the Plan proposed by the PUZ are shown in the table below:

Table  1-1  Life cycle description

| Life cycle stage | Characteristic activities |
| --- | --- |
| PLANNING AND DESIGN | Site evaluation |
|  | Wind, noise, shading studies, environmental impact studies, technical documentations and their endorsement by the competent authoritiesetc. |
|  | Design and obtaining the necessary authorizations/approvals/approvals for the execution plan |
| CONSTRUCTION | Realization of access roads (temporary or permanent) and storage areas for equipment/components/materials/surplus material; |
|  | Building technological platforms and foundations |
|  | Installing wind power plants |
|  | Making electrical connections and laying underground cables between the wind generator sets |
|  | Building transformer substations |
|  | Site organization |
|  | Land reclamation work at the end of construction |
| OPERATION and MAINTENANCE | Regular inspections |
|  | Maintenance and repairs |
|  | Technology upgrades to maximize energy output and extend equipment lifetime |
| DEFEAT Interventions and activities are similar to those in the construction phase | Site organization |
|  | Dismantling wind turbines |
|  | Decommissioning of technological platforms and foundations |
|  | Dismantling electrical connections and removing underground cables |
|  | Decommissioning of transformer stations |
|  | Decommissioning of logging roads |
|  | Land reclamation work on completion of construction |

Construction phase of the wind farm

Work areas shall be delineated prior to the commencement of construction work in such a way as to indicate the limits within which all construction-assembly activities will take place and to minimize the affected areas.

The area has no water or sewage networks.

For the implementation of the PUZ will be set up in the perimeter of the plan 3 site organizations, with an area of 3,000 m2 each, whose location is shown in the table below.

Table  1-2  Stereo 70 coordinates of site organizations

| Site organization | E (X) [m] | N (Y) [m] |
| --- | --- | --- |
| OS1 | 748717,741 | 264875,924 |
|  | 748647,678 | 264733,057 |
|  | 748574,038 | 264755,332 |
|  | 748644,036 | 264898,219 |
| OS2 | 749880,520 | 268760,540 |
|  | 749843,882 | 268528,385 |
|  | 749793,290 | 268535,543 |
|  | 749829,928 | 268767,698 |
| OS3 | 747373,991 | 264415,747 |
|  | 747357,911 | 264375,476 |
|  | 747100,574 | 264481,099 |
|  | 747116,612 | 264521,394 |

The site organization is of a temporary nature, requires the temporary set-aside of the occupied land area and will include:

- facilities for temporary storage of materials and parking of machinery and equipment (bulldozers, loaders, excavators, compactors, finishers, dumpers, cranes, dump trucks, dump trucks, truck mixers, trailers) in the central area of the wind farm, with quick access to the working points (platform and shed/warehouse);

- fueling facilities for machinery (mobile tanker);

- Staff facilities (office shed, workers' changing rooms, first aid point);

- fire-fighting facilities (PSI point);

- materials, installations, devices and control systems necessary for the execution in accordance with the provisions of the execution plan and the regulations in force;

- properly marked fencing to avoid direct access of strangers to the site.

Within the site organization, the electricity supply will be provided by connection to the existing networks in the area, based on specialized documentation, and the water supply to ensure hygienic-sanitary needs will be provided from tanks / cisterns brought on site for this purpose. In order to carry out the works, water will be used for possible sprinkling of the work fronts in order to reduce dust pollution in weather conditions favorable to the dusting of earth or powdery materials, but these quantities are small and do not require evacuation from the site. Drinking water will be bottled water brought on site.

The personnel involved in the execution of the works will be provided with sanitary groups - ecological toilets which will be emptied periodically by a firm authorized for this purpose or a drainable basin will be provided to which the sanitary groups - ecological toilets will be connected and which will be emptied periodically.

Access to the site is from the national road DN38 Constanta - Negru Voda, county road DJ391 Mangalia - Negru Voda - Cobadin - Tuzla, municipal roads (DC13, DC16, DC17, DC18, DC19) and existing exploitation roads (De). Access and connecting roads between the turbines and the assembly and maintenance platforms will be built on the sites.

The county road Dj 391 is a paved road, the communal roads are partially paved and all other roads are dirt roads.

The farm roads included in the PUZ will constitute the road access for:

- transportation of wind turbine components, transformer stations, ancillary components, as well as materials required for access roads, foundations and technological platforms during the construction phase;

- transportation of the various technical components during the operation and maintenance phase of the wind investment

As the access roads do not have a structure suitable for heavy traffic, it is necessary to make them suitable in order to provide the necessary turning radius for the trailers used to transport the components of the wind power plants.

The following technological operations are proposed to realize the roads:

- removal of the topsoil (or, where appropriate, excavation to the bedrock level in the case of sub-base layers), stabilization of the sub-base layer and compaction;

- backfill to the lower elevation of the crushed stone layer, where applicable;

- stabilized soil, applied to areas with excessive moisture;

- sand and compaction.

Roads and platforms shall be made of bituminized stone laid on a sand bed. Their structure must be able to withstand a maximum load of 12 tons per axle and a pressure of 18,5 tons/m2 . The width of the enclosure roads shall be 4.50 m with a radius of curvature of minimum 50.00 m in the access area from the service roads. The radii of curvature will be provided by temporary lease of land.

Slopes of min. 3% shall be provided for rainfall runoff and radii of curvature of min. 50 m at site access.

Each plot will have direct access to a landscaped farm road. The access roads, which are in the local public domain, will be built by the beneficiary of the investment under a contract concluded with Cerchezu Town Hall using bituminized stone laid on a sand bed.

Intersections shall be made with simple arched joints with a radius corresponding to the radius inscribed in the limits of the platform of the agabaritic transporter.

The arrangement of the intersections between operating roads and classified roads: national roads, county roads and municipal roads shall be carried out in accordance with the permits issued by the Road Administrator.

Water runoff will be discharged by draining stormwater down the embankment.

Earthworks consist of land leveling, digging, excavation, backfilling.

The technological platforms necessary for the assembly phase of the wind farm equipment do not require concreting but only leveling. Once the wind turbines have been installed, the land affected by the technological platforms will be restored to its original state and returned to agricultural use.

To lay the electrical cables (+ fiber optics) connecting the wind turbines to each other, trenches will be made between 1.0 and 1.5 m deep. After laying the cables, these will be filled with compacted earth and the land will be returned to its original shape. A protection zone of 1.00 to 1.50 m left to right of the axis of the cable will be provided for intervention in the event of damage.

The wind turbine pillar will be fixed in concrete foundations. The geotechnical analysis carried out on the PUZ site has determined that the recommended foundation type is on piles, designed to provide the strength and stiffness in the group required by the turbine manufacturer.

After installation, the foundation is covered with a layer of excavated earth and topsoil on which the texture of the natural vegetation in the area will be restored. Thus, the area of land permanently occupied by a wind turbine will be made up of the area occupied by the base of the mast, the maintenance platform and the connecting roads between the power plants and the access roads to the sites.

Installation of installations/equipment

The wind turbine has the following main components shown in the following figure and table:

Figure 1-2 Components of a wind turbine

Table  1-3 Components of a turbine

| Nacela Pale Pylon/ Pillar Foundation/Platform | The nacelle is mounted on the top section of the mast and has a fiberglass casing. Inside the nacelle is the generator which is equipped with a rotor and a control system. The blades are made of fiberglass injection-molded components, with a design based on their own load-bearing surfaces. The rotor has 3 blades, 170 m in diameter. The pillar supporting the nacelle is made of metal, conical in shape and will be 155 m high. The purpose of the foundation is to support the pylon, the rotor, the blades and the nacelle with all the equipment, as well as to transmit the specific loads mentioned to the ground, without producing deformations that compromise the safe operation of the work (without exceeding the bearing capacity of the foundation ground). The foundation of the wind generator set shall be an insulated, concrete slab type, weight bearing foundation made of monolithic cast reinforced concrete. It is proposed to build technological platforms for the period of installation of the wind generator sets. The platforms will be located adjacent to the road, in the immediate vicinity of each wind generator foundation. The technological platforms will be kept, they will be used in case of possible interventions on the wind generator sets for land leveling, followed by covering with ballast. The maximum height of the site platform will be max. 0,95 m. |
| --- | --- |

Electrical connections

In order to feed the energy produced into the grid, the wind turbines are connected to each other by an underground cable system (LES) to the 33 kV/ 110 kV transformer stations (two in number) located within the study area. In order to connect the wind farm to the National Energy System (SEN), an overhead/underground electrical connection from the 33kV/110kV transformer substations to the 110 kV LEA in the area will be realized, based on a specialized project not subject to the present documentation.

The electricity grid connecting the turbines to the 33kv - 110kv transformer substations, located on plots A 5/28 and A 177/16, will be buried at a depth of 1.00 - 1.50 m, and will be located in the developed/undeveloped farm roads in the study area.

The telecommunication cable, which transmits all data on the operation of the wind turbines to a process computer and, by radio, to a control unit where the proper functioning of the wind farm is monitored, will be laid in the electricity network trenches above the cables and separated by a layer of sand.

Cable routing for internal connections is planned where possible along existing roads and paths.

A protection zone equal to 1.00 - 1.50m left - right of the cable centerline shall be provided for intervention in the event of damage. A protection zone shall be provided with respect to the 20kV LEA = 245.90 m representing H + Lstâlppala +3.0 m+2.90 m (radius of the pole at the base).

The wind turbine array can be monitored automatically by the SCADA system or manually by the individual computers built into each turbine. The wind speed at which the wind turbines are programmed to stop is 23 - 25 m/s.

The site restoration works will be carried out once the construction works are completed, when the ecological reconstruction of all the land temporarily occupied by the different objectives of the construction site (site organizations, temporary technological platforms, etc.) will be carried out.

The main rehabilitation works to be carried out are:

- closure of site-related objectives (site organizations, temporary technological platforms, etc.);

- the existing constructions and installations will be dismantled and evacuated (loaded and transported off-site) and the site will be prepared for rehabilitation.

- landscaping will be carried out by sanitation, leveling and grassing works.

Wind farm operation phase

At this stage, the activities to be carried out are:

- electricity generation.

- maintenance and maintenance work.

Wind farm decommissioning phase

The lifetime of a wind turbine is 30-35 years. At the end of this period, there are two possibilities: decommissioning the wind turbine generating sets and restoring the site or replacing the wind turbine generating sets with new ones.

During the decommissioning period, site organization for decommissioning will involve the same activities and objectives as during the construction period.

The work involved in decommissioning the wind farm includes:

- dismantling of wind generator sets and poles with recovery and valorization of metals and generally reusable materials;

- demolition of foundations and use of crushed concrete for various improvements (road decks, various fillings)

- recovery and valorization of electrical cables; filling / leveling of the foundation pit and restoration of the ground cover.

Replacing wind generator sets with new ones requires fewer interventions.

### Natural resources needed to implement the PP

During the construction phase, the following natural resources will be necessary for the realization of the project proposed by PUZ:

- areas of land occupied by roads, technological platforms and foundations of wind power plant pillars. Excavated soil will be used for backfilling.

- Water (drinking, sanitary, technological): Bottled containers will be used to provide the drinking water required by the workers. The contractor will be responsible for supplying water for hygienic and sanitary use, which will be provided by tankers. The water will be used in small quantities for sprinkling work faces to reduce dust pollution of the air and for moistening foundations during the drying period. Water will be brought to the site as needed (usually by tanker).

- topsoil from stripping about 108600 m3 ,

- natural earth from excavation 24350 m ;3

- filling crushed stone 114250 m3 for the realization of foundations, technological platforms and for the arrangement / realization of access roads;

- sand for turbine connections.

For the implementation of the Plan, natural resources within protected natural areas will not be exploited.

During the operational period, the wind farm will use the wind energy potential, which is a renewable resource. No other natural resources will be used during this phase.

According to HG no. 1535/2003 on the approval of the Strategy for the utilization of renewable sources of energy, five distinct wind energy zones have been identified in Romania, according to the existing energy potential, environmental and topographical conditions. Within these regions, the favorable locations for the siting of wind turbines are those which aim at "exploiting the energy from the flow effect over the hilltop or the effect of channelling air currents".

The Dobrogean plateau which is the beneficiary of a "mild" climate is part of one of the wind areas with high energy potential.

|  | Legend. - Danube Delta; Solar energy. - Dobrogea; Solar and wind energy. - Moldova (plain and plains); Micro-hydro, wind, biomass. - Carpathian Mountains; Biomass, Microhydro. - Transylvanian Plateau; Microhydro. - Western Plain; Geothermal energy. - Subcarpathians; Biomass, Microhydro. - South Plain; Biomass, geothermal and solar energy. |
| --- | --- |

Source: Decision No 1069/2007 approving Romania's Energy Strategy for 2007-2020

Figure 1-3 Map of available renewable resources by region

Cerchezu Commune is located in a region with wind potential, which led to the proposal to develop "Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads". The development of the plan will take into account the challenge of sustainable management of natural resources, thus ensuring a balance between economic development and environmental protection.

The implementation of the proposed PUZ offers the opportunity to capitalize on renewable energy resources, contributing to reducing dependence on fossil fuels and reducing greenhouse gas emissions.

### Information on the production being carried out, information on raw materials, chemical substances or preparations used

This PUZ proposes the realization of a non-conventional energy complex - wind farm with a number of 46 wind turbines with an individual power of 6.6 MW/h and a total power of 303.6 MW, which has as its main purpose the production of green energy by exploiting the wind potential of the area.

The natural resources estimated to be used for the realization of the project proposed by the PUZ have been presented in chapter 1.1.5 Natural resources necessary for the implementation of the PP.

In addition, there are metals and alloys (steel for pillar structures and iron for foundations; copper in cables and electrical components, aluminum for light turbine components), composite materials (glass or carbon fiber for turbine blades), construction materials (concrete, fillers for foundations and platforms), asphalt mixtures for access roads.

All materials used will be stored throughout the execution according to the supplier's specifications to avoid damage, degradation or loss. All materials used in the execution of the investment objectives shall comply with manufacturing standards and norms and shall be accompanied by quality certificates. Upon receipt of the materials, the correspondence with the accompanying quality certificates shall be verified.

The concrete will be prepared in local authorized concrete plants under contract and transported to the site by means of concrete trucks.

The execution of the works will require the use of materials which by their composition or their potential effects on the health of employees are classified as hazardous chemical substances and preparations. These substances and materials are:

- fuels for supplying the vehicles transporting the construction materials and machinery needed to carry out the works

- fuel for vehicles transporting the team of workers

All these shall be purchased from third parties and not obtained by own production. Refueling of vehicles, oil changes, maintenance and repair work on vehicles and equipment will be carried out at vehicle fuel stations and in specialized workshops.

During wind farm operation, the only natural resource used is wind energy, which is inexhaustible. To this are added oils and waxes and greases (Vaseline) for the moving subassemblies, which are consumed during maintenance/maintenance periods.

More information about the construction details and the quantities of raw materials used will be presented in the technical design.

### Emissions of physical, chemical and biological pollutants generated by PP interventions and activities

Air emissions

The main possible forms of pollution caused by PUZ interventions are air quality changes, increased noise and vibration levels, increased light intensity, increased concentration of pollutants in the soil / accidental pollution, physical and behavioral barriers for wildlife, introduction / spread of invasive species.

Changes in air quality can only occur during the construction and decommissioning phases and are caused by dust pollution from site organization, earthworks, excavation works for foundations, platforms and connections, upgrading of mining roads, handling of earth masses and construction materials.

However, depending on climatic conditions and the activities carried out at the same time as the construction activities, there may be an increase in dust levels which may reach the local settlements in the area and the natural areas in Bulgaria.

The ADMS 6 software application was used to forecast air pollution as a result of the works related to the execution phase of the project. It uses a model based on a Gaussian pollutant plume equation for continuous emission sources. The model calculates dispersed pollutant concentrations in the environment based on a set of input data including:

- Characterization of meteorological conditions (main wind direction, wind speed, turbulence, turbulence, air temperature, rural/urban environment);

- Source characterization (source type, mass flow rate, height, gas velocity, gas temperature);

- Spatial characterization of the investigated area (with the possibility of graphical representation of concentration isolines on maps or images).

Regarding the modeling results, the following observations should be made:

- The dispersion of pollutants will be predominantly N-NE (in the predominant wind direction);

- In the case of the development phase of the turbine platforms (handling of earth masses), the intervention threshold and alert threshold values for the PM10 indicator (with an estimated maximum concentration of 60.66 μg/m3) and the hourly average values for the NO2 indicator (with an estimated maximum concentration of 239 μg/m3) will be exceeded, but these are estimated to occur only near the working fronts of the turbines. The concentration of PM10 (0.5 μg/m3) respectively NO2 (<30 μg/m3) estimated at the nearest sensitive receptor falls below the intervention and alert threshold values at this stage.

The maximum concentrations will be recorded at the level of the closest households located to the turbine platforms, i.e. households in Viroaga, but they are considerably lower than the maximum permissible concentrations according to Law 104/2011.

It was thus found that for all atmospheric pollutants emitted into the environment during the execution phase, at sensitive receptors in the area, the concentrations are estimated to be below the limits set by the legislation indicating no potential impact on ambient air quality.

We emphasize that there are no significant sources of air pollution in the analyzed area, so that the emissions generated from the analyzed activity could have a cumulative effect that could lead to exceeding the limits set by Law no. 104/2011. However, it can be assessed that the only cumulative effect with an impact on air quality in the area could be generated if the works for the development of the turbine platforms (stripping, earth moving) will overlap with the agricultural works carried out in the area. This effect will be temporary, short-term and reversible.

Figure 1-4 Dispersion of PM pollutants10 - Daily average concentration

Figure 1-5 CO pollutant dispersion - daily maximum concentration of 8-hour averages

Figure 1-6 Dispersion of SO pollutants2 - Maximum hourly concentration

Figure 1-7 Dispersion of NO pollutants2 - Hourly average concentration

Figure 1-8 Dispersion of NO pollutants2 - Annual average concentration

Figure 1-9 PM10 - Annual average concentration

Table  1-4 Maximum concentrations over different averaging ranges

| Source | Pollutant | Mediation interval | Maximum concentration |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cmax [μg/m ]3 | Alert threshold [μg/m ]3 | Limit value= intervention threshold [μg/m ]3 |  |
| Earth handling | PM10 | Daily average | 60,66 | 35 | 50 | <PA; <VL |
| Operating machinery and equipment | SO2 | Hourly maximum | 1,3 | 245 | 350 | <PA; <VL |
|  | NO2 | Hourly average | 239 | 140 | 200 | >AP; >VL |
|  | PM10 | Annual average | 1,62 | 35 | 40 | <PA; <VL |
|  | CO | Maximum 8-hour averages | 0,0334 | 7000 | 10000 | <PA; <VL |

Table  1-5 Comparison between maximum concentrations and limit values

| Source | Distance between point of maximum concentration and receiver (km) | Concentration/ concentration range at sensitive receptors ( g/m )3 | Limit value ( g/m )3 | Upper assessment threshold for vegetation protection (µg/m )3 | Lower assessment threshold for vegetation protection (µg/m )3 | Upper assessment threshold for health protection (µg/m )3 | Lower assessment threshold for health protection (µg/m )3 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Earth handling PM10 - daily average | 1,5 | Within the 0,5 | 50 | 28 | 20 | 35 | 25 | < limit |
| Operation of machinery and equipment - SO2 | 1,45 | Outside the 0,2 | 350 | 12 | 8 | 75 | 50 | < limit |
| Operation of machinery and equipment - NO2 | 1,56 | Outside the 30 | 200 | 24 | 19,5 | 140 | 100 | > limit |
| Operation of machinery and equipment - PM10 | 1,2 | Within the range of 0.03 | 50 | 28 | 20 | 35 | 25 | < limit |
| Operation of machinery and equipment - CO | 1,32 | Within the 0,005 | 10000 | - | - | 7000 | 5000 | < limit |

To reduce the negative impact on air quality during the construction of the wind farm, the following measures are proposed:

  - the use of water spraying of the working face to reduce dust, if the resulting dust is visible following the work on the plan.

  - as regards emissions from transport vehicles, they must comply with the technical conditions laid down in the roadworthiness tests that are carried out periodically throughout the period of use of all motor vehicles registered in the country.

  - using machinery/means of transport with high-performance engines (EURO 4 or EURO 5) and driving at low speed (maximum 30 km/h), especially on dirt or gravel roads (in very dry periods, it is recommended to spray them with water).

  - tipper trucks must be fitted with tarpaulins during transportation.

  - the topsoil on the site of projects on which foundation works are being carried out will be stored and reused to restore the land to its original state after the works have been completed.

  - dust-intensive technological processes will be reduced during periods of strong winds, or more intense wetting of surfaces under the action of working machinery or access roads, especially unpaved ones, will be pursued.

  - the application of paints on building elements shall be done by using minimum quantities of paint, primer and thinners and application with special devices that ensure minimum VOC emissions to the atmosphere.

There will be no emission sources of air pollutants during the operation of the plan.

Occasional maintenance operations may be carried out. These operations generate emissions of air pollutants mainly from vehicles used to move staff, but their contribution is insignificant.

In the decommissioning phase, the impact on air is similar to the construction phase.

Emissions into water

Sources of construction pollution that can affect water resources include:

  - improper management and storage of domestic waste water from the green toilets used on site;

  - improper disposal of construction waste;

  - excavation works, which can pollute surface water with small particles;

  - improper handling or storage of construction materials (concrete, earth, sand, etc.), which may end up in surface water carried by stormwater;

  - Accidental spills of fuels and lubricants from construction vehicles and machinery.

Manipulation of land masses (digging, grading, compaction) could have a minor impact on surface water quality through the deposition of dust sediments.

The works necessary to implement the plan are not significant sources of impact on groundwater and surface water quality.

To reduce the negative impact on water quality during the construction of the wind farm, the following measures are proposed:

  - the disposal of domestic fecaloid wastewater will be done in portable ecological toilets, will be emptied and transported to the nearest treatment plant.

  - construction materials shall not be stored in the vicinity of watercourses, in order to prevent deterioration of the status of water bodies (ecological status/ecological potential and chemical status for surface water bodies, respectively quantitative status and chemical status for groundwater bodies).

  - The waste will be selectively collected and handed over to specialized companies to prevent spills.

  - all works shall be supervised in such a way as to prevent possible contamination of the area due to accidental spillage of fuels or lubricants from equipment/utilities used in the works. In the event of accidental pollution, immediate action will be taken with absorbents/neutralizers, and breakdowns of means of transport and/or machinery will be remedied in specialized service units.

  - the works of undercrossing the watercourses (Cerchez and Magura rivers) will be carried out by horizontal drilling at a depth of 1 m between the upper generator of the protection pipeline and the watercourse slope

Emissions to soil/soil

The main potential sources of soil and subsoil contamination or degradation are:

  - accidental loss of petroleum products from construction machinery or motor vehicles transporting raw materials and materials.

  - uncontrolled storage of raw materials or construction waste directly on the ground.

Soil and subsoil contamination from construction works could only occur in accidental situations. To manage such situations, it is recommended to provide absorbent materials.

Noise and vibration

During the construction phase, the main form of physical pollution associated with the analyzed plan will be the noise and vibration generated by the operation of certain installations, equipment and vehicles during the construction period.

Noise level regulated by SR 10009:2017/C91:2020, "Acoustics. Permissible limits of environmental noise level" is 65 dB(A) at the site boundary. According to the Order of the Ministry of Health No. 119/ 2014 for the approval of the Hygiene and Public Health Norms on the living environment of the population, the continuous equivalent sound level measured outside the dwelling according to SR ISO 1996-2:2018, at 1.5 m above the ground, shall not exceed 55 dB(A) and the noise curve Cz 50. During the night (23:00 - 7:00), the continuous equivalent sound level shall not exceed 45 dB(A) and the noise curve Cz 40.

In terms of the effects of noise on wildlife, currently there is still a low level of knowledge. Barber et al. (2010) indicate that an increase in noise (relative to natural background noise) of 3 to 10 dB(A) can result in a reduction in wildlife alerting distances of 30 to 90%. However, most of the data to support this claim comes from the investigation of road noise. Less is known about the effects of intermittent noise over short periods of time. However, it is known that higher noise levels generally lead to the removal of wildlife species from the area, thus Foreman et al. (1998) indicate that a decline in grassland birds occurs at noise equivalent levels above 48 dB(A) and in woodland birds at levels above 42 dB(A).

The noise limit values according to current legislation and standards have been summarized in the following table.

Table  1-6: Reference values for noise level - db(A)

| Legislation | LAeq 31.5Hz | LAeq 63Hz | LAeq 125Hz | LAeq 250Hz | LAeq 500Hz | LAeq 1kHz | LAeq 2kHz | LAeq 4kHz | LAeq 8kHz | LAeq |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order 119/2014 - for residential areas - day | - | - | - | - | - | - | - | - | - | 55 |
| Order 119/2014 - for residential areas - night | - | - | - | - | - | - | - | - | - | 45 |
| SR 6156:2020 - for the hygiene and public health of the living environment of the population* | 89,4 | 75 | 65,2 | 58,5 | 53,5 | 50 | 47,2 | 45,2 | 43,5 | - |
| SR 10009:2017/C91:2020- for outdoor markets, commercial premises and catering/industrial premises | - | - | - | - | - | - | - | - | - | 65 |
| SR 6156:2020 - for markets, commercial and restaurant/industrial premises** | 96,3 | 82,9 | 74,2 | 67,8 | 63,2 | 60 | 57,4 | 55,4 | 53,8 | - |

The main sources of noise and vibration on site during the works will be:

- the operation of earth-moving machinery used to lay turbine platforms and foundations;

- handling of turbine components.

Noise sources will be temporary. In order to assess the noise level generated by the execution of the project, a worst-case situation has been considered, i.e. the operation of all equipment and machinery involved in the construction activities in three different work fronts: one for the excavation and preparation of the ground, one for the execution of the foundations and platforms of the turbines and one for the actual assembly of the installations.

According to data and information from the literature and similar projects, the machinery involved in the construction of wind infrastructure and the associated noise levels are represented by:

- Excavators - 2 pcs. (117 dB);

- Bulldozers - 2 pcs. (115 dB);

- Autogreder - 1 pc. (112 dB);

- Compaction cylinder - 4 pcs. (105 dB);

- Dumpers - 7 pcs. (107 dB);

- Truck crane - 1 pc. (96 dB);

- Automacara - 1 pcs. (83 dB);

- Truck mixer - 2 pcs. (107 dB);

- Hydraulic pile drill - 2 pcs. (120 dB);

- Autotrailer - 3 pcs. (90 dB);

- Excavator - 2 pcs. (116 dB);

- Ground stabilizing equipment - 1 pc. (105 dB).

In order to evaluate the noise level generated in the above scenario, noise source modeling was performed using the software application SoundPLANnoise 9.0. The input data used were represented by:

- digital terrain model of the analyzed area - ASCII format;

- position of the detailed scenario noise sources (STEREO 70 projection coordinates);

- position of the sensitive receivers relative to the noise sources (coordinates in STEREO 70 projection);

- information from the literature on the noise level of each type of equipment and machinery that are sources of noise.

Normally construction work will take place during the day between 08:00 and 18:00. There are, however, operations that need to be carried out continuously, such as pouring concrete for foundations, which may also require night work.

The results of the modeling carried out with the SoundPLANnoise software show that, during the construction phase, the noise level generated by the project will not generate a significant impact on the quality of living in the neighboring villages, at the level of the nearest receptors, the operation of the equipment used in the modeling generating a maximum noise level of about 36 dB. The noise generated by the construction activities is not able to change the current noise level induced mainly by the motor traffic on the DJ391.

Figure 1-10 Area of direct influence

Also in the most unfavorable situation (illustrated in the previous figure) it is highlighted that the area of significant impact where exceedances of the daytime limit values of 55 dB may occur (work being carried out exclusively during the day), has a maximum area of approximately 4.92 ha located strictly at the working front, at a maximum distance of 300 m from the working front.

No studies have been carried out in the analyzed area to determine the actual noise level. According to the literature a rural background pollution value can be considered as 40 dB during the day and 30 dB at night (Ir. F.W.J. van Deventer, 2014, Handboek Basiskennis Vliegtuiggeluid).

Taking into account the background noise level in the project area (rural area without significant noise sources) of 40 dB(A), the noise level resulting at sensitive receptors from modeling and applying the calculation formula for summing the noise levels.

L=10×log10(10L110+10L110)  dB

It results that the project will not have a major contribution on the current noise level at the identified sensitive receptors in the area, the calculation indicating very small changes, i.e.: between 40.008 dB to 40.016 dB.

Also, for the nearest identified protected natural areas in the project area (BG0000569 Kardam) the noise level estimated from modeling is 27 dB(A) which corresponds to a change on the current noise level of 0.2 dB (40.2 dB).

The modeling results indicate that the project will not have a significant adverse impact on sensitive receptors identified in the study area. For both the social and faunal components, it should be noted that the noise sources that have been estimated using the SoundPLANnoise program are temporary in nature, being present on the site only during the project execution period (about 30 days).

The noise sources presented above may have a potential negative impact on the staff directly involved in the specific project activities. For them, noise discomfort can be minimized by complying with occupational health and safety rules, i.e. by using personal noise protection equipment.

During operation, the noise produced by the wind turbine increases with wind speed and rotational speed. Modern turbine design incorporates nacelle insulation to prevent mechanical noise from being transmitted into the air. The nacelle is also insulated to prevent vibrations from moving parts (blades, hub, gearbox) from being transmitted into the tower and foundation. The aerodynamic noise of large turbines is quite dominant compared to mechanical noise and is dependent on the rotational speed of the blades (wind speed).

In general the noise level of a turbine varies between 92 - 107.7 dB. For the 6.6 MW turbine at 95% load the maximum noise level is 106.6 dB (level according to the noise study). From the noise modeling, in the receptor area (settlements in the area of implementation of the PUZ) the noise level is less than 40 dB, below the limit values set by Order No. 119/2014 for the approval of the Hygiene and Public Health Norms on the living environment of the population.

In the decommissioning phase, the impact of noise and vibration is similar to the construction phase.

The construction works will be carried out exclusively during daylight hours and only security lighting will be used for site organization, there will be no additional effects of increased light intensity other than those already existing.

### Waste generated by PP and how to manage it

During the execution phase of the investment foreseen by the PUZ (Construction), the waste generated will be selectively collected and temporarily stored in specially designated areas, for disposal / recovery by specialized companies, authorized, on a contract basis.

The categories of waste generated at this stage are as follows:

- Construction waste:

  - scrap metal (ferrous and non-ferrous) (code 17 04 07 iron and steel:);

  - excavated earth (code 17 05 04: earth and stones);

  - Concrete waste (code 17 01 01 Concrete waste);

  - waste insulation materials (code 17 06 04 insulation materials other than those mentioned in 17 06 01 and 17 06 03);

  - Waste electrical cables, waste electrical conductors (code 17 04 11Cables other than those mentioned in 17 04 10);

  - wood waste (code 17 02 01 wood);

  - plastics (code 17 02 03);

- Packaging waste:

  - paper/cardboard (code 15 01 01);

  - wood (code 15 01 03): pallets of wooden slats, wooden drums, wooden crates

  - plastics (code 15 01 02);

  - packaging containing residues or contaminated with dangerous substances (code 15 01 10*);

- Absorbents (for accidental oil spills) (code 15 02 02*);

- Household wastes arising from the social activities of staff involved in carrying out the work (code 20 03 01).

Waste oils from the machinery/equipment used to carry out the works, other engine, transmission and lubricating oils - code 13 02 08*), as well as used tires (16 01 03) and used batteries and accumulators (16 06 05) from the machinery/machinery/equipment involved in the works will not be generated on site, but in special service type sections. Oil, tires, batteries and accumulators will not be changed on the PUZ site.

Ferrous and non-ferrous metal waste will be collected and temporarily stored on the site organization's premises, on a waterproofed and covered surface and will be recovered by authorized economic operators.

Waste from construction materials (concrete waste) will be temporarily stored on site, in the area specially designed for each working face, to be used for backfilling the foundation pits.

The surplus soil resulting from the excavation of the foundation pits will be used as backfill for the vertical systematization.

Cable, conductor and insulation scrap will be collected at the site organization and handed over to an authorized economic operator.

Packaging waste will be recovered by authorized companies. Re-usable packaging (pallets, drums and wooden crates) will be temporarily stored within the site organization.

Household waste resulting from the personnel involved in the implementation of the plan, from the work points, will be collected in polyethylene bags and transferred daily in Eurocontainer or Eurobins, placed on a waterproofed surface and without leakage on the ground, on the site organization premises, from where they will be handed over to an authorized economic operator.

During the operation phase of the investment foreseen by the PUZ, there may be waste from maintenance activities as a result of repair work on mechanical, electrical and automation equipment, such waste being:

- waste oils (hydraulic 13 01 10* and transmission 13 02 05*);

- waxes and used fats (petroleum jelly) - 12 01 12*;

- other fluids - used brake fluid -16 01 13*;

- Discarded electronic and electrical equipment - 16 02 14*;

- packaging containing residues or contaminated with dangerous substances from category 15 01 10*;

- cable and conductor scrap - 17 04 11;

- absorbents, filter materials (including oil filters not otherwise specified), polishing materials, protective clothing 15 02 02*;

Oil change / topping up will be carried out as needed, depending on the technical specifications of the turbine, by specialized companies in the field, with which the wind farm manager will conclude a service and maintenance contract.

It should be noted that both quantitatively and qualitatively the resulting waste does not constitute a major problem from the point of view of environmental protection. All waste resulting from the site during both the current operation and maintenance periods will be collected in containers and transferred to a company specialized in waste storage and treatment.

Waste resulting from wind farm maintenance activities will not be deposited on the ground. It will be collected in special containers and removed from the site.

During the decommissioning phase of the wind farm foreseen by the PUZ, materials and equipment will be recovered and valorized as follows:

- poles - scrap metal - 17 04 07 - will be recovered as scrap metal at specialized centers.

- cable waste, conductor scrap -17 04 11 - will be recovered as recyclable metals at specialized centers;

- insulating elements - 17 06 04 - shall be removed by authorized companies;

- concrete resulting from the breaking of foundations - 17 01 07 - will be disposed of in inert landfills or as directed by the local authority.

- waste oils (hydraulic 13 01 10* and transmission 13 02 05*)- will be recovered/disposed of by authorized companies;

- Used waxes and fats (petroleum jelly) - 12 01 12* - to be recovered/disposed of by authorized companies;

- other fluids - used brake fluid -16 01 13* - will be recovered/disposed of by authorized companies;

- Scrapped electronic and electrical equipment - 16 02 14* - will be recovered/disposed of by authorized companies;

- parts and components of turbines, 10 11 03 - to be reclaimed by authorized companies

### Land use requirements necessary for the execution of PP

The land on which it is intended to realize the investment proposed by PUZ are the private property of individuals / legal entities with which the beneficiary of the investment SOUTH WIND SRL, has concluded surface contracts.

According to CU no. 129/28/11/2022 issued by Constanta County Council, the current use of the land is "arable", and the destination established by the approved urban and spatial planning plans is "land located in extravilan (TDE) - agricultural and electricity production area - wind farm and related constructions", with permitted functions: agricultural, renewable energy production, circulation and related facilities.

The total area of the proposed plan is 3,027.77 ha.

The area studied for the implementation of the objective includes agricultural land for arable land and land with special destination - exploitation roads in the public domain of the territorial administrative unit of Cerchezu commune, managed by the Local Council of Cerchezu commune and the public domain of county interest.

The plots of land on which the investment to be developed are represented by the parcels shown in the following table (see Situation Plan, Annex 3):

Table  1-7 Plots affected by investment works

| Investments | Go to |  |
| --- | --- | --- |
| Wind turbines | - T1 - A 439/2/15 - T15 - A 208/1/20 - T2 - A 411/1/10 - T16 - A 208/1/13 - central T3 - A 439/1/16 - T17 - A 78/22+A 78/23 - canceled - central T4 - A 439/3/3 - T18 - A 177/23 - T5 - A 208/2/3 - T19 - A 177/10 - T6 - A 430/2/15 - T20 - A 169/27 - T7 central - A 98/2/4/2 - T21 - A 359/12 - T8 - A 430/1/17 - T22 - A 359/3 - T9 - A 98/1/8 - T23 - A 359/25 - canceled - T10 - A 207/7 - T24 - A 363/2/9 - T11 - A 195/25 - T25 - A 363/1/23 - T12 - A 208/2/21 - T26 - A 72/4 - T13 - A 179/10/1 - T27 - A 90/9 | - T14 - A 174/11 - T28 - A 363/2/30; - T29 - A 367/49; - T30 - A 72/34; - T31 - A 367/12/2 - T32 central - A 9/12 lot1,A9/12 lot2,A9/12 lot3 - T33 - A 78/10; - T34 central - A 1/20/1, A 1/20/2 - T35 - A 277/2/11 - T43 - A 338/2/6 - T36 - A 277/2/3 - T44 - A 338/3/5; - T37 - A 124/11 - T45 - A 335/1/3; - T38 - A 250/26 - T46 - A 314/1/18/1, A 314/1/18/2; - T39 - A 251/28/1 - T47 - A 314/1/12 - T40 - A 250/10 - T48 - A 411/2/4; - T41 - A 338/1/21 - T42 - A 379/7/5 |
| Transformer stations | - SS01 - A5/28 - SS02 - A251/1/26 - canceled - SS03 - A177/16 - newly proposed (near turbine T19) | - SS03 - A367/49 will be subject to another authorization procedure - SS04 - A207/11 |
| Site organization (temporary) | - OS1 - A251/1/31 - OS2 - A367/15 - OS3 - A250/9 |  |
| Landscaped access roads on plots | - A163/18/2 - A277/4/7 - A251/31 |  |

Access to the site is from the existing national road (DN), communal roads (DC) and exploitation roads (De) listed below:

| Road type | Site access |
| --- | --- |
| National road | DN38 Constanta - Negru Voda |
| County Road | DJ391 Mangalia - Negru Voda - Cobadin - Tuzla |
| Existing municipal roads | DC13, DC16, DC17, DC18, DC19 |
| Existing logging roads | From 357, From 64, From 68, From 77, From 75/20, From 78/3, From 87, From 78/1, From 94/1, From 163/1,From 181, From 176, From 168, From 173, From169/1, From 172, From 208/1/1, From 198, From 195/16, From 98/1/1, From 98/2/1, From 430/1/1, From 430/2/1, From 432, From 440, From 439/2/1, From 411/1/1, From 439/1/1, From 224/4, From 251/1, From 130/2, From 130/1, From 250/1, From 130/1, From 129, From 128, From 125, From 271, From 258/10, From 379/2/1, From 361, From 356/1, From 356, From 356/2, From 339/1, From 339, From 337, From 336, From 314/1/1, From 315, From 277/8, From 277/7. |
| Landscaped access roads on plots | A163/18/2, A277/4/7 and A251/31. |

The main function of the studied area is agriculture, the agricultural land with the current arable function has an area of 2,908.24 ha, representing 96.05% of the total area studied. The land use categories are shown in the following table and figure.

|  | Current use categories; ha. Pasture; 64. Irrigation canal; 11,77. Major traffic (DN+Dc); 5,24. Roads (De); 36,65. Unproductive land; 1,87. Arable land; 2908,24. Total; 3027,77. |
| --- | --- |

Figure 1-11 Map of available renewable resources by region

For each plot on which the wind turbines will be installed, a request will be made in the DTAC phase for the permanent removal from agriculture of the areas occupied by the base of the pillar, the access roads to the turbines and the maintenance platforms. The energy park will lead to the definitive removal from agriculture of an area of approx. 30 ha.

For the areas occupied by the component parts storage and crane assembly platforms, temporary set-aside will be required. The energy park will lead to the temporary set-aside of approx. 70 ha.

The map with the distribution of land use categories in the PUZ coverage area is represented in the figure below.

Source: CORINE Land Cover, 2018, Version 2020_20u1mDate of publication: Jun 14, 2019 Revision date: May 13, 2020

Figure 1-12 Distribution of land use categories in the PUZ coverage area

Concerning the distribution of land use categories, the study area represents an area of arable land (majority - 90%), secondary grassland, predominantly agricultural land mixed with natural vegetation, discontinuous urban space and industrial or commercial units.

### Additional services required for PP implementation

The realization of the wind farm project envisaged by the PUZ does not require additional services such as: decommissioning / relocation of pipelines, high voltage lines, changes in the route of railroads or roads, etc.

The connection of the wind turbines will be detailed in the technical design.

### Activities generated as a result of PP implementation

The implementation of the PUZ generates the following activities:

- Elaboration of the technical implementation project;

- Obtaining permits, agreements and building authorization;

- The implementation of the wind farm generates the following activities:

  - Transportation of construction equipment and materials;

  - Construction activities - assembly;

  - Biodiversity/environmental impact monitoring activities in the area;

  - Activities of electricity generation using wind turbines;

  - Maintenance activities for wind turbines;

  - Waste collection and transportation activities during project implementation.

The following table summarizes, by life cycle stages, the main interventions that will result from the construction of the wind farm:

Table  1-8 Activities generated as a result of PP implementation

| Stage | Intervention type | Activities included |
| --- | --- | --- |
| Construction | Construction of access roads | - Clearing vegetation - Topsoil stripping works - Depositing uncovered soil - Compaction/leveling works - Ballast layer laying works - Site traffic of vehicles/operation of necessary machinery |
|  | Building technological platforms and foundations | - Clearing vegetation - Topsoil stripping works - Depositing uncovered soil - Compaction/leveling works - Ballast/foundation works - Site traffic of vehicles/operation of necessary machinery |
|  | Installing wind power plants | - Fitting the pillar, nacelle and blades - Running the necessary machinery |
|  | Making electrical connections and laying underground cables between the wind generator sets | - Clearing vegetation - Topsoil stripping works - Trench digging works - Disposal of excavated soil - Cable installation work - Filling works - Levelling works - Site traffic of vehicles/operation of necessary machinery |
|  | Building transformer substations | - Clearing vegetation - Topsoil stripping works - Foundation - excavation and concrete pouring - Disposal of excavated soil - Site traffic of vehicles/operation of necessary machinery - Installation of transformer station equipment - Fencing works |
|  | Site organization | - Landscaping - clearing vegetation - Topsoil stripping - Storage of uncovered soil - Ballast layer laying works - Compaction/leveling works - Site traffic of vehicles/operation of necessary machinery |
|  | Land reclamation work at the end of construction | - Restoration of the site where works and rehabilitation of temporarily used areas. - Work to remove all plant, machinery and waste and to reinstate topsoil on areas that have been temporarily used |
| OPER | Maintenance and repairs | - Vehicle traffic/operation of necessary machinery/equipment - Intervention and repair works wind power plants - Current repairs, equipment replacement - Replacing oils/lubricants - Disposal of repair and maintenance waste |
| DEFEAT Interventions and activities are similar to those in the build | Site organization | - Landscaping - clearing vegetation - Topsoil stripping - Storage of uncovered soil - Ballast layer laying works - Compaction/leveling works - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Dismantling wind farms | - Removing the blades, nacelle and pole |
|  | Decommissioning of technological platforms and foundations | - Clearing vegetation - Topsoil stripping works - Depositing uncovered soil - Foundation demolition works - Compaction/leveling works - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Dismantling electrical connections and removing underground cables | - Clearing vegetation - Topsoil stripping works - Trench digging works - Disposal of excavated soil - Removing electrical cables - Trench filling and land levelling works - Restoring the green carpet. - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Decommissioning of transformer stations | - Dismantling transformer substation equipment - Clearing vegetation - Excavation work - Disposal of excavated soil - Demolition of foundations - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Decommissioning of logging roads | - Topsoil stripping works - Disposal of excavated soil - Ballast layer removal works - Compaction/leveling works - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Site restoration work on completion of construction | - Restoration of the site where works and rehabilitation of temporarily used areas. - Work to remove all plant, machinery and waste and to reinstate topsoil on areas that have been temporarily used |

Local labor will be used as much as possible for these activities.

### Description of PP technological processes

The investment foreseen by the PUZ has a projected lifetime of at least 30 years, and the execution and commissioning period of the plan is 24 months.

The main works of the technological process of construction of the wind farm foreseen by the PUZ are described in detail in chapter 1.1.4 Description of the life cycle of the PP.

The technological process of wind power generation is the transfer of the kinetic energy of the wind kinetic energy of a wind turbine through its blades.

Thus, the principle of operation is that the wind drives the turbine blades, which in turn drive the turbine generator. In order to multiply the speed of action on the central axis, a speed multiplier is also included in the system.

The power generated by a wind turbine is directly proportional to the wind density, the area covered by a full movement of the rotor blades and the square of the wind speed.

The energy flow is as follows: kinetic energy of the wind → mechanical energy of the rotor → electrical energy of the generator → distribution network → (storage of batteries) → isolated loads (e.g. isolated villages).

### Characteristics of existing, proposed or approved PPs that may generate cumulative impact with the PP that is under assessment and may affect ANPIC

Regarding the relation with other plans and programs both on the territory of Romania and Bulgaria, according to the available information, in the vicinity of the proposed wind farm there are the following existing/proposed wind farms (see Figure : 1-6.)

- Existing:

  - "WIND ENERGY PARK 32 WIND POWER PLANTS, TOTAL POWER 80 MW, TRANSFORMATION STATION, ELECTRIC CONNECTION NETWORKS, CONSTRUCTION AND MODERNIZATION OF COMMUNICATION AND ACCESS ROADS", located in the municipalities of Chirnogeni and Independența. The distance between the proposed wind farm and the existing wind farm is approximately 1.5 km (distance between the proposed T24 turbine and the existing T1).

  - Wind farm on the territory of Bulgaria, located 5.6 km away from the proposed project (distance between the proposed T3 turbine and the existing T6).

- Proposed:

In order to assess the cumulative impact on the environment and on protected natural areas for the Zonal Urban Development Plan "Wind Energy Park 48 (46) CE, Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads", proposed to be located in Cerchezu, Constanta County, the competent authority for environmental protection Constanta was requested by address no./10.05.2024 information regarding the projects and plans/strategies approved/approved in the period 2020-2024 as well as those in the process of approval located on the territory of Cerchezu and the territory of neighboring localities (Măgura, Căscioarele, Viroaga, Chirnogeni, Negru Voda, Olteni, Dumbrăveni, Independența, Plopeni), operators carrying out activities on the administrative territory of Cerchezu, projects and plans/strategies approved/endorsed/ pending approval located in the protected areas located in the area of influence of the respective PUZ: ROSAC0071(ROSCI0071), ROSPA0036, ROSPA0166, ROSPA0166, ROSPA0094, ROSAC0157(ROSCI0157), data on the location of projects/plans in relation to the boundaries of these protected areas and the areas occupied by them within the boundaries of the protected areas, etc.

By APM Constanta address no. 5836/ 12.06.2024 the following information related to the projects were submitted:

  - Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța,

  - Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța,

  - Wind Farm Comana - Pecineaga - MV underground cables and fiber optic network,

  - Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța,

  - Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB location, internal road and access, land fencing and site organization located in Independența,

  - Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni,

  - Construction of underground fiber optic communications network for electronic services, Negru Voda,

  - Electricity supply to water household, Furnica village, Dumbrăveni commune, jud. Constanța,

  - General Urban Plan of Independența commune,

  - Setting up a smart natural gas distribution network in Independența commune, Constanta county,

  - Construction of a bridge in Movila Verde, Church Street, Movila Verde, Independența commune,

  - Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County

  - Construction of bridges, culverts and paving of rainwater drainage channels in Credința, Chirnogeni commune,

  - Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county.

- -RIG SERVICE SA,,, PUZ - WIND ENERGY PARK 17 CE - APPROX. 112,2 MW, TRANSFORMER STATION, ELECTRICAL CONNECTION NETWORKS, CONSTRUCTION AND MODERNIZATION OF COMMUNICATION AND ACCESS ROADS", IN INDEPENDENTA COMMUNE, EXTRAVILAN, CONSTANTA COUNTY

- -RCS & RDS SA - CONSTRUCTION OF UNDERGROUND FIBER OPTIC NETWORK FOR ELECTRONIC COMMUNICATIONS SERVICES'', IN CONSTANTA COUNTY, INDEPENDENTA COMMUNE, DUMBRAVENI.

- - INDEPENDENTA COMMUNE - GENERAL URBAN PLAN OF INDEPENDENTA COMMUNE

- - EMERGY STRIBOG SRL - CONSTRUCTION OF A WIND POWER PLANT INCLUDING: WIND TURBTNE, ACCESS ROADS AND ASSEMBLY/MAINTENANCE PLATFORMS, TRANSFORMER SUBSTATION (OWN), MEDIUM AND HIGH VOLTAGE ELECTRICAL NETWORK, TELECOMMUNICATION NETWORK", COM. INDEPENDENTA, JUD. CONSTANTA.

- -UAT INDEPENDENTA -,,, SETTING UP AN INTELLIGENT NATURAL GAS DISTRIBUTION NETWORK IN INDEPENDENTA COMMUNE, CONSTANTA COUNTY"

- -COMUNA INDEPENDENTA,, BUILDING A BRIDGE IN MOBILA VERDE, STR. BISERRCII, LOCALITATEA MOVILA VERDE, COMUNA INDEPENDENTA, JUDETUL CONSTANTA"

- -RAJA - ELECTRICITY SUPPLY FOR WATER HOUSEHOLD, FURNICA VILLAGE, COMMUNE DUMBRAVENI JUD. CONSTANTA

- -AGROTECH INVESTMENT SRL -,,DEMOLITION OF EXISTING BUILDINGS AND CONSTRUCTION OF PHOTOVOLTAIC POWER PLANT 3,212 MW, CONNECTION TO SEN, PC AND PTAB LOCATION, INTERNAL ROAD AND ACCESS, FENCING LAND AND SITE ORGANIZATION LOCATED IN THE LOCALITY INDEPENDENTA''

- -RCS&RDS SRL - CONSTRUCTION OF UNDERGROUND FIBER OPTIC NETWORK FOR ELECTRONIC COMMUNICATIONS SERVICES NEGRU VODA

- -ROMCM S.A. RELOCATION OF THE INTERNATIONAL NATURAL GAS TRANSPORTATION PIPELINE DN 1000T1 ISACCEA - NEGRU VODA, IN ROMCIM QUARRY AREA, PL MEDGIDIA"

- -PECINEAGA ENERGIES SRL. -,, WIND FARM COMANA - "PECINEAGA - UNDERGROUND ELECTRIC CABLES ROUTE MT AND FIBER OPTIC NETWORK'' PECINEAGA ENERGIES SRL

- -THE MUNICIPALITY OF COMANA COMMUNE THROUGH THE REPRESENTATIVE OF THE MAYOR ADAM ION - ASPHALTING STREETS COMANA, TATARU AND PELINU, STAGE I, COMANA COMMUNE, CONSTANTA COUNTY"

-COMUNA CHIRNOGENI - CONSTRUCTION OF BRIDGES, CULVERTS AND PAVING OF STORM WATER DRAINAGE CANALS IN THE LOCALITY OF CREDINTA", LOCALITY OF CREDINTA, COMMUNE OF CHIRNOGENI.

Figure 1-13 Existing, proposed or approved PPs likely to generate cumulative impacts with PPs

### Other information requested by ACPM

At the time of this study no further information has been requested on the proposed plan. We note, however, that it is possible that as the process and technical review meetings proceed, other new information may come to light that will be incorporated into the study.

### Summary of effects generated by the implementation of the PP

The plan does not intersect any Natura 2000 sites. In the vicinity of the PUZ at a distance of less than 5 km a number of 3 natural areas of community interest have been identified:

- 1 area situated on the territory of Romania ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake at a distance of about 2,79 km from the boundary of the PUZ-

- 2 areas on the territory of Bulgaria - BG0000569 Kardam (Bulgaria) at a distance of about 410 m from the boundary of the PUP and BG0000570 Izvorovo - Kraishte at a distance of about 3.64 km from the boundary of the PUP).

There are 5 natural protected areas within the Plan implementation area at a distance of 5-20 km:

- 5 protected natural areas for birdlife protection:

  - ROSPA0166 Plopeni-Chirnogeni at a distance of 6.21 km from the PUZ boundary

  - ROSPA0036 Dumbrăveni located at a distance of 7.07 km from the PUZ boundary

  - ROSPA0094 	Hagieni Forest at a distance of 18,39 km from the PUZ boundary

  - ROSPA0151 	Ciobănița-Osmancea at a distance of 18,55 km from the PUZ boundary

  - ROSPA0001 	Aliman - Adamclisi. at a distance of 19.12 km from the PUZ boundary

- 2 protected natural areas of Community interest:

  - ROSAC0157 Hagieni Forest - Cotul Văii at a distance of 13.81 km from the PUZ boundary,

  - BG0000130 Kraymorska Dobrudzha (Bulgaria) at a distance of 11 km from the PUZ boundary

According to the provisions of Annex no.6A of the Methodological Guide on the appropriate assessment of the potential effects of plans or projects on protected natural areas of community interest, integrated part of Order no. 1682/2023, the following criteria were applied to identify the PPAs potentially affected by the PUZ:

- intersection;

- neighborhood (area of influence);

- species mobility;

- ecological connectivity.

This identification was done using GIS spatial analysis. The identification of the ANPIC depended on the nature of the proposed PUZ, on the distance at which effects may occur and is carried out with a minimum set of data and information, such as: the location of the perimeter studied by the PUZ, the boundaries of the ANPIC, the boundaries of water bodies (aquatic connectivity zones) and the location of ecological corridors/areas with ecological connectivity/continuity.

Potential forms of impact considered:

- PH - habitat loss: permanent occupation of habitats or habitats favorable to the species;

- AH - habitat alteration;

- FH - habitat fragmentation;

- PAS - disturbing species activity: nesting, feeding, resting;

- REP - reduction in population numbers: casualties from machinery, nest destruction, collision with machinery

The entire list of protected natural areas has been subjected to an analysis of the possibility of being affected by the implementation of the Plan in terms of the mobility of species present in the areas, the existence of ecological corridors that could link the area of works and protected natural areas, and the identification of restrictive measures in the management plans of protected natural areas.

Analyzing the distance and the species/habitats for which these areas have been designated and taking into account the presentation memorandum as well as the initial classification decision no. 32/12.05.2024 issued by the Constanța Environmental Protection Agency, it has been identified that a number of five protected natural areas in Romania (ROSAC0071/ROSCI0071, ROSPA0036, ROSPA0094, ROSAC0157/ROSCI0157, ROSPA0166) and two areas in Bulgaria (BG0000569, BG0000570) may be potentially affected directly/indirectly by the PUZ, which have been analyzed in the following sections.

Table  1-9 Tabular presentation of PP interventions and components

| Stage | Type of intervention | Component | Location | Distance to the nearest ANPIC (km) |  |  |  |  |  |  | Other additional information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ROSAC0071 | ROSPA0036 | ROSPA0094 | ROSAC0157 | ROSPA0166 | BG0000569 | BG0000570 |  |
| Construction | Site organization and management (including site traffic) | Site organization | OS1 - turbine area T39 | 7,47 | 12,10 | 23,93 | 1967 | 12,90 | 3,72 | 8,36 | - |
|  |  |  | OS2 - turbine area T31 | 7,41 | 10,13 | 23,33 | 19,54 | 9,65 | 7,57 | 9,69 | - |
|  |  |  | OS3 - T40 turbine area | 6,50 | 11,63 | 25,25 | 20,80 | 14,12 | 3,67 | 6,89 | - |
|  |  | Construction site traffic | Turbine operation road | 3,22 | 8,27 | 18,79 | 14,10 | 7,12 | 0,74 | 29,42 | - |
|  | Building access roads | Reinforcement of the existing farm roads | Turbine operation road | 3,22 | 8,27 | 18,79 | 14,10 | 7,12 | 0,74 | 29,42 | - |
|  |  | Building access roads on the plots where the wind farms will be built | Turbine access road | 3,22 | 8,27 | 18,79 | 14,10 | 7,12 | 0,74 | 29,42 | - |
|  | Earthworks (land levelling, excavation, excavation, backfilling) | Transformer station | Transformer Station 1 | 4,48 | 7,90 | 25,92 | 22,12 | 11,17 | 8,25 | 8,27 | - |
|  |  |  | Transformer Station 2 (canceled) | 7,59 | 12,27 | 24,11 | 19,75 | 12,27 | 3,97 | 8,27 | - |
|  |  |  | Transformer Station 3 A177/16 - newly proposed (close to turbine T19)SS03 - A367/49 will be subject to another authorization procedure | 10,46 | 13,97 | 20,32 | 16,31 | 9,65 | 5,77 | 11,99 | - |
|  |  |  | Transformer Station 4 | 10,74 | 15,08 | 20,93 | 16,38 | 11,99 | 3,38 | 11,57 | - |
|  |  | Turbine platform | T1 turbine platform | 12,75 | 17,55 | 20,70 | 15,36 | 14,16 | 2,50 | 12,72 | - |
|  |  |  | T2 turbine platform | 11,52 | 16,53 | 22,04 | 16,76 | 14,57 | 1,12 | 11,34 | - |
|  |  |  | T3 turbine platform | 13,35 | 18,18 | 20,53 | 15,01 | 14,53 | 2,46 | 13,18 | - |
|  |  |  | T4 turbine platform | 11,84 | 16,66 | 21,30 | 16,12 | 13,96 | 1,86 | 11,91 | - |
|  |  |  | T5 turbine platform | 8,48 | 12,92 | 22,88 | 18,59 | 12,29 | 3,44 | 9,36 | - |
|  |  |  | T6 turbine platform | 11,71 | 16,19 | 20,54 | 15,66 | 12,62 | 2,86 | 12,25 | - |
|  |  |  | T7 turbine platform | 13,01 | 17,28 | 19,14 | 14,24 | 12,11 | 4,10 | 13,65 | - |
|  |  |  | T8 turbine platform | 11,83 | 16,09 | 20,03 | 15,33 | 11,87 | 3,57 | 12,54 | - |
|  |  |  | T9 turbine platform | 12,86 | 16,89 | 18,82 | 14,19 | 11,27 | 4,66 | 13,70 | - |
|  |  |  | T10 turbine platform | 11,06 | 15,28 | 20,51 | 16,02 | 11,63 | 3,56 | 11,88 | - |
|  |  |  | T11 turbine platform | 11,99 | 15,79 | 19,16 | 14,96 | 10,35 | 5,02 | 13,09 | - |
|  |  |  | T12 turbine platform | 10,02 | 14,19 | 21,34 | 16,99 | 11,51 | 3,71 | 10,95 | - |
|  |  |  | T13 turbine platform | 11,60 | 15,00 | 19,21 | 15,18 | 9,19 | 5,98 | 12,98 | - |
|  |  |  | T14 turbine platform | 10,66 | 14,28 | 20,25 | 16,19 | 9,94 | 5,22 | 11,94 | - |
|  |  |  | T15 turbine platform | 9,18 | 13,26 | 21,93 | 17,78 | 11,30 | 4,15 | 10,27 | - |
|  |  |  | T16 turbine platform | 8,52 | 12,69 | 22,55 | 18,44 | 11,61 | 4,12 | 9,64 | - |
|  |  |  | T17 turbine platform (canceled according to Transgaz opinion) | 11,05 | 14,76 | 18,92 | 14,99 | 8,17 | 6,98 | 13,35 | - |
|  |  |  | T18 turbine platform | 10,78 | 14,15 | 19,96 | 15,97 | 9,27 | 5,90 | 12,25 | - |
|  |  |  | T19 turbine platform | 10,78 | 14,15 | 19,96 | 15,97 | 9,27 | 5,90 | 12,25 | - |
|  |  |  | T20 turbine platform | 9,13 | 12,94 | 21,72 | 17,70 | 10,63 | 4,88 | 10,46 | - |
|  |  |  | T21 turbine platform | 9,96 | 14,42 | 18,82 | 15,04 | 7,09 | 8,05 | 13,73 | - |
|  |  |  | T22 turbine platform | 10,18 | 13,80 | 19,62 | 15,79 | 7,85 | 7,37 | 12,84 | - |
|  |  |  | T23 turbine platform (canceled according to Transgaz opinion) | 8,97 | 13,48 | 19,56 | 15,86 | 6,75 | 8,59 | 13,31 | - |
|  |  |  | T24 turbine platform | 9,07 | 12,81 | 20,33 | 16,59 | 7,44 | 8,12 | 12,44 | - |
|  |  |  | T25 turbine platform | 9,68 | 12,51 | 20,89 | 17,07 | 8,32 | 7,32 | 11,68 | - |
|  |  |  | T26 turbine platform | 8,90 | 11,65 | 21,78 | 17,97 | 8,75 | 7,30 | 10,89 | - |
|  |  |  | T27 turbine platform | 8,72 | 12,13 | 21,95 | 18,01 | 9,94 | 5,88 | 10,38 | - |
|  |  |  | T28 turbine platform | 8,98 | 11,93 | 21,27 | 17,52 | 7,98 | 7,91 | 11,62 | - |
|  |  |  | T29 turbine platform | 8,80 | 11,20 | 22,01 | 18,27 | 8,38 | 7,93 | 10,97 | - |
|  |  |  | T30 turbine platform | 8,10 | 10,93 | 22,57 | 18,75 | 9,32 | 7,20 | 10,13 | - |
|  |  |  | T31 turbine platform | 7,34 | 9,89 | 23,47 | 19,71 | 9,54 | 7,82 | 9,59 | - |
|  |  |  | T32 turbine platform | 5,43 | 7,48 | 25,65 | 21,94 | 10,87 | 9,16 | 7,95 | - |
|  |  |  | T33 turbine platform | 10,79 | 14,14 | 19,54 | 15,62 | 8,34 | 6,83 | 12,76 | - |
|  |  |  | T34 turbine platform | 4,01 | 8,34 | 26,51 | 22,57 | 13,15 | 7,10 | 5,91 | - |
|  |  |  | T35 turbine platform | 3,36 | 8,57 | 27,64 | 23,62 | 14,73 | 6,81 | 4,58 | - |
|  |  |  | T36 turbine platform | 3,80 | 8,72 | 27,04 | 23,04 | 14,13 | 6,59 | 5,20 | - |
|  |  |  | T37 turbine platform | 4,98 | 9,56 | 25,94 | 21,94 | 13,24 | 5,89 | 6,28 | - |
|  |  |  | T38 turbine platform | 6,24 | 10,95 | 24,84 | 20,69 | 13,15 | 4,57 | 7,19 | - |
|  |  |  | T39 turbine platform | 7,16 | 11,82 | 24,05 | 19,80 | 12,94 | 3,90 | 8,00 | - |
|  |  |  | T40 turbine platform | 6,49 | 11,60 | 25,25 | 20,82 | 14,16 | 3,75 | 6,90 | - |
|  |  |  | T41 turbine platform | 6,68 | 12,23 | 25,76 | 21,07 | 15,19 | 3,23 | 6,73 | - |
|  |  |  | T42 turbine platform | 7,70 | 13,44 | 25,45 | 20,50 | 15,76 | 2,20 | 7,36 | - |
|  |  |  | T43 turbine platform | 6,50 | 12,33 | 26,27 | 21,49 | 15,81 | 3,39 | 6,37 | - |
|  |  |  | T44 turbine platform | 5,97 | 12,11 | 27,20 | 22,41 | 16,49 | 4,17 | 5,57 | - |
|  |  |  | T45 turbine platform | 4,21 | 10,78 | 28,75 | 24,09 | 17,22 | 5,90 | 3,88 | - |
|  |  |  | T46 turbine platform | 3,31 | 9,91 | 29,22 | 24,71 | 17,17 | 6,69 | 3,11 | - |
|  |  |  | T47 turbine platform | 3,19 | 10,16 | 30,06 | 25,45 | 18,09 | 7,24 | 2,56 | - |
|  |  |  | T48 turbine platform | 10,85 | 15,88 | 22,49 | 17,32 | 14,48 | 0,72 | 10,73 | - |
|  | Foundation works | Turbine foundation | T1 turbine foundation | 12,76 | 17,56 | 20,77 | 15,43 | 14,19 | 2,64 | 12,83 | - |
|  |  |  | T2 turbine foundation | 11,66 | 16,72 | 22,09 | 16,77 | 14,77 | 1,30 | 11,52 | - |
|  |  |  | T3 turbine foundation | 13,46 | 18,33 | 20,63 | 15,08 | 14,76 | 2,58 | 13,31 | - |
|  |  |  | T4 turbine foundation | 11,90 | 16,71 | 21,32 | 16,14 | 13,98 | 2,07 | 12,08 | - |
|  |  |  | T5 turbine foundation | 8,49 | 12,93 | 22,95 | 18,68 | 12,35 | 3,72 | 9,48 | - |
|  |  |  | T6 turbine foundation | 11,80 | 16,33 | 20,62 | 15,73 | 12,84 | 2,99 | 12,38 | - |
|  |  |  | T7 turbine foundation | 13,13 | 17,44 | 19,18 | 14,26 | 12,33 | 4,28 | 13,83 | - |
|  |  |  | T8 turbine foundation | 11,95 | 16,26 | 20,05 | 15,34 | 12,08 | 3,75 | 12,72 | - |
|  |  |  | T9 turbine foundation | 12,92 | 17,02 | 18,90 | 14,26 | 11,50 | 4,79 | 13,82 | - |
|  |  |  | T10 turbine foundation | 11,07 | 15,30 | 20,58 | 16,11 | 11,68 | 3,84 | 11,99 | - |
|  |  |  | T11 turbine foundation | 12,05 | 15,85 | 19,18 | 15,01 | 10,37 | 5,32 | 13,26 | - |
|  |  |  | T12 turbine foundation | 10,08 | 14,33 | 21,41 | 17,06 | 11,72 | 3,80 | 11,07 | - |
|  |  |  | T13 turbine foundation | 11,67 | 15,13 | 19,24 | 15,20 | 9,40 | 6,12 | 13,15 | - |
|  |  |  | T14 turbine foundation | 10,73 | 14,43 | 20,26 | 16,20 | 10,13 | 5,35 | 12,12 | - |
|  |  |  | T15 turbine foundation | 9,28 | 13,43 | 21,95 | 17,80 | 11,48 | 4,26 | 10,45 | - |
|  |  |  | T16 turbine foundation | 8,59 | 12,75 | 22,57 | 18,49 | 11,62 | 4,41 | 9,82 | - |
|  |  |  | T17 turbine foundation (canceled according to Transgaz opinion) | 11,28 | 14,89 | 18,94 | 15,01 | 8,37 | 7,12 | 13,53 | - |
|  |  |  | T18 turbine foundation | 10,84 | 14,21 | 19,98 | 15,99 | 9,28 | 6,21 | 12,44 | - |
|  |  |  | T19 turbine foundation | 10,20 | 13,58 | 20,59 | 16,63 | 9,44 | 6,17 | 11,85 | - |
|  |  |  | T20 turbine foundation | 9,15 | 12,95 | 21,79 | 17,77 | 10,67 | 5,18 | 10,60 | - |
|  |  |  | T21 turbine foundation | 9,98 | 14,43 | 18,89 | 15,12 | 7,14 | 8,24 | 13,88 | - |
|  |  |  | T22 turbine foundation | 10,20 | 13,87 | 19,64 | 15,82 | 7,87 | 7,68 | 13,04 | - |
|  |  |  | T23 turbine foundation (canceled according to Transgaz opinion) | 9,00 | 13,55 | 19,59 | 15,90 | 6,77 | 8,90 | 13,52 | - |
|  |  |  | T24 turbine foundation | 9,30 | 12,89 | 20,40 | 16,65 | 7,64 | 8,22 | 12,57 | - |
|  |  |  | T25 turbine foundation | 9,79 | 12,60 | 20,96 | 17,14 | 8,51 | 7,42 | 11,81 | - |
|  |  |  | T26 turbine foundation | 9,96 | 11,78 | 21,78 | 17,99 | 8,91 | 7,40 | 11,07 | - |
|  |  |  | T27 turbine foundation | 8,73 | 12,14 | 22,03 | 18,10 | 10,00 | 6,17 | 10,51 | - |
|  |  |  | T28 turbine foundation | 9,01 | 11,97 | 21,21 | 17,58 | 8,02 | 8,21 | 11,81 | - |
|  |  |  | T29 turbine foundation | 8,82 | 11,21 | 22,08 | 18,35 | 8,44 | 8,22 | 11,16 | - |
|  |  |  | T30 turbine foundation | 8,12 | 11,03 | 22,64 | 18,82 | 9,50 | 7,29 | 10,26 | - |
|  |  |  | T31 turbine foundation | 7,36 | 9,90 | 23,54 | 19,79 | 9,61 | 8,10 | 9,79 | - |
|  |  |  | T32 turbine foundation | 5,49 | 7,51 | 25,83 | 22,13 | 10,94 | 9,38 | 8,14 | - |
|  |  |  | T33 turbine foundation | 11,02 | 14,28 | 19,56 | 15,64 | 8,53 | 6,96 | 12,94 | - |
|  |  |  | T34 turbine foundation | 4,03 | 8,35 | 26,73 | 22,79 | 13,28 | 7,28 | 6,08 | - |
|  |  |  | T35 turbine foundation | 3,38 | 8,60 | 27,80 | 23,79 | 14,80 | 7,00 | 4,70 | - |
|  |  |  | T36 turbine foundation | 3,81 | 8,75 | 27,20 | 23,21 | 14,20 | 6,97 | 5,33 | - |
|  |  |  | T37 turbine foundation | 5,05 | 9,72 | 25,99 | 21,97 | 13,42 | 5,90 | 6,46 | - |
|  |  |  | T38 turbine foundation | 6,27 | 11,00 | 25,07 | 20,92 | 13,30 | 4,70 | 7,32 | - |
|  |  |  | T39 turbine foundation | 7,19 | 11,87 | 24,28 | 20,03 | 13,01 | 4,01 | 8,31 | - |
|  |  |  | T40 turbine foundation | 6,69 | 11,76 | 25,27 | 20,84 | 14,17 | 3,79 | 7,23 | - |
|  |  |  | T41 turbine foundation | 6,90 | 12,40 | 25,77 | 21,09 | 15,21 | 3,22 | 7,05 | - |
|  |  |  | T42 turbine foundation | 7,91 | 13,61 | 25,47 | 20,52 | 15,77 | 2,15 | 7,69 | - |
|  |  |  | T43 turbine foundation | 6,52 | 12,34 | 26,47 | 21,71 | 15,89 | 3,55 | 6,51 | - |
|  |  |  | T44 turbine foundation | 5,99 | 12,13 | 27,26 | 22,27 | 16,53 | 4,16 | 5,73 | - |
|  |  |  | T45 turbine foundation | 4,24 | 10,80 | 28,84 | 24,22 | 17,24 | 5,99 | 4,01 | - |
|  |  |  | T46 turbine foundation | 3,34 | 9,93 | 29,31 | 24,83 | 17,19 | 6,80 | 3,26 | - |
|  |  |  | T47 turbine foundation | 3,21 | 10,19 | 10,19 | 25,62 | 18,16 | 7,35 | 2,65 | - |
|  |  |  | T48 turbine foundation | 10,99 | 16,07 | 25,54 | 17,33 | 14,68 | 0,90 | 10,91 | - |
|  | Installation of installations/ equipment | Turbine | T1 turbine | 12,76 | 17,56 | 20,77 | 15,43 | 14,19 | 2,64 | 12,83 | - |
|  |  |  | T2 turbine | 11,66 | 16,72 | 22,09 | 16,77 | 14,77 | 1,30 | 11,52 | - |
|  |  |  | T3 turbine | 13,46 | 18,33 | 20,63 | 15,08 | 14,76 | 2,58 | 13,31 | - |
|  |  |  | T4 turbine | 11,90 | 16,71 | 21,32 | 16,14 | 13,98 | 2,07 | 12,08 | - |
|  |  |  | T5 turbine | 8,49 | 12,93 | 22,95 | 18,68 | 12,35 | 3,72 | 9,48 | - |
|  |  |  | T6 turbine | 11,80 | 16,33 | 20,62 | 15,73 | 12,84 | 2,99 | 12,38 | - |
|  |  |  | T7 turbine | 13,13 | 17,44 | 19,18 | 14,26 | 12,33 | 4,28 | 13,83 | - |
|  |  |  | T8 turbine | 11,95 | 16,26 | 20,05 | 15,34 | 12,08 | 3,75 | 12,72 | - |
|  |  |  | T9 turbine | 12,92 | 17,02 | 18,90 | 14,26 | 11,50 | 4,79 | 13,82 | - |
|  |  |  | T10 turbine | 11,07 | 15,30 | 20,58 | 16,11 | 11,68 | 3,84 | 11,99 | - |
|  |  |  | T11 turbine | 12,05 | 15,85 | 19,18 | 15,01 | 10,37 | 5,32 | 13,26 | - |
|  |  |  | T12 turbine | 10,08 | 14,33 | 21,41 | 17,06 | 11,72 | 3,80 | 11,07 | - |
|  |  |  | T13 turbine | 11,67 | 15,13 | 19,24 | 15,20 | 9,40 | 6,12 | 13,15 | - |
|  |  |  | T14 turbine | 10,73 | 14,43 | 20,26 | 16,20 | 10,13 | 5,35 | 12,12 | - |
|  |  |  | T15 turbine | 9,28 | 13,43 | 21,95 | 17,80 | 11,48 | 4,26 | 10,45 | - |
|  |  |  | T16 turbine | 8,59 | 12,75 | 22,57 | 18,49 | 11,62 | 4,41 | 9,82 | - |
|  |  |  | T17 turbine (canceled according to Transgaz opinion) | 11,28 | 14,89 | 18,94 | 15,01 | 8,37 | 7,12 | 13,53 | - |
|  |  |  | T18 turbine | 10,84 | 14,21 | 19,98 | 15,99 | 9,28 | 6,21 | 12,44 | - |
|  |  |  | T19 turbine | 10,20 | 13,58 | 20,59 | 16,63 | 9,44 | 6,17 | 11,85 | - |
|  |  |  | T20 turbine | 9,15 | 12,95 | 21,79 | 17,77 | 10,67 | 5,18 | 10,60 | - |
|  |  |  | T21 turbine | 9,98 | 14,43 | 18,89 | 15,12 | 7,14 | 8,24 | 13,88 | - |
|  |  |  | T22 turbine | 10,20 | 13,87 | 19,64 | 15,82 | 7,87 | 7,68 | 13,04 | - |
|  |  |  | T23 turbine (canceled according to Transgaz opinion) | 9,00 | 13,55 | 19,59 | 15,90 | 6,77 | 8,90 | 13,52 | - |
|  |  |  | T24 turbine | 9,30 | 12,89 | 20,40 | 16,65 | 7,64 | 8,22 | 12,57 | - |
|  |  |  | T25 turbine | 9,79 | 12,60 | 20,96 | 17,14 | 8,51 | 7,42 | 11,81 | - |
|  |  |  | T26 turbine | 9,96 | 11,78 | 21,78 | 17,99 | 8,91 | 7,40 | 11,07 | - |
|  |  |  | T27 turbine | 8,73 | 12,14 | 22,03 | 18,10 | 10,00 | 6,17 | 10,51 | - |
|  |  |  | T28 turbine | 9,01 | 11,97 | 21,21 | 17,58 | 8,02 | 8,21 | 11,81 | - |
|  |  |  | T29 turbine | 8,82 | 11,21 | 22,08 | 18,35 | 8,44 | 8,22 | 11,16 | - |
|  |  |  | T30 turbine | 8,12 | 11,03 | 22,64 | 18,82 | 9,50 | 7,29 | 10,26 | - |
|  |  |  | T31 turbine | 7,36 | 9,90 | 23,54 | 19,79 | 9,61 | 8,10 | 9,79 | - |
|  |  |  | T32 turbine | 5,49 | 7,51 | 25,83 | 22,13 | 10,94 | 9,38 | 8,14 | - |
|  |  |  | T33 turbine | 11,02 | 14,28 | 19,56 | 15,64 | 8,53 | 6,96 | 12,94 | - |
|  |  |  | T34 turbine | 4,03 | 8,35 | 26,73 | 22,79 | 13,28 | 7,28 | 6,08 | - |
|  |  |  | T35 turbine | 3,38 | 8,60 | 27,80 | 23,79 | 14,80 | 7,00 | 4,70 | - |
|  |  |  | T36 turbine | 3,81 | 8,75 | 27,20 | 23,21 | 14,20 | 6,97 | 5,33 | - |
|  |  |  | T37 turbine | 5,05 | 9,72 | 25,99 | 21,97 | 13,42 | 5,90 | 6,46 | - |
|  |  |  | T38 turbine | 6,27 | 11,00 | 25,07 | 20,92 | 13,30 | 4,70 | 7,32 | - |
|  |  |  | T39 turbine | 7,19 | 11,87 | 24,28 | 20,03 | 13,01 | 4,01 | 8,31 | - |
|  |  |  | T40 turbine | 6,69 | 11,76 | 25,27 | 20,84 | 14,17 | 3,79 | 7,23 | - |
|  |  |  | T41 turbine | 6,90 | 12,40 | 25,77 | 21,09 | 15,21 | 3,22 | 7,05 | - |
|  |  |  | T42 turbine | 7,91 | 13,61 | 25,47 | 20,52 | 15,77 | 2,15 | 7,69 | - |
|  |  |  | T43 turbine | 6,52 | 12,34 | 26,47 | 21,71 | 15,89 | 3,55 | 6,51 | - |
|  |  |  | T44 turbine | 5,99 | 12,13 | 27,26 | 22,27 | 16,53 | 4,16 | 5,73 | - |
|  |  |  | T45 turbine | 4,24 | 10,80 | 28,84 | 24,22 | 17,24 | 5,99 | 4,01 | - |
|  |  |  | T46 turbine | 3,34 | 9,93 | 29,31 | 24,83 | 17,19 | 6,80 | 3,26 | - |
|  |  |  | T47 turbine | 3,21 | 10,19 | 10,19 | 25,62 | 18,16 | 7,35 | 2,65 | - |
|  |  |  | T48 turbine | 10,99 | 16,07 | 25,54 | 17,33 | 14,68 | 0,90 | 10,91 | - |
|  | Construction of the electrical network for discharging the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Transformer station | Transformer Station 1 | 4,48 | 7,90 | 25,92 | 22,12 | 11,17 | 8,25 | 8,27 | - |
|  |  |  | Transformer Station 2 (canceled) | 7,59 | 12,27 | 24,11 | 19,75 | 12,27 | 3,97 | 8,27 | - |
|  |  |  | Transformer Station 3 A177/16 - newly proposed (close to turbine T19)SS03 - A367/49 will be subject to another authorization procedure | 10,46 | 13,97 | 20,32 | 16,31 | 9,65 | 5,77 | 11,99 | - |
|  |  |  | Transformer Station 4 | 10,74 | 15,08 | 20,93 | 16,38 | 11,99 | 3,38 | 11,57 | - |
|  |  | LES cable route | Follow the route of existing logging roads, turbine plots, transformer stations | 3,22 | 8,27 | 18,79 | 14,10 | 7,12 | 0,74 | 29,42 | - |
|  | Site rehabilitation works on completion of construction | Transformer station | Transformer Station 1 | 4,48 | 7,90 | 25,92 | 22,12 | 11,17 | 8,25 | 8,27 | - |
|  |  |  | Transformer Station 2 (canceled) | 7,59 | 12,27 | 24,11 | 19,75 | 12,27 | 3,97 | 8,27 | - |
|  |  |  | Transformer Station 3 A177/16 - newly proposed (close to turbine T19)SS03 - A367/49 will be subject to another authorization procedure | 10,46 | 13,97 | 20,32 | 16,31 | 9,65 | 5,77 | 11,99 | - |
|  |  |  | Transformer Station 4 | 10,74 | 15,08 | 20,93 | 16,38 | 11,99 | 3,38 | 11,57 | - |
|  |  | Turbine platform | T1 turbine platform | 12,75 | 17,55 | 20,70 | 15,36 | 14,16 | 2,50 | 12,72 | - |
|  |  |  | T2 turbine platform | 11,52 | 16,53 | 22,04 | 16,76 | 14,57 | 1,12 | 11,34 | - |
|  |  |  | T3 turbine platform | 13,35 | 18,18 | 20,53 | 15,01 | 14,53 | 2,46 | 13,18 | - |
|  |  |  | T4 turbine platform | 11,84 | 16,66 | 21,30 | 16,12 | 13,96 | 1,86 | 11,91 | - |
|  |  |  | T5 turbine platform | 8,48 | 12,92 | 22,88 | 18,59 | 12,29 | 3,44 | 9,36 | - |
|  |  |  | T6 turbine platform | 11,71 | 16,19 | 20,54 | 15,66 | 12,62 | 2,86 | 12,25 | - |
|  |  |  | T7 turbine platform | 13,01 | 17,28 | 19,14 | 14,24 | 12,11 | 4,10 | 13,65 | - |
|  |  |  | T8 turbine platform | 11,83 | 16,09 | 20,03 | 15,33 | 11,87 | 3,57 | 12,54 | - |
|  |  |  | T9 turbine platform | 12,86 | 16,89 | 18,82 | 14,19 | 11,27 | 4,66 | 13,70 | - |
|  |  |  | T10 turbine platform | 11,06 | 15,28 | 20,51 | 16,02 | 11,63 | 3,56 | 11,88 | - |
|  |  |  | T11 turbine platform | 11,99 | 15,79 | 19,16 | 14,96 | 10,35 | 5,02 | 13,09 | - |
|  |  |  | T12 turbine platform | 10,02 | 14,19 | 21,34 | 16,99 | 11,51 | 3,71 | 10,95 | - |
|  |  |  | T13 turbine platform | 11,60 | 15,00 | 19,21 | 15,18 | 9,19 | 5,98 | 12,98 | - |
|  |  |  | T14 turbine platform | 10,66 | 14,28 | 20,25 | 16,19 | 9,94 | 5,22 | 11,94 | - |
|  |  |  | T15 turbine platform | 9,18 | 13,26 | 21,93 | 17,78 | 11,30 | 4,15 | 10,27 | - |
|  |  |  | T16 turbine platform | 8,52 | 12,69 | 22,55 | 18,44 | 11,61 | 4,12 | 9,64 | - |
|  |  |  | T17 turbine platform (canceled according to Transgaz opinion) | 11,05 | 14,76 | 18,92 | 14,99 | 8,17 | 6,98 | 13,35 | - |
|  |  |  | T18 turbine platform | 10,78 | 14,15 | 19,96 | 15,97 | 9,27 | 5,90 | 12,25 | - |
|  |  |  | T19 turbine platform | 10,78 | 14,15 | 19,96 | 15,97 | 9,27 | 5,90 | 12,25 | - |
|  |  |  | T20 turbine platform | 9,13 | 12,94 | 21,72 | 17,70 | 10,63 | 4,88 | 10,46 | - |
|  |  |  | T21 turbine platform | 9,96 | 14,42 | 18,82 | 15,04 | 7,09 | 8,05 | 13,73 | - |
|  |  |  | T22 turbine platform | 10,18 | 13,80 | 19,62 | 15,79 | 7,85 | 7,37 | 12,84 | - |
|  |  |  | T23 turbine platform (canceled according to Transgaz opinion) | 8,97 | 13,48 | 19,56 | 15,86 | 6,75 | 8,59 | 13,31 | - |
|  |  |  | T24 turbine platform | 9,07 | 12,81 | 20,33 | 16,59 | 7,44 | 8,12 | 12,44 | - |
|  |  |  | T25 turbine platform | 9,68 | 12,51 | 20,89 | 17,07 | 8,32 | 7,32 | 11,68 | - |
|  |  |  | T26 turbine platform | 8,90 | 11,65 | 21,78 | 17,97 | 8,75 | 7,30 | 10,89 | - |
|  |  |  | T27 turbine platform | 8,72 | 12,13 | 21,95 | 18,01 | 9,94 | 5,88 | 10,38 | - |
|  |  |  | T28 turbine platform | 8,98 | 11,93 | 21,27 | 17,52 | 7,98 | 7,91 | 11,62 | - |
|  |  |  | T29 turbine platform | 8,80 | 11,20 | 22,01 | 18,27 | 8,38 | 7,93 | 10,97 | - |
|  |  |  | T30 turbine platform | 8,10 | 10,93 | 22,57 | 18,75 | 9,32 | 7,20 | 10,13 | - |
|  |  |  | T31 turbine platform | 7,34 | 9,89 | 23,47 | 19,71 | 9,54 | 7,82 | 9,59 | - |
|  |  |  | T32 turbine platform | 5,43 | 7,48 | 25,65 | 21,94 | 10,87 | 9,16 | 7,95 | - |
|  |  |  | T33 turbine platform | 10,79 | 14,14 | 19,54 | 15,62 | 8,34 | 6,83 | 12,76 | - |
|  |  |  | T34 turbine platform | 4,01 | 8,34 | 26,51 | 22,57 | 13,15 | 7,10 | 5,91 | - |
|  |  |  | T35 turbine platform | 3,36 | 8,57 | 27,64 | 23,62 | 14,73 | 6,81 | 4,58 | - |
|  |  |  | T36 turbine platform | 3,80 | 8,72 | 27,04 | 23,04 | 14,13 | 6,59 | 5,20 | - |
|  |  |  | T37 turbine platform | 4,98 | 9,56 | 25,94 | 21,94 | 13,24 | 5,89 | 6,28 | - |
|  |  |  | T38 turbine platform | 6,24 | 10,95 | 24,84 | 20,69 | 13,15 | 4,57 | 7,19 | - |
|  |  |  | T39 turbine platform | 7,16 | 11,82 | 24,05 | 19,80 | 12,94 | 3,90 | 8,00 | - |
|  |  |  | T40 turbine platform | 6,49 | 11,60 | 25,25 | 20,82 | 14,16 | 3,75 | 6,90 | - |
|  |  |  | T41 turbine platform | 6,68 | 12,23 | 25,76 | 21,07 | 15,19 | 3,23 | 6,73 | - |
|  |  |  | T42 turbine platform | 7,70 | 13,44 | 25,45 | 20,50 | 15,76 | 2,20 | 7,36 | - |
|  |  |  | T43 turbine platform | 6,50 | 12,33 | 26,27 | 21,49 | 15,81 | 3,39 | 6,37 | - |
|  |  |  | T44 turbine platform | 5,97 | 12,11 | 27,20 | 22,41 | 16,49 | 4,17 | 5,57 | - |
|  |  |  | T45 turbine platform | 4,21 | 10,78 | 28,75 | 24,09 | 17,22 | 5,90 | 3,88 | - |
|  |  |  | T46 turbine platform | 3,31 | 9,91 | 29,22 | 24,71 | 17,17 | 6,69 | 3,11 | - |
|  |  |  | T47 turbine platform | 3,19 | 10,16 | 30,06 | 25,45 | 18,09 | 7,24 | 2,56 | - |
|  |  |  | T48 turbine platform | 10,85 | 15,88 | 22,49 | 17,32 | 14,48 | 0,72 | 10,73 | - |
| Operate | Energy production activities | Transformer station | Transformer Station 1 | 4,48 | 7,90 | 25,92 | 22,12 | 11,17 | 8,25 | 8,27 | - |
|  |  |  | Transformer Station 2 (canceled) | 7,59 | 12,27 | 24,11 | 19,75 | 12,27 | 3,97 | 8,27 | - |
|  |  |  | Transformer Station 3 A177/16 - newly proposed (close to turbine T19)SS03 - A367/49 will be subject to another authorization procedure | 10,46 | 13,97 | 20,32 | 16,31 | 9,65 | 5,77 | 11,99 | - |
|  |  |  | Transformer Station 4 | 10,74 | 15,08 | 20,93 | 16,38 | 11,99 | 3,38 | 11,57 | - |
|  |  | Turbine | T1 turbine | 12,76 | 17,56 | 20,77 | 15,43 | 14,19 | 2,64 | 12,83 | - |
|  |  |  | T2 turbine | 11,66 | 16,72 | 22,09 | 16,77 | 14,77 | 1,30 | 11,52 | - |
|  |  |  | T3 turbine | 13,46 | 18,33 | 20,63 | 15,08 | 14,76 | 2,58 | 13,31 | - |
|  |  |  | T4 turbine | 11,90 | 16,71 | 21,32 | 16,14 | 13,98 | 2,07 | 12,08 | - |
|  |  |  | T5 turbine | 8,49 | 12,93 | 22,95 | 18,68 | 12,35 | 3,72 | 9,48 | - |
|  |  |  | T6 turbine | 11,80 | 16,33 | 20,62 | 15,73 | 12,84 | 2,99 | 12,38 | - |
|  |  |  | T7 turbine | 13,13 | 17,44 | 19,18 | 14,26 | 12,33 | 4,28 | 13,83 | - |
|  |  |  | T8 turbine | 11,95 | 16,26 | 20,05 | 15,34 | 12,08 | 3,75 | 12,72 | - |
|  |  |  | T9 turbine | 12,92 | 17,02 | 18,90 | 14,26 | 11,50 | 4,79 | 13,82 | - |
|  |  |  | T10 turbine | 11,07 | 15,30 | 20,58 | 16,11 | 11,68 | 3,84 | 11,99 | - |
|  |  |  | T11 turbine | 12,05 | 15,85 | 19,18 | 15,01 | 10,37 | 5,32 | 13,26 | - |
|  |  |  | T12 turbine | 10,08 | 14,33 | 21,41 | 17,06 | 11,72 | 3,80 | 11,07 | - |
|  |  |  | T13 turbine | 11,67 | 15,13 | 19,24 | 15,20 | 9,40 | 6,12 | 13,15 | - |
|  |  |  | T14 turbine | 10,73 | 14,43 | 20,26 | 16,20 | 10,13 | 5,35 | 12,12 | - |
|  |  |  | T15 turbine | 9,28 | 13,43 | 21,95 | 17,80 | 11,48 | 4,26 | 10,45 | - |
|  |  |  | T16 turbine | 8,59 | 12,75 | 22,57 | 18,49 | 11,62 | 4,41 | 9,82 | - |
|  |  |  | T17 turbine (canceled according to Transgaz opinion) | 11,28 | 14,89 | 18,94 | 15,01 | 8,37 | 7,12 | 13,53 | - |
|  |  |  | T18 turbine | 10,84 | 14,21 | 19,98 | 15,99 | 9,28 | 6,21 | 12,44 | - |
|  |  |  | T19 turbine | 10,20 | 13,58 | 20,59 | 16,63 | 9,44 | 6,17 | 11,85 | - |
|  |  |  | T20 turbine | 9,15 | 12,95 | 21,79 | 17,77 | 10,67 | 5,18 | 10,60 | - |
|  |  |  | T21 turbine | 9,98 | 14,43 | 18,89 | 15,12 | 7,14 | 8,24 | 13,88 | - |
|  |  |  | T22 turbine | 10,20 | 13,87 | 19,64 | 15,82 | 7,87 | 7,68 | 13,04 | - |
|  |  |  | T23 turbine (canceled according to Transgaz opinion) | 9,00 | 13,55 | 19,59 | 15,90 | 6,77 | 8,90 | 13,52 | - |
|  |  |  | T24 turbine | 9,30 | 12,89 | 20,40 | 16,65 | 7,64 | 8,22 | 12,57 | - |
|  |  |  | T25 turbine | 9,79 | 12,60 | 20,96 | 17,14 | 8,51 | 7,42 | 11,81 | - |
|  |  |  | T26 turbine | 9,96 | 11,78 | 21,78 | 17,99 | 8,91 | 7,40 | 11,07 | - |
|  |  |  | T27 turbine | 8,73 | 12,14 | 22,03 | 18,10 | 10,00 | 6,17 | 10,51 | - |
|  |  |  | T28 turbine | 9,01 | 11,97 | 21,21 | 17,58 | 8,02 | 8,21 | 11,81 | - |
|  |  |  | T29 turbine | 8,82 | 11,21 | 22,08 | 18,35 | 8,44 | 8,22 | 11,16 | - |
|  |  |  | T30 turbine | 8,12 | 11,03 | 22,64 | 18,82 | 9,50 | 7,29 | 10,26 | - |
|  |  |  | T31 turbine | 7,36 | 9,90 | 23,54 | 19,79 | 9,61 | 8,10 | 9,79 | - |
|  |  |  | T32 turbine | 5,49 | 7,51 | 25,83 | 22,13 | 10,94 | 9,38 | 8,14 | - |
|  |  |  | T33 turbine | 11,02 | 14,28 | 19,56 | 15,64 | 8,53 | 6,96 | 12,94 | - |
|  |  |  | T34 turbine | 4,03 | 8,35 | 26,73 | 22,79 | 13,28 | 7,28 | 6,08 | - |
|  |  |  | T35 turbine | 3,38 | 8,60 | 27,80 | 23,79 | 14,80 | 7,00 | 4,70 | - |
|  |  |  | T36 turbine | 3,81 | 8,75 | 27,20 | 23,21 | 14,20 | 6,97 | 5,33 | - |
|  |  |  | T37 turbine | 5,05 | 9,72 | 25,99 | 21,97 | 13,42 | 5,90 | 6,46 | - |
|  |  |  | T38 turbine | 6,27 | 11,00 | 25,07 | 20,92 | 13,30 | 4,70 | 7,32 | - |
|  |  |  | T39 turbine | 7,19 | 11,87 | 24,28 | 20,03 | 13,01 | 4,01 | 8,31 | - |
|  |  |  | T40 turbine | 6,69 | 11,76 | 25,27 | 20,84 | 14,17 | 3,79 | 7,23 | - |
|  |  |  | T41 turbine | 6,90 | 12,40 | 25,77 | 21,09 | 15,21 | 3,22 | 7,05 | - |
|  |  |  | T42 turbine | 7,91 | 13,61 | 25,47 | 20,52 | 15,77 | 2,15 | 7,69 | - |
|  |  |  | T43 turbine | 6,52 | 12,34 | 26,47 | 21,71 | 15,89 | 3,55 | 6,51 | - |
|  |  |  | T44 turbine | 5,99 | 12,13 | 27,26 | 22,27 | 16,53 | 4,16 | 5,73 | - |
|  |  |  | T45 turbine | 4,24 | 10,80 | 28,84 | 24,22 | 17,24 | 5,99 | 4,01 | - |
|  |  |  | T46 turbine | 3,34 | 9,93 | 29,31 | 24,83 | 17,19 | 6,80 | 3,26 | - |
|  |  |  | T47 turbine | 3,21 | 10,19 | 10,19 | 25,62 | 18,16 | 7,35 | 2,65 | - |
|  |  |  | T48 turbine | 10,99 | 16,07 | 25,54 | 17,33 | 14,68 | 0,90 | 10,91 | - |
|  | Maintenance and maintenance works | Transformer station | Transformer Station 1 | 4,48 | 7,90 | 25,92 | 22,12 | 11,17 | 8,25 | 8,27 | - |
|  |  |  | Transformer Station 2 (canceled) | 7,59 | 12,27 | 24,11 | 19,75 | 12,27 | 3,97 | 8,27 | - |
|  |  |  | Transformer Station 3 A177/16 - newly proposed (close to turbine T19)SS03 - A367/49 will be subject to another authorization procedure | 10,46 | 13,97 | 20,32 | 16,31 | 9,65 | 5,77 | 11,99 | - |
|  |  |  | Transformer Station 4 | 10,74 | 15,08 | 20,93 | 16,38 | 11,99 | 3,38 | 11,57 | - |
|  |  | Turbine platform | T1 turbine platform | 12,75 | 17,55 | 20,70 | 15,36 | 14,16 | 2,50 | 12,72 | - |
|  |  |  | T2 turbine platform | 11,52 | 16,53 | 22,04 | 16,76 | 14,57 | 1,12 | 11,34 | - |
|  |  |  | T3 turbine platform | 13,35 | 18,18 | 20,53 | 15,01 | 14,53 | 2,46 | 13,18 | - |
|  |  |  | T4 turbine platform | 11,84 | 16,66 | 21,30 | 16,12 | 13,96 | 1,86 | 11,91 | - |
|  |  |  | T5 turbine platform | 8,48 | 12,92 | 22,88 | 18,59 | 12,29 | 3,44 | 9,36 | - |
|  |  |  | T6 turbine platform | 11,71 | 16,19 | 20,54 | 15,66 | 12,62 | 2,86 | 12,25 | - |
|  |  |  | T7 turbine platform | 13,01 | 17,28 | 19,14 | 14,24 | 12,11 | 4,10 | 13,65 | - |
|  |  |  | T8 turbine platform | 11,83 | 16,09 | 20,03 | 15,33 | 11,87 | 3,57 | 12,54 | - |
|  |  |  | T9 turbine platform | 12,86 | 16,89 | 18,82 | 14,19 | 11,27 | 4,66 | 13,70 | - |
|  |  |  | T10 turbine platform | 11,06 | 15,28 | 20,51 | 16,02 | 11,63 | 3,56 | 11,88 | - |
|  |  |  | T11 turbine platform | 11,99 | 15,79 | 19,16 | 14,96 | 10,35 | 5,02 | 13,09 | - |
|  |  |  | T12 turbine platform | 10,02 | 14,19 | 21,34 | 16,99 | 11,51 | 3,71 | 10,95 | - |
|  |  |  | T13 turbine platform | 11,60 | 15,00 | 19,21 | 15,18 | 9,19 | 5,98 | 12,98 | - |
|  |  |  | T14 turbine platform | 10,66 | 14,28 | 20,25 | 16,19 | 9,94 | 5,22 | 11,94 | - |
|  |  |  | T15 turbine platform | 9,18 | 13,26 | 21,93 | 17,78 | 11,30 | 4,15 | 10,27 | - |
|  |  |  | T16 turbine platform | 8,52 | 12,69 | 22,55 | 18,44 | 11,61 | 4,12 | 9,64 | - |
|  |  |  | T17 turbine platform (canceled according to Transgaz opinion) | 11,05 | 14,76 | 18,92 | 14,99 | 8,17 | 6,98 | 13,35 | - |
|  |  |  | T18 turbine platform | 10,78 | 14,15 | 19,96 | 15,97 | 9,27 | 5,90 | 12,25 | - |
|  |  |  | T19 turbine platform | 10,78 | 14,15 | 19,96 | 15,97 | 9,27 | 5,90 | 12,25 | - |
|  |  |  | T20 turbine platform | 9,13 | 12,94 | 21,72 | 17,70 | 10,63 | 4,88 | 10,46 | - |
|  |  |  | T21 turbine platform | 9,96 | 14,42 | 18,82 | 15,04 | 7,09 | 8,05 | 13,73 | - |
|  |  |  | T22 turbine platform | 10,18 | 13,80 | 19,62 | 15,79 | 7,85 | 7,37 | 12,84 | - |
|  |  |  | T23 turbine platform (canceled according to Transgaz opinion) | 8,97 | 13,48 | 19,56 | 15,86 | 6,75 | 8,59 | 13,31 | - |
|  |  |  | T24 turbine platform | 9,07 | 12,81 | 20,33 | 16,59 | 7,44 | 8,12 | 12,44 | - |
|  |  |  | T25 turbine platform | 9,68 | 12,51 | 20,89 | 17,07 | 8,32 | 7,32 | 11,68 | - |
|  |  |  | T26 turbine platform | 8,90 | 11,65 | 21,78 | 17,97 | 8,75 | 7,30 | 10,89 | - |
|  |  |  | T27 turbine platform | 8,72 | 12,13 | 21,95 | 18,01 | 9,94 | 5,88 | 10,38 | - |
|  |  |  | T28 turbine platform | 8,98 | 11,93 | 21,27 | 17,52 | 7,98 | 7,91 | 11,62 | - |
|  |  |  | T29 turbine platform | 8,80 | 11,20 | 22,01 | 18,27 | 8,38 | 7,93 | 10,97 | - |
|  |  |  | T30 turbine platform | 8,10 | 10,93 | 22,57 | 18,75 | 9,32 | 7,20 | 10,13 | - |
|  |  |  | T31 turbine platform | 7,34 | 9,89 | 23,47 | 19,71 | 9,54 | 7,82 | 9,59 | - |
|  |  |  | T32 turbine platform | 5,43 | 7,48 | 25,65 | 21,94 | 10,87 | 9,16 | 7,95 | - |
|  |  |  | T33 turbine platform | 10,79 | 14,14 | 19,54 | 15,62 | 8,34 | 6,83 | 12,76 | - |
|  |  |  | T34 turbine platform | 4,01 | 8,34 | 26,51 | 22,57 | 13,15 | 7,10 | 5,91 | - |
|  |  |  | T35 turbine platform | 3,36 | 8,57 | 27,64 | 23,62 | 14,73 | 6,81 | 4,58 | - |
|  |  |  | T36 turbine platform | 3,80 | 8,72 | 27,04 | 23,04 | 14,13 | 6,59 | 5,20 | - |
|  |  |  | T37 turbine platform | 4,98 | 9,56 | 25,94 | 21,94 | 13,24 | 5,89 | 6,28 | - |
|  |  |  | T38 turbine platform | 6,24 | 10,95 | 24,84 | 20,69 | 13,15 | 4,57 | 7,19 | - |
|  |  |  | T39 turbine platform | 7,16 | 11,82 | 24,05 | 19,80 | 12,94 | 3,90 | 8,00 | - |
|  |  |  | T40 turbine platform | 6,49 | 11,60 | 25,25 | 20,82 | 14,16 | 3,75 | 6,90 | - |
|  |  |  | T41 turbine platform | 6,68 | 12,23 | 25,76 | 21,07 | 15,19 | 3,23 | 6,73 | - |
|  |  |  | T42 turbine platform | 7,70 | 13,44 | 25,45 | 20,50 | 15,76 | 2,20 | 7,36 | - |
|  |  |  | T43 turbine platform | 6,50 | 12,33 | 26,27 | 21,49 | 15,81 | 3,39 | 6,37 | - |
|  |  |  | T44 turbine platform | 5,97 | 12,11 | 27,20 | 22,41 | 16,49 | 4,17 | 5,57 | - |
|  |  |  | T45 turbine platform | 4,21 | 10,78 | 28,75 | 24,09 | 17,22 | 5,90 | 3,88 | - |
|  |  |  | T46 turbine platform | 3,31 | 9,91 | 29,22 | 24,71 | 17,17 | 6,69 | 3,11 | - |
|  |  |  | T47 turbine platform | 3,19 | 10,16 | 30,06 | 25,45 | 18,09 | 7,24 | 2,56 | - |
|  |  |  | T48 turbine platform | 10,85 | 15,88 | 22,49 | 17,32 | 14,48 | 0,72 | 10,73 | - |
| Decommissioning | Site organization and management (including site traffic) | Site organization | OS1 - turbine area T39 | 7,47 | 12,10 | 23,93 | 1967 | 12,90 | 3,72 | 8,36 | - |
|  |  |  | OS2 - turbine area T31 | 7,41 | 10,13 | 23,33 | 19,54 | 9,65 | 7,57 | 9,69 | - |
|  |  |  | OS3 - T40 turbine area | 6,50 | 11,63 | 25,25 | 20,80 | 14,12 | 3,67 | 6,89 | - |
|  |  | Construction site traffic | Road operating turbines | 3,22 | 8,27 | 18,79 | 14,10 | 7,12 | 0,74 | 29,42 | - |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the project lifetime | Transformer station | Transformer Station 1 | 4,48 | 7,90 | 25,92 | 22,12 | 11,17 | 8,25 | 8,27 | - |
|  |  |  | Transformer Station 2 (canceled) | 7,59 | 12,27 | 24,11 | 19,75 | 12,27 | 3,97 | 8,27 | - |
|  |  |  | Transformer Station 3 A177/16 - newly proposed (close to turbine T19) SS03 - A367/49 will be subject to another authorization procedure | 10,46 | 13,97 | 20,32 | 16,31 | 9,65 | 5,77 | 11,99 | - |
|  |  |  | Transformer Station 4 | 10,74 | 15,08 | 20,93 | 16,38 | 11,99 | 3,38 | 11,57 | - |
|  |  | Turbine | T1 turbine | 12,76 | 17,56 | 20,77 | 15,43 | 14,19 | 2,64 | 12,83 | - |
|  |  |  | T2 turbine | 11,66 | 16,72 | 22,09 | 16,77 | 14,77 | 1,30 | 11,52 | - |
|  |  |  | T3 turbine | 13,46 | 18,33 | 20,63 | 15,08 | 14,76 | 2,58 | 13,31 | - |
|  |  |  | T4 turbine | 11,90 | 16,71 | 21,32 | 16,14 | 13,98 | 2,07 | 12,08 | - |
|  |  |  | T5 turbine | 8,49 | 12,93 | 22,95 | 18,68 | 12,35 | 3,72 | 9,48 | - |
|  |  |  | T6 turbine | 11,80 | 16,33 | 20,62 | 15,73 | 12,84 | 2,99 | 12,38 | - |
|  |  |  | T7 turbine | 13,13 | 17,44 | 19,18 | 14,26 | 12,33 | 4,28 | 13,83 | - |
|  |  |  | T8 turbine | 11,95 | 16,26 | 20,05 | 15,34 | 12,08 | 3,75 | 12,72 | - |
|  |  |  | T9 turbine | 12,92 | 17,02 | 18,90 | 14,26 | 11,50 | 4,79 | 13,82 | - |
|  |  |  | T10 turbine | 11,07 | 15,30 | 20,58 | 16,11 | 11,68 | 3,84 | 11,99 | - |
|  |  |  | T11 turbine | 12,05 | 15,85 | 19,18 | 15,01 | 10,37 | 5,32 | 13,26 | - |
|  |  |  | T12 turbine | 10,08 | 14,33 | 21,41 | 17,06 | 11,72 | 3,80 | 11,07 | - |
|  |  |  | T13 turbine | 11,67 | 15,13 | 19,24 | 15,20 | 9,40 | 6,12 | 13,15 | - |
|  |  |  | T14 turbine | 10,73 | 14,43 | 20,26 | 16,20 | 10,13 | 5,35 | 12,12 | - |
|  |  |  | T15 turbine | 9,28 | 13,43 | 21,95 | 17,80 | 11,48 | 4,26 | 10,45 | - |
|  |  |  | T16 turbine | 8,59 | 12,75 | 22,57 | 18,49 | 11,62 | 4,41 | 9,82 | - |
|  |  |  | T17 turbine (canceled according to Transgaz opinion) | 11,28 | 14,89 | 18,94 | 15,01 | 8,37 | 7,12 | 13,53 | - |
|  |  |  | T18 turbine | 10,84 | 14,21 | 19,98 | 15,99 | 9,28 | 6,21 | 12,44 | - |
|  |  |  | T19 turbine | 10,20 | 13,58 | 20,59 | 16,63 | 9,44 | 6,17 | 11,85 | - |
|  |  |  | T20 turbine | 9,15 | 12,95 | 21,79 | 17,77 | 10,67 | 5,18 | 10,60 | - |
|  |  |  | T21 turbine | 9,98 | 14,43 | 18,89 | 15,12 | 7,14 | 8,24 | 13,88 | - |
|  |  |  | T22 turbine | 10,20 | 13,87 | 19,64 | 15,82 | 7,87 | 7,68 | 13,04 | - |
|  |  |  | T23 turbine (canceled according to Transgaz opinion) | 9,00 | 13,55 | 19,59 | 15,90 | 6,77 | 8,90 | 13,52 | - |
|  |  |  | T24 turbine | 9,30 | 12,89 | 20,40 | 16,65 | 7,64 | 8,22 | 12,57 | - |
|  |  |  | T25 turbine | 9,79 | 12,60 | 20,96 | 17,14 | 8,51 | 7,42 | 11,81 | - |
|  |  |  | T26 turbine | 9,96 | 11,78 | 21,78 | 17,99 | 8,91 | 7,40 | 11,07 | - |
|  |  |  | T27 turbine | 8,73 | 12,14 | 22,03 | 18,10 | 10,00 | 6,17 | 10,51 | - |
|  |  |  | T28 turbine | 9,01 | 11,97 | 21,21 | 17,58 | 8,02 | 8,21 | 11,81 | - |
|  |  |  | T29 turbine | 8,82 | 11,21 | 22,08 | 18,35 | 8,44 | 8,22 | 11,16 | - |
|  |  |  | T30 turbine | 8,12 | 11,03 | 22,64 | 18,82 | 9,50 | 7,29 | 10,26 | - |
|  |  |  | T31 turbine | 7,36 | 9,90 | 23,54 | 19,79 | 9,61 | 8,10 | 9,79 | - |
|  |  |  | T32 turbine | 5,49 | 7,51 | 25,83 | 22,13 | 10,94 | 9,38 | 8,14 | - |
|  |  |  | T33 turbine | 11,02 | 14,28 | 19,56 | 15,64 | 8,53 | 6,96 | 12,94 | - |
|  |  |  | T34 turbine | 4,03 | 8,35 | 26,73 | 22,79 | 13,28 | 7,28 | 6,08 | - |
|  |  |  | T35 turbine | 3,38 | 8,60 | 27,80 | 23,79 | 14,80 | 7,00 | 4,70 | - |
|  |  |  | T36 turbine | 3,81 | 8,75 | 27,20 | 23,21 | 14,20 | 6,97 | 5,33 | - |
|  |  |  | T37 turbine | 5,05 | 9,72 | 25,99 | 21,97 | 13,42 | 5,90 | 6,46 | - |
|  |  |  | T38 turbine | 6,27 | 11,00 | 25,07 | 20,92 | 13,30 | 4,70 | 7,32 | - |
|  |  |  | T39 turbine | 7,19 | 11,87 | 24,28 | 20,03 | 13,01 | 4,01 | 8,31 | - |
|  |  |  | T40 turbine | 6,69 | 11,76 | 25,27 | 20,84 | 14,17 | 3,79 | 7,23 | - |
|  |  |  | T41 turbine | 6,90 | 12,40 | 25,77 | 21,09 | 15,21 | 3,22 | 7,05 | - |
|  |  |  | T42 turbine | 7,91 | 13,61 | 25,47 | 20,52 | 15,77 | 2,15 | 7,69 | - |
|  |  |  | T43 turbine | 6,52 | 12,34 | 26,47 | 21,71 | 15,89 | 3,55 | 6,51 | - |
|  |  |  | T44 turbine | 5,99 | 12,13 | 27,26 | 22,27 | 16,53 | 4,16 | 5,73 | - |
|  |  |  | T45 turbine | 4,24 | 10,80 | 28,84 | 24,22 | 17,24 | 5,99 | 4,01 | - |
|  |  |  | T46 turbine | 3,34 | 9,93 | 29,31 | 24,83 | 17,19 | 6,80 | 3,26 | - |
|  |  |  | T47 turbine | 3,21 | 10,19 | 10,19 | 25,62 | 18,16 | 7,35 | 2,65 | - |
|  |  |  | T48 turbine | 10,99 | 16,07 | 25,54 | 17,33 | 14,68 | 0,90 | 10,91 | - |
|  |  | Turbine access road | Turbine operation road | 3,22 | 8,27 | 18,79 | 14,10 | 7,12 | 0,74 | 29,42 | - |
|  |  |  | Turbine access road | 3,22 | 8,27 | 18,79 | 14,10 | 7,12 | 0,74 | 29,42 | - |

### Summary maps of all interventions potentially affecting ANPIC

This sub-chapter presents maps of all the interventions that are likely to affect ANPIC whether they are temporary or permanent or whether they are within or in the vicinity of ANPIC.

The analysis was conducted based on available information on the sites proposed in this Plan, taking into consideration an area large enough to include the final configuration of the Plan, including:

- areas to be temporarily affected during construction

- areas that will be permanently affected during construction

- any transportation infrastructure to serve the energy objective;

- the proposed sites for the construction of transformer stations, electricity transmission and connection to the existing electricity transmission grid;

- any proposed collection/disposal sites.

In the case of energy generation plans/projects, the following can be identified:

A zone of direct influence (the area where the effects of the plan are felt, such as noise, vibration, radiation, air pollutants, thermal pollutants, artificial lighting, dispersal of invasive species, and others);

An area of indirect influence (the area in which the effects of other activities occur, modified as a result of the implementation of the analyzed plan). An example is the modification of the transformation and transmission solutions of the electricity delivered to the grid (e.g. new electricity transmission infrastructures, modernization/increase of capacity of existing ones).

The main forms of impact that may occur in the ANPIC within the area of influence of a plan/project are habitat alteration (habitat degradation) and/or disturbance of species activity. Habitat alteration can lead over time to secondary impacts such as loss of habitat area or reductions in population numbers.

Figure 1-14 Area of direct influence

Figure 1-15 Area of indirect influence

## Effects of PP interventions

The effects generated by the implementation of the PP are tabulated for each intervention PP .

Table  1-10: Summary of effects generated by the implementation of the PP

| Stage | Impact | Type(s) of intervention generating the effect | Method of quantification | Quantifying the effects | Distance to the effects | ANPIC potentially affected | Other additional information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Construction | Changing air quality | Organization and conduct of the site, including site traffic | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mw Calculation methodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not capable of generating an impact on air quality, thus the resulting concentrations are considerably lower than the assessment threshold values established according to Law no. 104/2011 for the protection of vegetation: SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19,5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the air emission estimates presented in Section 1.1.7, Figures 1-4 - 1-9, the maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area. According to the air emission estimates presented in section 1.1.7, the maximum estimated pollutant concentration value: SO2 - maximum distance at which the concentration of 1.3 µg/m3 is experienced is 1.45 km from the turbine platform boundary NO2 (annual average) - maximum distance at which the concentration of 8.5 µg/m3 is experienced is 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 is experienced at a maximum distance of 1.2 km from the turbine platform boundary CO (8-hour average) - maximum value of 0.0334 mg/m3 is experienced at a maximum distance of 1.2 km from the edge of the turbine platform | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 Analysis | According to the noise level estimates presented in section 1.1.7, figure 1-10a, 1-10b, the noise level recorded at the designated receptors during the construction period will be below the limit values set by Order No. 119/2014 for the approval of the Hygiene and Public Health Standards on the living environment of the population (55 dB during the day, 45 dB at night), given the point sources (machinery involved in construction), their small number, their distance from sensitive receptors and the duration of execution The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | According to the noise level estimates presented in section 1.1.7, noise values above the maximum permissible limits may extend up to a distance of about 60 m from the working fronts and site organizations. The effects will be felt locally at distances not likely to affect sensitive receptors in the area. The 30 dB noise isoline will be felt at distances of maximum 1.5 km around the work area. Possible disturbance of the activity of up to 2 to 3 individuals of the species Spermophilus citellus and Mustela eversmanii. | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased brightness |  | Analyze ArcGIS Pro 3.2.1 Light pollution map | Construction work will be carried out during daylight hours only and only security lighting will be used during site organization | There will be no additional effects beyond those already existing | - | - |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution not exceeding 15 ha | Given the specific nature of the work, in case of accidental pollutions, no more than 20 m shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Physical and behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts). Possible disturbance of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii and avifauna*. | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Introduction / spread of invasive species |  | Field investigations to identify and map areas of high abundance of invasive plant species (April 2022 - March 2023). Spatial analysis to highlight the risk of spread of these species. Analysis ArcGIS Pro 3.2.1 | According to the literature and information provided by the reports of the project "Management of Invasive Species in Romania" benefiting the Ministry of Environment, the area is characterized by a low presence of invasive species. However, given the specificity of construction and agricultural activities, the possibility of the introduction and spread of invasive species is not excluded. The spread of invasive plant species may be favored by anemocory or hydrocorrhizal pathways, with the risk of invasive plant species occurring within the site, or in its vicinity, given the small distance between the site and the plane. There may also be contamination of soil with germinating material of invasive and potentially invasive ruderal and/or allochthonous invasive and potentially invasive species as a result of soil handling activities, as well as traffic of machinery and personnel | Maximum 500 m The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle was considered as it could potentially generate an impact. | BG0000569 Kardam | - |
|  | Changing air quality | Modernization of the operating roads | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mw Calculation methodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the assessment threshold values established according to Law no. 104/2011 for the protection of vegetation: SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19.5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the air emission estimates presented in Section 1.1.7, Figures 1-4 - 1-9, the maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum distance at which the concentration of 1.3 µg/m3 is felt is 1.45 km from the turbine platform boundary NO2 (annual average) - maximum distance at which the concentration of 8,5 µg/m3 is experienced is 1,56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 is experienced at a maximum distance of 1.2 km from the turbine platform boundary CO (8-hour average) - maximum value of 0.0334 mg/m3 is experienced at a maximum distance of 1.2 km from the edge of the turbine platform | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 analysis | The noise level recorded at the designated receptors during the construction period will be below the limit values set by Order No. 119/2014 for the approval of the Hygiene and Public Health Standards on the living environment of the population (55 dB during the day, 45 dB at night), given the point sources (machinery involved in construction), their small number, their distance from sensitive receptors and the duration of execution According to the air emission estimates presented in Section 1.1.7, the maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area Possible disruption of the activity of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts). Possible disruption of the activity of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Introduction / spread of invasive species |  | Field investigations to identify and map areas of high abundance of invasive plant species (April 2022 - March 2023). Spatial analysis to highlight the risk of spread of these species. Analysis ArcGIS Pro 3.2.1 | According to specialized literature and information provided by the reports of the project "Management of Invasive Species in Romania" benefiting the Ministry of Environment, the area is characterized by a low presence of invasive species. However, given the specificity of construction and agricultural activities, the possibility of the introduction and spread of invasive species is not excluded. The spread of invasive plant species may be favored by anemocory or hydrocorrhizal pathways, with the risk of invasive plant species occurring within the site, or in its vicinity, given the small distance between the site and the plane. There may also be contamination of soil with germinating material of invasive and potentially invasive ruderal and/or allochthonous invasive and potentially invasive species as a result of soil handling activities, as well as traffic of machinery and personnel | Maximum 500 m The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle was considered as it could potentially generate an impact. | BG0000569 Kardam | - |
|  | Changing air quality | Earthworks | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mw Calculation methodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the maximum permissible concentrations according to Law no. 104/2011 for vegetation protection: SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19.5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the air emission estimates presented in Section 1.1.7, Figures 1-4 - 1-9, the maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum value of 1,3 µg/m3 at a maximum distance of 1,45 km from the edge of the turbine platform NO2 (annual average) - maximum value of 8,5 µg/m3 at a maximum distance of 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 at a maximum distance of 1.2 km from the edge of the turbine platform CO (8-hour average) - maximum value of 0.0334 mg/m3 at a maximum distance of 1.2 km from the turbine platform boundary | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 Analysis | The noise level recorded at the designated receptors during the construction period will be below the limit values set by Order No. 119/2014 for the approval of the Hygiene and Public Health Standards on the living environment of the population (55 dB during the day, 45 dB at night), given the point sources (machinery involved in construction), their small number, their distance from sensitive receptors and the duration of execution According to the estimates presented in Section 1.1.7, of this report, the maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area. Possible disruption of the activity of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts) Possible disruption of the activity of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii and avifauna* | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Introduction / spread of invasive species |  | Field investigations to identify and map areas of high abundance of invasive plant species (April 2022 - March 2023). Spatial analysis to highlight the risk of spread of these species. Analysis ArcGIS Pro 3.2.1 | The spread of invasive plant species may be favored by anemocory or hydrocorrhizal pathways, with the risk of invasive plant species occurring within the site, or in its vicinity, given the small distance between the site and the plane. There may also be contamination of soil with germinating material of invasive and potentially invasive ruderal and/or allochthonous invasive and potentially invasive species as a result of soil handling activities, as well as traffic of machinery and personnel | Maximum 500 m The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle was considered as it could potentially generate an impact. | BG0000569 Kardam | - |
|  | Changing air quality | Foundation works | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mw Calculation methodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the maximum permissible concentrations according to Law no. 104/2011. SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19.5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 The maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum value of 1.3 µg/m3 at a maximum distance of 1.45 km from the edge of the turbine platform NO2 (annual average) - maximum value of 8.5 µg/m3 at a maximum distance of 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 at a maximum distance of 1.2 km from the edge of the turbine platform CO (8-hour average) - maximum value of 0.0334 mg/m3 at a maximum distance of 1.2 km from the turbine platform boundary | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 analysis | The noise level recorded at the designated receptors during the construction period will be below the limit values set by Order No. 119/2014 for the approval of the Hygiene and Public Health Standards on the living environment of the population (55 dB during the day, 45 dB at night), given the point sources (machinery involved in construction), their small number, their distance from sensitive receptors and the duration of execution The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area. Possible disruption of the activity of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts) Possible disruption of the activity of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii and avifauna* | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Increased noise and vibration | Installation of installations/ equipment | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 analysis | Noise level estimates presented in Section 1.1.7, Figure 1-10a The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | According to the noise level estimates presented in Section 1.1.7, noise values above the maximum permissible limits may extend up to a distance of about 60 m from the working fronts and site organizations. The effects will be felt locally at distances not likely to affect sensitive receptors in the area The 30 dB noise isoline will be felt at distances of maximum 1.5 km around the work area Possible disturbance of up to 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts) Possible disruption of the activity of up to 2 to 3 individuals of the mammalian species Spermophilus citellus and Mustela eversmanii and avifauna* | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Changing air quality | Construction of the electrical network for discharging the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optics), transformer stations | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mwhodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the maximum permissible concentrations according to Law no. 104/2011 for vegetstion protection. SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19,5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the air emission estimates presented in Section 1.1.7, Figures 1-4 - 1-9, the maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum value of 1.3 µg/m3 at a maximum distance of 1.45 km from the edge of the turbine platform NO2 (annual average) - maximum value of 8.5 µg/m3 at a maximum distance of 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 at a maximum distance of 1.2 km from the edge of the turbine platform CO (8-hour average) - maximum value of 0.0334 mg/m3 at a maximum distance of 1.2 km from the turbine platform boundary | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 Analysis | According to the air emission estimates presented in Section 1.1.7, Figures 1-10a, The noise level recorded at the designated receptors during the construction period will be below the limit values set by Order No. 119/2014 for the approval of the Hygiene and Public Health Standards on the living environment of the population (55 dB during the day, 45 dB at night), given the point sources (machinery involved in construction), their small number, their distance from sensitive receptors and the duration of execution The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts) | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Changing air quality | Site rehabilitation works on completion of construction | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mwhodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the maximum permissible concentrations according to Law no. 104/2011 for vegetation protection: SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19,5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the air emission estimates presented in Section 1.1.7, figures 1-4 – 1-9, the maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum value of 1.3 µg/m3 at a maximum distance of 1.45 km from the edge of the turbine platform NO2 (annual average) - maximum value of 8.5 µg/m3 at a maximum distance of 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 at a maximum distance of 1.2 km from the edge of the turbine platform CO (8-hour average) - maximum value of 0.0334 mg/m3 at a maximum distance of 1.2 km from the turbine platform boundary | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 Analysis | Estimates of noise levels presented in section 1.1.7, figure 1-10a The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | According to the noise level estimates presented in section 1.1.7, noise values exceeding the maximum permissible limits may extend up to a distance of approximately 60 m from the working fronts and site organization. The effects will be felt locally, at distances that are not likely to affect sensitive receptors in the area. The 30 dB noise contour will be felt at distances of up to 1.5 km around the work area. Possible disturbance of the activity of a maximum of 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts). Possible disturbance of the activity of up to 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii and avifauna* | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Introduction / spread of invasive species |  | Field investigations to identify and map areas of high abundance of invasive plant species (April 2022 - March 2023). Spatial analysis to highlight the risk of spread of these species. Analysis ArcGIS Pro 3.2.1 | The spread of invasive plant species may be favored by anemocory or hydrocorrhizal pathways, with the risk of invasive plant species occurring within the site, or in its vicinity, given the small distance between the site and the plane. There may also be contamination of soil with germinating material of invasive and potentially invasive ruderal and/or allochthonous invasive and potentially invasive species as a result of soil handling activities, as well as traffic of machinery and personnel | Maximum 500 m The distance at which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle, it was considered that this could generate a potential impact. | BG0000569 Kardam | - |
| Operate | Increased noise and vibration | Energy production activities (transformer stations, wind turbines) | Modeling of the noise level caused by turbine operation using windPRO 3.6.366 Analysis ArcGIS Pro 3.2.1 | The results of the noise modeling during the operational period reveal an affected area of about 2860 ha affected by a noise level of 45 dB at night (see noise level estimates presented in section 1.1.7, figure 1-10b) | About 400 m around each turbine. Possible disturbance of the activity of up to 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii, avifauna* and chiroptera**. | - | - |
|  | Increased brightness |  | Analyze ArcGIS Pro 3.2.1 Light pollution map | The activity of the species will not be disturbed by artificial lighting in the area of transformer substations. Safety lighting shall be used in the transformer substation area. To avoid disturbance of noct nocturnal species, UV spectrum-free lighting shall be used, downward oriented as recommended by the European Union on "Light pollution & Climate Change" and equipped with motion sensors. Nor will aviation obstruction light systems used for turbines have the potential to affect sensitive receptors in the area | There will be no additional effects beyond those already existing | - | - |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | Behavioral barriers that may arise with such projects could include turbine avoidance and wind farm siting. It is expected that there will be no significant potential impact on bird and chiropteran species during the construction and operation phase. However, it is recommended to implement a monitoring program covering the whole construction and operation period. For chiroptera, the Eurobats Guide 6 recommends: At least three years of monitoring during the operational phase of the wind farm is necessary to assess the impact on resident species (attractiveness, behavioral changes and mortality) and migratory species (mortality changes) and to highlight any annual variations. This usually involves one complete cycle of bat population fluctuations. Based on the results, a further 3 years may be needed to gain a full understanding of how. A comprehensive monitoring should focus on both activity levels and mortality rates. Post-construction activity monitoring will assess changes in bat activity and will also help to understand the results of mortality monitoring. All monitoring results will be reported annually to the relevant environmental authority. The analyzed area, according to the field inventory data and the "Study on recommendations on areas in Dobrogea, where the siting of wind power plants should be restricted due to the migration corridors of gliding birds (day raptors, storks, pelicans) respectively due to wintering of geese and swans" is in a moderate risk area. Recommendations for these areas are to carry out long-term monitoring (min 5 years) of soaring bird species and/or wintering populations of geese and swans. | Up to 5 km Possible disturbance of the activity of up to 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii, avifauna* and chiroptera**. | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Individual mortality (e.g. collision with wind turbines) |  | Analyze ArcGIS Pro 3.2.1 Collision study | Of the total of 63 bird species for which the presence in the area of the future Cerchezu Wind Farm has been established, only 18 are relevant to the risk of collision with wind turbine blades (mortality): these are raptor or gliding bird species (see Annexes of the Adequate Assessment Study - Annex A07 - Related Studies). For these species the risk of collision was calculated. The results of this study showed that the risk of collision is positive on a calendar year basis, but for none of the analyzed species the risk of collision showed an over unity, resulting in a final positive risk of collision on a year basis, i.e. for the species: Accipiter nisus (0,014 individuals/year), Aquila pomarina (0,066 individuals/year), Buteo buteo (0,605 individuals/year), Buteo rufinus (0,031 individuals/year), Ciconia ciconia (0,998 individuals/year), Circaetus gallicus (0,009 individuals/year), Circus aeruginosus (0,025 individuals/year), Circus cyaneus (0,008 individuals/year), Corvus corax (0,007 individuals/year), Falco subbuteo (0,006 individuals/year), Falco tinnunculus (0,201 individuals/year), Pernis apivorus (0,008 individuals/year). Of these, none of them show annual supra-unit values. This information has also been included in Table 1-10, Chapter 1-2 Effects generated by the plan interventions. Concerning the risk of collision of chiroptera with wind turbines, according to the Eurobats guide the species Pipistrellus nathusii/kuhlii , Pipistrellus pipistrellus, Eptesicus serotinus, Miniopterus schreibersii, Nyctalus leisleri, Nyctalus noctula are at high risk. There is no method to calculate the number of possibly affected individuals, but from the results of the monitoring carried out in the area of the wind farms in Dobrogea, the number of affected individuals was between 1-3 individuals/year. | In and around turbine areas, a buffer of at least 500 m radius should be considered Possible mortality of up to 2 to 3 individuals of bird* and bat** species during the first 3 years of operation. | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
| Decommissioning | Changing air quality | Site organization and site management, including site traffic | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mw Calculation methodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the maximum permissible concentrations according to Law no. 104/2011 for vegetation protection. SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19.5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the estimates presented in section 1.1.7 of this report, figures 1-4 – 1-9. The maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum value of 1.3 µg/m3 at a maximum distance of 1.45 km from the edge of the turbine platform NO2 (annual average) - maximum value of 8.5 µg/m3 at a maximum distance of 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 at a maximum distance of 1.2 km from the edge of the turbine platform CO (8-hour average) - maximum value of 0.0334 mg/m3 at a maximum distance of 1.2 km from the turbine platform boundary | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 analysis | Estimates of noise levels presented in section 1.1.7, figure 1-10a The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | According to the noise level estimates presented in section 1.1.7, noise values exceeding the maximum permissible limits may extend up to a distance of approximately 60 m from the working fronts and site organization. The effects will be felt locally, at distances that are not likely to affect sensitive receptors in the area. The 30 dB noise contour will be felt at distances of up to 1.5 km around the work area. Possible disturbance of the activity of a maximum of 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased brightness |  | Analyze ArcGIS Pro 3.2.1 Light pollution map | Construction work will be carried out during daylight hours only and only security lighting will be used during site organization | There will be no additional effects beyond those already existing | - | - |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts). Possible disturbance of the activity of up to 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii, avifauna* and chiroptera**. | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Introduction / spread of invasive species |  | Field investigations to identify and map areas of high abundance of invasive plant species (April 2022 - March 2023). Spatial analysis to highlight the risk of spread of these species. Analysis ArcGIS Pro 3.2.1 | The spread of invasive plant species may be favored by anemocory or hydrocorrhizal pathways, with the risk of invasive plant species occurring within the site, or in its vicinity, given the small distance between the site and the plane. There may also be contamination of soil with germinating material of invasive and potentially invasive ruderal and/or allochthonous invasive and potentially invasive species as a result of soil handling activities, as well as traffic of machinery and personnel | Maximum 500 m The distance at which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle, it was considered that this could generate a potential impact. | BG0000569 Kardam | - |
|  | Changing air quality | Demolition activities of foundations and all built elements | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mwhodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the maximum permissible concentrations according to Law no. 104/2011 for vegetation protection.. SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19.5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the estimates presented in section 1.1.7 of this report, figures 1-4 – 1-9, The maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum value of 1.3 µg/m3 at a maximum distance of 1.45 km from the edge of the turbine platform NO2 (annual average) - maximum value of 8.5 µg/m3 at a maximum distance of 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 at a maximum distance of 1.2 km from the edge of the turbine platform CO (8-hour average) - maximum value of 0.0334 mg/m3 at a maximum distance of 1.2 km from the turbine platform boundary | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 analysis | Estimates of noise levels presented in section 1.1.7, figure 1-10a The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | According to the noise level estimates presented in section 1.1.7, noise values exceeding the maximum permissible limits may extend up to a distance of approximately 60 m from the working fronts and site organization. The effects will be felt locally, at distances that are not likely to affect sensitive receptors in the area. The 30 dB noise contour will be felt at distances of up to 1.5 km around the work area. Possible disturbance of the activity of a maximum of 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts) Possible disturbance of the activity of up to 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii, avifauna* and chiroptera**. | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Introduction / spread of invasive species |  | Field investigations to identify and map areas of high abundance of invasive plant species (April 2022 - March 2023). Spatial analysis to highlight the risk of spread of these species. Analysis ArcGIS Pro 3.2.1 | The spread of invasive plant species may be favored by anemocory or hydrocorrhizal pathways, with the risk of invasive plant species occurring within the site, or in its vicinity, given the small distance between the site and the plane. There may also be contamination of soil with germinating material of invasive and potentially invasive ruderal and/or allochthonous invasive and potentially invasive species as a result of soil handling activities, as well as traffic of machinery and personnel | Maximum 500 m The distance at which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle, it was considered that this could generate a potential impact. | BG0000569 Kardam | - |
|  | Changing air quality | Land restoration/rehabilitation works at the end of the lifetime of the PP | Analyze ArcGIS Pro 3.2.1 EMEP/EEA Calculation Methodology - 1.A.4 Non road mobile machinery 2023, Tier 1 Mwhodology 1.A.3.b.i-iv Road transport 2023, Tier 2 ADMS 6 atmospheric pollutant dispersion modeling software | The contribution due to the activity carried out during the execution period is not able to generate an impact on air quality, thus the resulting concentrations are considerably lower than the maximum permissible concentrations according to Law no. 104/2011 for vegetation protection. SO2 - lower assessment threshold: 12 µg/m3 (winter) - upper assessment threshold: 8 µg/m3 (winter) NO2 (annual average) - lower assessment threshold: 24 µg/m3 - upper assessment threshold 19,5 µg/m3 PM10 (annual average) - - lower assessment threshold: 28 µg/m3 - upper assessment threshold: 20 µg/m3 CO (8-hour average) - lower assessment threshold: 7 mg/m3 - upper assessment threshold: 5 mg/m3 According to the estimates presented in section 1.1.7 of this report, figures 1-4 – 1-9, the maximum values obtained from pollutant dispersion for each of the above indicators were: SO2 - 1.3 µg/m3 NO2 (annual average) - 8.5 µg/m3 PM10 (annual average) - 1.62 µg/m3 CO (8-hour average) - 0.0334 mg/m3 | The maximum values will be felt locally at the turbine platforms at distances not likely to affect sensitive receptors in the area SO2 - maximum value of 1.3 µg/m3 at a maximum distance of 1.45 km from the edge of the turbine platform NO2 (annual average) - maximum value of 8.5 µg/m3 at a maximum distance of 1.56 km from the turbine platform boundary PM10 (annual average) - maximum value of 1.62 µg/m3 at a maximum distance of 1.2 km from the edge of the turbine platform CO (8-hour average) - maximum value of 0.0334 mg/m3 at a maximum distance of 1.2 km from the turbine platform boundary | BG0000570 Izvorovo - Kraishte ROSAC0071 | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased noise and vibration |  | Noise level modeling using SoundPLANnoise 9.0 ArcGIS Pro 3.2.1 Analysis | Estimates of noise levels presented in section 1.1.7, figure 1-10a The maximum value of the noise level obtained from the modeling was 85 dB, localized strictly at the working front, i.e. the wind turbine platforms. | According to the noise level estimates presented in section 1.1.7, noise values exceeding the maximum permissible limits may extend up to a distance of approximately 60 m from the working fronts and site organization. The effects will be felt locally, at distances that are not likely to affect sensitive receptors in the area. The 30 dB noise contour will be felt at distances of up to 1.5 km around the work area. Possible disturbance of the activity of a maximum of 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii | BG0000569 Kardam | Detailed information in subchapter 1.1.7 Emissions of physical, chemical and biological pollutants generated by PP interventions and activities |
|  | Increased concentration of pollutants in soil/ accidental pollution |  | Analyze ArcGIS Pro 3.2.1 | Areas potentially affected by accidental pollution 15 ha | In the case of accidental pollution, no more than 20 m areas shall be exceeded around new access roads, upgrading of existing access roads to be rehabilitated and power cable routes | Will not affect protected areas, will be locally felt in the area of the working front | - |
|  | Emerging behavioral barriers for wildlife |  | ArcGIS Pro analysis 3.2.1 Analysis of field inventories and monitoring (April 2022 - March 2023) based on observed species behavior. | The effect will be felt in the area of the working fronts, resulting in the temporary removal of foraging and resting in these areas, with the species moving to neighboring areas with similar conditions. It should be noted, however, that the effect will be insignificant and temporary, as the work will be carried out in stages. | Local, at the working fronts (3-5 km around the working fronts). Possible disturbance of the activity of up to 2 to 3 individuals of the mammal species Spermophilus citellus and Mustela eversmanii, avifauna* and chiroptera**. | BG0000569 Kardam BG0000570 Izvorovo - Kraishte ROSPA0166 Plopeni - Chirnogeni ROSPA0036 Dumbrăveni | - |
|  | Introduction / spread of invasive species |  | Field investigations to identify and map areas of high abundance of invasive plant species (April 2022 - March 2023). Spatial analysis to highlight the risk of spread of these species. Analysis ArcGIS Pro 3.2.1 | The spread of invasive plant species may be favored by anemocory or hydrocorrhizal pathways, with the risk of invasive plant species occurring within the site, or in its vicinity, given the small distance between the site and the plane. There may also be contamination of soil with germinating material of invasive and potentially invasive ruderal and/or allochthonous invasive and potentially invasive species as a result of soil handling activities, as well as traffic of machinery and personnel | Maximum 500 m The distance at which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle, it was considered that this could generate a potential impact. | BG0000569 Kardam | - |

*Aquila pomarina,  Falco peregrinus, Hieraaetus pennatus,  Lanius minor, Milvus migrans, Accipiter brevipes, Accipiter nisus, Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Alauda arvensis, Alcedo atthis, Anser albifrons, Anser anser, Anthus campestris, Aquila heliaca,Aquila pomarina, Bubo bubo, Burhinus oedicnemus, Buteo buteo, Buteo lagopus, Buteo rufinus, Calandrella brachydactyla, Caprimulgus europaeus, Carduelis cannabina, Carduelis carduelis, Carduelis chloris, Carduelis spinus, Ciconia ciconia, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coccothraustes coccothraustes, Coracias garrulus, Coturnix coturnix, Cygnus olor, Emberiza melanocephala, Falco cherrug, Falco columbarius, Falco peregrinus, Falco subbuteo, Falco tinnunculus, Falco vespertinus, Ficedula hypoleuca, Fringilla coelebs, Fringilla montifringilla, Galerida cristata, Gallinago gallinago, Hieraaetus pennatus, Hirundo daurica, Hirundo rustica, Jynx torquilla, Lanius collurio,  Lanius excubitor, Lanius minor, Lanius senator, Larus cachinnans, Larus melanocephalus, Larus minutus, Locustella fluviatilis, Locustella luscinioides, Lullula arborea, Melanocorypha calandra, Merops apiaster, Miliaria calandra, Milvus migrans, Monticola saxatilis, Motacilla alba, Motacilla flava, Muscicapa striata, Nycticorax nycticorax, Oenanthe isabellina, Oenanthe oenanthe, Oenanthe pleschanka, Oriolus oriolus, Pelecanus crispus, Pernis apivorus, Philomachus pugnax, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Porzana porzana/Calidris pugnax, Pyrrhula pyrrhula, Recurvirostra avosetta, Regulus ignicapillus, Regulus regulus, Riparia riparia, Saxicola torquata, Serinus serinus, Streptopelia turtur, Sturnus roseus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Tadorna ferruginea, Turdus iliacus, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus, Upupa epops

** Miniopterus schreibersii, Rhinolophus mehelyi, Rhinolophus ferrumequinum, Myotis capaccinii, Nyctalus noctula, Pipistrellus pipistrellus *** Spermophilus citellus, Mustela eversmanii

## Other PPs with which the analyzed PP can generate cumulative impact

In order to assess the impact on species and habitats of the Natura 2000 sites considered in the assessment, the cumulative impact of the plan on them was analyzed. For this purpose, the presence of pressures and threats in the Management Plans and Standard Forms of the sites, as well as other projects to be realized or currently in the process of being realized that have the potential to affect habitats and species of community interest, were analyzed.

The management plans of the potentially affected Natura 2000 sites indicate several pressures and threats that may affect habitats and species of Community interest, among the most common being urbanization and the expansion of land areas. Road or highway construction or road traffic are also mentioned as pressures/threats for some of the sites included in the assessment.

Gathering information on proposed plans and projects in the area was also done in consultation with local stakeholders, in particular the municipalities of the TAUs in the proposed plan area.

Table  1-11 Characteristics of other PPs (under implementation, approved or under evaluation) that may have a cumulative impact on ANPIC with the evaluated PP

| No. ctr. | Name PP | Location in relation to ANPIC (distance) | Effects generated |  |  |  |  |  |  |  |  | Impacts |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Changing air quality | Increased noise and vibration | Increased brightness | Increased concentration of pollutants in soil/pollution | Land occupation | Changes in vegetation | Introduction/ spread of invasive species | Wildlife mortality | Emerging behavioral barriers for wildlife | PH | AH | FH | PAS | REP |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 5.1 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 0.25 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 0.095 km | X | X | - | - | - | - | X | X | X | - | X | X | X | X |
| 4 | Comana - Pecineaga Wind Farm - MV underground electric cables and fiber optic network (proposed) | 0.59 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța (proposed) | 0.55 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 6 | Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența (proposed) | 2.1 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 7 | Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni (proposed) | Intersected in the area of Dumbraveni | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 8 | Construction of underground fiber optic communications network for electronic services, Negru Voda (proposed) | 13.07 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 9 | Electricity supply for water household, Furnica village, Dumbrăveni commune, jud. Constanța (proposed) | 0.56 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 10 | Relocation of the international natural gas transportation pipeline dn 1000T1 Isaccea - Negru Voda, in the ROMCIM quarry area, PL Medgidia (proposed) | 27.71 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 11 | General Urban Plan of Independența Commune (proposed) | Intersection in the area of Independeța | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 12 | Establishment of smart gas distribution network in Independența commune, Constanta county (proposed) | Intersection in the area of Independeța | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 13 | Construction of a bridge in Movila Verde locality, Church Street, Movila Verde locality, Independența commune, Constanta county (proposed) | 0.41 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 14 | Asphalting Comana, Tătaru and Pelinu streets, stage I, Comana commune, Constanta County (proposed) | 9.72 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 15 | Construction of bridges, culverts and rainwater drains in the locality of Credința, commune Chirnogeni (proposed) | Intersection in the area of Credința |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Street modernization and rehabilitation in Chirnogeni commune, Constanta county (proposed) | 6.61 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 17 | Karnobat wind farm (existing) | 16.89 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSCI0157/ROSAC157 Hagieni - Cotul Văii Forest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 13.57 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 20.50 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 19.05 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Wind Farm Comana - Pecineaga - MV underground cables and fiber optic network (proposed) | 1.66 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța (proposed) | 18.19 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 6 | Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența (proposed) | 23,73 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 7 | Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni (proposed) | 26.1 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 8 | Construction of underground fiber optic communications network for electronic services, Negru Voda (proposed) | 0.35 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 9 | Electricity supply for water household, Furnica village, Dumbrăveni commune, jud. Constanța (proposed) | 31.05 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 10 | Relocation of the international natural gas transportation pipeline dn 1000T1 Isaccea - Negru Voda, in the ROMCIM quarry area, PL Medgidia (proposed) | 42.45 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 11 | General Urban Plan of Independența Commune (proposed) | 14.53 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 12 | Establishment of smart gas distribution network in Independența commune, Constanta county (proposed) | 18.72 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 13 | Construction of a bridge in Movila Verde locality, Church Street, Movila Verde locality, Independența commune, Constanta county (proposed) | 21.51 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 14 | Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County (proposed) | 4.22 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 15 | Construction of bridges, culverts and rainwater drains in the locality of Credința, commune Chirnogeni (proposed) | 17.12 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 16 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 10.34 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 17 | Karnobat wind farm (existing) | 13.62 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSPA0036 Dumbrăveni |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 10.66 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 3.34 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 4.67 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Wind Farm Comana - Pecineaga - MV underground cables and fiber optic network (proposed) | 12.75 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța (proposed) | 3.48 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 6 | Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența (proposed) | 5.86 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 7 | Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni (proposed) | 0.03 km | - | X | - | - | - | X | X | - | - | - | - | - | X | - |
| 8 | Construction of underground fiber optic communications network for electronic services, Negru Voda (proposed) | 17.03 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 9 | Electricity supply for water household, Furnica village, Dumbrăveni commune, jud. Constanța (proposed) | 0.43 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 10 | Relocation of the international natural gas transportation pipeline dn 1000T1 Isaccea - Negru Voda, in the ROMCIM quarry area, PL Medgidia (proposed) | 36.90 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 11 | General Urban Plan of Independența Commune (proposed) | Intersected in the north-western part of the TAU | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 12 | Establishment of smart gas distribution network in Independența commune, Constanta county (proposed) | 5.19 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 13 | Construction of a bridge in Movila Verde locality, Church Street, Movila Verde locality, Independența commune, Constanta county (proposed) | 10.61 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 14 | Asphalting Comana, Tătaru and Pelinu streets, stage I, Comana commune, Constanta County (proposed) | 24.86 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 15 | Construction of bridges, culverts and rainwater drains in the locality of Credința, commune Chirnogeni (proposed) | 18.17 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 16 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 17.42 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 17 | Karnobat wind farm (existing) | 23.05 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSPA0166 Plopeni-Chirnogeni |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 3.18 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 4.37 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 7.68 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Wind Farm Comana - Pecineaga - MV underground cables and fiber optic network (proposed) | 1.84 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța (proposed) | 5.13 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 6 | Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența (proposed) | 9.08 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 7 | Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni (proposed) | 11.76 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 8 | Construction of underground fiber optic communications network for electronic services, Negru Voda (proposed) | 8.95 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 9 | Electricity supply for water household, Furnica village, Dumbrăveni commune, jud. Constanța (proposed) | 15.84 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 10 | Relocation of the international natural gas transportation pipeline dn 1000T1 Isaccea - Negru Voda, in the ROMCIM quarry area, PL Medgidia (proposed) | 33.01 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 11 | General Urban Plan of Independența Commune (proposed) | 2.36 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 12 | Establishment of smart gas distribution network in Independența commune, Constanta county (proposed) | 2.46 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 13 | Construction of a bridge in Movila Verde locality, Church Street, Movila Verde locality, Independența commune, Constanta county (proposed) | 5.36 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 14 | Asphalting Comana, Tătaru and Pelinu streets, stage I, Comana commune, Constanta County (proposed) | 7.55 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 15 | Construction of bridges, culverts and rainwater drains in the locality of Credința, commune Chirnogeni (proposed) | 4.39 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 16 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | Intersected | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 17 | Karnobat wind farm (existing) | 18.91 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSPA0094 Hagieni Forest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 17.05 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 23.36 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 23.04 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Wind Farm Comana - Pecineaga - MV underground cables and fiber optic network (proposed) | 4.26 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța (proposed) | 21.91 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 6 | Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența (proposed) | 27.59 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 7 | Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni (proposed) | 29.4 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 8 | Construction of underground fiber optic communications network for electronic services, Negru Voda (proposed) | 5.22 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 9 | Electricity supply for water household, Furnica village, Dumbrăveni commune, jud. Constanța (proposed) | 34.47 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 10 | Relocation of the international natural gas transportation pipeline dn 1000T1 Isaccea - Negru Voda, in the ROMCIM quarry area, PL Medgidia (proposed) | 44.95 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 11 | General Urban Plan of Independența Commune (proposed) | 18.46 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 12 | Establishment of smart gas distribution network in Independența commune, Constanta county (proposed) | 22.6 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 13 | Construction of a bridge in Movila Verde locality, Church Street, Movila Verde locality, Independența commune, Constanta county (proposed) | 25.37 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 14 | Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County (proposed) | 7.43 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 15 | Construction of bridges, culverts and rainwater drains in the locality of Credința, commune Chirnogeni (proposed) | 20.91 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 16 | Street modernization and rehabilitation in Chirnogeni commune, Constanta county (proposed) | 14.26 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 17 | Karnobat wind farm (existing) | 20 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| BG0000569 Kardam |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 9.65 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 13.05 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 7.38 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Wind Farm Comana - Pecineaga - MV underground cables and fiber optic network (proposed) | 8.86 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța (proposed) | 8.37 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 6 | Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența (proposed) | 14.74 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 7 | Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni (proposed) | 13.94 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 8 | Construction of underground fiber optic communications network for electronic services, Negru Voda (proposed) | 7.73 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 9 | Electricity supply for water household, Furnica village, Dumbrăveni commune, jud. Constanța (proposed) | 18.22 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 10 | Relocation of the international natural gas transportation pipeline dn 1000T1 Isaccea - Negru Voda, in the ROMCIM quarry area, PL Medgidia (proposed) | 50.31 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 11 | General Urban Plan of Independența Commune (proposed) | 8.28 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 12 | Establishment of smart gas distribution network in Independența commune, Constanta county (proposed) | 15.04 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 13 | Construction of a bridge in Movila Verde locality, Church Street, Movila Verde locality, Independența commune, Constanta county (proposed) | 17.71 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 14 | Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County (proposed) | 20.86 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 15 | Construction of bridges, culverts and rainwater drains in the locality of Credința, commune Chirnogeni (proposed) | 21.96 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 16 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 13.62 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 17 | Karnobat wind farm (existing) | 0.54 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 1BG0000570 Izvorovo - Kraishte |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 12.70 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 8.51 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 1.63 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Wind Farm Comana - Pecineaga - MV underground cables and fiber optic network (proposed) | 14.77 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța (proposed) | 10.55 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 6 | Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența (proposed) | 10.73 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 7 | Construction of underground fiber optic network for electronic communication services in Constanta County, Independența, Dumbrăveni (proposed) | 6.78 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 8 | Construction of underground fiber optic communications network for electronic services, Negru Voda (proposed) | 15.39 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 9 | Electricity supply for water household, Furnica village, Dumbrăveni commune, jud. Constanța (proposed) | 8.97 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 10 | Relocation of the international natural gas transportation pipeline dn 1000T1 Isaccea - Negru Voda, in the ROMCIM quarry area, PL Medgidia (proposed) | 47.32 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 11 | General Urban Plan of Independența Commune (proposed) | 0.75 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 12 | Establishment of smart gas distribution network in Independența commune, Constanta county (proposed) | 10.83 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 13 | Construction of a bridge in Movila Verde locality, Church Street, Movila Verde locality, Independența commune, Constanta county (proposed) | 15.7 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 14 | Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County (proposed) | 27.11 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 15 | Construction of bridges, culverts and rainwater drains in the locality of Credința, commune Chirnogeni (proposed) | 22,89 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 16 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 18.92 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 17 | Karnobat wind farm (existing) | 12.79 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |

| Legend | PH | habitat loss |
| --- | --- | --- |
|  | AH | altering habitat conditions |
|  | FH | habitat fragmentation |
|  | PAS | species disturbance |
|  | REP | Reducing stock numbers |

From the assessment of the effects generated by this PUZ taken into account for the cumulative impact assessment, it appears that the main forms of impact generated are: disruption of species activity caused by the emergence of behavioral barriers for wildlife and reduction of population numbers as a result of collisions with turbines.

# Information on protected natural areas of Community interest affected by the implementation of the PP

In order to identify the protected natural areas of Community interest potentially affected by the proposed plan, a spatial GIS analysis was carried out taking into account all elements of the plan (including remotely located elements). The selection of Natura 2000 sites potentially affected by the plan involved following several steps:

- Identify all Natura 2000 sites crossed by plan;

- Identifying Sites of Community Importance (SCIs) within a short distance of plan ;

- Identification of Special Protection Areas (SPAs) located within a short distance of plan;

- Identification of Sites of Community Importance (SCIs) where large mammal species are subject to conservation, and which are connected to the route area by ecological corridors;

- Identification of Natura 2000 sites hydrologically linked (via a river) to the area plan.

Based on the results of the analysis of the potentially affected sites according to the above mentioned steps, a final list of sites to be included in the impact analysis of the proposed plan has been established. It is important to note that none of the analyzed sites is intersected by the route of the expressway.

The full list of Natura 2000 sites included in the analysis is presented in the following table.

Table  2-1 Full list of Natura 2000 sites included in the impact assessment and reason for inclusion in the analysis

| Nr. crt | Natura 2000 Site | Intersection | Neighboring SCI | Neighboring SPA | Ecological corridor | Hydrological link |
| --- | --- | --- | --- | --- | --- | --- |
|  | ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake |  | X |  |  |  |
|  | ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii |  | X |  |  |  |
|  | ROSPA0036 Dumbrăveni |  |  | X |  |  |
|  | ROSPA0166 Plopeni-Chirnogeni |  |  | X |  |  |
|  | ROSPA0094 Hagieni Forest |  |  | X |  |  |
|  | BG0000569 Kardam |  |  | X |  |  |
|  | BG0000570 Izvorovo - Kraishte |  |  | X |  |  |

The area of direct influence of the project is represented by the footprint of the "Wind Energy Park 48 (46) wind power plants approx. 316.8MW (303.6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads, and the areas impacted by the project at all stages of the project (execution, operation, decommissioning). The distances up to which the effects of the project are felt were either calculated on the basis of the results of the noise study (Noise Calculation Model ISO9613-2 Poland, maximum operating capacity), the collision study and literature data. The following table shows the areas of direct influence according to the effects generated by the project, correlated with Section 2.3 of the Appropriate Assessment Study.

| Effects generated by the project | Potential impact | Boundary of the zone of influence | Justification |
| --- | --- | --- | --- |
| Construction/decommissioning phase |  |  |  |
| Permanent occupation of areas of land and removal of topsoil/vegetation | AH | Within the site | To intervention involving occupation permanent land |
| Temporary removal of topsoil | PH | Within the site | Habitat alteration on temporarily occupied areas (site organization, underground cable laying trenches) |
| Dispersal / spread of invasive species | AH | 0 m - 500 m depending on species (according to Lososová, Z., et al, 2023 and Pascal Vittoz and Robin Eng ler, 2007). | The penetration distance of species by air |
| Affected physical-chemical parameters of habitats | AH | Within the site | In the area of work fronts, on access roads, |
| Increased noise level | PAS | 60 m from the working frontage | In the area of the working fronts, on the access roads, according to the results of the noise estimates |
| Increased light pollution | PAS | Up to maximum 500 m from the site organization | The distance to which the light sources in site organization can function as wildlife attractant/repellent |
| Emissions of air pollutants | AH | 150-200 m | In the area of work fronts, on access roads, see the results of airborne dispersion of pollutants (section 1.1.7) |
| Potential species traps | REP | Within the site | Turbine foundations, underground cable trenches, transformer station foundations |
| Wildlife colliding with construction machinery and vehicles | REP | Within the site | In the area of work fronts, on access roads |
| Operating stage |  |  |  |
| Increased noise level | PAS | 500 m | The distance up to which noise is experienced during operation according to the results of the noise estimates, corresponding to the 42 dB isolinia |
| Emissions of air pollutants | AH | 150-200 m | In the area of work fronts where repair/maintenance work is being carried out, on access roads |
| Avoidance of to birds during operation | PAS | Up to 500 m from each turbine | The greatest distance of disturbance identified was considered taking into account the species identified in the project area |
| Wildlife collision with turbines (birds and bats) | REP | Rotating area shovels (a radius of 85 m) for each turbine | Collision caused by the spinning of the blades, regardless of the distance traveled |
| Barrier effect | FH | Up to 500 m from each turbine | The greatest distance of disturbance identified was considered taking into account the species identified in the project area |

As for the delimitation of the area of direct influence, an ArcGIS spatial analysis was used, using the buffer function, based on the distances presented in the table above. The minimum size of the zone of influence is 500 m around access roads and cable routes and 900 m around turbines. In areas where several project components may accumulate, the width of the buffer zone may exceed 900 m.

The area of indirect influence of the project is the existing activities that may be affected of the project, which is the case for car traffic on the existing road transportation infrastructure. The wind farm components will be transported by the shortest route from the Port of Constanta into the site. The additional traffic generated by the project is not likely to significantly influence the area of influence of these roads. Therefore, no additional indirect area of influence impacts were identified.

## Data on protected natural areas of Community interest

The following table summarizes information on protected natural areas of Community interest: location plan in relation to the site, surface area, importance/role, management plan and OM. by which it was approved, the Decision/Note approving the conservation objectives of the sites, the biogeographical regions in which the sites are located, the types of ecosystems in the sites, the overlap with other sites or protected natural areas, the relationship of the sites with other sites, other particularities.

Table  2-2 Data on species and habitats potentially affected by PP

| Name and ANPIC code | Area ( ha) | Importance/ Role | Management plan and OM no. by which it was approved | Decision/ Note approving the ANPIC Conservation Objectives | Biogeographical region(s) where ANPIC is located | Types of ecosystems | Overlap with other ANPICs or PAs | Relations with other AN PICs | Other particularities |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | 18024.4 | It supports 8 natural habitats and 2 species of amphibians, 5 species of fish, 2 species of plants, 4 species of invertebrates, 6 species of mammals and 4 species of reptiles | Order of the Minister of Environment, Waters and Forests no. 1557/2016 on the approval of the Management Plan of the protected natural areas ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Vederoasa Pond, 2.361 Dumbrăveni Forest, 2.350 Limestone walls of Petrosani - Deleni Commune, 2.351 Aliman fossiliferous site and IV.30 Vederoasa Lake | Decision No 414 of 03.08.2022 | Stepică | Rivers, lakes, swamps, bogs, peat bogs, natural meadows, steppes, agricultural land, grasslands, broadleaved forests, vineyards and orchards, artificial land, forest habitats | ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake overlaps ROSPA 0036 Dumbrăveni 2056 ha, Dumbrăveni Forest 2361 ha, ROSPA0001 Aliman - Adamclisi - 19.46 8 ha, Aliman Fossiliferous Site 2351 ha, ROSPA0007 Vederoasa Pond 2144 ha, Vederoasa Lake IV.30, ROSPA0039 Danube - Ostrove 16.224 ha, Bratca Forest IV.26, Limestone Walls of Petroșani 2350 ha | ROSPA0036 Dumbrăveni overlaps with ROSAC0071 DumbrăveniValea UrluiaValea Vederoasa Lake and Dumbrăveni Forest. | - |
| ROSCI0157/ROSAC157 Hagieni Forest | 3680.1 | It supports 7 natural habitats and 2 plant species, 4 invertebrate species, 7 mammal species and 3 reptile species | Order no. 1480/2016 on the approval of the Management Plan and Regulation of the protected natural areas ROSCI0157 Hagieni-Cotul Văii Forest, ROSPA0094 Hagieni Forest and 2.360 Hagieni Forest | Decision No 197 of 26.06.2020 and 426 of 24.09.2020 | Stepica and the Black Sea | Lacuri, tufărișuri, lizieră cu ierburi înalte, păduri de stejar, cer și gorun, stepe | Partial overlap with ROSPA0066 Limanu - Herghelia (approx. 423 ha) and full overlap with RONPA0377 Hagieni Forest, RONPA0375 Limanu Cave and ROSPA0094 Hagieni Forest | Overlap with ROSPA0066 Limanu - Herghelia, RONPA0377 Hagieni Forest, RONPA0375 Limanu Cave and ROSPA0094 Hagieni Forest | - |
| ROSPA0036 Dumbrăveni | 1903.6 | Particular importance through the 22 bird species of European conservation concern (bird species listed in Annex I of Council Directive 2009/147/EC) | Order of the Minister of Environment, Waters and Forests no. 1557/2016 on the approval of the Management Plan of the protected natural areas ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Vederoasa Pond, 2.361 Dumbrăveni Forest, 2.350 Limestone walls of Petrosani - Deleni Commune, 2.351 Aliman fossiliferous site and IV.30 Vederoasa Lake | Decision No 414 of 03.08.2022 | Stepică | Transitional forests, deciduous forests, sub-Mediterranean limestone ridges, reedbeds, dry grasslands, steppes and extensive cereal crops | It overlaps (partially 1903.6 ha) ROSCI0071 Dumbraveni-Valea Urluia-Lacul Vederoasa and RONPA0378 Dumbraveni Forest | Overlap with ROSCI0071 Dumbraveni-Valea Urluia-Lacul Vederoasa and RONPA0378 Dumbrăveni Forest | - |
| ROSPA0166 Plopeni-Chirnogeni | 137.2 | Add 9 bird species | No | Note No 8914/BT/ 28.03.2022 | Stepică | Predominantly agricultural land mixed with natural vegetation, vines, non-irrigated arable land | No overlap with other protected natural areas | Possible feeding habitat in the area ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake for the identified species | - |
| ROSPA0094 Hagieni Forest | 1414.8 | Add 121 bird species | Order no. 1480/2016 on the approval of the Management Plan and Regulation of the protected natural areas ROSCI0157 Hagieni-Cotul Văii Forest, ROSPA0094 Hagieni Forest and 2.360 Hagieni Forest | Decision No 197 of 26.06.2020 and 426 of 24.09.2020 | Stepică | Rivers, lakes, swamps, marshes, steppes, arable land, grasslands, deciduous forests, forests in transition | It overlaps (partially 1414.8 ha) with ROSCI0157/ROSAC157 Hagieni - Cotul Văii Forest and (entirely) RONPA0377 Dumbrăveni Forest | Overlap with ROSCI0157/ROSAC157 Hagieni - Cotul Văii Forest and RONPA0377 Dumbrăveni Forest | - |
| In a cross-border context |  |  |  |  |  |  |  |  |  |
| BG0000569 Kardam | 918.338 | Add 4 mammal and 4 reptile species | Order No. RD - 1021/17.12.2020 (promulgated by SG 17/2021) promulgated by the Ministry of Environment and Water | Unknown | Continental | Steppe, arable land, scrubland, deciduous forests, orchards, vineyards | No overlap with other protected natural areas | Possible ecological connectivity through the Cerchez stream (cadastral code - XIV_1.40.1.1.1) with ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | - |
| BG0000570 Izvorovo - Kraishte | 1082.27 | Home to 1 species of invertebrates, 6 species of mammals and 4 species of reptiles | Council of Ministers Decision No 122/02.03.2007 (promulgated by SG 21/2007) | Unknown | Continental | Steppes, arable land, scrublands, deciduous forests, water bodies, vineyards | No overlap with other protected natural areas | Possible ecological connectivity through the Ceair stream (cadastral code - XIV_1.40.1) with ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | - |

The description of the sites is in line with the specific Conservation Objectives, Standard Forms and Management Plans of the Natura 2000 sites potentially affected by plan.

### ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake

It was declared a Site of Community Importance by Order MMDD no. 1964/2007 on the establishment of the protected natural area regime of sites of Community Importance, as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

The area has been designated as a Site of Community Importance because it hosts at least 8 types of habitats protected at European level, of which 3 are priority, i.e. strictly protected: pont-sarmatic steppes - about 160 ha in the site, scrublands of pont-sarmatic deciduous trees present in coastal areas, on stony substrate, with limestone on the surface, with a total area of about 200 ha in the site and East European forests of downy oak, whose total area has been estimated at about 900 ha. Within these habitats 58 nationally protected rare plant species have been identified. Four of these are also protected at European level: Centaurea jankae, Echium russicum, Himantoglossum caprinum and Potentilla emilli-popii.

ROSCI0071 Dumbrăveni - Urluia Valley - Lake Vederoasa has an area of 17.971 ha and overlaps ROSPA0036 Dumbrăveni (2056 ha), Dumbrăveni Forest (2361 ha), ROSPA0001 Aliman - Adamclisi (19.468 ha), Aliman Fossiliferous Site (2351 ha), ROSPA0007 Vederoasa Pond (2144 ha), Vederoasa Lake (IV.30), ROSPA0039 Danube - Ostroave (16,224 ha), Bratca Forest (IV.26), Limestone Walls of Petroșani (2350 ha). ROSAC0071 (ROSCI0071) Dumbrăveni - Urluia Valley - Lake Vederoasa is located in the territory of the following localities: Vlahii, Aliman, Floriile, Adâncata, Crângu, Adamclisi.

Located in southern Dobrogea, the ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake site presents a rich biodiversity, which is due to a mosaic of different habitats developed on a varied geomorphology: here we find limestone cliffs with caves, rocky hills and loess, valleys with temporary waters, permanent watercourses and lakes. Interspersed with forests we find scrublands and ponto-sarmatic grasslands.

Habitat types present on the site are as follows:

- 62C0* Point-sarmatic stepping stones

- 91I0 * Eurosiberian forest vegetation with Quercus spp.

- 3150 Natural eutrophic lakes with Magnopotamion or Hydrocharition vegetation

- 40C0* Deciduous broad-leaved deciduous ponto-sarmatitic forests

- 6430 Lowland, upland to montane and alpine, tall-grass, hygrophilous upland communities

- 91F0 Mixed riparian forests with Quercus robur, Ulmus laevis, Fraxinus excelsior or Fraxinus angustifolia, along large rivers - Ulmenion minoris - -

- 91AA Forest vegetation punctuate-sarmatic with downy oak

- 91M0 Balkan-Pannonian sky and gorun forests

In terms of their importance for the conservation of endangered habitats, the site's forest-scrub formations are of the highest priority. Two plant types have been identified:

- Danubian-west-pontic mixed forests of the western-pontic forests of the red oak Quercus penduculiflora and silver lime Tilia tomentosa with topora Viola jordanii. This type of habitat has a small area of only about 3000 ha at national level and is of very high conservation value. Within the study area they represent 2% of the total area. It is characterized by the plant association Querco penduculiflorae-Tilietum tomentosae. The phytocenoses are edified by sub-Mediterranean and continental European and Caucasian species. The tree layer is composed of the European oak Quercus penduculiflora, silver lime Tilia tomentosa, these two species are edifying for the type of habitat, the European maple Acer tataricum, the European maple Fraxinus ornus, the European ash Fraxinus excelsior, the European ash Sorbus torminalis, the elm Ulmus procera.

- Pontic sylvatic vegetation of scrub oak and Scot's oak and Scot's maple. In the country, it is reported on relatively small areas totaling 16,000 ha in southern and south-eastern Romania. In the rest of Europe this habitat has not been reported so far. The characteristic association observed is Quercetum penduculiflorae-cerris. Edi edifying species observed: prickly maple Acer tataricum, Turkey maple Quercus tataricum, Turkey oak Quercus cerris, European oak Quercus penduculiflora. Threatened plant species in these forests: wood alder Asphodeline lutea, common ash Crocus pallisii, Mercurialis ovata, Myrrhoides nodosa, Pipitatherum virescens.

European protected plant species - Annex II of the Habitats Directive - are present on the site: Centaurea jankae, Himantoglossum caprinum, five-toed weed Potentilla emilii-popii, snake's head Echium russicum.

Of the invertebrate species the order Lepidoptera butterflies is best represented. Of these 6 butterflies are protected at European level under the Habitats Directive: Lycaena dispar a vulnerable species that prefers habitats with swampy or damp oak forests, rich in Polygonum bistorta roach, which is a plant that is the insect's staple food; Lucanus cervus, whose presence is determined by the old trees in the Dumbrăveni forest; Eriogaster catax moth, a very rarely reported species in Romania; two other species are Catopta thrips and Asthena anseraria.

Among the species of reptiles and amphibians present in the site are the Dobrogean crested newt Triturus dobrogicus, the red-bellied puff-bellied opossum Bombina bombina, the water turtle Emys orbicularis and others. One of the most important species is the Dobrogean turtle Testudo graeca, declared a natural monument and strictly protected by law. The population of this species is currently in drastic decline.

This site hosts a rich avifauna, with over 100 bird species identified. Some bird species have important flocks such as raptors, larks, woodpeckers, woodpeckers, blackcap and others. which is why it has been designated as a Special Avifauna Protection Area.

The mammalian fauna includes a significant number of species including the otter Lutra lutra lutra, the gopher Spermophilus citellus and many bat species.

Figure 2-1 Location plan in relation to ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake

### ROSPA0036 Dumbrăveni

It was established as a special avifaunistic protection area by GD no. 1284/2007 on the declaration of special avifaunistic protection areas as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

ROSPA0036 Dumbrăveni is of particular importance for its 22 species of birds of European conservation interest, being important because two globally threatened species are present here, the common moorhen Coracias garrulus, which nests here, and the white ib ibis, Circus macrourus.

The site is also important because it is an important nesting area for the EU threatened species Dendrocopos syriacus, Lanius minor, Hieraaetus pennatus, Emberiza hortulana, Lullula arborea, Accipiter brevipes, Milvus migrans, Buteo rufinus. The site is particularly important during migration for raptor species.

ROSPA0036 Dumbrăveni with an area of 2.056 ha overlaps with ROSCI0071 Dumbraveni-Valea Urluia-Lacul Vederoasa and Dumbrăveni Forest and is found in the territory of the following localities: Șipotele, Tufani, Furnica, Dumbrăveni, Olteni.

From the avifaunistic point of view, the Dumbrăveni Forest - together with its surroundings - hosts important flocks of protected bird species. The following categories have been identified within the site: 23 species listed in Annex 1 of the Birds Directive; 66 migratory species, listed in the Annexes of the Bonn Convention on Migratory Species; 2 globally endangered species.

The protected area is an important nesting area for many species such as the garden woodpecker Dendrocopos syriacus, the garden presura Emberiza hortulana, the black-browed spruce grouse Lanius minor, short-legged hawk Accipiter brevipes, short-tailed tit Anthus campestris, black-headed tit Circus cyaneus, little eagle Hieraaetus pennatus, white-tailed tit Circus macrourus, and the field martin Coracias garrulus, the last two species being globally threatened.

The site is crossed during migrations by numerous species of birds of prey, including the southern heron Circus pygargus, the red heron Circus cyaneus, the white heron Circus macrourus, the peregrine falcon Falco peregrinus, the black jay Milvus migrans.

The 22 species of birds listed in Annex I of Council Directive 2009/147/EC, comprise: A402 Accipiter brevipes, A403 Buteo rufinus, A092 Hieraaetus pennatus, A231 Coracias garrulus, A246 Lullula arborea, A339 Lanius minor, A379 Emberiza hortulana, A084 Circus pygargus, A082 Circus cyaneus, A083 Circus macrourus, A103 Falco peregrinus, A338 Lanius collurio, A242 Melanocorypha calandra, A533 Oenanthe pleschanka, A255 Anthus campestris, A089 Aquila pomarina, A243 Calandrella brachydactyla, A224 Caprimulgus europaeus, A321 Ficedula albicollis, A429 Dendrocopos syriacus, A072 Pernis apivorus, A073 Milvus migrans.

Figure 2-2 Location plan in relation to ROSPA0036 Dumbrăveni

### ROSPA0094 Hagieni Forest

It was established as a special avifauna protection area by the Government Decision no. 1284/2007 on the declaration of special avifauna protection areas as an integral part of the Natura 2000 European Ecological Network in Romania, with subsequent amendments and additions.

The site R0SPA0094 Hagieni Forest hosts important populations of protected bird species, i.e. 48 species of Community importance for conservation.

ROSPA 0094 Hagieni Forest has an area of 1374 ha and is located within ROSCI 0157 Hagieni-Cotul Văii Forest, partially overlapping with the Hagieni Forest Reserve (code 2360).

This site hosts important flocks of protected bird species: 48 species listed in Annex 1 of Directive 2009/147/EC of the European Parliament and of the Council of November 30, 2009 on the conservation of wild birds; 38 migratory species, listed in the annexes of Law no. 13/1998 for Romania's accession to the Convention on the Conservation of Migratory Species of Wild Animals, adopted in Bonn on June 23, 1979: 5 globally endangered species.

The site is important for nesting populations of the following species: Falco cherrug, Falco vespertinus, Coracias garrulus, Aquila heliaca, Accipiter brevipes, Antus campestris, Calandrella brachydactyla, Lanius minor, Milvus migrans, Pernis apivorus, Picus canus, Lullula arborea, Melanocorypha calandra.

The site is important during migration for the species Larus melanocephalus, Larus minutus, Pelecanus crispus, Phalacrocorax pygmaeus.

The site designated by the Romanian Ornithological Society as IBA - Important Birds Area, according to the following criteria elaborated by BirdLife International: C1, C2, C3, C6.

The 48 bird species listed in Annex I of Directive 2009/147/EC include: Accipiter brevipes, Alcedo atthis, Anthus campestris, Aquila heliaca, Burhinus oedicnemus, Buteo rufinus, Calandrella brachydactyla, Caprimulgus europaeus, Ciconia ciconia, Charadrius alexandrinus, Chlidonias hybridus, Chlidonias niger, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulous, Dendrocopos medius, Dendrocopos syriacus, Dryocopus martius, Egretta alba, Emberiza hortulana, Falco cherrug, Falco columbarius, Falco peregrinus, Falco vespertinus, Ficedula albicollis, Ficedula parva, Himantopus himantopus, Hieraaetus pennatus, Lanius minor, Lanius collurio, Larus melanocephalus, Larus minutus, Lullula arborea, Melanocorypha calandra, Milvus migrans, Nycticorax nycticorax, Oenanthe pleschanka, Pelecanus crispus, Pernis apivorus, Picus canus, Philomachus pugnax, Phalacrocorax pygmeus, Porzana porzana, Recurvirostra avosetta, Tadorna ferruginea.

Figure 2-3 Location plan in relation to ROSPA0094 Hagieni Forest

### ROSAC0157 Hagieni Forest - Cotul Văii

It was established by the Order of the Minister of Environment and Sustainable Development no. 1964/2007 on the establishment of the protected natural area regime of sites of Community importance, as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

ROSCI0157 The Hagieni-Cotul Văii Forest is of great scientific importance because of the 6 habitats and 18 species of Community interest for which it has been designated.

R0SCI0157 Hagieni-Cotul Văii Forest has an area of 3618 ha and overlaps with R0SPA0094 Hagieni Forest and Hagieni Forest Reserve-cod 2.360.

Habitat types present on the site - 6 habitat types:

- 3150 Natural eutrophic lakes with Magnopotamion or Hydrocharition vegetation

- 6430 Lowland, upland to montane and alpine, tall-grass, hygrophilous upland communities

- 91AA Forest vegetation punctuate-sarmatic with downy oak

- 40C0* Deciduous broad-leaved deciduous ponto-sarmatitic forests

- 62C0* Point-sarmatic stepping stones

- 91M0 Balkan-Pannonian sky and gorun forests

The site hosts:

- 7 species of mammals listed in Annex II to Directive 92/43/EEC Spermophilus citellus, Mesocricetus newtoni, Rhinolophus mehelyi, Miniopterus schreibersii, Rhinolophus ferrumequinum, Myotis capaccinii, Mustela eversmannii;

- 2 species of amphibians and reptiles listed in Annex II of Directive 92/43/EEC: Testudo graeca, Emys orbicularis;

- 4 plant species listed in Annex II of Directive 92/43/EEC: Himantoglossum caprinum, Potentilla emilii-popii, Centaurea jankae, Echium russicum:

- 42 Other important flora and fauna species:Anacamptis pyramidalis, Beta trigyna, Centaurea napulifera ssp. thirkei, Centaurea rutifolia ssp. jurineifolia, Centaurea varnensis, Chamaecytisus jankae, Colchicum fominii Convolvulus lineatus, Crocus chrysanthus, Dictamnus albus, Galanthus elwesii, Heliotropium dolosum, Hottonia palustris, Minuartia bilykiana, Myrrhoides nodosa, Nepeta parviflora, Ononis pusilla, Onosma taurica, Opopanax bulgaricus, Orchis purpurea, Orchis simian, Ornithogalum comosum, Ornithogalum oreoides, Paeonia peregrine, Paeonia tenuifolia, Parietaria lusitanica ssp. serbica, Phleum subulatum, Plumbago europaea, Prunus tenella, Rindera umbellate, Salvia ringens, Salvinia natans, Scabiosa micrantha, Scorzonera mollis, Seseli tortuosum, Silene dichotoma, Stachys oblique, Sternbergia colchiciflora, Stipa ucrainica, Trapa natans, Trigonella gladiate, Trigonella monspeliaca and animals: Vipera ammodytes, Lacerta viridis, Lacerta taurica, Coluber jugularis.

Figure 2-4 Location plan in relation to ROSAC0157 Hagieni Forest - Cotul Văii

### ROSPA0166 Plopeni-Chirnogeni

The natural avifaunistic protection area was declared in 2016 by Government Decision no. 663/2016 on the establishment of the protected natural area regime and the declaration of special avifaunistic protection areas as an integral part of the Natura 2000 ecological network in Romania. The site has an area of 137.20 ha being designated for 9 bird species namely: Anthus campestris, Buteo rufinus, Circus cyaneus, Dendrocopos syriacus, Emberiza hortulana, Falco columbarius, Falco vespertinus, Lanius collurio, Lanius minor. The site is aligned from north to south, between the localities of Plopeni and Chirnogeni (Constanța County), along the road linking the two localities. Evening vultures (Falco vespertinus) nest in the nests of the seed crow in the walnut trees lining the road. More or less degraded grassland is found along the sides of the road, also on the site. Also within the site, there are significant plantations of acacia, which may in the future harbor other colonies of red grouse. Surrounding the site are agricultural areas which are the hunting grounds for the evening red grouse.

Figure 2-5 Location plan in relation to ROSPA0166 Plopeni-Chirnogeni

### BG0000569 Kardam

It was established as a Site of Community Importance in 2008 and covers 918.92 ha. The site is important for the existing invertebrate fauna. Small isolated patches of steppe and dry karst valleys and canyons are important for small mammals and rare steppe bats.

The site has been designated for the protection of 8 fauna species: 4 reptiles (Testudo hermanni, Testudo graeca, Emys orbicularis, Elaphe sauromates) and 4 mammals (Vormela peregusna, Spermophilus citellus, Mustela eversmanii, Mesocricetus newtoni) and two habitats (6110 Rock meadows with pioneer species and 6210 Steppe meadows).

Figure 2-6 Location plan in relation to BG0000569 Kardam

### BG0000570 Izvorovo - Kraishte

It was established as a Site of Community Importance in 2008 and covers 1081.52 ha. The site is important for the existing invertebrate fauna. Small isolated patches of steppe and dry karst valleys and canyons are important for small mammals and rare steppe bats.

The site has been designated for the protection of 11 fauna species: 4 reptiles (Testudo hermanni, Testudo graeca, Rmys orbicularis, Elaphe sauromates), 6 mammals (Canis lupus, Vormela peregusna, Spermophilus citellus, Rhinolophus mehelyi, Mustela eversmanii, Mesocricetus newtoni) and one invertebrate (Lycaena dispar) and two habitats (6110 Rock meadows with pioneer species and 62C0 Ponto-Sarmatian Steppe).

Figure 2-7 Location plan in relation to BG0000570 Izvorovo - Kraishte

Figure 2-8 Location of the natural areas in the vicinity of the plan

## Data on habitats/ species in ANPIC possibly affected by PP

The following table presents data on the habitats and species in Natura 2000 sites potentially affected by the implementation of the plan. The information presented is in accordance with the Specific Conservation Objectives for each site, the management plans of the sites, data from the literature and databases available online. The plan is in the continental biogeographical region, and does not cross any Natura 2000 sites.

To establish baseline conditions (current stage "zero"/"pre-construction") on the presence and stock/area cover of community bird species and habitats in the area of the plan, the recommendations of the specialized guidelines on the conduct of appropriate assessment studies were used.

Table  2-3 Data on species and habitats potentially affected by PP

| Name and ANPIC code | Species/habitat name | Habitat & species localization | Population size (individuals) | Quantified information on the presence of individuals | Population dynamics | Habitat area of the species (ha) | Habitat area (ha) | Conservation status | Trends | Species ecology | Sensitivity to PP effects | Outlook - climate change |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSCI0071 | Spermophilus citellus (Gopher) | South-eastern part of the site | 1000-5000 | Population size between 1000-5000 | Unknown | 300 - 350 | - | Favorable | Maintenance | Highly mobile species It prefers steppe areas with low and very low grassy vegetation (grassland and well-drained soil) where it makes its galleries. A total of 39 individuals were observed | Habitat loss/alteration Reduced populations | Stable |
|  | Miniopterus schreibersii (Long-winged Bat) | Southern part of the site, mainly in the Viroaga area | To be defined within 2 years | At least 1500 | Unknown | To be defined within 2 years | - | Unfavorable | Improvement | Highly mobile species The species forages in more or less open habitats, even in peri-urban areas, and the foraging flight is high (10-20 m). It prefers heavily wooded areas. It often feeds far from its roost, about 30 km. A total of 85 contacts were obtained during the monitoring period | Declining herds - mortality risk | Stable |
|  | Rhinolophus mehelyi (Méhely's horseshoe bat) | Southern part of the site, mainly in the Viroaga area | To be defined within 2 years | At least 1500 | Unknown | To be defined within 2 years | - | Unfavorable | Improvement | Highly mobile species They hunt in deciduous forests, steppe areas, grasslands, sometimes agricultural land. Although a sedentary species, it can migrate distances of up to 94-100 km During monitoring campaigns the species was not identified | Declining herds - mortality risk | Stable |
| ROSCI0157 | Spermophilus citellus (Gopher) | Western part of the site, including the area of Cotu Văii | 1000-5000 | Population size between 1000-5000 | Unknown | 300 - 350 | - | Favorable | Maintenance | Highly mobile species Highly mobile species It prefers steppe areas with low and very low grassy vegetation (grassland and well-drained soil) where it makes its galleries. A total of 39 individuals were observed | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus ferrumequinum (Greater horseshoe bat) | Central area of the site, close to the village of Hagieni | 100 | Maximum 100 | Unknown | 30 | - | Unfavorable-inadequate | Improvement | Highly mobile species Foraging habitats include deciduous forests (especially spring) and grasslands (summer and fall). Also frequently flies in gardens, rocky and hilly areas. During monitoring campaigns the species was not identified | Declining herds - mortality risk | Stable |
|  | Miniopterus schreibersii (Long-winged Bat) | The central area of the site, but also in the area of Vâlcelelele | 1000 | 1000 | Unknown | 2 | - | Unfavorable-inadequate | Improvement | Highly mobile species The species forages in more or less open habitats, even in peri-urban areas, and the foraging flight is high (10-20 m). It prefers heavily wooded areas. It often feeds far from its roost, about 30 km. A total of 85 contacts were obtained during the monitoring period | Declining herds - mortality risk | Stable |
|  | Mustela eversmanii (Steppe ferret) | Western part of the site | 10-50 | 10-50 | Unknown | 150-250 | - | Favorable | Maintenance | Highly mobile species It prefers natural steppe grasslands with sparse shrubby vegetation, avoiding areas with agricultural monocultures or forested land. It can travel up to 18 km per night. One individual of the species was identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
| ROSPA0166 | Anthus campestris ( Field martin) | No information available on the location of the species | 20-30 | 20-30 | Unknown | 41.27 | - | Unknown | Maintain or improve | Highly mobile species Brooding in open areas in the lowland region on the ground. A total of 10 nesting or transient individuals were identified | Disturbance of species activity and removal Reducing populations | Stable |
|  | Buteo rufinus ( Great Bute) | No information available on the location of the species | 1 | 1 | Unknown | 41.27 | - | Unknown | Maintain or improve | Highly mobile species It grows in arid and semi-desert areas, but also in mountainous areas A total of 14 hunting or migrating individuals were identified | Declining herds - mortality risk | Stable |
|  | Circus cyaneus (Black-headed Erytra) | No information on the location of the species is available | 1 | 1 | Unknown | 41.27 | - | Unknown | Maintain or improve | Highly mobile species The species does not nest in Romania. A total of 7 individuals were identified as being in transit | Reduction in population numbers - risk of mortality Disruption of species activity and removal Habitat alteration Spread of invasive plant species in habitats | Stable |
|  | Falco columbarius ( Winter Falcon) | No information on the location of the species is available | 1 | 1 | Unknown | 136.62 | - | Unknown | Maintain or improve | Highly mobile species The species prefers rocky areas or rock hollows During the monitoring campaigns an individual in transit was identified | Reduction in population numbers - risk of mortality Disruption of species activity and removal | Stable |
|  | Falco vespertinus (Evening Goshawk) | Information on the location of the species is not available | 60-80 | 60-80 | Unknown | 136.62 | - | Favorable | Maintenance | Highly mobile species The species nests in marshes, floodplains, floodplains, farmland, meadows, pastures, scrublands, orchards, parks, heaths, heaths, poles, buildings. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disruption of species activity and removal | Stable |
|  | Lanius minor ( Black-fronted Shrike) | Information on the location of the species is not available | 6-10 | 6-10 | Unknown | 137.62 | - | Unknown | Maintain or improve | Highly mobile species Nesting species in agricultural land, meadows, pastures, scrublands, orchards, parks During the monitoring campaigns, a total of 58 individuals were identified as transiting or feeding | Reduction in population numbers - risk of mortality Disruption of species activity and removal | Stable |
| ROSPA0036 | Accipiter brevipes ( Short-legged Eurasian Short-legged) | Whole natural area | 8-16 | 8-16 | Unknown | 1789.57 | - | Favorable | Maintenance | Highly mobile species The species nests in tree branches at heights of over 4 m, preferring deciduous stands in the middle of meadows, plains and cultivated areas. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Anthus campestris ( Field martin) | Whole natural area | 1600-2000 | 1600-2000 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species Brooding in open areas in the lowland region on the ground. A total of 10 nesting or transient individuals were identified | Declining herds - mortality risk | Stable |
|  | Aquila pomarina (Lesser Spotted Eagle) | Whole natural area | 100-300 | 100-300 | Unknown | 1789.57 | - | Favorable | Maintenance | Highly mobile species The species nests in forests near grasslands, wet meadows and agricultural areas large enough to provide food. A total of 15 migrating or hunting individuals were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Bubo bubo ( Owl) | No information on the location of the species is available | 4 | 4 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species nests in cliffs, coniferous forests, deciduous forests, woodlands. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Buteo rufinus (Great Bute) | Whole natural area | 2-6 | 2-6 | Unknown | 1789.57 | - | Favorable | Maintenance | Highly mobile species It grows in arid and semi-desert areas, but also in mountainous areas A total of 14 hunting or migrating individuals were identified | Declining herds - mortality risk | Stable |
|  | Caprimulgus europaeus ( Caprimulg) | Whole natural area | 30-70 | 30-70 | Unknown | 1789.57 | - | Favorable | Maintenance | Highly mobile species The species nests on the ground, in hollows in meadows or in the shelter of trees or bushes. During the monitoring campaigns an individual in transit was identified | Declining herds - mortality risk | Stable |
|  | Circus cyaneus ( Black-headed Erytra) | Whole natural area | 150-200 | 150-200 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species does not nest in Romania. A total of 7 individuals were identified as being in transit | Declining herds - mortality risk | Stable |
|  | Circus macrourus ( White Erete) | Whole natural area | 60-70 | 60-70 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species prefers marshes near rivers, floodplains, farmland, meadows, pastures, grasslands, steppe, scrublands No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Circus pygargus (Erete sur) | Whole natural area | 150-200 | 150-200 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species nests in open areas with low natural vegetation, isolated scrublands, areas of meadows and grasslands, farmland, scrublands, mires, peat bogs or other marshy areas During the monitoring campaigns a number of 8 individuals in transit were identified | Declining herds - mortality risk | Stable |
|  | Coracias garrulus (Dusky) | Whole natural area | 20-60 | 20-60 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species nests in farmland, meadows, pastures, grasslands, banks, scrub, buildings. During the monitoring campaigns a total of 26 individuals were identified on the ground, in bushes or in transit | Declining herds - mortality risk | Stable |
|  | Falco peregrinus (Peregrine Falcon) | Mainly in the southern part of the area | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species They prefer rocky areas with abundant vegetation No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Hieraaetus pennatus (Little Eagle) | Whole natural area | 16-22 | 16-22 | Unknown | 1789.57 | - | Favorable | Maintenance | Highly mobile species They prefer nesting areas with reeds such as Typha sp., reeds, Phragmites sp., or any other dense aquatic vegetation. During the monitoring campaigns a number of two individuals were identified hunting | Declining herds - mortality risk | Stable |
|  | Lanius collurio ( Red-breasted Shrike) | Southern part of the area | 100-140 | 100-140 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species usually nests in farmland, meadows, pastures, scrublands, orchards, parks. During the monitoring campaigns, a total of 57 individuals were identified as transiting or feeding | Declining herds - mortality risk | Stable |
|  | Lanius minor (Black-fronted Shrike) | External boundaries of the area | 60-100 | 60-100 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species usually nests in farmland, meadows, pastures, scrublands, orchards, parks. During the monitoring campaigns a number of 3 individuals were identified as transiting or feeding | Declining herds - mortality risk | Stable |
|  | Melanocorypha calandra (Bărăgan's Woodpecker) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species nests at ground level or in bushes A total of 55 individuals in transit were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Milvus migrans ( Black Jay) | Whole natural area | 5-8 | 5-8 | Unknown | To be defined within 2 years | - | Favorable | Maintenance | Highly mobile species The species nests in rock hollows and tall trees. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Pernis apivorus ( Viespar) | Whole natural area | 8-16 | 8-16 | Unknown | 1789.57 | - | Favorable | Maintenance | Highly mobile species The species often nests in deserted nests of the common crow (Corvus frugilegus ). During the monitoring campaigns a number of 2 migrating individuals were identified | Declining herds - mortality risk | Stable |
|  | Regularly migratory species Species associated with extensively used agricultural land |  |  |  |  |  |  |  |  |  |  |  |
|  | Alauda arvensis (Woodlark) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species It nests in a variety of low, dense vegetation, often close to water or in damp depressions, including vegetation at the edges of pools, streams and canals, lowland marshes with mats, reedbeds, but avoiding reedbeds in standing water. A total of 560 resting or feeding individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disruption of species activity and removal Habitat loss/alteration Spread of invasive plant species in habitats | Stable |
|  | Carduelis cannabina (Hemp) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species The species nests in open areas with level, moist soil covered with herbaceous vegetation (including cereals), agricultural land, but can also be found in meadows or meadows, on the periphery of marshland, steppes and dunes or in areas of extensive deforestation. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disruption of species activity and removal Habitat loss/alteration Spread of invasive plant species in habitats | Stable |
|  | Carduelis carduelis (Stellar) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species Speci nests in medium-height trees, preferring acacias. During the monitoring campaigns a number of 100 individuals were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Coturnix coturnix (Quail) | No information available on the location of the species | 150-200 | 150-200 | Unknown | 4734.21 | - | Favorable | Maintenance | Highly mobile species The species nests on agricultural land, meadows, pastures, scrublands. During the monitoring campaigns a number of 4 individuals were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Emberiza melanocephala (Black-headed barnacle) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Falco tinnununculus (Red-rumped Vulture) | No information available on the location of the species | 40-60 | 40-60 | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species The species nests in marshes, floodplains, floodplains, farmland, meadows, pastures, scrublands, orchards, parks, heaths, heaths, poles, buildings. During the monitoring campaigns, a total of 55 individuals in transit or hunting were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Fringilla montifringilla (Winter finch) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species The species nests in coniferous and deciduous forests No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Galerida cristata (Woodlark) | No information on the location of the species is available | 200-300 | 200-300 | Unknown | 4734.21 | - | Favorable | Maintenance | Highly mobile species A total of 114 individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Lanius excubitor (Great Crested Crake) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species It forages in farmland, meadows, pastures, scrublands, orchards, parks. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Lanius senator (Red-headed Shrike) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Merops apiaster (Prigorie) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species The species nests on agricultural land, meadows, pastures, grasslands, banks, steppe. A total of 147 resting or feeding individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Miliaria calandra (Southern Mealy Borer) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Motacilla alba (White motacilla) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species It inhabits lakes, rivers, swamps, marshes, floodplains, anthropogenic aquatic systems, coastal areas. A total of 56 resting, feeding or migrating individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Motacilla flava (Yellow Codobat) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species The species nests in marshes, floodplains, meadows, grasslands. A total of 233 individuals in transit, migration or resting were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Oenanthe isabellina (Eastern Pietrar) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Oenanthe oenanthe (South Pietrar) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species Foraging on agricultural land, meadows, pastures, cliffs, buildings. During the monitoring campaigns a number of 15 individuals in transit were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Saxicola torquata (Black bramble) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species Grazing on meadows, farmland, scrubland No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Streptopelia turtur (Turtur) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species It forages in orchards, parks, deciduous forests, woodlands. During the monitoring campaigns a number of 5 individuals in transit were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Sturnus roseus (alternative name Pastor roseus) (Grasshopper) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Turdus iliacus (Vine Turd) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species It nests on the ground, in dense vegetation or at low altitude on a bush or tree. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Upupa epops (Kiss) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | 4734.21 | - | Unknown | Maintain or improve | Highly mobile species It nests in agricultural land, meadows, pastures, cliffs, scrublands, orchards, parks. During the monitoring campaigns, a total of 51 individuals were identified as being in transit or at rest | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Species associated with forest habitats |  |  |  |  |  |  |  |  |  |  |  |
|  | Carduelis chloris (Florinte) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species During the monitoring campaigns, a total of 46 resting or transient individuals were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Carduelis spinus | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Coccothraustes coccothraustes ( Botgros) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disruption of species activity and removal Habitat loss | Stable |
|  | Falco subbuteo (Sparrow hawk) | No information available on the location of the species | 22 | 22 | Unknown | To be defined within 2 years | - | Unknown | Maintenance | Highly mobile species The species nests in meadows, grassland, steppe, scrubland, poles, buildings During the monitoring campaigns a number of 2 resting individuals were identified | Reduction in population numbers - risk of mortality Disruption of species activity and removal Habitat loss | Stable |
|  | Ficedula hypoleuca (Black Muscar) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It forages in orchards, parks, deciduous forests, woodlands. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Fringilla coelebs (Finch) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in swamps, floodplains, orchards, parks, deciduous forests, heaths. During the monitoring campaigns a number of 4 individuals were identified at rest | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Jynx torquilla (Capintors) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Muscicapa striata (Muscar sur) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in rivers, scrublands, orchards, parks, deciduous forests, woodlands. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Oriolus oriolus (Grangur) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in orchards, parks, deciduous forests, woodlands. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Phylloscopus collybita (Lesser Collybita ) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in deciduous, mixed and coniferous forests, as well as in man-made habitats such as parks, orchards, old cemeteries and gardens. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Phylloscopus sibilatrix (Sizzling Wood-Dove) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in dense, mature forests with tall trees and abundant foliage, but no undergrowth, in lowland or foothill areas. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Phylloscopus trochilus (Whistling Wood-Dove) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in areas with abundant scrub (including alpine with juniper) and scrubby transition zones at the edge of forest habitats, or rarer forests with abundant scrub. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Pyrrhula pyrrhula (Mugurar) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species prefers deciduous, mixed and coniferous forests on the montane floor, where there are clearings or where there is a rich understory. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Regulus ignicapillus (Spur-browed Auk) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species prefers coniferous to mixed woodland, especially in spruce and fir forests. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Regulus regulus (Yellow-headed Auk) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in coniferous and deciduous forests, especially in areas where it finds pine trees, but in winter it also nests in parks and gardens. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Serinus serinus (Caper) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests from April to July in trees of different species, preferring conifers. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Sturnus vulgaris (Graur) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in various cavities, such as hollows, crevices in rocks, and in these cavities it will use any enclosed shelter it can find (cracks in walls, under roofs, in chimneys, etc.). A total of 992 resting or transient individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Sylvia atricapilla (Black-headed Sylvia) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in scrublands, orchards, parks, deciduous forests. During the monitoring campaigns a number of 3 resting individuals were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Sylvia curruca (Little Sylvia) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in meadows, grasslands, steppe, scrublands, deciduous forests. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Turdus merula (Blackbird) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in a wide range of habitats, occurring in most types of woodland, heaths, orchards, shrub lines, gardens and parks. During the monitoring campaigns, a total of 18 individuals at rest were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Turdus philomelos (Singing Sturz) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species prefers regenerating forests, but also tree-rich anthropogenic habitats such as gardens, parks and cemeteries During the monitoring campaigns a number of 7 individuals were identified in the repasu | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Turdus pilaris (Cock Cock) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in orchards, parks, woodlands. A total of 38 individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Turdus viscivorus (Mistle Thrush) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests on agricultural land, coniferous forests, deciduous forests No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Species associated with reed habitats |  |  |  |  |  |  |  |  |  |  |  |
|  | Acrocephalus arundinaceus (Great Grebe) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in reed vegetation near water bodies. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Acrocephalus palustris (Marsh wagtail) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species Foraging in grassy vegetation near water bodies, weeds in agricultural crops, during harvest or in drier, weedy stubble No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Acrocephalus schoenobaenus (Little Grebe) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in a variety of low, dense vegetation, often close to water or in damp depressions, including vegetation at the edges of pools, streams and canals, lowland marshes with mats, reedbeds, but avoiding reedbeds in standing water. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Acrocephalus scirpaceus (Reed Reed Reed) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It forages in areas of massive, compact, large expanses of reedbeds, most often associated with lowland aquatic habitats. It also forages on adjacent habitats (including agricultural land). No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Locustella fluviatilis (Red-crested Cricket) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It nests in dense vegetation, on nettle meadows (Urtica dioica) and tall vegetation, scrublands, riparian woodlands, heaths, brambles, anise and willow thickets, wet glades, brambles, oak swamps, and on bare agricultural land No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Locustella luscinioides (Reed Cricket) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in lakes, marshes, floodplains, meadows, pastures. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Riparia riparia (Mal's grass) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It nests in lakes, rivers, marshes, floodplains, meadows, pastures, grasslands, and banks. During the monitoring campaigns only one individual was identified at rest | Declining herds - mortality risk | Stable |
|  | Species associated with cliff habitats and urban areas |  |  |  |  |  |  |  |  |  |  |  |
|  | Monticola saxatilis (Stone blackbird) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species prefers hilly, sparsely vegetated areas No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Hirundo daurica (Red-crowned Swallowtail) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in open habitats where cliffs, gorges, gorges, steep valleys or quarries are present, but also in anthropogenic habitats No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Hirundo rustica (Swallow) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests on agricultural land, meadows, pastures, orchards, parks, poles, buildings. During the monitoring campaigns a total of 219 individuals were identified as transiting or feeding | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss | Stable |
| ROSPA0094 | Accipiter brevipes (Short-legged Eurasian Short-legged) | South-eastern part of the area | 30 | 30 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species nests in tree branches at heights of over 4 m, preferring deciduous stands in the middle of meadows, plains and cultivated areas. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Alcedo atthis (Blue Gull) | Eastern part of the site | 4 | 4 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species Their preferred nesting habitats are streams, small rivers and canals with steep, sandy banks During monitoring campaigns the species was not identified | Declining herds - mortality risk | Stable |
|  | Anthus campestris (Field martin) | Western part of the site | 150-200 | 150-200 | Unknown | 225.8 | - | Favorable | Maintenance | Highly mobile species Brooding in open areas in the lowland region on the ground. A total of 10 nesting or transient individuals were identified | Declining herds - mortality risk | Stable |
|  | Aquila heliaca (Field Eagle) | Whole natural area | 2-4 | 2-4 | Unknown | 313.65 | - | Favorable | Maintenance | Highly mobile species The species nests in forests or tree stands in steppe or open plains, but also in hilly areas, often near water. They hunt in open, often cultivated areas No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Aquila pomarina (Lesser Spotted Eagle) | Central part of the area | 2-4 | 2-4 | Unknown | 313.65 | - | Favorable | Maintenance | Highly mobile species The species nests in forests near grasslands, wet meadows and agricultural areas large enough to provide food. A total of 15 migrating or hunting individuals were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Burhinus oedicnemus (Common Oedocnemus ) | Western part of the site | 30-80 | 30-80 | Unknown | 225.8 | - | Favorable | Maintenance | Highly mobile species It prefers open steppe, grassland and agricultural crops. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Buteo rufinus (Great Bute) | Whole natural area | 4-8 | 4-8 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species It grows in arid and semi-desert areas, but also in mountainous areas A total of 14 hunting or migrating individuals were identified | Declining herds - mortality risk | Stable |
|  | Caprimulgus europaeus (Caprimulg) | Eastern and central part of the site | 20-40 | 20-40 | Unknown | 313.65 | - | Favorable | Maintenance | Highly mobile species The species nests on the ground, in hollows in meadows or in the shelter of trees or bushes. During the monitoring campaigns an individual in transit was identified | Declining herds - mortality risk | Stable |
|  | Calandrella brachydactyla (Flock of Common Woodpeckers) | Western and eastern part of the site | 60-80 | 60-80 | Unknown | 225.8 | - | Favorable | Maintenance | Highly mobile species They prefer open areas with scattered low vegetation, cultivated fields or roadsides | Declining herds - mortality risk | Stable |
|  | Ciconia ciconia (White Stork) | Whole natural area | 1300 | 1300 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species The species nests in cliffs, orchards, parks, coniferous forests, deciduous forests, poles, buildings. A total of 93 migrating individuals were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Circaetus gallicus (Snake) | Central and western part of the site | 2-4 | 2-4 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species prefers forests of different species alternating with open sunny, cultivated or uncultivated, open and sunny, cultivated or uncultivated, grassland and scrubland. One individual of the species was identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Circus aeruginosus (Reed cattail) | Whole natural area | 20-50 | 20-50 | Unknown | 330.378 | - | Favorable | Maintenance | Highly mobile species They prefer areas at the level of lakes, rivers, marshes and floodplains. A total of 37 resting or migrating individuals were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Circus cyaneus (Black-headed Erytra) | Whole natural area | 80-100 | 80-100 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species does not nest in Romania. A total of 7 individuals were identified as being in transit | Declining herds - mortality risk | Stable |
|  | Circus macrourus (White Erete) | Whole natural area | 30-40 | 30-40 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species prefers marshes near rivers, floodplains, farmland, meadows, pastures, grasslands, steppe, scrublands No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Circus pygargus (Erete sur) | Whole natural area | 50-300 | 50-300 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species nests in open areas with low natural vegetation, isolated scrublands, areas of meadows and grasslands, farmland, scrublands, mires, peat bogs or other marshy areas During the monitoring campaigns a number of 8 individuals in transit were identified | Declining herds - mortality risk | Stable |
|  | Coracias garrulus (Dusky) | Whole natural area | 2-40 | 2-40 | Unknown | 313.65 | - | Favorable | Maintenance | Highly mobile species The species nests in farmland, meadows, pastures, grasslands, banks, scrub, buildings. During the monitoring campaigns a total of 26 individuals were identified on the ground, in bushes or in transit | Declining herds - mortality risk | Stable |
|  | Falco cherrug (Saker falcon) | Whole natural area | 3-4 | 3-4 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species It prefers open, arid steppe areas with forest and grassland No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Falco columbarius (Wintering Vulture) | Whole natural area | 1-2 | 1-2 | Unknown | 330.37 | - | Favorable | Maintenance | Highly mobile species The species prefers rocky areas or hollows in the rock During the monitoring campaigns an individual in transit was identified | Declining herds - mortality risk | Stable |
|  | Falco peregrinus (Peregrine Falcon) | Whole natural area | 1-2 | 1-2 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species They prefer rocky areas with abundant vegetation No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Falco vespertinus (Evening Goshawk) | Whole natural area | 200-300 | 200-300 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species nests in marshes, floodplains, floodplains, farmland, meadows, pastures, scrublands, orchards, parks, heaths, heaths, poles, buildings. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Hieraaetus pennatus (Little Eagle) | Whole natural area | 15-20 | 15-20 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species They prefer nesting areas with reeds such as Typha sp., reeds, Phragmites sp., or any other dense aquatic vegetation. During the monitoring campaigns a number of two individuals were identified hunting | Declining herds - mortality risk | Stable |
|  | Lanius collurio (Red-breasted Shrike) | Whole natural area | 60-100 | 60-100 | Unknown | 225.82 | - | Favorable | Maintenance | Highly mobile species The species usually nests in farmland, meadows, pastures, scrublands, orchards, parks. During the monitoring campaigns, a total of 57 individuals were identified as transiting or feeding | Declining herds - mortality risk | Stable |
|  | Lanius minor (Black-fronted Shrike) | Whole natural area | 40-80 | 40-80 | Unknown | 225.82 | - | Favorable | Maintenance | Highly mobile species The species usually nests in farmland, meadows, pastures, scrublands, orchards, parks. During the monitoring campaigns a number of 3 individuals were identified as transiting or feeding | Declining herds - mortality risk | Stable |
|  | Larus melanocephalus (Black-headed Gull) | Western and eastern extremity of the site | 5500-6000 | 5500-6000 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species They prefer natural, vegetated, stagnant pools rich in aquatic invertebrates and small fish. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Larus minutus (Little Gull) | Western and eastern extremity of the site | 100-200 | 100-200 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species The species prefers bare or sparsely vegetated land, riverine and lacustrine, coastal, shelf and open ocean ecosystems. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Lullula arborea (Woodlark) | Whole natural area | 60-240 | 60-240 | Unknown | 259.28 | - | Favorable | Maintenance | Highly mobile species Characteristic of open areas in deciduous or coniferous forests. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Melanocorypha calandra (Bărăgan's Woodpecker) | Western and eastern extremity of the site | 80-120 | 80-120 | Unknown | 225.82 | - | Favorable | Maintenance | Highly mobile species The species nests at ground level or in bushes A total of 55 individuals in transit were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Milvus migrans (Black Jay) | Whole natural area | 5-10 | 5-10 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species nests in rock hollows and tall trees. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Nycticorax nycticorax (Night heron) | Western and eastern extremity of the site | 10 | 10 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species The species nests in lakes, rivers, swamps, marshes or floodplains. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Oenanthe pleschanka (Black Petrel) | Southeast end of the site | 8-12 | 8-12 | Unknown | 225.8 | - | Favorable | Maintenance | Highly mobile species The species nests in stony and arid areas with sparse vegetation, in areas with abundant cliffs (gorges, rock walls, cliffs), abandoned quarries, eroded ravines, but also on steep mountain slopes and high plateaus with cliffs and meadows. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Pelecanus crispus (Crested Pelican) | Western and eastern extremity of the site | To be defined within 2 years | Unknown | Unknown | 225.8 | - | Favorable | Maintenance | Highly mobile species They prefer nesting habitats similar to those occupied by the common pelican, rivers, lakes, lagoons, estuaries, usually nesting as small colonies on islands or in extensive reedbeds No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Pernis apivorus (Viespar) | Whole natural area | 2-6 | 2-6 | Unknown | 418.2 | - | Favorable | Maintenance | Highly mobile species The species often nests in deserted nests of the common crow (Corvus frugilegus ). During the monitoring campaigns a number of 2 migrating individuals were identified | Declining herds - mortality risk | Stable |
|  | Philomachus pugnax (Bully) | Western and eastern extremity of the site | 20-100 | 20-100 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species Prefer wetlands but sometimes also agricultural fields No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Porzana porzana porzana/Calidris pugnax (Spotted Crested) | Eastern end of the site | 100 | 100 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species The species nests in swampy wetlands with a lot of vegetation No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Recurvirostra avosetta (Ciocîntors) | Western and eastern extremity of the site | 10-20 | 10-20 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species They prefer brackish or brackish aquatic habitats with shallow standing water and little vegetation (with exposed mudflats). No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Tadorna ferruginea (Red-crested Shelduck) | Western and eastern extremity of the site | 8 | 8 | Unknown | 104.6 | - | Favorable | Maintenance | Highly mobile species The species prefers brackish or saltwater aquatic habitats in open areas, but also occurs in freshwater aquatic surfaces. No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Regularly migratory species Species associated with open/agricultural land |  |  |  |  |  |  |  |  |  |  |  |
|  | Alauda arvensis (Woodlark) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It nests in a variety of low, dense vegetation, often close to water or in damp depressions, including vegetation at the edges of pools, streams and canals, lowland marshes with mats, reedbeds, but avoiding reedbeds in standing water. A total of 560 resting or feeding individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disruption of species activity and removal Habitat loss | Stable |
|  | Buteo lagopus (Shoed Buteo) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It prefers open habitats with sparse or absent trees in the lowlands of tundra and taiga. No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Emberiza melanocephala (Black-headed barnacle) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Lanius senator (Red-headed Shrike) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Merops apiaster (Prigorie) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests on agricultural land, meadows, pastures, grasslands, banks, steppe. A total of 147 resting or feeding individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Miliaria calandra (Southern Mealy Borer) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Motacilla alba (White motacilla) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It inhabits lakes, rivers, swamps, marshes, floodplains, anthropogenic aquatic systems, coastal areas. A total of 56 resting, feeding or migrating individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disruption of species activity and removal | Stable |
|  | Oenanthe oenanthe (South Pietrar) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species Foraging on agricultural land, meadows, pastures, cliffs, buildings. During the monitoring campaigns a number of 15 individuals in transit were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Streptopelia turtur (Turtur) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It forages in orchards, parks, deciduous forests, woodlands. During the monitoring campaigns a number of 5 individuals in transit were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Upupa epops (Kiss) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It nests in agricultural land, meadows, pastures, cliffs, scrublands, orchards, parks. During the monitoring campaigns, a total of 51 individuals were identified as being in transit or at rest | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Falco tinnununculus (Red-rumped Vulture) | No information on the location of the species is available | 80 | 80 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in marshes, floodplains, floodplains, farmland, meadows, pastures, scrublands, orchards, parks, heaths, heaths, poles, buildings. During the monitoring campaigns, a total of 55 individuals in transit or hunting were identified | Declining herds - mortality risk | Stable |
|  | Anser albifrons (Great Tern) | No information available on the location of the species | 300-400 | 300-400 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species Prefer low bushes with low, marshy areas with open water holes No individuals of the species were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Galerida cristata (Woodlark) | No information available on the location of the species | 240-280 | 240-280 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species A total of 114 individuals were identified during the monitoring campaigns | Reduction in population numbers - risk of mortality Disruption of species activity and removal | Stable |
|  | Coturnix coturnix (Quail) | No information available on the location of the species | 600 | 600 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests on agricultural land, meadows, pastures, scrublands. During the monitoring campaigns a number of 4 individuals were identified | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Regularly migratory species Species associated with riparian and coastal habitats |  |  |  |  |  |  |  |  |  |  |  |
|  | Gallinago gallinago (Common Snipe) | No information on the location of the species is available | 44 | 44 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species They prefer a wide range of wetland habitats, from wet meadows to salt marshes. During the monitoring campaigns an individual was identified at rest | Declining herds - mortality risk | Stable |
|  | Hirundo rustica (Swallow) | No information on the location of the species is available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests on agricultural land, meadows, pastures, orchards, parks, poles, buildings. During the monitoring campaigns a total of 219 individuals were identified as transiting or feeding | Declining herds - mortality risk | Stable |
|  | Regularly migratory species Species associated with urban habitats |  |  |  |  |  |  |  |  |  |  |  |
|  | Hirundo rustica (Swallow) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests on agricultural land, meadows, pastures, orchards, parks, poles, buildings. During the monitoring campaigns a total of 219 individuals were identified as transiting or feeding | Declining herds - mortality risk | Stable |
|  | Regularly migratory species Species associated with mixed terrestrial habitats forests and open lands |  |  |  |  |  |  |  |  |  |  |  |
|  | Accipiter nisus (Bird's-billed Woodlark) | Information on the location of the species is not available | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species prefers coniferous and mixed forests, pine plantations, parks with large trees or isolated groups of trees During the monitoring campaigns a number of 5 individuals were identified as resting or migrating | Reduction in population numbers - risk of mortality Disturbance of species activity and removal Habitat loss/alteration | Stable |
|  | Buteo buteo (Common buzzard) | No information available on the location of the species | 200 | 200 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species Foraging on agricultural land, meadows, pastures, coniferous forests, deciduous forests, heaths During the monitoring campaigns a total of 162 individuals were identified as transiting, hunting or migrating | Declining herds - mortality risk | Stable |
|  | Falco subbuteo (Sparrow hawk) | No information available on the location of the species | 50 | 50 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in meadows, grassland, steppe, scrubland, poles, buildings During the monitoring campaigns a number of 2 resting individuals were identified | Declining herds - mortality risk | Stable |
|  | Regularly migratory species Species associated with reed habitats |  |  |  |  |  |  |  |  |  |  |  |
|  | Motacilla flava (Yellow Codobat) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in marshes, floodplains, meadows, grasslands. A total of 233 individuals in transit, migration or resting were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Regularly migratory species Species associated with open aquatic habitats |  |  |  |  |  |  |  |  |  |  |  |
|  | Anser albifrons (Great Tern) | Information on the location of the species is not available | 3400 | 3400 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species Prefer low bushes with low, marshy areas with open water holes No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Cygnus olor (Summer swan) | Information on the location of the species is not available | 32 | 32 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species prefers natural, extensive aquatic habitats, wetlands and lakes with reed surfaces No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Anser anser (Summer Goose) | Information on the location of the species is not available | 80 | 80 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species prefers low-lying, lowland areas rich in fall agricultural crops or areas with natural grass vegetation No individuals of the species were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Larus cachinnans ( Mediterranean Gull ) | Information on the location of the species is not available | 1800 | 1800 | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species It nests in the area of lakes surrounded by extensive reedbeds in steppe and semi-desert regions, on reservoirs, rivers and river islands with short vegetation of grass and scrubs A total of 88 individuals in transit, resting or migrating were identified during the monitoring campaigns | Declining herds - mortality risk | Stable |
|  | Regularly migratory species Species associated with forest habitats |  |  |  |  |  |  |  |  |  |  |  |
|  | Sylvia atricapilla (Black-headed Sylvia) | No information available on the location of the species | To be defined within 2 years | Unknown | Unknown | To be defined within 2 years | - | Unknown | Maintain or improve | Highly mobile species The species nests in scrublands, orchards, parks, deciduous forests. During the monitoring campaigns a number of 3 resting individuals were identified | Declining herds - mortality risk | Stable |
| BG0000569 Kardam | 6110* White Alysso-Sedion calcareous or basifyllic rocky grasslands | Central area of the area along the watercourse | - | - | - | - | Unknown | Unknown | Unknown | - | Habitat alteration Spread of invasive plant species in habitat | Uncertain |
|  | 6210* - Semi-natural xerophytic and facies xerophytic grasslands with bushes on calcareous substrata (Festuco-Brometalia) | In the Romanian border area | - | - | - | - | Unknown | Unknown | Unknown | - | Habitat alteration Spread of invasive plant species in habitat | Uncertain |
|  | Vertigo moulinsiana (Coiled Snail) | Wetland, central to the site | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Vertigo angustior | Wetland, central to the site | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Unio crassus (Little river clam) | Wetland, central to the site | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Osmoderma eremita (Hermit Beetle) | Wooded area on the Romanian border | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Morimus funereus (Grey tailor) | Wooded area on the Romanian border | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Lycaena dispar (Red Swamp Butterfly) | Wooded area on the Romanian border and Kardam area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Lucanus cervus (Red) | Wooded area on the Romanian border and Kardam area | - | - | - | - | Unknown | Unknown | Unknown | Medium-mobile species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Hypodryas maturna (Rare Fritillary) | Area of the whole natural area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Euplagia quadripunctaria (Striped butterfly) | In the Romanian border area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Dioszeghyana schmidtii | Area of the whole natural area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Cucujus cinnaberinus | Wooded area on the Romanian border | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Cerambyx cerdo (Great oak tailor) | Wooded area on the Romanian border | - | - | - | - | Unknown | Unknown | Unknown | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Reduced | Uncertain |
|  | Rhodeus sericeus amarus (Barnacle) | The area of the river crossing the area, in the Kardam area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Cobitis taenia (Cobitis taenia ) | The whole area of the river running through the area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Barbus meridionalis (Black-eared Murre) | The area of the river crossing the area, in the Kardam area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Sabanejewia aurata (Dunari) | The area of the river crossing the area, in the Kardam area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Elaphe sauromates (Dobrogean dragon) | Area of the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Habitat-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Stable |
|  | Bombina variegata (Yellow-bellied Pond Owl) | Wetlands throughout the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Wetland-dependent species No individuals of the species were identified during the monitoring campaigns | Very low | Stable |
|  | Emys orbicularis (European pond turtle) | Wetlands throughout the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Mesocricetus newtoni (Romanian Hamster) | Agricultural areas at site level | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Mustela eversmanii (Steppe fer ferret) | Areas of scrubland on the outskirts of the localities within the site | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species It prefers arid and barren plains One individual of the species was identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Spermophilus citellus (Gopher) | Grassland areas at site level | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species A total of 39 individuals were identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Triturus dobrogicus (Dobrogic crested newt) | Ograzhden wetland | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility One individual of the species was identified during the monitoring campaigns | Very low | Uncertain |
|  | Triturus karelinii (Crested newt) | Wetlands throughout the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Testudo graeca (Dobrogea land tortoise) | Wetlands throughout the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility During the monitoring campaigns a number of 6 individuals were identified | Very low | Uncertain |
|  | Testudo hermanni ( Land tortoise) | Wetlands throughout the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility One individual of the species was identified during the monitoring campaigns | Very low | Uncertain |
|  | Lynx lynx (Laughs) | The wooded and highly-vegetated areas of the site, especially in the area of Ograzhden, Kardam, Snyagovo | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species One individual of the species was identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Uncertain |
|  | Lutra lutra (Otter) | A small wetland near Kardam | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species One individual of the species was identified during the monitoring campaigns | Very low | Uncertain |
|  | Vormela peregusna (Spotted ferret) | Pasture areas around Ograzhden, Kardam, Snyagovo | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species One individual of the species was identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus mehelyi (Méhely's horseshoe bat) | Reduced forest areas in the center of the natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species They hunt in deciduous forests, steppe areas, grasslands, sometimes agricultural land. Although a sedentary species, it can migrate distances of up to 94-100 km During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus hipposideros (Little horseshoe bat) | The border area starting from the Romanian border and continuing in the direction of Ograzhden, Kardam, General Toshevo | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus ferrumequinum (Greater horseshoe bat) | The border area from the border with Romania continues in the direction of Ograzhden, Kardam | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus euryale (Mediterranean horseshoe bat) | The border area from the Romanian border to the village of Ograzhden | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus blasii (Blasius' horseshoe bat) | The border area from the Romanian border to the village of Ograzhden | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Miniopterus schreibersii (Long-winged Bat) | The border area stretching from the border with Romania towards Ograzhden and Kardam | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species The species forages in more or less open habitats, even in peri-urban areas, and the foraging flight is high (10-20 m). It prefers heavily wooded areas. It often feeds far from its roost, about 30 km. A total of 85 contacts were obtained during the monitoring period | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis myotis (Common bat) | Romanian border area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis emarginatus (Hollow-eared bat) | The border area stretching from the border with Romania towards Ograzhden and Kardam | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis capaccinii (Long-legged bat) | Border areas near the settlements of Ograzhden and Kardam | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis blythii (Little common bat) | Border areas near the settlements of Ograzhden and Kardam | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis bechsteinii (Big-eared bat) | Forest area on the Romanian border | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Barbastella barbastellus (Bar bar barbartelltellus) | Forest area on the Romanian border | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
| BG0000570 Izvorovo - Kraishte | 6110* White Alysso-Sedion calcareous or basifyllic rocky grasslands | Border area with Romania and the adjacent areas of Rosen, Krasen, Zhiten and Izvorovo | - | - | - | - | Unknown | Unknown | Unknown | - | Very low | Uncertain |
|  | 6210* - Semi-natural xerophytic and facies xerophytic grasslands with bushes on calcareous substrata (Festuco-Brometalia) | Border area with Romania and the adjacent areas of Rosen, Krasen, Zhiten and Izvorovo | - | - | - | - | Unknown | Unknown | Unknown | - | Very low | Uncertain |
|  | 62C0* Point-sarmatic stepping stones | Areas adjacent to Rosen and Zhiten | - | - | - | - | Unknown | Unknown | Unknown | - | Very low | Uncertain |
|  | Echium russicum (Snake's head) | Border with Romania, north of Kraishte | - | - | - | - | Unknown | Unknown | Unknown | - | Very low | Uncertain |
|  | Potentilla emilii-popii (Five-toed weed) | Romanian border area | - | - | - | - | Unknown | Unknown | Unknown | - | Very low | Uncertain |
|  | Vertigo moulinsiana (Coiled Snail) | Wetlands in the river crossing the area | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Vertigo angustior | Wetlands in the river crossing the area | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Unio crassus (Little river clam) | Wetlands in the river crossing the area | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Pseudophilotes bavius | Grassland in the natural area | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Paracaloptenus caloptenoides (Devil Horse) | Grassland in the vicinity of Kraishte and Rosen | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Osmoderma eremita (Hermit beetle) | Border area with Romania and small parts bordering forest areas | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Ophiogomphus cecilia (Green Gomphida) | Wetlands throughout the whole natural area | - | - | - | - | Unknown | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Morimus funereus (Grey tailor) | Wooded area of the entire natural area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Lycaena dispar (Red Swamp Butterfly) | Wooded area of the entire natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Very low | Stable |
|  | Lucanus cervus (Red) | Wooded area of the entire natural area | - | - | - | - | Unknown | Unknown | Unknown | Medium-mobile species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Hypodryas maturna (Rare Fritillary) | Area of the whole natural area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Euplagia quadripunctaria (Striped butterfly) | Area-wide wetlands and woodlands | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Dioszeghyana schmidtii | Wooded area of the entire natural area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Cucujus cinnaberinus | Wooded area of the entire natural area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Coenagrion ornatum | The area of the river crossing the area | - | - | - | - | Unknown | Unknown | Unknown | Species with reduced mobility During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Cerambyx cerdo (Great oak tailor) | Wooded area on the Romanian border | - | - | - | - | Unknown | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Reduced | Uncertain |
|  | Rhodeus sericeus amarus (Boarță) | The area of the river crossing the area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Cobitis taenia (Cobitis taenia ) | The whole area of the river running through the area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Barbus meridionalis (Black-eared Murre) | The whole area of the river running through the area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Sabanejewia aurata (Dunari) | The whole area of the river running through the area | - | - | - | - | Unknown | Unknown | Unknown | Aquatic habitat-dependent species During monitoring campaigns the species was not identified | Very low | Uncertain |
|  | Canis lupus (Wolf) | The wooded areas around Rosen and Krasen | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration | Stable |
|  | Bombina variegata (Yellow-bellied Pond Owl) | Wetlands throughout the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Wetland-dependent species During monitoring campaigns the species was not identified | Very low | Stable |
|  | Elaphe sauromates (Dobrogean dragon) | Whole site area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Habitat-dependent species During monitoring campaigns the species was not identified | Very low | Stable |
|  | Emys orbicularis (European pond turtle) | Grassland areas of the site | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Mesocricetus newtoni (Romanian Hamster) | Grassland areas of the site | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species No individuals of the species were identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Mustela eversmanii (Steppe fer ferret) | Agricultural areas in the vicinity of Rosen and Krasen | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species It prefers arid and barren plains One individual of the species was identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Lynx lynx (Laughs) | Wooded and highly vegetated areas on the site | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species One individual of the species was identified during the monitoring campaigns | Habitat loss/alteration | Uncertain |
|  | Lutra lutra (Otter) | Wetlands around the watercourses that cross the site | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species One individual of the species was identified during the monitoring campaigns | Very low | Uncertain |
|  | Spermophilus citellus (Gopher) | Grassland in the natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species A total of 39 individuals were identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Vormela peregusna (Spotted ferret) | Grassland in the natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species One individual of the species was identified during the monitoring campaigns | Habitat loss/alteration Reduced populations | Stable |
|  | Testudo graeca (Dobrogea land tortoise) | Whole site area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility During the monitoring campaigns a number of 6 individuals were identified | Very low | Uncertain |
|  | Testudo hermanni ( Land tortoise) | Grassland areas of the site | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility One individual of the species was identified during the monitoring campaigns | Very low | Uncertain |
|  | Triturus dobrogicus (Dobrogic crested newt) | The lake near the Romanian border | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility One individual of the species was identified during the monitoring campaigns | Very low | Uncertain |
|  | Triturus karelinii (Crested newt) | Wetlands throughout the whole natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Species with reduced mobility No individuals of the species were identified during the monitoring campaigns | Very low | Uncertain |
|  | Rhinolophus mehelyi (Méhely's horseshoe bat) | Forested and highly vegetated areas throughout the whole area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species They hunt in deciduous forests, steppe areas, grasslands, sometimes agricultural land. Although a sedentary species, it can migrate distances of up to 94-100 km During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus hipposideros (Little horseshoe bat) | Border and wetland areas of the whole area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus ferrumequinum (Greater horseshoe bat) | Border and wetland areas of the whole area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus euryale (Mediterranean horseshoe bat) | Border and wetland areas of the whole area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Rhinolophus blasii (Blasius' horseshoe bat) | Wooded area of the entire natural area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Miniopterus schreibersii (Long-winged Bat) | Border and wetland areas of the whole area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species The species forages in more or less open habitats, even in peri-urban areas, and the foraging flight is high (10-20 m). It prefers heavily wooded areas. It often feeds far from its roost, about 30 km. A total of 85 contacts were obtained during the monitoring period | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis myotis (Common bat) | Border and wetland areas of the whole area | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis emarginatus (Hollow-eared bat) | Wetlands at site level | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis capaccinii (Long-legged bat) | Wetlands at site level | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis blythii (Little common bat) | Wetlands at site level | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Myotis bechsteinii (Big-eared bat) | Locally, the wetland of the river running through the area in the immediate vicinity of Rosen | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |
|  | Barbastella barbastellus (Bar bar barbartelltellus) | Forest area on the Romanian border | Unknown | Unknown | Unknown | Unknown | - | Unknown | Unknown | Highly mobile species During monitoring campaigns the species was not identified | Habitat loss/alteration Reduced populations | Stable |

## Structural and functional relationships that create and maintain the integrity of ANPIC

A plan may affect the integrity of a Natura 2000 site if it has a negative impact on the factors determining the maintenance of the favorable conservation status of the protected natural area of Community interest or if it alters the dynamics of the relationships that define the structure and/or function of the protected natural area of Community interest.

Factors that can affect the integrity of a natural area, independently or in combination with other plans/projects include:

- significantly reducing the area of one or more habitat types of Community interest within the Natura 2000 site;

- significant reduction in the area of habitats and/or the number of specimens of species of Community interest;

- significant fragmentation of habitats of community interest;

- significant fragmentation of ecologically suitable habitats of species of Community interest;

- the occurrence of a significant negative impact on the factors determining the maintenance of the favorable conservation status of the protected natural area;

- changes in the dynamics of the relationships defining the structure and/or function of the protected natural area.

The structural and functional relationships that create and maintain the integrity of the area are related to feeding, shelter and reproduction conditions on the one hand, and on the other hand to anthropogenic pressure and all the external factors that can affect the biodiversity of the area under analysis.

Invertebrates play an essential role in the functioning of ecosystems due on the one hand to their feeding regime - primary, secondary and decomposers consumers, and on the other hand due to their functional ecological plurivalence - pollinator species, they are also a food source for other invertebrate and vertebrate species: amphibians, birds and mammals).

Amphibians and reptiles have the dual quality in food webs of being both prey and predator.

Where amphibian populations are abundant, they can consume significant quantities of prey organisms, serving to limit population explosions. As prey, herpetofauna are an important trophic resource for other taxonomic groups.

The importance of bird species for the optimal functioning of natural ecosystems is extremely varied. Some birds are considered keystone species because their presence in or disappearance from an ecosystem directly affects other species in the food chain.

The main ecological functions provided by birds are:

- Regulating services: seed dispersal (for frugivorous species), pest control (invertebrate and vertebrate-feeding bird species), carcass removal (scavenging species);

- Supporting services: nutrient deposition (aquatic species), ecosystem "shaping" services (burrowing species).

Bird species are highly mobile and not strictly habitat-dependent, they are not so strongly affected by anthropogenic activities and can retreat from the disturbed area to the undisturbed areas of the characteristic habitat. A prerequisite is that the characteristic (favorable) habitat must not be destroyed and anthropogenic works must not be carried out during vulnerable life-history stages (breeding, nesting, brood rearing).

Depending on the ecological and/or trophic niche they occupy within an ecosystem, mammals play important roles in its functioning:

- Micromammals - contribute to the diversity of life both as predators and prey. Through this interaction with other animal groups, mammals influence food webs and control the population levels of predators, insects and host species for parasites.

- Macromammals - facilitate the flow of nutrients by connecting adjacent ecosystems and occupy a unique place in food webs that cannot be occupied by other animals, through direct seed dispersal or consumption by seed-dispersing animals. Like other predator species, medium-sized mammals also control the population levels of prey species - small mammals, reptiles, amphibians and birds.

Natura 2000 sites are a Europe-wide network of protected areas aimed at protecting biodiversity and maintaining species and habitats of Community interest. These sites are managed on the basis of structural and functional relationships that ensure their ecological integrity. These structural and functional relationships can be:

Structural relations

Green infrastructure

Waterways

Migration routes

Functional relationships

Regulating ecosystem services

Biological cycles and energy flow

Protection and recovery of species of Community interest - conservation actions

Sustainable management - economic activities on or near sites should be managed in a way that does not adversely affect biodiversity

### Green infrastructure

Green infrastructure is a strategically planned network of natural and semi-natural areas with other environmental characteristics, designed and managed to provide a wide range of ecosystem services .

Within natural and semi-natural ecosystems, there are established intra- and interspecific connections that mediate material, energy and information exchanges that ensure their productivity, adaptability and resilience.

Natural and semi-natural ecosystems account for about 47% of the country's land area, 45% is agricultural ecosystems and the remaining 8% is buildings and infrastructure.

Semi-natural habitats arise from the interaction between human activities and the natural environment. These semi-natural habitats may occur as a result of traditional agricultural activities (grazing, mowing), forest management (traditional forestry, broadleaved forests and wooded meadows), water management (wetlands: marshes, artificial lakes or irrigation systems), traditional landscape conservation, land clearing and abandonment (natural reforestation), creation of buffer or protection zones.

The characteristics of semi-natural habitats are: high biodiversity, need for continuous management, cultural and ecological value.

Three types of high nature value agricultural land have been identified at European level:

- land characterized by large areas of semi-natural vegetation (low human intervention),

- land characterized by mosaic-type landscapes (hedgerows, rows of trees, etc.).

- land of low natural value but important ecological corridors for the maintenance of rare habitats and species, important nesting areas for certain rare bird species or migratory birds (cereal crops) .

In Romania, land with high nature value can be classified according to the criteria proposed by the European Forum for Nature Conservation and Pastoralism into natural and semi-natural meadows in the mountainous area; Extensive traditional orchards (the old meadows are almost entirely preserved); mosaic landscapes (meadows, trees, shrubs and agricultural plots with abundant biodiversity); meadows close to forests characterized by a high diversity of fauna (birds, invertebrates, mammals, etc.)

In the area where the PUZ is located, it was found that the largest share is represented by agricultural areas, occupying about 88% of the total. A smaller share is represented by meadows (about 4.5%) and settlements (about 5%).

Figure 2-9  Green infrastructure in the site area

### Waterways

From the hydrographical point of view, the studied site is located in the Danube-Littoral Watershed. The nearest water bodies identified in the area studied by the PUZ are located at distances greater than 33 km (Marea Neagra-Eforie Nord-Vama Veche - code ROCT02_B2, at a distance of about 33.75 km, east of the site boundary, the Danube-Black Sea Canal - Poarta Alba- Midia Canal - RORW15-1-10B_B2, at a distance of about 34.99 km, north of the site boundary).

Watercourses are also considered ecological corridors. The following watercourses have been identified in the project area:

No surface water bodies have been delimited on these watercourses. According to the Management Plan of the Dan Danube Littoral River Basin, at the level of the river basin, no interdependencies have been identified between the watercourses in the area analyzed by the PUZ and the natural habitats in Natura 2000 sites. The site is crossed by non-permanent watercourses.

In the area, in the vicinity of wind power plants T1, T4, T12, T26, T27 and T28 there are irrigation canals, which are not operational.

Figure 2-10  Watercourses in the area of the site

### Migration routes

Analyzing the distance and the species/habitats for which these areas have been designated, it has been identified that a number of 5 natural protected areas on the territory of Romania can be potentially affected directly/indirectly by the PUZ.

According to data from the literature, the site of the PUZ is located on the eastern albicard - Danube migration route .  According to this study, the highest number of migratory species was reported to the north, north-east of the analyzed Park site (Figure 2-11).

Figure 2-11  Watercourses in the area of the site

At the end of 2012 the National Institute for Research and Development of the Danube Delta (INCDDD) finalized, under a contract funded by the Ministry of Environment and Forests, the "Study on recommendations on areas in Dobrogea where the siting of wind power plants should be restricted due to the migration corridors of birds of gliding flight (day raptors, storks, pelicans) respectively due to wintering geese and swans". The following areas, spatially localized in Figure 2-12, were proposed for the study:

- Areas of exclusion of wind power plants and related infrastructure, where it is recommended to prohibit their construction: Natura 2000 sites (SPAs and SCIs), the Danube Delta Biosphere Reserve, national and nature parks and nature reserves/ bird sanctuaries on the territory of continental Dobrogea were included in these areas. The surface area of the proposed exclusion zones represents 50.4% of the region.

- High-risk areas: in these areas a number of restrictions on the siting and operation of wind power plants are needed to minimize the risk of collision (e.g. monitoring using radar technology, embedded microphones to monitor collision rates, etc.), and long-term monitoring (minimum 10 years) of soaring bird species and/or wintering populations of geese and swans is required. The area of the proposed high risk areas represents 36.4% of Dobrogea;

- Moderate risk areas: in these areas, long-term monitoring of soaring bird species and/or wintering populations of geese and swans is required. The surface area of the proposed moderate risk areas represents 13.2% of Dobrogea.

These areas only take into account important areas for bird species and do not include other types of areas with construction restrictions for wind farms (e.g. important areas for bat species, settlements, public road network, nesting information for bird species).

The conclusions of the "Study on the recommendations for areas in Dobrogea where the siting of wind power plants should be restricted due to the migration corridors of gliding birds (day raptors, storks, pelicans), respectively due to the wintering of geese and swans", conducted by the National Institute for Research and Development of the Danube Delta in 2012, show that the perimeter studied by the PUZ is located in an area with moderate risk in terms of conservation of bird species and natural habitats. Moderate risk areas. The moderate risk areas total approximately 206100 ha which represents 13.2% of Dobrogea. In moderate-risk areas, long-term monitoring of soaring bird species and/or wintering populations of geese and swans is necessary to establish conditions for the construction or operation of wind power plants.

Figure 2-12  Map showing areas where construction of wind turbines may be allowed, but their construction or operation will have to be restricted due to the migration of soaring birds and/or wintering populations of geese and swans

The perimeter of the PUZ is located in the vicinity of the distribution areas of the Red-throated Goose (Branta ruficollis). In Romania, the Red-throated Goose population is found in the lowlands of Bărăgan and Dobrogea, near large lakes, which it uses for wintering. In milder winters, several thousand geese remain to winter in Ukraine and even in southern Russia. When it's cold, the geese quickly move south to the Dobrogea region of Romania and Bulgaria. At present, the most important areas for the species remain the sites in south-eastern Romania, mainly the regions of Bărăgan, the Danube Delta and Dobrogea (Figure 2-13), which have several declared protected areas for the species: Balta Albă - Amara - Jirlău, Ianca - Plopu - Sărat, Strachina Lake, Călărași estuary, Gura Ialomiței - Berteștii de Sus, Balta Mică Natural Park of Brăilei, Oltina Lake, Danube Delta and Razim-Sinoe Lagoon Complex.

The latest estimate of the national red-breasted goose population for the period 2013-2022 is about 9,915-23,641 individuals wintering in Romania, representing about 42% of the global population. The following figure shows the distribution of the red-breasted goose (Branta ruficollis) species in Romania, in the period 2000-2022 (EEA grid, 10x10 km, in ETRS89 projection system).The following tables present the structural and functional relationships and the analysis of the relationships between habitats/species and ecosystems at the analyzed sites.

Figure 2-13  Red-breasted goose (Branta ruficollis) species distribution map in Romania, 2000-2022

There are currently no data in Romania on red-necked geese colliding with power lines and wind turbines. Research in Western and Northern Europe shows that wind turbines have a negative impact on goose populations. The main recorded effect is the abandonment of foraging areas leading to the effective loss of key habitats (Larsen & Madsen, 2000; Larsen & Clausen, 2002, Madsen & Boertmann, 2008 and etc.).

From the analysis of the map showing the correlation between the important areas for the red-throated goose (Branta ruficollis) in Dobrogea and the wind farms, it can be observed that the perimeter of the analyzed energy park is located in an area of low importance (low sensitivity).

Figure 2-14  Correlation between important areas for red-throated goose (Branta ruficollis) in Dobrogea and wind farms. Authorized (black) and operational (green) wind turbines in Dobrogea and Bărăgan approved in Romania, in the period 2000-2022

Movements of the species in wintering areas, which were carried out within the framework of the National Action Plan for the conservation and population management of Branta ruficollis.

Figure 2-15  Species movements in wintering areas, based on satellite transmitter data in the south-east of the country, 2011-2020

The following table presents the migration routes of the bird species identified in the area of the PUZ according to the information available on the Bird Migration Map and the results of the projects results of the projects, which are studying birds with the help of satellite transmitters in central and south-eastern Europe .

Table  2-4  Migration routes for the migratory bird species analyzed in this study

| No. crt. | Species | Species | Ind. no. Signs in the study area for PUZ | Migration route | Data source |
| --- | --- | --- | --- | --- | --- |
| 1 | Accipiter gentilis | Uliu porumbar | 2 | - | - |
| 2 | Accipiter nisus | Uliu bird | 5 | Northern Europe - South Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 3 | Alauda arvensis | Field woodlark | 550 | - | - |
| 4 | Anthus campestris | Fieldfare | 10 | - | - |
| 5 | Apus apus | Black Drepnea | 99 | Northern Europe - South Eastern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 6 | Pomarina eagle | Little screaming eagle | 19 | Northern Europe - Central Africa | Birdtrackingmap - https://bbecquet.github.io/birdtracking/ |
| 7 | Buteo buteo | Shared bus | 162 | Northern Europe - South Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 8 | Buteo rufinus | Largeorecar | 19 |  |  |
| 9 | Caprimulgus europaeus | Caprimulg | 1 | Northern Europe - South Eastern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 10 | Carduelis carduelis | Sticlete | 100 | - | - |
| 11 | Carduelis chloris | Florinte | 46 | - | - |
| 12 | Ciconia ciconia | White Stork | 93 | South East Europe - East Africa | Satellitetracking - https://satellitetracking.eu/inds/show map/?check_273=273 |
| 13 | Ciconia nigra | Black stork | 1 | East Africa - North Europe | Birdmap - https://birdmap.5dvision.ee/EN/202 3/autumn/?line=1&track=0&speed=1 |
| 14 | Circaetus gallicus | Saw | 1 | Northern Europe - South Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 15 | Circus aeruginosus | Reed reed | 37 | - | - |
| 16 | Circus cyaneus | Erete vânăt | 7 | - | - |
| 17 | Circus pygargus | Southern Erete | 8 | - | - |
| 18 | Columba palumbus | Collared pigeon | 2 | - | - |
| 19 | Coracias garrulus | Dumbrăveancă | 26 | South East Europe - Southern Africa | Satellitetracking - https://satellitetracking.eu/inds/show map/?check_273=273 |
| 20 | Corvus corax | Corbul | 1 | - | - |
| 21 | Corvus frugilegus | Seed crow | 28 | South East Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 22 | Coturnix coturnix | Quail | 4 | South-Central Europe Eastern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 23 | Cuculus canorus | Cuc | 6 | Northern Europe - South Eastern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 24 | Delichon urbica | Homemade locust | 50 | Northern Europe - South Africa | Migrationatlas - https://migrationatlas.org/node/1830 |
| 25 | Emberiza calandra | Depressure | 200 | - | - |
| 26 | Emberiza hortulana | Garden Presley | 3 | - | - |
| 27 | Erithacus rubecula | Măcăleandru | 3 | Northern Europe - South Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 28 | Falco columbarius | Winter Falcon | 2 | Northern Europe - South Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 29 | Subbuteo falcon | Sparrow hawk | 2 | Northern Europe - South Africa | Migrationatlas - https://migrationatlas.org/node/1830 |
| 30 | Falco tinnununculus | Red fannet | 55 | Northern Europe - North Africa | Migrationatlas - https://migrationatlas.org/node/1830 |
| 31 | Fringilla coelebs | Finches | 4 | - | - |
| 32 | Galerida cristata | Woodpecker | 26 | Southern Europe - South Eastern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 33 | Gallinago gallinago | Common grant | 1 | - | - |
| 34 | Garrulus glandarius | Skip | 4 | - | - |
| 35 | Hieraaetus pennatus | Little eagle | 2 | - | - |
| 36 | Hirundo rustica | Swallow | 219 | Northern Europe - South Africa | Migrationatlas - https://migrationatlas.org/node/1830 |
| 37 | Lanius collurio | Reddish red spurge | 58 | Northern Europe - South Africa | Migrationatlas - https://migrationatlas.org/node/1830 |
| 38 | Lanius minor | Black-headed spurge | 3 | - | - |
| 39 | Larus cachinnans | Pond Gull | 88 | - | - |
| 40 | Luscinia luscinia | The watcher of the wattle | 1 | Northern Europe - East Africa | Migrationatlas - https://migrationatlas.org/node/1830 |
| 41 | Luscinia megarhynchos | Redheaded nightwatcher | 4 | South East Europe - North Africa | Migrationatlas - https://migrationatlas.org/node/1830 |
| 42 | Melanocorypha calandra | Bărăgan woodlark | 55 | - | - |
| 43 | Merops apiaster | Prison | 147 | Center of Europe - Southern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 44 | White motacilla | White codobat | 56 | - | - |
| 45 | Motacilla flava | Yellow codobat | 233 | - | - |
| 46 | Oenanthe oenanthe | South Pietrar | 15 | - | - |
| 47 | Parus major | Big duck | 3 | - | - |
| 48 | Passer domesticus | Homemade barley | 70 | - | - |
| 49 | Perdix perdix | Partridge | 30 | - | - |
| 50 | Pernis apivorus | Viespar | 2 | - | - |
| 51 | Phasianus colchicus | Fazan | 12 | - | - |
| 52 | Phoenicurus phoenicurus | Forest cod | 4 | - | - |
| 53 | Pica pica | Coțofană | 69 | - | - |
| 54 | Riparia riparia | Shore grass | 1 | - | - |
| 55 | Saxicola rubetra | Large bramble | 10 | - | - |
| 56 | Streptoptopelia decaocto | Guguștiuc | 29 | - | - |
| 57 | Streptopelia turtur | Turturică | 5 | Northern Europe - South Eastern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 58 | Sturnus vulgaris | Common starling | 982 | Eastern Europe - South Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 59 | Sylvia atricapilla | Black-headed Silvie | 3 | Northern Europe - West Asia | Migrationatlas - https://migrationatlas.org/node/1830 |
| 60 | Turdus merula | Blackbird | 18 | - | - |
| 61 | Turdus philomelos | Singing Sturgeon | 7 | Southern Europe - Asia Center | Migrationatlas - https://migrationatlas.org/node/1830 |
| 62 | Turdus pilaris | Cock | 38 | Northern Europe - South Europe | Migrationatlas - https://migrationatlas.org/node/1830 |
| 63 | Upupa epops | Kiss | 51 | Central Europe - Southern Europe | Migrationatlas - https://migrationatlas.org/node/1830 |

Migration routes for bat species

The study area is not located in Natura 2000 protected areas, but species with European and National protected status are present in the study area (all chiroptera are protected).

The following species of bats have been identified in the PUZ area: Eptesicus serotinus, Miniopterus schreibersii, Myotis daubentonii, Myotis mystacinus/brandtii, Nyctalus leisleri, Nyctalus noctula, Pipistrellus nathusii/kuhlii, Pipistrellus pipistrellus, Pipistrellus pygmaeus, Plecotus auritus, Vespertilio murinus.

The most abundant taxon recorded in the 2022 monitoring campaign was Pipistrellus nathusii/kuhlii, followed by Nyctalus noctula. All groups/species recorded in the study area are very common for open areas in southern Romania.

The species Pipistrellus pipistrellus pipistrellus is among the regionally migratory species, seasonally migrating within a few hundred kilometers but dispersing or migrating up to 800 km . Pipistrellus nathusii can migrate somewhat longer distances (over 2000 km)

For these species a project has been developed to identify migration routes, and to provide operational maps (see figure X) to guide wind farm project planning .

Figure 2-16  Migration routes of Pipistrellus nathusi

Nyctalus noctula is a long-distance migrant species that can regularly fly 3,000-4,000 km in a round-trip flight from its summer breeding area to its winter habitat and back.

### Functional relationships

a) Regulating ecosystem services: Sites maintain essential ecological functions such as water purification, carbon storage, erosion protection and climate regulation, which are vital for both ecosystems and society.

b) Biological cycles and energy flow: Sites maintain functional ecological cycles (e.g. nitrogen cycle, carbon cycle), which are fundamental for biodiversity. The proper functioning of these cycles allows the natural balance to be maintained.

c) Protection and recovery of species of Community interest: Conservation actions in Natura 2000 sites aim to protect rare or threatened species and restore their populations. By managing habitat and reducing threats (such as pollution or unsustainable exploitation), the long-term survival of these species is ensured.

d) Sustainable management: It is essential that economic activities on or in the vicinity of sites are managed so as not to adversely affect biodiversity. Agricultural, forestry or tourism practices must be compatible with conservation objectives.

The structural and functional relationships of Natura 2000 sites are essential for maintaining their integrity and biodiversity, and sustainable management, ecological connectivity and the protection of ecosystem services play a crucial role in this process.

Table  2-5 Structural and functional relationships

| Species/ habitat name | Dependencies between ANPIC and groundwater and surface water bodies | Dependency relationships between species and habitats of Community interest | Dependency relationships between species/ habitats and other features (relief, geological, altitudinal, other) | Relationships between species of Community interest based on trophic or other interspecific relationships | Linking species and ecological corridors |
| --- | --- | --- | --- | --- | --- |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake |  |  |  |  |  |
| Spermophilus citellus (Gopher) | According to the Updated Management Plan of the Dobrogea Litoral Hydrographic Basin 2022-2027 the community habitats within ROSCI0071 depend on the groundwater aquifer RODL10 The natural area is not crossed by surface water bodies but it is crossed by a number of watercourses such as: Corvin cadastral code XIV_1.39a.2, Urluia cadastral code XIV_1.40.40, Ceair cadastral code XIV_1.40.1, Cerchez cadastral code XIV_1.40.1.1.1, Valea Baciului cadastral code XIV_1.40.2 | The characteristic habitat of the species includes meadows, islets, marshes, glades, and the like, their presence depending on the maintenance of sparse vegetation e.g. by grazing and tolerance by the local people. The species occurs in well drained grassland/island habitats at the site. | They are found in meadows, islets, marshes, glades, and the like, their presence depending on the maintenance of sparse vegetation e.g. by grazing and tolerance by local people. | A site resident species, it consumes plant species (seeds, leaves) from grassland habitats such as Trifolium campestre, T. arvense, T. repens, T. media, T. pratense, Medicago minima, Coronilla varia, Ononis spinosa, Plantago lanceolata, P. media, Pimpinella saxifrage, Festuca spp, Dactylis glomerata, Agropyron repens, Taraxacum sp., Achillea millefolium (Janák, , Marhoul, & Matějů, 2013). Action plan for the conservation of the European ground squirrel Spermophilus citellus in the European Union. European Commission.). In addition to plants, it also consumes large terrestrial atropods. In some habitats it could be an important consumer able to limit the spread of some plant species, including invasive plants (e.g. Solanum elaeagnifolium) The species is a trophic resource for birds of prey (e.g. Aquila heliaca, A. pomarina, Falco cherrug, Buteo buteo etc.), mammals (e.g: Mustela eversmanii, M. peregusna). | No special requirements for connectivity. No ecological corridors identified |
| Miniopterus schreibersii (Long-winged Bat) |  | Habitats of Community interest that the species uses at the site: 40C0, 62C0, 6430, 91AA, 91F0, 91I0, 91M0 | The long-winged bat shows a preference for heavily wooded areas. It is a species that shelters in caves or other underground cavities and needs special protection/specific gates at the entrance to the shelter | Migratory species, Insectivorous mammal that feeds mainly on invertebrates (nocturnal lepidopterans and coleopterans) It is a food resource for nocturnal birds of prey. The long-winged bat shows a preference for heavily wooded areas. It has a fast and maneuverable flight, hunting under the forest canopy, over water surfaces, or close to vegetation. | It is a medium-distance migratory species of 40-100 km, with a maximum recorded distance of 833 km No special requirements for connectivity. |
| Rhinolophus mehelyi (Méhely's horseshoe bat) |  | Habitats of Community interest that the species uses at site level: 6110*, 6210* | Individuals of the species can be found in caves in karst areas at altitudes up to 700 m. Summer and winter roosts are almost exclusively in caves, sometimes in mines | It feeds mainly on moths, but also consumes mosquitoes and cockroaches as a trophic resource for predatory birds | Hunt in forests of deciduous steppe, grassland, sometimes land agricultural. Although is a species sedentary, it can migrate at a distance up to 94- 100 km. No special requirements for connectivity. |
| ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii |  |  |  |  |  |
| Spermophilus citellus (Gopher) | According to the Updated Management Plan of the Dobrogea Litoral Hydrographic Basin 2022-2027 the ROSCI0157 community habitats depend on the groundwater aquifer RODL10 The natural area is not crossed by surface water bodies but it is also crossed by the watercourse: Albești cadastral code XV_1.12 | - | - | It consumes plant species (seeds, leaves) in grassland habitats such as Trifolium campestre, T. arvense, T. repens, T. media, T. pratense, Medicago minima, Coronilla varia, Ononis spinosa, Plantago lanceolata, P. media, Pimpinella saxifrage, Festuca spp, Dactylis glomerata, Agropyron repens, Taraxacum sp., Achillea millefolium (Janák, , Marhoul, & Matějů, 2013). Action plan for the conservation of the European ground squirrel Spermophilus citellus in the European Union. European Commission.). In addition to plants, it also consumes large terrestrial atropods. | No special requirements for connectivity. No ecological corridors identified |
| Rhinolophus ferrumequinum (Greater horseshoe bat) |  | Uses mixed habitats including specific forests, grasslands, scrublands and orchards and protected natural areas | Being a large species, it prefers underground cavities for hibernation and often forms breeding colonies in underground cavities as well as in high bridge buildings (churches) | Insectivorous mammal that feeds mainly on moths and beetles, rarely dipterans or hymenoptera | They only use the habitats on the site surface for foraging, with the species using buildings outside the site boundary for resting and hibernation. No special requirements for connectivity. |
| Mustela eversmannii (Steppe fer ferret) |  | Habitat of Community interest that the species uses at site level: 62C0 | They prefer arid and barren plains | Usually conditioned by the presence of the gopher Spermophilus citellus but also by other species of the families Microtidae and Muridae | No special requirements for connectivity. |
| Miniopterus schreibersii (Long-winged Bat) |  | Habitats of Community interest that the species uses from the site: 40C0, 62C0, 6430, 91AA, 91M0 | - | Insectivorous mammal whose main food source is invertebrates (nocturnal lepidopterans and coleopterans) and is a trophic resource for nocturnal birds of prey | Forest ecosystems are important ecological corridors for this species as they require hollow hollow trees for shelter. No special requirements for connectivity. |
| ROSPA0166 Plopeni - Chirnogeni |  |  |  |  |  |
| Anthus campestris ( Field Fern) | According to the Updated Management Plan of the Dobrogea Litoral Hydrographic Basin 2022-2027 the community habitats within ROSPA0166 depend on the groundwater aquifer RODL10 | - | It nests in lowland and low hill regions. The species prefers open, dry habitats with sparse vegetation and isolated shrublands such as steppe habitats, farmland margins, grasslands, as well as semi-arid habitats. | It is a predominantly insectivorous species, feeding on the ground, sometimes also in flight, feeding mostly on insects (Orthoptera, Isoptera, Odonata, Mantodea, Coleoptera), but also on other invertebrates (Mollusca), seeds and less rarely small vertebrates (reptiles). | No special requirements for connectivity. |
| Buteo rufinus ( Great Bute) |  | A nesting species, it is present on the site in wooded areas as well as in areas with isolated trees or on rocky valleys using mainly grasslands but also arable land as feeding areas. | The great tit is specific to steppe or steppe-influenced habitats. It nests in open and semi-open areas with meadows/grasses and mosaics with farmland | It feeds mainly on small mammals (occasionally reptiles, small birds or insects such as orthopterans or beetles), which it hunts from a high vantage point, flying in wide circles or sitting directly on the ground. | No special requirements for connectivity. |
| Circus cyaneus (Black-headed Erytra) |  | - | It prefers meadows/grasslands for nesting, but also marshy areas, young conifer plantations, taiga peat bogs, lowland or hilly farmland. Winters in open areas, especially at lower altitudes and is often found on agricultural land (SOR). The site includes agricultural land and meadows, meadows. | It feeds mainly on small mammals (mice, rats, rabbits) and small birds. During the breeding season, young birds or small birds are the predominant food. Occasionally they also eat invertebrates, reptiles, amphibians or bird eggs. Prey is mainly caught on the ground (SOR) | No special requirements for connectivity. |
| Falco columbarius ( Winter Falcon) |  | - | The winter hawk is characteristic of low wooded areas, grasslands and marshes. | It feeds mainly on small birds such as larks, skylarks, sparrows. It also feeds on insects, small mammals and snakes. | No special requirements for connectivity. |
| Falco vespertinus ( Evening Goshawk) |  | - | It nests mainly in semi-open habitats such as meadows/grasslands or traditional agricultural mosaics with mature trees, small forests (acacia plantations), ditches, where corvid nests are present: colonies of the common crow or isolated nests of the griffon and magpie. They nest in nests of these species. | It feeds mainly on insects (mainly Orthoptera, such as crickets, grasshoppers, grasshoppers, grasshoppers, but also other species), which it hunts by flying at a fixed point, at a height of several meters. Small vertebrates (small mammals, lizards, small birds, etc.) can make up a very important part of the diet, especially during the chick-feeding period. | No special requirements for connectivity. |
| Lanius minor ( Black-fronted Shrike) |  | - | Nests in open grassland or agricultural mosaic habitats with trees; sometimes also nests in orchards. Prefer grassland or meadow habitats with trees or in alignments (poplars), including ditches. Frequently nests in roadside trees. | They eat large insects (especially Orthoptera and Coleoptera). Occasionally eat spiders or other invertebrates. Very rarely eat small mammals or small birds. | No special requirements for connectivity. |
| ROSPA0036 Dumbrăveni |  |  |  |  |  |
| Accipiter brevipes ( Short-legged Eurasian Short-legged) | According to the Updated Management Plan of the Dobrogea Litoral Hydrographic Basin 2022-2027, the community habitats within ROSCI0071 depend on the groundwater aquifer RODL10 The natural area is not crossed by surface water bodies, but it is crossed by a number of watercourses such as Ceair cadastral code XIV_1.40.1 | The species prefers to nest in the wooded areas of the site, including plantations, feeding preferentially on grassland adjacent to them. | The short-legged elm is a species characteristic of low-lying wooded areas near water | It feeds on insects, lizards, birds and small mammals. | No special requirements for connectivity. |
| Anthus campestris ( Field Fern) |  | - | It nests in lowland and low hill regions. The species prefers open, dry habitats with sparse vegetation and isolated shrublands such as steppe habitats, farmland margins, grasslands, as well as semi-arid habitats. | It is a predominantly insectivorous species, feeding on the ground, sometimes also in flight, feeding mostly on insects (Orthoptera, Isoptera, Odonata, Mantodea, Coleoptera), but also on other invertebrates (Mollusca), seeds and less rarely small vertebrates (reptiles). | No special requirements for connectivity. |
| Aquila pomarina (Lesser Spotted Eagle) |  | - | Nesting in coniferous forests, deciduous forests, woodlands. The Lesser Spotted Eagle prefers mature deciduous forests, mostly oak, in hilly, lowland and lowland meadow areas for nesting. Some pairs also ascend into the mountains where they nest in beech and spruce forests. They nest in forests near pastures, wet meadows and agricultural areas that are large enough to provide food.Lesser Spotted Eagle nests in old trees with strong branches. Eagles often occupy nests built by other nesting species, such as the common buzzard - Buteo buteo, the common buzzard - Accipiter gentilis or even the black stork - Ciconia nigra. There are no set rules for the use of nests, which is why the protection of any raptor nests in eagle-occupied territories can be just as important. | A carnivorous species that feeds mainly on small mammals, amphibians, reptiles, birds and some insects. | No special requirements for connectivity. |
| Bubo bubo ( Owl) |  | - | It prefers wooded or semi-open areas with cliffs, steep slopes and mature trees. During the day it can be seen resting in old trees, rock crevices or caves. It prefers wild, undisturbed areas, river valleys with gorges, overgrown quarries, etc. It can also be seen near farmland in rocky areas, as well as near rubbish dumps. | The species feeds on mammals (mice, rats, hedgehogs, rabbits) and birds (crows, gulls, ducks or even birds of prey). Their diet may vary according to the season and the abundance of prey, so they occasionally eat amphibians, reptiles, fish or invertebrates | No special requirements for connectivity. |
| Buteo rufinus (Great Bute) |  | A nesting species, it is present on the site in wooded areas as well as in areas with isolated trees or on rocky valleys using mainly grasslands but also arable land as feeding areas. | The great tit is specific to steppe or steppe-influenced habitats. It nests in open and semi-open areas with meadows/grasses and mosaics with farmland | It feeds mainly on small mammals (occasionally reptiles, small birds or insects such as orthopterans or beetles), which it hunts from a high vantage point, flying in wide circles or sitting directly on the ground. | No special requirements for connectivity. |
| Caprimulgus europaeus ( Caprimulg) |  | - | It is characteristic of open, arid areas of coniferous or mixed coniferous forest and grasslands. It nests on the ground, in hollows in meadows or in the shelter of trees or bushes. The nest can be used for several successive years. | It feeds on insects flying at dusk or at night, which it catches in flight. | No special requirements for connectivity. |
| Circus cyaneus ( Black-headed Erytra) |  | - | It prefers meadows/grasslands for nesting, but also marshy areas, young conifer plantations, taiga peat bogs, lowland or hilly farmland. Winters in open areas, especially at lower altitudes and is often found on agricultural land (SOR). The site includes agricultural land and meadows, meadows. | It feeds mainly on small mammals (mice, rats, rabbits) and small birds. During the breeding season, young birds or small birds are the predominant food. Occasionally they also eat invertebrates, reptiles, amphibians or bird eggs. Prey is mainly caught on the ground (SOR) | No special requirements for connectivity. |
| Circus macrourus ( White Erete) |  | During the passage period, the site is a feeding ground for a number of species that mainly use arable land and grassland. | During the migration period it feeds mainly in open lowland areas, including farmland and wetlands. | It feeds mainly on small mammals (mice, rats) and small birds; additionally it consumes reptiles or large insects (especially in wintering areas). | No special requirements for connectivity. |
| Circus pygargus (Erete sur) |  | As far as nesting pairs are concerned, the site is home to a very small number of nesting pairs that mainly use arable land and grassland, | It nests in open areas with low natural vegetation and isolated scrub. Nests in meadows and pastures, farmland, scrubland, mires, peat bogs or other marshy areas. During the migration period it feeds mainly in open lowland areas, including farmland or wetlands. | Feeds mainly on small birds (especially freshly fledged juveniles) and small mammals (mice, rats), reptiles or large insects. They hunt by flying at low altitudes with infrequent wing beats. | No special requirements for connectivity. |
| Coracias garrulus (Dusky) |  | - | It is a species of open, wide, sunny areas with lower rainfall. It nests in areas of meadows/grassland or mosaics with agricultural crops (small areas), with mature trees with bark, in which it nests. Often occurs in areas with sandy or loamy soils, with breaks or landslides, where the soil is exposed, relatively vertical, in which it can burrow (SOR). | It is mostly insectivorous, with large insect species making up the majority of its diet (crickets, coropiscipids, various beetles, butterfly larvae, etc.). It also often eats other invertebrate species that are present on the ground (worms, centipedes, snails, scorpions), as well as small vertebrates (lizards, snakes, frogs, small mammals) (SOR). | No special requirements for connectivity. |
| Falco peregrinus (Peregrine Falcon) |  | - | It nests in montane or submontane habitats with cliffs and abundant vegetation, forest or scrub. The presence of open cliffs without vegetation is required. Generally avoids compact forested areas. | It feeds mainly on birds, with Columbiformes (pigeons) being the main food source in many areas. In coastal areas, marine species (gulls, petrels) can make up a large part of the food. Occasionally they also consume other prey such as small mammals (including bats), lizards or large insects. | No special requirements for connectivity. |
| Hieraaetus pennatus ( Little Eagle) |  | The species nests in the forest and uses adjacent land - predominantly grassland, not included in the SPA, as feeding areas | The species prefers forest habitats for nesting where there are open, natural areas or agricultural mosaics suitable for foraging. The forest types in which the species nests can vary. | The species' diet consists of small and medium-sized birds, small and medium-sized mammals (rabbits, squirrels, etc.), reptiles and sometimes insects. | No special requirements for connectivity. |
| Lanius collurio ( Red-breasted Shrike) |  | - | It forages in all open habitats, meadows and pastures with shrubs, or agricultural mosaics, of crops alternating with semi-natural habitats, with isolated shrubs or in alignments. It also enters localities where it finds suitable habitats (suburban wasteland, parks, gardens, etc.). | An opportunistic carnivorous species, it feeds mainly on large insects (Orthoptera, Coleoptera, Odontidae, etc.) and small vertebrates (rodents, lizards, frogs, small birds). In the fall, they also eat small fruits (wild cherries, elderberries, etc.). | No special requirements for connectivity. |
| Lanius minor (Black-fronted Shrike) |  | - | Nests in open grassland or agricultural mosaic habitats with trees; sometimes also nests in orchards. Prefer grassland or meadow habitats with trees or in alignments (poplars), including ditches. Frequently nests in roadside trees. | They eat large insects (especially Orthoptera and Coleoptera). Occasionally eat spiders or other invertebrates. Very rarely eat small mammals or small birds. | No special requirements for connectivity. |
| Melanocorypha calandra (Bărăgan's Woodpecker) |  | Present in open areas, predominantly in arable land on the site, but also in meadows, being less common than the field lark | Characteristic species of arid and cultivated lowlands, meadows and natural steppes | During the nesting season it feeds predominantly on insects and in winter on seeds and roots. | No special requirements for connectivity. |
| Milvus migrans ( Black Jay) |  | During passage periods, the species transits the area or uses the site as a feeding and resting area, preferring grasslands for feeding or wooded areas for resting. | Red Gaia is characteristic of forests near wetlands. | Insects, small mammals and remains of large mammals, birds, snakes, frogs and fish, vegetable matter | No special requirements for connectivity. |
| Pernis apivorus ( Viespar) |  | - | It is characteristic of deciduous forests with glades. | It feeds on insect larvae and adults, especially wasps and bees, but also rodents, birds, lizards and snakes. | No special requirements for connectivity. |
| Regularly migratory species Species associated with extensively used agricultural land |  |  |  |  |  |
| Alauda arvensis (Woodlark) | According to the Updated Management Plan of the Dobrogea Litoral Hydrographic Basin 2022-2027, the community habitats within ROSCI0071 depend on the groundwater aquifer RODL10 The natural area is not crossed by surface water bodies, but it is crossed by a number of watercourses such as Ceair cadastral code XIV_1.40.1 | - | Prefer steppe and lowland habitats with abundant grassy vegetation | Eat insects, seeds | No special requirements for connectivity. |
| Carduelis cannabina (Hemp) |  | - | The species is characteristic of open and semi-open habitats, especially ecotonal areas, such as grasslands, meadows with shrubs or spreading trees, forest margins, or extensive scrublands, orchards, vineyards or gardens, arable land with intermittent strips of uncultivated land, etc. | They feed on a wide variety of seeds, but also eat the fruits and buds of plants. In addition, it feeds on invertebrates, especially insects and their larvae, with the young feeding almost exclusively on animal food for the first few days after hatching. | No special requirements for connectivity. |
| Carduelis carduelis (Stellar) |  | - | It nests in a wide range of habitats, in trees or shrubs, such as open woodland habitats, parks with abundant trees, orchards and gardens, tree lines or ditches along rivers. It occupies any kind of semi-open habitat, including anthropogenic habitats (settlements). | It has a wide range of food, predominantly plants (buds, flowers, fruits), and in the cold season mainly seeds and dried fruits - with a preference for Asteraceae species; additionally it consumes small invertebrates and their larvae. | No special requirements for connectivity. |
| Coturnix coturnix (Quail) |  | - | Forages in large open areas, such as meadows with few shrubs and farmland. It prefers plains or broad depressions with grassland, farmland or mosaic areas; also occurs in extensive agricultural monocultures | It is an opportunistic species; it consumes mainly grass seeds, ruderal plants and cereals. It also feeds on invertebrates (worms, molluscs, ants, spiders, etc.), which it gathers from vegetation or can extract from the soil. | No special requirements for connectivity. |
| Emberiza melanocephala (Black-headed barnacle) |  | - | The species nests in open, dry, scrubby and sparsely wooded areas, often on farmland with bushes between plots or scraped patches of trees, in orchards, and on heaths bordering farmland. | It has a wide range of food, eating seeds (millet, rice, wheat, maize) or other plant parts. During the breeding season it feeds its young on invertebrates: butterfly larvae, crickets, ants, flies, etc. | No special requirements for connectivity. |
| Falco tinnununculus (Red-rumped Vulture) |  | - | Nesting in marshes, floodplains, farmland, meadows, pastures, grasslands, scrublands, orchards, parks, heaths, ridgelines, poles, buildings. | It feeds mainly on rodents but also reptiles, small birds and insects | No special requirements for connectivity. |
| Fringilla montifringilla (Winter finch) |  | - | In the nesting range, it prefers deciduous forests dominated by birch, or a mixture of resinous and birch. It also occupies forest habitats with willows and alders. Outside the nesting period, occurs in a wide variety of habitats, including broadleaved and mixed forests (beech and hornbeam forests preferred), especially ecotonal areas bordering arable or grassland. It also frequently intrudes into the interior of settlements, and in winter may be a visitor to feeders. | It feeds predominantly on seeds and fruits, especially the seeds of deciduous species, preferring the fruits of beech, hornbeam or alder. They also eat invertebrates and their larvae. The young feed mainly on lepidopteran larvae. | No special requirements for connectivity. |
| Galerida cristata (Woodlark) |  | - | It prefers open habitats, including human-made ones such as railways or airports. | It is omnivorous, feeding on plants in winter and invertebrates in summer | No special requirements for connectivity. |
| Lanius excubitor (Great Crested Crake) |  | - | Nests in open grassland or agricultural mosaic habitats with tall trees; sometimes also in orchards. Prefer nesting habitats with tall isolated trees or in alignments (poplars, including gullies). | Carnivorous, feeding mainly on small vertebrates (rodents, lizards, frogs, small birds) and large insects. | No special requirements for connectivity. |
| Lanius senator (Red-headed Shrike) |  | - | Nests in open habitats, mainly meadow/grassland, but also agricultural mosaics with isolated trees and scrub; sometimes also in orchards. Prefer nesting habitats classified as open woodland (or wooded meadows) in dry, sunny areas. | An almost exclusively insectivorous species, it consumes large insects (especially Coleoptera and Orthoptera). Occasionally consumes small reptiles (lizards) and very rarely small birds. | No special requirements for connectivity. |
| Merops apiaster (Prigorie) |  | - | Prefers open terrestrial habitats/agricultural land - nests in areas with sandy or loamy soils, with breaks or landslides, where the soil is exposed, relatively vertical, in which it can burrow. Also nests in high, muddy banks of low-lying rivers. | It is strictly insectivorous, consuming mainly species of the Hymenoptera family: bumblebees, wasps, bees | No special requirements for connectivity. |
| Miliaria calandra (Southern Mealy Borer) |  | - | It is characteristic of arid and cultivated lowlands, meadows and natural steppes. The nest is a hollow in the ground lined with straw and dried plant stems, over which a fine mat of grass is laid. Nests are generally sheltered under bushes or other plants | During the nesting season it feeds predominantly on insects and in winter on seeds and roots. Outside the nesting season it also feeds together with other species (southern southern prey). Monogamous and nests are solitary. | No special requirements for connectivity. |
| Motacilla alba (White motacilla) |  | - | The species nests in open and semi-open habitats such as: various wetlands, lakeshores and coastal areas, riparian areas, agricultural habitats, parks, gardens, man-made areas, etc. | It feeds mainly on terrestrial and aquatic invertebrates, including: insects and their larvae, spiders, snails, crustaceans, etc. | No special requirements for connectivity. |
| Motacilla flava (Yellow Codobat) |  | - | It prefers moist, low-vegetated habitats such as grasslands, meadows, fens and reed marshes. Outside the nesting season it can also be found near farmland, preferring lakes, ponds or rivers. | It catches insect prey on the ground or at water level, but can also fly short distances in the air to catch them, often following herds of cows or sheep to feed on insects that settle on them. | No special requirements for connectivity. |
| Oenanthe isabellina (Eastern Pietrar) |  | - | It is a specialist of arid, steppe-like areas, preferring wide meadows, sometimes with gravel or isolated cliffs, with sparse or absent bushes. It often shares its habitat with the gopher, whose galleries it uses for nesting. | Predominantly insectivorous species; feeds mainly on insects (especially beetles and ants), but also on other invertebrates (worms, spiders, scorpions, etc.). Occasionally eats plant seeds. | No special requirements for connectivity. |
| Oenanthe oenanthe (South Pietrar) |  | - | The species nests in open, stony areas with meadows, pastures, often also on uncultivated land, coastal meadows and stone-fenced farms, but also in low, gravelly coastal areas or on steep, earthy banks. In some ranges, the species also occurs in high, alpine areas. | Predominantly insectivorous species; feeds on larvae and adults of: beetles, lepidoptera, hymenoptera, grasshoppers, locusts, but also other invertebrates (spiders, earthworms, small snails, etc.). Occasionally they also eat small fruits (blackberry, blueberry, gooseberry, elderberry) especially in late summer/fall. | No special requirements for connectivity. |
| Saxicola torquata (Black bramble) |  | - | The black bramble nests in open and semi-open areas with low vegetation, often with ruderal plants. Its main nesting habitats are meadows with scrubs, preferring more open sites than the large bramble. It also nests in areas of agricultural mosaic, but requires bush and semi-natural areas. | A predominantly insectivorous species, it consumes larvae and adults of: beetles, butterflies, flies, locusts, ants, but also other invertebrates (spiders and earthworms). It occasionally consumes small vertebrates (small fish and lizards), small fruits and seeds. | No special requirements for connectivity. |
| Streptopelia turtur (Turtur) |  | - | It nests in open deciduous forests in low lying areas with abundant understory near agricultural areas, clearings, shrub thickets or alignments with abundant understory. In wintering areas it occurs in semi-open areas as well as steppe and semi-desert; it also occupies habitats with olive, palm, acacia and scrub plantations. | It feeds mainly on plant food, especially seeds, which it gathers from the ground. Occasionally also eat invertebrates: insects, pupae, worms and small snails. | No special requirements for connectivity. |
| Sturnus roseus (alternative name Pastor roseus) (Grasshopper) |  | - | Prefer steppe habitats and open farmland | It feeds mainly on plant food, especially berries, cereals, but also insects, mostly grasshoppers and beetles of the Lucanidae or Elateridae | No special requirements for connectivity. |
| Turdus iliacus (Vine Turd) |  | - | Nests in birch or mixed woodland, shrublands, woodland edges, river courses and the edges and floodplains of lakes. In winter it is found in open woodlands, tree hedgerows, sheepfolds, orchards, parks and gardens. | It is omnivorous, consuming a wide range of insects and earthworms throughout the year, supplemented by berries in the fall and winter | No special requirements for connectivity. |
| Upupa epops (Kiss) |  | - | Foraging mainly in open and semi-open habitats, such as meadows/grasses with mature trees, orchards, tree lines, tree lines, hedgerows. It also occurs in areas of farmland with traditional agriculture (mosaics of small areas alternating with natural vegetation). | It is predominantly insectivorous, with large ground-dwelling species making up the majority of its diet (crickets, coropiscip, various beetles, butterfly larvae, etc.). It also consumes other invertebrate species that are present on or in the soil (worms), as well as small vertebrates (lizards, snakes, frogs). | No special requirements for connectivity. |
| Species associated with forest habitats |  |  |  |  |  |
| Carduelis chloris (Florinte) |  | - | It nests in a wide range of habitats, in trees or shrubs, such as open woodland habitats, towns, parks with abundant trees, orchards and gardens, tree lines or ditches along rivers. They occupy any semi-open habitat, including anthropogenic habitats. | It has a wide range of food, predominantly plants (buds, flowers, fruits), including nuts (ash, hornbeam). In addition, it consumes invertebrates, especially during the breeding season. | No special requirements for connectivity. |
| Carduelis spinus |  | - | During the nesting season, the species prefers coniferous and mixed forests on the montane floor. Outside the nesting period the species can be observed in a wide range of habitats: most types of forests, orchards, parks, arable areas with stubble stubble, grasslands, etc. | The snail feeds predominantly on plants (buds, flowers, etc.), in the cold season feeding mainly on seeds and dried fruits. They also eat invertebrates. | No special requirements for connectivity. |
| Coccothraustes coccothraustes ( Botgros) |  | - | It occurs in forest habitats, especially in deciduous forests with hornbeam, mixed with hornbeams or other species. Sometimes also occurs in mixed resinous forests, especially in the lower altitudinal parts. | They eat mainly plant food, especially seeds (even cherry), buds or flowers. Carpen seeds are a significant part of their diet. | No special requirements for connectivity. |
| Falco subbuteo (Sparrow hawk) |  | - | It nests in semi-open, forest-like habitats (steppe areas with sparsely or sparsely wooded or open woodland). It is found in meadow/grassland or traditional agricultural mosaics with mature trees, small woodland, small woodland, and ditches. It also often occurs in large city parks. | It feeds mainly on large insects (mainly Orthoptera, such as crickets, grasshoppers, grasshoppers, grasshoppers, but also other species) and small birds, which it catches in flight. Occasionally it also eats other animals (lizards, small mammals). | No special requirements for connectivity. |
| Ficedula hypoleuca (Black Muscar) |  | Forest Habitats of Community Interest in ROSAC0071: 91AA, 91F0 , 91I0, 91M0 | It prefers mature deciduous forests (but also nests in mixed or coniferous forests, especially in northern regions) with trees with natural cavities used for nesting. Sometimes also nests in natural-looking parks | It usually feeds on flying insects, which it catches in short flights. It eats a wide range of invertebrates (insects and their larvae, spiders, snails, etc.) but occasionally small fruits or seeds. | No special requirements for connectivity. |
| Fringilla coelebs (Finch) |  | - | Foraging in forest habitats, parks with abundant and mature trees, sometimes in tree lines or ditches along rivers. It occupies all types of forest habitat, from coniferous forests to lowland oak or poplar woodlands. | It feeds on small invertebrates and their larvae, as well as seeds and shoots | No special requirements for connectivity. |
| Jynx torquilla (Capintors) |  | - | It forages mainly in open habitats such as meadows/grasslands with mature trees, orchards, heaths, heaths, heaths. The presence of mature trees with natural hollows or hollows excavated by other woodpecker species is obligatory (since it does not dig its own hollow). | Predominantly insectivorous, with ants making up the majority of the diet (especially larvae and pupae). Also eat other insect species that are present on the ground, or spiders and other invertebrates. Occasionally eat small fruits of ground-level plants. | No special requirements for connectivity. |
| Muscicapa striata (Muscar sur) |  | - | The species prefers bright forests, often with small glades and openings. It nests in woodland edges, gardens, parks, orchards, but also in mature trees, along streams, rivers and the edges of standing water. | It is a predominantly insectivorous species; like other species of flycatchers, it chooses a high perch (usually in the canopy of trees) from which it hunts insects in short flights. It feeds mainly on flies, bees and wasps, but also on larvae and adults of butterflies and dragonflies. It also eats other invertebrates (spiders, snails, earthworms) or small fruits (scorus, porumbar, mullein, dogwood, horn, myrtle, etc.). | No special requirements for connectivity. |
| Oriolus oriolus (Grangur) |  | - | It forages in a wide variety of habitats where trees are present, including broadleaved and mixed broadleaved forests, riparian forests, parks, orchards, gardens, as well as arable areas where isolated stands of trees are present. | It is omnivorous, feeding mainly on invertebrates and fruit, but occasionally on seeds, nectar, pollen, and less frequently on small reptiles, small mammals, eggs and young of other bird species, usually small. | No special requirements for connectivity. |
| Phylloscopus collybita (Lesser Collybita ) |  | Forest Habitats of Community Interest in ROSAC0071: 91AA, 91F0 , 91I0, 91M0 | It prefers forest habitats with a well-developed shrub layer. It occurs in deciduous, mixed and coniferous forests, areas with abundant scrub (including alpine areas). It can also nest in parks or gardens with a natural appearance and abundant undergrowth. | It is insectivorous, consuming mainly insects (including eggs and larvae) but also other invertebrates such as worms, spiders, etc. Outside the breeding season it also consumes small fruits or seeds | No special requirements for connectivity. |
| Phylloscopus sibilatrix (Sizzling Wood-Dove) |  | Forest Habitats of Community Interest in ROSAC0071: 91AA, 91F0 , 91I0, 91M0 | It prefers forest habitats with a well-developed shrub layer. It occurs in deciduous, mixed and coniferous forests, areas with abundant scrub (including alpine areas). It can also nest in parks or gardens with a natural appearance and abundant undergrowth. | It is insectivorous, consuming mainly insects (including eggs and larvae) but also other invertebrates such as worms, spiders, etc. Outside the breeding season it also consumes small fruits or seeds | No special requirements for connectivity. |
| Phylloscopus trochilus (Whistling Wood-Dove) |  | - | It prefers areas with abundant scrub (including alpine with juniper) and scrubby transition zones at the edge of forest habitats, or rarer forests with abundant scrub. | The species is insectivorous, consuming mainly insects (including eggs and larvae) but also other invertebrates such as worms, spiders, etc. Outside the breeding season it also consumes plant material. | No special requirements for connectivity. |
| Pyrrhula pyrrhula (Mugurar) |  | - | The species is found in deciduous, mixed and coniferous forests on the montane floor, where there are clearings or where there is a rich understory. | They eat mainly seeds, fruits, buds and shoots from a wide variety of plant species including beech, spruce and oak. It also occasionally eats invertebrates, more intensively during the nesting period, with the young feeding mainly on insects and their larvae. | No special requirements for connectivity. |
| Regulus ignicapillus (Spur-browed Auk) |  | - | It can be found in a variety of woodland habitats, from coniferous to deciduous, mainly in oak and beech forests, but also in parks. | It feeds on small invertebrates. | No special requirements for connectivity. |
| Regulus regulus (Yellow-headed Auk) |  | Forest Habitats of Community Interest in ROSAC0071: 91AA, 91F0 , 91I0, 91M0 | It nests in coniferous and deciduous forests, especially in areas where it finds pine trees, but in winter it also reaches parks and gardens. | Feeds on small insects, spiders, seeds | No special requirements for connectivity. |
| Serinus serinus (Caper) |  | - | It prefers hilly and mountainous regions and can be seen in forests, clearings, open cultivated areas, high ridges, orchards, plantations, urban parks and gardens | It feeds mostly on seeds on the ground, but also on plant parts, buds, seeds and flowers, as well as insects, moth larvae, spiders. | No special requirements for connectivity. |
| Sturnus vulgaris (Graur) |  | - | The species nests in open habitats where suitable nesting sites are present, such as hollow cavities in hollowed out trees and man-made constructions with access to foraging sites such as agricultural areas or other areas with low vegetation, including parks and gardens. Outside the nesting period it is present in a wide variety of habitats, but mainly in agricultural habitats. | It is predominantly insectivorous, especially during the breeding period, preferring a wide range of insects (ants, butterflies, bees, wasps, beetles, beetles, flies, etc.), but also other invertebrates (snails, spiders, earthworms, centipedes, etc.). It also feeds on vertebrates, preferring frogs, newts and lizards. Plant food is very variable, including apple, pear, cherry, cherry, plum, horn, vine, elder, elderberry, sorb, etc., as well as cereals. | No special requirements for connectivity. |
| Sylvia atricapilla (Black-headed Sylvia) |  | Forest Habitats of Community Interest in ROSAC0071: 91AA, 91F0 , 91I0, 91M0 | It prefers forest habitats with a well-developed shrub layer. Occurs in deciduous and mixed forests, especially in scrub-rich heathland. May also nest in natural-looking parks or gardens with abundant undergrowth. | The species is omnivorous, but during the nesting season it is predominantly insectivorous (consuming other invertebrates such as worms, spiders, etc.). Outside the breeding season it is predominantly frugivorous, consuming small fruits, but also other plants (buds, seeds, pollen, nectar). | No special requirements for connectivity. |
| Sylvia curruca (Little Sylvia) |  | - | It prefers areas with dense shrubs, garden hedges and young groves. It nests in mosaic farmland, scrubby parks, young conifers, scrubby gardens and fruit shrubs, even in dense thickets. | Their food consists mainly of invertebrates (butterflies, ants, flies, spiders) and their larvae, especially during the breeding season. Outside the breeding period they also eat fruit, nectar or pollen from various plants. | No special requirements for connectivity. |
| Turdus merula (Blackbird) |  | - | The species is omnivorous and opportunistic, its diet consisting of insects and their larvae, earthworms, snails, spiders, small vertebrates (newts, frogs, lizards, young of other birds, etc.), as well as fruits of: pigeon, hawthorn, hornwort, hornwort, blackthorn, mulberry, elder, elderberry, apple, pear, pear and others. During the breeding season, they prefer animal food, which is more abundant, while in winter they rely more on plant food. | The species nests in a wide range of habitats, occurring in most types of woodland, heaths, orchards, shrub lines, gardens and parks. | No special requirements for connectivity. |
| Turdus philomelos (Singing Sturz) |  | - | Does not nest in Romania. On migration it prefers flooded channels, puddles, lake shores, narrow ditches, often with vegetation, etc. | The species is omnivorous, but feeds predominantly on aquatic and terrestrial insects, especially beetle adults and larvae, dragonfly larvae, ants, worms, small crustaceans, spiders and fish. Occasionally they also eat plants (parts of plants). | No special requirements for connectivity. |
| Turdus pilaris (Cock Cock) |  | - | It prefers mixed nesting habitats with both woodland and open habitats, but also occurs in parks and gardens. Outside the nesting season it occurs at lower altitudes, feeding on arable or grassland, but also in scrub. | It is an omnivorous species. Animal food consists of insect adults and larvae, but also other invertebrates (snails, spiders, etc.). Plant food consists mainly of seeds and fruits of pigeon pea, hawthorn, elder, elderberry, blackberry, strawberry and others | No special requirements for connectivity. |
| Turdus viscivorus (Mistle Thrush) |  | Forest Habitats of Community Interest in ROSAC0071: 91AA, 91F0 , 91I0, 91M0 | It forages in medium to high altitude mature deciduous, mixed deciduous and coniferous forests, where there are clearings, glades, rarified forests, or forest edges. | They eat mainly invertebrates (insects and their larvae, spiders, snails, earthworms, etc.), rarely vertebrates (young of other birds), and during the cold season they eat fruits and seeds. | No special requirements for connectivity. |
| Species associated with reed habitats |  |  |  |  |  |
| Acrocephalus arundinaceus (Great Grebe) | Surface watercourse Ceair cadastral code XIV_1.40.1 | - | It can be found in areas with aquatic vegetation, especially in reed and reed areas in shallow water. | Feeds on insects and other invertebrates, occasionally on small vertebrates, and in the fall it also feeds on fruit | No special requirements for connectivity. |
| Acrocephalus palustris (Marsh wagtail) | Surface watercourse Ceair cadastral code XIV_1.40.1 | - | It nests in areas with natural or semi-natural vegetation near wetlands: stretches of rockroses or pipirig, areas with sparse reed, roadsides, canals or farmland with tall ruderal plants. Does not nest in wetlands proper (reedbeds) but occupies any peripheral natural or semi-natural habitat. | They eat a wide range of invertebrates (especially insects, arachnids, small snails, worms). In late summer and fall, they also eat small fruits. | No special requirements for connectivity. |
| Acrocephalus schoenobaenus (Little Grebe) | Surface watercourse Ceair cadastral code XIV_1.40.1 | - | It nests in areas of reed and reedbeds, most often in large areas associated with a very wide range of aquatic habitats (lakes, rivers, canals, estuaries, etc.). And during migration it usually follows aquatic habitats. | They eat a wide range of invertebrates (especially insects, arachnids, small snails, worms). | No special requirements for connectivity. |
| Acrocephalus scirpaceus (Reed Reed Reed) | Surface watercourse Ceair cadastral code XIV_1.40.1 | - | It forages in areas of massive, compact, large expanses of reedbeds, most often associated with lowland aquatic habitats. It also forages on adjacent habitats (including agricultural land). | Predominantly insectivorous but also consumes a wide range of invertebrates (arachnids, small snails, worms). In late summer and fall, it also eats small fruits | No special requirements for connectivity. |
| Locustella fluviatilis (Red-crested Cricket) |  | - | Breeding in marshes, floodplains, farmland, meadows, pastures, scrublands. | Feeds on invertebrates | No special requirements for connectivity. |
| Locustella luscinioides (Reed Cricket) | Surface watercourse Ceair cadastral code XIV_1.40.1 | - | It nests in areas of massive, compact, wide-spreading reedbeds, most often associated with lowland aquatic habitats | It is a predominantly insectivorous species, but also consumes a wide range of invertebrates (arachnids, small snails, worms). | No special requirements for connectivity. |
| Riparia riparia (Mal's grass) | Possible surface watercourse Ceair cadastral code XIV_1.40.1 | - | The species nests mainly in open areas with sandy and high banks of flowing and standing water, sometimes within sand pits where erosion has created vertical walls within which the species digs galleries for nesting. It also sometimes nests at considerable distances where it finds smooth walls. Feeds in open habitats in the nesting area, in grassland, meadows, hay meadows, arable and wetland surfaces. It does not avoid man-made habitats, especially if there are sites for building galleries. | The species consumes mainly insects and, to a lesser extent, spiders. Diet varies greatly depending on nesting area and multi-year and seasonal variation in insect populations. | No special requirements for connectivity. |
| Species associated with cliff habitats and urban areas |  |  |  |  |  |
| Monticola saxatilis (Stone blackbird) |  | - | For nesting they prefer hilly, sparsely vegetated areas | The species mainly eat insects, berries and small reptiles | No special requirements for connectivity. |
| Hirundo daurica (Red-crowned Swallowtail) |  | - | The species nests in open habitats where cliffs, gorges, gorges, steep valleys or quarries are present, but also in anthropogenic habitats. Outside the nesting period it can be seen in grassland and cultivated areas, usually nesting in reedbeds. | The species is almost exclusively insectivorous, consuming insects (Diptera, Coleoptera, Hemiptera, Hymenoptera) that it catches in flight, sometimes also feeding at ground level. It hunts individually, in pairs, or sometimes in mixed groups with other Hirundinidae. | No special requirements for connectivity. |
| Hirundo rustica (Swallow) |  | - | It forages mainly in open, rural, man-made areas with patchy areas of agricultural habitats, grasslands and meadows, which it uses intensively for foraging. They also often enter cities, especially in outlying areas. On migration, they use reed meadows as resting places. | An almost exclusively insectivorous species, it consumes mainly flying insects that it catches in flight. It also eats other invertebrates (spiders, other arthropods). Occasionally consumes seeds or small fruits in wintering quarters. | No special requirements for connectivity. |
| ROSPA0094 Hagieni Forest |  |  |  |  |  |
| Accipiter brevipes (Short-legged Eurasian Short-legged) | - | The species prefers to nest in the wooded areas of the site, including plantations, feeding preferentially on grassland adjacent to them. | The short-legged elm is a species characteristic of low-lying wooded areas near water | It feeds on insects, lizards, birds and small mammals. | No special requirements for connectivity. |
| Alcedo atthis (Blue Gull) | Possible watercourse: Albești cadastral code XV_1.12 | - | It needs steep, exposed, bare banks (muddy, clayey or otherwise) in which it can burrow to nest. | Predominantly ihthiophagous, consuming small fish species, after which it dives and submerges from its perch above the water.In addition, it also eats invertebrates (dragonflies, worms, snails, shrimps, etc.) or amphibians. Very rarely in winter, they also eat small fruits (elderberries) or reed stems | No special requirements for connectivity. |
| Anthus campestris (Field martin) | - | - | It nests in lowland and low hill regions. The species prefers open, dry habitats with sparse vegetation and isolated shrublands such as steppe habitats, farmland margins, grasslands, as well as semi-arid habitats. | It is a predominantly insectivorous species, feeding on the ground, sometimes also in flight, feeding mostly on insects (Orthoptera, Isoptera, Odonata, Mantodea, Coleoptera), but also on other invertebrates (Mollusca), seeds and less rarely small vertebrates (reptiles). | No special requirements for connectivity. |
| Aquila heliaca (Field Eagle) | - | - | It forages in forests or tree stands in steppe or open plains, but also in hilly areas, often near water. Hunts in open, often cultivated areas. | It mainly eats medium-sized mammals (rabbits, foxes, gophers) and small mammals (rodents) but also birds (corvids, waterfowl and night birds of prey). It also occasionally feeds on reptiles, amphibians, crustaceans or insects. | No special requirements for connectivity. |
| Aquila pomarina (Lesser Spotted Eagle) | - | - | Nesting in coniferous forests, deciduous forests, woodlands. The Lesser Spotted Eagle prefers mature deciduous forests, mostly oak, in hilly, lowland and lowland meadow areas for nesting. Some pairs also ascend into the mountains where they nest in beech and spruce forests. They nest in forests near pastures, wet meadows and agricultural areas that are large enough to provide food.Lesser Spotted Eagle nests in old trees with strong branches. Eagles often occupy nests built by other nesting species, such as the common buzzard - Buteo buteo, the common buzzard - Accipiter gentilis or even the black stork - Ciconia nigra. There are no set rules for the use of nests, which is why the protection of any raptor nests in eagle-occupied territories can be just as important. | A carnivorous species that feeds mainly on small mammals, amphibians, reptiles, birds and some insects. | No special requirements for connectivity. |
| Burhinus oedicnemus (Common Oedocnemus ) | - | - | The common owlet is a species characteristic of open steppe, grassland and agricultural crops. | It feeds on insects and larvae, snails, worms, earthworms, frogs, seeds, small mammals and birds. | No special requirements for connectivity. |
| Buteo rufinus (Great Bute) | - | - | Specific to steppe or steppe-influenced habitats. It nests in open and semi-open areas with meadows/grasses and mosaics with agricultural land | It feeds mainly on small mammals (occasionally reptiles, small birds or insects such as orthopterans or beetles), which it hunts from a high vantage point, flying in wide circles or sitting directly on the ground. | No special requirements for connectivity. |
| Caprimulgus europaeus (Caprimulg) | - | - | It is characteristic of open, arid areas of coniferous or mixed coniferous forest and grasslands. It nests on the ground, in hollows in meadows or in the shelter of trees or bushes. The nest can be used for several successive years. | It feeds on insects flying at dusk or at night, which it catches in flight. | No special requirements for connectivity. |
| Calandrella brachydactyla (Flock of Common Woodpeckers) | - | - | A species characteristic of lowland, open and arid regions, cultivated or natural. | Feeds mainly on insects and seeds | No special requirements for connectivity. |
| Ciconia ciconia (White Stork) | - | - | Nesting in cliffs, orchards, parks, coniferous forests, deciduous forests, poles, buildings. | Micromammals (mice, mosquitoes), lizards, snakes, amphibians, small birds (especially chickens, sometimes eggs), large insects. In aquatic areas, food is diversified to include fish and aquatic invertebrates (molluscs, crustaceans). They also eat plant matter. | No special requirements for connectivity. |
| Circaetus gallicus (Snake) | - | - | A species that prefers a mosaic of habitats with wooded areas used for nesting and open areas preferred for feeding. | It feeds mainly on snakes and chooses non-venomous species. It also feeds on lizards, frogs, small mammals and rarely birds or invertebrates. | No special requirements for connectivity. |
| Circus aeruginosus (Reed cattail) | - | - | It prefers wetlands with extensive marsh habitats. It is also present and feeds in other habitats such as farmland, grassland and woodland, where these are close to wetlands | It feeds on small to medium-sized birds, their young and eggs, mammals (especially rodents and rabbits), but also fish, reptiles, amphibians and invertebrates. | No special requirements for connectivity. |
| Circus cyaneus (Black-headed Erytra) | - | - | It prefers meadows/grasslands for nesting, but also wetlands, young conifer plantations, taiga peat bogs, lowland or hilly farmland. It winters in open areas, especially at lower altitudes and is often found on agricultural land. The site includes farmland and meadows, meadows. | It feeds mainly on small mammals (mice, rats, rabbits) and small birds. During the breeding season, young birds or small birds are the predominant food. Occasionally they also eat invertebrates, reptiles, amphibians or bird eggs. Prey is mainly caught on the ground | No special requirements for connectivity. |
| Circus macrourus (White Erete) | - | - | Foraging in open areas with natural meadow/grassland vegetation with steppe influences. Also foraging in semi-desert and woodland areas/ During migration feeds mainly in open lowland areas, including agricultural land or wetlands. | It feeds mainly on small mammals (mice, rats) and small birds; additionally it consumes reptiles or large insects (especially in wintering areas). | No special requirements for connectivity. |
| Circus pygargus (Erete sur) | - | - | It nests in open areas with low natural vegetation and isolated scrub. Nests in meadows and pastures, farmland, mires, peat bogs or other marshy areas. During the migration period it feeds mainly in open lowland areas, including farmland or wetlands. | Feeds mainly on small birds (especially freshly fledged juveniles) and small mammals (mice, rats), reptiles or large insects. They hunt by flying at low altitudes with infrequent wing beats. | No special requirements for connectivity. |
| Coracias garrulus (Dusky) | Possible watercourse: Albești cadastral code XV_1.12 | - | It is a species of open, wide, sunny areas with lower rainfall. It nests in areas of meadows/grassland or mosaics with agricultural crops (small areas), with mature trees with bark, in which it nests. Often occurs in areas with sandy or loamy soils, with breaks or landslides, where the soil is exposed, relatively vertical, in which it can burrow its galleries | It is mostly insectivorous, with large insect species making up the majority of its diet (crickets, coropiscipids, various beetles, butterfly larvae, etc.). It also often eats other invertebrate species that are present on the ground (worms, centipedes, snails, scorpions), as well as small vertebrates (lizards, snakes, frogs, small mammals) | No special requirements for connectivity. |
| Falco cherrug (Saker falcon) | - | - | The species prefers open, steppe-like areas, such as meadows and agricultural areas, with access to a minimum of nesting habitat (ditches, tree lines, rocky areas, quays, etc.). | It feeds mainly on small mammals, in Romania it feeds mainly on gophers (Spermophilus citellus), but also on other rodents. It may also eat medium-sized birds (corvids, pigeons, partridges, quails, pheasants, etc.), but also lizards or large insects. | No special requirements for connectivity. |
| Falco columbarius (Wintering Falcon) | - | - | The winter hawk is characteristic of low wooded areas, grasslands and marshes | Feeds mainly on small birds such as larks, skylarks, sparrows. It prefers young, inexperienced chicks. Its diet is also supplemented with insects, small mammals and snakes. | No special requirements for connectivity. |
| Falco peregrinus (Peregrine Falcon) | - | - | It nests in montane or submontane habitats with cliffs and abundant vegetation, forest or scrub. The presence of open cliffs without vegetation is required. Generally avoids compact forested areas. | It feeds mainly on birds, with Columbiformes (pigeons) being the main food source in many areas. In coastal areas, marine species (gulls, petrels) can make up a large part of the food. Occasionally they also consume other prey such as small mammals (including bats), lizards or large insects. | No special requirements for connectivity. |
| Falco vespertinus (Evening Goshawk) | - | - | It mainly nests in semi-open habitats such as meadows/grasslands or traditional agricultural mosaics with mature trees, small forests (acacia plantations), ditches, where corvid nests are present: colonies of Common Crow present: colonies of Common Crow or isolated nests of Common Crow or isolated nests of Common and Common Tit. For nesting they occupy nests of these species. | It feeds mainly on insects (especially Orthoptera, such as crickets, grasshoppers, grasshoppers, grasshoppers, but also other species), which it hunts by flying at a fixed point, at a height of several meters. Small vertebrates (small mammals, lizards, small birds, etc.) can make up a very important part of the diet, especially during the chick-feeding period. | No special requirements for connectivity. |
| Hieraaetus pennatus (Little Eagle) | - | - | The species prefers forest habitats for nesting where there are open, natural areas or agricultural mosaics suitable for foraging. The forest types in which the species nests can vary. | The species' diet consists of small and medium-sized birds, small and medium-sized mammals (rabbits, squirrels, etc.), reptiles and sometimes insects. | No special requirements for connectivity. |
| Lanius collurio (Red-breasted Shrike) | - | - | It forages in all open habitats, meadows and grasslands with shrubs, or agricultural mosaics, of crops alternating with semi-natural habitats, with isolated shrubs or in alignments. It also enters localities where it finds suitable habitats (suburban wasteland, parks, gardens, etc.). | An opportunistic carnivorous species, it feeds mainly on large insects (Orthoptera, Coleoptera, Odontidae, etc.) and small vertebrates (rodents, lizards, frogs, small birds). They also eat small fruits in the fall (wild cherries, elderberries, etc.). | No special requirements for connectivity. |
| Lanius minor (Black-fronted Shrike) | - | - | Nests in open grassland or agricultural mosaic habitats with trees; sometimes also nests in orchards. Prefer grassland or meadow habitats with trees or in alignments (poplars), including ditches. Frequently nests in roadside trees | They eat large insects (especially Orthoptera and Coleoptera). Occasionally eat spiders or other invertebrates. Very rarely eat small mammals or small birds. | No special requirements for connectivity. |
| Larus melanocephalus (Black-headed Gull) | - | - | An aquatic species, both in and out of the nesting season, it is tied to vegetated, brackish waters. During migration they remain in certain areas of large standing water in flocks of hundreds or thousands for longer periods. | It feeds predominantly on aquatic invertebrates and small fish. Often, especially during migration, it feeds on farmland or meadows near its resting places, on invertebrates but also on small mammals or plant matter (seeds left after harvesting). They also sometimes visit landfill sites. | No special requirements for connectivity. |
| Larus minutus (Little Gull) | - | - | They prefer reed-rich lakes, marshes or lagoon shores with brackish or marine water. | It feeds on insects, including dragonflies, worms and small fish, but prefers chironomid larvae. | No special requirements for connectivity. |
| Lullula arborea (Woodlark) | - | - | It prefers open areas in deciduous or coniferous forests with abundant herbaceous vegetation. | It feeds on insects and seeds. | No special requirements for connectivity. |
| Melanocorypha calandra (Bărăgan's Woodpecker) | - | - | It is characteristic of arid and cultivated lowlands, meadows and natural steppes. The nest is a hollow in the ground lined with straw and dried plant stems, over which a fine mat of grass is laid. Nests are generally sheltered under bushes or other plants | During the nesting season it feeds predominantly on insects and in winter on seeds and roots. Outside the nesting season it also feeds together with other species (southern southern prey). Monogamous and nests are solitary. | No special requirements for connectivity. |
| Milvus migrans (Black Jay) | - | - | It is characteristic of forests near wetlands. | Feeds mainly on insects, small mammals and the remains of large mammals, birds, snakes, frogs and fish, Plant matter | No special requirements for connectivity. |
| Nycticorax nycticorax (Night heron) | - | - | They prefer extensive, natural, aquatic habitats with rich vegetation in which to locate their colonies and extensive marshy areas for foraging. Nests are built of twigs or reeds. Nesting usually occurs in more secluded, hidden areas in dense vegetation, in trees or on reeds, near or over water. | It is an opportunistic carnivorous species, feeding on a very wide range of aquatic or wetland organisms, especially small fish, larvae, amphibians, molluscs or reptiles. Occasionally they also hunt in peripheral wetland habitats, especially orthopterans, beetles, leeches, small mammals or even other small bird species. | No special requirements for connectivity. |
| Oenanthe pleschanka (Black Petrel) | - | - | The species nests in stony and arid areas with sparse vegetation, in areas with abundant cliffs (gorges, rock walls, cliffs), abandoned quarries, eroded ravines, but also on steep mountain slopes and high plateaus with cliffs and meadows. | During the breeding season it feeds on invertebrates: ants, flies, moths, spiders, mites, spider mites, grasshoppers, grasshopper larvae, small snails. It uses high supports from which it flies to the ground to catch insects. Outside the breeding season they eat small fruits and seeds. | No special requirements for connectivity. |
| Pelecanus crispus (Crested Pelican) | - | - | They prefer nesting habitats similar to those occupied by the common pelican, rivers, lakes, lagoons, estuaries, usually nesting in small colonies on islands or in extensive reedbeds. | It is an ichthyophagous species, generally consuming carp, redfish, perch, etc. It hunts singly or in small groups, sometimes with groups of cormorants. | No special requirements for connectivity. |
| Pernis apivorus (Viespar) | - | - | It is characteristic of deciduous forests with glades. | It feeds on insect larvae and adults, especially wasps and bees, but also rodents, birds, lizards and snakes. | No special requirements for connectivity. |
| Philomachus pugnax (Bully) | - | - | It is characteristic of swamps, marshes and wet grasslands, especially in the Arctic tundra. In migration it is common on shorelines, wet meadows, marshlands and arable. | On migration it feeds on insects, molluscs, spiders, frogs, small fish, aquatic plants and rice or cereal seeds. | No special requirements for connectivity. |
| Porzana porzana porzana/Calidris pugnax (Spotted Crested) | - | - | They prefer moist, marshy, vegetation-rich places: marshes, wet meadows, the edges of drainage channels, puddles, grassy edges of lakes, and slow-flowing rivers. For nesting they prefer shallow water, reedy, reedy, reedy, reedy areas of steppe, semi-desert, forest, woodland and woodland. | It feeds on both animal food (small aquatic insects and their larvae, molluscs, snails, worms, earthworms, spiders and even small fish) and plant food (seeds, leaves, shoots, roots of various plants, algae). | No special requirements for connectivity. |
| Recurvirostra avosetta (Ciocîntors) | Possible watercourse: Albești cadastral code XV_1.12 | - | Preferred nesting habitats during nesting periods are the edges of brackish or brackish aquatic habitats with shallow, standing water and sparse vegetation (with exposed patches of mud). They prefer areas isolated from the shore, often islands or areas of emergent vegetation, to be safe from predators. Outside the nesting period it can be found in most aquatic habitats in shallow water where it can forage. | It is a carnivorous species that feeds mainly on invertebrates living in aquatic habitats, but also fish and plant material. | No special requirements for connectivity. |
| Tadorna ferruginea (Red-crested Shelduck) | - | - | The redhorse prefers brackish or saltwater aquatic habitats in open areas, but also occurs on freshwater aquatic surfaces. | The species feeds mainly on grasses, leaves, seeds and stems of aquatic and marsh plants as well as crop plant material (rice and cereals), aquatic invertebrates (worms, insects, crustaceans, molluscs), small fish and amphibians. They feed relatively frequently on land. | No special requirements for connectivity. |
| Regularly migratory species Species associated with open/agricultural land |  |  |  |  |  |
| Alauda arvensis (Woodlark) | - | - | Prefer steppe and lowland habitats with abundant grassy vegetation | Eat insects, seeds | No special requirements for connectivity. |
| Buteo lagopus (Shoed Buteo) | - | - | The shoebill is specific to open habitats with sparse or absent trees in the lowlands of tundra and taiga. It nests on cliffs, ravines or high riverbanks (rarely in trees), and in winter it forages in wide open areas: farmland, meadows and pastures. | It feeds mainly on small mammals (sometimes more than 80% of its food comes from lemmings and mice). Occasionally it also eats small birds, reptiles, insects or carcasses. Feeding habits are similar to those of other species, stalking prey from the air (hovering or hovering) or from a high perch. They often hunt directly on the ground. | No special requirements for connectivity. |
| Emberiza melanocephala (Black-headed barnacle) | - | - | The species nests in open, dry, scrubby and sparsely wooded areas, often on farmland with bushes between plots or scraped patches of trees, in orchards, and on heaths bordering farmland. | It has a wide range of food, eating seeds (millet, rice, wheat, maize) or other plant parts. During the breeding season it feeds its young on invertebrates: butterfly larvae, crickets, ants, flies, etc. | No special requirements for connectivity. |
| Lanius senator (Red-headed Shrike) | - | - | Nests in open habitats, mainly meadow/grassland, but also agricultural mosaics with isolated trees and scrub; sometimes also in orchards. Prefer nesting habitats classified as open woodland (or wooded meadows) in dry, sunny areas. | An almost exclusively insectivorous species, it consumes large insects (especially Coleoptera and Orthoptera). Occasionally consumes small reptiles (lizards) and very rarely small birds. | No special requirements for connectivity. |
| Merops apiaster (Prigorie) | - | - | Prefers open terrestrial habitats/agricultural land - nests in areas with sandy or loamy soils, with breaks or landslides, where the soil is exposed, relatively vertical, in which it can burrow. Also nests in high, muddy banks of low-lying rivers. | It is strictly insectivorous, consuming mainly species of the Hymenoptera family: bumblebees, wasps, bees | No special requirements for connectivity. |
| Miliaria calandra (Southern Mealy Borer) | - | - | It is characteristic of arid and cultivated lowlands, meadows and natural steppes. The nest is a hollow in the ground lined with straw and dried plant stems, over which a fine mat of grass is laid. Nests are generally sheltered under bushes or other plants | During the nesting season it feeds predominantly on insects and in winter on seeds and roots. Outside the nesting season it also feeds together with other species (southern southern prey). Monogamous and nests are solitary. | No special requirements for connectivity. |
| Motacilla alba (White motacilla) | - | - | The species nests in open and semi-open habitats such as: various wetlands, lakeshores and coastal areas, riparian areas, agricultural habitats, parks, gardens, man-made areas, etc. | It feeds mainly on terrestrial and aquatic invertebrates, including: insects and their larvae, spiders, snails, crustaceans, etc. | No special requirements for connectivity. |
| Oenanthe oenanthe (South Pietrar) | - | - | The species nests in open, stony areas with meadows, pastures, often also on uncultivated land, coastal meadows and stone-fenced farms, but also in low, gravelly coastal areas or on steep, earthy banks. In some ranges, the species also occurs in high, alpine areas. | Predominantly insectivorous species; feeds on larvae and adults of: beetles, lepidoptera, hymenoptera, grasshoppers, locusts, but also other invertebrates (spiders, earthworms, small snails, etc.). Occasionally they also eat small fruits (blackberry, blueberry, gooseberry, elderberry) especially in late summer/fall. | No special requirements for connectivity. |
| Streptopelia turtur (Turtur) | - | - | It nests in open deciduous forests in low lying areas with abundant understory near agricultural areas, clearings, shrub thickets or alignments with abundant understory. In wintering areas it occurs in semi-open areas as well as steppe and semi-desert; it also occupies habitats with olive, palm, acacia and scrub plantations. | It feeds mainly on plant food, especially seeds, which it gathers from the ground. Occasionally they also eat invertebrates: insects, pupae, worms and small snails. | No special requirements for connectivity. |
| Upupa epops (Kiss) | - | - | Foraging mainly in open and semi-open habitats, such as meadows/grasses with mature trees, orchards, tree lines, tree lines, hedgerows. It also occurs in areas of farmland with traditional agriculture (mosaics of small areas alternating with natural vegetation). | It is predominantly insectivorous, with large ground-dwelling species making up the majority of its diet (crickets, coropiscip, various beetles, butterfly larvae, etc.). It also consumes other invertebrate species that are present on or in the soil (worms), as well as small vertebrates (lizards, snakes, frogs). | No special requirements for connectivity. |
| Falco tinnununculus (Red-rumped Vulture) | - | - | It nests mainly in open habitats such as meadows/grassland or traditional agricultural mosaics with mature trees, orchards, orchards, heaths, heaths. May also nest in towns, parks. | Feeds mainly on rodents (but also reptiles, small birds or insects) | No special requirements for connectivity. |
| Anser albifrons (Great Tern) | - | - | Prefer open tundra or low-lying shrubland with marshy areas and open water holes in nesting areas. It nests both coastal and inland. In wintering areas it also prefers low, lowland areas rich in agricultural crops. | In nesting territories they feed on plant species from the Siberian tundra, and in wintering quarters in south-eastern Europe mainly on plant material from agricultural crops. At first they feed on grains (corn, wheat or other grasses) left over after harvest (when available) and later on the sprouting leaves of wheat, rape or other fall agricultural crops. | No special requirements for connectivity. |
| Galerida cristata (Woodlark) | - | - | It lives in open habitats, including those made by humans, such as railways or airports. | It is omnivorous, feeding on plants in winter and invertebrates in summer | No special requirements for connectivity. |
| Coturnix coturnix (Quail) | - | - | Forages in large open areas, such as meadows with few shrubs and farmland. It prefers plains or broad depressions with grassland, farmland or mosaic areas; also occurs in extensive agricultural monocultures | It is an opportunistic species; it mainly eats seeds of grasses, ruderal plants and cereals. It also feeds on invertebrates (worms, molluscs, ants, spiders, etc.), which it gathers from vegetation or can extract from the soil. | No special requirements for connectivity. |
| Regularly migratory species Species associated with riparian and coastal habitats |  |  |  |  |  |
| Gallinago gallinago (Common Snipe) | - | - | Prefer swampy and muddy terrain | It feeds on larvae in the soil, which it searches for with its long, straight beak, much like a nuthatch. | No special requirements for connectivity. |
| Hirundo rustica (Swallow) | - | - | It forages mainly in open, rural, man-made areas with patchy areas of agricultural habitats, pastures and meadows, which it uses intensively for foraging. They also often enter cities, especially in outlying areas. On migration, they use reed meadows as resting places. | An almost exclusively insectivorous species, it consumes mainly flying insects that it catches in flight. It also eats other invertebrates (spiders, other arthropods). Occasionally consumes seeds or small fruits in wintering quarters. | No special requirements for connectivity. |
| Regularly migratory species Species associated with urban habitats |  |  |  |  |  |
| Hirundo rustica (Swallow) | - | - | It forages mainly in open, rural, man-made areas with patchy areas of agricultural habitats, pastures and meadows, which it uses intensively for foraging. They also often enter cities, especially in outlying areas. On migration, they use reed meadows as resting places. | An almost exclusively insectivorous species, it consumes mainly flying insects that it catches in flight. It also eats other invertebrates (spiders, other arthropods). Occasionally consumes seeds or small fruits in wintering quarters. | No special requirements for connectivity. |
| Regularly migratory species Species associated with mixed terrestrial habitats forests and open lands |  |  |  |  |  |
| Accipiter nisus (Bird's-billed Woodlark) | - | - | It is found in woodland areas, but prefers to hunt in open spaces, such as heaths, parks and gardens near cities. | They hunt small birds and sometimes small mammals. | No special requirements for connectivity. |
| Buteo buteo (Common buzzard) | - | - | It forages in forested areas (mainly forests, but also plantations with larger forest areas), in areas where there is sufficient open space nearby (meadows, pastures, agricultural land), which it uses for feeding. | It feeds mainly on small mammals (but also reptiles, small birds and insects), which it hunts by flying at a fixed point several meters high. Occasionally they also eat carcasses, especially in winter. | No special requirements for connectivity. |
| Falco subbuteo (Sparrow hawk) | - | - | It nests in semi-open, forest-like habitats (steppe areas with sparsely or sparsely wooded or open woodland). It is found in meadows/grasslands or traditional agricultural mosaics with mature trees, small forests, and ditches. It also often occurs in large city parks. | It feeds mainly on large insects (mainly Orthoptera, such as crickets, grasshoppers, grasshoppers, grasshoppers, but also other species) and small birds, which it catches in flight. Occasionally it also eats other animals (lizards, small mammals). | No special requirements for connectivity. |
| Regularly migratory species Species associated with reed habitats |  |  |  |  |  |
| Motacilla flava (Yellow Codobat) | - | - | It prefers moist, low-vegetated habitats, such as grasslands, meadows, fens and reed marshes. Outside the nesting season it can also be found near farmland, preferring lakes, ponds or rivers. | It catches insect prey on the ground or at water level, but can also fly short distances in the air to catch them, often following herds of cows or sheep to feed on insects that settle on them. | No special requirements for connectivity. |
| Regularly migratory species Species associated with open aquatic habitats |  |  |  |  |  |
| Anser albifrons (Great Tern) | - | - | Prefer open tundra or low-lying shrubland with marshy areas and open water holes in nesting areas. It nests both coastal and inland. In wintering areas it also prefers low, lowland areas rich in agricultural crops. | In nesting territories they feed on plant species from the Siberian tundra, and in wintering quarters in south-eastern Europe mainly on plant material from agricultural crops. At first they feed on grains (corn, wheat or other grasses) left over after harvest (when available) and later on the sprouting leaves of wheat, rape or other fall agricultural crops. | No special requirements for connectivity. |
| Cygnus olor (Summer swan) | Possible watercourse: Albești cadastral code XV_1.12 | - | Open aquatic habitats - areas of marshes and lakes with reed surfaces where they nest. | Food consists mainly of aquatic plants (including submerged ones, which they reach using their long necks, but without diving). In addition, it eats grasses and agricultural plants (including seeds). Occasionally it may also consume animal food from aquatic areas (aquatic insects, worms, snails, tadpoles, etc.). | No special requirements for connectivity. |
| Anser anser (Summer Goose) | Possible watercourse: Albești cadastral code XV_1.12 | - | During the nesting season they prefer extensive wetlands associated with large rivers in lowland areas. The highest density is in the Danube Delta and lagoon system. During the wintering season, they prefer low, lowland areas, rich in fall agricultural crops or areas with natural grass vegetation. | The species is herbivorous, consuming a wide variety of plant matter: grasses, shoots, roots, etc. During the nesting period they feed mainly on plant material from agricultural crops, such as the leaves of wheat, rape or other fall crops. | No special requirements for connectivity. |
| Larus cachinnans ( Mediterranean Gull ) | - | - | It nests near lakes surrounded by reed beds, river islands. Outside the breeding season it is more common in coastal areas | Eat invertebrates, reptiles, small mammals | No special requirements for connectivity. |
| Regularly migratory species Species associated with forest habitats |  |  |  |  |  |
| Sylvia atricapilla (Black-headed Sylvia) | According to the Updated Management Plan of the Dobrogea Litoral Hydrographic Basin 2022-2027 the community habitats within ROSCI0157 depend on the groundwater aquifer RODL10 | Forest Habitats of Community Interest in ROSAC0071: 91AA, 91F0 , 91I0, 91M0 | It prefers forest habitats with a well-developed shrub layer. Occurs in deciduous and mixed forests, especially in scrub-rich heathland. May also nest in natural-looking parks or gardens with abundant undergrowth. | The species is omnivorous, but during the nesting season it is predominantly insectivorous (consuming other invertebrates such as worms, spiders, etc.). Outside the breeding season it is predominantly frugivorous, consuming small fruits, but also other plants (buds, seeds, pollen, nectar). | No special requirements for connectivity. |
| BG0000569 Kardam |  |  |  |  |  |
| 6110* White Alysso-Sedion calcareous or basophilous rocky grasslands | - | Potential habitat for Elaphe sauromates, Mesocricetus newtoni, Mustela eversmanii, Spermophilus citellus | Characteristic species: Allium montanum, Sedum acre, S. album, S. rupestre, S. rupestre, S. sexangulare, Scleranthus perennis, Rumex acetosella; mosses - Polytrichum piliferum, Ceratodon purpureus | Important for several species of invertebrates, thus also supporting other animal species that feed on them. | - |
| 6210* - Semi-natural xerophytic and facies xerophytic grasslands with bushes on calcareous substrata (Festuco-Brometalia) | - | Potential habitat for Elaphe sauromates, Mesocricetus newtoni, Mustela eversmanii, Spermophilus citellus | Grasslands are mesoxerophilous in character and are only considered to have priority status if they support orchid populations. In these meadows vegetation can reach a height of 20-100 cm with a substrate thickness of less than 1%. In this type of habitat there may also be shrubs in mosaic with the herbaceous carpet. The plant associations of this habitat type consist mostly of perennial species, the associations are at an advanced successional stage and the habitat is in climax. | Important for several species of invertebrates, and thus a support for other animal species that feed on them. | - |
| Vertigo moulinsiana (Coiled Snail) | - | It can also be found in habitat 6210* | It is mainly found in wetlands with calcareous substrate at low altitudes. | - | No special requirements for connectivity. |
| Vertigo angustior | - | Populates a wide range of open habitats: wet or marshy grasslands, calcareous stream banks, river or lake banks, marshes, fixed coastal dunes | It can be found in ecotonal zones between aquatic habitats and grassy meadow habitats. It prefers permanently wet soils, but without frequent flooding. | Food resource for herpetofauna and bird species | No special requirements for connectivity. |
| Unio crassus (Little river clam) | - | - | Populates rivers and lakes with clean water and sandy or sand and gravel substrate. It is sensitive to changes in the physico-chemical parameters of the water and in the species composition of ichthyofauna and to the occurrence of pollutants in the water. | - | No special requirements for connectivity. |
| Osmoderma eremita (Hermit Beetle) | - | - | It prefers deciduous forests from the steppe to the beech, being an indicator for deciduous ecosystems | - | No special requirements for connectivity. |
| Morimus funereus (Grey tailor) | - | - | It occurs in deciduous and mixed forests with medium deadwood densities. Higher abundances occur in beech and oak forests, especially in wetter areas | - | No special requirements for connectivity. |
| Lycaena dispar (Red Swamp Butterfly) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.1.1 | - | Prefer riparian habitats | The larvae develop on Rumex spp. (e.g. R. aquaticus, R. obtusifolius, R. crispus) and Polygonum bistorta. In the adult stage, it prefers species such as Mentha spp., Lythrum salicaria, Eupatorium cannabinum, Cirsium palustre, Origanum vulgare, Cirsium arvense, Valeriana officinalis, etc (Iorgu et. al. 2015, ). | No special requirements for connectivity. |
| Lucanus cervus (Red) | - | They mainly occupy deciduous forests at low and medium altitudes | A polyphagous species that thrives in decayed wood (below ground level) of many deciduous species, but prefers quercus (Iorgu et. al. 2015) | - | No special requirements for connectivity. |
| Hypodryas maturna (Rare Fritillary) | - | - | It is a species found on hardwoods along forest roads and in riparian forests. | - | No special requirements for connectivity. |
| Euplagia quadripunctaria (Striped butterfly) | - | - | Habitats are mesophytic, shrubby, light deciduous forests (with ecotonal areas with flowering plants, areas with abundant vegetation cliffs, shrubby edges and wooded areas) in temperate or Mediterranean climates. | - | No special requirements for connectivity. |
| Dioszeghyana schmidtii (Comb Owl Butterfly) | - | - | Its life cycle is linked to oak forests, especially Acer tataricum, but it can also be found in different types of xerophytic oak forests. | - | No special requirements for connectivity. |
| Cucujus cinnaberinus | - | - | It is associated with oak (Quercus spp.), maple (Acer spp.) and poplar (Populus spp.) species. It can be found in different habitat types, including forests and urban areas. | - | No special requirements for connectivity. |
| Cerambyx cerdo (Great oak tailor) | - | - | It prefers Quercus live wood in central Europe, but also other species such as Castanea, Juglans, Ceratonia in the southern regions of the continent. The dispersal capacity of the species is low, it does not exceed 500 meters from the place where they live. | Food resource for birdlife | No special requirements for connectivity. |
| Rhodeus sericeus amarus (Boarță) | - | - | It lives in many freshwater ponds, lakes and low-flow freshwater rivers, except in brackish and alpine lakes | - | No special requirements for connectivity. |
| Cobitis taenia (Cobitis taenia ) | - | - | It prefers gently flowing or stagnant waters whose facies consists of sand, clay and rarely gravel. It avoids aquatic ecosystems whose facies consists of mud. | Food source for Lutra lutra, Emys orbicularis, birds associated with aquatic habitats | No special requirements for connectivity. |
| Barbus meridionalis (Black-eared Murre) | - | - | It lives mainly in hilly (hill) and mountain rivers (especially in Transylvania and in the Bistrița basin), with clear, flowing and well oxygenated water, especially in low-flowing waters, together with trout and lipan. It has also been found on the southern slopes of the Carpathians and even in small hill streams in the Bucharest region. It can also live in flowing waters that dry up a lot during droughts | Food source for Lutra lutra, Emys orbicularis, birds associated with aquatic habitats | No special requirements for connectivity. |
| Sabanejewia aurata (Dunari) | - | - | It prefers flowing waters whose facies consists of sand and clay mixed with sand; the altitude of reophilous aquatic ecosystems does not condition its presence. A common habit/behavior is to burrow into the substrate/facies of the aquatic ecosystem. | - | No special requirements for connectivity. |
| Elaphe sauromates (Dobrogean dragon) | - | Habitats of Community Interest used by individuals of the species: 6110* and 6210* | It occupies a wide variety of natural habitats, including wooded steppes and even quasi-desert habitats. Shelters under rock piles, stone walls or in bushes. | The main source of food is birds, including eggs, but also many species of mammals (especially mice) and reptiles (lizards and even other snakes) | No special requirements for connectivity. |
| Bombina variegata (Yellow-bellied Pond Owl) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.0 | - | It occupies any body of water, mainly temporary pools, reproducing even in soil unevenness containing less than a liter of water - It is found almost everywhere where it finds a minimum of moisture, from 150 m to almost 2000 m altitude (Goriup, 2008). | The species feeds on invertebrates, and in turn may be a food source for some mammals, reptiles, birds | No special requirements for connectivity. |
| Emys orbicularis (European pond turtle) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.1.1 | - | It prefers marshes, ponds, ponds, lakes, smooth-flowing rivers) in lowland, hilly and hilly areas, with herbaceous and shrubby vegetation on the banks, aquatic vegetation and significant populations of fish and aquatic invertebrates. It is sensitive to water quality and is not found in polluted waters (Török, Ghira and Zamfirescu, 2013). | Feeds on invertebrates, fish, amphibians, plant matter | No special requirements for connectivity. |
| Mesocricetus newtoni (Romanian Hamster) | - | - | Avoid arable areas, usually galleries are found only on the outskirts of crops or in the unpopulated, unguarded, wooded areas in the immediate vicinity. | Food source for a wide range of predators. Role in dispersal of some seeds they lose during transportation. | No special requirements for connectivity. |
| Mustela eversmanii (Steppe fer ferret) | - | Habitat of Community interest that the species uses at site level: 6210 | They prefer arid and barren plains | Usually conditioned by the presence of the gopher Spermophilus citellus but also by other species of the families Microtidae and Muridae | No special requirements for connectivity. |
| Spermophilus citellus (Gopher) | - | - | The grasslands within the site may represent a favorable habitat for the species | It consumes plant species (seeds, leaves) in grassland habitats such as Trifolium campestre, T. arvense, T. repens, T. media, T. pratense, Medicago minima, Coronilla varia, Ononis spinosa, Plantago lanceolata, P. media, Pimpinella saxifrage, Festuca spp, Dactylis glomerata, Agropyron repens, Taraxacum sp., Achillea millefolium (Janák, , Marhoul, & Matějů, 2013). Action plan for the conservation of the European ground squirrel Spermophilus citellus in the European Union. European Commission.). In addition to plants, it also consumes large terrestrial atropods. | No special requirements for connectivity. |
| Triturus dobrogicus (Dobrogic crested newt) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.1.1 | - | It is an aquatic and terrestrial species. It prefers deeper, still or gently flowing water in open or wooded areas. In the terrestrial phase they are found at the edge of puddles hidden in reed lying on the ground and mixed with silt or in the litter of shrubs and undergrowth (Török et al, 2013). | Food source for bird species dependent on open aquatic habitats and reedbeds | No special requirements for connectivity. |
| Triturus karelinii (Crested newt) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.2 | - | The southern crested newts live in a variety of montane habitats, including deciduous and coniferous forests, slopes and plateaus. | - | No special requirements for connectivity. |
| Testudo graeca (Dobrogea land tortoise) | - | - | It is very well adapted to arid habitats, being specific to steppe areas, occurring in meadows as well as in forests or anthropized habitats. | They prefer vegetable matter, fruits; to a lesser extent, they consume animal matter such as invertebrates (insects, snails, slugs) and decaying matter (necrophagy, coprophagy), occasionally inorganic matter (geophagy - limestone, kaolin). | No special requirements for connectivity. |
| Testudo hermanni ( Land tortoise) | - | - | Prefers inland and coastal forest habitats. | They eat edible weeds and plants that make up their habitat. | No special requirements for connectivity. |
| Lynx lynx (Laughs) | - | It prefers large, rugged forest massifs and interspersed glades. | The main prey of the lynx is the roe deer, followed by rabbits, young deer, black goat and lesser wild boar | Depending on forest habitats, their connectivity is important. | No special requirements for connectivity. |
| Lutra lutra (Otter) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.2 | - | The species prefers the banks of flowing and stagnant water, its presence being an indicator of clean water, the species is sensitive to pollution. It has no preference for particular habitat types, living on the banks of lightly polluted waters in the immediate vicinity of the waterhole. | Feeds on fish (e.g. Cottus gobio, Phoxinus phoxinus) aquatic invertebrates, amphibians (e.g. Rana esculenta) | No special requirements for connectivity. |
| Vormela peregusna (Spotted ferret) | - | - | It also lives in open plains, bush and scrub fields, but also enters rare lowland woodlands | They hunt a range of animals, with small rodents, birds and reptiles predominating among the vertebrates, and also eat insects when necessary. | No special requirements for connectivity. |
| Rhinolophus mehelyi (Méhely's horseshoe bat) | - | Habitats of Community interest that the species uses at site level: 6110*, 6210* | Individuals of the species can be found in caves in karst areas at altitudes up to 700 m. Summer and winter roosts are almost exclusively in caves, sometimes in mines | It feeds mainly on moths, but also consumes mosquitoes and cockroaches as a trophic resource for predatory birds | No special requirements for connectivity. |
| Rhinolophus hipposideros (Little horseshoe bat) | - | - | Requires the presence of natural or man-made cavities for breeding (including crevices in cliffs or tree bark, generally hunts close to vegetation, at low to medium height, even in dense canopy, in mature deciduous or mixed forests and on their edges | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity, Moves 2-5 km from shelters to feeding areas |
| Rhinolophus ferrumequinum (Greater horseshoe bat) | - | - | It can frequently be found in limestone regions, near streams or standing water. It hibernates in caves, abandoned mining galleries and in May - October it can be found in the attics of houses, church towers | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Rhinolophus euryale (Mediterranean horseshoe bat) | - | - | They are usually found in warm, forested foothills and mountain regions. They prefer karst formations with numerous caves near water sources. They also prefer deciduous forests. They spend their summers roosting in caves and underground shelters. (Russo, et al., 2002) | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Rhinolophus blasii (Blasius' horseshoe bat) | - | - | A thermophilous species, it uses caves as a year-round roosting colony and foraging habitat in warm, open, calcareous areas with scrubs or sparse trees. They prefer warm caves with temperatures between 9 and 11°C for hibernation. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Miniopterus schreibersii (Long-winged Bat) | - | - | The species forages in more or less open habitats , even in peri-urban areas and the feeding flight is high (10- 20 m), prefers heavily wooded areas, often forages at great distances from the roost, about 30 km. Regionally migratory species, with distances of 400-600 km between maternity and wintering roosts. Colonies are mixed (mostly with common bat). Summer and winter roosts are underground (mostly caves), almost without exception. Colonies usually roost in caves throughout the year, but less often in mines or other underground roosts, Prefers caves with large entrances, in karst regions of hill and mountain areas, Solitary specimens or small groups may be found in a variety of roosts, in buildings, in bridge structures, Prefers heavily wooded areas, hunting under the forest canopy, but also over water surfaces, or close to vegetation, | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc. and is a trophic resource for nocturnal predatory birds | No special requirements for connectivity. |
| Myotis myotis (Common bat) | - | - | Foraging habitats are forest edges, groves and grasslands. Main roosts are caves, used all year round or only for hibernation.They hunt most frequently in mature deciduous or mixed forests, less frequently in coniferous forests with semi-open understory, taking a large proportion of prey directly from the ground. May travel significant distances (10-25 km) from roosts to feeding habitats. | Insectivorous species | No special requirements for connectivity. |
| Myotis emarginatus (Hollow-eared bat) | - | - | Summer colonies can be found in the attics of buildings, sometimes even in big cities, or in caves, They frequently form large colonies of hundreds of individuals, often together with other species, primarily Rhinolophus and Myotis myotis, Hibernate in caves, mines, cellars, solitary or in small groups, at relatively high temperatures (6-12°C). They hunt in deciduous forests, above tree pastures, scrub, avoiding open habitats. It flies close to vegetation and in the canopy, also catching prey on leaves. For foraging they will travel up to 10 km from roosts | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc. and is a trophic resource for nocturnal predatory birds | No special requirements for connectivity. |
| Myotis capaccinii (Long-legged bat) | - | - | This species prefers mild continental habitats and calcareous areas rich in caves. It may also occur in other habitats if underground sites are available. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc. and is a trophic resource for nocturnal predatory birds | No special requirements for connectivity. |
| Myotis blythii (Little common bat) | - | - | They prefer habitats with tall grasses and shrubs or natural grasslands in limestone areas. They frequent localities from sea level up to 1000 m altitude. Warm caves are the most sought-after roosts, and in the absence of caves, it uses the attics of dwellings and the hollows of trees | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Myotis bechsteinii (Big-eared bat) | - | - | It prefers mature deciduous forests (Fagus sp, Quercus spp, Capinus sp) with many old trees. May also occur in mixed or even coniferous forests if these are located near optimal habitats for the species. Breeding colonies of 10-30 females are located in frequently alternating hollows or, less frequently, in buildings. Individuals of the species feed in deciduous forests, flying about 1-3 km away from the barrens. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Barbastella barbastellus (Bar bar barbartelltellus) | - | - | Individuals can be seen in caves, rock crevices, rock crevices, hollows and under the bark of trees, but they also enter dwellings, seeking dark places such as pantries, cellars, cellars, attics. The sausage bat is a predominantly forest-dwelling species, widespread in the Carpathian Mountains, in the Eastern and Southern Carpathians and in south-western Romania up to 1100 m altitude. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| According to the European reporting data for the Water Directive the natural area overlaps with the following groundwater bodies BG1G000000N049, BG1G00000000J3K051, BG2G0000000000N044, BG2G00000PG026, BG2G000J3K1041, RODL04, RODL06, RODL10 As far as surface water bodies are concerned, the natural area is crossed by the river BG1DJ900R1015 Paran Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.1 |  |  |  |  |  |
| BG0000570 Izvorovo - Kraishte |  |  |  |  |  |
| 6110* White Alysso-Sedion calcareous or basifyllic rocky grasslands | - | Potential habitat for Elaphe sauromates, Mesocricetus newtoni, Mustela eversmanii, Spermophilus citellus | Characteristic species: Allium montanum, Sedum acre, S. album, S. rupestre, S. rupestre, S. sexangulare, Scleranthus perennis, Rumex acetosella; mosses - Polytrichum piliferum, Ceratodon purpureus | Important for several species of invertebrates, and thus a support for other animal species that feed on them. | - |
| 6210* - Semi-natural xerophytic and facies xerophytic grasslands with bushes on calcareous substrata (Festuco-Brometalia) | - | Potential habitat for Elaphe sauromates, Mesocricetus newtoni, Mustela eversmanii, Spermophilus citellus | The habitat includes arid and semi-arid grasslands with shrubs and scattered facies, which in the mountainous landscape change from calcareous to neutral substrates. The habitat is located on shallow, well-drained, primitive, poorly developed soils rich in calcium and poor in organic matter and nutrients - which have developed over time on various calcareous substrates. | Important for several species of invertebrates, and thus a support for other animal species that feed on them. | - |
| 62C0* Point-sarmatic stepping stones | - | Foraging habitat for bat species of community interest at the site. Habitat where the mammalian species of Community interest Spermophilus citellus, Vormela peregusna can be found. Important habitat for reptile species of Community interest: Elaphae sauromates, Testudo graeca, Testudo hermanni. | The habitat extends geographically throughout the steppe and forest-steppe area from north of the Black Sea to the Caspian Sea. On loess and alluvial deposits, on soils of chernozemic, castanozemic and phaeotizemic types, the most typical meadows of this habitat occur. | - | - |
| Echium russicum (Snake's head) |  | Usually found in habitats 6110*, 6210*, 62C0 | Suitable habitats are meadows and scrublands | Is a primary producer | No special requirements for connectivity. |
| Potentilla emilii-popii (Five-toed weed) | - | Usually found in habitat 62C0* | Acid, grassy places through scrublands, forest edges, calcareous soils | Is a primary producer | No special requirements for connectivity. |
| Vertigo moulinsiana (Coiled Snail) |  | It can also be found in habitat 6210* | It is mainly found in wetlands with calcareous substrate at low altitudes. | - | No special requirements for connectivity. |
| Vertigo angustior | - | Populates a wide range of open habitats: wet or marshy grasslands, calcareous stream banks, river or lake banks, marshes, fixed coastal dunes | It can be found in ecotonal zones between aquatic habitats and grassy meadow habitats. It prefers permanently wet soils, but without frequent flooding. | Food resource for herpetofauna and bird species | No special requirements for connectivity. |
| Unio crassus (Little river clam) | - | - | Populates rivers and lakes with clean water and sandy or sand and gravel substrate. It is sensitive to changes in the physico-chemical parameters of the water and in the species composition of ichthyofauna and to the occurrence of pollutants in the water. | - | No special requirements for connectivity. |
| Pseudophilotes bavius | - | Usually found in habitats 6110*, 6210*, 62C0 | The species occurs in small, isolated populations in dry, flower-rich meadows on dry, stony slopes, as well as in open patches in scrublands and in vineyards on calcareous soils. | - | No special requirements for connectivity. |
| Paracaloptenus caloptenoides (Devil Horse) | - | - | It prefers arid soils with small patches of vegetation, arranged in a mosaic of bare patches where the substrate is visible. Adults emerge late July through September. | - | No special requirements for connectivity. |
| Osmoderma eremita (Hermit beetle) | - | - | It prefers deciduous forests from the steppe to the beech, being an indicator for deciduous ecosystems | - | No special requirements for connectivity. |
| Ophiogomphus cecilia (Green Gomphida) | - | - | Specific to stenotope, preferring cold, clear, very clean, slow-flowing, very clean flowing waters with sandy substrate. | - | No special requirements for connectivity. |
| Morimus funereus (Grey tailor) | - | - | It occurs in deciduous and mixed forests with medium deadwood densities. Higher abundances occur in beech and oak forests, especially in wetter areas | - | No special requirements for connectivity. |
| Lycaena dispar (Red Swamp Butterfly) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.1.1 | - | Prefer riparian habitats | The larvae develop on Rumex spp. (e.g. R. aquaticus, R. obtusifolius, R. crispus) and Polygonum bistorta. In the adult stage, it prefers species such as Mentha spp., Lythrum salicaria, Eupatorium cannabinum, Cirsium palustre, Origanum vulgare, Cirsium arvense, Valeriana officinalis, etc (Iorgu et. al. 2015, ). | No special requirements for connectivity. |
| Lucanus cervus (Red) | - | They mainly occupy deciduous forests at low and medium altitudes | A polyphagous species that thrives in decayed wood (below ground level) of many deciduous species, but prefers quercus (Iorgu et. al. 2015) | - | No special requirements for connectivity. |
| Hypodryas maturna (Rare Fritillary) | - | - | It is a species found on hardwoods along forest roads and in riparian forests. | - | No special requirements for connectivity. |
| Euplagia quadripunctaria (Striped butterfly) | - | - | Habitats are mesophytic, shrubby, light deciduous forests (with ecotonal areas with flowering plants, areas with abundant vegetation cliffs, shrubby edges and wooded areas) in temperate or Mediterranean climates. | - | No special requirements for connectivity. |
| Dioszeghyana schmidtii | - | - | Its life cycle is linked to oak forests, especially Acer tataricum, but it can also be found in different types of xerophytic oak forests. | - | No special requirements for connectivity. |
| Cucujus cinnaberinus | - | - | It is associated with oak (Quercus spp.), maple (Acer spp.) and poplar (Populus spp.) species. It can be found in different habitat types, including forests and urban areas. | - | No special requirements for connectivity. |
| Coenagrion ornatum | - | - | The larvae live in flowing water, in mud-bottomed mudflats, the adults of this species prefer emergent vegetation, they live in the phytocenoses with Carex, Eleocharis on the banks of the water, between the leaves of which they can hide, and the females lay their eggs in the stems of these (endophytic oviposition), | - | No special requirements for connectivity. |
| Cerambyx cerdo (Great oak tailor) | - | - | It prefers Quercus live wood in central Europe, but also other species such as Castanea, Juglans, Ceratonia in the southern regions of the continent. The dispersal capacity of the species is low, it does not exceed 500 meters from the place where they live. | Food resource for birdlife | No special requirements for connectivity. |
| Rhodeus sericeus amarus (Boarță) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Ceair cadastral code XIV_1.40.1 | - | It lives in many freshwater ponds, lakes and low-flow freshwater rivers, except in brackish and alpine lakes | - | No special requirements for connectivity. |
| Cobitis taenia (Cobitis taenia ) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Ceair cadastral code XIV_1.40.1 | - | It prefers gently flowing or stagnant waters whose facies consists of sand, clay and rarely gravel. It avoids aquatic ecosystems whose facies consists of mud. | Food source for Lutra lutra, Emys orbicularis, birds associated with aquatic habitats | No special requirements for connectivity. |
| Barbus meridionalis (Black-eared Murre) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Ceair cadastral code XIV_1.40.1 | - | It lives mainly in hilly (hill) and mountain rivers (especially in Transylvania and in the Bistrița basin), with clear, flowing and well oxygenated water, especially in low-flowing waters, together with trout and lipan. It has also been found on the southern slopes of the Carpathians and even in small hill streams in the Bucharest region. It can also live in flowing waters that dry up a lot during droughts | Food source for Lutra lutra, Emys orbicularis, birds associated with aquatic habitats | No special requirements for connectivity. |
| Sabanejewia aurata (Dunari) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Ceair cadastral code XIV_1.40.1 | - | It prefers flowing waters whose facies consists of sand and clay mixed with sand; the altitude of reophilous aquatic ecosystems does not condition its presence. A common habit/behavior is to burrow into the substrate/facies of the aquatic ecosystem. | - | No special requirements for connectivity. |
| Canis lupus (Wolf) | - | - | The species does not have specific requirements for certain forest habitats, preferring areas that provide an abundant trophic base of both wild and domestic animals. | Plays an important role in regulating pop | Depending on forest habitats, their connectivity is important. |
| Bombina variegata (Yellow-bellied Pond Owl) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.0 | - | It occupies any body of water, mainly temporary pools, reproducing even in soil unevenness containing less than a liter of water - It is found almost everywhere where it finds a minimum of moisture, from 150 m to almost 2000 m altitude (Goriup, 2008). | The species feeds on invertebrates, and in turn may be a food source for some mammals, reptiles, birds | No special requirements for connectivity. |
| Elaphe sauromates (Dobrogean dragon) | - | Habitats of Community Interest used by individuals of the species: 6110* and 62C0* | It occupies a wide variety of natural habitats, including wooded steppes and even quasi-desert habitats. Shelters under rock piles, stone walls or in bushes. | The main source of food is birds, including eggs, but also many species of mammals (especially mice) and reptiles (lizards and even other snakes) | No special requirements for connectivity. |
| Emys orbicularis (European pond turtle) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Ceair cadastral code XIV_1.40.1 | - | It prefers marshes, ponds, ponds, lakes, smooth-flowing rivers) in lowland, hilly and hilly areas, with herbaceous and shrubby vegetation on the banks, aquatic vegetation and significant populations of fish and aquatic invertebrates. It is sensitive to water quality and is not found in polluted waters (Török, Ghira and Zamfirescu, 2013). | Feeds on invertebrates, fish, amphibians, plant matter | No special requirements for connectivity. |
| Mesocricetus newtoni (Romanian Hamster) | - | - | Avoid arable areas, usually galleries are found only on the outskirts of crops or in the unpopulated, unpopulated, wooded areas in the immediate vicinity. | Food source for a wide range of predators. Role in dispersal of some seeds they lose during transportation. | No special requirements for connectivity. |
| Mustela eversmanii (Steppe fer ferret) | - | Habitat of Community interest that the species uses at site level: 6210 | They prefer arid and barren plains | Usually conditioned by the presence of the gopher Spermophilus citellus but also by other species of the families Microtidae and Muridae | No special requirements for connectivity. |
| Lynx lynx (Laughs) | - | It prefers large, rugged forest massifs and interspersed glades. | The main prey of the lynx is the roe deer, followed by rabbits, young deer, black goat and lesser wild boar | Depending on forest habitats, their connectivity is important. | No special requirements for connectivity. |
| Lutra lutra (Otter) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.2 | - | The species prefers the banks of flowing and stagnant waters, its presence being an indicator of clean waters, the species is sensitive to pollution. It has no preference for particular habitat types, living on the banks of lightly polluted waters in the immediate vicinity of the waterhole. | Feeds on fish (e.g. Cottus gobio, Phoxinus phoxinus) aquatic invertebrates, amphibians (e.g. Rana esculenta) | No special requirements for connectivity. |
| Spermophilus citellus (Gopher) | - | - | The grasslands within the site may represent a favorable habitat for the species | It consumes plant species (seeds, leaves) in grassland habitats such as Trifolium campestre, T. arvense, T. repens, T. media, T. pratense, Medicago minima, Coronilla varia, Ononis spinosa, Plantago lanceolata, P. media, Pimpinella saxifrage, Festuca spp, Dactylis glomerata, Agropyron repens, Taraxacum sp., Achillea millefolium (Janák, , Marhoul, & Matějů, 2013). Action plan for the conservation of the European ground squirrel Spermophilus citellus in the European Union. European Commission.). In addition to plants, it also consumes large terrestrial atropods. | No special requirements for connectivity. |
| Vormela peregusna (Spotted ferret) | - | - | It also lives in open plains, bush and scrub fields, but also enters sparse, lowland forests | They hunt a range of animals, with small rodents, birds and reptiles predominating among the vertebrates; they also eat insects when necessary. | No special requirements for connectivity. |
| Testudo graeca (Dobrogea land tortoise) | - | - | It is very well adapted to arid habitats, being specific to steppe areas, occurring in meadows as well as in forests or anthropized habitats. | They prefer vegetable matter, fruits; to a lesser extent, they consume animal matter such as invertebrates (insects, snails, slugs) and decaying matter (necrophagy, coprophagy), occasionally inorganic matter (geophagy - limestone, kaolin). | No special requirements for connectivity. |
| Testudo hermanni ( Land tortoise) | - | - | Prefers inland and coastal forest habitats. | They eat edible weeds and plants that make up their habitat. | No special requirements for connectivity. |
| Triturus dobrogicus (Dobrogic crested newt) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.2 | - | It is an aquatic and terrestrial species. It prefers deeper, still or gently flowing water in open or wooded areas. In the terrestrial phase they are found at the edge of puddles hidden in reed lying on the ground and mixed with silt or in the litter of shrubs and undergrowth (Török et al, 2013). | Food source for bird species dependent on open aquatic habitats and reedbeds | No special requirements for connectivity. |
| Triturus karelinii (Crested newt) | River BG1DJ900R1015 Paran Dere Dere DJRWB1015 which ensures the continuity of the watercourse on the territory of Romania, namely Cerchez cadastral code XIV_1.40.1.2 | - | The southern crested newts live in a variety of montane habitats, including deciduous and coniferous forests, slopes and plateaus. | - | No special requirements for connectivity. |
| Rhinolophus mehelyi (Méhely's horseshoe bat) | - | Habitats of Community interest that the species uses at site level: 6110*, 62C0* | Individuals of the species can be found in caves in karst areas at altitudes up to 700 m. Summer and winter roosts are almost exclusively in caves, sometimes in mines | They feed mainly on moths, but also eat mosquitoes and cockroaches | No special requirements for connectivity. |
| Rhinolophus hipposideros (Little horseshoe bat) | - | - | Requires the presence of natural or man-made cavities for breeding (including crevices in cliffs or tree bark, generally hunts close to vegetation, at low to medium height, even in dense canopy, in mature deciduous or mixed forests and on their edges | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity, Moves 2-5 km from shelters to feeding areas |
| Rhinolophus ferrumequinum (Greater horseshoe bat) | - | - | It can frequently be found in limestone regions, near streams or standing water. It hibernates in caves, abandoned mining galleries and from May to October it can be found in the attics of houses, church towers | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Rhinolophus euryale (Mediterranean horseshoe bat) | - | - | They are usually found in warm, forested foothills and mountain regions. They prefer karst formations with numerous caves near water sources. They also prefer deciduous forests. They spend their summers roosting in caves and underground shelters. (Russo, et al., 2002) | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Rhinolophus blasii (Blasius' horseshoe bat) | - | - | A thermophilous species, it uses caves as a year-round roosting colony and foraging habitat in warm, open, calcareous areas with scrubs or sparse trees. They prefer warm caves with temperatures between 9 and 11°C for hibernation. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Miniopterus schreibersii (Long-winged Bat) | - | - | The species forages in more or less open habitats , even in peri-urban areas and the feeding flight is high (10- 20 m), prefers heavily wooded areas, often forages at great distances from the roost, about 30 km. Regionally migratory species, with distances of 400-600 km between maternity and wintering roosts. Colonies are mixed (mostly with common bat). Summer and winter roosts are underground (mostly caves), almost without exception. Colonies usually roost in caves all year round, but less often in mines or other underground roosts, Prefers caves with large entrances, in karst regions of hill and mountain areas, Solitary specimens or small groups may be found in a variety of roosts, in buildings, in bridge structures, Prefers heavily wooded areas, hunting under the forest canopy, but also over water surfaces, or close to vegetation, | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc. and is a trophic resource for nocturnal predatory birds | No special requirements for connectivity. |
| Myotis myotis (Common bat) | - | - | Foraging habitats are forest edges, groves and grasslands. Main roosts are caves, used all year round or only for hibernation.They hunt most frequently in mature deciduous or mixed forests, less frequently in coniferous forests with semi-open understory, taking a large proportion of prey directly from the ground. May travel significant distances (10-25 km) from roosts to feeding habitats. | Insectivorous species | No special requirements for connectivity. |
| Myotis emarginatus (Hollow-eared bat) | - | - | Summer colonies can be found in the attics of buildings, sometimes even in big cities, or in caves, They frequently form large colonies of hundreds of individuals, often together with other species, primarily Rhinolophus and Myotis myotis, Hibernate in caves, mines, cellars, solitary or in small groups, at relatively high temperatures (6-12°C). They hunt in deciduous forests, above tree pastures, scrub, avoiding open habitats. It flies close to vegetation and in the canopy, also catching prey on leaves. For foraging they will travel up to 10 km from roosts | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc. and is a trophic resource for nocturnal predatory birds | No special requirements for connectivity. |
| Myotis capaccinii (Long-legged bat) | - | - | This species prefers mild continental habitats and calcareous areas rich in caves. It may also occur in other habitats if underground sites are available. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc. and is a trophic resource for nocturnal predatory birds | No special requirements for connectivity. |
| Myotis blythii (Little common bat) | - | - | They prefer habitats with tall grasses and shrubs or natural grasslands in limestone areas. They frequent localities from sea level up to 1000 m altitude. Warm caves are the most sought-after roosts, and in the absence of caves, it uses the attics of dwellings and the hollows of trees | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Myotis bechsteinii (Big-eared bat) | - | - | It prefers mature deciduous forests (Fagus sp, Quercus spp, Capinus sp) with many old trees. May also occur in mixed or even coniferous forests if these are located near optimal habitats for the species. Breeding colonies of 10-30 females are located in frequently alternating hollows or, less frequently, in buildings. Individuals of the species feed in deciduous forests, flying about 1-3 km away from the barrens. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |
| Barbastella barbastellus (Bar bar barbartelltellus) | - | - | Individuals can be seen in caves, rock crevices, rock crevices, hollows and under the bark of trees, but they also enter dwellings, seeking dark places such as pantries, cellars, cellars, attics. The sausage bat is a predominantly forest-dwelling species, widespread in the Carpathian Mountains, in the Eastern and Southern Carpathians and in south-western Romania up to 1100 m altitude. | It feeds mainly on lepidoptera, dipterans, hymenoptera, coleoptera, etc., being a trophic resource for predatory birds | No special requirements for connectivity. |

Figure 2-17  Species and habitat functional relationships ROSI0071/ROSAC0071

Figure 2-18  Functional relationships between species and habitats ROSI0157/ROSAC0157

Figure 2-19  Functional relationships between species and habitats ROSPA0094

Figure 2-20  Functional relationships between species and habitats ROSPA0036

Figure 2-21  Functional relationships between species and habitats ROSPA0036

Figure 2-22  Functional relationships between species and habitats BG0000569

Figure 2-23  Functional relationships between species and habitats BG000000570

## ANPIC's conservation objectives

The conservation objectives for species and habitats in potentially affected Natura 2000 sites are presented in the annex CSO Analysis, attached, part of this study . The objectives have been set by the National Agency for Protected Natural Areas through Decisions and Notes presented in the Documents section, attached to this study. For the 2 protected natural areas on the territory of Bulgaria, namely BG0000569 Kardam and BG0000570 Izvorovo - Kraishte, the data used in the present Study are those provided by the Bulgarian Ministry of Environment and Forests, following correspondence (see address no. 99-00-243-1 dated 30.05.2024 and address no. 99-00-243-3 dated 13.06.2024)

Below is the current status (presence/absence) of the specific conservation objectives and management plans for each area analyzed.

Table  2-6 Status of specific conservation objectives and management plans of the protected natural areas analyzed in the study

| ANPIC code and name | Intersected | Conservation objectives | Management Plan |
| --- | --- | --- | --- |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | No | From | Management plan approved by Order of the Minister of Environment, Water and Forests no. 1557/2016 |
| ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii | No | From | Management plan approved by Order of the Minister of Environment, Water and Forests no. 1480/2016 |
| ROSPA0036 Dumbrăveni | No | From | Management plan approved by Order of the Minister of Environment, Water and Forests no. 1557/2016 |
| ROSPA0166 Plopeni-Chirnogeni | No | From | - |
| ROSPA0094 Hagieni Forest | No | From | Management Plan, approved by Order of the Minister of Environment, Water and Forests No 1480/2016 |
| BG0000569 Kardam | No | No | - |
| BG0000570 Izvorovo - Kraishte | No | No | - |

## Analysis of the conservation measures in the management plan/ ANPIC regulation that may limit/ influence the interventions and activities proposed by the PP

The following table presents the conservation/ management measures of the analyzed protected natural areas that may limit/ influence the interventions and activities proposed by the PP.

Table  2-7 Analysis of the conservation measures in the management plan/ ANPIC regulation that may limit/ influence the interventions and activities proposed by the PP

| ANPIC code and name | Connected (Yes/No) | Restrictive measures in PM/regulatory act/administrative act |
| --- | --- | --- |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | No | The management plan, being common with other protected natural areas in the area (ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Balta Vederoasa which overlap with the boundaries of ROSCI0071) establishes restrictive measures for bird species, not to locate wind farms at a distance of less than 3 km from the boundary of the sites. Site ROSCI0071 has not been declared for the protection of bird species |
| ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii | No | The management plan is common with that of ROSPA0094. The plan establishes restrictions for bird species. The site has been declared for the projection of bat, invertebrate mammal and reptile species |
| ROSPA0036 Dumbrăveni | No | The management plan, being common with other protected natural areas in the area (ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Balta Vederoasa which overlap with the boundaries of ROSCI0071) establishes restrictive measures to ensure the conservation objective for bird species. The restrictive measures are; no wind farms within 3 km of the site boundary. |
| ROSPA0166 Plopeni-Chirnogeni | No | - |
| ROSPA0094 Hagieni Forest | No | Restrictive measures to reduce disturbance to bird species on the site. For new siting applications the following set of restrictions will be introduced: a) no constructions of any kind shall be placed on the meadow, b) no new construction shall be located within 400 meters of forests or wetlands - lakes, marshes, streams, and the like. Buildings may only be located on arable land, subject to the requirements of point b). |
| BG0000569 Kardam | No | - |
| BG0000570 Izvorovo - Kraishte | No | - |

## Other information relevant to ANPIC conservation, including possible changes in its natural evolution

Analyzing the information publicly available so far as well as the trends in the natural evolution of protected natural areas in Romania, no possible changes or other relevant data on the conservation status of the sites analyzed in this study were identified. However, it should be noted that, in view of the upward trend in climate change, a possible change in the ecology of the species identified in each area analyzed should be considered. At present it is not possible to predict this change, but ongoing monitoring of biodiversity (both fauna and flora) will confirm or refute this.

# Presenting the results of field activities

Field work to identify the aspects related to biodiversity components in the location area of the "Wind Energy Park 48 (46) wind power plants approx. 316.8MW (303.6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads", were carried out in the period from April 2022 to March 2023. A detail of the monitoring program conducted for the proposed plan is provided, includes field activities over a 12-month period, capturing a complete biological cycle.

The field data were collected by an independent team (specialized on the taxonomic components plants and habitats, amphibians and reptiles, birds, mammals, chiroptera and invertebrates), including a total of 24 field trips.

The field activities covered have extended beyond the direct area of influence of the project (see species and habitat distribution maps based on field data).

The following table presents the results of the field activities and how the identified uncertainties have been addressed.

Table  3-1 Results of field activities

| Uncertainty identified |  | Proposed approach | Aspects analyzed | Clarifying uncertainties | The uncertainty has been clarified (Yes/No/Partial) |
| --- | --- | --- | --- | --- | --- |
| The presence, distribution and activity of the species is unknown |  |  |  |  |  |
| Accipiter gentilis | Uliu porumbar | Field trips during the optimal study period with application of three monitoring methods (transects, fixed points, monitoring cameras) | Presence of the species | Species observed in winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, but may use the area as feeding habitat | YES |
|  |  |  | Activity of the species | Within a year, two individuals were reported | YES |
| Accipiter nisus | Uliu bird |  | Presence of the species | Species observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, but may use the area as feeding habitat | YES |
|  |  |  | Activity of the species | In one year, five individuals have been reported | YES |
| Alauda arvensis | Field woodlark |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, but may use the area as foraging habitat | YES |
|  |  |  | Activity of the species | 560 individuals have been reported in one year | YES |
| Anthus campestris | Fieldfare |  | Presence of the species | Species observed in summer | YES |
|  |  |  | Species distribution | The species nests on the site | YES |
|  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
| Apus apus | Black Drepnea |  | Presence of the species | Species observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species nests on the site but has also been observed transiting and migrating | YES |
|  |  |  | Activity of the species | In one year, 90 individuals have been reported | YES |
| Pomarina eagle | Little screaming eagle |  | Presence of the species | Species observed in spring - summer - fall | YES |
|  |  |  | Species distribution | The species does not nest on the site but has also been observed transiting and migrating | YES |
|  |  |  | Activity of the species | In one year, 19 individuals have been reported | YES |
| Buteo buteo | Shared bus |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the surface of the site as feeding habitat | YES |
|  |  |  | Activity of the species | In one year 162 individuals have been reported | YES |
| Buteo rufinus | Big earcarrier |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the surface of the site as feeding habitat | YES |
|  |  |  | Activity of the species | In the space of a year, 240 cases were reported | YES |
| Caprimulgus europaeus | Caprimulg |  | Presence of the species | The species was sighted in the summer season | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 1 individual was reported | YES |
| Carduelis carduelis | Sticlete |  | Presence of the species | Species observed in summer - fall - winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 100 individuals have been reported | YES |
| Carduelis chloris | Florinte |  | Presence of the species | Species observed in summer | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 46 individuals have been reported | YES |
| Ciconia ciconia | White Stork |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, but uses the area for migration | YES |
|  |  |  | Activity of the species | 93 individuals have been reported in one year | YES |
| Ciconia nigra | Black stork |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | Within a year, an individual was reported | YES |
| Circaetus gallicus | Saw |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | Within a year, an individual was reported | YES |
| Circus aeruginosus | Reed reed |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 355 individuals have been reported | YES |
| Circus cyaneus | Erete vânăt |  | Presence of the species | The species was observed in winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | 162 individuals have been reported in one year | YES |
| Circus pygargus | Southern Erete |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 8 individuals have been reported | YES |
| Columba palumbus | Collared pigeon |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
| Coracias garrulus | Dumbrăveancă |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
| Corvus corax | Corbul |  | Presence of the species | The species was observed in spring | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | Within a year, an individual was reported | YES |
| Corvus frugilegus | Seed crow |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 28 individuals have been reported | YES |
| Coturnix coturnix | Quail |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Cuculus canorus | Cuc |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
| Delichon urbica | Homemade locust |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, but uses the surface of the site for migration or feeding | YES |
|  |  |  | Activity of the species | In one year, 50 individuals have been reported | YES |
| Emberiza calandra | Depressure |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 200 individuals have been reported | YES |
| Emberiza hortulana | Garden Presley |  | Presence of the species | The species was observed in spring | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
| Erithacus rubecula | Măcăleandru |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
| Falco columbarius | Winter Falcon |  | Presence of the species | The species was observed in the fall period | YES |
|  |  |  | Species distribution | The species does not nest on the site, using the surface area for feeding | YES |
|  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
| Subbuteo falcon | Sparrow hawk |  | Presence of the species | The species has been observed in summer and fall | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
| Falco tinnununculus | Red fannet |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, but uses the area for transit, migration and feeding | YES |
|  |  |  | Activity of the species | 675 individuals were reported in one year | YES |
| Fringilla coelebs | Finches |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Galerida cristata | Woodpecker |  | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
| Gallinago gallinago | Common grant |  | Presence of the species | The species was observed in winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
| Garrulus glandarius | Skip |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Hieraaetus pennatus | Little eagle |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
| Hirundo rustica | Swallow |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, but uses the area for transit, migration and feeding | YES |
|  |  |  | Activity of the species | In one year 219 individuals have been reported | YES |
| Lanius collurio | Reddish red spurge |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 58 individuals have been reported | YES |
| Lanius minor | Black-headed spurge |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
| Larus cachinnans | Pond Gull |  | Presence of the species | The species has been observed in spring - summer - fall | YES |
|  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  | Activity of the species | In one year 88 individuals have been reported | YES |
| Luscinia luscinia | The watcher of the wattle |  | Presence of the species | The species was observed in spring | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
| Luscinia megarhynchos | Red-headed nightwatcher |  | Presence of the species | The species was observed in spring | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Melanocorypha calandra | Bărăgan woodlark |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 55 individuals have been reported | YES |
| Merops apiaster | Prison |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  | Activity of the species | In one year 147 individuals have been reported | YES |
| White motacilla | White codobat |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, using the area for transit, migration and feeding | YES |
|  |  |  | Activity of the species | In one year 57 individuals have been reported | YES |
| Motacilla flava | Yellow codobat |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  | Activity of the species | 233 individuals have been reported in one year | YES |
| Oenanthe oenanthe | South Pietrar |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 15 individuals have been reported | YES |
| Parus major | Big duck |  | Presence of the species | The species was observed in winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
| Passer domesticus | Homemade barley |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 70 individuals have been reported | YES |
| Perdix perdix | Partridge |  | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  | Species distribution | The species nests on the site | YES |
|  |  |  | Activity of the species | In one year, 30 individuals have been reported | YES |
| Pernis apivorus | Viespar |  | Presence of the species | The species was observed in the summer - fall period | YES |
|  |  |  | Species distribution | The species does not nest on the site, but uses the area for migration | YES |
|  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
| Phasianus colchicus | Fazan |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 12 individuals have been reported | YES |
| Phoenicurus phoenicurus | Forest cod |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Pica pica | Coțofană |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species nests on the site but also uses the area for transit | YES |
|  |  |  | Activity of the species | In one year, 69 individuals have been reported | YES |
| Riparia riparia | Shore grass |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
| Saxicola rubetra | Large bramble |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
| Streptoptopelia decaocto | Guguștiuc |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 39 individuals have been reported | YES |
| Streptopelia turtur | Turturică |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 5 individuals have been reported | YES |
| Sturnus vulgaris | Common starling |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 992 individuals have been reported | YES |
| Sylvia atricapilla | Black-headed Silvie |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
| Turdus merula | Blackbird |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 18 individuals have been reported | YES |
| Turdus philomelos | Singing Sturgeon |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 7 individuals have been reported | YES |
| Turdus pilaris | Cock |  | Presence of the species | The species was observed in winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 38 individuals have been reported | YES |
| Upupa epops | Kiss |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  | Activity of the species | In one year, 51 individuals have been reported | YES |
| Apodemus agrarius | Striped field mouse |  | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 19 individuals have been reported | YES |
| Apodemus sylvaticus | Forest mouse |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
| Bufo viridis | Green toad |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species uses wetlands for breeding | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Canis aureus | Jackal |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Capreolus capreolus | Deer |  | Presence of the species | The species was observed during the summer-winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 14 individuals have been reported | YES |
| Crocidura leucodon | Field Thistle |  | Presence of the species | The species was observed during the summer-winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
| Dolicophis caspius | Evil snake |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
| Erinaceus roumanicus | Eastern hedge hedge hedgehog |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Lacerta viridis | Gușter |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 11 individuals have been reported | YES |
| Lepus europaeus | Wild rabbit |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
| Meles meles | Bursuc/Viezure |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 15 individuals have been reported | YES |
| Mustela eversmanii | Steppe fer ferret |  | Presence of the species | The species was observed during the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 1 individual was reported | YES |
| Podarcis muralis | Wall lizard |  | Presence of the species | The species was observed during the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
| Podarcis taurica | Steppe Lizard |  | Presence of the species | The species was observed during the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 14 individuals have been reported | YES |
| Rattus norvegicus | Brown rat |  | Presence of the species | The species was observed during the winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
| Spalax leucodon | Small blind |  | Presence of the species | The species was observed during the winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 7 individuals have been reported | YES |
| Spermophilus citellus | Gopher |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species uses the site area for nesting, feeding and transit | YES |
|  |  |  | Activity of the species | In one year, 39 individuals have been reported | YES |
| European mole | Mole |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species, uses the site area for foraging and transit and there are also mounds within the site | YES |
|  |  |  | Activity of the species | In one year, 8 individuals have been reported | YES |
| Testudo graeca | Dobrogean land tortoise |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 6 individuals have been reported | YES |
| Vulpes vulpes | Fox |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 55 individuals have been reported | YES |
| Eptesicus serotinus | Late Lilac |  | Presence of the species | The species was detected between May and October | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year 88 contacts have been registered | YES |
| Miniopterus schreibersii | Long-winged bat |  | Presence of the species | The species was detected between May and October | YES |
|  |  |  | Species distribution | The species, uses the area of the site for transit and feeding | YES |
|  |  |  | Activity of the species | In one year 85 contacts have been registered | YES |
| Myotis daubentonii | Water bat |  | Presence of the species | The species was detected between May and October | YES |
|  |  |  | Species distribution | The species, uses the area of the site for transit and feeding | YES |
|  |  |  | Activity of the species | 52 contacts were registered in one year | YES |
| Myotis mystacinus/brandtii | Mustached bat |  | Presence of the species | The species was detected in May and September | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year, 2 contacts have been registered | YES |
| Nyctalus leisleri | Small dusk bat |  | Presence of the species | The species was detected between May and November | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year 193 contacts have been registered | YES |
| Nyctalus noctula | Twilight lilac |  | Presence of the species | The species was detected between May and November | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year, 284 contacts were registered | YES |
| Pipistrellus nathusii/kuhlii | Nathusius's dwarf bat |  | Presence of the species | The species was detected between May and November | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year 2798 contacts were registered | YES |
| Pipistrellus pipistrellus | Dwarf bat |  | Presence of the species | The species was detected between May and November | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year 125 contacts have been registered | YES |
| Pipistrellus pygmaeus | Pygmy bat |  | Presence of the species | The species was detected between May and August | YES |
|  |  |  | Species distribution | The species, uses the area of the site for transit and feeding | YES |
|  |  |  | Activity of the species | 45 contacts were registered in one year | YES |
| Plecotus auritus | Eared bat |  | Presence of the species | The species was detected between May and August | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year, 26 contacts have been registered | YES |
| Vespertilio murinus | Liliac bicolor |  | Presence of the species | The species was detected between May and July | YES |
|  |  |  | Species distribution | The species, uses the site area for transit and feeding | YES |
|  |  |  | Activity of the species | In one year, 7 contacts were registered | YES |

In order to identify as accurately as possible the flora and fauna species in the plan implementation area, field activities were carried out over a 12-month period, starting in spring 2022, thus capturing a complete biological cycle. A breakdown of the monitoring program conducted for the proposed plan can be found in the table below.

Table  3-2 Methodological details biodiversity monitoring Cerchezu wind farm

| Biological groups | Moon monitoring |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  | Calendar month |  |  |  |  |  |  |  |  |  |  |  |
|  | 2022 |  |  |  |  |  |  |  |  | 2023 |  |  |
|  | April | May | June | July | August | September | October | November | December | January | February | March |
| Plants and habitats | 12 hours | 12 hours | 14 hours | 16 hours | 16 hours | 14 hours | 12 hours | - | - | - | - | 12 hours |
| Nevertebrate - Visual transects | - | 12 hours | 14 hours | 16 hours | - | 14 hours | 12 hours | - | - | - | - | - |
| Reptiles and amphibians - Visual transects | 12 hours | 12 hours | 14 hours | 16 hours | 16 hours | - | - | - | - | - | - | - |
| Mammals - Transect | 12 hours | 12 hours | 14 hours | 16 hours | 16 hours | 14 hours | 12 hours | 12 hours | 10 hours | 10 hours | 10 hours | 12 hours |
| Mammals - Monitoring rooms | ongoing | ongoing | ongoing | ongoing | ongoing | ongoing | ongoing | ongoing | ongoing | ongoing | ongoing | ongoing |
| Birds - Transects | 12 hours | 12 hours | 14 hours | 16 hours | 16 hours | 4 hours | 4 hours | 4 hours | 10 hours | 10 hours | 10 hours | 4 hours |
| Birds - Fixed points | - | - | - | - | - | 10 hours | 10 hours | 10 hours | - | - | - | 12 hours |
| Bats - Transects | 5 hours | 5 hours | 6 hours | 6 hours | 6 hours | 6 hours | 6 hours | - | - | - | - | - |
| Bats - Fixed points in transects | 10 points - 10 minutes / point | 10 points - 10 minutes / point | 10 points - 10 minutes / point | 10 points - 10 minutes / point | 10 points - 10 minutes / point | 10 points - 10 minutes / point | 10 points - 10 minutes / point | - | - | - | - | - |
| Bats - static detectors | 5 nights | 5 nights | 5 nights | 5 nights | 5 nights | 5 nights | 5 nights | - | - | - | - | - |

The results of the monitoring campaigns are described below.

## Plants and habitats

### Description of the working method

In order to analyze the habitats in the study area, a land-use category analysis was initially conducted to identify areas of land with natural or semi-natural land use regimes. The Corine Land Cover 2018 database, available at www.copernicus.eu, was used for this analysis. The study area in terms of habitats was established by intersecting the level three administrative-territorial units (municipalities) with the total proposed wind turbines. It was found that all the wind turbines will be located in the administrative territory of Cerchezu Commune, in areas in agricultural use, without intersecting grassland areas.

### Results achieved

It was found that the largest share at the level of the Cerchezu TAU is represented by agricultural areas, occupying about 88% of the total. A smaller share is represented by meadows (about 4.5%) and settlements (about 5%). The smallest shares are held by industrial or commercial areas (less than 1%) and agricultural land with significant areas of natural vegetation (about 1.4%).

Table  3-3 Land use regime in the study area and important habitat categories

| Category CLC level 1 | Category CLC level 2 | Category CLC level 3 | Area (ha) | Share (%) |
| --- | --- | --- | --- | --- |
| Artificial surfaces | Urban fabric | Municipalities - Continuous built-up areas | 425.38 | 5.14 |
|  | Industrial, commercial and transportation establishments | Industrial or commercial zones | 31.17 | 0.373 |
| Agricultural area | Meadows | Meadows | 371.35 | 4.49 |
|  | Heterogeneous agricultural areas | Largely agricultural land with significant areas of natural vegetation | 117.21 | 1.41 |
|  | Agricultural area | Non-irrigated arable land | 7322.09 | 88.56 |

The area of grassland south of Cerchezu

Predominantly herbaceous vegetation, with sparse scrubland, visibly affected by grazing, as it is surrounded by agricultural land. Due to this limitation the vegetation is overgrazed, with less than 100% ground cover. The vegetation consists of predominantly herbaceous, native species, with Asteraceae and Poaceae being the families with the most representatives:

Artemisia austriaca, Artemisia absinthium, Botriochloa ischaemum, Marrubium peregrinum, Hordeum murinum, Xeranthemum annuum, Daucus carota, Chondrilla juncaea, Euphorbia glareosa nicaeensis, Eryngium campestre, Achillea setacea, Agropyron cristatum, Cichorium intybus, Plantago lanceolata, Sisymbrium orientale, Teucrium polium subsp. capitatum, Salvia aethiopis, Linaria genistifolia, Sideritis montana, Tunica prolifera, Bupleurum commutatum subsp. glaucocarpum, Festuca valesiaca, Poa bulbosa, Cynodon dactylon (herbaceous base; shows slight salinity of the soils, specific), Aegylops cylindrica, Galium humifusum, Asperula tenella, Bromus cf. tectorum, Allium rotundum, Potentilla sp., Vulpia myuros, Trifolium sp., Medicago lupulina, Sambucus ebulus, Carduus acanthoides, Cannabis ruderalis, Prunus spinosa, Verbascum thapsus, Urtica dioica, Torilis nodosa, Onopordum acanthium, Rosa canina, Consolida regalis, Centaurea diffusa, Cephalaria transsilvanica, Elymus repens, Lolium perenne, Convolvulus arvensis.

The landscape is typically Dobrogean, emphasizing the limestone deposits of the canara type, at the level of which can also be found rare species, typically Dobrogean, calciphilous. The meadow is also bordered and partly covered by a plantation of acacia (Robinia pseudoacacia, an invasive allogenic species).

The distribution of vegetation is heterogeneous, alternating areas with abundant vegetation with specimens in good state of conservation with areas with sparse vegetation, with reduced size and cover and affected by the presence of ruderal and some segetal species.

The area of grassland north of Viroaga

Characterized by herbaceous species, dominated by species of no conservation value, with a predominance of taxa of the Poaceae family:

Carduus acanthoides (monodominate, close, over large areas), Lolium perenne, Festuca valesiaca, Poa angustifolia, Achillea setacea, Papaver dubium, Daucus carota, Tunica prolifera, Berteroa incana, Plantago lanceolata, Vulpia myuros, Bromus cf. tectorum, Stellaria media, Centaurea diffusa, Reseda lutea, Cannabis ruderalis, Artemisia austriaca, Marrubium peregrinum, Onopordom acanthium, Galium humifusum, Galum verum, Stachys germanica, Medicago falcata, Aegylops cilindrica, Potentilla argentea, Bromus tectorum (predominant species over large areas), Phragmites australis, Poa prratensis (predominant species over large areas), Bupleurum commutatum subsp. glaucocarpum.

The plant associations that characterize the plant communities in the analyzed areas are Cynodonti-Poëtum angustifoliae Rapaics ex Soó 1957 and Artemsio austriacae-Poëtum bulbosae Pop 1970, which form the steppe grassland vegetation. Cynodonti-Poëtum angustifoliae is a secondary association, its composition being heterogeneous according to location, type and degree of use, and thus "enriched" with numerous ruderal elements, species resistant to these pressures. Today, located between agricultural land, these areas form islets and areas where sheepfolds have been established. The vegetation, however, still preserves some elements of Dobrogean steppe vegetation such as Teucrium polium subsp. capitatum, Festuca valesiaca, Sideritis montana, Asperula tenella, Allium rotundum, etc. Ruderalization is conferred by the degradation of the original communities, the reduction of ground cover and the presence of species that form true scrub such as Carduus acanthoides, Onopordon acanthium, Urtica dioica, Sambucus ebulus, Cannabis ruderalis, etc. The two associations belong to the habitat of the national classification (Doniță et al. 2005) R3420 West-pontic meadows with Poa bulbosa, Artemisia austriaca, Cynodon dactylon and Poa angustifolia (land covered, before anthropization, with vegetation specific to the Natura 2000 priority habitat 62C0* Ponto-sarmatic steppes).

The habitat R8702 Anthropogenic communities with Onopordum acanthium, Carduus nutans and Centaurea calcitrapa (Doniță et al. 2005) occurs in areas where Carduus and Onopordon species are highly distributed, with 70-80% cover, where they form thickets of scrub that prevent the development of natural native vegetation. However, it is a source of food in the late summer and fall season for migratory and sedentary species of the order Passeriformes.

The area of grassland to the west of Căscioarele

The analyzed grassland is covered predominantly by herbaceous vegetation, with rare scrubs, heavily affected by grazing, being bordered by agricultural land and inhabited areas. The vegetation is mosaic, alternating between grazed and overgrazed areas, with plant communities of steppe characteristic grasses (e.g. Stipa capillata, Chrysopogon gryllus). The vegetation consists of predominantly native species, with Asteraceae and Poaceae being the families with the most representatives:

Berteroa incana, Plantago lanceolata, Galium humifusum, Achillea setacea, Torilis arvensis, Allium rotundum, Eryngium campestre, Daucus carota, Lolium perenne, Dicanthium ischaemum (predominant species on large areas), Cynodon dactylon (predominant species on large areas), Tunica prolifera, Festuca valesiaca, Bromus cf. tectorum (predominant species on large areas), Sisymbrium orientale, Poa bulbosa, Centaurea diffusa, Artemisia austriaca, Nigella arvensis, Consolida regalis, Xeranthemum annuum, Chondrilla juncaea, Euphorbia glareosa nicaeensis, Asperula tenella, Dactylis glomerata, Crataegus monogyna, Cephalaria transsilvnica, Cichorium intybus, Stipa capillata, Marrubium peregrinum, Chaiturus marrubiastrum, Oenothera sp., Echiunm vulgare, Heliotropium suaveolens, Sanguisorba minor, Malva neglecta, Sedum sp., Agropyron cristatum, Chrysopogon gryllus, Carduus acanthoides (predominant species over large areas), Onopordum acanthium (predominant species over large areas), C. lanceolatum, Verbascum thapsus, V. nigrum, Sambucus ebulus, Urtica dioica, Xanthium spinosum (invasive allogamous species, characteristic of crevice areas).

Typical Dobrogean landscape with wide open areas, covered with herbaceous vegetation with xerophytic elements resistant to the periods of lack of precipitation in summer, installed secondary to the steppe vegetation, and limestone deposits of the canara type, at the level of which rare calciferous species usually grow. Woody species are rare (e.g. Crataegus monogyna).

The distribution of vegetation is heterogeneous, alternating areas with abundant vegetation and areas with sparse vegetation, with reduced size and cover and affected by the presence of ruderal species (e.g. Carduus acanthoides, C. lanceolatum, Verbascum thapsus, Sambucus ebulus, Urtica dioica, Xanthium spinosum), and some segetal (e.g. Consolida regalis).

The grassland is a mosaic of plant communities, among the most prominent are the plant associations Cynodonti-Poëtum angustifoliae Rapaics ex Soó 1957, Artemsio austriacae-Poëtum bulbosae Pop 1970 and Botriochloetum ischaemi (Kristinsen 1937) Pop 1977, which form the secondary steppe grassland vegetation. All three associations represent secondary successions of vegetation, the phytokenotic composition being quite heterogeneous according to location, soil type, exposure, type and degree of use, and thus being "enriched" with numerous species denoting ruderalization. The vegetation still preserves, however, some elements of Dobrogean steppe vegetation such as Chrysopogon gryllus, Stipa capillata, Festuca valesiaca, Eryngium campestre, Plantago lanceolata, Sanguisorba minor, Asperula tenella, Allium rotundum, etc. Ruderalization is conferred by the degradation of the original communities, the reduction of ground cover and the presence of species that form veritable thickets such as Carduus acanthoides, Onopordon acanthium, Urtica dioica, Sambucus ebulus, etc. The first two associations belong to the habitat of the national classification (Doniță et al. 2005) R3420 West-pontic grasslands with Poa bulbosa, Artemisia austriaca, Cynodon dactylon and Poa angustifolia, and the association Botriochloetum ischaemi defines the habitat R3415 Ponto-balcanic grasslands of Botriochloa ischaemum and Festuca valesiaca, both habitats with a compositional structure strongly affected by grazing (land probably covered, prior to anthropization, by vegetation characteristic of Natura 2000 habitat 62C0* Ponto-sarmatic Steppes).

Plant communities with Chrysopogon gryllus (ass. veg. Thymio pannonici-Chrysopogonetum grylli Doniță et al. 1992, syn.: Chrysopogonetum grylli Soó), with limited distribution on the limestone slopes bordering the flat area of the meadow, obviously degraded by grazing.

The limestone rocks of the canara type still preserve elements of vegetation characterized by Agropyron cristatum, Satureja coerulea, Thymus sp., these apparently unattractive areas for domestic animals are affected by the passage of animals, vectors of dispersal and for non-native plant species, such as Xanthium spinosum - invasive non-native species in our country, characteristic of the târlite areas

The meadow is crossed by a drainage channel mostly devoid of vegetation, with meso-hygrophilous species such as Chaiturus marrubiastrum and some non-native species such as Oenothera sp. (a taxon of South American origin) in certain areas where excess moisture is maintained.

Figure 3-1 Grassland vegetation aspects of the analyzed areas

An obvious effect of the degree of utilization and degradation of the grassland is the presence of the habitat R8702 Anthropogenic communities with Onopordum acanthium, Carduus nutans and Centaurea calcitrapa (Doniță et al. 2005). It is defined by the areas where Carduus and Onopordon species have a high distribution (observed especially near localities), with a high cover of 70-80%, forming thickets (scrublands) that prevent the development of natural native vegetation. It is a source of food at the end of the summer and in the fall season for species of the order Passeriformes, migratory or sedentary.

Figure 3-2 Aspects of areas affected by grazing, where ruderal species predominate and invasive alien species (e.g. Xanthium spinosum) are growing in places

Although the main use of the grassland is as an island, it is a refuge for numerous species of wildlife, some of which are protected species (e.g. Spermophilus citellus), due to the plant species that constitute the trophic base, but also due to its role as a breeding habitat. The grassland vegetation is clearly affected by grazing, the land is highly ruderalized due to its island function, which makes its conservation value low, the presence of alien taxa contributing to this condition.

The area of grassland east of Căscioarele (not identified by CLC2018)

This area of grassland is used as sheep pasture, with a sheepfold identified nearby. The vegetation in this area of grassland is predominantly segetal and ruderal (with rare elements of xerophytic stepic, spontaneous xerophytic vegetation), consisting of the following 41 taxa. Spring annuals are replaced by summer season vegetation. The highest share is held by representatives of the families Poaceae (grasses), Fabaceae (legumes) and Caryophyllaceae (carnation family):

| Achillea setacea | Trifolium repens | Cynodon dactylon |
| --- | --- | --- |
| Poa bulbosa | Marrubium peregrinum | Veronica Verna |
| Plantago lanceolata | Poa pratense | Holosteum umbellatum |
| Cerastium semidecandrum | Anchusa sp. | Filago arvensis |
| Stellaria media | Scleranthus annuus | Chenopodium album |
| Galium humifusum | Androsace elongata | Geranium sp. |
| Erodium cicutarium | Xeranthemum annuum | Silver cinquefoil |
| Bromus tectorum | Hordeum vulgare | Centaurea sp. |
| Bromus hordeaceus | Lepidium draba | Matricaria sp. |
| Medicago minima | Veronica polita | Thymus sp. |
| Lathyrus tuberosus | Malva neglecta | Alyssum desertorum |
| Taraxacum officinale | Carduus nutans | Verbascum thapsus |
| Eryngium campestre | Sysimbrium loeselii | Daucus carota |
| Chondrilla juncacea | Draba verna |  |

The vegetation forms a grassland, but this is a grazed grassland, a grassland that has been established secondarily on former steppe-specific vegetation habitats that have been disturbed by anthropogenic factors in the past (most probably areas in agricultural use). Often this type of vegetation can also be found on the margins of roadsides or agricultural crops, better understood. Some species such as Achillea setacea, Thymus sp., Medicago minima, Veronica verna, Centaurea sp., Xeranthemum annuum, etc. are species indicating secondary grassland quality, some of which are found in the phytokenotic composition of the xerophytic vegetation.

The grassland is a large, extensive area, covered mainly with Poa pratensis, a mesophilic species, indicating sandy-loamy soils, probably with an excess of water in the vernal period. The vegetation of this meadow can be classified as Poëtum pratensis Răvăruț et al. 1956.

There is a lack of non-native species (at least so far), which can be considered a positive aspect, but given the proximity of agricultural land, the presence of non-native species is not to be excluded.

Figure 3-3 Vegetation in the area plan

### Distribution map

The distribution by CLC2018 classification of land use categories is shown in the map below:

Figure 3-4 Location of land use categories

### Pressures identified, other important issues

During the field activities, in areas with natural or semi-natural vegetation, i.e. grasslands and lands mostly occupied by agriculture, with significant areas of natural vegetation, the presence of some forms of activities that represent pressures on plant communities, i.e. grazing and the presence of roads that favor the expansion of non-native species, was reported.

Figure 3-5 Photos relevant to habitat pressures

## Mammals

### Description of the working method

For the monitoring of mammal activity in the study area a mixed methods approach was chosen, i.e. by applying the transect method and the camera-trap method.

During the monitoring period, transects were conducted on the area of the future Cerchezu wind farm and all signs of presence (element that may indicate the presence of an individual of the target species, such as tracks, scats, droppings, etc.) of mammal species were noted. This method was also applied in the southern sector of the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake , which borders the area of the plan.

The camera trap method has started to be applied inside the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Lake Vederoasa, but in the next period it will be extended inside the site plan.

### Results achieved

During the monitoring period, in the studied area (by which we mean both the location of the plan and the southern extremity of the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake) the presence of 14 mammal species was established, as listed in the table below.

Table  3-4 Land use regime in the study area and important habitat categories

| Species | Species (ro) | No. of reported ind. | Comments activity species | IUCN* | Habitats Directive* | Priority species | GEO 57/2007* | Red Book of Romanian Vertebrates |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apodemus agrarius | Striped field mouse | 19 | The species has been observed in summer and winter | LC | - | No | - | - |
| Apodemus sylvaticus | Forest mouse | 1 | The species does not roost on the site, it uses the site area for transit | LC | - | No | - | - |
| Canis aureus | Jackal | 26 | Species, uses the site area for transit | NT | - | No | Annex 5A | Vulnerable species |
| Capreolus capreolus | Deer | 14 | The species has been observed in the summer-winter period, using the site for transit | LC | - | No | Annex 5B | Vulnerable species |
| Crocidura leucodon | Field Thistle | 2 | The species does not roost on the site, it uses the site area for transit | LC | - | No | - | Vulnerable species |
| Erinaceus roumanicus | Eastern hedge hedge hedgehog | 6 | The species was observed in the spring-summer period Species, uses the site area for transit | LC | - | No | - | - |
| Lepus europaeus | Wild rabbit | 26 | The species was observed throughout the year Species, uses the site area for transit | LC | - | No | Annex 5B | - |
| Meles meles | Bursuc/Viezure | 15 | The species was observed throughout the year Species, uses the site area for transit | LC | - | No | Annex 5B | - |
| Mustela eversmanii | Steppe fer ferret | 1 | The species was observed throughout the year Species, uses the site area for transit | EN | Annex II, Annex IV | No | Annex 3, Annex 4A | Vulnerable species |
| Rattus norvegicus | Brown rat | 1 | The species was observed during the winter period Species, uses the site area for transit | LC | - | No | - | - |
| Spalax leucodon | Small blind | 9 | The species was observed during the winter period Species, uses the site area for transit | LC | - | No | Annex 4B | - |
| Spermophilus citellus | Gopher | 39 | The species uses the site area for nesting, feeding and transit | VU | Annex II, Annex IV | No | Annex 3, Annex 4A | Vulnerable species |
| European mole | Mole | 8 | The species was observed in the spring-summer period The species, uses the site area for foraging and transit and there are also mounds within the site | LC | - | No | - | - |
| Vulpes vulpes | Fox | 55 | The species was observed throughout the year Species, uses the site area for transit | LC | - | No | Annex 5B | - |

IUCN: LC - Least Concern; VU - Vulnerable; NT - Near Threatened, EN - Endangered.

Habitats Directive: Annex II - animal and plant species of Community interest whose conservation requires the designation of special areas of conservation; Annex IV - animal and plant species of Community importance requiring strict protection;

OUG57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna: Annex 3 - Species of plants and animals whose conservation requires the designation of special areas of conservation and special avifauna protection areas; Annex 4A - Species of Community interest. Plant and animal species requiring strict protection; Annex 5A - Species of Community interest. Species of plants and animals of Community interest, with the exception of bird species, whose taking from the wild and exploitation are subject to management measures; Annex 5B - Species of animals of national interest whose taking from the wild and exploitation are subject to management measures.

### Distribution map

The distribution of occurrence locations for mammal species is shown in the map below.

Figure 3-6 Distribution of occurrence locations for mammal species

| Jackal (Canis aureus) in the wind farm site area | Red deer (Capreolus capreolus) in the area of Natura 2000 site ROSCI0071 |
| --- | --- |
| Fox (Vulpes vulpes) in the area of Natura ROSCI0071 site | Motion-sensor triggered photo-video camera in the wind farm site area |
| Gopher (Spermophilus citellus) in the wind farm site area | Small spurge (Spalax leucodon) in the area of the wind farm site |
| Bad badger (Meles meles) den in the wind farm site area | Steppe ferret (Mustela eversmanii) victim of road traffic in the wind farm site area |
| Bad Badger (Meles meles) in ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | Fox (Vulpes vulpes vulpes) in the wind farm site area |
| Hed hedge hedgehog (Erinaceus roumanicus) in the area of the wind farm site | Wild hare (Lepus europaeus) in the wind farm site area |

Figure 3-7 Distribution of occurrence locations for mammal species

### Pressures identified, other important issues

The following pressures on mammal species were reported during fieldwork:

- Road presence;

- The presence of companion dogs at the sheepfold;

- Presence of roaming cats;

- Presence of roaming dogs.

### How to address identified uncertainties

For species in potentially affected protected areas in the site area, data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the boundaries of the PUZ. Thus it was conventionally assumed that any species or habitat may have distribution over the entire area of the protected natural area (worst case scenario).

In order to resolve uncertainties related to the presence of the species, the results of the monitoring for the nearby Chirnogeni Wind Farm were also analyzed.

The following table presents the results of the field activities and how the identified uncertainties have been addressed.

Table  3-5  Results of mammalian field activities

| Uncertainty identified |  | Proposed approach | Aspects analyzed | Clarifying uncertainties | The uncertainty has been clarified (Yes/No/Partial) |
| --- | --- | --- | --- | --- | --- |
| The presence, distribution and activity of the species is unknown |  |  |  |  |  |
| Apodemus agrarius | Striped field mouse | Field trips during the optimal survey period, transect method, traps and video cameras | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  | Species distribution | The species does not roost on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 19 individuals have been reported | YES |
| Apodemus sylvaticus | Forest mouse |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | The species does not roost on the site, it uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 1 individual was reported | YES |
| Bufo viridis | Green toad |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species uses wetlands for breeding | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Canis aureus | Jackal |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Capreolus capreolus | Deer |  | Presence of the species | The species was observed during the summer-winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 14 individuals have been reported | YES |
| Crocidura leucodon | Field Thistle |  | Presence of the species | The species was observed during the summer-winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 1 individual was reported | YES |
| Erinaceus roumanicus | Eastern hedge hedge hedgehog |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
| Lepus europaeus | Wild rabbit |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
| Meles meles | Bursuc/Viezure |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 15 individuals have been reported | YES |
| Mustela eversmanii | Steppe fer ferret |  | Presence of the species | The species was observed during the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 1 individual was reported | YES |
| Rattus norvegicus | Brown rat |  | Presence of the species | The species was observed during the winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year 1 individual was reported | YES |
| Spalax leucodon | Small blind |  | Presence of the species | The species was observed during the winter period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 7 individuals have been reported | YES |
| Spermophilus citellus | Gopher |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | The species uses the site area for nesting, feeding and transit | YES |
|  |  |  | Activity of the species | In one year, 39 individuals have been reported | YES |
| European mole | Mole |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species, uses the site area for foraging and transit and there are also mounds within the site | YES |
|  |  |  | Activity of the species | In one year, 8 individuals have been reported | YES |
| Vulpes vulpes | Fox |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 55 individuals have been reported | YES |

During the monitoring the climatic conditions were normal for the seasonal period covered by the field trips, although negative temperatures (around -1°C/-2°C) were recorded, considered as minimum temperatures for the monitoring activity, no atypical activities were observed in the behavior of the species, the activity being normal.

## Amphibians and reptiles

### Description of the working method

During the monitoring period, transects were made on the area of the future Cerchezu wind farm and all signs of presence (visual observation of individuals, regardless of their stage of development: instar, larva or adult or their state: alive, dead) of amphibian and reptile species were noted. This method was also applied in the southern sector of the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, which borders the area plan.

Table  3-6  Amphibian and reptile species identified in the study area

| Species | Species (ro) | No. of reported ind. | IUCN* | Habitats Directive* | Priority species | GEO 57/2007* | Red Book of Romanian Vertebrates |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bufo viridis | Green toad | 4 | LC | Annex IV | No | Annex 4A | Near Threatened |
| Dolicophis caspius | Evil snake | 3 | LC | Annex IV | No | Annex 4A, Annex 4B | Vulnerable species |
| Lacerta viridis | Gușter | 11 | LC | Annex IV | No | Annex 4A | - |
| Podarcis muralis | Wall lizard | 2 | LC | Annex IV | No | Annex 4A | Vulnerable species |
| Podarcis taurica | Steppe Lizard | 14 | LC | Annex IV | No | Annex 4A | Near Threatened species |
| Testudo graeca | Dobrogean land tortoise | 6 | VU | Annex II, Annex IV | No | Annex 3, Annex 4A | Endangered species |

IUCN: LC - Least Concern; VU - Vulnerable;

Habitats Directive: Annex II - animal and plant species of Community interest whose conservation requires the designation of special areas of conservation; Annex IV - animal and plant species of Community importance requiring strict protection;

OUG57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna: Annex 3 - Species of plants and animals whose conservation requires the designation of special areas of conservation and special avifauna protection areas; Annex 4A - Species of Community interest. Plant and animal species requiring strict protection; Annex 4B - Species of national interest. Animal and plant species requiring strict protection.

### Results achieved

During the monitoring period, in the studied area (by which we mean both the location of the plan and the southern extremity of the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake), the presence of one amphibian species and five reptile species, as listed in the table below, was established.

.

### Distribution map

The distribution of occurrence locations for amphibian and reptile species is shown in the map below

Figure 3-8 Distribution of occurrence locations for amphibian species

Figure 3-9 Distribution of occurrence locations for reptile species

| Green toad larvae (Bufo viridis) in a temporary pond on the farm road | Green toad (Bufo viridis) victim of road traffic, in the area of Natura 2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake |
| --- | --- |
| Dobrogean land tortoise (Testudo graeca) observed inside ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | Guster (Lacerta viridis) observed in the wind farm site area |
| Bad snake (Dolicophis caspius) victim of road traffic in the wind farm site area | Wall lizard (Podarcis muralis) observed in the wind farm area |
| Steppe lizard (Podarcis taurica) observed in the wind farm area |  |

Figure 3-10 Mammal-relevant photos

### Pressures identified, other important issues

The following pressures on mammal species were reported during fieldwork:

- Road presence;

- Presence of companion dogs in the herd

- Presence of roaming dogs

- Presence of roaming cats.

### How to address identified uncertainties

According to the Presentation Memorandum, the following uncertainty was found for the location of habitats and species:

-	The exact distribution of the species and habitats of Community interest within the protected natural areas is not known and flight routes are not known for all species on the site.

In order to resolve the uncertainties related to the presence of species, the results of the monitoring for the nearby Chirnogeni Wind Farm were analyzed

Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the PUZ boundaries. Thus, it was conventionally considered that any species or habitat may have distribution over the entire area of the protected natural area (the worst case scenario).

The following table presents the results of the field activities and how the identified uncertainties have been addressed.

Table  3-7  Results of field activities clarifying uncertainties

| Uncertainty identified |  | Proposed approach | Aspects analyzed | Clarifying uncertainties | The uncertainty has been clarified (Yes/No/Partial) |
| --- | --- | --- | --- | --- | --- |
| The presence, distribution and activity of the species is unknown |  |  |  |  |  |
| Bufo viridis | Green toad | Field trips during the optimal study period with application of three monitoring methods (transects, fixed points) | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | The species uses wetlands for breeding | YES |
|  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  |  |  | Activity of the species | In one year 1 individual was reported | YES |
| Dolicophis caspius | Evil snake |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
| Podarcis muralis | Wall lizard |  | Presence of the species | The species was observed during the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
| Podarcis taurica | Steppe Lizard |  | Presence of the species | The species was observed during the summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 14 individuals have been reported | YES |
|  |  |  | Species distribution | The species, uses the site area for foraging and transit and there are also mounds within the site | YES |
|  |  |  | Activity of the species | In one year, 8 individuals have been reported | YES |
| Testudo graeca | Dobrogean land tortoise |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  | Species distribution | Species, uses the site area for transit | YES |
|  |  |  | Activity of the species | In one year, 6 individuals have been reported | YES |
|  |  |  | Species distribution | The species, uses the area of the site for transit and feeding | YES |
|  |  |  | Activity of the species | In one year, 7 contacts have been registered | YES |

In addition, as mentioned in the presentation memorandum for potentially affected protected area species in the site area, data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the boundaries of the PUZ. Thus, it was conventionally considered that any species or habitat may have distribution over the entire area of the protected natural area (the worst case scenario, applying the precautionary principle).

## Birdlife

### Description of the working method

During the monitoring period, transects were made on the area of the future Cerchezu wind farm and all signs of presence (visual observation of individuals, auditory signaling) of bird species were noted. This method was also applied in the southern sector of the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, which borders the area plan.

### Results achieved

During the monitoring period, in the studied area (by which we mean both the location plan and the southern extremity of the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake), the presence of 63 bird species (totaling almost 3800 individuals) was established, as listed in the table below.

Table  3-8 Bird species identified in the study area

| No. crt. | Species | Species | No. of reported ind. | IUCN* | Birds Directive* | GEO 57/2007* | Red Book of Vertebrates | Bern Convention* | Bonn Convention* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Accipiter gentilis | Uliu porumbar | 2 | LC | - | - | - | - | - |
| 2 | Accipiter nisus | Uliu bird | 5 | LC | - | - | - | - | - |
| 3 | Alauda arvensis | Field woodlark | 550 | LC | Annex II B | Annex 5C | - | - | - |
| 4 | Anthus campestris | Fieldfare | 10 | LC | Annex I | Annex 3 | - | - | - |
| 5 | Apus apus | Black Drepnea | 99 | LC | - | - | - | - | - |
| 6 | Pomarina eagle | Little screaming eagle | 19 | LC | Annex I | Annex 3 | Vulnerable species | - | - |
| 7 | Buteo buteo | Shared bus | 162 | LC | - | - | - | - | - |
| 8 | Buteo rufinus | Largeorecar | 19 | LC | Annex I | Annex 3 | Vulnerable species | - | - |
| 9 | Caprimulgus europaeus | Caprimulg | 1 | LC | Annex I | Annex 3 | - | - | - |
| 10 | Carduelis carduelis | Sticlete | 100 | LC | - | Annex 4B | - | Annex II | - |
| 11 | Carduelis chloris | Florinte | 46 | LC | - | Annex 4B | - | Annex II | - |
| 12 | Ciconia ciconia | White Stork | 93 | LC | Annex I | Annex 3 | Vulnerable species | - | Annex II |
| 13 | Ciconia nigra | Black stork | 1 | LC | Annex I | Annex 3 | Vulnerable species | - | Annex II |
| 14 | Circaetus gallicus | Saw | 1 | LC | Annex I | Annex 3 | Vulnerable species | - | - |
| 15 | Circus aeruginosus | Reed reed | 37 | LC | Annex I | Annex 3 | - | - | - |
| 16 | Circus cyaneus | Erete vânăt | 7 | LC | Annex I | Annex 3 | - | - | - |
| 17 | Circus pygargus | Southern Erete | 8 | LC | Annex I | Annex 3 | Endangered species | - | - |
| 18 | Columba palumbus | Collared pigeon | 2 | LC | Annex II A, Annex III A | Annex 5C | - | Annex III | - |
| 19 | Coracias garrulus | Dumbrăveancă | 26 | LC | Annex I | Annex 3 | - | Annex II | Annex II |
| 20 | Corvus corax | Corbul | 1 | LC | - | Annex 4B | Endangered species | - | - |
| 21 | Corvus frugilegus | Seed crow | 28 | LC | Annex II B | Annex 5C | - | Annex III | - |
| 22 | Coturnix coturnix | Quail | 4 | LC | Annex II B | Annex 5C | - | - | Annex II |
| 23 | Cuculus canorus | Cuc | 6 | LC | - | - | - | - | - |
| 24 | Delichon urbica | Homemade locust | 50 | LC | - | - | - | - | - |
| 25 | Emberiza calandra | Depressure | 200 | LC | - | Annex 4B | - | - | - |
| 26 | Emberiza hortulana | Garden Presley | 3 | LC | Annex I | Annex 3 | - | - | - |
| 27 | Erithacus rubecula | Măcăleandru | 3 | LC | - | Annex 4B | - | Annex II | - |
| 28 | Falco columbarius | Winter Falcon | 2 | LC | - | - | - | - | - |
| 29 | Subbuteo falcon | Sparrow hawk | 2 | LC | - | Annex 4B | - | - | Annex II |
| 30 | Falco tinnununculus | Red fannet | 55 | LC | - | Annex 4B | - | - | Annex II |
| 31 | Fringilla coelebs | Finches | 4 | LC | - | - | - | - | - |
| 32 | Galerida cristata | Woodpecker | 26 | LC | - | - | - | - | - |
| 33 | Gallinago gallinago | Common grant | 1 | LC | Annex II A, Annex III B | Annex 5C, Annex 5E | - | - | - |
| 34 | Garrulus glandarius | Skip | 4 | LC | Annex II B | Annex 5C | - | Annex III | - |
| 35 | Hieraaetus pennatus | Little eagle | 2 | LC | Annex I | Annex 3 | Critically endangered species | - | - |
| 36 | Hirundo rustica | Swallow | 219 | LC | - | - | - | - | - |
| 37 | Lanius collurio | Reddish red spurge | 58 | LC | Annex I | Annex 3 | - | - | - |
| 38 | Lanius minor | Black-headed spurge | 3 | LC | Annex I | Annex 3 | - | - | - |
| 39 | Larus cachinnans | Pond Gull | 88 | LC | - | - | - | - | - |
| 40 | Luscinia luscinia | The watcher of the wattle | 1 | LC | - | - | - | - | - |
| 41 | Luscinia megarhynchos | Red-haired nightwatcher | 4 | LC | - | - | - | - | - |
| 42 | Melanocorypha calandra | Bărăgan woodlark | 55 | VU | Annex I | Annex 3 | - | Annex II | - |
| 43 | Merops apiaster | Prison | 147 | LC | - | Annex 4B | - | Annex II | Annex II |
| 44 | White motacilla | White codobat | 56 | LC | - | Annex 4B | - | - | - |
| 45 | Motacilla flava | Yellow codobat | 233 | LC | - | Annex 4B | - | - | - |
| 46 | Oenanthe oenanthe | South Pietrar | 15 | LC | - | - | - | Annex II | - |
| 47 | Parus major | Big duck | 3 | LC | - | - | - | - | - |
| 48 | Passer domesticus | Homemade barley | 70 | LC | - | - | - | Annex III | - |
| 49 | Perdix perdix | Partridge | 30 | LC | Annex II A, Annex III A | Annex 5C | - | - | - |
| 50 | Pernis apivorus | Viespar | 2 | LC | Annex I | Annex 3 | Vulnerable species | - | - |
| 51 | Phasianus colchicus | Fazan | 12 | LC | Annex II A, Annex III A | Annex 5C | - | - | - |
| 52 | Phoenicurus phoenicurus | Forest cod | 4 | LC | - | Annex 4B | - | Annex II | - |
| 53 | Pica pica | Coțofană | 69 | LC | Annex II B | Annex 5C | - | Annex III | - |
| 54 | Riparia riparia | Shore grass | 1 | LC | - | - | - | - | - |
| 55 | Saxicola rubetra | Large bramble | 10 | LC | - | - | - | Annex II | - |
| 56 | Streptoptopelia decaocto | Guguștiuc | 29 | LC | Annex II B | Annex 5C | - | - | - |
| 57 | Streptopelia turtur | Turturică | 5 | NT | Annex II B | Annex 5C | Vulnerable species | - | - |
| 58 | Sturnus vulgaris | Common starling | 982 | LC | Annex II B | Annex 5C | - | Annex III | - |
| 59 | Sylvia atricapilla | Black-headed Silvie | 3 | LC | - | - | - | - | - |
| 60 | Turdus merula | Blackbird | 18 | LC | - | - | - | - | - |
| 61 | Turdus philomelos | Singing Sturgeon | 7 | LC | Annex II B | Annex 5C | - | - | - |
| 62 | Turdus pilaris | Cock | 38 | VU | Annex II B | Annex 5C | - | - | - |
| 63 | Upupa epops | Kiss | 51 | LC | - | Annex 4B | Vulnerable species | - | - |

IUCN: LC - Least Concern; NT - Near Threatened; VU - Vulnerable;

Birds Directive: Annex I - Species subject to special measures for the conservation of their habitats in order to ensure their survival and reproduction in their range; Annex II A - Species which may be hunted in the geographical sea and land area where this Directive applies; Annex II B - Species which may be hunted only in certain Member States indicated in the Directive; Annex II B - Species which may be hunted only in certain Member States indicated in the Directive; Annex III A - Species for which the sale, transport for sale, keeping for sale and offering for sale live or dead birds and any parts of birds or readily recognizable poultry products are not prohibited provided that the birds have been killed or captured by legal means or have been obtained by legal means; Annex III B - Species of birds for which certain Member States may permit the sale, transport for sale, keeping for sale and offering for sale live or dead birds and any parts of birds or readily recognizable poultry products within their territory, subject to certain restrictions, provided that the birds have been killed or captured by lawful means or have been obtained by lawful means.

OUG57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna: Annex 3 - Species of plants and animals whose conservation requires the designation of special conservation areas and areas of special avifauna protection; Annex 4B - Species of national interest. Species of animals and plants requiring strict protection; Annex 5C - Species of Community interest which may be hunted; Annex 5E - Bird species of Community interest which may be traded under special conditions.

Bern Convention: Appendix II - Strictly protected fauna species; Appendix III - Protected fauna species.

Bonn Convention: Annex II - Migratory species to be the subject of agreements.

### Distribution map

The distribution of occurrence locations for bird species is shown in the maps below.

Figure 3-11 Distribution of occurrence locations for bird species (A)

Figure 3-12 Distribution of occurrence locations for bird species (B)

| White Stork (Ciconia ciconia) nest in Olteni locality | Mixed nesting colony (Merops apiaster, Sturnus vulgaris) near the future wind farm |
| --- | --- |
| Red (Emberiza calandra) and yellowtail (Motacilla flava) at a temporary pond | Large bramble (Saxicola rubetra) |
| Deaf Presura (Emberiza calandra) singing | Pheasant (Phasianus colchicus) |
| Field Martin (Alauda arvensis) | Little screech eagle (Aquila pomarina) |
| Common buzzard (Buteo buteo) | Great Spotted Curlew (Buteo rufinus) |
| White Stork (Ciconia ciconia ciconia) feeding on am placement plane | White Stor Storks (Ciconia ciconia) migrating over I placement plane |
| Reed cattail (Circus aeruginosus) | Common Moorhen (Coracias garrulus) |
| Southern Mealy Bunting (Emberiza calandra) | Red-backed Vulture (Falco tinnununculus) |
| Little Eagle (Hieraaetus pennatus) | Red-breasted Shrike (Lanius collurio) |
| Black-browed Shrike (Lanius minor) | Common Kemp (Merops apiaster) |
| White motacilla (Motacilla alba) | Yellow dodo (Motacilla flava) |
| Common partridge (Perdix perdix) | Large bramble (Saxicola rubetra) |
| Turtle turtle (Streptopelia turtur) | Eupal (Upupa epops) |

Figure 3-13 Photos relevant to bird species

### Pressures identified, other important issues

The following pressures on mammal species were reported during fieldwork:

- Road presence;

- Presence of companion dogs in the herd

- Presence of roaming dogs

- Presence of roaming cats.

### How to address identified uncertainties

According to the Presentation Memorandum, the following uncertainty was found for the location of habitats and species:

- The exact distribution of the species and habitats of Community interest within the protected natural areas is not known and the flyways for all the species on the site are unknown.

Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the PUZ boundaries. Thus, it was conventionally considered that any species or habitat may have distribution over the entire area of the protected natural area (the worst case scenario).

The following table presents the results of the field activities and how the identified uncertainties have been addressed.

Table  3-9  How to address identified uncertainties

| Uncertainty identified | Species name |  | Proposed approach | Aspects analyzed | Clarifying uncertainties | The uncertainty has been clarified (Yes/No/Partial) |
| --- | --- | --- | --- | --- | --- | --- |
| The exact distribution of species and habitats of Community interest within the protected natural areas is not known and the flyways for all species in the protected areas are unknown | Accipiter gentilis | Uliu porumbar | Field trips during the optimal study period with application of three monitoring methods (transects, fixed points, monitoring cameras) Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the PUZ boundaries. Thus it was considered conventional that any species or habitat can have distribution over the whole area of the protected natural area (worst case scenario). | Presence of the species | Species observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but may use the area as foraging habitat | YES |
|  |  |  |  | Activity of the species | Within a year, two individuals were reported | YES |
|  | Accipiter nisus | Uliu bird |  | Presence of the species | Species observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but may use the area as feeding habitat | YES |
|  |  |  |  | Activity of the species | In one year, five individuals have been reported | YES |
|  | Alauda arvensis | Field woodlark |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but may use the area as feeding habitat | YES |
|  |  |  |  | Activity of the species | 560 individuals have been reported in one year | YES |
|  | Anthus campestris | Fieldfare |  | Presence of the species | Species observed in summer | YES |
|  |  |  |  | Species distribution | The species nests on the site | YES |
|  |  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
|  | Apus apus | Black Drepnea |  | Presence of the species | Species observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species nests on the site but has also been observed transiting and migrating | YES |
|  |  |  |  | Activity of the species | In one year, 90 individuals have been reported | YES |
|  | Pomarina eagle | Little screaming eagle |  | Presence of the species | Species observed in spring - summer - fall | YES |
|  |  |  |  | Species distribution | The species does not nest on the site but has also been observed transiting and migrating | YES |
|  |  |  |  | Activity of the species | In one year, 19 individuals have been reported | YES |
|  | Buteo buteo | Shared bus |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the surface of the site as feeding habitat | YES |
|  |  |  |  | Activity of the species | In one year, 162 individuals have been reported | YES |
|  | Buteo rufinus | Largeorecar |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the surface of the site as feeding habitat | YES |
|  |  |  |  | Activity of the species | In the space of a year, 240 cases were reported | YES |
|  | Caprimulgus europaeus | Caprimulg |  | Presence of the species | The species has been observed in the summer season | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 1 individual was reported | YES |
|  | Carduelis carduelis | Sticlete |  | Presence of the species | Species observed in summer - fall - winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 100 individuals have been reported | YES |
|  | Carduelis chloris | Florinte |  | Presence of the species | Species observed in summer | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 46 individuals have been reported | YES |
|  | Ciconia ciconia | White Stork |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the area for migration | YES |
|  |  |  |  | Activity of the species | 93 individuals have been reported in one year | YES |
|  | Ciconia nigra | Black stork |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within a year, an individual was reported | YES |
|  | Circaetus gallicus | Saw |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within a year, an individual was reported | YES |
|  | Circus aeruginosus | Reed reed |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 355 individuals have been reported | YES |
|  | Circus cyaneus | Erete vânăt |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 162 individuals have been reported | YES |
|  | Circus pygargus | Southern Erete |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 8 individuals have been reported | YES |
|  | Columba palumbus | Collared pigeon |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Coracias garrulus | Dumbrăveancă |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
|  | Corvus corax | Corbul |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within a year, an individual was reported | YES |
|  | Corvus frugilegus | Seed crow |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 28 individuals have been reported | YES |
|  | Coturnix coturnix | Quail |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Cuculus canorus | Cuc |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
|  | Delichon urbica | Homemade locust |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the surface of the site for migration or feeding | YES |
|  |  |  |  | Activity of the species | In one year, 50 individuals have been reported | YES |
|  | Emberiza calandra | Depressure |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 200 individuals have been reported | YES |
|  | Emberiza hortulana | Garden Presley |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Erithacus rubecula | Măcăleandru |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Falco columbarius | Winter Falcon |  | Presence of the species | The species was observed in the fall period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the surface area for feeding | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Subbuteo falcon | Sparrow hawk |  | Presence of the species | The species has been observed in summer and fall | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Falco tinnununculus | Red fannet |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the area for transit, migration and feeding | YES |
|  |  |  |  | Activity of the species | 675 individuals have been reported in one year | YES |
|  | Fringilla coelebs | Finches |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Galerida cristata | Woodpecker |  | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
|  | Gallinago gallinago | Common grant |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses its surface for transit | YES |
|  |  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
|  | Garrulus glandarius | Skip |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Hieraaetus pennatus | Little eagle |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Hirundo rustica | Swallow |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the area for transit, migration and feeding | YES |
|  |  |  |  | Activity of the species | In one year 219 individuals have been reported | YES |
|  | Lanius collurio | Reddish red spurge |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 58 individuals have been reported | YES |
|  | Lanius minor | Black-headed spurge |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Larus cachinnans | Pond Gull |  | Presence of the species | The species has been observed in spring - summer - fall | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  |  | Activity of the species | In one year 88 individuals have been reported | YES |
|  | Luscinia luscinia | The watcher of the wattle |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 1 individual was reported | YES |
|  | Luscinia megarhynchos | Red-headed nightwatcher |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Melanocorypha calandra | Bărăgan woodlark |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 55 individuals have been reported | YES |
|  | Merops apiaster | Prison |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  |  | Activity of the species | In one year 147 individuals have been reported | YES |
|  | White motacilla | White codobat |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit, migration and feeding | YES |
|  |  |  |  | Activity of the species | In one year 57 individuals have been reported | YES |
|  | Motacilla flava | Yellow codobat |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  |  | Activity of the species | 233 individuals were reported in one year | YES |
|  | Oenanthe oenanthe | South Pietrar |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 15 individuals have been reported | YES |
|  | Parus major | Big duck |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Passer domesticus | Homemade barley |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 70 individuals have been reported | YES |
|  | Perdix perdix | Partridge |  | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  |  | Species distribution | The species nests on the site | YES |
|  |  |  |  | Activity of the species | In one year, 30 individuals have been reported | YES |
|  | Pernis apivorus | Viespar |  | Presence of the species | The species was observed in the summer - fall period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses its surface for migration | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Phasianus colchicus | Fazan |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 12 individuals have been reported | YES |
|  | Phoenicurus phoenicurus | Forest cod |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Pica pica | Coțofană |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species nests on the site, but also uses the area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 69 individuals have been reported | YES |
|  | Riparia riparia | Shore grass |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
|  | Saxicola rubetra | Large bramble |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
|  | Streptoptopelia decaocto | Guguștiuc |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 39 individuals have been reported | YES |
|  | Streptopelia turtur | Turturică |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 5 individuals have been reported | YES |
|  | Sturnus vulgaris | Common starling |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 992 individuals have been reported | YES |
|  | Sylvia atricapilla | Black-headed Silvie |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Turdus merula | Blackbird |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 18 individuals have been reported | YES |
|  | Turdus philomelos | Singing Sturgeon |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 7 individuals have been reported | YES |
|  | Turdus pilaris | Cock |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 38 individuals have been reported | YES |
|  | Upupa epops | Kiss |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  |  | Activity of the species | In one year, 51 individuals have been reported | YES |
| The number of accidental casualties cannot be quantified The degree of disturbance to species and/or the likelihood of removing individuals Yes their current habitats. | Accipiter gentilis Accipiter nisus Alauda arvensis Anthus campestris Apus apus Pomarina eagle Buteo buteo Buteo rufinus Caprimulgus europaeus Carduelis carduelis Carduelis chloris Ciconia ciconia Ciconia nigra Circaetus gallicus Circus aeruginosus Circus cyaneus Circus pygargus Columba palumbus Coracias garrulus Corvus corax Corvus frugilegus Coturnix coturnix Cuculus canorus Delichon urbica Emberiza calandra Emberiza hortulana Erithacus rubecula Falco columbarius Subbuteo falcon Falco tinnununculus Fringilla coelebs Galerida cristata Gallinago gallinago Garrulus glandarius Hieraaetus pennatus Hirundo rustica Lanius collurio Lanius minor Larus cachinnans Luscinia luscinia Luscinia megarhynchos Melanocorypha calandra Merops apiaster White motacilla Motacilla flava Oenanthe oenanthe Parus major Passer domesticus Perdix perdix Pernis apivorus Phasianus colchicus Phoenicurus phoenicurus Pica pica Riparia riparia Saxicola rubetra Streptoptopelia decaocto Streptopelia turtur Sturnus vulgaris Sylvia atricapilla Turdus merula Turdus philomelos Turdus pilaris Upupa epops |  | calculations and estimates | Quantifying the impact | Collision risk calculation for all birds observed in the field was carried out considering that all species fly in the risk of collision with turbine blades. Calculation of noise and air emissions (section 1.1.7) | YES |

## Chiroptere

### Description of the working method

In order to observe chiropteran activity at the site, best chiropteran practices were implemented using state-of-the-art equipment.

Detecting chiroptera is difficult at night, so a non-invasive and highly efficient method was chosen: recording the ultrasound emitted by the animals at night.

Three monitoring methods were used:

- Qualitative

- Quantitative - transect

- Quantitative - passive detection

The qualitative method involved driving ultrasound transects, using a vehicle, along a pre-determined route at the proposed site. The transects were taken half an hour before sunset until 03:00 AM the following day to record the full optimal spectrum for the chiropteran species at the site. An Anabat Walkabout type device with GPS included was used. The data were processed using Anabat Insight software and ultrasound identification guides emitted by the chiroptera (Pocora & Pocora, 2012). Lunar transects were carried out, as far as possible in periods with meteorologically optimal conditions for animal flight (no rain, temperatures close to the lunar average at night). The total length of transects for each night was 60.7 km.

Qualitative methods were: 1 - recording of chiropteran activity at fixed points in the transects (10 points), where chiropteran activity was monitored for 10 minutes, 2 - automatic recording of chiropteran activity using static detectors placed in various positions in the field (stratified radomic sampling method), Anabat Chorus type, with a frequency of one full night of activity per month, at 5 points in the proposed site. Data recorded with static detectors were manually identified using Kaleidoscope Pro - Wildlife Acoustics software.

For any ultrasound analysis, the number of contacts is not equal to the number of individuals, an animal may be detected several times in the same place (particularly for quantitative monitoring). The number of contacts is directly proportional to the number of individuals.

The Bat Activity Index (BAI) was calculated for the quantitative method, providing a less subjective perspective in quantifying the potential impact (no. of contacts/business nights).

All spatial data were processed using ArcGIS ESRI 2023.

Figure 3-14 Hand-held ultrasonic transect detector - Anabat Walkabout type with built-in GPS - fixed point observations in transects (visual plus ultrasonic detector)

Figure 3-15 Static ultrasonic ultrasonic detector type Anabat Chorus

### Results achieved

Qualitative method

Climatic conditions during monitoring were generally optimal (one sub-optimal) for 2022.

During the monitoring period, 285 records were captured.

5 potentially 6 bat species/groups were determined. Some species cannot be determined directly by ultrasound analysis due to their similarity in echolocation, thus groupings such as Pipistrellus nathusii/Pipistrellus kuhlii or Myotis sp. (with various grouping variations) are accepted. Species determinations from the qualitative method can be found in the sonograms below.

The most abundant taxon recorded in the 2022 monitoring campaign was Pipistrellus nathusii/kuhlii, followed by Nyctalus noctula. All groups/species recorded in the study area are very common for open areas in southern Romania.

The evolution of chiropteran activity per month of monitoring indicates a peak in May 2022, which may coincide with late migrations of some species or with the beginning of the maternity period for species of the genus Pipistrellus sp.

Activity has gradually decreased since May 2022 and diversity has remained roughly similar, with few species identified during transects (maximum 5). The data indicate the possibility of the site being used as a migratory area in the spring season, but also as a summer maternity site.

Figure 3-16 Number of ultrasound transect contacts per monitoring month and per species

Table  3-10 Number of ultrasound contacts per species or group and conservation status - qualitative method - ultrasound transects

| No. Crt. | Species | O.U.G. 57/2007 | Habitats Directive | No. of contacts |
| --- | --- | --- | --- | --- |
| 1 | Pipistrellus nathusii/kuhlii | Annex IV A | Annex IV | 239 |
| 2 | Pipistrellus pipistrellus | Annex IV A | Annex IV | 7 |
| 3 | Nyctalus leisleri | Annex IV A | Annex IV | 10 |
| 4 | Nyctalus noctula | Annex IV A | Annex IV | 26 |
| 5 | Eptesicus serotinus | Annex IV A | Annex IV | 3 |

Quantitative method

Static points collected during the transect showed variable results, with relatively uniform values across the site, in terms of species diversity. The highest values for contact abundance were recorded at T2 in the northern part of the site. In general the highest values were recorded along linear vegetation features (shrub lines, roads and irrigation channels).

The static detectors were located near former irrigation canals (MS1, MS2, MS3, MS5), but also in open agricultural land (MS4). The 5 static detectors recorded a total of 31060 gross contacts, with a maximum abundance for MS2 (9986 gross contacts). The sonograms of the identified species (minus the common ones from the qualitative analysis), can be seen in The BAI activity index had the highest value for point MS2, located in the central-eastern area of plan, near an abandoned irrigation canal. The species with the highest abundance was Pipistrellus nathusii/kuhlii, but also with the highest BAI index. The evolution of ultrasound contacts per month indicates a maximum during the fall migration period (August), and a high during the spring migration period. The species Miniopterus schreibersii was more active during migration periods.

Figure 3-17 Evolution of ultrasound contacts from static detection per month of monitoring

Table  3-11 Results of monitoring at static points in transects (10 min per point), year 2022

| No. Crt. | Point | Nyctalus noctula | Pipistrellus nathusii/kuhlii | Nyctalus leisleri | Pipistrellus pipistrellus | Eptesicus serotinus | No. Total Contacts | Species diversity | BAI index | X | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T1 | 1 | 6 | 0 | 0 | 1 | 8 | 3 | 1.14 | 747156.56 | 269972.55 |
| 2 | T2 | 1 | 24 | 2 | 0 | 0 | 27 | 3 | 3.86 | 749533.96 | 269277.57 |
| 3 | T3 | 1 | 10 | 0 | 0 | 0 | 11 | 2 | 1.57 | 754284.66 | 267601.72 |
| 4 | T4 | 1 | 6 | 1 | 0 | 0 | 8 | 3 | 1.14 | 751460.76 | 267045.82 |
| 5 | T5 | 0 | 11 | 1 | 1 | 0 | 13 | 3 | 1.86 | 752550.39 | 263288 |
| 6 | T6 | 2 | 10 | 1 | 0 | 0 | 13 | 3 | 1.86 | 751620.1 | 261352.01 |
| 7 | T7 | 0 | 7 | 0 | 0 | 0 | 7 | 1 | 1.00 | 747061.81 | 263898.26 |
| 8 | T8 | 2 | 3 | 1 | 0 | 0 | 6 | 3 | 0.86 | 744156.1 | 264540.74 |
| 9 | T9 | 1 | 13 | 0 | 1 | 0 | 15 | 3 | 2.14 | 746451.69 | 266593.71 |
| 10 | T10 | 3 | 9 | 0 | 1 | 0 | 13 | 3 | 1.86 | 749278.07 | 265257.38 |

Table  3-12 No. Gross contacts registered in 2022 - Static ultrasonic detectors

| No. Crt. | No. point Static | May | June | July | August | September | October | November | X | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MS1 | 441 | 532 | 40 | 2955 | 111 | 98 | 70 | 28.13310450000 | 43.87493150000 |
| 2 | MS2 | 1436 | 206 | 2081 | 5873 | 76 | 153 | 161 | 28.12776970000 | 43.84928450000 |
| 3 | MS3 | 172 | 359 | 5750 | 18 | 225 | 54 | 9 | 28.13737450000 | 43.80321580000 |
| 4 | MS4 | 252 | 962 | 46 | 4465 | 402 | 116 | 130 | 28.03096880000 | 43.84771990000 |
| 5 | MS5 | 1442 | 240 | 1247 | 528 | 186 | 110 | 114 | 28.07506220000 | 43.87936810000 |

Table  3-13 No. of static ultrasound contacts processed - year 2022 - per static detector

| No. Crt. | No. point Static | May | June | July | August | September | October | November | Total | Species diversity | BAI index |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MS1 | 143 | 120 | 2 | 357 | 24 | 14 | 12 | 672 | 8 | 96.00 |
| 2 | MS2 | 247 | 27 | 107 | 281 | 13 | 76 | 26 | 777 | 11 | 111.00 |
| 3 | MS3 | 102 | 39 | 485 | 3 | 29 | 11 | 0 | 669 | 8 | 95.57 |
| 4 | MS4 | 24 | 258 | 8 | 226 | 153 | 22 | 4 | 695 | 10 | 99.29 |
| 5 | MS5 | 195 | 84 | 136 | 113 | 44 | 29 | 6 | 607 | 9 | 86.71 |
|  | Total | 711 | 528 | 738 | 980 | 263 | 152 | 48 | 3420 |  |  |

Table  3-14 No. of static ultrasound contacts processed - year 2022 - by species

| No. Crt. | Species | May | June | July | August | September | October | November | Total | BAI index |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Eptesicus serotinus | 45 | 16 | 2 | 7 | 12 | 3 | 0 | 85 | 12.14 |
| 2 | Miniopterus schreibersii | 2 | 5 | 7 | 23 | 42 | 6 | 0 | 85 | 12.14 |
| 3 | Myotis daubentonii | 29 | 12 | 0 | 2 | 7 | 2 | 0 | 52 | 7.43 |
| 4 | Myotis mystacinus/brandtii | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 2 | 0.29 |
| 5 | Nyctalus leisleri | 36 | 56 | 16 | 19 | 37 | 15 | 4 | 183 | 26.14 |
| 6 | Nyctalus noctula | 44 | 25 | 19 | 108 | 29 | 26 | 7 | 258 | 36.86 |
| 7 | Pipistrellus nathusii/kuhlii | 524 | 386 | 644 | 742 | 132 | 97 | 34 | 2559 | 365.57 |
| 8 | Pipistrellus pipistrellus | 17 | 18 | 29 | 48 | 2 | 2 | 2 | 118 | 16.86 |
| 9 | Pipistrellus pygmaeus | 1 | 2 | 13 | 26 | 1 | 1 | 1 | 45 | 6.43 |
| 10 | Plecotus auritus | 11 | 6 | 4 | 5 | 0 | 0 | 0 | 26 | 3.71 |
| 11 | Vespertilio murinus | 1 | 2 | 4 | 0 | 0 | 0 | 0 | 7 | 1.00 |
|  | Total | 711 | 528 | 738 | 980 | 263 | 152 | 48 | 3420 |  |

Table  3-15 Conservation status of bat species

| No. Crt. | Species | Total no. of transects | Total static no. | Total | Annexes Habitats Directive | GEO 57/2007 | Red Book of Romanian Vertebrates | Conservation status IUCN Global / European |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Eptesicus serotinus | 3 | 85 | 88 | IV | VAT | Vulnerable | LC - stable |
| 2 | Miniopterus schreibersii |  | 85 | 85 | II, IV | III, VAT | Vulnerable | VU - decrease |
| 3 | Myotis daubentonii |  | 52 | 52 | IV | VAT | Critically endangered | LC - stable |
| 4 | Myotis mystacinus/brandtii |  | 2 | 2 | IV | VAT | Periclitată | LC - unknown/ LC - stable |
| 5 | Nyctalus leisleri | 10 | 183 | 193 | IV | VAT | Periclitată | LC - unknown |
| 6 | Nyctalus noctula | 26 | 258 | 284 | IV | VAT |  | LC - unknown |
| 7 | Pipistrellus nathusii/kuhlii | 239 | 2559 | 2798 | IV | VAT | Periclitată | LC - unknown / LC - unknown |
| 8 | Pipistrellus pipistrellus | 7 | 118 | 125 | IV | VAT |  | LC - stable |
| 9 | Pipistrellus pygmaeus |  | 45 | 45 | IV | VAT |  | LC - unknown |
| 10 | Plecotus auritus |  | 26 | 26 | IV | VAT | Vulnerable | LC - stable |
| 11 | Vespertilio murinus |  | 7 | 7 | IV | VAT, BTI | Periclitată | LC - stable |
|  | Total | 285 | 3420 |  |  |  |  |  |

Figure 3-18 Transit sound sonogram Pipistrellus nathusii / kuhlii

Figure 3-19 Transit sound sonogram Nyctalus leisleri

Figure 3-20 Transit sound sonogram Nyctalus noctula

Figure 3-21 Transit sound sonogram Eptesicus serotinus

Figure 3-22 Transit sound sonogram Pipistrellus pipistrellus

Figure 3-23 Transit sound sonogram Myotis mystacinus/brandtii

Figure 3-24 Transit sound sonogram Myotis daubentonii

Figure 3-25 Transit sound sonogram Miniopterus schreibersii

Figure 3-26 Transit sound sonogram Plecotus auritus

Figure 3-27 Transit sound sonogram Vespertilio murinus

### Distribution map

The study area is not located in Natura 2000 protected areas, but species with European and National protected status are present in the study area (all chiroptera are protected).

A wind project does not need to cross a Natura 2000 site to have an impact, given that resident and migratory species of chiroptera are highly mobile, particularly during feeding or dispersal.

The chiropteran species mentioned in the site's Management Plan are Rhinolophus mehelyi, Rhinolophus ferrumequinum, Myotis myotis and Pipistrellus nathusii. Species in the plan that have not been identified at the site may be present at times in the plan area.

The Cherchezu wind farm is located in the southern part of Dobrogea, in the Romanian sector. Dobrogea is part of the Via Pontica flyway, predominantly relevant for birds, but in recent years this flyway has also been identified for chiroptera. Among the identified migratory species, which have also been identified in the proposed project, are: Nyctalus noctula, Nyctalus leisleri, Nyctalus lasiopterus, Vespertilio murinus, Pipistrellus nathusii and Miniopterus schreibersii. During migration, which takes place in spring (April-May) and fall (August-September), these species are at high risk of impact with wind turbines. The most sensitive and notable migrations may be from the species Miniopterus schreibersii, which realizes maternity colonies in the eastern part of the site and crosses part of the site to north-eastern Bulgaria for hibernation. During the maternity period there are some relevant roosts in the site, especially in the rock crevices in the area of the canaries. These crevices may not support very large populations of chiroptera, but there are species such as Eptesicus serotinus, Pipistrellus pipistrellus, Pipistrellus pipistrellus, Pipistrellus kuhlii, Hypsugo savii and possibly Vespertilio murinus, which may establish small maternity colonies during May - August. The risk of collision increases when the young are growing, i.e. July-August.

Species of the genus Rhinolopus sp. are present at the site. The direct impact is most likely to be very small, as there have been no frequent cases of collisions with wind turbine blades, but the change in habitat brought about by the installation of wind turbines may affect these rare species. Recent studies show how these species avoid certain areas of light or noise pollution, which can reduce their feeding habitat.

In order to observe the turbines with high potential risk of impact, taking into account the data presented, but also the optimal conditions described for bats in the EUROBATS guidelines, a spatial analysis was performed, which includes elements of risk for bats (distances of 200 m from the lineages, water, settlements, potential roosts or tree lineages). The turbines with high risk (4 units - T26, T27, T28 and T36) are located less than 200 m from a tree and bush lineage. The medium risk turbines (n=6), which are assessed based on field data (5 units - T5, T12, T15, T16, T20) and the remaining turbines, are considered, at least methodologically, low risk, but this will only be visible after a first year of post-construction monitoring.

The nearest SCI (Site of Community Importance) is ROSCI0071 Dumbrăveni - Urluia Lake - Vederoasa Lake. The management plan of the site indicates that no new wind farms will be approved within a radius of at least 3 km from the site. There are no turbines intersecting this 3 km boundary with the ROSCI0071 site, but 5 turbines are very close to this boundary (T23, T22, T11, T10 and T07), the shortest distance being 3030.6 m. This limit is indicative but the wind farm can affect the site in the long term (direct and cumulative impact), thus it is necessary to implement a monitoring plan for accidental kills generated by the wind farm, throughout its operation.

Figure 3-28 Sensitive areas for chiroptera and proposed turbines by chiroptera impact risk classes

Figure 3-29 Chiropteran contacts recorded during ultrasonic ultrasound tracing

Figure 3-30 Results from static monitoring points during transects - figures within points represent species diversity

Figure 3-31 Abundance of bats (BAI index) from monitoring with static ultrasonic detectors - figures within dots represent species diversity

### Pressures identified, other important issues

The most sensitive species identified at the site, but also with a high risk of impact from wind energy is Miniopterus schreibersii. This species is regionally migratory and migrations have been documented between southern Dobrogea and central Bulgaria. In Romania, the species migrates to form maternity colonies, but there are also cases of hibernation colonies. The migration corridors are unknown, but in southern Dobrogea they are most probably closely correlated with the canyons (dry canyon valleys). In the south-western and western area of the proposed plan there are such valleys (Măgura Valley - Olteni and Căciulați Valley, Aron's Valley - between Chercezu and Căscioarele localities) which can be used by all chiroptera species.

Other sensitive chiropteran species identified are all species of the genus Nyctalus sp, Pipistrellus sp, Vespertilio murinus and Eptesicus serotinus. These species may have a high risk of impact during operation (accidental kills). The rest of the identified species fly much lower to feed or migrate and the risk of impact is low (but not absent). For these species, the park may exert a greater impact through a light and noise barrier effect, but also through the alteration of habitats (roads, LES), even ruderal vegetation, used by all species that hunt in close proximity to the ground.

Eleven species of chiroptera have been identified, 8 of which are at high risk of impact from wind energy.

The study area is part of the southern sector of Dobrogea, a migration area for chiroptera. This was evidenced in the 2022 campaign ultrasound monitoring in the studied site.

One vulnerable species (Miniopterus schreibersii) has been identified, being typical of the southern part of Dobrogea, with declining populations. It may suffer heavily from wind energy impacts.

It is recommended to continue monitoring during the operational period (the entire period) to verify the effectiveness of the proposed measures. Optimal post-construction monitoring should be treated with very high importance as it will show the real impact generated by the park on resident and migratory bat populations.

The monitoring program needs to be aligned with international recommendations, as at the end of each year, a fatality estimator needs to be implemented, which will have comparable results with other monitored sites in Europe. This will help to scale the magnitude of impact and determine the exact reduction measures to be implemented.

The minimum study period is at least 3 years, with annual reports. This usually implies one complete cycle of bat population fluctuations. The optimal study period according to EUROBATS is 6 years, but in our experience, post-construction monitoring should be continued throughout the life-cycle of the project, as impacts may change and with them the proposed mitigation measures (from year to year).

The post-construction bat monitoring survey should include the following:

- Casing searches on all turbines, at least twice a month for each turbine

- A monitoring calendar at least from April to November (year-round is recommended, even in winter).

- Ultrasonic monitoring at hub height (or with microphone extensions from the nacelle) and at ground level.

- Monitoring the bats twice a month with ultrasonic transects throughout the park on a predetermined route.

- Monitoring search accuracy with at least 3 Mus musculus carcass plots (30 carcasses per plot) during spring, summer and fall.

- Monitoring scavenger carcass consumption for the 3 selected plots after performing search precision.

- Monitoring available bat roosts within a 20 km radius during spring, summer and fall

- Statistical analysis with meteorological data from each turbine, ultrasounds recorded during both static and mobile campaigns.

- GIS analysis of bat sensitivity zones

- Calculation of total and per turbine mortality rate, estimated in no. of bat carcasses/MW/year and using only research-proven methods (e.g. U.S.G.S. Fatality Estimator).

- Formulate reduction measures and recommendations for the next operational year.

### How to address identified uncertainties

According to the Presentation Memorandum, the following uncertainty was found for the location of habitats and species:

- The exact distribution of the species and habitats of Community interest within the protected natural areas is not known and the flyways for all the species on the site are unknown.

Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the PUZ boundaries. Thus, it was conventionally considered that any species or habitat may have distribution over the entire area of the protected natural area (the worst case scenario).

The following table presents the results of the field activities and how the identified uncertainties have been addressed.

Table  3-16  How to address identified uncertainties

| Uncertainty identified | Species name |  | Proposed approach | Aspects analyzed | Clarifying uncertainties | The uncertainty has been clarified (Yes/No/Partial) |
| --- | --- | --- | --- | --- | --- | --- |
| The exact distribution of species and habitats of Community interest within the protected natural areas is not known and the flyways for all species in the protected areas are unknown | Accipiter gentilis | Uliu porumbar | Field trips during the optimal study period with application of three monitoring methods (transects, fixed points, monitoring cameras) Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the PUZ boundaries. Thus, it was conventionally considered that any species or habitat may have distribution over the entire area of the protected natural area (the worst case scenario). | Presence of the species | Species observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but may use the area as feeding habitat | YES |
|  |  |  |  | Activity of the species | Within a year, two individuals were reported | YES |
|  | Accipiter nisus | Uliu bird |  | Presence of the species | Species observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but may use the area as feeding habitat | YES |
|  |  |  |  | Activity of the species | In one year, five individuals have been reported | YES |
|  | Alauda arvensis | Field woodlark |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but may use the area as foraging habitat | YES |
|  |  |  |  | Activity of the species | 560 individuals have been reported in one year | YES |
|  | Anthus campestris | Fieldfare |  | Presence of the species | Species observed in summer | YES |
|  |  |  |  | Species distribution | The species nests on the site | YES |
|  |  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
|  | Apus apus | Black Drepnea |  | Presence of the species | Species observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species nests on the site but has also been observed transiting and migrating | YES |
|  |  |  |  | Activity of the species | In one year, 90 individuals have been reported | YES |
|  | Pomarina eagle | Little screaming eagle |  | Presence of the species | Species observed in spring - summer - fall | YES |
|  |  |  |  | Species distribution | The species does not nest on the site but has also been observed transiting and migrating | YES |
|  |  |  |  | Activity of the species | In one year, 19 individuals have been reported | YES |
|  | Buteo buteo | Shared bus |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the surface of the site as feeding habitat | YES |
|  |  |  |  | Activity of the species | In one year 162 individuals have been reported | YES |
|  | Buteo rufinus | Largeorecar |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the surface of the site as feeding habitat | YES |
|  |  |  |  | Activity of the species | In the space of a year, 240 cases were reported | YES |
|  | Caprimulgus europaeus | Caprimulg |  | Presence of the species | The species has been observed in the summer season | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses its surface for transit | YES |
|  |  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
|  | Carduelis carduelis | Sticlete |  | Presence of the species | Species observed in summer - fall - winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 100 individuals have been reported | YES |
|  | Carduelis chloris | Florinte |  | Presence of the species | Species observed in summer | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 46 individuals have been reported | YES |
|  | Ciconia ciconia | White Stork |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the area for migration | YES |
|  |  |  |  | Activity of the species | 93 individuals have been reported in one year | YES |
|  | Ciconia nigra | Black stork |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within a year, an individual was reported | YES |
|  | Circaetus gallicus | Saw |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within a year, an individual was reported | YES |
|  | Circus aeruginosus | Reed reed |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 355 individuals have been reported | YES |
|  | Circus cyaneus | Erete vânăt |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 162 individuals have been reported | YES |
|  | Circus pygargus | Southern Erete |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 8 individuals have been reported | YES |
|  | Columba palumbus | Collared pigeon |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Coracias garrulus | Dumbrăveancă |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
|  | Corvus corax | Corbul |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within a year, an individual was reported | YES |
|  | Corvus frugilegus | Seed crow |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 28 individuals have been reported | YES |
|  | Coturnix coturnix | Quail |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Cuculus canorus | Cuc |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
|  | Delichon urbica | Homemade locust |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the surface of the site for migration or feeding | YES |
|  |  |  |  | Activity of the species | In one year, 50 individuals have been reported | YES |
|  | Emberiza calandra | Depressure |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 200 individuals have been reported | YES |
|  | Emberiza hortulana | Garden Presley |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Erithacus rubecula | Măcăleandru |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Falco columbarius | Winter Falcon |  | Presence of the species | The species was observed in the fall period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the surface area for feeding | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Subbuteo falcon | Sparrow hawk |  | Presence of the species | The species has been observed in summer and fall | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Falco tinnununculus | Red fannet |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the area for transit, migration and feeding | YES |
|  |  |  |  | Activity of the species | In one year, 675 individuals have been reported | YES |
|  | Fringilla coelebs | Finches |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Galerida cristata | Woodpecker |  | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 26 individuals have been reported | YES |
|  | Gallinago gallinago | Common grant |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 1 individual was reported | YES |
|  | Garrulus glandarius | Skip |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Hieraaetus pennatus | Little eagle |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Hirundo rustica | Swallow |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the area for transit, migration and feeding | YES |
|  |  |  |  | Activity of the species | In one year 219 individuals have been reported | YES |
|  | Lanius collurio | Reddish red spurge |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 58 individuals have been reported | YES |
|  | Lanius minor | Black-headed spurge |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Larus cachinnans | Pond Gull |  | Presence of the species | The species has been observed in spring - summer - fall | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the area for transit and migration | YES |
|  |  |  |  | Activity of the species | In one year 88 individuals have been reported | YES |
|  | Luscinia luscinia | The watcher of the wattle |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 1 individual was reported | YES |
|  | Luscinia megarhynchos | Red-headed nightwatcher |  | Presence of the species | The species was observed in spring | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Melanocorypha calandra | Bărăgan woodlark |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 55 individuals have been reported | YES |
|  | Merops apiaster | Prison |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses its surface for transit and migration | YES |
|  |  |  |  | Activity of the species | In one year 147 individuals have been reported | YES |
|  | White motacilla | White codobat |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit, migration and feeding | YES |
|  |  |  |  | Activity of the species | In one year 57 individuals have been reported | YES |
|  | Motacilla flava | Yellow codobat |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  |  | Activity of the species | 233 individuals have been reported in one year | YES |
|  | Oenanthe oenanthe | South Pietrar |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 15 individuals have been reported | YES |
|  | Parus major | Big duck |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Passer domesticus | Homemade barley |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 70 individuals have been reported | YES |
|  | Perdix perdix | Partridge |  | Presence of the species | The species has been observed in summer and winter | YES |
|  |  |  |  | Species distribution | The species nests on the site | YES |
|  |  |  |  | Activity of the species | In one year, 30 individuals have been reported | YES |
|  | Pernis apivorus | Viespar |  | Presence of the species | The species was observed in the summer - fall period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, but uses the area for migration | YES |
|  |  |  |  | Activity of the species | In one year, 2 individuals have been reported | YES |
|  | Phasianus colchicus | Fazan |  | Presence of the species | The species has been observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 12 individuals have been reported | YES |
|  | Phoenicurus phoenicurus | Forest cod |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 4 individuals have been reported | YES |
|  | Pica pica | Coțofană |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species nests on the site but also uses the area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 69 individuals have been reported | YES |
|  | Riparia riparia | Shore grass |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | Within one year 1 individual was reported | YES |
|  | Saxicola rubetra | Large bramble |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 10 individuals have been reported | YES |
|  | Streptoptopelia decaocto | Guguștiuc |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 39 individuals have been reported | YES |
|  | Streptopelia turtur | Turturică |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 5 individuals have been reported | YES |
|  | Sturnus vulgaris | Common starling |  | Presence of the species | The species was observed throughout the year | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year 992 individuals have been reported | YES |
|  | Sylvia atricapilla | Black-headed Silvie |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 3 individuals have been reported | YES |
|  | Turdus merula | Blackbird |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 18 individuals have been reported | YES |
|  | Turdus philomelos | Singing Sturgeon |  | Presence of the species | The species was observed in the summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 7 individuals have been reported | YES |
|  | Turdus pilaris | Cock |  | Presence of the species | The species was observed in winter | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, it uses the site area for transit | YES |
|  |  |  |  | Activity of the species | In one year, 38 individuals have been reported | YES |
|  | Upupa epops | Kiss |  | Presence of the species | The species was observed in the spring-summer period | YES |
|  |  |  |  | Species distribution | The species does not nest on the site, using the area for transit and migration | YES |
|  |  |  |  | Activity of the species | In one year, 51 individuals have been reported | YES |
| The number of accidental casualties cannot be quantified The degree of species disturbance and/or the likelihood of removing individuals Yes their current habitats. | Accipiter gentilis Accipiter nisus Alauda arvensis Anthus campestris Apus apus Pomarina eagle Buteo buteo Buteo rufinus Caprimulgus europaeus Carduelis carduelis Carduelis chloris Ciconia ciconia Ciconia nigra Circaetus gallicus Circus aeruginosus Circus cyaneus Circus pygargus Columba palumbus Coracias garrulus Corvus corax Corvus frugilegus Coturnix coturnix Cuculus canorus Delichon urbica Emberiza calandra Emberiza hortulana Erithacus rubecula Falco columbarius Subbuteo falcon Falco tinnununculus Fringilla coelebs Galerida cristata Gallinago gallinago Garrulus glandarius Hieraaetus pennatus Hirundo rustica Lanius collurio Lanius minor Larus cachinnans Luscinia luscinia Luscinia megarhynchos Melanocorypha calandra Merops apiaster White motacilla Motacilla flava Oenanthe oenanthe Parus major Passer domesticus Perdix perdix Pernis apivorus Phasianus colchicus Phoenicurus phoenicurus Pica pica Riparia riparia Saxicola rubetra Streptoptopelia decaocto Streptopelia turtur Sturnus vulgaris Sylvia atricapilla Turdus merula Turdus philomelos Turdus pilaris Upupa epops |  | calculations and estimates | Quantifying the impact | Collision risk calculation for all birds observed in the field was carried out considering that all species fly in the risk of collision with turbine blades. Calculation of noise and air emissions (section 1.1.7) | YES |

According to the Presentation Memorandum, the following uncertainty was found for the location of habitats and species:

- The exact distribution of the species and habitats of Community interest within the protected natural areas is not known and the flyways for all the species on the site are unknown.

Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the PUZ boundaries. Thus, it was conventionally considered that any species or habitat may have distribution over the entire area of the protected natural area (the worst case scenario).

The following table presents the results of the field activities and how the identified uncertainties have been addressed.

Table  3-9  Results of field activities

| Uncertainty identified | Species name |  | Proposed approach |  | Aspects analyzed |  | Clarifying uncertainties |  | The uncertainty has been clarified (Yes/No/Partial) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The presence, distribution and activity of the species is unknown Quantifying the impact | Eptesicus serotinus | Late Lilac | Field trips during the optimal study period with application of three monitoring methods (transects, fixed points, static detectors) Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the PUZ boundaries. Thus, it was conventionally considered that any species or habitat can have distribution over the whole area of the protected natural area (the worst case scenario). Using the best methodologies allows the application of a precautionary approaches in collision risk analysis for bats | Presence of the species |  | The species was detected between May and October |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the area of the site for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year 88 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  |  |  |
|  | Miniopterus schreibersii | Long-winged bat |  | Presence of the species |  | The species was detected between May and October |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the site area for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year 85 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  |  |  |
|  | Myotis daubentonii | Water bat |  | Presence of the species |  | The species was detected between May and October |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the site area for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | Within one year, 52 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  |  |  |
|  | Myotis mystacinus/brandtii | Mustached bat |  | Presence of the species |  | The species was detected in May and September |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the area of the site for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year, 2 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  |  |  |
|  | Nyctalus leisleri | Small dusk bat |  | Presence of the species |  | The species was detected between May and November |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the area of the site for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year 193 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  |  |  |
|  | Nyctalus noctula | Twilight lilac |  | Presence of the species |  | The species was detected between May and November |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the area of the site for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year, 284 contacts were registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  | YES |  |
|  | Pipistrellus nathusii/kuhlii | Nathusius's dwarf bat |  | Presence of the species |  | The species was detected between May and November |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the site area for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year 2798 contacts were registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  | YES |  |
|  | Pipistrellus pipistrellus | Dwarf bat |  | Presence of the species |  | The species was detected between May and November |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the site area for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year 125 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  | YES |  |
|  | Pipistrellus pygmaeus | Pygmy bat |  | Presence of the species |  | The species was detected between May and August |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the site area for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | 45 contacts were registered in one year |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  | YES |  |
|  | Plecotus auritus | Eared bat |  | Presence of the species |  | The species was detected between May and August |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the area of the site for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year, 26 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  |  |  |
|  | Vespertilio murinus | Liliac bicolor |  | Presence of the species |  | The species was detected between May and July |  | YES |  |
|  |  |  |  | Species distribution |  | The species, uses the area of the site for transit and feeding |  | YES |  |
|  |  |  |  | Activity of the species |  | In one year, 7 contacts have been registered |  | YES |  |
|  |  |  |  | Quantifying the impact |  | No a methodology that can indicate number of individuals that can be collision victims or barotrauma following surgery of the wind farm. The use of best methodologies allows the application of a precautionary approaches to collision risk analysis for bats. Thus, although an accurate quantification of the number of individuals who may be victims of the collision is not possible, the significance of the impact can be established |  | YES |  |

## Natura 2000 sites in Bulgaria

According to the correspondence with the Ministry of Environment and Water of Bulgaria, from 31.05. to 14.06. 2024, a series of data as well as a database on the biodiversity elements within the two natural areas potentially affected by the implementation of the plan were received from the regulator. Using the data made available by the Bulgarian Ministry, a series of maps (described below) were produced for each biodiversity component, detailed by species.

The species identified both in the databases and in the other information received were used in the assessment of the impacts of the future wind farm on the designated areas. It should be noted that many of the species identified in the submitted data are not included in the Standard Data Form. According to the official documents for the two areas, at the time of the study, no Specific Conservation Objectives, Management Plans or Regulations had been developed.

Figure 3-32 Habitat types of protected natural areas BG0000569 Kardam and BG0000570 Izvorovo - Kraishte

Figure 3-33 Plant species within protected natural areas BG0000569 Kardam and BG0000570 Izvorovo - Kraishte

Figure 3-34 Mammal species inside protected natural areas BG0000569 Kardam and BG0000570 Izvorovo - Kraishte

Figure 3-35 Invertebrate species within the protected natural area BG0000569 Kardam

Figure 3-36 Invertebrate species within the protected natural area BG000000570 Izvorovo - Kraishte

Figure 3-37 Amphibian species within protected natural areas BG0000569 Kardam and BG0000570 Izvorovo - Kraishte

Figure 3-38 Fish species within protected natural areas BG0000569 Kardam and BG0000570 Izvorovo - Kraishte

Figure 3-39 Chiropteran species within the protected natural area BG0000569 Kardam

Figure 3-40 Species of chiroptera within the protected natural area BG000000570 Izvorovo - Kraisht

# Analysis of pressures and threats

The following table shows the main pressures and threats identified in the Management Plans and Standard Forms of the Natura 2000 sites potentially affected by the proposed plan

Table  4-1 Analysis of pressures and threats

| ANPIC | Species/ habitat |  | Parameter/target affected | Pressure/threat according to ANPIC PM/FS |  | Pressure/threat level according to ANPIC PM/FS | PP contributing to pressure/ threat | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | Spermophilus citellus (Gopher) |  | Habitat area of the species | Existing/future pressures | A02.03 - Replacement of pasture by arable land. | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it The plan will increase pressure by occupying large areas of the species' potential habitat |
|  |  |  |  |  | A04.01 Intensive grazing | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  | Population size |  | D01.02 Roads, highways | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | E01.01Continuous urbanization | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Miniopterus schreibersii (Long-winged bat) Rhinolophus mehelyi (Méhely's horseshoe bat) |  | Population size |  | G01 Outdoor sport and recreation activities, recreational activities | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A10.01 Removal of hedges and vegetation or bushes | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A10.02 Removal of stone walls and stonework | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A11 Agricultural activities not mentioned above | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | B02 Management and use of forests and plantations | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B02.01 Forest replanting | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B02.01.01 Replanting of forests (indigenous trees) | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B02.01.02 Forest replanting (non-native trees) | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B02.02 Forest clearing | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B02.03 Removal of forest vegetation | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B02.04 Removal of dry and drying trees | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | C03.03 Wind energy production (this pressure is not mentioned in the management plan for this species, it is mentioned for birds) | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
| ROSCI0157/ROSAC157 Hagieni - Cotul Văii Forest | Spermophilus citellus (Gopher) |  | Habitat area of the species Habitat grass layer height | Existing pressures | A02.03 - Replacement of pasture by arable land. | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it The plan will increase pressure by occupying large areas of the species' potential habitat |
|  |  |  |  |  | A04.01 Intensive grazing | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  | Future pressures | E01.01 Continued urbanization | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drains in Credința village, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it but also on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure by increasing the area of potential foraging and nesting habitat for the species |
|  |  |  | Population size | Existing pressures | A.07 Use of biocides, hormones and chemicals | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A10.01 Removal of hedges and hedgerows and bushes | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A.11 Agricultural activities other than those listed above | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | E03.01 Disposal of household waste | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | Implementation of the plan may generate a cumulative effect on the size of the species through potential casualties The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | G01.03.02 Off-road driving of motorized vehicles | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it but also on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  | Future pressures | E01.01 Continued urbanization | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land enclosure and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it but also on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  | Rhinolophus ferrumequinum (Greater horseshoe bat) |  | Population size Foraging habitat area of the species | Existing pressures | G01 Outdoor sport and recreation, amusement and recreation activities | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A10.01 Removal of hedges and vegetation or bushes | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A10.02 Removal of stone walls and stonework | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A11 Agricultural activities not mentioned above | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | B02 Management and use of forests and plantations | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Rhinolophus ferrumequinum by reducing its surface area and disturbing the activity of the species, as well as on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.01.02 Forest replanting (non-native trees) | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Rhinolophus ferrumequinum by reducing its surface area and disturbing the activity of the species, as well as on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.02 Forest clearing | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the habitat area of Rhinolophus ferrumequinum by reducing its surface area and disturbing the activity of the species, as well as on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.03 Removal of forest vegetation | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the Rhinolophus ferrumequinum habitat area by reducing it but also disturbing the species' activity and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.04 Removal of dried or drying trees | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the Rhinolophus ferrumequinum habitat area by reducing it but also disturbing the species' activity and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B03 Exploitation of forests without replanting or natural regrowth | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the Rhinolophus ferrumequinum habitat area by reducing it but also disturbing the species' activity and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | E01.01 Continued urbanization | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the Rhinolophus ferrumequinum habitat area by reducing it but also disturbing the species' activity and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | C03.03 Wind energy production (this pressure is not mentioned in the management plan for this species, it is mentioned for birds | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the Rhinolophus ferrumequinum habitat area by reducing it but also disturbing the species' activity and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  | Mustela eversmannii (Steppe fer ferret) |  | Population size | Existing pressures | A02.03 - Replacement of pasture by arable land. | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it The plan will increase pressure by occupying large areas of the species' potential habitat |
|  |  |  |  |  | A04.01 Intensive grazing | Average | - | The plan will not increase the pressure given since it does not involve the same type of activities |
|  |  |  | Habitat area of the species |  | A.07 Use of biocides, hormones and chemicals | Unknown | - | The plan will not increase the pressure given since it does not involve the same type of activities |
|  |  |  |  |  | A10.01 Removal of hedges and hedgerows and bushes | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A.11 Agricultural activities other than those listed above | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | E03.01 Disposal of household waste | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | G01.03.02 Off-road driving of motorized vehicles | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the species' habitat area by reducing it but also on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  | Miniopterus schreibersii (Long-winged Bat) |  | Population size Foraging habitat area of the species | Existing pressures | G01 Outdoor sport and recreation activities, recreational activities | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A10.01 Removal of hedges and vegetation or bushes | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A10.02 Removal of stone walls and stonework | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | A11 Agricultural activities not mentioned above | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | B02 Management and use of forests and plantations | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land enclosure and site organization located in Independența locality | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.01.02 Forest replanting (non-native trees) | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land enclosure and site organization located in Independența locality | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.02 Forest clearing | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.03 Removal of forest vegetation | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B02.04 Removal of dried or drying trees | Retrieved | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | B03 Exploitation of forests without replanting or natural regrowth | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | A07 Use of biocides, hormones and chemicals | Unknown | - | The plan will not increase the pressure given since it does not involve the same type of activities |
|  |  |  |  |  | E01.01 Continued urbanization | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  |  | C03.03 Wind energy production (this pressure is not mentioned in the management plan for this species, it is mentioned for birds) | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
|  |  |  |  | Future pressures | E01.01 Continued urbanization | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may generate a cumulative effect on the size of the habitat area of Miniopterus schreibersii by reducing its area but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
| ROSPA0166 Plopeni - Chirnogeni | Anthus campestris (Field martin) |  | Population size Distribution pattern Habitat area | Existing pressures | A02.01 Intensive agriculture (off-site) | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Buteo rufinus (Great Bute) |  |  |  | G01.03.01 Regular driving of motorized vehicles (on-site) | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Circus cyaneus (Black-headed Erytra) |  |  |  |  |  |  |  |
|  | Falco columbarius (Winter Falcon) |  | Population size trend |  |  |  |  |  |
|  | Falco vespertinus (Evening Goshawk) |  |  |  |  |  |  |  |
|  | Lanius minor (Black-fronted Shrike) |  |  |  |  |  |  |  |
| ROSPA0036 Dumbrăveni | Accipiter brevipes (Short-legged Eurasian Short-legged) | Species associated with reed habitats | Population size Population trends Distribution pattern | Existing pressures for all bird species of conservation concern | A02.03 Use of grassland for arable land | Retrieved | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase pressure by occupying large areas of the species' potential habitat |
|  | Anthus campestris (Field martin) | Acrocephalus arundinaceus (Great Grebe) |  |  | A04.01 Intensive grazing | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Aquila pomarina (Lesser Spotted Eagle) | Acrocephalus palustris (Marsh wagtail) |  |  | A07 Use of biocides, hormones and chemicals | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Bubo bubo (Owl) | Acrocephalus schoenobaenus (Little Grebe) |  |  | A08 Fertilization - with fertilizer | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Buteo rufinus (Great Bute) | Acrocephalus scirpaceus (Reed Reed Reed) |  |  | A11 Agricultural activities not mentioned above | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Caprimulgus europaeus (Caprimulg) | Locustella fluviatilis (Red-crested Cricket) |  |  | B02 Management and use of forests and plantations | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Circus cyaneus (Black-headed Erytra) | Locustella luscinioides (Reed Cricket) |  |  | B02.02. Deforestation | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Circus macrourus (White Erete) | Riparia riparia (Mal's grass) |  |  | B02.01.02 Replanting of forests (non-native trees) | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Circus pygargus (Erete sur) |  |  |  | B02.04 Removal of dried or drying trees | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Coracias garrulus (Dusky) |  | Population size trend Habitat surface |  | B06 Grazing in forest areas | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Falco peregrinus (Peregrine Falcon) |  |  |  | B03 Exploitation of forests without replanting or natural regeneration | Retrieved | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Hieraaetus pennatus (Little Eagle) |  |  |  | C03.03 Wind energy production | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Lanius collurio (Red-breasted Shrike) |  |  |  | D02.01.01 Overhead power lines | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The plan will intensify the pressure through potential accidental casualties |
|  | Lanius minor (Black-fronted Shrike) |  |  |  | G01 Outdoor sport and recreation, amusement and recreation activities | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Melanocorypha calandra (Bărăgan's Woodpecker) |  |  |  | E01.01 Continuous urbanization | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Milvus migrans (Black Jay) |  |  |  |  |  |  |  |
|  | Pernis apivorus (Viespar) |  |  |  |  |  |  |  |
|  | Regularly migratory species Species associated with extensively used agricultural land | Species associated with cliff habitats and urban areas | Population size Population trends Distribution pattern |  | A02.03 Use of grassland for arable land | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase pressure by occupying large areas of the species' potential habitat |
|  | Alauda arvensis (Woodlark) | Monticola saxatilis (Stone blackbird) |  |  | A04.01 Intensive grazing | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Carduelis cannabina (Hemp) | Hirundo daurica (Red-crowned Swallowtail) |  |  | A07 Use of biocidal products, hormones and hymic substances | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Carduelis carduelis (Stellar) | Hirundo rustica (Swallow) | Population size trend Habitat area |  | A08 Fertilization - with fertilizer | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Coturnix coturnix (Quail) |  |  |  | A11 Agricultural activities not mentioned above | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Emberiza melanocephala (Black-headed barnacle) |  |  |  | B02 Management and use of forests and plantations | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Falco tinnununculus (Red-rumped Vulture) |  |  |  | B02.02. Deforestation | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Fringilla montifringilla (Winter finch) |  | Population size Population trends Distribution pattern |  | B02.01.02 Replanting of forests (non-native trees) | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Galerida cristata (Woodlark) |  |  |  | B02.04 Removal of dried or drying trees | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Lanius excubitor (Great Crested Crake) |  |  |  | B06 Grazing in forest areas | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Lanius senator (Red-headed Shrike) |  |  |  | B03 Exploitation of forests without replanting or natural regeneration | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Merops apiaster (Prigorie) |  |  |  | C03.03 Wind energy production | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Miliaria calandra (Southern Mealy Borer) |  |  |  | D02.01.01 Overhead power lines | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The plan will intensify the pressure through potential accidental casualties |
|  | Motacilla alba (White motacilla) |  |  |  | G01 Outdoor sport and recreation, amusement and recreation activities | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Motacilla flava (Yellow Codobat) |  |  |  | E01.01 Continuous urbanization | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Oenanthe isabellina (Eastern Pietrar) |  |  |  |  |  |  |  |
|  | Oenanthe oenanthe (South Pietrar) |  |  |  |  |  |  |  |
|  | Saxicola torquata (Black bramble) |  |  |  |  |  |  |  |
|  | Streptopelia turtur (Turtur) |  |  |  |  |  |  |  |
|  | Sturnus roseus (alternative name Pastor roseus) (Grasshopper) |  |  |  |  |  |  |  |
|  | Turdus iliacus (Vine Turd) |  |  |  |  |  |  |  |
|  | Upupa epops (Kiss) |  |  |  |  |  |  |  |
|  | Species associated with forest habitats |  |  |  |  |  |  |  |
|  | Carduelis chloris (Florinte) |  |  |  |  |  |  |  |
|  | Carduelis spinus |  |  |  |  |  |  |  |
|  | Coccothraustes coccothraustes (Botgros) |  |  |  |  |  |  |  |
|  | Falco subbuteo (Sparrow hawk) |  |  |  |  |  |  |  |
|  | Ficedula hypoleuca (Black Muscar) |  |  |  |  |  |  |  |
|  | Fringilla coelebs (Finch) |  |  |  |  |  |  |  |
|  | Jynx torquilla (Capintors) |  |  |  |  |  |  |  |
|  | Muscicapa striata (Muscar sur) |  |  |  |  |  |  |  |
|  | Oriolus oriolus (Grangur) |  |  |  |  |  |  |  |
|  | Phylloscopus collybita (Lesser Collybita) |  |  |  |  |  |  |  |
|  | Phylloscopus sibilatrix (Sizzling Wood-Dove) |  |  |  |  |  |  |  |
|  | Phylloscopus trochilus (Whistling Wood-Dove) |  |  |  |  |  |  |  |
|  | Pyrrhula pyrrhula (Mugurar) |  |  |  |  |  |  |  |
|  | Regulus ignicapillus (Spur-browed Auk) |  |  |  |  |  |  |  |
|  | Regulus regulus (Yellow-headed Auk) |  |  |  |  |  |  |  |
|  | Serinus serinus (Caper) |  |  |  |  |  |  |  |
|  | Sturnus vulgaris (Graur) |  |  |  |  |  |  |  |
|  | Sylvia atricapilla (Black-headed Sylvia) |  |  |  |  |  |  |  |
|  | Sylvia curruca (Little Sylvia) |  |  |  |  |  |  |  |
|  | Turdus merula (Blackbird) |  |  |  |  |  |  |  |
|  | Turdus philomelos (Singing Sturz) |  |  |  |  |  |  |  |
|  | Turdus pilaris (Cock Cock) |  |  |  |  |  |  |  |
|  | Turdus viscivorus (Mistle Thrush) |  |  |  |  |  |  |  |
| ROSPA0094 Hagieni Forest | Accipiter brevipes (Short-legged Eurasian Short-legged) | Lanius collurio (Red-breasted Shrike) | Population size Habitat surface | Existing pressures for all bird species of conservation concern | A02 Changing cultivation practices | Retrieved | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase pressure by occupying large areas of the species' potential habitat |
|  | Alcedo atthis (Blue Gull) | Lanius minor (Black-fronted Shrike) |  |  | A02.03 Replacement of grassland by arable land | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase pressure by occupying large areas of the species' potential habitat |
|  | Anthus campestris (Field martin) | Larus melanocephalus (Black-headed Gull) |  |  | A04.01 Intensive grazing | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Aquila heliaca (Field Eagle) | Larus minutus (Little Gull) |  |  | B02 Forest and plantation management and use | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Aquila pomarina (Lesser Spotted Eagle) | Lullula arborea (Woodlark) |  |  | C03.03 Use of wind energy | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Burhinus oedicnemus (Common Oedicnemus) | Melanocorypha calandra (Bărăgan's Woodpecker) |  |  | E04 Infrastructure, buildings in the landscape | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Buteo rufinus (Great Bute) | Milvus migrans (Black Jay) |  |  | A07 Use of biocides, hormones and chemicals | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Caprimulgus europaeus (Caprimulg) | Nycticorax nycticorax (Night heron) |  |  |  |  |  |  |
|  | Calandrella brachydactyla (Flock of Common Woodpeckers) | Oenanthe pleschanka (Black Petrel) |  |  | B01 Forest planting on open land | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Ciconia ciconia (White Stork) | Pelecanus crispus (Crested Pelican) |  |  |  |  |  |  |
|  | Circaetus gallicus (Snake) | Pernis apivorus (Viespar) |  |  |  |  |  |  |
|  | Circus aeruginosus (Reed cattail) | Philomachus pugnax (Bully) |  |  |  |  |  |  |
|  | Circus cyaneus (Black-headed Erytra) | Porzana porzana porzana/Calidris pugnax (Spotted Crested) |  |  |  |  |  |  |
|  | Circus macrourus (White Erete) | Recurvirostra avosetta (Ciocîntors) |  |  |  |  |  |  |
|  | Circus pygargus (Erete sur) | Tadorna ferruginea (Red-crested Shelduck) |  |  |  |  |  |  |
|  | Coracias garrulus (Dusky) |  |  |  |  |  |  |  |
|  | Falco cherrug (Saker falcon) |  |  |  |  |  |  |  |
|  | Falco columbarius (Wintering Falcon) |  |  |  |  |  |  |  |
|  | Falco peregrinus (Peregrine Falcon) |  |  |  |  |  |  |  |
|  | Falco vespertinus (Evening Goshawk) |  |  |  |  |  |  |  |
|  | Hieraaetus pennatus (Little Eagle) |  |  |  |  |  |  |  |
|  |  |  | Population size trend | Future pressures | A02 Changing cultivation practices | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase pressure by occupying large areas of the species' potential habitat |
|  |  |  |  |  | A02.03 Replacement of grassland by arable land | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase pressure by occupying large areas of the species' potential habitat |
|  |  |  |  |  | A04.01 Intensive grazing | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | B02 Forest and plantation management and use | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | C03.03 Use of wind energy | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | E04 Infrastructure, buildings in the landscape | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | A07 Use of biocides, hormones and chemicals | Unknown | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | B01 Forest planting on open land | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | a. Regularly migratory species |  | Population size Habitat surface | For these species the Site Management Plan does not identify specific pressures. The general pressures identified at site level have been taken into account. |  |  |  |  |
|  | Species associated with open/agricultural land |  |  | Existing pressures | A02.03 Replacement of grassland by arable land | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase pressure by occupying large areas of the species' potential habitat |
|  | Alauda arvensis (Woodlark) | Oenanthe oenanthe (South Pietrar) |  |  | A04.01 Intensive grazing | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Buteo lagopus (Shoed Buteo) | Streptopelia turtur (Turtur) |  |  | A.07 Use of biocides, hormones and chemicals | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Emberiza melanocephala (Black-headed barnacle) | Upupa epops (Kiss) |  |  | A.08 Fertilization - with fertilizer | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Lanius senator (Red-headed Shrike) | Falco tinnununculus (Red-rumped Vulture) |  |  | A10.01 Removal of hedges and hedgerows and bushes | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Merops apiaster (Prigorie) | Anser albifrons (Great Tern) |  |  | A.11 Agricultural activities other than those listed above | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Miliaria calandra (Southern Mealy Borer) | Galerida cristata (Woodlark) | Population size trend for each species |  | B02 Management and use of forests and plantations B02 01.02 | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Motacilla alba (White motacilla) | Coturnix coturnix (Quail) |  |  | Forest replanting - non-native trees B02.02 Forest clearance | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B02.03 Shoot removal | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | B02.04 Removal of dried or drying trees | Retrieved | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B03 Farming without replanting | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on the size of the population by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | B06 Forest/woodland grazing | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan i may have a cumulative effect on the size of the area of nesting habitat for bird species by reducing it but also disturbing the activity of the species and on population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | D01.02 Roads, motor roads | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of the species by reducing it but also on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) Habitat surface |  | D02.01.01 Power lines | Low | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | E01.01 Continuous urbanization | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Wind Park Comana - Pecineaga - route of MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | E03.01 Disposal of household waste | Low | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | Implementation of the plan may have a cumulative effect on the size of the area of foraging habitat for the species by reducing it The plan will intensify the pressure through potential accidental casualties |
|  |  |  |  |  | F02 Fishing and harvesting of aquatic resources | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | G.01 Outdoor sports and leisure and recreation, recreational activities | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  |  |  |  |  | G01.03.02 Off-road driving of motorized vehicles | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify pressure by reducing the stock due to accidental mortality |
|  | b. Regularly migratory species |  | Population size Habitat surface |  | H01 Water pollution | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Species associated with riparian and coastal habitats |  |  |  | I01 Invasive non-native/halogenic species | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electricity network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The plan will increase pressure by spreading invasive non-native species |
|  | Gallinago gallinago (Common Snipe) | Hirundo rustica (Swallow) |  |  | K03.01 Competition - invasive species | Retrieved | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The plan will increase pressure by spreading invasive non-native species |
|  |  |  | Population size trend for each species |  |  |  |  |  |
|  | c. Regularly migratory species |  | Population size Habitat surface |  |  |  |  |  |
|  | Species associated with urban habitats |  |  |  |  |  |  |  |
|  | Hirundo rustica (Swallow) |  |  |  |  |  |  |  |
|  |  |  | Population size trend for each species |  |  |  |  |  |
|  | d. Regularly migratory species |  | Population size Habitat surface |  |  |  |  |  |
|  | Species associated with terrestrial mixed woodland and open land habitats |  |  |  |  |  |  |  |
|  | Accipiter nisus (Bird's-billed Woodlark) | Falco subbuteo (Sparrow hawk) |  |  |  |  |  |  |
|  |  |  | Population trends for each species |  |  |  |  |  |
|  | Buteo buteo (Common buzzard) |  |  |  |  |  |  |  |
|  | e. Regularly migratory species |  | Population size Habitat surface |  |  |  |  |  |
|  | Species associated with reed habitats |  |  |  |  |  |  |  |
|  | Motacilla flava (Yellow Codobat) |  | Population trends for each species |  |  |  |  |  |
|  | f. Regularly migratory species |  | Population size Habitat surface |  |  |  |  |  |
|  | Species associated with open aquatic habitats |  |  |  |  |  |  |  |
|  | Anser albifrons (Great Tern) | Anser anser (Summer Goose) |  |  |  |  |  |  |
|  |  |  | Population trends for each species Habitat surface |  |  |  |  |  |
|  | Cygnus olor (Summer swan) | Larus cachinnans (Mediterranean Gull) |  |  |  |  |  |  |
|  | g. Regularly migratory species |  | Population size Habitat surface |  |  |  |  |  |
|  | Species associated with forest habitats |  |  |  |  |  |  |  |
|  | Sylvia atricapilla (Black-headed Sylvia) |  | Population trends for each species |  |  |  |  |  |
|  | Applicable for all above species (a-g) with specific assessment parameters |  |  |  |  |  |  |  |
|  | Applicable for all above species (a-g) with specific assessment parameters |  |  | Future pressures | E01.01 urbanization continues | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county Establishment of a smart gas distribution network in Independența commune, Constanta county Construction of underground fiber optic communications network for electronic services, Negru Voda Construction of underground fiber optic network for electronic communication services in Constanta county, Independența commune, Dumbrăveni | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of bird species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will increase the pressure by reducing the stock due to accidental mortality |
| BG0000569 Kardam | 6110* White Alysso-Sedion calcareous or basifyllic rocky grasslands |  | Habitat area | Existing pressures | A07 Use of biocides, hormones and chemicals | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | 6210* - Semi-natural xerophytic and facies xerophytic grassland with scrubs on calcareous substrata (Festuco-Brometalia) |  |  |  | D02.01 Electric and telephone lines | Low | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land enclosure and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The plan will intensify the pressure through potential accidental casualties |
|  | Vertigo moulinsiana (Coiled Snail) | Testudo graeca (Dobrogea land tortoise) | Habitat area of the species |  | A04 Grazing | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Vertigo angustior | Testudo hermanni (Land tortoise) |  |  | A05.02 Feeding of animals | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Unio crassus (Little River Clam) | Lynx lynx (Laughter) |  |  | L09 Fires (natural) | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Osmoderma eremita (Hermit Beetle) | Lutra lutra (Otter) |  |  | K01.01 Erosion | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Morimus funereus (Grey croaker) | Vormela peregusna (Spotted ferret) |  |  | F03.02.03 Trapping, poisoning, poaching | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Lycaena dispar (Red Swamp Butterfly) | Rhinolophus mehelyi (Méhely's horseshoe bat) |  |  | A03 Mowing/mowing of pasture | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land enclosure and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | Implementation of the plan may have a cumulative effect on the amount of foraging habitat area for the identified species by reducing it The plan will increase pressure through potential accidental casualties and reduced areas of potential habitat |
|  | Lucanus cervus (Red-breasted) | Rhinolophus hipposideros (Little horseshoe bat) |  |  | K01.03 Drying | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Hypodryas maturna (Rare Fritillary) | Rhinolophus ferrumequinum (Greater horseshoe bat) |  |  | F02.03 Recreational fishing | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Euplagia quadripunctaria (Striped butterfly) | Rhinolophus euryale (Mediterranean horseshoe bat) | Population size |  | A01 Cultivation | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Dioszeghyana schmidtii (Comb Owl Butterfly) | Rhinolophus blasii (Blasius' horseshoe bat) |  |  | A08 Fertilization | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Cucujus cinnaberinus | Miniopterus schreibersii (Long-winged Bat) |  |  | B01 Forest planting on open land | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Cerambyx cerdo (Great Oak Bunting) | Myotis myotis (Common bat) |  |  | B02.02 Forest clearing | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the nesting habitat area of the identified species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Rhodeus sericeus amarus | Myotis emarginatus (Hollow-eared bat) |  |  | D01.02 Roads, highways | Average | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of the identified species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Cobitis taenia (Cobitis taenia) | Myotis capaccinii (Long-legged bat) |  |  | B Forestry | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Barbus meridionalis (Black-crowned Murre) | Myotis blythii (Little Common Bat) |  |  | D01.04 Railways | Low | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of the identified species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Sabanejewia aurata (Dunariță) | Myotis bechsteinii (Big-eared bat) |  |  | A05.01 Farming of animals | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Elaphe sauromates (Dobrogean dragon) | Barbastella barbastellus (Barbastella barbastellus) |  |  | F03.01 Hunting | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Bombina variegata (Yellow-bellied Pond Owl) |  |  |  | A04.03 Abandonment of pastoral systems, lack of grazing | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Emys orbicularis (European pond turtle) |  |  |  | E01.03 Dispersed living - scattered, dispersed dwellings | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Mesocricetus newtoni (Romanian Hamster) |  |  |  |  |  |  |  |
|  | Mustela eversmanii (Steppe ferret) |  |  |  |  |  |  |  |
|  | Spermophilus citellus (Gopher) |  |  |  |  |  |  |  |
|  | Triturus dobrogicus (Dobrogean crested newt) |  |  |  |  |  |  |  |
|  | Triturus karelinii (Crested newt) |  |  |  |  |  |  |  |
| BG0000570 Izvorovo - Kraishte | 6110* White Alysso-Sedion calcareous or basifyllic rocky grasslands |  | Habitat area | Existing pressures | D02.01 Electric and telephone lines | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land enclosure and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The plan will intensify the pressure through potential accidental casualties |
|  | 6210* Semi-natural xerophytic and facies xerophytic grassland with shrubs on calcareous substrata (Festuco-Brometalia) |  |  |  | A03 Mowing/mowing of pasture | Average | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground cables and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB location, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | Implementation of the plan may have a cumulative effect on the amount of foraging habitat area for the identified species by reducing it The plan will increase pressure through potential accidental casualties and reduced areas of potential habitat |
|  | 62C0* Point-sarmatic stepping stones |  | Habitat area of the species |  | E01.03 Dispersed living - scattered, dispersed dwellings | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Echium russicum (Snake's head) | Elaphe sauromates (Dobrogean dragon) |  |  | A04 Grazing | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Potentilla emilii-popii (Five-toed weed) | Emys orbicularis (European pond turtle) |  |  | A08 Fertilization | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Vertigo moulinsiana (Coiled Snail) | Mesocricetus newtoni (Romanian Hamster) |  |  | K01.03 Drying | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Vertigo angustior | Mustela eversmanii (Steppe ferret) |  |  | B Forestry | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Unio crassus (Little River Clam) | Lynx lynx (Laughter) |  |  | D01.02 Roads, highways | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing constructions and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County Construction of bridges, culverts and paving of rainwater drainage channels in Credința locality, Chirnogeni commune Street modernization and rehabilitation in Chirnogeni commune, Constanta county | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of the identified species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Pseudophilotes bavius | Lutra lutra (Otter) |  |  | K01.01 Erosion | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Paracaloptenus caloptenoides (Devil Horse) | Spermophilus citellus (Gopher) |  |  | D01.04 Railways | Low | Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality Chirnogeni wind farm (existing) Karnobat wind farm (existing) | The implementation of the plan may have a cumulative effect on the size of the foraging habitat area of the identified species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Osmoderma eremita (Hermit Beetle) | Vormela peregusna (Spotted ferret) |  |  | F03.01 Hunting | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Ophiogomphus cecilia (Green Gomphida) | Testudo graeca (Dobrogea land tortoise) |  |  | F02.03 Recreational fishing | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Morimus funereus (Grey croaker) | Testudo hermanni (Land tortoise) |  |  | L09 Fires (natural) | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Lycaena dispar (Red Swamp Butterfly) | Triturus dobrogicus (Dobrogean crested newt) | Population size |  | F03.02.03 Trapping, poisoning, poaching | Average | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Lucanus cervus (Red-breasted) | Triturus karelinii (Crested newt) |  |  | A05.01 Farming of animals | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Hypodryas maturna (Rare Fritillary) | Rhinolophus mehelyi (Méhely's horseshoe bat) |  |  | A01 Cultivation | Retrieved | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Euplagia quadripunctaria (Striped butterfly) | Rhinolophus hipposideros (Little horseshoe bat) |  |  | A04.03 Abandonment of pastoral systems, lack of grazing | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Dioszeghyana schmidtii | Rhinolophus ferrumequinum (Greater horseshoe bat) |  |  | B02.02 Forest clearing | Low | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța Comana - Pecineaga Wind Farm - MV underground electric cables route and fiber optic network Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, com. Independența, jud. Constanța Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB installation, internal road and access, land fencing and site organization located in Independența locality | The implementation of the plan may have a cumulative effect on the size of the nesting habitat area of the identified species by reducing it but also disturbing the activity of the species and on the population size by reducing the population numbers due to accidental mortality The plan will intensify the pressure through potential accidental casualties |
|  | Cucujus cinnaberinus | Rhinolophus euryale (Mediterranean horseshoe bat) |  |  | B01 Forest planting on open land | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Coenagrion ornatum | Rhinolophus blasii (Blasius' horseshoe bat) |  |  | A07 Use of biocides, hormones and chemicals | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Cerambyx cerdo (Great Oak Bunting) | Miniopterus schreibersii (Long-winged Bat) |  |  | A05.02 Feeding of animals | Low | - | The plan will not increase the pressure given It does not involve the same type of activities. |
|  | Rhodeus sericeus amarus | Myotis myotis (Common bat) |  |  |  |  |  |  |
|  | Cobitis taenia (Cobitis taenia) | Myotis emarginatus (Hollow-eared bat) |  |  |  |  |  |  |
|  | Barbus meridionalis (Black-crowned Murre) | Myotis capaccinii (Long-legged bat) |  |  |  |  |  |  |
|  | Sabanejewia aurata (Dunariță) | Myotis blythii (Little Common Bat) |  |  |  |  |  |  |
|  | Canis lupus (Wolf) | Myotis bechsteinii (Big-eared bat) |  |  |  |  |  |  |
|  | Bombina variegata (Yellow-bellied Pond Owl) | Barbastella barbastellus (Barbastella barbastellus) |  |  |  |  |  |  |

# Impact assessment

The assessment of the impacts on PNACIs was carried out on the basis of the conservation objectives of each PNACI established by the authority responsible for the management/administration of protected natural areas of Community interest. The assessment was carried out for all the habitats and species for the protection of which the PANIs have been designated as potentially affected by the plan, at the level of each CSO parameter.

At this stage, the potentially affected ANPICs are known and therefore the Specific Conservation Objectives (SCOs) of the habitats and species of Community interest in these sites are available.

## Identifying and quantifying impacts

### Identification of potentially significant impacts based on the Specific Conservation Objectives

The type and magnitude of impact depends to a large extent on the species involved, their ecology and conservation status, and the location, size and design of the entire plan.

The approach proposed in this study is based on the relationship: CAUSE - EFFECTS - IMPACTS.

- The causes are the interventions proposed in the plan.

- Impacts are physical, chemical and biological changes to the environment as a result of a cause (examples: increased noise levels, increased concentrations of pollutants in air, water or soil, increased light intensity, invasive species, other impacts).

- Impacts are changes to sensitive receptors (habitats and species) as a result of interaction with effects.

The main forms of impact that could affect the structure and functions of the protected natural areas identified for the proposed plan are as follows:

- Habitat loss

This form of impact consists in the loss of areas of habitats of community interest, i.e. areas of habitats favorable for the different stages of development and activities of species of community interest (breeding, resting, feeding, etc.), as a result of works.

The significance of habitat loss depends on the rarity and vulnerability of the habitats affected and/or their importance as foraging, breeding or resting areas for certain species, in particular for the conservation of species of European interest. Consideration should also be given to the potential role of certain habitats as components of important corridors or crossing points for dispersal and migration as well as for different local movements, for example between feeding and nesting sites.

- Habitat alteration

This form of impact occurs as a result of physical, chemical and biological alterations to terrestrial and aquatic habitats, and includes those structural and functional changes that lead to a decrease in their carrying capacity (e.g. populations of flora species of Community interest are altered as a result of decreased trophic support or increased competition with non-native/invasive species). Over time, altered habitats can lead to habitat loss for species of Community interest.

Habitat alteration is, broadly speaking, a process of temporary or long-term loss of the original, characteristic qualities of the affected areas, expressed by those transformations that diminish their structure and composition as well as their favorability for wildlife species. Habitat alteration refers to both Natura 2000 habitat types and species habitats (environments defined by abiotic and biotic factors, in which species live at any stage of their life cycle).

In the construction phase, habitat alteration occurs both on the areas where works are being carried out and in the areas adjacent to them. In the operational phase, habitat alteration occurs mainly on areas affected by the presence of pollutants.

Interventions that may lead to habitat alteration during the construction, operation and decommissioning phases can be:

- Temporary occupation of a habitat area with materials or machinery without removal of natural vegetation;

- Crossing an area of habitat by vehicles without destruction/removal of natural vegetation;

- The presence of pollutants that can inhibit the growth of vegetation or other organisms without destroying them;

- The invasiveness and spread of invasive species;

- Modification of the physical, chemical and biological parameters of the habitat without the removal of individuals of species characteristic of the habitat.

- Fragmentation of specific habitats

While the activities related to the implementation of the plan (construction-assembly phases) may result in the local destruction of natural habitats on the surfaces occupied by the related structures, in the operational phase the degradation of habitats ceases, the impact becoming negligible

At the present level plan habitat fragmentation may be due to:

- Creating physical barriers (fences);

- Emergence of behavioral barriers (e.g. due to noise, artificial lighting, human presence, and others).

Noise, artificial lighting and human presence are factors that are conventionally analyzed under the impact form "species disturbance" and are easier to quantify from this perspective.

- Disruption of wildlife species

This form of impact is associated with human presence and human activity occurring during both the construction and operation phases.

In the case of the implementation of the project, the disturbance of wildlife species is due to :

- increased noise levels - noise disturbance affects not only nesting, but also inter- and intraspecific communication, reproduction or feeding of wildlife species;

- artificial lighting - affects the nesting and foraging activities of some bird species, or can induce behavioral changes in the activity of some nocturnal species, such as invertebrates, amphibians, birds or bats;

- human presence.

The following changes may occur as a result of land occupation, human presence, increased noise levels, artificial lighting sources or other contributing effects:

- Inter- and intraspecific communication impairment;

- Abandon nest/breeding areas;

- Changes in travel routes with increased energy consumption of affected individuals;

- Removal of individuals of a species (with relocation within or outside the Natura 2000 site).

- Reducing the size of stocks

At the level of a Natura 2000 site, population declines may occur:

Directly as a result of:

- accidental / deliberate killing of individuals;

- accidental / deliberate destruction of eggs, bridges.

Indirectly, as a result of the other forms of impact:

- Loss of habitat area (including destruction of breeding habitats/shelters). Reduced habitat area may lead to reduced population numbers;

- Habitat alteration which may lead to a reduction in trophic resources and indirectly to a reduction in populations;

- Fragmentation of habitats that may affect the reproduction of individuals or prevent their access to favorable habitats on the site;

- Disturbance of species activity that may lead to relocation of individuals off-site.

The risk of mortality of individual wildlife species may occur at all stages of the plan (construction, operation, decommissioning).

Mortality risk - chiroptera

Regarding the mortality risk analysis for bat species, this was carried out using NatureScot's Bats and onshore wind turbines - survey, assessment and mitigation and takes into account both the risk of collision of bat species and the risk of barotrauma due to pressure differentials generated by wind turbine operation, the vulnerability of the species, their activity in the project area (assessed based on field observations), the location of the project and the types of interventions proposed.

Based on the methodology described the project site presents a moderate-value habitat, i.e. an area with buildings, trees or other structures with moderate-high potential to serve as roosts, which could be widely used by foraging bats. In terms of the size of the project it fits in with larger projects (> 40 turbines) with other wind energy projects under development within a 5km radius of it, with turbines greater than 100m in height. Analyzing the initial risk assessment matrix the project site falls into the high risk category.

Taking into consideration the information listed above as well as the monitoring data and using the general risk assessment matrix, the plan site has a high risk of collision and mortality for chiroptera.

However, existing methodologies do not allow the calculation of a potential number of casualties for bat species, as is done for bird species.

Bird mortality risk

The collision risk of the observed species is calculated according to the Scottish National Heritage guide "Wind farms and Birds: Calculating a theoretical collision risk assuming no avoiding action".

In a first step, the risk of collision was assessed with two different computational models, depending on the phenology of the species (transient migratory or resident/migratory spawning species):

- Calculation model 1: used for flights during migration but also transit flights

- Calculation model 2: used for irregular flights.

- Model for calculating the risk of birds colliding with wind turbines during migratory flights

This approach is used for birds regularly flying through the wind farm in a defined and known direction. This applies, for example, to seasonally migrating birds. The calculation method consists of 5 steps:

    - Define a "risk zone" equal to the width of the wind farm and a height equal to the maximum height of the tallest wind turbine. Cross sectional area W = width x height.

    - Estimated number of birds n flying through this risk window per year

    - Calculate the area A described by the wind farm turbines.  A = N x πR2 where N is the number of rotors and R is the rotor radius

    - Expressing the total rotor area as proportion A/W in the risk window

    - Identify the number of birds passing through the rotors as the number of birds in the risk window x proportion occupied by rotors = n x (A / W)

In the case of the Cerchezu wind farm, the following input data were used:

- Number of turbines (N): 46;

- Maximum height of wind turbines: 170 m;

- Rotor radius (R): 56 m;

- Average width of the risk zone: 9764 m;

- Area described by the wind farm turbines: 82354461 m ;2

- Area at risk (W): 2011384 m ;2

- Model for calculating the risk of birds colliding with wind turbines in irregular flights

This approach is appropriate for birds such as raptors that occupy a recognized territory and where observations have led to some understanding of the probability of flight distribution in that territory. The calculation method consists of 6 steps:

- Identification of a 'flight risk volume' Vw which is the area of the wind farm multiplied by the height of the turbines.

- Calculation of the combined volume of the wind farm rotor area Vr = N x πR2 x (d + l ) where N is the number of wind turbines, d is the rotor depth and l is the bird length.

- Estimation of bird occupancy n within the flight risk volume. This is the number of birds present multiplied by the time spent in flight in the flight risk volume within the period for which the collision is estimated.

- Determination of the volume occupied by birds within the volume of the wind farm rotors

n x ( Vr / Vw ) bird-sec

- Calculate the time it takes a bird to make a complete transit through the rotor:

t = (d + l ) / v where v m/sec is the speed of the bird through the rotor

- Calculation of the number of passages of birds through the rotors, dividing the total occupancy of the volume of the wind farm rotor area in bird-seconds by the transit time t

Number of birds passing through the rotors = n x ( Vr / Vw ) t

In the case of the Cerchezu wind farm, the following input data were used:

- Maximum height of wind turbines: 170 m;

- Wind farm area - 82354461 m ;3

- "Volume of flight risk" (Vw) - 16965019042 m ;3

- Rotor radius (R): 56 m;

- rotor depth (d) - 4 m.

The second step, regardless of the computational model used, was to calculate the probability of the bird being hit as it flies through the rotor area.

Total probability = (1/πR2 ) // p(r, ϕ) r dr dϕ

Again,

Probability p of collision for a bird at a radius r from the stump

p(r) = ( bΩ/2πv ) [ K | ± c sinγ + α c cosγ | + l for α < β

wαF for α > β 
where b = number of rotor blades

Ω = rotor angular speed (radians/sec)

c = blade chord width

γ = blade pitch angle

R = outer rotor radius

In order to obtain a more realistic value of collision risk for both Model 1 and Model 2, an avoidance rate recommended by Scottish National Heritage: "Use of Avoidance Rates in the SNH Wind Farm Collision Risk Model" was considered.This avoidance rate is different for each species and is presented in the table in the next section.

- RESULTS OF THE BIRD COLLISION RISK ASSESSMENT OF WIND TURBINES

Out of the total of 63 bird species for which the presence on the future Cerchezu wind farm has been established, only for 18 species it is relevant to assess the risk of collision with the blades of the wind turbines: these are the species of birds of prey or gliding birds, which are highlighted in red in the table below. Of the 446 individuals of the raptor and gliding bird species, only 197 were considered for the collision risk assessment, representing those individuals that were observed inside the wind farm, flying at heights that put them at risk of collision (50 - 170 m).

Table  5-1 Selection of bird species and individuals for collision risk assessment

| Nr. crt. | Species | Risk of collision | Total number of individuals observed | Total number of individuals observed inside the wind farm | Total no. of individuals observed at risk heights (50-170 m) | No. individuals Model 1 | No. individuals Model 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Accipiter gentilis | From | 2 | 2 | 0 | 0 | 0 |
| 2 | Accipiter nisus | From | 5 | 5 | 2 | 2 | 0 |
| 3 | Alauda arvensis | No | 550 | 500 | - | - | - |
| 4 | Anthus campestris | No | 10 | 6 | - | - | - |
| 5 | Apus apus | No | 99 | 83 | - | - | - |
| 6 | Pomarina eagle | From | 19 | 12 | 8 | 8 | 0 |
| 7 | Buteo buteo | From | 162 | 149 | 73 | 70 | 3 |
| 8 | Buteo rufinus | From | 19 | 11 | 4 | 4 | 0 |
| 9 | Caprimulgus europaeus | No | 1 | 1 | - | - | - |
| 10 | Carduelis carduelis | No | 100 | 91 | - | - | - |
| 11 | Carduelis chloris | No | 46 | 46 | - | - | - |
| 12 | Ciconia ciconia | From | 93 | 92 | 89 | 89 | 0 |
| 13 | Ciconia nigra | From | 1 | 0 | 0 | 0 | 0 |
| 14 | Circaetus gallicus | From | 1 | 1 | 1 | 1 | 0 |
| 15 | Circus aeruginosus | From | 37 | 27 | 3 | 3 | 0 |
| 16 | Circus cyaneus | From | 7 | 7 | 2 | 2 | 0 |
| 17 | Circus pygargus | From | 8 | 6 | 0 | 0 | 0 |
| 18 | Columba palumbus | No | 2 | 0 | - | - | - |
| 19 | Coracias garrulus | No | 26 | 19 | - | - | - |
| 20 | Corvus corax | From | 1 | 1 | 1 | 1 | 0 |
| 21 | Corvus frugilegus | From | 28 | 0 | 0 | 0 | 0 |
| 22 | Coturnix coturnix | No | 4 | 2 | - | - | - |
| 23 | Cuculus canorus | No | 6 | 3 | - | - | - |
| 24 | Delichon urbica | No | 50 | 41 | - | - | - |
| 25 | Emberiza calandra | No | 200 | 195 | - | - | - |
| 26 | Emberiza hortulana | No | 3 | 0 | - | - | - |
| 27 | Erithacus rubecula | No | 3 | 3 | - | - | - |
| 28 | Falco columbarius | From | 2 | 2 | 0 | 0 | 0 |
| 29 | Subbuteo falcon | From | 2 | 2 | 1 | 1 | 0 |
| 30 | Falco tinnununculus | From | 55 | 41 | 12 | 12 | 0 |
| 31 | Fringilla coelebs | No | 4 | 0 | - | - | - |
| 32 | Galerida cristata | No | 26 | 12 | - | - | - |
| 33 | Gallinago gallinago | No | 1 | 1 | - | - | - |
| 34 | Garrulus glandarius | No | 4 | 0 | - | - | - |
| 35 | Hieraaetus pennatus | From | 2 | 1 | 0 | 0 | 0 |
| 36 | Hirundo rustica | No | 219 | 184 | - | - | - |
| 37 | Lanius collurio | No | 58 | 50 | - | - | - |
| 38 | Lanius minor | No | 3 | 3 | - | - | - |
| 39 | Larus cachinnans | No | 88 | 74 | - | - | - |
| 40 | Luscinia luscinia | No | 1 | 1 | - | - | - |
| 41 | Luscinia megarhynchos | No | 4 | 3 | - | - | - |
| 42 | Melanocorypha calandra | No | 55 | 50 | - | - | - |
| 43 | Merops apiaster | No | 147 | 68 | - | - | - |
| 44 | White motacilla | No | 56 | 50 | - | - | - |
| 45 | Motacilla flava | No | 233 | 215 | - | - | - |
| 46 | Oenanthe oenanthe | No | 15 | 10 | - | - | - |
| 47 | Parus major | No | 3 | 3 | - | - | - |
| 48 | Passer domesticus | No | 70 | 62 | - | - | - |
| 49 | Perdix perdix | No | 30 | 29 | - | - | - |
| 50 | Pernis apivorus | From | 2 | 2 | 1 | 1 | 0 |
| 51 | Phasianus colchicus | No | 12 | 10 | - | - | - |
| 52 | Phoenicurus phoenicurus | No | 4 | 4 | - | - | - |
| 53 | Pica pica | No | 69 | 55 | - | - | - |
| 54 | Riparia riparia | No | 1 | 1 | - | - | - |
| 55 | Saxicola rubetra | No | 10 | 10 | - | - | - |
| 56 | Streptoptopelia decaocto | No | 29 | 26 | - | - | - |
| 57 | Streptopelia turtur | No | 5 | 0 | - | - | - |
| 58 | Sturnus vulgaris | No | 982 | 866 | - | - | - |
| 59 | Sylvia atricapilla | No | 3 | 3 | - | - | - |
| 60 | Turdus merula | No | 18 | 1 | - | - | - |
| 61 | Turdus philomelos | No | 7 | 0 | - | - | - |
| 62 | Turdus pilaris | No | 38 | 21 | - | - | - |
| 63 | Upupa epops | No | 51 | 34 | - | - | - |

Results of modeling the risk of collision of birds with wind turbines during migratory flights

Out of the 18 species for which the risk of collision with wind turbines was assessed using model calculation 1, in the case of migrations, taking into account the avoidance rate, for 12 species the risk of collision was positive, both reported on a calendar year basis. For none of the 12 species, the collision risk did not show an over-unitary result, suggesting that the mortality risk for the analyzed species is below one individual/year.

The results of the collision risk assessment of raptor and gliding bird species using model 1 calculation taking into account the avoidance rate are presented in the table below.

Table  5-2 Selection of bird species and individuals for collision risk assessment

| Nr. crt. | Species | Flight frequency | No. of birds flying through the risk area | Average no. of birds/ observation hour | Total number of hours the species is active in a year (180 days)* | Number of birds flying through the risk area all season | "p" | Risk of collision without taking into account an avoidance rate | Avoidance rate | Number of collisions per year with avoidance rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Accipiter gentilis | 1 | 0,00 | 0,00 | 2174,4 | 0,00 | 0,063 | 0,000 | 98% | 0,000 |
| 2 | Accipiter nisus | 1 | 0,55 | 0,01 | 2174,4 | 15,14 | 0,046 | 0,699 | 98% | 0,014 |
| 3 | Pomarina eagle | 1 | 2,20 | 0,03 | 2174,4 | 60,57 | 0,054 | 3,289 | 98% | 0,066 |
| 4 | Buteo buteo | 1 | 19,25 | 0,24 | 2174,4 | 529,95 | 0,057 | 29,959 | 98% | 0,599 |
| 5 | Buteo rufinus | 1 | 1,10 | 0,01 | 2174,4 | 30,28 | 0,052 | 1,573 | 98% | 0,031 |
| 6 | Ciconia ciconia | 1 | 24,48 | 0,31 | 2174,4 | 673,79 | 0,074 | 49,898 | 98% | 0,998 |
| 7 | Ciconia nigra | 2 | 0,00 | 0,00 | 2174,4 | 0,00 | 0,066 | 0,000 | 98% | 0,000 |
| 8 | Circaetus gallicus | 1 | 0,28 | 0,00 | 2174,4 | 7,57 | 0,057 | 0,434 | 98% | 0,009 |
| 9 | Circus aeruginosus | 1 | 0,83 | 0,01 | 2174,4 | 22,71 | 0,056 | 1,263 | 98% | 0,025 |
| 10 | Circus cyaneus | 1 | 0,55 | 0,01 | 2174,4 | 15,14 | 0,053 | 0,808 | 99% | 0,008 |
| 11 | Circus pygargus | 1 | 0,00 | 0,00 | 2174,4 | 0,00 | 0,050 | 0,000 | 98% | 0,000 |
| 12 | Corvus corax | 1 | 0,28 | 0,00 | 2174,4 | 7,57 | 0,049 | 0,374 | 98% | 0,007 |
| 13 | Corvus frugilegus | 2 | 0,00 | 0,00 | 2174,4 | 0,00 | 0,047 | 0,000 | 98% | 0,000 |
| 14 | Falco columbarius | 1 | 0,00 | 0,00 | 2174,4 | 0,00 | 0,044 | 0,000 | 98% | 0,000 |
| 15 | Subbuteo falcon | 1 | 0,28 | 0,00 | 2174,4 | 7,57 | 0,042 | 0,320 | 98% | 0,006 |
| 16 | Falco tinnununculus | 1 | 3,30 | 0,04 | 2174,4 | 90,85 | 0,044 | 4,023 | 95% | 0,201 |
| 17 | Hieraaetus pennatus | 1 | 0,00 | 0,00 | 2174,4 | 0,00 | 0,051 | 0,000 | 98% | 0,000 |
| 18 | Pernis apivorus | 1 | 0,28 | 0,00 | 2174,4 | 7,57 | 0,054 | 0,409 | 98% | 0,008 |

* Average day length in Romania (12.08 hours) in the study area

Model results for calculating the risk of birds colliding with wind turbines in irregular flights

Out of the 18 species for which the risk of collision with wind turbines was assessed using calculation model 2, in the case of irregular flights, taking into account the avoidance rate, for only one species the risk of collision was positive on a calendar year basis: Buteo buteo. However, based on the information collected so far, the collision risk is low, 0.006 individuals per year.

The results of the collision risk assessment of raptor and gliding bird species using model 2 calculation taking into account the avoidance rate are presented in the table below.

Table  5-3 Collision risk of predatory and gliding bird species using model 2 calculation

| Nr. crt. | Species | No. of birds flying through the risk area | "flight risk volume" Vw (bird-sec) | Estimated occupancy time per day (h/day) | Total days of species activity | 'n' - Estimated occupancy time per season (bird-sec) | "l" - length of bird (m) | 'v' - flight speed of the bird (m/s) | "Vr" - the combined volume of the combined area of action of the rotors (mc) | Estimated time of bird occupancy within the combined volume of the rotor flight range (bird-sec) | "t" - Time taken for a bird to transit through the rotor area (sec) | No. of birds passing through the rotors | "p" | Risk of collision without avoidance | Avoidance rate | No. of collisions per year with avoidance rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Accipiter gentilis | 0 | 0,00 | 0,00 | 0 | 0,00 | 0,69 | 11,80 | 4446830,59 | 0,00 | 0,40 | 0,00 | 0,0634 | 0,000 | 98% | 0,000 |
| 2 | Accipiter nisus | 0 | 0,00 | 0,00 | 180 | 0,00 | 0,35 | 11,80 | 4124459,08 | 0,00 | 0,37 | 0,00 | 0,0462 | 0,000 | 98% | 0,000 |
| 3 | Pomarina eagle | 0 | 0,00 | 0,00 | 180 | 0,00 | 0,63 | 14,75 | 4385200,74 | 0,00 | 0,31 | 0,00 | 0,0543 | 0,000 | 98% | 0,000 |
| 4 | Buteo buteo | 3 | 0,09 | 0,01 | 180 | 6938,53 | 0,54 | 11,55 | 4304607,87 | 2,08 | 0,39 | 5,28 | 0,0565 | 0,299 | 98% | 0,006 |
| 5 | Buteo rufinus | 0 | 0,00 | 0,00 | 180 | 0,00 | 0,58 | 11,55 | 4337793,17 | 0,00 | 0,40 | 0,00 | 0,0519 | 0,000 | 98% | 0,000 |
| 6 | Ciconia ciconia | 0 | 0,00 | 0,00 | 180 | 0,00 | 1,08 | 14,15 | 4811868,93 | 0,00 | 0,36 | 0,00 | 0,0741 | 0,000 | 98% | 0,000 |
| 7 | Ciconia nigra | 0 | 0,00 | 0,00 | 0 | 0,00 | 0,98 | 15,55 | 4717053,77 | 0,00 | 0,32 | 0,00 | 0,0663 | 0,000 | 98% | 0,000 |
| 8 | Circaetus gallicus | 0 | 0,00 | 0,00 | 0 | 0,00 | 0,65 | 13,60 | 4404163,78 | 0,00 | 0,34 | 0,00 | 0,0573 | 0,000 | 98% | 0,000 |
| 9 | Circus aeruginosus | 0 | 0,00 | 0,00 | 180 | 0,00 | 0,52 | 11,55 | 4285644,84 | 0,00 | 0,39 | 0,00 | 0,0556 | 0,000 | 98% | 0,000 |
| 10 | Circus cyaneus | 0 | 0,00 | 0,00 | 90 | 0,00 | 0,48 | 11,60 | 4247718,78 | 0,00 | 0,39 | 0,00 | 0,0533 | 0,000 | 99% | 0,000 |
| 11 | Circus pygargus | 0 | 0,00 | 0,00 | 0 | 0,00 | 0,45 | 12,55 | 4219274,23 | 0,00 | 0,35 | 0,00 | 0,0501 | 0,000 | 98% | 0,000 |
| 12 | Corvus corax | 0 | 0,00 | 0,00 | 180 | 0,00 | 0,50 | 14,50 | 4266681,81 | 0,00 | 0,31 | 0,00 | 0,0494 | 0,000 | 98% | 0,000 |
| 13 | Corvus frugilegus | 0 | 0,00 | 0,00 | 180 | 0,00 | 0,45 | 14,50 | 4219274,23 | 0,00 | 0,31 | 0,00 | 0,0471 | 0,000 | 98% | 0,000 |
| 14 | Falco columbarius | 0 | 0,00 | 0,00 | 90 | 0,00 | 0,33 | 12,75 | 4105496,05 | 0,00 | 0,34 | 0,00 | 0,0439 | 0,000 | 98% | 0,000 |
| 15 | Subbuteo falcon | 0 | 0,00 | 0,00 | 0 | 0,00 | 0,33 | 14,55 | 4105496,05 | 0,00 | 0,30 | 0,00 | 0,0423 | 0,000 | 98% | 0,000 |
| 16 | Falco tinnununculus | 0 | 0,00 | 0,00 | 180 | 0,00 | 0,335 | 12,75 | 4110236,81 | 0,00 | 0,34 | 0,00 | 0,0443 | 0,000 | 95% | 0,000 |
| 17 | Hieraaetus pennatus | 0 | 0,00 | 0,00 | 0 | 0,00 | 0,49 | 13,3 | 4257200,29 | 0,00 | 0,34 | 0,00 | 0,0506 | 0,000 | 98% | 0,000 |
| 18 | Pernis apivorus | 0 | 0,00 | 0,00 | 0 | 0,00 | 0,56 | 13,35 | 4323570,90 | 0,00 | 0,34 | 0,00 | 0,0540 | 0,000 | 98% | 0,000 |

- Interpretation of results

Of the 18 species for which the risk of collision with wind turbines has been assessed, 12 species show a positive final collision risk reported at one year, namely for the species: Accipiter nisus (0.014 individuals/yr), Aquila pomarina (0.066 individuals/yr), Buteo buteo (0.605 individuals/yr), Buteo rufinus (0.031 individuals/yr), Ciconia ciconia (0.998 individuals/yr), Circaetus gallicus (0.009 individuals/yr), Circus aeruginosus (0,025 individuals/year), Circus cyaneus (0,008 individuals/year), Corvus corax (0,007 individuals/year), Falco subbuteo (0,006 individuals/year), Falco tinnunculus (0,201 individuals/year), Pernis apivorus (0,008 individuals/year). Of these, none of them show annual supra-unit values.

All 12 species for which the final positive collision risk results in a one-year positive final collision risk are bird species that, according to the Red List of European Bird Species (Birdlife International, 2015), are classified as species of Least Concern. At Romanian level, according to the Red List of bird species in Romania, three species are classified as Near Threatened, one is assessed as Vulnerable and two have not been assessed (Not Evaluated).

In order to determine the significance of the impact of the Cerchezu wind farm operation on raptor and gliding bird species, the population size of the species in the study area was used (where possible) as a reference, wishing to compare the collision victims with the relevant population of the species, either the nesting or wintering population or the passage population. Thus, the population values of the species in the neighboring Natura 2000 sites ROSPA0036 Dumbraveni and ROSPA0166 Plopeni-Chirnogeni were consulted, considering that a 1% reduction of the minimum population within the site represents a significant impact on the species, and may hinder the achievement of the site's conservation objectives. However, there have been situations where within the conservation objectives of the sites ROSPA0036 Dumbraveni and ROSPA0166 Plopeni-Chirnogeni did not show species population values or did not show relevant population values (such as the case of the species Accipiter gentilis, Accipiter nisus, Buteo buteo, Buteo rufinus, Ciconia ciconia, Ciconia nigra, Circaetus gallicus, Circus aeruginosus, Corvus corax, Corvus frugilegus, Falco columbarius, Falco subbuteo, Falco tinnunculus, Pernis apivorus). The national population sizes of the species as reported by Romania in accordance with Art. 12 of the Birds Directive (available at https://nature-art12.eionet.europa.eu/article12/summary) were consulted in this situation, considering that a 0.05% reduction of the national minimum population is considered a significant impact.

### Quantifying the impact

- Presentation of the impact quantification methodology

The quantification of the effects due to the implementation of this plan has been done in a cumulative way, considering:

- possible temporal and spatial overlap of the interventions needed to implement the plan ;

- the contribution of other PPs, as well as other activities generating similar effects in the implementation area of the plan .

- Quantifying habitat loss

Expressed in area units (hectares). The loss is expressed as a percentage as a share of the total area of the Natura 2000 habitat or habitat of the species in the site and not in relation to the total area of the Natura 2000 site.

Depending on the way the parameters of the conservation objectives were formulated, habitat loss was calculated separately for: resting habitats, breeding habitats, foraging habitats, other habitat types of the species.

In a precautionary way, the assessment of the degree of habitat alteration will take into account the maximum area that may be affected (worst case scenario) without any measures to avoid or limit this impact. Consideration of spatial and temporal dynamics in quantifying the impact will be done using a case-by-case approach, depending on the habitat affected and the nature of the alteration (pollutant identity, invasive species identity).

Depending on how the conservation target parameters were formulated, habitat alteration was calculated based on the units of measurement provided for each parameter (e.g. % invasive species, % bare soil, water quality class, other units of measurement).

- Quantifying the impact of fragmentation

It will be done in such a way as to respond to the targets and units of measurement set out in the CST.

- Quantifying the impact of disruption

It was realized like this:

- the potentially affected areas for each species have been estimated and their spatial localization is presented;

- the impact has been quantified based on the targets and units of measurement provided by the CSO. As an example, if the CSO parameter being analyzed is the 'distribution pattern' of the species, the area (and/or duration) over which disturbance may occur.

- Quantifying collision risk

There will be no risk of collision, while avoiding any risk of accidental electrocution of birds.

All potential environmental effects identified for each activity that is subject to impact assessment are analyzed in order to determine the value of the final impact.

This value is given by the following formula:

Impact = Consequence X Probability

Multi-criteria analysis is widely used to identify significant effects. Common criteria are established for assessing the significance of an impact, which are quantified for each individual PP.

The significance of an impact can be major (significant), moderate, minor, negligible, negligible, of no value or positive.

The magnitude of the impact, which is given by the characteristics of the PP and the effects it generates, such as:

- Type of effect: direct, indirect, secondary, cumulative;

- Reversibility of effect: reversible, irreversible;

- Duration of effect: temporary, short-term, long-term.

- Type of impact

- Direct - impacts resulting from the direct interaction between a plan activity and an environmental factor (e.g. habitat occupancy during construction);

- Indirect - impacts resulting from other activities or as a consequence or circumstance of PP (e.g. increased road traffic);

- Secondary - direct or indirect impact as a result of repeated interaction between PP components and environmental factors (e.g. direct secondary impact - an impact on wildlife due to collisions; indirect secondary impact - an impact on wildlife due to habitat loss);

- Cumulative - impact that acts together with another impact (including the impacts of other plans/projects/activities), affecting the same environmental factor or receptor (e.g. the combined effect of other similar projects in the area of influence).

- Impact reversibility

- Reversible - an impact is reversible when the affected environmental factor (the receptor) can return to its original state (before the impact action), e.g. water turbidity can return to its original state after the cause of the turbidity - construction activities - has ceased;)

- Irreversible - an impact is irreversible if the environmental factor can no longer return to its original state (e.g. permanent land occupation).

- Duration of impact

- Temporary - the impact is short-lived and possibly intermittent/occasional (e.g. temporary earth deposits during works);

- Short term - the impact is expected to be active for a limited, short period of time and will cease entirely upon completion of the activity causing it (e.g. noise and vibration generated during construction). The impact is also short-lived if it is eliminated by appropriate measures or the environmental factor is restored (e.g. shutting down a facility if the noise it produces affects receptors);

- Long-term - impacts occur over a long period of time (over the entire period of operation), but cease with the closure of the PP (e.g. plant noise, emissions, etc.). Likewise, impacts are long-lasting even if intermittent, but occur throughout the lifetime of the PP (e.g. disturbance to biodiversity during maintenance operations of the installation);

- Permanent - impacts occur in all phases of the PP and remain active even after the PP closes. In other words, it causes permanent changes to biotic and abiotic resources or receptors.

The assessment of the significance of the impact was based on the following two common criteria used in biodiversity impact assessment:

- the magnitude of the effect which takes into account the characteristics of the change (timing, scale, size and duration of the impact) that would be likely to affect the target receptor as a result of the implementation of the proposed PP;

- Sensitivity of the area taking into account the changes and capacity to adapt to the changes brought to the area by the implementation of the PP objectives.

Identification and quantification of impacts during construction and operation and are presented in the table below.

F.    Impact significance.

Two classes were used to assess significance:

- significant

- insignificant.

The following parameters were used to assess the significance of the impact:

Quantitative Parameters - percentage of damage of the target value

As a guideline percentage it was considered that habitat loss should be <1% (analyzed on a case-by-case basis, taking into account the criteria below), and for priority habitats any habitat loss is considered as a significant impact.

Qualitative Parameters

- Conservation status of the species/habitat at site, biogeographical region and national level

- Presence of the species/habitat in other Natura 2000 sites;

- Area where the habitat is affected (marginal or central)

- Species at the edge of its range

Ecological functions

- Maintain/restore ecological connectivity;

- Maintenance of critical physico-chemical parameters such as water quality, air quality, noise level.

Impact form parameters such as:

- Project implementation stage (construction, operation, decommissioning);

- Type of impact (positive, negative);

- Nature of impact (direct, secondary, indirect, cumulative);

- Spatial extent (local, zonal, county, regional, national, transboundary);

- Duration (short, medium, long term);

- Frequency of occurrence (accidental, intermittent, periodic, permanent, one-off/temporary);

- Likelihood of occurrence (uncertain, unlikely, probable, very likely);

- Reversibility of effects (reversible, irreversible).

The following table presents the parameters that have been considered for the impact assessment.

Table  5-3 Parameters taken into account to assess impacts

| Evaluation parameter | Evaluation parameter variables | Description of the characteristics of the variables of the assessment parameters |
| --- | --- | --- |
| Impact type | Positive | The changes contribute to improving the condition/achieving the objectives of the component under analysis. |
|  | Negative | The changes contribute to the worsening of the state/non-achievement of the objectives of the component under analysis. |
| Nature impact | Direct | The main form of impact produced by the occurrence of an effect. |
|  | Secondary | Form of impact generated by a direct impact. |
|  | Indirect | The form of impact that occurs not because of an effect generated by the project, but because of activities that are encouraged to occur as a consequence of the project. |
| Spatial expansion | Local | Equates to a small number of habitat locations on the site. |
|  | Zonal | Equivalent to the entire area of habitat on the site. |
|  | County | Equivalent to the surface area of several sites. |
|  | Regional | Equivalent to the biogeographical region level. |
|  | National | The impact produces changes felt throughout the country. |
|  | Cross-border | The impact is felt in neighboring countries. |
| Duration | Short deadline | The impact lasts up to 1 year. |
|  | Medium term | Impacts occur during construction and for a short post-construction period. |
|  | Long term | The impact is felt over several years. |
| Frequency | Accidental | The impact only occurs as a result of an accident (an accidental pollution). |
|  | Flashing | The impact occurs repeatedly/discontinuously with unknown frequency. |
|  | Periodical | The impact occurs repeatedly, with a known frequency. |
|  | Permanent | The impact is continuous after the moment of onset. |
|  | One-off/ temporary | The impact occurs once in one of the project phases. Most often associated with a short duration. |
| Probability | Unlikely | The likelihood of an impact is low - it is possible. |
|  | Probably | The probability of an impact is high - it is very likely to occur. |
|  | Very likely | Impact is safe. |
| Reversibility | Reversible | Once the impact is gone, the affected component can return to its original condition. |
|  | Irreversible | The impact does not allow a return to baseline conditions of the affected environmental component. |

Identification and quantification of impacts during construction and operation and are presented in the table below

Table  5-4 Identification and quantification of impacts

| Intervention |  | Impact | Impacts direct | Impacts indirect | Impacts secondary | Impacts cumulative | Impacts impacts and long | Species/habitat | Parameter / Target affected | Quantification impact | How to quantify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus | Habitat area of the species | 0 ha of protected area Areas temporarily occupied by proeict outside the boundaries of protected areas where the presence of sopecia has been reported: 0.9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the construction elements of the energy park Potential disruption of up to 2 to 3 The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) The impact will be felt outside the protected area at the boundary of the wind farm, at the boundary/neighborhood of the working front Noise level modeling study results | Spatial analysis of field inventory results Noise level estimates |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Habitat area of the species |  |  |
|  | Earthworks (land levelling, excavation, excavation, backfilling) | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size |  |  |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size | 1/individual (accidental) (outside the boundary of the protected area, within the wind farm) | Risk analysis of mortality of the species |
|  | Installation of installations/ equipment | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Areas temporarily occupied by proeict outside the boundaries of protected areas where the presence of sopecia has been reported: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis of field inventory results |
|  | Construction of the electrical network to discharge the energy produced by the wind power plant to the transformer station and telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 30-40 square meters in the working front area, land outside the protected area (agricultural land). | Spatial analysis of field inventory results |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |
| Operate | Energy production activities | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Spermophilus citellus | Population size (distribution pattern) | Potential disruption of up to 2 to 3 The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) Impacts will be felt outside the protected area of the wind farm boundary, in the vicinity of the wind turbines Noise level modeling study results | Noise level estimates |
|  |  | Lighting at night | PAS/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not identified | No potentially affected species have been identified | Not the case | Not the case | Not the case |
|  |  | Waste generation | AH/PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not identified | Not identified | Not the case | Not the case | Not the case |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus | Population size | 1/individual (accidental) (outside the protected area boundary, within the wind farm boundary) | Risk analysis of mortality of the species |
|  |  | Collision with turbine blades | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing) | Long-term direct impact | Miniopterus schreibersii Rhinolophus mehelyi | Population size | Miniopterus schreibersii - maximum one individual every 5 years | Precautionary principle And literature data Eurobats Report 2023 |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing | Long-term direct impact | Miniopterus schreibersii | Population size | Miniopterus schreibersii - maximum one individual every 5 years | Precautionary principle And literature data Eurobats Report 2023 |
|  |  | Introducing invasive species | AH | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus | Habitat area of the species | 0 ha of protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0.9 ha temporarily occupied, area of site organization | Spatial analysis and data received from the beneficiary |
|  |  | Temporary occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus | Habitat area of the species |  |  |
|  |  | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus | Population size (distribution pattern) | Potential disruption of up to 2 to 3 The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) Impacts will be felt outside the protected area of the wind farm boundary, in the vicinity of the wind turbines | Noise level modeling study results |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus | Population size | 1/individual (accidental) outside the boundary of the protected area, within the wind farm boundary) | Risk analysis of mortality of the species |
|  |  | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Areas temporarily occupied by proeict outside the boundaries of protected areas where the presence of sopecia has been reported: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park The impact will be felt outside the protected area at the boundary of the wind farm, at the boundary/neighborhood of the working front | Spatial analysis of field inventory results |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Direct short-term impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 30-40 square meters in the working front area, agricultural land outside the protected area | Spatial analysis of field inventory results |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |
| ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mustela eversmanii | Habitat area of the species | 0 ha of protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis and data received from the beneficiary |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Habitat area of the species |  |  |
|  | Earthworks (land levelling, digging, excavation, backfilling) | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Habitat area of the species |  |  |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size | 1/individual (accidental) (outside the protected area boundary, within the wind farm boundary) | Risk analysis of mortality of the species |
|  | Installation of installations/ equipment | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Areas temporarily occupied by proeict outside the boundaries of protected areas where the presence of sopecia has been reported: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park The impact will be felt outside the protected area at the boundary of the wind farm, at the boundary/neighborhood of the working front | Spatial analysis of field inventory results |
|  | Construction of the electrical network to discharge the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 30-40 square meters in the working front area, land outside the protected area (agricultural land). | Spatial analysis of field inventory results |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |
| Operate | Energy production activities | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Spermophilus citellus, Mustela eversmanii | Population size | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at distances of maximum 1.5 km around the work area. (see section 1.1.7, fig 1-10a) Impacts will be felt outside the protected area of the wind farm boundary, in the vicinity of the wind turbines | Noise level modeling study results |
|  |  | Lighting at night | PAS/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | No potentially affected species have been identified | Not the case | Not the case | Not the case |
|  |  | Waste generation | AH/PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not identified | Not identified | Not the case | Not the case | Not the case |
|  |  | Collision with turbine blades | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing | Long-term direct impact | Miniopterus schreibersii, Rhinolophus ferrumequinum | Population size | Rhinolophus ferrumequinum Miniopterus schreibersii ca. 1-2 individuals/year (accidental) | Literature data Eurobats Report 2023 |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing | Long-term direct impact | Miniopterus schreibersii, Rhinolophus ferrumequinum | Population size | Rhinolophus ferrumequinum Miniopterus schreibersii - maximum one individual every 5 years | Literature data Eurobats 2023 Report |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | direct impact | Spermophilus citellus, Mustela eversmanii | Population size | 1/individual (accidental) | Risk analysis of mortality of the species |
|  |  | Introducing invasive species | AH | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mustela eversmanii | Habitat area of the species | 0 ha of protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis and data received from the beneficiary |
|  |  | Temporary occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mustela eversmanii |  |  |  |
|  |  | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mustela eversmanii | Population size (distribution pattern) | Potential disruption of up to 2 to 3 The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) Impacts will be felt outside the protected area of the wind farm boundary, in the vicinity of the wind turbines 1/individual (accidental) (outside the protected area boundary, within the wind farm boundary) | Noise level modeling study results |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mustela eversmanii | Population size |  | Risk analysis of mortality of the species |
|  |  | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Areas temporarily occupied by proeict outside the boundaries of protected areas where the presence of sopecia has been reported: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park The impact will be felt outside the protected area at the boundary of the wind farm, at the boundary/neighborhood of the working front | Spatial analysis of field inventory results |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 30-40 square meters in the working front area, agricultural land outside the protected area | Spatial analysis of field inventory results |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |
| ROSPA0166 Plopeni - Chirnogeni |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Anthus campestris Buteo rufinus Circus cyaneus Falco columbarius Falco vespertinus Lanius minor | Population size trend Population size | 0 individuals The impact will be felt outside the boundary of the protected area, in the area of the working fronts, with the effect of temporarily removing foraging/resting species from these areas, with the species moving to neighboring areas with similar conditions. The effect will be insignificant and temporary, the works will be phased. | Analysis of inventories and field monitoring based on observed species behavior. GIS analysis |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size trend Population size | The impact will be felt outside the boundary of the protected area, in the area of the working fronts, with the effect of temporarily removing foraging/resting species from these areas, with the species moving to neighboring areas with similar conditions. The effect will be insignificant and temporary, the works will be phased | Analysis of inventories and field monitoring based on observed species behavior. GIS analysis |
|  | Earthworks (land levelling, digging, excavation, backfilling) | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size trend Population size | Potential disruption of up to 2 to 3 The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) The impact occurs outside the boundaries of the protected area | Analysis of inventories and field monitoring based on observed species behavior. GIS analysis Noise level modeling |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  | Installation of installations/ equipment | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  | Construction of the electrical network to discharge the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
| Operate | Energy production activities | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Anthus campestris Buteo rufinus Circus cyaneus Falco columbarius Falco vespertinus Lanius minor | Population size trend | Potential disruption of up to 2 to 3 The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) The impact will be felt outside the protected area, within the wind farm boundary, in the vicinity of the wind turbines | Noise level modeling study |
|  |  | Lighting at night | PAS/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  |  | Waste generation | AH/PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  |  | Collision with turbine blades | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing | Long-term direct impact | Anthus campestris Buteo rufinus Circus cyaneus Falco columbarius Falco vespertinus Lanius minor | Population size | Anthus campestris - insignificant Buteo rufinus - 0.193 individuals/ year (1 individual every approx. 5 years), Circus cyaneus - 0,012 individuals/ year (1 individual every approx. 83 years) Falco columbarius - not significant Falco vespertinus - not significant Lanius minor - insignificant, it is a small species, the effects on these species are rarely considered to be significant at population level due to (in most cases) relatively large populations sizes | Scottish National Heritage guide: "Wind farms and Birds: Calculating a theoretical collision risk assuming no avoiding action" |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Anthus campestris Buteo rufinus Circus cyaneus Falco columbarius Falco vespertinus Lanius minor | Population size trend | Negligible. The impact will be felt outside the protected area, in the area of the working fronts, with the effect of temporarily removing wildlife using these areas for feeding and resting. These species will move to neighboring areas with similar habitat conditions. The effect will be of a temporary and insignificant nature, concentrated on specific work periods. The works will be phased and impact monitoring and prevention measures will be implemented to ensure the protection of wildlife in these areas. The works will also respect the periods of prohibition on interventions during sensitive periods for vulnerable species. 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Temporary occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Anthus campestris Buteo rufinus Circus cyaneus Falco columbarius Falco vespertinus Lanius minor | Population size trend | Insignificant. The impact will be felt outside the protected area in the area of the working fronts, with the effect of temporarily removing foraging/resting species from these areas, with the species moving to neighboring areas with similar conditions. The effect will be insignificant and temporary, the works will be phased 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Anthus campestris Buteo rufinus Circus cyaneus Falco columbarius Falco vespertinus Lanius minor | Population size trend | Insignificant considering the results of the estimates of the noise level likely to be recorded in the area of the working front, within the park's footprint, during the construction period, similarly during the decommissioning period (see section 1.1.7, fig 1-10a) The impact will be felt outside the protected area, within the wind farm boundary, in the vicinity of the wind turbines | Noise level modeling result |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Anthus campestris Buteo rufinus Circus cyaneus Falco columbarius Falco vespertinus Lanius minor | Population size trend | Insignificant. The impact will be felt outside the protected area in the area of the working fronts, with the effect of temporarily removing foraging/resting species from these areas, with the species moving to neighboring areas with similar conditions. The effect will be insignificant and temporary, the works will be phased Maximum 1 individual of Anthus campestris at this stage | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
| BG0000569 Kardam |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Lynx lynx (only if presence is confirmed) | Habitat area of the species | 0 ha of protected area Occupied areas outside the protected area where presence has been reported following monitoring : 0.9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by construction elements of the energy park Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) Impacts will be felt outside the protected area of the wind farm boundary, in the vicinity of the wind turbines | Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the beneficiary Noise level estimates |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Habitat area of the species |  |  |
|  | Earthworks (land leveling, excavation, excavation, backfilling) | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size (distribution) |  |  |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size | 1 individual (accidental) (outside the boundary of the protected area, within the wind farm) | Literature data Precautionary principle Risk analysis of mortality of the species |
|  | Installation of installations/ equipment | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  | Construction of the electrical network to discharge the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  | Land reclamation work on completion of construction | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | During the construction period of the project, a potential risk of damage by invasive plant species (Ailanthus altissima, Ambrosia artemisiifolia, Xanthium orientale, Conyza canadensis potentially present in the vicinity of the roads) was identified as a potential risk of damage to the habitat, mainly by anemocory, during the construction phase (during excavation works through excavated soil and transportation activities - on site machinery or vehicles) Considering that the project does not foresee any kind of works on the site, but only in its vicinity, the spread of invasive species is possible only at the edge of the habitat. According to the results of data from specialized studies (Invasive Species Management in Romania, beneficiary Ministry of Environment) the richness of invasive species in the project area is very low (1-3 species), the hot spots of distribution of invasive species are in the insignificant range. 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |
| Operate | Energy production activities | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna | Population size | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) Impacts will be felt outside the protected area of the wind farm boundary, in the vicinity of the wind turbines | Noise modeling |
|  |  | Lighting at night | PAS/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  |  | Waste generation | AH/PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna | Population size | 1 individual (accidental) (outside the boundary of the protected area, within the wind farm) | Literature data Risk analysis of mortality of the species |
|  |  | Collision with turbine blades | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformation, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing | Long-term direct impact | Rhinolophus mehelyi Rhinolophus hipposideros Rhinolophus ferrumequinum Rhinolophus euryale Rhinolophus blasii Miniopterus schreibersii Myotis myotis Myotis emarginatus Myotis capaccinii Myotis blythii Myotis bechsteinii Barbastella barbastellus | Population size | Rhinolophus mehelyi - maximum one individual every 10 years Rhinolophus hipposideros - maximum one individual every 10 years Rhinolophus ferrumequinum - maximum two individuals every 10 years Rhinolophus euryale - maximum one individual every 10 years Rhinolophus blasii - maximum one individual every 10 years Miniopterus schreibersii - maximum two individuals every 10 years Myotis myotis - maximum two individuals every 10 years Myotis emarginatus - maximum one individual every 10 years Myotis capaccinii - maximum one individual every 10 years Myotis blythii - maximum two individuals every 10 years Myotis bechsteinii - maximum one individual every 10 years Barbastella barbastellus - maximum two individuals every 10 years | literature data |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing |  | Rhinolophus mehelyi Rhinolophus hipposideros Rhinolophus ferrumequinum Rhinolophus euryale Rhinolophus blasii Miniopterus schreibersii Myotis myotis Myotis emarginatus Myotis capaccinii Myotis blythii Myotis bechsteinii Barbastella barbastellus | Population size | Rhinolophus mehelyi - maximum one individual every 10 years Rhinolophus hipposideros - maximum one individual every 10 years Rhinolophus ferrumequinum - maximum two individuals every 10 years Rhinolophus euryale - maximum one individual every 10 years Rhinolophus blasii - maximum one individual every 10 years Miniopterus schreibersii - maximum two individuals every 10 years Myotis myotis - maximum two individuals every 10 years Myotis emarginatus - maximum one individual every 10 years Myotis capaccinii - maximum one individual every 10 years Myotis blythii - maximum two individuals every 10 years Myotis bechsteinii - maximum one individual every 10 years Barbastella barbastellus - maximum two individuals every 10 years | literature data |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case | Not the case | Not the case | Not the case |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Lynx lynx (only if presence is confirmed) | Habitat area of the species | 0 ha of protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0.9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis and data received from the beneficiary Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the beneficiary |
|  |  | Temporary occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Lynx lynx (only if presence is confirmed) | Population size | 0 ha fragmented habitats in the protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis and data received from the beneficiary Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the beneficiary |
|  |  | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Lynx lynx (only if presence is confirmed) | Population size (distribution pattern) | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) | Spatial analysis and data received from the beneficiary Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the beneficiary Noise level measurements |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Lynx lynx (only if presence is confirmed) | Population size | 1/individual (accidental) (outside the boundary of the protected area, within the wind farm) | The precautionary principle and evidence from the literature Risk analysis of mortality of the species |
|  |  | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park The impact will be felt outside the protected area at the boundary of the wind farm, at the boundary/neighborhood of the working front | Spatial analysis of field inventory results |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Direct short-term impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 30-40 square meters in the working front area, agricultural land outside the protected area | Spatial analysis of field inventory results |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |
| BG0000570 Izvorovo - Kraishte |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Canis lupus (only if presence is confirmed) Lynx lynx (only if presence is confirmed) | Habitat area of the species | 0 ha of protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0.9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis and data received from the beneficiary Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the beneficiary |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Habitat area of the species |  |  |
|  | Earthworks (land levelling, excavation, excavation, backfilling) | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Habitat area of the species |  |  |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  | Population size | 1/individual (accidental) (outside the boundary of the protected area, within the wind farm) | The precautionary principle and evidence from the literature Risk analysis of mortality of the species |
|  | Installation of installations/ equipment | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Areas temporarily occupied by proeict outside the boundaries of protected areas where the presence of sopecia has been reported: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis of field inventory results |
|  | Construction of the electrical network to discharge the energy produced by the wind power plant to the transformer station and telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 30-40 square meters in the working front area, land outside the protected area (agricultural land). | Spatial analysis of field inventory results |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |
| Operate | Energy production activities | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Canis lupus (only if presence is confirmed) Lynx lynx (only if presence is confirmed) | Population size (distribution pattern) | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) Impacts will be felt outside the protected area of the wind farm boundary, in the vicinity of the wind turbines | Spatial analysis and data received from the beneficiary Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the beneficiary Wind farm noise modeling |
|  |  | Lighting at night | PAS/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | No potentially affected species have been identified | Not the case | Not the case | Not the case |
|  |  | Waste generation | AH/PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not identified | Not identified | Not the case | Not the case | Not the case |
|  |  | Collision with turbine blades | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Rhinolophus mehelyi Rhinolophus hipposideros Rhinolophus ferrumequinum Rhinolophus euryale Rhinolophus blasii Miniopterus schreibersii Myotis myotis Myotis emarginatus Myotis capaccinii Myotis blythii Myotis bechsteinii Barbastella barbastellus | Population size | Rhinolophus mehelyi - maximum one individual every 10 years Rhinolophus hipposideros - maximum one individual every 10 years Rhinolophus ferrumequinum - maximum two individuals every 10 years Rhinolophus euryale - maximum one individual every 10 years Rhinolophus blasii - maximum one individual every 10 years Miniopterus schreibersii - maximum two individuals every 10 years Myotis myotis - maximum two individuals every 10 years Myotis emarginatus - maximum one individual every 10 years Myotis capaccinii - maximum one individual every 10 years Myotis blythii - maximum two individuals every 10 years Myotis bechsteinii - maximum one individual every 10 years Barbastella barbastellus - maximum two individuals every 10 years | The precautionary principle and evidence from the literature Eurobats Report 2023 |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Rhinolophus mehelyi Rhinolophus hipposideros Rhinolophus ferrumequinum Rhinolophus euryale Rhinolophus blasii Miniopterus schreibersii Myotis myotis Myotis emarginatus Myotis capaccinii Myotis blythii Myotis bechsteinii Barbastella barbastellus | Population size | Rhinolophus mehelyi - maximum one individual every 10 years Rhinolophus hipposideros - maximum one individual every 10 years Rhinolophus ferrumequinum - maximum two individuals every 10 years Rhinolophus euryale - maximum one individual every 10 years Rhinolophus blasii - maximum one individual every 10 years Miniopterus schreibersii - maximum two individuals every 10 years Myotis myotis - maximum two individuals every 10 years Myotis emarginatus - maximum one individual every 10 years Myotis capaccinii - maximum one individual every 10 years Myotis blythii - maximum two individuals every 10 years Myotis bechsteinii - maximum one individual every 10 years Barbastella barbastellus - maximum two individuals every 10 years | The precautionary principle and evidence from the literature Eurobats Report 2023 |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Canis lupus (only if presence is confirmed) Lynx lynx (only if presence is confirmed) | Population size | 1/individual (accidental) (outside the boundary of the protected area, within the wind farm) | The precautionary principle and evidence from the literature Risk analysis of mortality of the species |
|  |  | Introducing invasive species | AH | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case | Not the case |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Canis lupus (only if presence is confirmed) Lynx lynx (only if presence is confirmed) | Habitat area of the species | 0 ha of protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the project beneficiary GIS field data analysis |
|  |  | Temporary occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Canis lupus (only if presence is confirmed) Lynx lynx (only if presence is confirmed) | Population size | 0 ha fragmented habitats in the protected area Occupied areas outside the protected area where presence has been reported following monitoring: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the project beneficiary GIS field data analysis |
|  |  | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Canis lupus (only if presence is confirmed) Lynx lynx (only if presence is confirmed) | Population size | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) | Spatial analysis and data submitted by the Ministry of Environment Bulgaria at the request of the project beneficiary GIS field data analysis Noise level modeling |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna Canis lupus (only if presence is confirmed) Lynx lynx (only if presence is confirmed) | Population size | 1/individual (accidental) | Risk analysis of mortality of the species |
|  |  | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Areas temporarily occupied by proeict outside the boundaries of protected areas where the presence of sopecia has been reported: 0,9 ha temporarily occupied by site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park The impact will be felt outside the protected area at the boundary of the wind farm, at the boundary/neighborhood of the working front | Spatial analysis of field inventory results |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Direct short-term impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area 30-40 square meters in the working front area, agricultural land outside the protected area | Spatial analysis of field inventory results |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Agricultural land outside the protected natural area | Habitat area of the species | 0 ha of protected area Maximum 500 m from the limit of the working front. The distance to which the effects are felt is between 5 m (in the case of rhizomes/root fragments) and 200-500 m (in the case of seeds), the habitat is located 200-700 m from the area where the excavation works are carried out. Taking into account the precautionary principle it was considered that it could potentially generate an impact | Spatial analysis of field inventory results |

Table  5-5 Identification and quantification of impacts (continued)

| Intervention |  | Impact | Direct impacts | Indirect impacts | Secondary impacts | Cumulative impacts | Short- and long-term impacts | Species |  |  |  |  | Parameter/target affected | Quantifying the impact | How to quantify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSPA0036 Dumbrăveni |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | - | - | - | Short | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | - | - | - | Short |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Earthworks (land levelling, excavation, excavation, backfilling) | Increased noise level | PAS | - | - | - | Short |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | - | - | - | Short | - |  |  |  |  | - | - | - |
|  | Installation of installations/ equipment | Emissions of pollutants into the air (operation of vehicles and machinery) | AH | - | - | - | Short | - |  |  |  |  | - | - | - |
|  | Construction of the electrical network for discharging the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | - | - | - | Short | - |  |  |  |  | - | - | - |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | - | - | - | Lung | - |  |  |  |  | - | - | - |
| Operate | Energy production activities | Increased noise level | PAS | - | - | - | Lung | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Lighting at night | PAS/PH | - | - | - | Lung | - |  |  |  |  | - | - | - |
|  |  | Waste generation | AH/PAS | - | - | - | Short | - |  |  |  |  | - | - | - |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | - | - | - | Short | - |  |  |  |  | - | - | - |
|  |  | Collision with turbine blades | REP | - | - | - | Lung | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size | Alauda arvensis - 0,083 individuals/ year (1 individual every approx. 12 years) Buteo rufinus - 0,193 individuals/ year (1 individual every approx. 5 years) Circus cyaneus - 0,012 individuals/ year (1 individual every approx. 83 years) Coracias garrulus - 0,009 individuals/ year (1 individual every approx. 111 years) Falco tinnununculus - 0,964 individuals/ year (1 individual every approx. 1 year) Falco subbuteo - 0,154 individuals/ year (1 individual every approx. 7 years) Galerida cristata - 0,006 individuals/ year (1 individual every approx. 167 years) Hirundo rustica - 0,019 individuals/ year (1 individual approx. 53 years) Melanocorypha calandra - 0,005 individuals/ year (1 individual every approx. 200 years) Merops apiaster - 0,019 individuals/ year (1 individual every approx. 53 years) Miliaria calandraandra - 0,014 individuals/ year (1 individual every approx. 71 years) Motacilla alba - 0,006 individuals/ year (1 individual at approx. 167 years) Motacilla flava - 0,003 individuals/ year (1 individual every approx. 333 years) Pernis apivorus - 0,028 individuals/ year (1 individual every approx. 36 years) Streptopelia turtur - 0,004 individuals/ year (1 individual every approx. 250 years) Upupa epops - 0,005 individuals/ year (1 individual every approx. 20 years) | Scottish National Heritage guide: "Wind farms and Birds: Calculating a theoretical collision risk assuming no avoiding action" |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | - | - | - | Lung | - |  |  |  |  | - | - | - |
|  |  | Introducing invasive species | AH | - | - | - | Lung | - |  |  |  |  | - | - | - |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | - | - | - | Short | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Temporary occupation of areas of land | FH | - | - | - | Short |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Increased noise level | PAS | - | - | - | Short |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | - | - | - | Short |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Emissions of pollutants into the air (operation of vehicles and machinery) | AH | - | - | - | Short | - |  |  |  |  | - | - | - |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | - | - | - | Short | - |  |  |  |  | - | - | - |
|  |  | Introducing invasive species | AH | - | - | - | Short | - |  |  |  |  | - | - | - |

Table  5-6 Identification and quantification of impacts (continued)

| Intervention |  | Impact | Direct impacts | Indirect impacts | Secondary impacts | Cumulative impacts | Short- and long-term impacts | Species |  |  |  |  |  |  |  | Parameter/target affected | Quantifying the impact | How to quantify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSPA0094 Hagieni Forest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | - | - | - | Short | Accipiter brevipes, Alcedo atthis, Anthus campestris, Aquila heliaca, Aquila pomarina, Burhinus oedicnemus, Buteo rufinus, Caprimulgus europaeus, Calandrella brachydactyla, Ciconia ciconia, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco cherrug, Falco columbarius, Falco peregrinus Falco vespertinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Larus melanocephalus, Larus minutus, Lullula arborea, Melanocorypha calandra, Milvus migrans, Nycticorax nycticorax, Oenanthe pleschanka, Pelecanus crispus, Pernis apivorus, Philomachus pugnax, Porzana porzana/Calidris pugnax, Recurvirostra avosetta, Tadorna ferruginea | Regularly migratory species Species associated with open land/ agricultural species Alauda arvensis, Buteo lagopus, Emberiza melanocephala, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Oenanthe oenanthe, Streptopelia turtur, Upupa epops, Falco tinnunculus Anser albifrons, Galerida cristata, Coturnix coturnix | Regularly migratory species Species associated with riparian and coastal habitats Gallinago gallinago, Hirundo rustica | Regularly migratory species Species associated with urban habitats Hirundo rustica | Regularly migratory species Species associated with terrestrial mixed woodland and open land habitats Accipiter nisus, Falco subbuteo, Buteo buteo | Regularly migratory species Species associated with reed habitats Motacilla flava | Regularly migratory species Species associated with open aquatic habitats Anser albifrons, Anser anser, Cygnus olor, Larus cachinnans | Regularly migratory species Species associated with forest habitats Sylvia atricapilla | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | - | - | - | Short |  |  |  |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Earthworks (land levelling, digging, excavation, backfilling) | Increased noise level | PAS | - | - | - | Short |  |  |  |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |
|  | Installation of installations/ equipment | Emissions of pollutants into the air (operation of vehicles and machinery) | AH | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |
|  | Construction of the electrical network for discharging the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | - | - | - | Lung | - |  |  |  |  |  |  |  | - | - | - |
| Operate | Energy production activities | Increased noise level | PAS | - | - | - | Lung | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Lighting at night | PAS/PH | - | - | - | Lung | - |  |  |  |  |  |  |  | - | - | - |
|  |  | Waste generation | AH/PAS | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |
|  |  | Collision with turbine blades | REP | - | - | - | Lung | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica |  |  |  | Population size | Alauda arvensis - 0,083 individuals/ year (1 individual every approx. 12 years) Aquila pomarina - 0,197 individuals/ year (1 individual every approx. 5 years) Buteo buteo - 1,285 individuals/ year (1 individual every approx. 1 year) Buteo rufinus - 0,193 individuals/ year (1 individual every approx. 5 years) Circaetus gallicus - 0,261 individuals/ year (2 individuals every approx. 5 years) Circus aeruginosus - 0,181 individuals/ year (1 individual every approx. 6 years) Circus cyaneus - 0,012 individuals/ year (1 individual every approx. 83 years) Coracias garrulus - 0,009 individuals/ year (1 individual every approx. 111 years) Falco subbuteo - 0,077 individuals/ year (1 individual every approx. 12 years) Falco tinnununculus - 0,964 individuals/ year (1 individual every approx. 1 year) Galerida cristata - 0,006 individuals/ year (1 individual every approx. 167 years) Hirundo rustica - 0,019 individuals/ year (1 individual approx. 53 years) Melanocorypha calandra - 0,005 individuals/ year (1 individual every approx. 200 years) Merops apiaster - 0,019 individuals/ year (1 individual every approx. 53 years) Miliaria calandraandra - 0,014 individuals/ year (1 individual every approx. 71 years) Motacilla alba - 0,006 individuals/ year (1 individual at approx. 167 years) Motacilla flava - 0,003 individuals/ year (1 individual every approx. 333 years) Pernis apivorus - 0,028 individuals/ year (1 individual every approx. 36 years) Streptopelia turtur - 0,004 individuals/ year (1 individual every approx. 250 years) Upupa epops - 0,005 individuals/ year (1 individual every approx. 20 years) | Scottish National Heritage guide: "Wind farms and Birds: Calculating a theoretical collision risk assuming no avoiding action" |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | - | - | - | Lung | - |  |  |  |  |  |  |  | - | - | - |
|  |  | Introducing invasive species | AH | - | - | - | Lung | - |  |  |  |  |  |  |  | - | - | - |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | - | - | - | Short | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Temporary occupation of areas of land | FH | - | - | - | Short |  |  |  |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Increased noise level | PAS | - | - | - | Short |  |  |  |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | - | - | - | Short |  |  |  |  |  |  |  |  | Population size trend | Could not quantify | To be decided following monitoring campaigns |
|  |  | Emissions of pollutants into the air (operation of vehicles and machinery) | AH | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |
|  |  | Introducing invasive species | AH | - | - | - | Short | - |  |  |  |  |  |  |  | - | - | - |

Table  5-8 Identifying and quantifying impacts (continued)

| Intervention |  | Impact | Impacts direct | Impacts indirect | Impacts secondary | Impacts cumulative | Short- and long-term impacts | Species |  |  |  |  |  |  |  | Parameter / Target affected | Quantification impact | How to quantify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSPA0036 Dumbrăveni |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus |  | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops |  | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus |  | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Earthworks (land leveling, excavation, excavation, backfilling) | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  | Population size trend | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Foundation works | Collision of wildlife with vehicular traffic on access/use roads in the area of the working front | REP | No indirect i impacts have been identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Installation of installations/ equipment | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Construction of the electrical network to discharge the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
| Operate | Energy production activities | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus |  | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops |  | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus |  | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size trend | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Lighting at night | PAS/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Waste generation | AH/PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Collision with turbine blades | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Comana - Pecineaga Wind Farm, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing | Long-term direct impact | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops |  | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus |  | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia |  | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size | Alauda arvensis - 0,083 individuals/ year (1 individual every approx. 12 years) Buteo rufinus - 0,193 individuals/ year (1 individual every approx. 5 years) Circus cyaneus - 0,012 individuals/ year (1 individual every approx. 83 years) Coracias garrulus - 0,009 individuals/ year (1 individual every approx. 111 years) Falco tinnununculus - 0,964 individuals/ year (1 individual every approx. 1 year) Falco subbuteo - 0,154 individuals/ year (1 individual every approx. 7 years) Galerida cristata - 0,006 individuals/ year (1 individual every approx. 167 years) Hirundo rustica - 0,019 individuals/ year (1 individual approx. 53 years) Melanocorypha calandra - 0,005 individuals/ year (1 individual every approx. 200 years) Merops apiaster - 0,019 individuals/ year (1 individual every approx. 53 years) Miliaria calandraandra - 0,014 individuals/ year (1 individual every approx. 71 years) Motacilla alba - 0,006 individuals/ year (1 individual at approx. 167 years) Motacilla flava - 0,003 individuals/ year (1 individual every approx. 333 years) Pernis apivorus - 0,028 individuals/ year (1 individual every approx. 36 years) Streptopelia turtur - 0,004 individuals/ year (1 individual every approx. 250 years) Upupa epops - 0,005 individuals/ year (1 individual every approx. 20 years) | Scottish National Heritage guide: "Wind farms and Birds: Calculating a theoretical collision risk assuming no avoiding action" |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus |  | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops |  | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus |  | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Temporary occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  | Population size trend | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  | Population size trend | The population trend will be stable, 0% change The impact will be felt in the area of the working fronts, resulting in temporary removal of foraging/ resting in these areas, with the species moving to neighboring areas with similar conditions. The effect will be insignificant and temporary, the works will be phased | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |

Table  5-9 Identifying and quantifying impacts (continued)

| Intervention |  | Impact | Impacts direct | Impacts indirect | Impacts secondary | Impacts cumulative | Impacts in the and long | Species |  |  |  |  |  |  |  |  |  |  |  | Parameter / Target affected | Quantification impact | How to quantify |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSPA0094 Hagieni Forest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Construction | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Accipiter brevipes, Alcedo atthis, Anthus campestris, Aquila heliaca, Aquila pomarina, Burhinus oedicnemus, Buteo rufinus, Caprimulgus europaeus, Calandrella brachydactyla, Ciconia ciconia, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco cherrug, Falco columbarius, Falco peregrinus Falco vespertinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Larus melanocephalus, Larus minutus, Lullula arborea, Melanocorypha calandra, Milvus migrans, Nycticorax nycticorax, Oenanthe pleschanka, Pelecanus crispus, Pernis apivorus, Philomachus pugnax, Porzana porzana/Calidris pugnax, Recurvirostra avosetta, Tadorna ferruginea |  | Regularly migratory species Species associated with open/ agricultural Alauda arvensis, Buteo lagopus, Emberiza melanocephala, Lanius senator, Merops apiaster, Merops apiaster, Miliaria calandra, Motacilla alba, Oenanthe oenanthe, Streptopelia turtur, Upupa epops, Falco tinnunculus Anser albifrons, Galerida cristata, Coturnix coturnix |  | Regularly migratory species Species associated with riparian and coastal habitats Gallinago gallinago, Hirundo rustica |  | Regularly migratory species Species associated with urban habitats Hirundo rustica | Regularly migratory species Species associated with terrestrial mixed woodland and open land habitats Accipiter nisus, Falco subbuteo, Buteo buteo |  | Regularly migratory species Species associated with reed habitats Motacilla flava | Regularly migratory species Species associated with open aquatic habitats Anser albifrons, Anser anser, Cygnus olor, Larus cachinnans | Regularly migratory species Species associated with forest habitats Sylvia atricapilla | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by trenching for cable laying 309,24 ha areas permanently occupied by the constructive elements of the energy park | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Building access roads | Temporary/permanent occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  |  |  |  |  | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Earthworks (land levelling, excavation, excavation, backfilling) | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  |  |  |  |  | Population size trend | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Foundation works | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Installation of installations/ equipment | Emissions of pollutants to air (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Construction of the electrical network to discharge the energy produced by the wind power plant to the transformer station and the telecommunication network (fiber optic), transformer stations | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Site rehabilitation works on completion of construction | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
| Operate | Energy production activities | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Long-term direct impact | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus |  | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops |  | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus |  | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica |  |  |  |  | Population size trend | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a | Analysis of inventories and field monitoring based on observed species behavior |
|  |  | Lighting at night | PAS/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Waste generation | AH/PAS | No indirect impacts have been identified | No secondary impacts were identified | No cumulative impacts have been identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Collision with turbine blades | REP | No indirect impacts identified | No secondary impacts identified | Cumulative impact with : Chirnogeni wind farm (existing) Wind energy park 14 EC-92,4 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) Wind energy park 17 CE-112,2 MW, wind power station transformer, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) Wind Farm Comana - Pecineaga, intravilan loc Tătaru and extravilan Comana (proposed Karnobat wind farm (existing | Long-term direct impact | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops |  | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus |  | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia |  |  | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica |  |  |  | Population size | Alauda arvensis - 0,083 individuals/ year (1 individual every approx. 12 years) Aquila pomarina - 0,197 individuals/ year (1 individual every approx. 5 years) Buteo buteo - 1,285 individuals/ year (1 individual every approx. 1 year) Buteo rufinus - 0,193 individuals/ year (1 individual every approx. 5 years Circaetus gallicus - 0,261 individuals/ year (2 individuals every approx. 5 years) Circus aeruginosus - 0,181 individuals/ year (1 individual every approx. 6 years) Circus cyaneus - 0,012 individuals/ year (1 individual every approx. 83 years) Coracias garrulus - 0,009 individuals/ year (1 individual every approx. 111 years) Falco subbuteo - 0,077 individuals/ year (1 individual every approx. 12 years) Falco tinnununculus - 0,964 individuals/ year (1 individual every approx. 1 year) Galerida cristata - 0,006 individuals/ year (1 individual every approx. 167 years) Hirundo rustica - 0,019 individuals/ year (1 individual approx. 53 years) Melanocorypha calandra - 0,005 individuals/ year (1 individual every approx. 200 years) Merops apiaster - 0,019 individuals/ year (1 individual every approx. 53 years) Miliaria calandraandra - 0,014 individuals/ year (1 individual every approx. 71 years) Motacilla alba - 0,006 individuals/ year (1 individual approx. 167 years) Motacilla flava - 0,003 individuals/ year (1 individual every approx. 333 years) Pernis apivorus - 0,028 individuals/ year (1 individual every approx. 36 years) Streptopelia turtur - 0,004 individuals/ year (1 individual every approx. 250 years) Upupa epops - 0,005 individuals/ year (1 individual every approx. 20 years) | Scottish National Heritage guide: "Wind farms and Birds: Calculating a theoretical collision risk assuming no avoiding action" |
|  |  | Sudden change of pressure within the turbine blades and also below the turbine blades (about 5 m) - barotrauma | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  | Maintenance and maintenance works | Collision of wildlife with vehicular traffic on access/use roads in the area of the repair front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
| Decommissioning | Site organization and management (including site traffic) | Removing topsoil and vegetation | AH/PH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact | Accipiter brevipes, Anthus campestris, Aquila pomarina, Bubo bubo, Buteo rufinus, Caprimulgus europaeus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulus, Falco peregrinus, Hieraaetus pennatus, Lanius collurio, Lanius minor, Melanocorypha calandra, Milvus migrans, Pernis apivorus |  | Regularly migratory species/Species associated with extensively used agricultural land Alauda arvensis, Carduelis cannabina, Carduelis carduelis, Coturnix coturnix, Emberiza melanocephala, Falco tinnunculus, Fringilla montifringilla, Galerida cristata, Lanius excubitor, Lanius senator, Merops apiaster, Miliaria calandra, Motacilla alba, Motacilla flava, Oenanthe isabellina, Oenanthe oenanthe, Saxicola torquata, Streptopelia turtur, Sturnus roseus (alternative name Pastor roseus), Turdus iliacus, Upupa epops |  | Species associated with forest habitats Carduelis chloris, Carduelis spinus, Coccothraustes coccothraustes, Falco subbuteo, Ficedula hypoleuca, Fringilla coelebs, Jynx torquilla, Muscicapa striata, Oriolus oriolus, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Pyrrhula pyrrhula, Regulus ignicapillus, Regulus regulus, Serinus serinus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus |  | Species associated with reed habitats Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Locustella fluviatilis, Locustella luscinioides, Riparia riparia | Species associated with cliff habitats and urban areas Monticola saxatilis, Hirundo daurica, Hirundo rustica |  |  |  |  | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches 309,24 ha areas permanently occupied by the constructive elements of the energy park | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Temporary occupation of areas of land | FH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  |  |  |  |  | Population size trend | 0 ha of protected area Occupied areas outside the protected area that could constitute feeding habitats: 0.9 ha temporarily occupied, area of site organization + area temporarily occupied by cable laying trenches | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Increased noise level | PAS | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  |  |  |  |  | Population size trend | Potential disruption of up to 2 to 3 individuals The effects will be felt locally, at distances not likely to affect sensitive receptors in the area The noise level of 30 dB shall be experienced at a maximum distance of 1.5 km around the working area (see section 1.1.7, fig 1-10a) | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  | Decommissioning / demolition works Site restoration / rehabilitation works at the end of the plan's lifetime | Collision of wildlife with vehicular traffic on access roads/vehicle tracks in the area of the work front | REP | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Short-term direct impact |  |  |  |  |  |  |  |  |  |  |  |  | Population size trend | The population trend will be stable, 0% change The impact will be felt in the area of the working fronts, resulting in temporary removal of foraging/ resting in these areas, with the species moving to neighboring areas with similar conditions. The effect will be insignificant and temporary, the works will be phased | Analysis of inventories and field monitoring based on observed species behavior GIS analysis. |
|  |  | Emissions of pollutants into the atmosphere (operation of vehicles and machinery) | AH | No indirect impacts identified | No secondary impacts have been identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Accidental spills of petroleum products from machinery - pollutants seeping into soil | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |
|  |  | Introducing invasive species | AH | No indirect impacts identified | No secondary impacts identified | No cumulative impacts identified | Not the case | Not the case |  |  |  |  |  |  |  |  |  |  |  | Not the case | Not the case | Not the case |

## Assessing the significance of impacts

The detailed analysis of the significance of the impact is presented for each habitat and species in the Natura 2000 sites potentially affected by the plan in the table in the Annex to this study.

The table attached to this study shows the potential impacts if the proposed measures are not implemented. In the following, details of the forms of impact generated by the plan are briefly presented for each Natura 2000 site analyzed.

### ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake

- Habitat loss

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Habitat alteration

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Fragmentation of specific habitats

The plan is not likely to cause habitat fragmentation or disruption of connectivity for species characteristic of this site.

- Disturbing the activity of wildlife species

Both during construction and operation, there is a risk that the plan may lead to disturbance of species due to reduced food resources, noise and vibration or atmospheric emissions, especially for Spermophilus citellus and Miniopterus schreibersii.

- Reducing the size of the herd

There is potential for the plan to generate casualties during both the construction and operational phases. In the construction period possible victims of the species Spermophilus citellus and in the operational period individuals of the species Miniopterus schreibersii.

### ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii

- Habitat loss

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Habitat alteration

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Fragmentation of specific habitats

The plan is not likely to cause habitat fragmentation or disruption of connectivity for species characteristic of this site.

- Disturbing the activity of wildlife species

During both construction and operation, there is a risk that the plan may lead to disturbance to species due to reduced food resources, noise and vibration or atmospheric emissions, especially for Spermophilus citellus and Miniopterus schreibersii and Rhinolophus ferrumequinum.

- Reducing the size of the herd

There is a possibility that the plan may generate casualties during both the execution and operational phases. In the execution phase, possible victims of the species Spermophilus citellus and in the operational phase, individuals of the species Miniopterus schreibersii and Rhinolophus ferrumequinum.

### ROSPA0166 Plopeni - Chirnogeni

- Habitat loss

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Habitat alteration

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Fragmentation of specific habitats

The plan is not likely to cause habitat fragmentation or disruption of connectivity for species characteristic of this site.

- Disturbing the activity of wildlife species

During both construction and operation, there is a risk that the plan may lead to disturbance of species due to reduced food resources, noise and vibration or atmospheric emissions, especially for Buteo rufinus, Circus cyaneus, Falco columbarius, Falco vespertinus, Lanius minor.

- Reducing the size of the herd

There is a possibility that plan may generate casualties in the operational phase. During the operational period possible casualties of Buteo rufinus, Circus cyaneus, Falco columbarius, Falco vespertinus, Lanius minor.

### ROSPA0036 Dumbrăveni

- Habitat loss

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Habitat alteration

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Fragmentation of specific habitats

The plan is not likely to cause habitat fragmentation or disruption of connectivity for species characteristic of this site.

- Disturbing the activity of wildlife species

During both construction and operation, there is a risk that the plan may lead to disturbance to species due to reduced food resources, noise and vibration or atmospheric emissions, in particular for the species Buteo rufinus, Circus cyaneus, Circus pygargus, Falco tinnununculus, Falco subbuteo, Jynx torquilla.

- Reducing the size of the herd

There is a possibility that plan may generate casualties in the operational phase. During the operational period possible casualties of Buteo rufinus, Circus cyaneus, Circus pygargus, Falco tinnunculus, Falco subbuteo, Jynx torquilla.

### ROSPA0094 Hagieni Forest

- Habitat loss

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Habitat alteration

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Fragmentation of specific habitats

The plan is not likely to cause habitat fragmentation or disruption of connectivity for species characteristic of this site.

- Disturbing the activity of wildlife species

During both construction and operation, there is a risk that the plan may lead to disturbance of species due to reduced food resources, noise and vibration or atmospheric emissions, especially for Anser albifrons species, Buteo rufinus, Circus cyaneus, Circus pygargus, Falco cherrug, Falco columbarius, Falco peregrinus, Falco peregrinus, Falco subbuteo, Falco tinnununculus, Falco vespertinus, Hieraaetus pennatus, Melanocorypha calandra, Pernis apivorus.

- Reducing the size of the herd

There is a possibility that plan may generate casualties during the operational phase. During the operational period possible casualties of Anser albifrons, Buteo rufinus, Circus cyaneus, Circus pygargus, Falco cherrug, Falco columbarius, Falco peregrinus, Falco subbuteo, Falco tinnunculus, Falco vespertinus, Hieraaetus pennatus, Melanocorypha calandra, Pernis apivorus.

### BG0000569 Kardam

- Habitat loss

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Habitat alteration

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Fragmentation of specific habitats

The plan is not likely to cause habitat fragmentation or disruption of connectivity for species characteristic of this site.

- Disturbing the activity of wildlife species

During the construction period, there is a risk that the plan may lead to disturbance of species due to reduced food resources, noise and vibrations or atmospheric emissions, especially for Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna.

- Reducing the size of the herd

There is a possibility that the plan may generate victims during the execution phase. Possible victims during the execution phase are Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna.

### BG0000570 Izvorovo - Kraishte

- Habitat loss

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Habitat alteration

The area proposed by the PUZ does not intersect the site, and no interventions within the site are necessary for the construction of the plan . There is no risk of losing areas of favorable habitats of species declared of community interest within the site, but potential habitats of species that are present within the area studied by the PUZ and that are also declared at the level of the area may be affected.

- Fragmentation of specific habitats

The plan is not likely to cause habitat fragmentation or disruption of connectivity for species characteristic of this site.

- Disturbing the activity of wildlife species

During both construction and operation, there is a risk that the plan may lead to disturbance of species due to reduced food resources, noise and vibration or atmospheric emissions, in particular for Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna, Canis lupus (only if confirmed), and Rhinolophus mehelyi.

- Reducing the size of the herd

There is a possibility that plan may generate casualties in both the execution and operational phases. In the execution period possible victims of the species Spermophilus citellus, Mesocricetus newtoni, Mustela eversmanii, Vormela peregusna, Canis lupus (only if presence is confirmed) and in the operational period individuals of the species Rhinolophus mehelyi.

# Prevention, avoidance and mitigation measures

## Proposed measures to avoid and reduce impacts

The principles applied in identifying and establishing avoidance and mitigation measures are:

General principles:

- Monitoring. On-going monitoring during the implementation phases (during construction, during the first years of operation - minimum 3 years) is necessary to ensure that the data and knowledge base of the plan is updated and informed decisions can be made;

- Adaptive management. Avoidance and mitigation measures should be continuously adapted on the basis of the latest available information in the area where the plan is implemented;

- Ensuring specialized expertise. It is preferable that biodiversity officers differ from environmental officers in order to ensure that requirements for the protection of biodiversity components are adequately addressed;

- Ongoing consultation with stakeholders. During construction and operation, it is necessary to ensure a framework for ongoing collaboration with the main stakeholders concerned with biodiversity management (at least Natura 2000 site managers/ custodians). Collaboration should focus on the exchange of recent data and information as well as details on the implementation of avoidance and mitigation measures;

- Effectiveness and complementarity of measures. Any of the measures implemented must achieve their aim through a high degree of effectiveness, without hampering/limiting the effectiveness of other measures and without creating other forms of significant impacts or risks to biodiversity or the human population;

- Control of impacts. The measures formulated and implemented must directly address the impacts identified, ensuring that these impacts are always kept below the thresholds of significance.

For habitat loss and alteration:

- Reduction of pollutant concentrations in areas adjacent to the plan implementation area.

- Rehabilitation of all temporarily damaged areas with the use of native species and ensuring the ecological functionality of the rehabilitated areas.

Disruption of wildlife species:

- Minimize the effects associated with human presence, noise and lighting during the construction and operation of the future park.

Reducing the size of stocks

- Minimize mortality rates from wildlife collisions with road traffic.

The following aspects were taken into account in formulating the avoidance and mitigation measures:

- Address those impacts whose occurrence is a clear consequence of the activities proposed in the plan (in this case they are more specific and better quantified/ localized);

- a cautious approach to the protection of sensitive components that could be affected during construction or operation by certain changes to the plan or decisions of the moment;

- Specifying precisely only those parameters that are absolutely necessary to ensure the functionality of the proposed measures, but not providing details that may limit options during the design and construction period.

Some of the measures formulated address several strands of Community interest, but to avoid redundancy they have been described only once and then only mentioned under the other strands.

The measures presented below are based both on recommended best practices for the realization of wind energy projects and on the analysis and adaptation of experiences and solutions identified at national and international level for different situations encountered in the construction and operation of wind farms.

All measures formulated for the construction phase are valid in the case of a possible decommissioning phase.

Table  6-1 Proposed measures to protect biodiversity

| Measure- description |  | Type of measure (P/E/R) | Affected species/habitat | Parameter addressed by the measure | Target impact | Period of implementation of the measure | Location of implementation of the measure |
| --- | --- | --- | --- | --- | --- | --- | --- |
| General measures |  |  |  |  |  |  |  |
| M1 | Conduct regular training for all staff involved in construction / decommissioning works on general environmental issues, protected habitats and species and measures to avoid and minimize impacts. Particular attention will be given to issues relating to the prohibition of the collection of plants and animals or the deliberate injuring/killing of protected species. | E | All Natura 2000 components related to protected natural areas ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species Habitat area | All forms of impact | Construction/Decommissioning | Plan implementation area |
| M2 | Implementation of low-glare, ground-only lighting systems (excluding incandescent heat-generating luminaires) | P | All potentially affected bird* species and chiroptera** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | PAS, REP | Construction/Operation/Decommissioning | Plan implementation area |
| M3 | Compliance with the proposed schedule of works, as well as compliance with the period proposed in this plan. | P | All potentially affected bird* species and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | PAS | Construction/Decommissioning | Plan implementation area |
| M4 | Compliance with the perimeter of the site organization proposed to be located in the immediate vicinity of the work area. | P | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | PAS | Construction/Decommissioning | Plan implementation area |
| M5 | Carry out perimeter activities on the strictly necessary areas. | P | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | AH,PAS | Construction/Decommissioning | Plan implementation area |
| M6 | The storage of construction materials shall be only in the areas provided for by the plan within the site organization and work points, without affecting the adjacent areas. | P,E | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | AH, PAS | Construction/Decommissioning | Plan implementation area |
| M7 | Avoid any spills of liquid fuels, oils, paints, etc. on the ground. In case of accidental pollution, they will be removed by applying absorbent materials and removed from the site by contracting companies specialized in the management of these types of hazardous waste. | P,E | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | AH,PAS | Construction/Operation/Decommissioning | Plan implementation area |
| M8 | Ensure proper waste management with regular waste disposal without the use of intermediate and non-conforming landfills. It is forbidden to abandon waste in the immediate vicinity of the site organization and beyond. | P,E | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | AH,PAS | Construction/Operation/Decommissioning | Plan implementation area |
| M9 | Access to the work points will be via existing access roads so as not to affect additional areas of land. | P,E | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | AH,PAS | Construction/Operation/Decommissioning | Plan implementation area |
| M10 | The use of machinery and equipment for carrying out the works that produce a minimum level of noise and vibration, that are efficient, low-polluting and quiet, so that wildlife species are not affected. | P,E | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | PAS | Construction/Operation/Decommissioning | Plan implementation area |
| M11 | Topsoil or topsoil resulting from excavations will be properly stockpiled and protected, then reused. Revegetation of temporarily occupied areas. | P,E | All potentially affected bird species* and chiroptera** and mamals*** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | AH,PAS | Construction/Decommissioning | Plan implementation area |
| Specific measures |  |  |  |  |  |  |  |
| M12 | If nests are identified, postpone activities in these areas beyond the April-May period. | P,E | All Natura 2000 components related to protected natural areas ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | Construction?Decommissioning | Plan implementation area |
| M13 | Turbines should be signaled at night with flashing lights, with long intervals between two consecutive lights. These turbines are more easily recognized by birds if flashing rather than continuous light is used. | P,E,R | All potentially affected bird species* and chiroptera** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | Operate | Wind farm area |
| M14 | Regular mowing of vegetation around the turbines to maintain a low abundance of insect species that are a food source for both chiropteran and bird species. | P,E | All potentially affected bird species* and chiroptera** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | Operate | Areas around turbines |
| M15 | Installation of an Aircraft Detection Lighting System (ADLS) on turbines T47, T46, T35, T36, T34 to reduce the risk of mortality of species sensitive to artificial lighting (birds and bats). The measure will apply from the first year of operation of the project | P,E,R | All potentially affected bird species* and chiroptera** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | Operate | Turbine area T47, T46, T35, T36, T34 |
| M16 | To reduce the risk of mortality it is proposed to limit the turbines (T47, T46, T35, T36, T34) to a wind speed of 6.5 m/s during the sensitive period (migration), starting half an hour before sunset until sunrise and bat protection systems that emit acoustic ultrasonic deterrent signals to remove bats from the swept rotor area. The measure will apply from the wind farm's first year of operation. | R | All potentially affected bird species* and chiroptera** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | Operate | Turbine area T47, T46, T35, T36, T34 |
| M17 | The installation of collision risk prevention systems will be implemented from the first year of operation of the wind farm. An automatic shutdown/power-down control system (e.g. DTBird, IdentiFlight, Robin Radar Max, STRIX BirdTrack). The system will target bird species for which a potentially significant impact has been identified and will be implemented from the first year of operation. | R | All potentially affected bird species* in ROSPA0166, ROSPA0036, ROSPA0094, | Population size Population trends for each species | REP | Operate | Turbine area T8, T18, T29. T32 si T37 |
| M18 | Installation of video bird detection system on turbines T8, T18, T29. T32 and T37. High-resolution camera systems shall be installed.The system to emit deflection signals shall be installed from the first year of operation of the wind farm. The camera system detects birds from a distance of up to 600 m and emits chasing sounds. If the bird continues to approach the turbine, after 300 meters the blades start to slow down to a speed of 3 rpm, which eliminates the risk of collision | E,R | All potentially affected bird species* in , ROSPA0166, ROSPA0036, ROSPA0094, | Population size Population trends for each species | REP | Operate | Turbine area T8, T18, T29. T32 si T37 |
| M19 | Installation of day and night radar and video camera systems (on turbines T8, T18, T29. T32 and T37) that detect and prevent the risk of chiroptera collision with wind farm structures. The deflection beacon system shall be installed from the first year of operation of the wind farm. | E,R | All potentially affected chiroptera** species from ROSCI0071, ROSCI0157, BG0000569, BG0000570 | Population size Population trends for each species | REP | Operate | Turbine area T8, T18, T29. T32 si T37 |

*Aquila pomarina, Falco peregrinus, Hieraaetus pennatus, Lanius minor, Milvus migrans, Accipiter brevipes, Accipiter nisus, Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Alauda arvensis, Alcedo atthis, Anser albifrons, Anser anser, Anthus campestris, Aquila heliaca,Aquila pomarina, Bubo bubo, Burhinus oedicnemus, Buteo buteo, Buteo lagopus, Buteo rufinus, Calandrella brachydactyla, Caprimulgus europaeus, Carduelis cannabina, Carduelis carduelis, Carduelis chloris, Carduelis spinus, Ciconia ciconia, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coccothraustes coccothraustes, Coracias garrulus, Coturnix coturnix, Cygnus olor, Emberiza melanocephala, Falco cherrug, Falco columbarius, Falco peregrinus, Falco subbuteo, Falco tinnunculus, Falco vespertinus, Ficedula hypoleuca, Fringilla coelebs, Fringilla montifringilla, Galerida cristata, Gallinago gallinago, Hieraaetus pennatus, Hirundo daurica, Hirundo rustica, Jynx torquilla, Lanius collurio, Lanius excubitor, Lanius minor, Lanius senator, Larus cachinnans, Larus melanocephalus, Larus minutus, Locustella fluviatilis, Locustella luscinioides, Lullula arborea, Melanocorypha calandra, Merops apiaster, Miliaria calandra, Milvus migrans, Monticola saxatilis, Motacilla alba, Motacilla flava, Muscicapa striata, Nycticorax nycticorax, Oenanthe isabellina, Oenanthe oenanthe, Oenanthe pleschanka, Oriolus oriolus, Pelecanus crispus, Pernis apivorus, Philomachus pugnax, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Porzana porzana/Calidris pugnax, Pyrrhula pyrrhula, Recurvirostra avosetta, Regulus ignicapillus, Regulus regulus, Riparia riparia, Saxicola torquata, Serinus serinus, Streptopelia turtur, Sturnus roseus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Tadorna ferruginea, Turdus iliacus, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus, Upupa epops

** Miniopterus schreibersii, Rhinolophus mehelyi, Rhinolophus ferrumequinum, Myotis capaccinii, Nyctalus noctula, Pipistrellus pipistrellus

*** Spermophilus citellus, Mustela eversmanii

Further details on some proposed measures

M16 - To reduce the risk of mortality, it is proposed to limit the turbines (T47, T46, T35, T36, T34) to a wind speed of 6.5 m/s during the sensitiveperiod (migration), from half an hour before sunset until sunrise, and to install bat protection systems that emit ultrasonic deterrent acoustic signals to remove bats from the rotor swept area. The measure will apply from the first year of operation of the wind farm.

It is proposed to mitigate the impact on population declines of chiropteran and avian species. The study by Arnett et al. (2010) showed that the measure of increasing the parameter “cut-in speed” to 5 m/s reduces mortality from 44% to 93%, with less than 1% production losses.

The same effectiveness is similar for measures M15 and M17.

M18 – Installation of Video Detection System for Birds on Turbines T8, T18, T29, T32, and T37  High-resolution video camera systems will be installed.

The cameras will be installed and commissioned at the same time as the wind farm is brought into operation. Thus, monitoring will begin immediately, reducing the risk of collisions from the very start of turbine operations.

The camera system can detect birds from a distance of up to 2 km. This extended detection range allows for early identification of approaching birds and helps prevent potential incidents.

Through real-time tracking, the system can provide instantaneous data on bird movements, which is essential for taking quick and effective action in the event of a collision risk.

The system can also track and detect bird activity at night using thermal spectrum cameras, enabling continuous, round-the-clock monitoring.

The effectiveness of these systems is generally estimated at around 80%, although it can vary depending on weather conditions, light levels, or the type of camera used.

A case study from Norway indicates detection effectiveness for bird flights near turbines ranging between 76% and 96% within a 300-meter radius around the turbines for one of the video-based systems. At shorter distances (150 m), the detection rate was estimated between 86% and 96% (May et al., 2012).

Another case from Sweden reports an 88% collision avoidance rate with turbine blades (Litsgard et al., 2016).

For another video system, the detection and collision risk reduction rate ranges between 77% and 85%, according to a case study from California (Duerr et al., 2023).

Another study from the United States reports an 82% reduction in mortality rates for raptor species after implementing this system in a wind farm (McClure et al., 2021).

Similar effectiveness is expected for measure M19.

The measures are Specific, Measurable, Achievable, Relevant, and Time-bound – SMART. Verification can be carried out by the evaluator using a set of key questions presented in the following table.

Table  6-2 Verification of the fulfillment of the SMART criteria for the proposed measures

| Attribute | Key question | YES/NO | Explanation of the answer to the key question |
| --- | --- | --- | --- |
| Specific Measurable | Does it address specific habitat(s)/species? | YES | Measures are proposed for all Natura 2000 components likely to be impacted by the implementation of the plan |
|  | Can it be useful for other habitats/species? | YES | The nature of the proposed general measures is also likely to be useful to other Natura 2000 components |
|  | Does it address a Conservation Objective parameter? | YES | The proposed measures are specific to the CSO parameters on which impacts due to the implementation of the plan have been identified |
|  | Does it address a significant impact identified for the project? | YES | The aim of the proposed measures is to minimize this impact to zero |
|  | Are the constructive dimensions of the measure (height, length, width, etc.) defined? | YES | These dimensions are defined and, if necessary, they will be rescaled according to further results of field monitoring |
|  | Can the contribution to impact reduction be quantified? | YES | The contribution to reducing the impact can only be quantifiable with true values after the operation of the objective and implementation of the proposed measures |
|  | Is the unit of measurement defined in accordance with the unit of measurement of the Conservation Objective parameter? | YES | The proposed measures have been designed in line with the specific conservation objectives of each of the ANPICs analyzed |
|  | Does the quantification method make it possible to establish an indicator that can be monitored during the implementation of the measure? | YES | It allows comparison with the field studies that preceded the plan and the studies that will precede the implementation of the plan |
| Applicable Relevant | Is there evidence of the practical feasibility/implementation of the measure? | YES | The measures are recommended by both national and international best practice guidelines and are in line with the specific ecology of the site |
|  | Is there evidence of this measure being applied and working in the past? | YES | Studies in the field as well as previously developed projects prove the effectiveness and applicability of the proposed measures |
|  | Can this be achieved without disproportionate costs? | YES | The costs of implementing the measures are not likely to give rise to high costs affecting the financial integrity of the plan |
|  | Is it the best applicable measure for the impacts identified? | YES | Yes, the proposed measures are used in the development of plans proposing the same activity |
|  | Can it lead to a negligible residual impact? | YES | The proposed measures lead to insignificant impact after implementation |
| Framed in time | Is the stage of the project at which it is realized/implemented clearly mentioned? | YES | Yes, the details have been detailed in this study and in the Environmental Report |
|  | Is the stage of the project at which the expected results are achieved clearly stated? Is there a specific timeframe? | YES | Yes, the details have been detailed in this study and in the Environmental Report |

## Timetable for implementation and monitoring

The following table shows the timetable for the implementation of the measures foreseen in this plan for the different stages of the plan, together with the relevant monitoring component.

Table  6-3 Timetable for implementation and monitoring of impact mitigation measures

| Measure |  | Affected species/ habitat | Parameter addressed by the measure | The targeted impact | Timetable for implementing the measures |  |  |  |  |  |  |  |  |  |  |  | Responsible | Budget |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |  |
| General measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M1 | Conduct regular training for all staff involved in construction / decommissioning works on general environmental issues, protected habitats and species and measures to avoid and minimize impacts. Particular attention will be given to issues relating to the prohibition of the collection of plants and animals or the deliberate injuring/killing of protected species. | All potentially affected bird species and chiroptera in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species Habitat area | All forms of impact | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M2 | Implementation of low-glare, ground-only lighting systems (excluding incandescent heat-generating luminaires) | All potentially affected bird species and chiroptera in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | PAS, REP | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M3 | Compliance with the proposed schedule of works, as well as compliance with the period proposed in this plan. | All potentially affected bird species and chiroptera in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population trends for each species Habitat area | PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M4 | Compliance with the perimeter of the site organization proposed to be located in the immediate vicinity of the work area. | All potentially affected bird species and chiroptera and mams in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M5 | Carry out perimeter activities on the strictly necessary areas. | All potentially affected bird species and chiroptera and mammals in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | AH,PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M6 | The storage of construction materials shall be only in the areas provided for by the plan within the site organization and work points, without affecting the adjacent areas. | All potentially affected bird species and chiroptera and mammals in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | AH, PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M7 | Avoid any spills of liquid fuels, oils, paints, etc. on the ground. In case of accidental pollution, they will be removed by applying absorbent materials and removed from the site by contracting companies specialized in the management of these types of hazardous waste. | All potentially affected bird species and chiroptera and mammals in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | AH,PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M8 | Ensure proper waste management with regular waste disposal without the use of intermediate and non-compliant landfills. It is forbidden to abandon waste in the immediate vicinity of the site organization and beyond. | All potentially affected bird species and chiroptera and mammals in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | AH,PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M9 | Access to the work points will be via existing access roads so as not to affect additional areas of land. | All potentially affected bird species and chiroptera and mammals in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | AH,PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M10 | The use of machinery and equipment for carrying out the works that produce a minimum level of noise and vibration, that are efficient, low-polluting and quiet, so that wildlife species are not affected. | All potentially affected bird species and chiroptera and mammals in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M11 | Topsoil or topsoil resulting from excavations will be properly stockpiled and protected, then reused. Revegetation of temporarily occupied areas. | All potentially affected bird species and chiroptera and mammals in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population trends for each species Habitat area | AH,PAS | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| Specific measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M12 | Construction and erection works shall be planned so that they take place outside the breeding and rearing periods of the broods of the species of Community interest, irrespective of the presence or absence of nests on the site. The planning of activities shall take into account the biological calendar of the species concerned in order to prevent any negative impact on them. | All components Natura 2000 protected natural areas ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 Spermophillus citellus | Population size Population trends for each species | REP | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M13 | Turbines should be signaled at night with flashing lights, with long intervals between two consecutive lights. These turbines are more easily recognized by birds if flashing rather than continuous light is used. | All bird species potentially affected and chiroptera from ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | X | X | X | X | X | X | X | X | X | X | X | X | Builder | At no extra cost |
| M14 | Regular mowing of vegetation around turbines to maintain a low abundance of insect species that are a food source for both chiropteran and bird species. | All bird species potentially affected and chiroptera from ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP |  |  |  | X | X | X | X | X | X | X |  |  | Builder | 10000 euro/year |
| M15 | Installation of an Aircraft Detection Lighting System (ADLS) on turbines T47, T46, T35, T36, T34 to reduce the risk of mortality of species sensitive to artificial lighting (birds and bats). The measure will apply from the first year of operation of the project | All potentially affected bird species* and chiroptera** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | X | X | X | X | X | X | X | X | X | X | X | X | Contractor/Experts certified and experienced in biodiversity monitoring | 100000 euro/year |
| M16 | The installation of collision risk prevention systems will be implemented from the first year of operation of the wind farm. An automatic shutdown/power-down control system (e.g. DTBird, IdentiFlight, Robin Radar Max, STRIX BirdTrack). The system will target bird species for which a potentially significant impact has been identified and will be implemented from the first year of operation. | All potentially affected bird species* and chiroptera** in ROSCI0071, ROSCI0157, ROSPA0166, ROSPA0036, ROSPA0094, BG0000569, BG0000570 | Population size Population trends for each species | REP | X | X | X | X | X | X | X | X | X | X | X | X | The park owner / Certified and experienced biodiversity monitoring experts | At no extra cost |
| M17 | The installation of collision risk prevention systems will be implemented from the first year of operation of the wind farm. | All potentially affected bird species* in ROSPA0166, ROSPA0036, ROSPA0094, | Population size Population trends for each species | REP |  |  | X | X | X |  |  |  | X | X | X |  | Park keeper / Certified and experienced biodiversity monitoring experts | 150000 euro/year |
| M18 | Installation of video bird detection system on turbines T8, T18, T29. T32 and T37. High-resolution camera systems shall be installed.The system to emit deflection signals shall be installed from the first year of operation of the wind farm. The camera system detects birds from a distance of up to 600 m and emits chasing sounds. If the bird continues to approach the turbine, after 300 meters the blades start to slow down to a speed of 3 rpm, which eliminates the risk of collision | All potentially affected bird species* in , ROSPA0166, ROSPA0036, ROSPA0094, | Population size Population trends for each species | REP | X | X | X | X | X | X | X | X | X | X | X | X | Park keeper / Certified and experienced biodiversity monitoring experts | 200000 euro/year |
| M19 | Installation of day and night radar and video camera systems (on turbines T8, T18, T29. T32 and T37) that detect and prevent the risk of chiroptera collision with wind farm structures. The deflection beacon system shall be installed from the first year of operation of the wind farm. | All chiroptera species potentially affected of ROSCI0071, ROSCI0157, BG0000569, BG0000570 | Population size Population trends for each species | REP |  |  |  | X | X | X | X | X | X | X |  |  | The park keeper /Experienced and certified experts in biodiversity monitoring | 200000 euro/year |

*Aquila pomarina, Falco peregrinus, Hieraaetus pennatus, Lanius minor, Milvus migrans, Accipiter brevipes, Accipiter nisus, Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Alauda arvensis, Alcedo atthis, Anser albifrons, Anser anser, Anthus campestris, Aquila heliaca,Aquila pomarina, Bubo bubo, Burhinus oedicnemus, Buteo buteo, Buteo lagopus, Buteo rufinus, Calandrella brachydactyla, Caprimulgus europaeus, Carduelis cannabina, Carduelis carduelis, Carduelis chloris, Carduelis spinus, Ciconia ciconia, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coccothraustes coccothraustes, Coracias garrulus, Coturnix coturnix, Cygnus olor, Emberiza melanocephala, Falco cherrug, Falco columbarius, Falco peregrinus, Falco subbuteo, Falco tinnunculus, Falco vespertinus, Ficedula hypoleuca, Fringilla coelebs, Fringilla montifringilla, Galerida cristata, Gallinago gallinago, Hieraaetus pennatus, Hirundo daurica, Hirundo rustica, Jynx torquilla, Lanius collurio, Lanius excubitor, Lanius minor, Lanius senator, Larus cachinnans, Larus melanocephalus, Larus minutus, Locustella fluviatilis, Locustella luscinioides, Lullula arborea, Melanocorypha calandra, Merops apiaster, Miliaria calandra, Milvus migrans, Monticola saxatilis, Motacilla alba, Motacilla flava, Muscicapa striata, Nycticorax nycticorax, Oenanthe isabellina, Oenanthe oenanthe, Oenanthe pleschanka, Oriolus oriolus, Pelecanus crispus, Pernis apivorus, Philomachus pugnax, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Porzana porzana/Calidris pugnax, Pyrrhula pyrrhula, Recurvirostra avosetta, Regulus ignicapillus, Regulus regulus, Riparia riparia, Saxicola torquata, Serinus serinus, Streptopelia turtur, Sturnus roseus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Tadorna ferruginea, Turdus iliacus, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus, Upupa epops

** Miniopterus schreibersii, Rhinolophus mehelyi, Rhinolophus ferrumequinum, Myotis capaccinii, Nyctalus noctula, Pipistrellus pipistrellus

*** Spermophilus citellus, Mustela eversmanii

# Monitoring prevention, avoidance and mitigation measures

The monitoring of the impact that the construction and operation of the expressway will have on the environmental components is intended, on the one hand, to confirm or refute the quantifications of the residual impact made prior to the implementation of the plan , to quantify the effectiveness of the measures already implemented and to identify, where necessary, the need for additional measures or new locations where it is necessary to implement impact mitigation measures.

The implementation of the monitoring programme requires the existence of a specialized team including at least one expert for each Natura 2000 component (habitats/plants, amphibians and reptiles, birds, mammals (including bats).

The results of the monitoring will feed into a database of data and information with which the following objectives can be achieved:

- Reporting the results to the relevant environmental authorities and other stakeholders (e.g. custodians of protected natural areas);

- Data analysis to assess the actual residual impact;

- Justification of the need for potential additional measures or additional sites for implementation.

Independently of the monitoring program, the owner/contractors have the obligation to report, according to the legal requirements in force, any accidental killing of bird species, as well as strictly protected species listed in Annexes no. 4A and 4B of GEO no. 57/2007 (both during the construction and during the operation period).

The methodological requirements of the methodological guidelines for monitoring the conservation status of species and habitats in Romania, based on Article 17 of the Habitats Directive, published on the website of the Romanian Academy's Institute of Biology in Bucharest (http://www.ibiol.ro/posmediu/rezultate.htm ), respectively:

- A synthetic monitoring guide for habitats of community interest (salt marshes, continental dunes, grasslands, freshwater) in Romania;

- Summary monitoring guide for habitats of community interest: scrublands, peat bogs and fens, cliffs, forests;

- Summary guide for monitoring invertebrate species of community interest in Romania;

- Synthetic guide for monitoring community species of reptiles and amphibians in Romania;

- Synthetic monitoring guide for community fish species in Romania;

- Synthetic monitoring guide for mammalian species of Community interest in Romania;

- Guidelines for monitoring the conservation status of caves and bat species of Community interest in Romania;

as well as:

- Standard Guide for the monitoring of bird species of community interest in Romania, elaborated by the Romanian Ornithological Society and Milvus Group in 2014, http://monitorizareapasarilor.cndd.ro/documents/Ghid-standard-de-monitorizare-pasari-2014.pdf.

For the purpose of this report, a "field campaign" is a field trip that ensures that all locations to be monitored are fully covered, within the entire study territory and with the application of all appropriate survey methods.

It is very important that throughout the construction period and at least during the first three years of operation, managers and custodians of potentially affected Natura 2000 sites have access to detailed monitoring results in order to be able to link these data and information to activities related to the assessment of the conservation status of habitats and species within the sites.

All data and information collected in the monitoring program should be expressed in quantitative terms, with a clear indication of the units of measurement, the size of the areas investigated, the method applied and the time periods (including hourly) during which the field activities were carried out. The information shall be presented in both raw data (tabulated) and graphical form (map representation of all data collected). Each set of data shall be accompanied by an interpretation of the results as well as qualitative and quantitative assessments of the trends and the prospects for changes in the value of the indicators monitored.

The monitoring reports will be prepared by the designated monitoring team(s) and will be made available to the Beneficiary and upon request to the interested public and to the competent environmental protection authority.

Table  7-1 Program for monitoring measures

| ANPIC affected (COD, name) | / Conservation objective/ Species/ Habitat affected/ Parameter | Form of impact | Reduction measure | Period of implementation of the measure | Location of the measure | Monitoring indicators | Units of measurement | Frequency of monitoring | Monitoring locations | Duration of monitoring | Degree of effectiveness of the measure | Budget | Monitoring Officer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake | Terrestrial mammals*** | AH/PH/FH/FH/REP/PAS | M1, M4, M5, M6, M8, M10, | Construction and operation | The proposed location of the wind farm | Species distribution Activity level | Spatial and temporal spatial pattern Intensity of use of the future park site | Lunar | Location of the future park | Construction/operation period (at least 3 years) | High | No additional costs are required | Contractor/Experts certified and experienced in biodiversity monitoring |
|  | Chiroptere** | PAS/REP | M1, M12, M14, M16, M19 | Operate |  | Species distribution Activity index - BAI | Spatial and temporal spatial pattern Intensity of use of the future park site Number of individuals | Lunar | Location of the future park | The entire operational period of the wind farm | Assumed high (effectiveness to be confirmed by monthly monitoring) | Approximately € 210000/ operational period of the wind farm | Certified and experienced operator/experts in biodiversity monitoring |
| ROSCI0157/ROSAC157 Hagieni - Cotul Văii Forest | Terrestrial mammals*** | AH/PH/FH/FH/REP/PAS | M1, M4, M5, M6, M8, M10 | Construction and operation |  | Species distribution Activity level | Spatial and temporal spatial pattern Intensity of use of the future park site | Lunar | Location of the future park | Construction/operation period (at least 3 years with the possibility of extension for a further 3 years) | High | The amount will be determined following field monitoring over a period of at least 3 years for measure M18 for the other measures no additional construction is necessary | Contractor/Experts certified and experienced in biodiversity monitoring |
|  | Chiroptere** | PAS/REP | M1, M12, M14, M16, M19 | Operate |  | Species distribution Activity index - BAI | Spatial and temporal spatial pattern Intensity of use of the future park site Number of individuals | Lunar | Location of the future park | The entire operational period of the wind farm | Assumed high (effectiveness to be confirmed by monthly monitoring) | Approximately € 210000/ operational period of the wind farm | Certified and experienced operator/experts in biodiversity monitoring |
| ROSPA0166 Plopeni - Chirnogeni | Birdlife* | AH/PH/FH/FH/REP/PAS | M1, M12, M14, M15, M16,M17, M18, M19 | Construction and operation |  | Species distribution Species abundance Population size | Spatial and temporal spatial pattern Intensity of use of the future park site Number of individuals | Lunar | Location of the future park | The entire operational period of the wind farm | Assumed high (effectiveness to be confirmed by monthly monitoring) | Approximately € 560000/ operational period of the wind farm | Certified and experienced operator/experts in biodiversity monitoring |
| ROSPA0036 Dumbrăveni | Birdlife* | AH/PH/FH/FH/REP/PAS | M1, M12, M14, M15, M16, M17, M18 | Construction and operation |  | Species distribution Species abundance Population size | Spatial and temporal spatial pattern Intensity of use of the future park site Number of individuals | Lunar | Location of the future park | The entire operational period of the wind farm | Assumed high (effectiveness to be confirmed by monthly monitoring) | Approximately € 560000/ operational period of the wind farm | Certified and experienced operator/experts in biodiversity monitoring |
| ROSPA0094 Hagieni Forest | Birdlife* | AH/PH/FH/FH/REP/PAS | M1, M12, M14, M15, M16, M17, M18 | Construction and operation |  | Species distribution Species abundance Population size | Spatial and temporal spatial pattern Intensity of use of the future park site Number of individuals | Lunar | Location of the future park | The entire operational period of the wind farm | Assumed high (effectiveness to be confirmed by monthly monitoring) | Approximately € 560000/ operational period of the wind farm | Certified and experienced operator/experts in biodiversity monitoring |
| BG0000569 Kardam | Terrestrial mammals*** | AH/PH/FH/FH/REP/PAS | M1, M4, M5, M6, M8, M10 | Building |  | Species distribution Activity level | Spatial and temporal spatial pattern Intensity of use of the future park site | Lunar | Location of the future park | Construction/operation period (at least 3 years) | High | The amount will be determined following field monitoring over a period of at least 3 years for measure M18 for the other measures no additional construction is necessary | Contractor/Experts certified and experienced in biodiversity monitoring |
|  | Chiroptere** | PAS/REP | M1, M12, M14, M16, M19 | Operate |  | Species distribution Activity index - BAI | Spatial and temporal spatial pattern Intensity of use of the future park site Number of individuals | Lunar | Location of the future park | The entire operational period of the wind farm | Assumed high (effectiveness to be confirmed by monthly monitoring) | Around € 210000/ operational period of the wind farm | Certified and experienced operator/experts in biodiversity monitoring |
| BG0000570 Izvorovo - Kraishte | Terrestrial mammals* | AH/PH/FH/FH/REP/PAS | M1, M4, M5, M6, M8, M10 | Construction and operation |  | Species distribution Activity level | Spatial and temporal spatial pattern Intensity of use of the future park site | Lunar | Location of the future park | Construction/operation period (at least 3 years) | High | The amount will be determined following field monitoring over a period of at least 3 years for measure M18 for the other measures no additional construction is necessary | Contractor/Experts certified and experienced in biodiversity monitoring |
|  | Chiroptere** | PAS/REP | M1, M12, M14, M16, M19 | Operate |  | Species distribution Activity index - BAI | Spatial and temporal spatial pattern Intensity of use of the future park site Number of individuals | Lunar | Location of the future park | The entire operational period of the wind farm | Assumed high (effectiveness to be confirmed by monthly monitoring) | Around € 210000/ operational period of the wind farm | Certified and experienced operator/experts in biodiversity monitoring |

*Aquila pomarina, Falco peregrinus, Hieraaetus pennatus, Lanius minor, Milvus migrans, Accipiter brevipes, Accipiter nisus, Acrocephalus arundinaceus, Acrocephalus palustris, Acrocephalus schoenobaenus, Acrocephalus scirpaceus, Alauda arvensis, Alcedo atthis, Anser albifrons, Anser anser, Anthus campestris, Aquila heliaca,Aquila pomarina, Bubo bubo, Burhinus oedicnemus, Buteo buteo, Buteo lagopus, Buteo rufinus, Calandrella brachydactyla, Caprimulgus europaeus, Carduelis cannabina, Carduelis carduelis, Carduelis chloris, Carduelis spinus, Ciconia ciconia, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coccothraustes coccothraustes, Coracias garrulus, Coturnix coturnix, Cygnus olor, Emberiza melanocephala, Falco cherrug, Falco columbarius, Falco peregrinus, Falco subbuteo, Falco tinnunculus, Falco vespertinus, Ficedula hypoleuca, Fringilla coelebs, Fringilla montifringilla, Galerida cristata, Gallinago gallinago, Hieraaetus pennatus, Hirundo daurica, Hirundo rustica, Jynx torquilla, Lanius collurio, Lanius excubitor, Lanius minor, Lanius senator, Larus cachinnans, Larus melanocephalus, Larus minutus, Locustella fluviatilis, Locustella luscinioides, Lullula arborea, Melanocorypha calandra, Merops apiaster, Miliaria calandra, Milvus migrans, Monticola saxatilis, Motacilla alba, Motacilla flava, Muscicapa striata, Nycticorax nycticorax, Oenanthe isabellina, Oenanthe oenanthe, Oenanthe pleschanka, Oriolus oriolus, Pelecanus crispus, Pernis apivorus, Philomachus pugnax, Phylloscopus collybita, Phylloscopus sibilatrix, Phylloscopus trochilus, Porzana porzana/Calidris pugnax, Pyrrhula pyrrhula, Recurvirostra avosetta, Regulus ignicapillus, Regulus regulus, Riparia riparia, Saxicola torquata, Serinus serinus, Streptopelia turtur, Sturnus roseus, Sturnus vulgaris, Sylvia atricapilla, Sylvia curruca, Tadorna ferruginea, Turdus iliacus, Turdus merula, Turdus philomelos, Turdus pilaris, Turdus viscivorus, Upupa epops

** Miniopterus schreibersii, Rhinolophus mehelyi, Rhinolophus ferrumequinum, Myotis capaccinii, Nyctalus noctula, Pipistrellus pipistrellus

*** Spermophilus citellus, Mustela eversmani

# Residual impact assessment

For the residual impact analysis, a quantification of the potential impacts remaining after the implementation of the avoidance and mitigation measures proposed in this study was carried out. The analysis was carried out for each form of impact:

- Habitat loss

The plan will not lead to the loss of habitat areas and will not affect Natura 2000 sites. The measures do not address this form of impact as it is not relevant in the context of the appropriate assessment for this plan.

- Habitat alteration

In the absence of avoidance and mitigation measures, plan may contribute to habitat alteration. This risk has been addressed by the measures proposed in the study and the risk is low.

- Habitat fragmentation

The plan is not likely to cause fragmentation of habitats or favorable habitats of species within the Natura 2000 sites analyzed. There is a potential to affect local migration corridors for bats and birds as well as foraging areas as a result of new constructions (turbines and related facilities). This risk is addressed by the proposed measures and the residual impact is considered insignificant.

- Disruption of species activity

Disturbance to species activity can occur as a result of increased noise levels, increased light levels and disturbance to food resources. In the case of this plan, disruption to species activity may occur mainly at SPAs. To address this form of impact, specific measures have been proposed in the EA study.

- Reducing the size of the herd

Population reduction may occur most frequently as a result of collision with the turbines of the future park. Quantification of this form of impact was estimated using a risk zone for both chiroptera and bird species. The residual impact was analyzed by calculating the collision risk for each type of species analyzed.

Taking into account the impacts identified and the measures proposed in this study, no significant residual impact has been identified for any of the analyzed species in the protected natural areas potentially affected by the implementation of the plan .

# Alternative solutions

Alternative zero (no project implementation)

The advantages of not implementing the project are:

- Biodiversity - maintaining unchanged land use throughout the area proposed for the future park and maintaining the integrity of the species present and likely to be present on the future site.

- Water - maintaining water quality (there will no longer be potential risks of contamination of water bodies crossed by the proposed route in case of accidental pollution);

- Soil - maintaining vegetated soil surfaces;

- Landscape - maintaining existing semi-natural and man-made elements;

- Social and economic environment - business as usual (no new emissions will be generated in the local area, no new sources of noise).

However, these disadvantages could be reduced or even excluded by implementing a set of measures for each component potentially affected by the project (social-economic component, biodiversity, surface and groundwater bodies, soil and subsoil, air).

Not implementing the project, however, will lead to the maintenance of current conditions regarding greenhouse gas emissions, particulate matter, nitrogen oxides and sulphur, etc. At the same time, the decision not to implement the proposed investment may contribute to the failure to meet the targets set at European and national level of reducing the environmental impact of the energy sector by 80%-95% by 2050.

The lack of an adequate infrastructure suitable for the conditions of increased energy consumption to attract various economic operators for the economic development of the region, leads to a significant negative impact on the economic environment. The realization of the project will also create new jobs both during the implementation and later during the operation of the future park.

Considering the above, Alternative "0" is not a viable option in terms of environmental impacts. The construction of the future park is a necessary option as it can meet the following two major objectives:

- Reducing the current significant negative impacts mainly due to fossil fuel energy production and thus reducing GHG emissions;

- Avoiding the generation of additional significant impacts through the choice of location and construction solutions of the future investment.

Alternative I - was initiated in 2011 (consisting of 3 wind farms) - 117 wind power plants with a total installed capacity of 351 MW

Alternative II - started in 2022 - 48 wind power plants with a total installed capacity of 316.8 MW

Alternative III - was initiated in 2023 after obtaining the TRANSGAZ SA approval no. 34618/948/10.05.2023, as a result of which 2 power plants T17 and T23 were abandoned, thus the new wind farm will comprise 46 wind power plants with a total installed capacity of 303.6 MW

# Compensatory measures

Given the fact that the implementation of the PUZ will not affect the structure, functions or conservation objectives of the analyzed ANPICs and especially that it will not affect the habitats and/or species of community interest of the Natura 2000 sites, no compensatory measures will be necessary.

# Methods used to collect information on the species and/or habitats of Community interest affected

## Field data collection for plant habitats/communities and flora species

To analyze the horizontal structure of the plant phytocenoses, the line transect method was used, complemented with the phytocenological survey method. The line transect method involves the identification and notation of plant species/plant associations along a line whose length is determined according to the complexity of the habitat.

The survey method is based on the notation of species abundance-dominance indices, according to the methodology developed by the Central European Floristic School (Braun-Blanquet), in order to graphically transpose relevant elements for the description of floristic associations.

The survey includes the list of plant species recorded in the sample area accompanied by the abundance-dominance (AD) index rating for each species. The abundance-dominance index is assessed according to the Braun-Blanquet scale, complemented by Tüxen and Ellenberg, which comprises seven main steps as follows:

- r = rare or isolated individuals (0,01-0,1 %);

- += rare individuals with very low coverage (0.1-1 %);

- 1 = individuals numerous but with low coverage or rare but with high coverage (1-10 %);

- 2 = very numerous individuals or 10-25% coverage of the sample area;

- 3 = 25-50% coverage of the sample area, number of individuals is indifferent;

- 4 = 50-75% coverage of the sample area, number of individuals is indifferent

- 5 = cover 75-100% of the sample area, number of individuals indifferent.

The identification of plant species was done by using specialized works, such as Flora României vol. I-XIII (Săvulescu et al., 1952-1976), Flora ilustrată a României. Pteridophyta et Spermatophyta (Ciocârlan, 2009), Vascular Plants of Romania: illustrated field determinator (Sârbu I., Ștefan N., Oprea A., 2013), Red List of Higher Plants of Romania (Oltean et al., 1994), Red Book of Vascular Plants of Romania (Dihoru, G., & Negrean, G. (2009), Lista critică a plantelor vasculare din România (Oprea, A., 2005), Lista roșie a plantelor vasculare dispărute, periclitate, vulnerabile și rare din flora României (Boșcaiu N. et al., 1994). The nomenclature used for the naming of plant species is in accordance with the current regulations on aspects of taxonomy and systematic botany (https://europlusmed.org/). Plant associations and natural habitats were identified by using specialized works such as Phytocenoses of Romania (Sanda et al., 2008), the European Union Natural Habitats Interpretation Manual (EUR 28), complemented with the national habitat classification - Habitats of Romania (Doniță et al., 2005).

## Field data collection for inver invertebrates

To highlight the entomofauna of the plan area, the field investigation method by visual transects was used.

The principle of the method is to walk along a transect for a fixed distance. The person carrying out the monitoring moves for a fixed duration of time in terrestrial habitats, visually detecting individuals or traces of their activity. The data collection methodology was prepared according to the diurnal visual transect model predominantly used for data collection for the orders Orthoptera, Coleoptera, Odonata and Lepidoptera, according to the Synthetic Guide for monitoring invertebrate species of community interest in Romania (Pârvulescu L. et al., 2015). The data collection on the presence of invertebrate species involved diurnal visual transects, as well as the identification and detailed investigation of microhabitats favorable to invertebrate species of community interest, but also other species present in the area. Mainly microhabitat areas associated with the species of community interest reported in the area, wetland habitats, river banks, irrigation canals, etc. were investigated.

## Field data collection for amphibians and reptiles

The species of amphibians and reptiles that may use the habitats in the area of the plan or in its immediate vicinity were surveyed in the field using the diurnal transect method according to the Synthetic Guide for the monitoring of reptile and amphibian species in Romania (Török (Zs.) et al., 2013).During the monitoring period, transects were carried out on the area of the future Cerchezu wind farm and all signs of presence (visual observation of individuals, regardless of their developmental stage: pupa, larva or adult or their condition: alive, corpse) of amphibian and reptile species were noted. This method has also been applied in the southern sector of the Natura 2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, which borders the plan area .

## Field data collection for avifauna

Bird monitoring was carried out according to the "Standard Guidelines for Monitoring of Bird Species of Community Interest in Romania, 2021".

- The main purposes of implementing these methodologies:

- Bird migration data collection

- Identification and description of flight corridors relevant to the species transiting the site;

- Identify possible impacts to these species from the construction and operation of the wind farm;

- Identify and propose site-specific mitigation measures.

Thus:

During the nesting and wintering period, transects were carried out on the area of the future Ansthall wind farm and all signs of presence (visual observation of individuals, auditory signaling) of bird species were recorded.

During the fall migration period, the fixed point method was used to monitor birds. This method involves observing all birds transiting the site during the migration from fixed points that allow the whole site (locations of all turbines) to be visualized as far as possible and observed.

During the reporting period, for the Ansthall wind farm, 5-hour observations were made from three fixed monitoring points. Monitoring involved recording all individuals/groups of birds passing through the observer's field of view, their flight path, flight height.

## Field data collection for mammals (including chiroptera)

The following methods were used to analyze the presence of mammal species in the area of the plan , adapted according to the recommendations of the Synthetic Monitoring Guide for the species of mammals of community interest in Romania, elaborated by the Institute of Biology in Bucharest:

- For diurnal mammals - diurnal transects for identification of tracks, individuals and camera trapping;

- For chiroptera - ultrasound recordings within dedicated transects and investigations of optimal areas for colonies or individuals (e.g. abandoned buildings, tree hollows).

In the case of chiropterans, best practices have been implemented using state-of-the-art equipment.

Detecting chiroptera is difficult at night, so a non-invasive and very effective method was chosen: recording the ultrasound emitted by the animals at night.

Three monitoring methods were used:

- Qualitative

- Quantitative - transect

- Quantitative - passive detection

The qualitative method involved driving ultrasound transects, using a vehicle, along a pre-determined route at the proposed site. Transects were taken half an hour before sunset until 03:00 AM the following day to record the full optimal spectrum for the chiropteran species at the site. An Anabat Walkabout type device with GPS included was used. The data were processed using Anabat Insight software and ultrasound identification guides emitted by the chiroptera (Pocora & Pocora, 2012). Lunar transects were carried out, as far as possible in periods with meteorologically optimal conditions for animal flight (no rain, temperatures close to the lunar average at night). The total length of transects for each night was 60.7 km.

Qualitative methods were: 1 - recording of chiropteran activity at fixed points in the transects (10 points), where chiropteran activity was monitored for 10 minutes, 2 - automatic recording of chiropteran activity using static detectors placed in various positions in the field (stratified radomic sampling method), Anabat Chorus type, with a frequency of one full night of activity per month, at 5 points in the proposed site. Data recorded with static detectors were manually identified using Kaleidoscope Pro - Wildlife Acoustics software.

For any ultrasound analysis, the number of contacts is not equal to the number of individuals, an animal may be detected several times in the same place (particularly for quantitative monitoring). The number of contacts is directly proportional to the number of individuals.

The Bat Activity Index (BAI) was calculated for the quantitative method, providing a less subjective perspective in quantifying the potential impact (no. of contacts/business nights).

All spatial data were processed using ArcGIS ESRI 2023.

# Appropriate evaluation findings

This document represents the Adequate Assessment Study on the potential effects that the implementation of the "Plan Urbanistic Zonal Wind Energy Park 48 (46) wind power plants approx. 316.8MW (303.6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads" may generate on the protected natural areas of community interest in its area. The study was prepared in order to obtain the environmental permit for the investment.

The appropriate assessment study was prepared in accordance with the requirements of the Methodological Guide on the appropriate assessment of the potential effects of plans or projects on protected natural areas of community interest (Order no. 1.682/2023, subsequently amended and supplemented by Order no. 2.452/2023), in accordance with the provisions of art. 28 of the Government Emergency Ordinance no. 57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna, approved by Law no. 49/ 2011, as subsequently amended and supplemented and in compliance with the provisions of the specific Guide (approved by Order no. 1679/2023 for the approval of the specific Methodological Guide on the adequate assessment of the potential effects of plans/projects in the areas of interest) taking into account that the Plan in question is included in the field of "Energy Production".

From the administrative point of view, the study area of the plan is located in the Cerchezu locality, Constanța County.

The plan area does not cross any Natura 2000 sites, but it borders 5 Natura 2000 sites in Romania that have the potential to be influenced by its construction and operation: ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii, ROSPA0036 Dumbrăveni, ROSPA0166 Plopeni-Chirnogeni, ROSPA0094 Hagieni Forest and with 2 Natura 2000 sites in Bulgaria: BG0000569 Kardam, BG0000570 Izvorovo - Kraishte.

These sites have been analyzed in this study in terms of the impact of the plan on their integrity.

The assessment of the impact of the plan on the Natura 2000 sites likely to be affected has been carried out on the basis of the Specific Conservation Objectives set by ANANP for the period 2021-2023 for all the sites considered in the assessment. The assessment has considered the potential cumulative impact, generated together with the proposed and existing wind farms both on Romanian and Bulgarian territory.

The most important potential form of impact associated with the plan is the reduction in wildlife populations, which may occur during the construction and operational phases as a result of construction works, collisions with construction traffic and mainly collisions with wind turbines (especially for bird and chiropteran species). Population reduction is likely to have a significant impact on wildlife populations and affect the population size parameters of the specific conservation objectives set for the species.

The measures proposed in this study for impact avoidance and mitigation address all forms of impacts identified, which may affect the parameters of the conservation objectives set for the habitats and species of Community interest in the potentially affected Natura 2000 sites.

The Appropriate Assessment Study has identified the need to implement measures that can ensure that the residual impact remains insignificant. In order to validate the effectiveness of the avoidance and mitigation measures, a Monitoring Program has been proposed which includes provisions for both the construction and operational periods. The implementation of the monitoring program is essential to ensure the correct implementation and functionality of the avoidance and mitigation measures.

The conclusions of the Appropriate Assessment Study detailed for each affected ANPIC can be found as Annex to this study.

# Selected bibliography

Anastasiu P. - coord., Sîrbu C., Urziceanu M., Camen-Comănescu P., Oprea A., Nagodă E., Gavrilidis A.-A., Miu I., Memedemin D., Sîrbu I., Manta N., 2019, Guide for inventory and mapping of the distribution of invasive and potentially invasive alien plant species in Romania Bucharest;

Anastasiu P., Negrean G., 2007, Plant invaders in Romania, București: Editura Universității din București;

Boșcaiu N., Coldea Gh., Horeanu C. (1994). Lista roșie a plantas vasculare dispărute, periclitate, vulnerabile si rare din flora Romaniei. Ocrotirea Naturii și a Mediului înconjurător, București, 38 (1) :45-56

Craioveanu, C., Rakosy, L., (2011), Fauna of lepidopterans in semi-natural mountain habitats of the Băișorii Mountain area (Cluj County), Bul. inf. Entomol, 21:3-4;

Dihoru, G., & Negrean, G. (2009). Red Book of Romanian vascular plants. Ed. Academiei Române;

Doniță, N., Paucă-Comănescu, M., Popescu, A., Mihăilescu, S., Biriș I.-A., 2005, Habitats in Romania, Editura Tehnică Silvică, Bucharest;

Natura 2000 site standard forms

Gafta, D., Mountford, O., 2008, Manual de interpretare a habitatelor Natura 2000 din România, Risoprint Publishing House, Cluj-Napoca;

Goriup P., (2008), Natura 2000 in Romania: Species fact sheet;

Grilo, C., Koroleva, E., Andrášik, R., Bíl, M., & González-Suárez, M., 2020, Roadkill risk and population vulnerability in European birds and mammals. Frontiers in Ecology and the Environment, 18(6), 323-328.

Gutzwiller KJ (2002) Applying landscape ecology in biological conservation Springer

Specific Guide (approved by Order no. 1679/2023 for the approval of the Specific Methodological Guide on the adequate assessment of the potential effects of plans/projects in the areas of interest) taking into account the fact that the Plan in question is included in the field of "Energy Production"

IGSU (2020). Synthesis on disaster risk manamgent in Romania. https://www.igsu.ro/Resources/COJ/RapoarteStudii/RO%20-%20Raport%20de%20tara%20evaluare%20riscuri%20si%20capabilitati%20final%202020%20Mec%20Pr%20Civ.pdf

Iuell, Bjorn Bekker, Hans Cuperus, Ruud Dufek, Jiri Fry, Gary Hicks, Claire Hlavac, Vaclav Keller, Verena Rosell, Carme Sanwine, Tony Torslov, Niels Wandall. (2003) COST 341 Handbook.

Oltean M., Neagrean G., Popescu A., Roman N., Dihoru G., Sanda V., Miilescu S. (1994). Red List of Romanian higher plants. Inst. de biologie, studii, sinteze, documentații de ecologie, Bucuresti, 1 :1-52.

Order of the Minister of Environment, Waters and Forests no. 1557/2016 on the approval of the Management Plan of the protected natural areas ROSCI0071 Dumbrăveni - Urluia Valley - Lake Vederoasa, ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Vederoasa Pond, 2.361 Dumbrăveni Forest, 2.350 Limestone walls of Petrosani - Deleni Commune, 2.351 Aliman fossiliferous site and IV.30 Vederoasa Lake

Order no. 1480/2016 on the approval of the Management Plan and Regulation of the protected natural areas ROSCI0157 Hagieni-Cotul Văii Forest, ROSPA0094 Hagieni Forest and 2.360 Hagieni Forest

Order of the Minister of Environment, Waters and Forests no. 1557/2016 on the approval of the Management Plan of the protected natural areas ROSCI0071 Dumbrăveni - Urluia Valley - Lake Vederoasa, ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Vederoasa Pond, 2.361 Dumbrăveni Forest, 2.350 Limestone walls of Petrosani - Deleni Commune, 2.351 Aliman fossiliferous site and IV.30 Vederoasa Lake

Order no. 1480/2016 on the approval of the Management Plan and Regulation of the protected natural areas ROSCI0157 Hagieni-Cotul Văii Forest, ROSPA0094 Hagieni Forest and 2.360 Hagieni Forest

Order No. RD - 1021/17.12.2020 (promulgated by SG 17/2021) promulgated by the Ministry of Environment and Water

Council of Ministers Decision No 122/02.03.2007 (promulgated by SG 21/2007)

Oprea A., 2005, Lista critică a plantelor vasculare din România, Editura Universității "Alexandru Ioan Cuza", Iași;

Romanian Ornithological Society, 2015. Atlas of bird species of community interest in Romania. National Center for Sustainable Development (Ed.). Noi Media Print S. A. & Media Nature Consulting S. R. L. Bucharest;

Arnett, E. B., Baerwald, E. F., Mathews, F., Rodrigues, L., Rodríguez-Durán, A., Rydell, J., Villegas-Patraca, R., & Voigt, C. C. (2016). Impact of Wind Energy Development on Bats: A Global Perspective. In C. C. C. Voigt & T. Kingston (Eds.), Bats in the Anthropocene: Conservation of Bats in a Changing World (pp. 295-323). Springer International Publishing. https://doi.org/10.1007/978-3-319-25220-9 https://doi.org/10.1007/978-3-319-25220-9

Arnett, E. B., Brown, K. B., Erickson, W. P., Fielder, J. K., Hamilton, B. L., Henry, T. H., Jain, A., Johnson, G. D., Kerns, J., Koford, R. R., Nicholson, C. P., O'Connel, T. J., Piorkowski, M. D., & Tankersley, R. D. (2008). Patterns of Bat Fatalities at Wind Energy Facilities in North America. The Journal of Wildlife Management, 72(1), 61-78. https://doi.org/10.2193/2007-221 https://doi.org/10.2193/2007-221

Cryan, P. M., Gorresen, P. M., Hein, C. D., Schirmacher, M. R., Diehl, R. H., Huso, M. M., Hayman, D. T. S., Fricker, P. D., Bonaccorso, F. J., Johnson, D. H., Heist, K., & Dalton, D. C. (2014). behavior of bats at wind turbines. Proceedings of the National Academy of Sciences, U.S.A., 1-6.

Doba, A., Papp, T., Nistorescu, M., Nagy, A. A., Stănescu, S., & Măntoiu, D. Ștefan. (2016) Good practice guide for planning and implementing wind energy investments (Issue August). Milvus Group Association, EPC Consultanță de Mediu SRL.

EC (2011) Wind energy developments and Natura 2000. In Wind Energy. https://doi.org/10.2779/98894

Kunz, T. H., Arnett, E. B., Cooper, B. M., Erickson, W. P., Larkin, R. P., Mabee, T., Morrison, M. L., Strickland, D. M., Szewczak, J. M., Strickland, M. D., & Szewczak, J. M. (2007). Assessing Impacts of Wind-Energy Development on Nocturnally Active Birds and Bats: A Guidance Document. Journal of Wildlife Management, 71(8), 2449-2486. https://doi.org/10.2193/2007-270 https://doi.org/10.2193/2007-270

Măntoiu, D. Ștefan, Chișamera, G., Chachula, O. M., Mărginean, G., Irina, P., Viorel, P., Hodor, C., Stanciu, C. R., Popescu-Mirceni, R., Telea, A., Bălășoiu, D., & Șandric, I. C. (2015). A bat fatality risk model at wind farms in Dobrogea, Romania , using a GIS approach. 4Th Inernational Berlin Bat Meeting, July.

Măntoiu, D. Ștefan, Kravchenko, K., Lehnert, L. S., Kramer-Schadt, S., Vlashcenko, A., Mirea, I.-C., Stanciu, C. R., Popescu-Mirceni, R., Zaharia, R., Chișamera, G. B., Chachula, O. M., Nistorescu, M. C., Moldovan, T.. O., & Voight, C. C. (2016). Bat migration in the western Black Sea area: stable isotopes analysis (δ2 Hf ), ultrasound monitoring and wind turbine mortality events. International Zoological Congres of "Grigore Antipa" Museum, 74-75.

Măntoiu, D. Ștefan, Kravchenko, K., Lehnert, L. S., Vlaschenko, A. S., Moldovan, T., T. O., Mirea, I.-C., Stanciu, C. R., Zaharia, R., Popescu-Mirceni, R., Nistorescu, M. C., & Voight, C. C. (2020). Wildlife and infrastructure: Impact of wind turbines on bats in the Black Sea Coast region. European Journal of Wildlife Management, 66(44), 1-13. https://doi.org/10.1007/s10344-020-01378-x https://doi.org/10.1007/s10344-020-01378-x

Pocora, I., & Pocora, V. (2012) Practical guide for bat identification using sonograms. Editura Universității "Alexandru Ioan Cuza".

Rodrigues, L., Bach, L., Dubourg-Savage, M.-J., Karapandža, B., Kovač, D., Kervyn, T., Dekker, J., Kepel, A., Bach, P., Collins, J., Harbusch, C., Park, K., Micevski, B., & Minderman, J. (2015). Guidelines for consideration of bats in wind farm projects Revision 2014. In Eurobats: Vol. Publicatio. https://doi.org/10.1017/CBO9781107415324.004

Walsh, K., & England, N. (2012). Bats and wind turbines (Issue June 2012).

Ciocârlan, V. (2009) Flora ilustrată a României: Pteridophyta et Spermatophyta. Edit.: Ceres, București.

Combroux, I. & Schwoerer, C. (2007) Assessment of the conservation status of habitats and species of Community interest in Romania - methodological guide. Edit. Timișoara.

Interpretation manual of European Union habitats-EUR 28.

Dihoru G., Negrean G. (2009) The red book of Romanian vacular plants. Edit. Editura Academiei Române, Bucharest.

Doniță N., Popescu A., Paucă-Comănescu M., Mihăilescu S., Biriș I.A. (2005) Habitats in Romania. Editura Tehnică Silvică, Bucharest.

Gafta D., Mountford O. (coord.), Alexiu V., Anastasiu P., Bărbos M., Burescu P., Coldea G., Drăgulescu C., Făgăraș M., Goia I., Groza G., Micu D., Mihăilescu S., Moldovan O., Nicolin A., Niculescu M., Oprea A., Oroian S., Paucă Comănescu M., Sârbu I., Suteu A. (2008). Manual de interpretare a habitatelor Natura 2000 din România. Publisher: Rosprint, Cluj-Napoca.

Oltean, M., Negrean, G., Popescu, A., Roman, N., Dihoru, G., Sanda, V. & Mihăilescu, S. (1994). Lista roșie a plantas superioare din România. Studii, sinteze, documentații de ecologia, 1, 1-52.

Sanda V., Öllerer K., Burescu P. (2008) Phytocenoses in Romania. Syntaxonomy, structure, dunamics and evolution. Edit: Ars Docendi, București.

Sârbu, I., Ștefan, N., Oprea, A. (2013). vascular plants of Romania: illustrated field determinator. Edit.: Victor B Victor, București.

Botnariuc, N., Tatole, V., The Red Book of Romanian Vertebrates, Romanian Academy, Bucharest, 2005

GEO 57/2007 on protected natural areas, conservation of natural habitats, wild flora and fauna

Birds Directive - Directive 2009/147/EC of the European Parliament and of the Council of November 30, 2009 on the conservation of wild birds

Habitats Directive - Council Directive 92/43/EEC of 21 May 1992 on the conservation of natural habitats and of wild fauna and flora

Bonn Convention - Convention of June 23, 1979 on the Conservation of Migratory Species of Wild Animals

Bern Convention - Wildlife Conservation CONVENTION of September 19, 1979

[2] European Commission (2013), Green Infrastructure (GI): Enhancing Europe's Natural Capital, COM/2013/0249 final, Green Infrastructure Strategy, p. 3.

Thematic publication of the National Network for Rural Development no. 42, year II,

Drugescu and Geacu, 2002

"Study on the recommendations on areas in Dobrogea where the siting of wind power plants should be restricted due to the migration corridors of birds of gliding flight (day raptors, storks, pelicans), respectively due to the wintering of geese and swans", conducted by the National Institute for Research and Development Danube Delta, 2012

Annex to the Order of the Minister of Environment, Waters and Forests No. 2.480/2022 on the approval of the National Action Plan for the conservation and management of the population of the red-throated goose - Branta ruficollis, in the period

Annex to the Order of the Minister of Environment, Waters and Forests No. 2.480/2022 on the approval of the National Action Plan for the conservation and management of the population of the red-throated goose - Branta ruficollis, in the period

Birdmap (5dvision.ee)

SATELLITETRACKING.EU

According to Hutterer, 2005

Bat migration routes in Europe, 2021

https://apmct.anpm.ro/biodiversitate