Benefit:

SC South Wind SRL

Title Report:

Presentation memorandum

Date

October 2025

Presentation memorandum Wind Energy Park 48 (46) Wind Power Plant approx. 316.8MW (303.6MW), Transformer substations, Connecting power grids, construction and modernization of communication and access routes place. Cerchezu, Constanta County

Document Control Sheet

| Project | "Wind power park 48 (46) wind power plants approx. 316.8MW (303.6MW), Transformer stations, Electrical connection networks, construction and modernization of communication and access routes" |
| --- | --- |
| Contracting Entity | SC South Wind SRL |
| Provider | SC Ramboll South East Europe SRL |
| Version | [Final Version] |
| Date | October 2025 |
| Document title | PRESENTATION MEMORANDUM |

| Made: Expert GIS Field activity and special contributions: | Vlad Dinu Ruxandra Florentina Buga Oana Falup Cristina Daniela Bordei Dan George Constantin Stelian Valentin STĂNESCU |
| --- | --- |
| Checked: | Gabriela Mușat |
| Approved | Ileana Fălcescu |

Contained I. Project name 6 II. Project owner 6 III. Description of the project according to the specifics of the investment, products and by-products obtained, size, capacities 7 III.1. Project summary 7 III.2. Justification of the need for the project 8 III.3. Investment Value 9 III.4. Proposed implementation period 9 III.5. Drawings representing the boundaries of the project site, including any land area required for temporary use (site plans and sites) 9 III.6. Description of the physical characteristics of the project, of the production processes of the proposed project, depending on the specifics of the investment, products and by-products obtained, size, capacity 11 III.6.1. Land use 11 III.6.2. Transport route for turbine components 12 III.6.3. Wind turbines 12 III.6.4. Foundations 13 III.6.5. Technology platforms 14 III.6.6. Transformer stations 15 III.6.7. Medium voltage LES (underground power lines) network (33 kV) 17 III.6.8. Profile and production capacities 17 III.6.9. Description of the installation and the technological flows existing on the site 17 III.6.10. Description of the production processes of the proposed project, depending on the specifics of the investment, products and by-products obtained, size, capacity 17 III.6.11. Raw materials, energy and fuels used, with how they are insured 17 III.6.12. Connection to the existing utility networks in the area 18 III.6.13. Description of the site restoration works in the area affected by the execution of the investment 18 III.6.14. New access routes or changes to existing ones 18 III.6.15. Natural resources used in construction and operation 20 III.6.16. Methods used in construction/demolition 20 III.6.17. Execution plan, including the construction, commissioning, operation, restoration and subsequent use phase 20 III.6.18. Relationship with other existing or planned projects 20 III.6.19. Details of the alternatives that were considered 23 III.6.20. Other activities that may arise as a result of the project 25 III.6.21. Other permits required for the project 25 IV. Description of the necessary demolition works 26 IV.1. Execution plan for demolition, restoration and subsequent use of the land 26 IV.2. Description of the site restoration works 26 IV.3. New access routes or changes to existing ones, as appropriate 26 IV.4. Methods used in demolition 26 IV.5. Details of the alternatives that were considered 27 IV.6. Other activities that may occur as a result of demolition 27 V. Description of the project location 28 V.1. The distance from borders for projects falling within the scope of the Convention on Environmental Impact Assessment in a Transboundary Context, adopted in Espoo on 25 February 1991, ratified by Law no. 22/2001, with subsequent additions 28 V.2. The location of the site in relation to the cultural heritage according to the List of Historical Monuments, updated, approved by the Order of the Minister of Culture and Cults no. 2.314/2004, as subsequently amended, and to the National Archaeological Repertoire provided by Government Ordinance no. 43/2000 on the protection of the archaeological heritage and the declaration of some archaeological sites as areas of national interest, republished, with subsequent amendments and completions 28 V.3. Maps, site photographs that can provide information on the physical characteristics of the environment, both natural and man-made, and other information 30 III.3.1. Current and planned land uses both on the site and in areas adjacent to it 30 III.3.2. Information on zoning and land use policies 31 III.3.3. Information on sensitive areas 31 V.4. The geographical coordinates of the project site, which will be presented in the form of a vector in digital format with geographical reference, in the Stereo 1970 national projection system 32 V.5. Details of any site variants that have been considered 32 YOU. Description of all significant effects of the project on the environment 33 VI.1. Sources of pollutants and installations for the retention, discharge and dispersion of pollutants in the environment 33 III.1.1. Protection of water quality 33 III.1.2. Air quality protection 34 III.1.3. Noise and vibration protection 35 III.1.4. Radiation protection 36 III.1.5. Soil and subsoil protection 36 III.1.6. Protection of terrestrial and aquatic ecosystems 37 III.1.7. Protection of human settlements and other objectives of public interest 41 III.1.8. Prevention and management of waste generated on site during project implementation/during operation, including disposal 42 III.1.9. Management of hazardous chemicals 46 VI.2. Use of natural resources, in particular soil, land, water and biodiversity 46 VII. Description of environmental aspects likely to be significantly affected by the project 47 VII.1. Characteristics of the potential impact 47 III.1.1. Impact on population and human health 47 III.1.2. Impact on flora and fauna outside Natura 2000 sites 48 III.1.3. Impact on soil and subsoil 48 III.1.4. Impact on material uses and goods 48 III.1.5. Impact on water quality and quantitative regime 48 III.1.6. Impact on air quality and climate. Climate 49 III.1.7. Impact on noise and vibration 49 III.1.8. Impact on the landscape and visual environment 50 VII.2. Extent of impact (geographical area, number of population/habitats/species affected) 50 VII.3. Magnitude and complexity of the impact 50 VII.4. Likelihood of impact 50 VII.5. Duration, frequency and reversibility of impact 50 VII.6. Measures to avoid, reduce or improve significant environmental impact 51 VII.7. Cross-border nature of the impact 51 VII.8. Exposure of the area to climate change 55 VIII. Provisions for environmental monitoring 77 IX. Link with other normative acts and/or plans/programs/strategies/planning documents 78 X. Works necessary for the organization of the site 79 X.1. Description of the works necessary for the organization of the site 79 X.2. Location of the site organization 79 X.3. Description of the environmental impact of the site organization works 80 X.4. Sources of pollutants and installations for the retention, discharge and dispersion of pollutants in the environment during site organisation 81 X.5. Facilities and measures provided for the control of pollutant emissions into the environment 81 XI. Site restoration works upon completion of the investment, in case of accidents and/or cessation of activity, to the extent that this information is available 82 XI.1. Proposed works for the restoration of the site at the completion of the investment, in case of accidents and/or at the cessation of the activity 82 XI.2. Aspects of prevention and response to accidental pollution cases (Plan of Accidental Pollution Response Measures and Provision of Necessary Means) 82 XI.3. Issues related to the closure/decommissioning/demolition of the facility 83 XI.4. Ways to restore the initial condition/rehabilitation for further use of the land 83 XII. Appendices – Cartoons 84 XIII. Information on protected natural areas of community interest 85 XIII.1.Brief description of the project and distance from the protected natural areas of community interest (ANPIC) 85 XIII.2.Name and code of the protected natural area of community interest 101 XIII.3.Presence and herds/areas covered by species and habitats of community interest in the project area 115 XIII.4.Clarifications regarding the direct link of the proposed project with or if necessary for the management of the conservation of the protected natural area of community interest 227 XIII.5.Estimation of the potential impact of the project on the species and habitats of the protected natural area of community interest 227 III.5.1. Identifying and estimating the impact 227 III.5.2. Results of the impact assessment based on the Specific Conservation Objectives 236 III.5.3. Identifying uncertainties 282 XIII.6. Conclusions on the description and quantification of impacts and why it is or is not necessary to continue the procedure with the appropriate assessment study stage 283 XIV. Additional data regarding the classification of the project under the provisions of art. 48 and art. 54 of the Water Law 107/1996 287 XIV.1. Localization of the project in relation to water bodies 287 III.1.1.River basin 287 III.1.2.Surface watercourses 287 III.1.3.Surface water bodies 287 III.1.4. Groundwater bodies 289 XIV.2. Indication of the ecological status/ecological potential and the chemical status of the surface water body; for the groundwater body, the quantitative status and chemical status of the water body shall be indicated 292 XIV.3. Indication of the environmental objective(s) for each identified body of water, specifying the exceptions applied and the related deadlines, as applicable 292 XV. Additional data regarding the classification of the project under the provisions of art. 48 and art. 54 of the Water Law 107/1996 294 XV.1. Features of the project 294 XV.2. Location of the project 295 XV.3. Types and characteristics of potential impact 296

# Project name

The name of the project is "Wind energy park 48 (46) wind power plants approx. 316.8MW (303.6MW), Transformer stations, Electrical networks for connection, construction and modernization of communication and access routes", proposed to be located in the outskirts of Cerchezu, Constanta County.

For the construction of the wind farm, the urban planning certificate no. 129/28.11.2022 with extended validity, by the Constanta County Council.

According to the Decision of the initial evaluation stage no. 54/07.02.2025, issued by the Environmental Protection Agency of Constanta, the following is mentioned:

- the project falls under the scope of Law 292/2018, falling under Annex no. 2, item 3, letter i) "Installations intended for the production of energy through the exploitation of wind energy" and 10 e - "Construction of roads, ports and port facilities, including fishing ports, other than those provided for in Annex no. 1”;

- the proposed project falls under the scope of art. 28 of GEO no. 57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna, with subsequent amendments and completions. The project site is located at a distance of approximately 2.8 km from the boundary of the site ROSAC007 I Dumbrăveni - Urluia Valley - Vederoasa Lake, approx. 6.3 km from ROSPA0l 66 Plopeni - Chimogeni, approx. 7.2 km from ROSPA0036 Dumbraveni, approx. 14.4 km from ROSACO157 Hagieni Forest Cotul Văii, approx. 18.6 km from ROSPA0094 Hagieni Forest,  ROSPA0l 51 Ciobăniţa Osmancea, approx 19.4 km from ROSPA000I Aliman-Adamclisi, at a distance of approx. 410 m from the protected natural area BG0000569 Kardam and at a distance of approx. 3.64 km from the protected natural area BG0000570 lzvorovo Kraiste;

- The proposed project does not fall under the provisions of art. 48 and 54 of the Water Law no. 107/1996, with subsequent amendments and completions.

- the project falls under the scope of Law no. 22 of 22 February 2001, for the ratification of the Convention on Environmental Impact Assessment in a Transboundary Context, adopted in Espoo on 25 February 1991, with subsequent amendments and completions, being included in Annex 1, item 22;

The presentation memorandum is prepared in accordance with Law no. 292/2018, respectively Annex no. 5E "The Framework Content of the Presentation Memorandum".

# Project owner

Name of the owner: SOUTH WIND SRL

Address of the holder: 4 Comana Street, 1st floor, sector 1 Bucharest, Romania, Phone: +40 728 030 019, E-mail: andreea.raducu@ox2.com ,Website: https://www.ox2.com/ro/romania

Contact person: Andreea Răducu, phone: +40 728 030 019, e-mail: andereea.raducu@ox2.com

Legal representatives/proxies: Lăcrămioara Diaconu-Pințea, Administrator South Wind SRL.

# Description of the project according to the specifics of the investment, products and by-products obtained, size, capacities

  - Project summary

The present project involves the construction of a wind farm (wind turbines), wind power plant construction including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer stations (owned), medium and high voltage electrical network and telecommunications network. The land on which the investment is to be made is located outside the built-up area of Cerchezu commune, Constanta County. The location of the wind farm will be arranged exclusively on the surface of land with the "arable" use category.

SOUTH WIND SRL intends to build and operate a wind power plant equipped with 48 (46) wind turbines with an individual power of 6.6 MW/h.

For the introduction of the energy produced into the grid, the wind turbines are connected to each other by an underground cable system (LES) up to the 33 kV/110 kV transformer stations (two in number) located within the studied area. In order to connect the wind farm to the National Energy System (SEN), an overhead/underground electrical connection will be made from the 33kV/110kV transformer stations to the 110 kV OHL in the area.

The electrical network connecting the turbines to the 33kv - 110kv transformer substations, located on plots A 5/28 and A 177/16, will be buried at a depth of 1.00 - 1.50 m, and will be located in the arranged/undeveloped exploitation roads in the studied area.

Based on the opinion of TRANSGAZ SA no. 34618/948/10.05.2023, 2 sites, the T17 and T23 power plants, which did not comply with the legal distances from the main pipes identified in the notice, were abandoned. The resulting wind energy park will consist of 46 wind power plants of 6.6 MW each, ultimately totaling a power of 303.6 MW.

For the implementation of the project, the following works will be carried out:

- Construction of foundations and technological platforms for the installation of wind turbines;

- Installation of SIEMENS GAMESA wind turbines of 6.6 MW each, with a total power of about 316.8MW (303.6 MW);

- Arrangement of agricultural exploitation roads for heavy traffic, within the limit of 5.5 m;

- Construction of 4 (2) 33 kV/110 kV transformer stations to the 33/110 kV transformer substation;

- Installation of 33 kV underground power lines for the interconnection of wind turbines with transformer stations.

In order to carry out the project, works will be carried out for the construction of turbine foundations and soil improvement, modernization of existing roads and construction of new roads inside the plots, construction of assembly platforms, construction of electrical transformer stations and connections with turbines, site organizations, installation of video monitoring poles, installation of settlement witnesses for monitoring the behavior of foundations.

Based on the solution study on the connection of the wind farm to the National Energy System, the project beneficiary will later decide on the connection solution, respectively through underground cables laid along the roads. These connection cables between the internal transformer stations and the connection point(s) to the SEN, as well as the main transformer substation/works necessary in the existing ones, are not the subject of this project, and will be established later, after the approval of the Solution Study for connection.

  - Justification of the need for the project

The transition to alternative energy sources is an imperative necessity for contemporary society.

Wind turbines, which produce electricity by converting the kinetic energy of the wind, contribute significantly to the sustainable development of the region due to their ecological and non-polluting characteristics. The installation and operation of these turbines is aligned with the major objectives of the European Union and Romania regarding the implementation of green energy production technologies.

Romania's efforts to obtain renewable and clean energy are in line with the legislation in force, such as EU Directive 2018/844 of 30 May 2018 and Directive 2009/28/EC of 23 April 2009. According to studies, the electricity generated by wind turbines has one of the lowest carbon footprints, with most of the emissions being generated in the process of manufacturing and building turbines (United Nations Economic Commission for Europe. Carbon Neutrality in the UNECE Region: Integrated Life-cycle Assessment of Electricity Sources, 2022).

During the period of operation, the emissions generated during maintenance operations are very low. Thus, wind turbines contribute to the prevention of CO2, SO2, NOx and dust emissions, which would have been generated by the use of non-renewable energy sources.

The implementation of the project in the studied territory totaling a power of 316.8MW (303.6MW) is based on the national and international Conventions on energy and climate change, based on which Romania developed the National Integrated Plan in the Field of Energy and Climate Change 2020 – 2030, as well as Romania's Energy Strategy 2022-2030, with the perspective of 2050 and Romania's Long-term Strategy for reducing GHG Neutral Romania in 2050.

The proposed project responds to the general objective of Romania's Energy Strategy, namely the growth of the energy sector in sustainable conditions, in compliance with national, European and global benchmarks, the implementation of this plan contributing to the development of the national energy system and also to the compliance with Romania's international obligations (Paris Agreement and subsequent strategies).

The continuous increase in the use of renewable resources has been identified as the main solution to reduce the environmental pressures generated by the energy sector due to the exploitation of fossil fuels.

In addition, building a "resilient Energy Union with a forward-looking climate change policy" is a major priority of the European Commission. The Energy Union Strategy, through its five dimensions: energy security, an integrated internal energy market, energy efficiency, decarbonisation of the economy and research and innovation, provides a framework for achieving this priority.

As part of the Clean Energy for All Europeans package, the EU also revised the EU's 2030 energy targets, with at least 32% of renewable energy, which was later revised as part of the Fit for 55 package to at least 40% by 2030.

Encouraging projects that enable the use of renewable energy ensures the dimension of decarbonising the economy. This dimension states that "an ambitious climate change policy is an integral part of our Energy Union" and aims to place the EU at the forefront of the world in renewable energy.

Directly, the project is not related to protected natural areas and is not necessary in the process of conserving species and habitats of community interest within protected natural areas. However, ensuring the use of energy from renewable sources will reduce greenhouse gas emissions, improve the quality of the environment and will have a positive impact on biodiversity.

It is estimated that the proposed project represents a major investment in the area, an investment that will generate viable opportunities, direct and indirect, to improve the long-term socio-economic situation of the community, without creating significant effects on environmental factors.

  - Investment Value

The value of the investments proposed in the project is in the range of approximately 500 - 550 million Euro.

  - Proposed implementation period

The duration of the execution period of the works is about 3 years from the date of their start. Estimated, the start date of the works is Q4 2026 with an end in Q4 2029.

  - Drawings representing the boundaries of the project site, including any land area required for temporary use (site plans and sites)

Wind energy park 48 (46) wind power plants approx. 316.8MW (303.6MW), Transformer stations, Electrical networks for connection, construction and modernization of communication and access roads" will be located outside the built-up area, in the western and eastern part of the administrative territory of Cerchezu commune, on the border of Chirnogeni, Independența, Dumbrăveni and Negru Vodă communes.

The location of the energy park is in the immediate vicinity of the border line with Bulgaria.

The land on which the project will be built borders on:

- North: border limit of the Independence ATU – privately owned agricultural land;

- South: border with Bulgaria;

- East: border limit of Chirnogeni and Negru Vodă TAU – privately owned agricultural land;

- West: border limit of the Independența and Dumbrăveni TAUs - privately owned agricultural land.

The main function of the studied area is agriculture, the agricultural land with the current arable function, has an area of 114.71 ha.

Figure no. 1 Project location

  - Description of the physical characteristics of the project, of the production processes of the proposed project, depending on the specifics of the investment, products and by-products obtained, size, capacity

The main components of the wind farm are:

- Wind turbines together with their support structure;

- Wind turbine foundations;

- Technology platforms;

- Transformer stations;

- Medium voltage LES (underground power lines) network;

- Technological roads for access to wind turbines.

The wind farm will most likely benefit from a lighting and video surveillance system.

    - Land use

The lands on which the investment proposed by the project is intended to be carried out are the private property of individuals/legal entities with whom the beneficiary of the investment SOUTH WIND SRL has concluded surface contracts.

According to CU no. 129/28/11/2022 issued by the Constanta County Council, the current use of the land is "arable", and the destination established by the approved urban and spatial planning plans is "land located outside the built-up area (TDE) – agricultural and electricity production area – wind farm and related constructions", with permitted functions: agricultural, production of electricity from renewable sources, circulations and related installations.

The total area of the proposed project is 114.71 ha.

The area studied for the implementation of the objective includes agricultural land for arable land and land for special use - exploitation roads in the public domain of the territorial administrative unit Cerchezu commune, administered by the Local Council of Cerchezu commune and the public domain of county interest.

The lands on which the investment subject to this documentation is to be developed are represented by the plots presented in the following table:

Table 1 Territorial balance sheet

| Component | Total ha |
| --- | --- |
| Foundations | 3,25 |
| Technology platforms | 55,76 |
| Transformer stations | 3,03 |
| The internal network of access roads, superimposed on the existing agricultural roads, 5.5 m wide that will be paved | 49,06 |
| Site organization | 3,61 |
| Total area | 114,71 |

The construction of the energy park will lead to the permanent occupation of some land areas:

- 58.79 ha for the location of wind power plants and complementary buildings and transformer stations

- 49.06 ha permanently occupied by exploitation roads and developed roads.

For each plot on which the wind farms will be installed, the definitive removal from the agricultural circuit of the areas occupied by the base of the pole, the access roads to the plants and the maintenance platforms will be requested in the DTAC phase. The energy park will lead to the definitive removal from the agricultural circuit of an area of approx. 30 ha and the temporary removal from the agricultural circuit of an area of approx. 70 ha.

The land on which the proposed project will be located is typical of the Dobrogean plateau, without considerable differences in level. The land is free of construction.

    - Transport route for turbine components

So far, the transport routes for the components necessary for the implementation of the wind farm have not been established.

    - Wind turbines

The height of the wind turbine pillar will be between 155-165 m, measured at the level of the rotor shaft, to which is added the rotor, with 3 blades of 75-85 m long. The main components of the wind turbine are mentioned below:

- The tower (pillar) – consisting of several sections, with a height of 155-165 m (in front of the nacelle), is a metal structure with a slightly conical shape

- The boom lift – positioned at the top of the tower, includes as component parts: the rotor shaft, the gearbox, the generator and the transformer. A medium intensity light warning will be installed on the gondola, according to the requirements of the Romanian Civil Aviation Authority.

For the choice of turbine locations, the safety and protection distances from the elements of urban infrastructure were taken into account, indicated by the "Technical Norm on the delimitation of the protection and safety areas related to the energy capacities", approved by Order no. 239/2019 of ANRE.

The following figure illustrates the vertical dimension for the wind turbine.

Figure no. 2 Schematic representation of vertical dimensions for wind turbine

    - Foundations

The reinforced concrete foundations for the installation of wind turbines will have a cylindrical shape plus foundation pillars, variable diameter between 18-30 m and variable depth (depending on local foundation conditions). The depth and diameter of the foundation, for each turbine, will comply with the minimum requirements imposed by the turbine manufacturer and will be calculated according to the geological structure, which will be identified by the Geotechnical Study.

The following figure shows the turbine foundation section.

Figure no. 3 Turbine Foundation Section

The area occupied by the foundations of the wind turbines is 3.25 ha.

    - Technology platforms

The technological platforms for mounting the cranes will be located next to the turbines and will have average surfaces between 700 and 1,200 m2, meaning cobbled land surfaces on which cranes used for the installation, maintenance and decommissioning of wind turbines will be mounted. In some cases, between the technological platform and the exploitation road at the end of the plot there will be a short section of inner road, with a width of approx. 5 m. The following figure schematically presents an example of a technological platform proposed for the implementation of the project.

Figure no. 4 Schematic representation of the wind turbine technology platform

The area occupied by the technological platforms for wind turbines is 55.76 ha.

    - Transformer stations

The proposed wind farm will have 3 areas for internal 33/110 kV transformer stations whose location has been optimized according to the distances from the wind turbines served.

The internal 110/33 kV transformer stations will be indoor-outdoor type, the 110 kV equipment being located outdoors and the 33 kV indoor ones. The stations will be equipped with a 110 kV cell, 33/110 kV transformers of different capacities (e.g. 63 MVA) and the medium voltage cells will be connected to a simple unsectioned busbar. Also, the stations will be equipped with a 30/0.4 kV internal service transformer, generator set, own service installations, communication, protection and electricity metering systems, as well as anti-burglary and anti-fire systems.

The areas allocated to the ground construction of the components of the 33/110 kV transformer stations are 3.03 ha.

The transformer stations will operate in automatic mode and will not have permanent operating personnel. There will be no water supply or sewerage systems at the transformer stations.

The proposed locations for these transformer stations are shown in the following map.

Figure no. 5 Proposed locations for transformer stations

    - Medium voltage LES (underground power lines) network (33 kV)

The external LES network (connection to the SEN) is not the subject of this Presentation Memorandum, it will be regulated from the point of view of environmental protection through a separate procedure. However, it was considered cautious that the current version of the proposed solution should be considered, as it is an essential part of the draft. However, the current version is not a definitive one, given that the connection study is not completed.

The total length of the underground turbine interconnection lines (internal) in the wind farm is 57.78 km.

The areas necessary for the underground power lines were not included in the territorial balance because the ditches will be located along the exploitation roads (under one of the sides), which constitute the access roads of the park, already quantified separately.

    - Profile and production capacities

It is proposed to build a wind farm by placing 46 SIEMENS GAMESA wind turbines of 6.6 MW each, with a total power of about 316.8MW (303.6 MW). The production capacity of the wind power plant depends mainly on the wind speed.

    - Description of the installation and the technological flows existing on the site

At the moment there are no installations on the site and no technological flows are carried out. The site of the future wind farm is on land currently used for agriculture.

    - Description of the production processes of the proposed project, depending on the specifics of the investment, products and by-products obtained, size, capacity

Within the proposed wind farm, electricity will be produced and delivered to the National Energy System (S.E.N.). Connection cables between the internal transformer stations and the connection point(s) to the SEN are not the subject of this memorandum and will be finalized later, once the Solution Study for connection is approved.

    - Raw materials, energy and fuels used, with how they are insured

In the execution phase of the wind farm, construction materials will be used that will be necessary to carry out the works provided for in the project: different types of concrete, reinforcements for the construction of foundations, ballast, sand, crushed stone, geotextile, etc. At this time, no information is available on the quantities of raw materials and fuels needed in the execution phase. It should be noted that it may undergo small changes.

Also, during the execution period, fuels and oils necessary for the operation of the vehicles and machinery involved in carrying out the works will be used, these being stored in the area allocated to the site organizations.

During the operating period

The operation of wind turbines does not require the use of raw materials, but only consumable materials (oil and air filters) and various chemical substances and preparations (oils, lubricants, greases, paints), which must be changed at certain intervals of time, as part of maintenance activities. At this time, the quantities of consumables needed in the operation phase are not known.

    - Connection to the existing utility networks in the area

The proposed project does not involve the connection to the existing utility networks, telephony-data, drinking water, sewerage, gas or electricity (other than the power lines included), neither during the works nor during the exploitation of the investment.

    - Description of the site restoration works in the area affected by the execution of the investment

Upon completion of the project execution works, the temporarily occupied land areas will be brought back to their initial state and as far as possible they will be returned to the agricultural circuit.

    - New access routes or changes to existing ones

The access roads overlap on the routes of the existing agricultural and communal roads, except for the curve sectors where widening works are necessary, for the registration of oversized vehicles, on some private land (contracted by the beneficiary). The widening in curves will be ensured in all cases by the use of private properties in the vicinity of the road.

The following figure shows an example of widening of access roads in areas with intersections.

Figure no. 6 Schematic representation of the widening of access roads in areas with intersections

The widening of the agricultural roads included in the project will contribute to ensuring road access roads for:

- Transport of wind turbines, components of 33/110 kV transformer stations, auxiliary components, as well as construction materials necessary for the construction of foundations and technological platforms during the execution phase.

- Transportation of various technical components and construction materials during the operation and maintenance phase of the wind energy investment.

Each turbine will have an adjacent and an internal technological access road, all of which will be located on land outside the built-up area

    - Natural resources used in construction and operation

In the execution phase of the project, natural resources and construction materials will be used that will be provided by the companies involved in the implementation of the project, such as different types of concrete, reinforcements for the construction of foundations, sand, ballast, crushed stone, geotextile, etc.

In the operation phase of the project, it is not necessary to use natural resources, only in exceptional cases for various maintenance works. However, the quantities needed will be reduced.

    - Methods used in construction/demolition

At the construction works, the level of quality will be ensured according to the requirements imposed by Law 10/1995 on quality in construction. New traditional and specific products, processes and equipment will be used, for which there will be appropriate technical approvals.

For the implementation of the project, no demolition works are required.

    - Execution plan, including the construction, commissioning, operation, restoration and subsequent use phase

The execution period of the works is estimated at 3 years from the beginning. The commissioning will be established according to the completion date of the transformer and connection stations.

The duration of the exploitation phase of the wind farm is 40 years.

    - Relationship with other existing or planned projects

In order to establish the relationship with other existing or planned projects for the project "Wind Energy Park 48 (46) EC, Transformer stations, Electrical connection networks, construction and modernization of communication and access roads", proposed to be located in Cerchezu, Constanta County, information regarding the projects approved in the period 2020-2024 as well as those in progress were requested from the competent authority for environmental protection of Constanta through the address no. 37/10.05.2024 approved on the territory of Cerchezu locality and the territory of the neighboring localities (Măgura, Căscioarele, Viroaga, Chirnogeni, Negru Vodă, Olteni, Dumbrăveni, Independența, Plopeni), the operators who carry out activities on the administrative territory of Cerchezu locality, the data on the location of the projects/plans in relation to the limits of these protected areas and the areas occupied by them within the limits of the protected areas, etc.

By the address of APM Constanța no. 5836/ 12.06.2024 were submitted regarding the projects/plans under the environmental assessment procedure/completed, identified as being in the project's area of influence.

According to the available information, in the vicinity of the proposed wind farm, both on the territory of Romania and Bulgaria, there are the following existing/proposed projects:

- Existent:

  - "Wind energy park 32 wind power plants, total power 80 mw, transformer station, electrical connection networks, construction and modernization of communication and access routes", located in the communes of Chirnogeni and Independencia. The distance between the proposed and existing wind farm is approximately 1.5 km (the distance between the proposed T24 turbine and the existing T1).

  - Wind farm on the territory of Bulgaria, located at a distance of 5.6 km from the proposed plan (distance between the proposed T3 turbine and the existing T6).

- Proposed:

  - Wind energy park 14 CE-92.4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - outside the built-up area of Fântâna Mare, Independența commune, county. Constanta

  - Wind energy park 17 CE-112.2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extra-built-up area of Independența commune and Cerchezu commune. Constanta

  - Comana - Pecineaga Wind Farm - MV underground power cables and fiber optic network route

  - Construction of a wind power plant including: wind turbines, access roads and assembly/maintenance platforms, transformer substation (owned), medium and high voltage electrical network, telecommunications network, Independența commune, Bucharest county. Constanta

  - Demolition of existing buildings and construction of a 3.212 MW photovoltaic power plant, connection to the SEN, location of PC and PTAB arrangement of internal road and access, fencing of land and organization of the site located in Independența locality

  - Construction of underground fiber optic network for electronic communications services in Constanta County, Independența commune, Dumbrăveni (proposed)

  - Construction of underground fiber optic communications network for electronics services, Negru Vodă (proposed)

  - Electricity supply for the water household, Furnica village, Dumbrăveni commune, Bucharest county. Constanta (proposed)

  - Relocation of the international natural gas transmission pipeline dn 1000T1 Isaccea - Negru Vodă, in the ROMCIM quarry area, PL Medgidia (proposed)

  - General Urban Plan of Independența commune (proposed)

  - Establishment of an intelligent natural gas distribution network in Independența commune, Constanta county (proposed)

  - Construction of a bridge in Movila Verde, str. Bisericii, Movila Verde locality, Independența commune, Constanta county (proposed)

  - Asphalting of Comana, Tătaru and Pelinu streets, stage I, Comana commune, Constanta County (proposed)

  - Construction of bridges, bridges and drainage canals in Credința, Chirnogeni commune (proposed)

  - Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed)

Figure no. 7 Location of existing or planned projects in the project area

    - Details of the alternatives that were considered

In the process of implementing the project, 4 alternatives were studied:

- The "zero" alternative - no project implementation

- Alternative I – was initiated in 2011 (consisting of 3 wind farms) – 117 wind power plants, with a total installed capacity of 351 MW

- Alternative II - was initiated in 2022 - 48 wind power plants, with a total installed capacity of 316.8 MW

- Alternative III - was initiated in 2023 after obtaining the approval of TRANSGAZ SA no. 34618/948/10.05.2023, following which 2 T17 and T23 plants were abandoned, thus the new wind farm will include 46 wind farms, with a total installed capacity of 303.6 MW.

The advantages and disadvantages of each alternative and the justification for choosing the optimal alternative are presented below.

The 'zero' alternative was taken into account as a reference element against which the other alternatives are compared for the different elements of the project analysed. The main forms of impact associated with the adoption of the "zero" alternative are:

- the loss of the opportunity to capitalize on the high wind potential in this area;

- loss of job opportunities (estimated at 30 ÷ 75 direct hires in the pre-construction and construction phases);

- the loss of economic opportunities that would support the local community in socio-economic development that would allow the resolution of environmental problems;

- loss of interest of private investors in future industrial development projects in the region and in Romania;

- loss of support for the development of a reliable alternative for electricity production in accordance with policies developed at European and national level;

- failure to achieve the objectives assumed at national level regarding the decarbonization of the energy sector, the promotion of renewable energy production resources for a neutral Romania in terms of GHG emissions by 2050.

Alternative I consists of building three wind farms consisting of 117 wind turbines of 3 MW/turbine, the total power of the park being 351 MW and 3 transformer stations.

For this variant, a total land area of 473.09 ha is required (according to the Decisions of the classification phase issued in 2013: DEI 14866/29.04.2013, DEI 14867/22.04.2013, DEI 14868/22.04.2013) of which 3.90 ha permanently occupied by constructions and 469.19 ha temporarily occupied by works. In this variant, the land with the current agricultural-arable function requires the transformation of the category of use into "technical-urban facilities", which implies the removal of 5 ha of arable land from the agricultural circuit.

The advantage of this alternative is the total capacity of 351 MW of the three wind farms that will replace an equivalent power of electricity produced from fossil fuels, thus contributing to the fulfillment of national and European commitments to decarbonize the energy system and diversify energy production sources, ensuring energy independence.

The disadvantage of Alternative I is represented by the large number of wind turbines provided for by the project, which implies a high density of turbine placement with effects on inhabited areas through an increase in noise levels, an intensification of the shading phenomenon, an agglomeration of the natural landscape.

To these is added the land area affected by the construction of new roads, foundations and platforms related to the 117 wind turbines and the 3 transformer stations.

In addition, in 2011, the date of the Adequate Assessment Study for the 3 parks around Cerchezu, from which the analysed project emerged, the site ROSPA0166 was not declared as a protected area and the Adequate Assessment Study was not based on the assessment in relation to the conservation objectives of the Natura 2000 sites, since at that time the specific conservation objectives were not defined. The major conclusion of this Adequate Assessment Study was that the location of the wind turbines in the area of interest, an area already anthropized, will have an insignificant impact on the local vegetation, flora and fauna.

Alternative II consists of building a wind farm consisting of 48 wind turbines with a power of 6.6 MW/turbine, the total power of the park being 316.8 MW and 4 transformer stations.

For this variant, a total land area of 3,027.77 ha is required, of which 309.34 ha is permanently occupied by constructions and 2,718.43 ha is temporarily occupied by works. In this variant, the land with the current agricultural-arable function required the transformation of the category of use into "technical-urban facilities", which implies the removal of 309.34 ha of arable land from the agricultural circuit.

The advantage of this alternative is the significantly smaller number of turbines, which minimizes the disadvantages of the first variant and the placement of turbines at greater distances from each other.

By reducing the number of turbines envisaged by the project, the distance between wind farms is increased to over 600 m, compared to around 350 m in the situation approved in 2011, and the lower density has the effect of a more balanced use of natural resources and reduces the pressure on biodiversity both during the implementation period of the plan and during the operation period of the wind farm.

The disadvantage of Alternative II is represented by the lower power of the energy park and the incompatibility of the locations of the T17 and T23 turbines with the main natural gas transmission network, which led to the emergence of Alternative III.

Alternative III consists of building a wind farm consisting of 46 wind turbines with a power of 6.6 MW/turbine, the total power of the park being 303.6 MW and 2 transformer stations.

For this variant, a total land area of 3,027.77 ha is required, of which 292.52 ha is permanently occupied by constructions and 2,735.25 ha temporarily occupied by works. In this variant, the land with the current agricultural-arable function required the transformation of the category of use into "technical-urban facilities", which implied the removal of 100 ha of arable land from the agricultural circuit.

The advantage of this alternative is compatibility with the natural gas transmission network, to which are added the advantages of Alternative II related to the lower number of wind turbines and lower density.

The disadvantage of Alternative III is the lower power of electricity produced.

Following the analysis of the alternatives, alternative III is considered optimal, for the following reasons:

- minimizing negative environmental impacts by reducing the number of wind turbines;

More efficient use of air currents through optimal positioning of turbines relative to each other, at greater distances, which leads to their operation in the best conditions and to an increase in the efficiency of the wind farm.

    - Other activities that may arise as a result of the project

The implementation of the project will not lead to the emergence of other activities in the area. However, the implementation of the project brings a contribution to achieving the objectives of Romania's Energy Strategy to grow the energy sector in conditions of sustainability and economic growth, taking into account the EU targets for 2030. The development of the renewable energy sector is a strategic objective for national energy security and economic development.

At the local level, as a result of the project, new jobs will be generated, especially during the construction period, but also later in the operation and maintenance phase of the wind farm.

    - Other permits required for the project

The approvals and authorizations requested for the project according to the Urban Planning Certificates no. 40 of 30.03.2022, no. 74 of 22.06.2022 and no. 210 of 14.12.2023 issued by the Vaslui County Council are as follows:

- Water supply – RAJA SA Constanta or the alternative system used in the absence of the public network;

- Electricity supply – E-Distribuție Dobrogea SA;

- Telephony – Telekom Romania Comunication SA/ Orange Romania SA/ RCS&RDS;

- Sanitation – Construction waste collection operator;

- Office of Cadastre and Real Estate Advertising Constanta;

- Constanta County Directorate for Culture;

- M.A.D.R. – Directorate for Agriculture of Constanta County;

- A.N.I.F. – I.F. Constanta Territorial Branch;

- M.A.I, S.R.I, M.A.P.N – Major Defense Village;

- Special Telecommunications Service;

- the Romanian Civil Aviation Authority;

- R.A.J.D.P. Constanța

- S.N.T.G.N. Transgaz SA Mediaș – Territorial exploitation of Constanta;

- C.N.T.E.E.  Transelectrica SA – S.T. Constanța;

- Agricover Credit IFN SA – registration of the right to the mortgage;

- Agreement for private owners of individuals/legal entities;

- Geotechnical study;

- Coexistence study with other objectives and activities;

- Project verification reports for quality requirements in accordance with the provisions of Law no. 10/1995 (republished) on quality in constructions;

- Situation plan on topographic/cadastral support drawn up in the Stereographic 1970 projection system, endorsed by the cadastre and real estate advertising office, in .pdf and .dwg format or .dxf.

- The point of view/administrative act of the competent authority for environmental protection.

# Description of the necessary demolition works

  - Execution plan for demolition, restoration and subsequent use of the land

The project does not involve the demolition of existing objectives. The area of approximately 114.71 ha will be used exclusively for the construction of a wind farm, foundations, technological platforms, access roads, internal electrical network, 33-110 kV transformer stations and site organization.

At the end of the operating period, the necessary works will be those of dismantling/demolishing the buildings, bringing the soil to level, then returning the land to the agricultural circuit or leaving it in its natural state, depending on the agreement with the owner of the land.

  - Description of the site restoration works

Upon completion of the construction works, the Contractor will ensure the restoration of the natural framework of the temporarily occupied areas, but which are not occupied by the project interventions. The areas affected by the construction works will be brought to a state that ensures the landscape integration of the elements subject to restoration works. These works will be carried out by sanitizing the area (completely removing the waste resulting from the activities specific to the work fronts, including household waste), planting species from the vegetation specific to the area, etc. The restoration works have both the purpose of ensuring the landscape restoration of the affected areas, and that of reducing the risk of entry and installation of invasive alien plant species on the affected areas, which would endanger the areas in the vicinity of the proposed project.

  - New access routes or changes to existing ones, as appropriate

Given the fact that demolition works are not necessary for the implementation of the project, new access roads or changes to the existing ones will not be necessary in this regard.

  - Methods used in demolition

This is not the case. The project does not involve demolition works of existing objectives. The demolition works at the end of the operation phase will be carried out mechanized. If it is necessary to decommission the wind farm, it will mainly consist of the following activities:

- Works for the establishment of the site organization - similar to the stage of execution of the construction works of the wind farm. The chosen location will be used temporarily and will be returned to its original state upon completion of the works; Dismantling the turbines and removing them from the site;

- Demolition of the foundations to a depth that allows the resumption of agricultural activities (about 1 m depth from ground level) and soil supply to the demolished area;

- Digging up the underground electrical cables and removing them from the construction site;

- Decommissioning of transformer substations - dismantling equipment and installations and removing them from the site; Decommissioning of the technological platforms and access roads inside the plots in order to hand them over to the agricultural circuit;

- Construction site restoration works in the affected areas.

  - Details of the alternatives that were considered

Alternatives for demolition were not considered. This is not the case.

  - Other activities that may occur as a result of demolition

This is not the case, given the fact that this project does not propose demolition works.

# Description of the project location

  - The distance from borders for projects falling within the scope of the Convention on Environmental Impact Assessment in a Transboundary Context, adopted in Espoo on 25 February 1991, ratified by Law no. 22/2001, with subsequent additions

Given that the project has a common border with the border with the Republic of Bulgaria, it is mentioned that the project falls under the Convention on Environmental Impact Assessment in a Transboundary Context, adopted in Espoo on 25 February 1991, ratified by Law no. 22/2001, with subsequent additions, being included in Annex 1, item 22.

  - The location of the site in relation to the cultural heritage according to the List of Historical Monuments, updated, approved by the Order of the Minister of Culture and Cults no. 2.314/2004, as subsequently amended, and to the National Archaeological Repertoire provided by Government Ordinance no. 43/2000 on the protection of the archaeological heritage and the declaration of some archaeological sites as areas of national interest, republished, with subsequent amendments and completions

According to the List of Historical Monuments 2015 in Constanta County approved by Order no. 2361/2010, the following objective of historical interest was identified in the area of the proposed project.

Table 2 Objectives of interest in the site area

| LMI code | Locality | Name | Dating |
| --- | --- | --- | --- |
| CT-IV-m-B-02967 | sat VIROAGA; commune SEARCH | Obelisk in memory of the heroes of the First World War | 1936 |

Also, in the ona area, there are archaeological sites located exactly that are not included in the List of Historical Monuments, but have been identified with the help of the National Archaeological Repertoire. These are:

- Rustic villa in Viroaga – RAN code 61201.01 - Viroaga village, Cerchezu commune

- Căscioarele Mound - RAN code 61185.03 - Bacu Movil - Căscioarele village, Cerchezu commune

- The legionary camp of Căscioarele - RAN code 61185.01 - Căscioarele village, Cerchezu commune

Figure no. 8 Locations of historical monuments in relation to project elements

  - Maps, site photographs that can provide information on the physical characteristics of the environment, both natural and man-made, and other information

The project will be developed exclusively on land that currently has the category of agricultural-arable use, pasture, communal road, exploitation roads. In the following figures are presented some images of the site on which the project is proposed to be implemented.

Figure no. 9 Images of the existing situation of the project area

    - Current and planned land uses both on the site and in areas adjacent to it

According to the urban planning certificate no. 129 of 28.11.2022, the land related to the investment has the category of agricultural-arable use and the proposed destination "land located outside the built-up area (TDE) – agricultural and electricity production area – wind farm and related constructions."

The land in the vicinity of the project is used for agricultural purposes.

    - Information on zoning and land use policies

According to the urban planning certificate, the project will be developed in the outskirts of the localities on land that has an agricultural-arable use category. Also, previously for the project analyzed in this presentation memorandum, the following zonal urban plan was carried out: "The wind energy park 48 (46) wind power plants approx. 316.8MW (303.6MW), Transformer stations, Electrical connection networks, construction and modernization of communication and access roads".

    - Information on sensitive areas

The sensitive areas in the project area that require a closer analysis in terms of the potential effects that the project may have on them, are represented by: protected natural areas, surface water bodies, inhabited areas, ecological corridors and forest areas.

Protected areas

In the vicinity of the project, at a distance of less than 5 km, a number of 3 natural areas of community interest have been identified:

- 1 area located on the territory of Romania ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake at a distance of about 2.79 km from the project limit;

- 2 areas located on the territory of Bulgaria - BG0000569 Kardam (Bulgaria) located at a distance of about 410 m from the project limit and BG0000570 Izvorovo – Kraishte located at a distance of about 3.64 km from the project limit).

In the project implementation area at distances between 5 - 20 km there are a number of 5 protected natural areas:

- 5 protected natural bird protection areas:

  - ROSPA0166 Plopeni-Chirnogeni at a distance of 6.21 km from the project limit

  - ROSPA0036 Dumbrăveni located at a distance of 7.07 km from the project limit

  - ROSPA0094 	Hagieni Forest at a distance of 18.39 km from the project limit

  - ROSPA0151 	Ciobănița-Osmancea at a distance of 18.55 km from the project limit

  - ROSPA0001 	Aliman – Adamclisi. at a distance of 19.12 km from the project boundary

- 2 protected natural areas of community interest:

  - ROSAC0157 Hagieni Forest - Bend of the Valley at a distance of 13.81 km from the project limit

  - BG0000130 Kraymorska Dobrudzha (Bulgaria) at a distance of 11 km from the project boundary

Surface watercourses

The inland power lines intersect 2 surface water courses (at two points each), namely Cerchez cadastral code XIV_1.40.1.1. and Magura cadastral code XIV_1.40.1.1.

More details about water bodies are presented and analyzed in chapter 14 of this memorandum.

Localities

The wind farm is proposed to be built in the outskirts of Cerchezu commune, the nearest localities being: T32 at 1.1 km from Viroaga, T5 at 1 km from Cerchezu, T35 at 1 km from Magura, T3 at 1.56 km from the Bulgarian locality of Iovkov, T47 at 2.1 km from the Bulgarian locality Kraiste, T9 at 3.98 km from Negru Vodă.

Culturally important areas

The project does not intersect any historical monument, the nearest site, the Căscioarele Movila Bacu Mound is located at a distance of about 636 m from T27 and about 1 km from T30. The legionary camp is located about 742.26 m from the T27 turbine.

Detailed presentations of the culturally important areas are presented in section 5.2 of this memorandum.

Eco-friendly corridors

The nearest ecological corridors for otters are located approx. 14 km away.

  - The geographical coordinates of the project site, which will be presented in the form of a vector in digital format with geographical reference, in the Stereo 1970 national projection system

The STEREO 70 coordinates of the project are presented in excel format and shapefile in Annex B of the presentation memorandum.

  - Details of any site variants that have been considered

From the point of view of location, it was chosen in 2011 based on the general wind maps for Romania, which showed that Constanta County is located in an area with very good potential with a constant speed of 6.65 m/s. The Cerchezu ATU was chosen due to the presence of flat relief and generally favorable orientation NNV-S.

In order to establish the locations of the wind turbines, the following aspects were taken into account:

- the willingness to easily contract private land;

- logistics/transport accessibility;

- energy potential (calculated using the windPRO program)

- distances to localities; distances from the network of protected areas / Natura 2000;

- proximity distances from the energy grid.

# Description of all significant effects of the project on the environment

  - Sources of pollutants and installations for the retention, discharge and dispersion of pollutants in the environment

    - Protection of water quality

Sources of pollutants for water, discharge site or outfall

Inland power lines intersect 2 surface watercourses.

However, the implementation of the project does not involve surface water abstractions, groundwater supply boreholes or discharges into surface waters.

In the execution phase, the main sources with potential pollutants for water are represented by:

- Soil handling works - the generation of soil particles that can reach surface waters by stream. In the case of large amounts of suspensions, they can accumulate in watercourses, generating changes in water turbidity with effects on aquatic flora and fauna;

- Site traffic to and from the work fronts or areas from which construction materials are brought;

- Accidental leaks of chemicals, fuels and oils from the operation of the machinery involved in the construction works or due to the faulty handling of transport vehicles;

- Improper storage and management of waste resulting from construction works;

- Inadequate storage and management of domestic wastewater resulting in sanitary groups within the site organization, the management being properly ensured through authorized operators.

It is specified that the organization of the site is not proposed in the vicinity of surface water bodies, the risk of contamination associated with the sources presented above being thus very low.

In the operation phase of the project there will be no direct sources of emissions in the water. The project does not propose the discharge of water into groundwater or surface water bodies. The only sources with the potential for contamination of water bodies are accidental in nature and can be caused as a result of accidental leakage of substances used in maintenance work carried out on turbines or in the transformer station (mainly oils and other substances that grease the mechanical part of turbines).

In the decommissioning phase, the main sources are associated with the organization of the site and the storage areas of the materials resulting from the demolition. The main potential sources for groundwater can be associated with temporary storage and inadequate management of waste resulting from demolitions, in particular waste oil collected from wind turbine generators.

The potential sources that can generate negative effects on surface and groundwater at this stage are similar to the execution stage.

Wastewater treatment or pre-treatment plants provided for

The project does not provide for wastewater treatment or pre-treatment facilities in any of its stages. In the execution phase, ecological toilets will be provided in the site organization that will be periodically emptied by the economic operator that will provide these facilities.

    - Air quality protection

Sources of air pollutants, pollutants, including sources of odours

In the execution phase, the main sources of air pollutants are:

- Activities of handling land masses (uncovering fertile soil, excavations, filling, leveling, loading, unloading, transport), construction materials (sand, gravel, ballast) – undirected stationary sources. Pollutants: particulate matter and sedimentable dust;

- Temporary storage of powdery materials (sand, soil) that can be carried by the wind. – stationary undirected sources. Pollutants: particulate matter and sedimentable dust;

- Wind erosion on disturbed or vegetated land areas – undirected stationary sources. Pollutants: particulate matter;

- Generator sets for ensuring energy supply in site organizations and work fronts – stationary directed sources. Pollutants: NO2, SO2, CO, dust;

- Mobile emission sources (vehicles and machinery participating in the landscaping as well as in the transport of materials and equipment during the execution of construction works. – stationary directed sources. Pollutants: NOx, SOx, CO, particulate matter, heavy metal particles;

- Activities of handling and assembly of wind turbine components – stationary undirected sources.

The stationary undirected sources of air purification will appear during the execution period of the works proposed for the achievement of the objective and will be represented by the activities of handling the land masses (excavation works, soil uncovering, loading – unloading, transport), and by the activities of handling and assembling the components of the wind turbines. The dust generated by material handling and wind erosion is mainly of natural origin (soil particles, mineral dust).

The sources specific to the construction period will be mainly open surface sources. Their operation will be intermittent, depending on the work schedule and the schedule of the works.

During the period of operation of the objective, there will be no sources of emission of air pollutants. Occasionally, maintenance operations may be carried out. These operations generate emissions of air pollutants mainly represented by the vehicles used for the movement of personnel, but their contribution is insignificant.

During the decommissioning period, the emission sources involved will be similar to those mentioned during the execution phase.

Installations for the retention and dispersion of pollutants in the atmosphere

During the execution phase of the project, significant amounts of air pollutants are not generated. The sources of pollution during the execution period are free, open. For this reason, it is not necessary to provide facilities for capturing – purifying – discharging impurified air/waste gases into the atmosphere. If, during periods without precipitation and with higher wind speeds, larger amounts of dust will be generated from the surfaces without vegetation, the wetting of these surfaces will be considered.

This  type of installation is not required during the operating period.

In the decommissioning phase, the quantities of air pollutants will be similar to those in the execution phase, so no other actions will be proposed.

    - Noise and vibration protection

Noise and vibration sources

During the execution period of the wind farm, the noise sources will be temporary. It manifests itself locally and for a limited period of time.

As previously specified, the noise sources related to the execution stage of the works are temporary. We specify that the machines that will be sources of noise during the entire execution period will operate mainly during the day.

The main sources of noise and vibrations on the site during the execution of the works will be represented by:

- Traffic in the area of the work fronts (temporary turbine platforms) and on the access roads;

- Excavation activities, material handling, respectively loading and unloading;

- Operation of machinery (transport machines, heavy trucks, concrete mixers, excavators, cranes, bulldozers, compressors) – operation of engines, handling and transport of loads;

- Activities of handling and assembly of wind turbine components.

In the construction activities that will be carried out within the analyzed project, the vibrations will manifest themselves repetitively, for a short period of time. The area of vibration manifestation is limited to the area of the work fronts.

In the operation stage there are two main categories of noise sources that will be generated:

- Noise caused by the rotation of the blades (contact with air);

- The mechanical noise created by the operation of the elements in the nacelle (generator, gearbox, drivetrain).

The noise sources in the decommissioning phase are similar to those in the execution phase.

Arrangements and equipment for protection against noise and vibrations

In order to avoid and reduce the noise and vibrations generated during the execution phase, the speed of movement of machinery and vehicles will be limited (maximum 30 km/h), especially in the areas near Viroaga, Căscioarele, Magura, Cerchez. Only equipment and machinery with a low noise level will be used, and the vehicles will be periodically checked to keep them in good working order and will be stopped while stationary.

    - Radiation protection

Radiation sources

During the activities carried out in the project stages, no radioactive substances will be used or transported. According to specialized studies, wind farms do not pose a risk of exposure to radiation. The electromagnetic field levels in the vicinity of wind turbines are actually lower than those produced by the most common household electrical devices.

Arrangements and facilities for radiation protection

No additional arrangements and equipment are required for protection against radiation sources.

    - Soil and subsoil protection

Sources of pollutants for soil, subsoil, groundwater and deepwater

In the execution phase of the works, the potential sources of contamination/degradation for the soil, subsoil and groundwater are represented by:

- Improper storage of construction equipment and materials;

- Improper management and storage of waste resulting from the works, as well as household waste resulting from the personnel involved in the execution of the works;

- Traffic of vehicles and machinery involved in achieving the objective. With the impurity of the air, there is a possibility that a certain amount of atmospheric pollutants (SO2, NOx, heavy metals) can reach the soil, which can lead to changes in its characteristics;

- Accidental leakage of fuels, lubricants and other substances from vehicles and machinery involved in construction works or from their improper storage;

- Degradation of soil quality by improper handling/storage of the discovered/excavated material, implicitly the occurrence of erosion and/or spreading phenomena;

- Soil contamination with germinal material belonging to invasive and potentially invasive ruderal and/or non-native species, as a result of soil handling activities, as well as traffic of machinery and work personnel;

- Deposition of dust resulting from excavation, loading, transport and unloading of construction materials;

- Inadequate management of domestic wastewater resulting from the organization of the site and work fronts.

In the operation phase, the potential sources of pollution will be the following:

- Road traffic of maintenance crew vehicles, which are an occasional source of atmospheric emissions (CO, NOX, SO2, PM10 and heavy metals) from exhaust gases.

- Accidental leakage of fuels, lubricants from waste transport vehicles and personnel involved in maintenance activities.

In the decommissioning phase of the project, the potential sources of soil pollution will be similar to those in the construction phase, the works being carried out with the same types of machinery.

Works and facilities for soil and subsoil protection

The project provides for the following works and facilities for the protection of the soil and subsoil:

- The layer of topsoil removed from the areas where the trenches for laying the LES cables on the site will be made will be stored in separate piles and will be reinstalled after the completion of the works, in order to make possible the natural reinstallation of the vegetation;

- The layer of soil removed for the construction of the foundations will be used as necessary in the execution phase and the rest of the quantities can be used as fillers in the case of other projects;

- Upon completion of the excavation works for laying the LES cables, restoration works will be carried out on the affected land by leveling the earth;

- The transformer stations will be provided with oil-resistant floors, so as to avoid environmental pollution with possible oil leaks from the power transformer tank.

    - Protection of terrestrial and aquatic ecosystems

Identification of sensitive areas that may be affected by the project

The project does not intersect protected natural areas of national, community or international interest.

From the point of view of the location of the turbines, a number of 3 natural areas of community interest have been identified:

- 1 area located on the territory of Romania ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake at a distance of about 2.79 km from the project limit;

- 2 areas located on the territory of Bulgaria - BG0000569 Kardam (Bulgaria) located at a distance of about 410 m from the project limit and BG0000570 Izvorovo – Kraishte located at a distance of about 3.64 km from the project limit).

Their location in relation to the project are shown in the figures below.

In the project implementation area at distances between 5 - 20 km there are a number of 5 protected natural areas:

- 5 protected natural bird protection areas:

  - ROSPA0166 Plopeni-Chirnogeni at a distance of 6.21 km from the project limit

  - ROSPA0036 Dumbrăveni located at a distance of 7.07 km from the project limit

  - ROSPA0094 	Hagieni Forest at a distance of 18.39 km from the project limit

  - ROSPA0151 	Ciobănița-Osmancea at a distance of 18.55 km from the project limit

  - ROSPA0001 	Aliman – Adamclisi. at a distance of 19.12 km from the project boundary

- 2 protected natural areas of community interest:

  - ROSAC0157 Hagieni Forest - Bend of the Valley at a distance of 13.81 km from the project limit

  - BG0000130 Kraymorska Dobrudzha (Bulgaria) at a distance of 11 km from the project boundary

Further details on the Natura 2000 sites in the vicinity of the project are presented in Chapter 13 of this presentation memorandum.

From the point of view of protected natural areas of national interest, the closest to the analyzed project are the nature reserves RONPA0378 the Dumbraveni Forest located at a distance of about 7.51 km and RONPA0367 the Petroșani Limestone Walls, located at about 12.95 km, RONPA0370 the Credința fossil site located at about 12.83 km and RONPA0377 the Hagieni Forest located at about 23.64 km.

The nearest protected natural area of international interest is the RAMSAR site – RORMS0017 Ostroavele Danube - Bucgeac - Iortmac, located 3.67 km from the project.

Figure no. 10 Location of the project in relation to the nearest Natura 2000 sites

Connectivity is crucial for species that require large territories and move long distances. Ecological corridors of migration and dispersal depend on the existence of favorable habitats, where individuals can feed and shelter during transit. In a broad sense, green corridors are formed thanks to coherent green infrastructure. If ecological corridors are the functional element of the dispersion of individuals or populations, green infrastructure is the structural element. The analysis of ecological corridors carried out within the project by Fedorca A. and Ionescu G. underlines the importance of these aspects.

The project does not have the potential to lead to disruption of connectivity and fragmentation of ecological corridors. In the area of the site, no watercourses were identified whose connectivity could be affected by the implementation of the analyzed project.

The lands on which the investment is to be made are included in the built-up area and have the category of agricultural-arable use, pasture, communal road, exploitation roads, not being sensitive areas from the point of view of biodiversity.

Works, facilities and measures for the protection of biodiversity, nature monuments and protected areas

In order to protect the biodiversity components, the  following measures are provided for in the execution phase:

- Clear delimitation of the work front in order to minimize the disturbance of additional surfaces to those necessary to carry out the activities provided for in the project;

- For planting shrubs and herbaceous vegetation, only native plant species will be used, non-invasive, which will have an ecological role not only aesthetic;

- The control of the spread of invasive or potentially invasive plant species will be ensured, if their presence is found in the areas where the works are carried out. For the control of invasive alien plant species, it is recommended to use mechanical methods, pruning to be carried out before the flowering period. For invasive alien and potentially invasive alien plant species that are already in bloom, it is recommended that the clipper contains a bag, so as to reduce seed dispersal;

- The lighting devices will have the lamps with the lights positioned downwards (perpendicular to the ground) and will be equipped with motion sensors to minimize the risk of attracting certain species of invertebrates.

During the operating period, no chemicals that can penetrate the soil, groundwater and surface water will be used for the management of vegetation on the site.

In the decommissioning phase of the project, the measures will be similar to the execution phase.

    - Protection of human settlements and other objectives of public interest

Identification of objectives of public interest, distance from human settlements, respectively from historical and architectural monuments, other areas on which a restriction regime is instituted, areas of traditional interest and others

Objectives of public interest

The implementation of the project will not lead to the affectation of objectives of public interest. The road access to the project elements will be made through the existing communal and agricultural roads. During the period of execution and operation of the project, these public utility roads will not be affected.

Human settlements

The park is developed in the Cerchezu TAU site, in its outskirts, the nearest localities being: T32 at 1.1 km from Viroaga, T5 at 1 km from Cerchezu, T35 at 1 km from Magura, T3 at 1.56 km from the Bulgarian locality of Iovkov, T47 at 2.1 km from the Bulgarian locality Kraiste, T9 at 3.98 km from Negru Vodă.

Historical monuments and archaeological sites

The project does not intersect any historical monument, the closest being at a distance of approx. 636 m from T27, this site being the Tumulus from Căscioarele Movila Bacu.

The underground power line for connection to the SEN, which is not the subject of this presentation memorandum and does not have the definitive route, is located in the vicinity of several historical monuments. The detailed analysis of the location of the project in relation to the historical monuments in the area was previously presented in chapter 5.2 of this presentation memorandum.

Works, facilities and measures for the protection of human settlements and protected and/or public interest objectives

In the execution phase of the works, the following measures are proposed:

- Informing the citizens of the area about the program of works;

- Daily cleaning of access roads in the vicinity of work areas and maintenance of these roads;

- Protection and signaling of work areas, with clear markings regarding the safety limit in the perimeter of the works;

- Use of new vehicles, equipment and machinery, technically compliant with the best existing technologies;

- In the event that objects of archaeological importance are identified in the work fronts, during the development of the works, the works will be stopped, and the competent authorities will be contacted for expertise and the establishment of the necessary solutions for archaeological unloading.

During the operating period , measures can be proposed to reduce the intermittent shading effect, in case turbines are identified that can generate values above the materiality threshold. These measures consist of implementing a turbine operating schedule, which will reduce the exposure time to the shading effect.

In the decommissioning phase of the project, the measures will be similar to the execution phase.

    - Prevention and management of waste generated on site during project implementation/during operation, including disposal

List of waste (classified and codified in accordance with the provisions of European and national waste legislation), quantities of waste generated

The estimated waste to be generated in the execution phase of the works and in the operation phase, as well as their management are presented in the following table.

Table 3 Estimated waste to be generated in the execution and operation stages

| Waste name | Estimated quantity (tonnes) | Physical condition* | Waste code** | Management mode |
| --- | --- | --- | --- | --- |
| Execution stage |  |  |  |  |
| Mixed municipal deşeuri | 41,48 | S | 20 03 01 | Specially arranged spaces will be built with bin-type containers. Periodically they will be picked up by authorized operators and transported to landfills. |
| Plastics | 0,12 | S | 17 02 03 | They will be collected separately in temporary storage spaces specially arranged within the site organization and in the work fronts. Periodically they will be picked up by authorized operators and transported for recovery. |
| Paper and cardboard packaging | 0,85 | S | 15 01 01 |  |
| Plastic packaging | 0,49 | S | 15 01 02 |  |
| Construction wood waste | 0,89 | S | 17 02 01 |  |
| Metal mixtures | 259,25 | S | 17 04 07 |  |
| Concrete | 32,93 | S | 17 01 01 |  |
| Resturi de balast | 20,73 | S | 17 05 08 |  |
| Fire electrice | 0,03 | S | 17 04 11 | Specially designed spaces |
| Excavated material | 48,54 | S | 17 05 04 | Stored in the area of the work fronts and later reused as filler. |
| Packaging containing residues or contaminated with hazardous substances | 0,76 | S | 15 01 10* | They will be collected and stored selectively, in order to be transported to the disposal facilities by authorized operators. Exceptions are packaging that is returned to the manufacturer (e.g. IBCs). |
| Waste oils (Other engine, transmission and lubrication oils) | 1,31 | L | 13 02 08* | They will be collected in closed, labeled containers, stored in a closed enclosure provided with a concrete platform. They will be handed over to authorized units for collection and recovery. |
| Filter of ulei | 0,15 | S | 16 01 07* | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Used tyres | 0,76 | S | 16 01 03 | They will be collected on concrete platforms from the site organizations and handed over to authorized units for collection and recovery. |
| Batteries & Accumulators | 0,49 | S | 16 06 05 | They will be collected in metal containers and stored in specially arranged spaces and will be handed over to authorized operators for recovery. |
| Operation Stage |  |  |  |  |
| Non-chlorinated hydraulic mineral oils (hydraulic braking system oil) | 0,34 | L | 13 01 10* | They will be collected in metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Non-chlorinated mineral oils for engine, transmission and lubrication (emergency lubrication system) | n.a. | L | 13 02 05* |  |
| Synthetic engine, transmission and lubrication oils (transmission system, return system) | 2,3 | L | 13 02 06* |  |
| Non-chlorinated thermal insulating mineral oils and heat transfer | 23 | L | 13 03 07* |  |
| Packaging waste containing residues or contaminated with hazardous substances (metal or plastic packaging made of chemical substances and preparations used in maintenance) | 0,19 | S | 15 01 10* | They will be collected and stored selectively, in order to be transported to the disposal facilities by authorized operators. Exceptions are packaging that is returned to the manufacturer (e.g. IBCs). |
| Plastic Packaging Waste | 0,17 | S | 15 01 02 | They will be collected separately in temporary storage spaces specially arranged within the site organization and in the work fronts. Periodically they will be picked up by authorized operators and transported for recovery. |
| Filter material (filtre de aer) | 0,17 | S | 15 02 03 | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Filter media (oil filters), polishing materials | n.a. | S | 15 02 02* | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Electrical and electronic waste | n.a. | S | 16 02 14 | They will be collected in metal containers and stored in specially arranged spaces and will be handed over to authorized operators for recovery. |
| Decommissioning stage |  |  |  |  |
| Iron and steel | 2500 | S | 17 04 05 | They will be collected separately in temporary storage spaces specially arranged within the site organization and in the work fronts. Periodically they will be picked up by authorized operators and transported for recovery. |
| Aluminium | 75 | S | 17 04 02 |  |
| Copper, bronze, brass | 65 | S | 17 04 01 |  |
| Concrete | 10700 | S | 17 01 01 |  |
| Non-hazardous mixtures | 650 | S | 17 01 07 |  |
| Plastics (composites) | 1100 | S | 17 02 03 | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Non-hazardous electrical equipment | 57 | S | 16 02 13 | They will be collected in metal containers and stored in specially arranged spaces and will be handed over to authorized operators for recovery. |
| Hazardous electrical equipment | 7 | S | 16 02 14* | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Non-chlorinated hydraulic mineral oils (hydraulic braking system oil) | 10,5 | L | 13 01 10* | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Non-chlorinated mineral oils for engine, transmission and lubrication (emergency lubrication system) | 3,2 | It | 13 02 05* | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Batteries/accumulators (Pb, Ni) | 0,9 | S | 16 06 01* | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |
| Paper/cardboard packaging | 2,3 | S | 15 01 01 | They will be collected separately in temporary storage spaces specially arranged within the site organization and in the work fronts. Periodically they will be picked up by authorized operators and transported for recovery. |
| Packaging waste containing residues or contaminated with hazardous substances (metal or plastic packaging made of chemical substances and preparations used in maintenance) | 1,7 | S | 15 01 10* | They will be collected in airtight bags/metal containers and stored in specially arranged spaces and will be handed over to authorized operators for disposal. |

*Physical Condition: Solid-S, Liquid-L

** In accordance with the List of wastes, provided for in the European Commission Decision 2014/955/EU and in Annex no. 2 of GD no. 856/2002 on the record of waste management and for the approval of the list of wastes, including hazardous wastes, with subsequent amendments and completions.

Program for the prevention and reduction of the quantities of waste generated

At all stages of the project, contracts will be concluded with authorized companies that will ensure the elimination/recovery of all types of waste generated. All the waste generated as a result of the project, in all its stages, will be temporarily deposited only on specially arranged surfaces, on concrete platforms, in appropriate conditions, so as not to influence the development of activities in the area of the work fronts.

The temporary storage of waste is carried out in accordance with the specific legislation in force, as follows:

- on covered/uncovered concrete platforms;

- specially arranged spaces;

- in transportable containers, metal barrels;

- in closed and covered spaces.

At all stages of the project, the waste management record will be maintained according to GEO no. 92/2021 on the waste regime, with subsequent amendments and completions, GD no. 856/2002 and respectively Law no. 249/2015 on the management of packaging and packaging waste, with subsequent amendments and completions. All persons involved in all stages of the project will be trained on the handling of waste as well as how to sort it by categories, in the containers specially provided for each category of waste. In the case of hazardous waste, special measures will be taken to manage it (by separate storage only on impermeable surfaces), so as not to contaminate the rest of the waste or the soil. Within the site organizations, the contractor will set up platforms specially designed for the collection and management of all types of waste that will result from the execution of the works, provided with bins, containers and containers specially designed for the temporary storage of waste. The platforms will be arranged in such a way as to allow the handling of waste by the contracted authorized companies, in safe conditions. The temporary storage of waste will be done separately, on each type of waste, each container or container intended for storage being labeled with the corresponding waste code, according to GD 856/2002 with subsequent amendments and completions.

The company will have a waste management plan generated on the site, which will specify the name of the waste produced, the waste code, the quantity produced, the quantity recovered, the destination of the waste, as well as the stock existing at the end of the year. Pollution, due to waste generation, is considered insignificant. During the execution period as well as during the operation, the beneficiary will take all the necessary measures, so that the disposal, recovery of waste is carried out in a controlled manner, without generating pollution to the environment, so that a direct and significant impact on environmental factors is not expected, but only an indirect impact by the disposal of this waste by specialized companies. In the case of WEEE (waste electrical and electronic equipment) - Electrical and electronic tools, the beneficiary has the obligation to recycle them according to GD 448/2005 on waste electrical and electronic equipment. The collection of WEEE will be done separately, and their temporary storage will be carried out in a specially arranged, waterproof, properly marked space. The collection of hazardous waste (waste transmission and hydraulic oil) will be carried out with a specialized company / authorized according to the contract for the change / disposal of the waste oils generated from the site.

Waste Management Plan

The waste management plan involves the way of collecting/storing/disposing of both solid and hazardous and non-hazardous liquid waste.

Solid waste management

- Scrap metal. They will be collected separately in temporary storage spaces specially arranged within the site organization and in the work fronts. Periodically they will be picked up by authorized operators and transported for recovery.

- Packaging waste. Raw material packaging is paper and cardboard packaging that is collected and handed over to authorized collection units.

Regarding the management of packaging, the provisions of Law no. 249 of 28 October 2015 (*updated*) on the management of packaging and packaging waste:

- The waste from construction materials will be used for landscaping, it will be reused at other locations.

- Household waste is pre-collected in containers (bins) placed in site organizations. The disposal and storage of household waste is done by an authorized company;

- Hazardous chemical  packaging waste will be collected in airtight bags/metal containers and stored in specially designed spaces and will be handed over to authorized operators for disposal.

- Oil filters will be collected in airtight bags/metal containers and stored in specially designed spaces and will be handed over to authorized operators for disposal.

Liquid waste management

- The used oils will be collected in metal barrels and recovered by authorized companies.

    - Management of hazardous chemicals

Hazardous chemical substances and preparations used and/or produced

The necessary fuels and oils used during the project execution period will be stored in the area allocated to the site organization, according to the legislation in force.

How to manage hazardous chemical substances and preparations and ensure the conditions for the protection of environmental factors and the health of the population

Maintenance activities will be carried out in accordance with the manufacturer's technical instructions for wind turbines. It should be noted that materials, consumables and chemicals have different lifespans, ranging from one year to the entire lifespan of wind turbines. In some cases, it is sufficient to replenish an oil level that has been indicated, rather than to replace all the oil in a turbine component. Less than 1 liter to many hundreds of liters of oil are included in each of the few parts of a wind turbine. It is impossible to predict the annual amount of each category that will be used for maintenance work at this time, especially for this type of wind turbine.

In the case of the transformer station, the insulating oils in the transformers and the diesel needed to power the electric generator are classified as hazardous chemicals. Periodic oil testing is recommended for transformers to function properly (with a frequency of about 2 years for determining the dielectric strength and about 8 years for full chemical tests). The requirement to fill transformer oil is determined based on test results.

In the area of the wind farm, materials containing polychlorinated biphenyls (PCBs) will not be used.

During periods when maintenance work is carried out, consumables, hazardous compounds and preparations will not be kept on site, but will be brought in as needed.

The management of hazardous substances will be done in compliance with the legislation in force and the indications on the packaging of these products, as well as in the safety data sheets accompanying the products.

  - Use of natural resources, in particular soil, land, water and biodiversity

The main natural resources used for the implementation of the project are water and natural aggregates (ballast, broken stone and sand).

The land on which the project will be implemented is located outside the built-up areas of the localities, and the category of use is agricultural-arable, pasture, communal road and exploitation roads, being free of constructions.

# Description of environmental aspects likely to be significantly affected by the project

  - Characteristics of the potential impact

    - Impact on population and human health

The potential impact on the human population can take the following forms:

- From the point of view of population size, an insignificant positive impact can be generated, given the possibility of increasing the number of jobs in the area both during the execution period and partially during the operation period, thus leading to the attraction of the population in this area, including the young population.

Secondly, from an economic point of view, the same form of impact can be generated as a result of the development of the area but also of the subsequent attraction of other investments in the area.

- From the point of view of human health, the following aspects were taken into account: noise level, concentration of pollutants and shadow flicker.

In the execution phase, an insignificant negative impact may be generated, even if exceedances above the permissible limit of the noise level and the concentration of atmospheric pollutants may be generated, these will be for a short period of time. Measures to reduce the impact will also be applied, but these can only be established following noise and air modeling, taking into account all the characteristics of the project.

In the operating phase in terms of noise level, the maximum noise level will most likely be recorded in the turbine area, without being able to propagate values above the permissible limit to the sensitive receivers, both during the day and at night.

Wind turbines do not generate atmospheric emissions, they can be generated through maintenance activities, but they are not able to produce a significant negative impact on human health.

Through the operation of the turbines, the shadow flicker effect occurs, when the sun's rays reach the back of the turbine tower, thus generating a shadow that can overlap the properties located in its vicinity. This phenomenon is characteristic of clear sky days, when the turbine is located between the sun and the receiver. Another phenomenon is that of intermittent shading (flickering of shadows) and occurs when the sun is on the horizon and its light penetrates between the blades in the rotational movement. This phenomenon could occur on the population in the immediate vicinity of the wind farm.

The project will produce a strongly positive socio-economic impact and will also have positive influences on the environment. These voting benefits offset the unavoidable impacts associated during the construction and operation period.

It is considered that the activities that will be carried out within this investment objective and the change of the functional destination of the area, do not create the premises of negatively affecting the comfort and health of the population.

    - Impact on flora and fauna outside Natura 2000 sites

    - Impact on soil and subsoil

The activities carried out within the project that could affect the soil and subsoil are closely related to the work areas and the footprint of the constructions related to the foundations of the wind turbines, the arrangement of the access roads, the construction of the power station and the laying of the underground electrical network (LES).

It should be noted that the land areas definitively occupied by the construction elements of the project are reduced, however, in relation to the total area of the park, and as for the land areas temporarily occupied during the construction period, they will be returned to their original state once the works are completed.

The uncovered topsoil, as well as the excavated earth, will be stored separately, in order to be used for the ecological restoration of the areas affected by the temporary works. Thus, the impact on the soil will be felt directly as a result of the uncovering / excavation works, or the settlement / compaction in the work areas, but it is reversible once the works cease.

In accidental situations, fuel or lubricant leaks may occur, with a direct negative impact on the quality of the soil, strictly localized, on the work alignments, but involving small amounts of pollutants. The probability of such events is reduced, due to the application of strict procedures for refueling vehicles and the obligation to comply with the maintenance schedules of all equipment and machinery by the works contractor.

The impact on the soil, both during the construction period and during the operating period, under normal operating conditions of the machinery, is insignificant.

    - Impact on material uses and goods

The implementation of the project does not imply the impairment of material assets or the irreparable loss of material assets on which local communities depend (communication networks, water and sewerage networks, gas pipelines, electricity networks, real estate or movable property).  However, an accidental damage to the material assets of the local distributors in the project area during the construction phase (for example: during the excavation works, an accidental damage to: water pipes/ communication cables / sewerage network, etc.), can have a potential impact on the population.

    - Impact on water quality and quantitative regime

The works proposed in the project are not likely to have an impact on the quality and quantitative regime of the water. A potential impact on water quality can occur as a result of massive accidental pollution on the ground, which migrates into the groundwater. Compliance with the safety measures provided for in the project and the implementation of an appropriate maintenance program for vehicles and equipment, will minimize the risk of accidental leaks of fuel, chemicals and wastewater during the construction and operation of the project. It is estimated that in the event of an accidental pollution situation, the potential impact will be negative, direct and short-term.

    - Impact on air quality and climate. Climate

In the execution phase, the impact generated on the air environment aspect will most likely be negative, insignificant, considering the short duration of the works, but also the complexity of the works.

The operation of the turbines does not generate atmospheric emissions, but they could be generated as a result of maintenance activities, but they cannot lead to a significant negative impact.

The impacts associated with climate change are analysed from the point of view of:

- climate change mitigation

- adaptation to climate change

Climate change mitigation

Climate change mitigation takes into account the impact that the project will have on climate change, mainly through greenhouse gas emissions.

The quantities of pollutants generated by mobile machinery used in construction depend on the technological level and power of the engine, fuel consumption per unit of power, capacity, age of the machine and the equipment of pollution reduction devices.

Adaptation to climate change (climate resilience)

According to Annex IV of Law 292/2018, within the framework of the assessment of the project's resilience to climate change, it is necessary to assess the vulnerability and climate risks on the project, by identifying, assessing and implementing climate change adaptation measures.

The Cerchezu Wind Farm project has a life cycle of 30-35 years and can be exposed, over time, to a changing climate, with extreme weather phenomena and increasingly unfavorable and frequent climatic effects.

As for the components of wind turbines, they are designed in such a way as to withstand extreme weather phenomena.

Analyses of the effects of climate change predict annual increases in air temperature for the future, with higher rates during the summer season. As a result, a decrease in the annual amount of precipitation and a higher frequency of storms are expected.

The climatic variables that can have an effect on the project are: temperature increase, extreme storms and winds, extreme precipitation, cold and frost.

Climate change adaptation takes into account the project's vulnerability to future climate change and its ability to adapt to the impacts of climate change, which may be uncertain.

    - Impact on noise and vibration

The nature of the sources of noise and vibration within the implementation of the project is specific to the activities carried out.  During the execution period, the technological processes related to the projected works involve the use of specific machinery, which are sources of noise.

The noise produced by the turbines in the rotor's area of action is estimated at 96 - 101 dB, but the noise level decreases as the distance from the noise source increases. Taking into account the distance from the inhabited area of the localities, it is estimated that the execution works of the project will generate a noise level within permissible limits, insignificant, felt locally and temporarily, in the area where the project is located, and during  the period of operation of the wind farm The noise level will be monitored in order to comply with the limits provided by STAS 10009, Acoustics.

    - Impact on the landscape and visual environment

The impact on the landscape and the visual environment will be felt through the presence of construction equipment, as well as the carrying out of construction works.  The main visual impact during the operation period is related to the modification of the landscape by introducing new elements into the landscape, wind turbines and access roads, which can be perceived by sensitive receivers as a major change. This impact is expected to be direct, locally and long-term.

  - Extent of impact (geographical area, number of population/habitats/species affected)

The impact on the environmental factors likely to be affected will be felt locally, without the risk of its extension outside the project area.

  - Magnitude and complexity of the impact

By implementing the wind turbine construction project, there is the possibility of generating significant forms of potential impact on environmental aspects.

As mentioned in the previous section, a significant negative impact could be generated on the population and human health, at the operating stage, as a result of the shadow flicker effect.

  - Likelihood of impact

All the forms of impact mentioned above have a high probability of occurrence.

  - Duration, frequency and reversibility of impact

The forms of impact identified so far start with the start of the works. The duration of their manifestation is proportional to the duration of the works.

All forms of impact can be reversible (at different time scales).

The identified significant negative impact starts in the operation phase and manifests itself depending on the weather conditions.

  - Measures to avoid, reduce or improve significant environmental impact

Where potential significant negative impacts on the environmental components as a result of the implementation of the project were estimated, different sets of measures to avoid and reduce the impact were analyzed, the main ones being presented in Chapter VI for each environmental component.

  - Cross-border nature of the impact

Given the nature of the project, its location and its characteristics, we believe that there may be a potential to generate direct or indirect impacts of a cross-border nature. The distance between the project and Romania's border with the Bulgarian Republic is less than 100 m.

Thus, each environmental component likely to be affected by the construction and operation of the project was analyzed, the information being described below.

Water

The nearest bodies of surface water identified in the project area on the territory of Bulgaria are located south of the site boundary: BG1DJ900R1015 at a distance of about 1 km, BG2DO700L017 at a distance of about 34 km, respectively BG2BS000C001 at a distance of about 36 km, south-east of the site boundary.

The project area is located in the vicinity of two bodies of groundwater in Bulgaria (BG1G000000N049 and BG1G0000J3K051).

The construction and operation of the wind farm outside the built-up area of Cerchezu commune will not have a significant negative impact on the quality of surface or groundwater bodies in Bulgaria.

Air

According to the information published in the area of the outdated territory of Bulgaria with the project site (North region), the following aspects were highlighted:

- Sulphur dioxide (SO2): According to reports for the fourth quarter of 2022, SO2 standards were not exceeded.

- Nitrogen dioxide (NO2): NO2 levels were within the permissible limits and did not show significant exceedances.

- Fine particulate matter (PM10 and PM2.5): Problems with PM10 pollution continued, with occasional exceedances of norms, especially during the cold season.

As can be seen above, in the area of implementation of the plan, there were no significant exceedances of the limit values set by the legislation in force on the territory of Bulgaria.

Taking into account the activities currently carried out, the air quality can be considered to be generally good, being an open rural area uncluttered by urban constructions and without intense industrial activities.

The construction of the wind farm outside the built-up area of Cerchezu commune will not have a significant negative impact on air quality in Bulgaria. At the construction stage, dust emissions from excavation, vehicular and installation of construction materials, as well as emissions of pollutants specific to the combustion of fossil fuels associated with machinery and means of transport, are the main sources of emissions. Erosion caused by excavation work can also alter local air quality

In operation, wind turbines contribute to reducing greenhouse gas (GHG) emissions by replacing an amount of electricity produced from fossil fuels, thus contributing to climate change mitigation and having a significant positive impact on air quality.

Floor/Basement

The northern area of Bulgaria, on the border with Romania, is relatively anthropized and has predominantly fertile soils. The distribution of land use categories in the coverage area, on the Bulgarian territory, in the vicinity of the proposed project, presents an area of arable land (majority), with small intercalations of deciduous forests, predominantly agricultural land mixed with natural vegetation, discontinuous urban space and industrial or commercial units.

As regards the impact on soil and subsoil, there is no significant impact on soil and subsoil in the neighbouring territory of Bulgaria. In the execution stage, the impact on the ground is temporary and reversible, limited to the perimeter of the construction works. Changes in soil structure and water infiltration regime are managed through ecological restoration measures. Possible sources of pollution from the activities of vehicles and machinery are monitored and controlled to prevent contamination. In addition, wind turbines have a reduced footprint on the ground compared to other forms of energy, which limits soil erosion and compaction. During operation, the wind farm does not generate pollutants that affect the quality of the soil or subsoil, thus ensuring long-term environmental protection.

Use of natural resources

The implementation of the wind farm plan in Cerchezu commune, Romania, offers the opportunity to capitalize on renewable energy resources. For the realization of the park, the following resources will be used: topsoil from uncovering, natural earth from excavation, filling with broken stone, and insignificant amounts of water for various activities on site.

The construction of the wind farm will not use natural resources on the territory of Bulgaria nor will it affect its natural resources. All the resources necessary for the plan will be extracted and managed on the territory of Romania. Thus, Bulgarian soils and subsoils will remain unaffected by the activities related to the construction and operation of the wind farm outside the built-up area of Cerchezu commune.

Waste management

As part of the implementation of the wind farm in Cerchezu commune, Romania, adequate waste management will be ensured, through its proper collection, storage and disposal. The waste generated will include construction materials, packaging and household waste, all managed according to Romanian legislation. No waste will be transported or managed on the territory of Bulgaria, thus ensuring that  Bulgaria's resources and environment will not be affected by activities related to the construction and operation of the wind farm.

Noise and vibration

In the execution phase, the means of transport and machinery used, as well as the activities on the work front, will generate noise and vibrations. However, given that the area is crossed by local roads and agricultural exploitation, the impact will be local, temporary and insignificant, with no impact on the territory of the neighboring state of Bulgaria.

In the operating stage, the noise produced by wind turbines will increase with wind speed and blade rotational speed. The modern design of the turbine includes the isolation of the nacelle to prevent the transmission of mechanical noise and vibrations. The noise level varies between 92 and 107.7 dB, and the noise modeling shows that in the reception areas the level will be below 40 dB.

Thus, the construction and operation of the wind farm will not generate significant noise and vibrations that would affect the territory of Bulgaria. Mitigation measures, such as the strategic location of turbines and the use of modern technologies, will reduce noise and vibration emissions. Thus, communities and the environment in Bulgaria will not be affected.

Biodiversitate

In the vicinity of the project, at a distance of less than 5 km, a number of 2 natural areas of community interest in Bulgaria  were identified: BG0000569 Kardam, located at a distance of about 410 m from the project boundary, and BG0000570 Izvorovo – Kraishte, located at a distance of about 3.64 km from the project boundary.

A significant negative impact on some species within the protected natural areas is expected: ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSCI0157/ROSAC157 Hagieni Forest - Bend of the Valley, ROSPA0036 Dumbrăveni, ROSPA0166 Plopeni-Chirnogeni, ROSPA0094 Hagieni Forest - Cotul Văii, BG0000569 Kardam, BG0000570 Izvorovo - Kraishte. The groups that will be significantly affected by the implementation and operation of the wind energy plan will be terrestrial mammals, bats and birds.

Population and human health

The park is developed in the Cerchezu TAU site, in its outskirts, the nearest localities on the Bulgarian territory being the village of Iovkov at 1.56 km from the T3 turbine and the Kraiste locality at 2.1 km from the T47 turbine.

Iovkovo and Kraiste are two villages in Gheneral-Toşevo Municipality, Dobrich Region, South Dobrudja, Bulgaria38F

The construction and operation of the wind farm in the outskirts of Cerchezu commune will not affect the population on the territory of Bulgaria. As presented in the previous chapters, in the execution phase, noise, vibrations and dust emissions will be local and temporary, with a low impact. In the operating stage, the noise produced by the turbines will be below 40 dB in the reception areas, respecting the legal limits. The shading effect is insignificant, and the visual impact is subjective. In the decommissioning stage, the impact will be similar to that of the construction stage, being local and temporary.

Landscape

According to the map with the types of landscape characteristic of the area where the project is proposed, it can be seen that the types of landscape in the area of the plan are similar for both countries, as follows:

- Continental landscape - this area represents hills and arable land with sedimentary soils. It is predominant in the largest part of the project's coverage area.

- Steppe landscape - this area represents a steppe landscape with hills and sedimentary soils.

On the territory of Bulgaria, in the vicinity of the project perimeter, very little fragmentation prevails. The relief is moderately fragmented, suggesting the presence of low hills and valleys. This type of landscape can support diversified agricultural activities and exhibits moderate biodiversity.

The construction and operation of the wind farm in the outskirts of Cerchezu commune will not affect the landscape on the territory of Bulgaria.

Climate

According to the National Report on the State and Protection of the Environment in Bulgaria41F, major GHG emissions - for the period 1988-2021 tend to decrease. In 2021, total GHG emissions were generated – 53917.27 Gg CO2-eq. This means that GHG emissions have fallen by 52.55% compared to emissions in the 1988 reference year.

The National Report on the State and Protection of the Environment in Bulgaria42F presents the following aspects regarding climate factors:

- average annual temperature - between 1988 and 2021, the average annual air temperature for the lower part of the country fluctuates between 10.6°C and 13.3°C

In 2021, the average annual temperature for the lower part of the country was 12.3 ° C, which is 0.3 ° C above the norm. This is the thirteenth warmest year since 1988-2021, and January is the fifth warmest since 1930 – an average of 2.5 ° C above the monthly norm (from +1.2 ° C in the village of Kostenets to + 3.7 ° C in Ruse).

- The average annual amount of precipitation - in the period 1988-2021 for regions with altitude up to 800 m varies in the range from 377 mm to 1013 mm, while maintaining the positive trend of this indicator (+4.2 mm / year). In 2021, the average annual rainfall was 741 mm, which is about 20% above the norm for the period 1991-2020.

- Snow - in the period 1988-2021 there was no significant decreasing trend of fluctuations in the average maximum height of the snow cover in areas with altitude between 800-1800 m. The value for 2021 is 43 cm – below the average for the period 1991-2020.

- climatic phenomena - during the period 8-12.01.2021, torrential and river floods were recorded, mainly in western and southeastern Bulgaria, due to heavy rainfall, combined in some places with melting snow. In Sofia, Blagoevgrad, Yambol and Burgas districts, the amount of precipitation for 72 hours exceeded three times the monthly norm. A state of disaster due to floods, destroyed bridges and broken roads was declared in 8 districts. Heavy snowfall in northwestern Bulgaria led to power outages in many places.

Cultural heritage

No data sources have been found that completely list all historical monuments in Bulgaria on each region, as they exist in Romania. However, the construction of the proposed park will not affect the territory of Bulgaria, thus ensuring the preservation of the country's cultural and historical heritage.

In accordance with Act 22/2001 on the ratification of the Convention on Access to Information, Public Participation in Decision-making and Access to Justice in Environmental Matters (Aarhus Convention), it is essential to ensure full transparency and the involvement of the local and cross-border community at all stages of the plan. This includes access to relevant information on environmental and health impacts and active public participation in decision-making. Compliance with this law ensures that all stakeholders are informed and have the opportunity to contribute to decision-making, thus ensuring effective and fair management of environmental impact.

  - Exposure of the area to climate change

In order to integrate the climate change dimension into the project's analysis and to identify appropriate adaptation options, a chapter dedicated to climate change has been developed. It summarises the main conclusions and complies with the methodological requirements and guidelines established at European level.

The chapter is based on the following reference documents:

- Communication from the European Commission "Technical guidance on immunising infrastructure to climate change 2021–2027" (2021/C 373/01);

- Guidebook „How to develop a Sustainable Energy and Climate Action Plan (SECAP)”, elaborat de Comisia Europeană;

- „Guidelines for Project Managers: Making vulnerable investments climate resilient”, elaborat de Directoratul General pentru Politici Climatice (DG CLIMA);

- „Climate change and major projects”, elaborat de Comisia Europeană;

- „The Basics of Climate Change Adaptation Vulnerability and Risk Assessment”, versiunea 1, iunie 2017, elaborat de JASPERS.

The following steps will be considered:

- Climate change mitigation

- Screening stage (attenuation)

- Complex analysis

- Framing the project in the fields of application;

- Climate change adaptation/resilience

- Screening stage (attenuation)

- Identification of the project's climate sensitivity;

- Exposure assessment in the project implementation area (without considering the project);

- Vulnerability analysis.

- Complex analysis

- Risk assessment;

- Identification of adaptation options;

- Assessment of adaptation options.

- Climate change mitigation

According to the requirements of the European Commission's Guide "Technical Guidelines on Climate Change Immunisation of Infrastructure 2021–2027" and the European Investment Bank (EIB) Methodology, the quantification and monetisation of greenhouse gas (GHG) emissions is the basis for the cost-benefit analysis and, implicitly, for the selection of the most advantageous alternative for the project.

Wind farm development projects are classified as those for which such an assessment is not required.

Therefore, the analyzed project does not require the quantification and monetization of greenhouse gas emissions.

According to the United States Department of Energy (2023), greenhouse gas emissions from renewable sources are considerably lower than emissions from natural gas and coal. Wind power produces about 11 grams of CO2 per kilowatt-hour (g CO2/kWh) of electricity generated, compared to about 980 g CO2/kWh for coal and about 465 g CO2/kWh for natural gas.

- Climate change adaptation/resilience

Climate sensitivity was identified for each of the 4 components of industrial projects: physical assets, inputs, outputs and transmission and distribution networks.

- Physical assets and processes – These are represented by the wind turbines themselves, underground power transmission lines, transformer stations and power generation;

- Inputs – are represented by wind energy

- Outputs or services – are represented by the production of electricity;

- Interdependencies: Transmission networks – comprise the road infrastructure that provides access to the project site, in the case of interventions for maintenance works and energy transmission networks of the SN.

Climate variables include both the primary effects of climate change and the side effects that are directly dependent on the primary ones. The components of the project are interdependent, so affecting one can have consequences on the others, generating feedback effects.

Table 4 Identifying the sensitivity of project types to climate variables

| No. | Climate variable | Entries | Assets and processes | Exits | Interdependencies | Final |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | 1 | 2 | 1 | 1 | 2 |
| 2 | Evolution of heat waves | 1 | 1 | 1 | 1 | 1 |
| 3 | Evolution of cold waves | 1 | 1 | 1 | 1 | 1 |
| 4 | Evolution of the thermal comfort indicator | 1 | 1 | 1 | 1 | 1 |
| 5 | Evolution of the drought phenomenon | 1 | 1 | 1 | 1 | 1 |
| 6 | Evolution of the amount of solar radiation | 1 | 1 | 1 | 1 | 1 |
| 7 | Evolution of the multiannual average amount of precipitation | 1 | 2 | 1 | 2 | 2 |
| 8 | Evolution of the precipitation deficit indicator | 1 | 1 | 1 | 1 | 1 |
| 9 | Evolution of the indicator on the abundance of precipitation | 1 | 1 | 1 | 1 | 1 |
| 10 | Evolution of the flow of the snow layer | 1 | 1 | 1 | 1 | 1 |
| 11 | Evolution of the percentage of relative humidity | 1 | 1 | 1 | 1 | 1 |
| 12 | Evolution of average wind speed | 2 | 2 | 2 | 2 | 2 |
| 13 | Annual average of tornadoes | 1 | 1 | 1 | 1 | 1 |
| 14 | Flood risk index | 2 | 3 | 2 | 3 | 3 |
| 15 | Wind Erosion Risk Index | 1 | 1 | 1 | 1 | 1 |
| 16 | Landslide/avalanche risk index | 2 | 3 | 2 | 3 | 3 |
| 17 | Wildfire Risk Index | 1 | 2 | 1 | 2 | 2 |

Legend:

| Climate sensitivity | No Sensitivity (0) | Low (1) | medium (2) | Elevated (3) |
| --- | --- | --- | --- | --- |

From the analysis carried out in the previous table, it results that a medium sensitivity was assessed for 4 climatic variables and a high sensitivity was assessed for 4 other climatic variables.

These are:

- Evolution of the average annual thermal regime,

- Evolution of the multiannual average amount of precipitation,

- Evolution of the average wind speed,

- Flood risk index,

- Landslide/avalanche risk index

- Wildfire Risk Index

Exposure assessment

Evolution of the average annual thermal regime

Extreme temperatures are the exceptionally high or low temperature values in a particular region or place at a specific time. These temperatures significantly exceed the normal or usual temperature limits for that area or period of time.

The phenomena of temperature increase intensified after 2000, the winter of 2006-2007 being considered the warmest since instrumental measurements have been made in Romania. In that year, pronounced deviations of the maximum/minimum temperature from the multiannual average regime persisted over long periods of time.

As for the trends of thermal extremes, at national level, the results recorded at the weather stations indicated a decrease in the number of frost days in some areas in the south of the country and an increase in heat waves in most areas of the country.

Figure no. 11 Historical average annual maximum temperature values at the level of the project's impact area

Regarding the evolution of the maximum annual average temperature in the area of influence of the proposed project, in 2050 an increase in temperature by about 130C is expected, with values between 19.01÷19.360C recorded in the area.

Figure no. 12 Maximum temperature values in the project's impact area in 2050

According to the global climate data for the period 1970-2000 available on WorldClim, the minimum average annual temperature in the area of influence of the project, at the level of Cerchezu commune, is between 0.68÷0.330C.

Figure no. 13 Historical average minimum temperature values at the level of the project's impact area

As for the evolution of the minimum annual average temperature in the area of influence of the proposed project, at the level of Cerchezu commune, in 2050 an increase in temperature by about 90C is expected, with values between 9.49÷9.840C recorded in the area.

Figure no. 14 Average annual minimum temperature values in the project's impact area in 2050

Evolution of the multiannual average amount of precipitation

Precipitation can take various forms, such as drops of liquid water, snow crystals or ice granules. Extreme rainfall can have significant effects on water supply and sewerage infrastructure by increasing the turbidity of water supply sources from surface sources.

According to the global climate data for the period 1970-2000 available, the annual amount of precipitation in the area of influence of the project, at the level of Cerchezu commune, varies between 460÷476 mm.

Figure no. 15 Historical rainfall values in the project's impact area

Regarding the evolution of the annual amount of precipitation in the area of influence of the proposed project, in 2050 a decrease in the amount of precipitation at the level of Cerchezu commune by about 10 mm is expected, with values between 450÷475 mm recorded in the area.

Flood risk index

Floods are events in which the soil is covered with excess water, usually due to heavy rainfall, rapid melting of snow, or overflowing watercourses.

Exposure to flood risk, already considerable in most of the country, is projected to increase amid climate change. According to the data available in the flood hazard and risk maps - cycle 2, the impact area of the proposed project is not subject to flood risks, the distance to the water bodies at risk of flooding being at least 15 km.

Figure no. 16 Map of areas with significant flood risk

Wildfire Risk Index

Wildfires are fires that spread through areas with vegetation, such as forests, grasslands, or bushes. These fires can have multiple causes, including negligent human activity, electrical discharges, or natural phenomena such as drought.

In recent years, amid global warming, there has been an increase in the duration and magnitude of favorable weather conditions for forest fires (Davis et al. 2018).

Between 1995 and 2018, the number of forest fires and the area affected by them showed significant annual variations, the highest values being found in 2000, 2007 and 2012. This result can be explained by the fact that these years were considered to be the hottest since the reference year 1951, when the maximum temperature record in our country was recorded (Busuioc et al 2007). Of the total fires that affected the forests in Romania between 1995 and 2018, most occurred in March, August and November.

Figure no. 17 Map of the probability of forest fires

Landslide/avalanche risk index

Landslides are caused by gravitational forces, but they can be triggered by a variety of processes. Some of the most common triggers include earthquakes and periods of prolonged and/or intense rainfall. Deforestation can also increase the likelihood of landslides

Figure no. 18 Landslide hazard zoning map

To identify the evolution of the average wind speed variable, climate data from the Climate Copernicus platform, CMIP6, in NetCDF format, were used, which were later processed in ArcGIS Pro 2.5. This is how the situation for the Baseline period (1970 – 2000) was obtained, as well as for the future, more precisely the year 2050. Thus, through a decrease in rasters, the evolution of this variable was obtained. As can be seen from the map inserted above, there is a positive evolution of the average wind speed in the project area

Figure no. 19 Map of the evolution of the average wind speed expected for the period 2030 – 2050

Exposure assessment

Based on the analysis of the available information on climate change in the study area, an increasing trend was identified: average temperature, extreme maximum temperatures (heat waves), solar radiation, average precipitation.

The following table summarizes the results of the exposure assessment in the study area, both to current and future climatic conditions.

Identifying the sensitivity of project types to climate variables

Table 5 Assessment of the exposure of the study area in relation to climate variables

| No. | Climate variable | Exposure | Explanation |
| --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | 2 | From the point of view of the evolution of average temperatures, in the study area there is a decrease between -0.24 - 0 °C. |
| 2 | Evolution of heat waves | 1 | The phenomenon of heat waves will register an increase between the intervals 5.1 – 10.4 days in which it will make its presence felt. |
| 3 | Evolution of cold waves | 1 | The cold waves will register a decrease in the number of days in which they will manifest, they are expected to be -1.99 and 1 days less. |
| 4 | Evolution of the thermal comfort indicator | 1 | As for the thermal comfort index for the period 2030 – 2050, a decrease of up to -25° is expected |
| 5 | Evolution of the drought phenomenon | 1 | The drought variable is expected to intensify by (0.1 – 1.5 days). |
| 6 | Evolution of the amount of solar radiation | 1 | Solar radiation is expected to increase between 0.1 and 1 W/m2. |
| 7 | Evolution of the multiannual average amount of precipitation | 2 | The multiannual average amount of precipitation shows multiple signals. At the level of the project area, increases are expected to be recorded between 99 and 15 |
| 8 | Evolution of the precipitation deficit indicator | 1 | In the project area, the number of days in which precipitation is expected to be free is expected to decrease by 0.1 – 11.3 days |
| 9 | Evolution of the indicator on the abundance of precipitation | 1 | As for the abundance of precipitation, an increase of between 1 and 10 consecutive days of positive extreme precipitation is expected at the level of the project area. |
| 10 | Evolution of the flow of the snow layer | 1 | The amount of snow expected for the winter season between 2030 and 2050 is -0.19 and -0.12 mm/day in winter |
| 11 | Evolution of the percentage of relative humidity | 1 | In the area of the project area, an increase in the percentage of relative humidity is expected, around 0.1 – 0.5 % |
| 12 | Evolution of average wind speed | 2 | The variable on the average wind speed is expected to register multiple responses. In the south-east and north-western extremities there is a decrease of -0.030 – 0 m/s. |
| 13 | Annual average of tornadoes | 1 | In terms of the number of tornadoes incidences in the project area, there are between 0.1 – 0.25 tornadoes per year |
| 14 | Flood risk index | 0 | The project does not overlap with a flood risk |
| 15 | Wind Erosion Risk Index | 1 | The project is located in areas without moderate risk of wind erosion |
| 16 | Landslide/avalanche risk index | 0 | Due to the location of the project in a relief stage with low altitudes and planes, there is no risk of this phenomenon occurring in the study area. |
| 17 | Wildfire Risk Index | 2 | The project area is exposed to low and moderate risk of wildfires. |

These scores on exposure to current and future environmental conditions, related to the climate context, that of climate change, were awarded in accordance with the analysis of multiple sources, both at local and regional level.

Vulnerability analysis

The results of the analysis of the project's vulnerability to climate change are presented below, considering the highest score awarded for exposure (current or future).

Table 6 Current vulnerability of the project to climate variables

| No. | Climate variable | Sensibility | Exposure | Vulnerability |
| --- | --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | 2 | 2 | 4 |
| 2 | Evolution of heat waves | 3 | 2 | 6 |
| 3 | Evolution of cold waves | 1 | 1 | 1 |
| 4 | Evolution of the thermal comfort indicator | 1 | 1 | 1 |
| 5 | Evolution of the drought phenomenon | 1 | 1 | 1 |
| 6 | Evolution of the amount of solar radiation | 1 | 1 | 1 |
| 7 | Evolution of the multiannual average amount of precipitation | 2 | 1 | 2 |
| 8 | Evolution of the precipitation deficit indicator | 1 | 1 | 1 |
| 9 | Evolution of the indicator on the abundance of precipitation | 2 | 2 | 4 |
| 10 | Evolution of the flow of the snow layer | 1 | 1 | 2 |
| 11 | Evolution of the percentage of relative humidity | 1 | 1 | 1 |
| 12 | Evolution of average wind speed | 1 | 2 | 2 |
| 13 | Annual average of tornadoes | 3 | 3 | 9 |
| 14 | Flood risk index | 3 | 0 | 0 |
| 15 | Wind Erosion Risk Index | 1 | 3 | 3 |
| 16 | Landslide/avalanche risk index | 3 | 0 | 0 |
| 17 | Wildfire Risk Index | 2 | 2 | 4 |

Legend:

| Sensibility | No Sensitivity (0) | Small (1) | medium (2) | Elevated (3) |
| --- | --- | --- | --- | --- |
| Exposure | No Exposure (0) | Small (1) | medium (2) | Elevated (3) |
| Vulnerability | No Vulnerability (0) | Small (1-2) | medium (3-4) | High (6-9) |

According to the vulnerability analysis, the climate variables that recorded the highest scores are:

- Evolution of the average annual thermal regime

- Evolution of heat waves

- Evolution of the indicator on the abundance of precipitation

- Annual average of tornadoes

- Wind Erosion Risk Index

- Wildfire Risk Index

Complex analysis

Risk assessment

The risk assessment was carried out for those climate variables with high and medium future vulnerability, relevant to the project in question.

The following table shows the identified climate variables, their trend, as well as the risk related to changes in these variables.

Table 7 Risk assessment of future climate variables in the project area

| No. | Climate variable | Climate variable trend | Risk |
| --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | Increase in average temperatures | Overloading of mechanical systems |
|  |  |  | Overloading of electrical power generation systems |
|  |  |  | Creating unfavorable conditions for staff |
|  |  |  | Damage to the installation and the need for more frequent maintenance work |
| 2 | Evolution of heat waves | Increase in duration and frequency | Exposure of wind turbine maintenance personnel to adverse environmental conditions |
|  |  |  | Overloading the electricity production and transmission system |
| 3 | Evolution of the indicator on the abundance of precipitation | Increasing the intensity and amount of precipitation but also the frequency | Heavy rainfall in a short period of time can cause surface water erosion, which can erode the soil and reduce the anchoring capacity of renewable energy production systems. |
|  |  |  | Restricting access to technology platforms during periods of extreme rainfall |
| 4 | Annual average of tornadoes | Increasing frequency and intensity | High wind speeds can break or damage wind turbine blades |
|  |  |  | System deactivation due to exceeding the maximum speeds at which wind turbines are programmed to interrupt their activity |
| 5 | Wind Erosion Risk Index | Increased risk of erosion Wind | Degradation of wind turbine structures |
| 6 | Wildfire Risk Index | Amplification of the appearance, intensity and duration of wildfires | Deterioration of renewable energy production infrastructure |
|  |  |  | Damage to the transport infrastructure for the energy generated by the wind farm |

The following table shows the probability of occurrence of the identified climate risks.

Table 8 Assessing the likelihood of climate risks

| No. | Climate variable | Risk | Estimate | Justification |
| --- | --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | Overloading of mechanical systems | Probable | The analysis of the recorded historical data highlighted trends of increasing average temperatures throughout Romania. Also, all climate models developed so far indicate a global increase in average temperatures |
|  |  | Overloading of electrical power generation systems |  |  |
|  |  | Creating unfavorable conditions for staff |  |  |
|  |  | Damage to the installation and the need for more frequent maintenance work |  |  |
| 2 | Evolution of heat waves | Exposure of wind turbine maintenance personnel to adverse environmental conditions | Probable | The analysis of the recorded historical data reported significant changes in the trends of extreme maximum temperatures throughout Romania. The annual frequency of tropical summer days has increased, while the frequency of winter days has decreased (ANM, 2021). The increase in extreme ambient temperatures will most likely lead to the electrical and mechanical overload of renewable energy production technologies |
|  |  | Overloading the electricity production and transmission system |  |  |
| 3 | Evolution of the indicator on the abundance of precipitation | Heavy rainfall in a short period of time can cause surface water erosion, which can erode the soil and reduce the anchoring capacity of renewable energy production systems. | Moderate | According to the report "Climate change – from physical bases to risks and adaptation", prepared by the ANM in 2021, the number of days with extreme rainfall episodes is higher in hill and mountain areas, compared to lowlands. The analyzed project is located in a lowland area, where the probability of extreme precipitation phenomena is much lower. |
|  |  | Restricting access to technology platforms during periods of extreme rainfall |  |  |
| 4 | Annual average of tornadoes | High wind speeds can break or damage wind turbine blades | Reduced | According to the study "Climate change - from the physical basis to risks and adaptation" - Roxana Bojariu et al., 2021, for the period 2021-2050, extreme tornado-type phenomena will intensify in Romania, with a maximum frequency and intensity in the southeast, manifesting themselves in restricted areas (locally) and being difficult to forecast |
|  |  | System deactivation due to exceeding the maximum speeds at which wind turbines are programmed to interrupt their activity |  |  |
| 5 | Wind Erosion Risk Index | Degradation of wind turbine structures | Sure | Due to the amplification of the thermal regime and the intensification of the drought phenomenon, recently proven to be direct consequences of climate change, as well as the change in wind patterns, wind soil erosion is expected to increase in the future |
| 6 | Wildfire Risk Index | Deterioration of renewable energy production infrastructure | Moderate | In the current climate context, based on measurements of various climatic parameters and scientific literature, climate change researchers are convinced that more wildfires will occur in the future, according to current trends |
|  |  | Damage to the transport infrastructure for the energy generated by the wind farm |  |  |

The following table presents the assessment of the previously identified risks, estimated on the basis of the probability of occurrence and the severity of the impact on the risk areas considered in the assessment.

Table 9 Probability and severity of risks associated with climate variables

| No. | Climate variable | Risk | Areas of risk | Probability | Analyze Impact | PxI |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | Overloading of mechanical systems Overloading of electrical power generation systems Creating unfavorable conditions for staff Damage to the installation and the need for more frequent maintenance work | Active damage, engineering, functional aspects | Probable | Moderate | Elevated |
|  |  |  | Safety & Health |  | Minor |  |
|  |  |  | Environment, cultural heritage |  | Minor |  |
|  |  |  | Social |  | Insignificant |  |
|  |  |  | Finance |  | Minor |  |
|  |  |  | Reputation |  | Insignificant |  |
|  |  |  | Any other areas de risc relevante |  | Insignificant |  |
| 2 | Evolution of heat waves | Exposure of wind turbine maintenance personnel to adverse environmental conditions Overloading the electricity production and transmission system | Active damage, engineering, functional aspects | Probable | Moderate | Elevated |
|  |  |  | Safety & Health |  | Moderate |  |
|  |  |  | Environment, cultural heritage |  | Moderate |  |
|  |  |  | Social |  | Moderate |  |
|  |  |  | Finance |  | Moderate |  |
|  |  |  | Reputation |  | Minor |  |
|  |  |  | Any other areas de risc relevante |  | Insignificant |  |
| 3 | Evolution of the indicator on the abundance of precipitation | Heavy rainfall in a short period of time can cause surface water erosion, which can erode the soil and reduce the anchoring capacity of renewable energy production systems. Restricting access to technology platforms during periods of extreme rainfall | Active damage, engineering, functional aspects | Moderate | Minor | Elevated |
|  |  |  | Safety & Health |  | Insignificant |  |
|  |  |  | Environment, cultural heritage |  | Minor |  |
|  |  |  | Social |  | Insignificant |  |
|  |  |  | Finance |  | Insignificant |  |
|  |  |  | Reputation |  | Insignificant |  |
|  |  |  | Any other areas de risc relevante |  | Insignificant |  |
| 4 | Annual average of tornadoes | High wind speeds can break or damage wind turbine blades System deactivation due to exceeding the maximum speeds at which wind turbines are programmed to interrupt their activity | Active damage, engineering, functional aspects | Reduced | Moderate | Elevated |
|  |  |  | Safety & Health |  | Moderate |  |
|  |  |  | Environment, cultural heritage |  | Minor |  |
|  |  |  | Social |  | Minor |  |
|  |  |  | Finance |  | Moderate |  |
|  |  |  | Reputation |  | Minor |  |
|  |  |  | Any other areas de risc relevante |  | Insignificant |  |
| 5 | Wind Erosion Risk Index | Degradation of wind turbine structures | Active damage, engineering, functional aspects | Sure | Moderate | Elevated |
|  |  |  | Safety & Health |  | Minor |  |
|  |  |  | Environment, cultural heritage |  | Minor |  |
|  |  |  | Social |  | Minor |  |
|  |  |  | Finance |  | Minor |  |
|  |  |  | Reputation |  | Insignificant |  |
|  |  |  | Any other areas de risc relevante |  | Insignificant |  |
| 6 | Wildfire Risk Index | Deterioration of renewable energy production infrastructure Damage to the transport infrastructure for the energy generated by the wind farm | Active damage, engineering, functional aspects | Moderate | Moderate | Elevated |
|  |  |  | Safety & Health |  | Minor |  |
|  |  |  | Environment, cultural heritage |  | Minor |  |
|  |  |  | Social |  | Minor |  |
|  |  |  | Finance |  | Minor |  |
|  |  |  | Reputation |  | Minor |  |
|  |  |  | Any other areas de risc relevante |  | Insignificant |  |

Identifying options for adaptation to climate change

For the project-specific climate change risks identified in the previous phase, a number of related adaptation measures have been identified, presented in the table.

Table 10 Identification of adaptation measures associated with climate variables

| No. | Climate variable | Risk | Adaptation measures |
| --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | - Overloading of mechanical systems - Overloading of electrical power generation systems - Creating unfavorable conditions for staff - Damage to the installation and the need for more frequent maintenance work | - Electrical units (inverters, UPS, transformers) equipped with active or passive cooling systems, designed for hot climates - Installation of permanent local weather systems to monitor temperature, humidity and heat stress accumulation - Use of light/reflective color for enclosures and technical containers (passive cooling effect) - Adaptation of maintenance work schedules to cooler hours (early morning/evening) |
| 2 | Evolution of heat waves | - Exposure of wind turbine maintenance personnel to adverse environmental conditions - Overloading the electricity production and transmission system | - Certified wind turbines for warm climates – selection of models tested to operate efficiently at high ambient temperatures (above 40°C) - Planning activities outside peak heating hours – maintenance work scheduled early in the morning or evening - Adaptive SCADA systems – which dynamically adjust the output power or rotation of the blades according to the anticipated thermal stress. - Creating green areas with vegetation adapted to drought – which contributes to reducing the thermal island effect. |
| 3 | Evolution of the indicator on the abundance of precipitation | - Heavy rainfall in a short period of time can cause surface water erosion, which can erode the soil and reduce the anchoring capacity of renewable energy production systems. - Restricting access to technology platforms during periods of extreme rainfall | - Monitoring the drainage level of the surfaces within the site |
| 4 | Annual average of tornadoes | - High wind speeds can break or damage wind turbine blades - System deactivation due to exceeding the maximum speeds at which wind turbines are programmed to interrupt their activity | - Using flexible blades that withstand high wind speeds - Building solid foundations adapted for extreme conditions - Obtaining insurance to cover the costs of damage caused by extreme weather events |
| 5 | Wind Erosion Risk Index | - Degradation of wind turbine structures | - Proper fastening of wind turbines - Grassing the project site |
| 6 | Wildfire Risk Index | - Deterioration of renewable energy production infrastructure - Damage to the transport infrastructure for the energy generated by the wind farm | - Carrying out vegetation maintenance works around the turbine site - Use of insulating materials for electricity transmission infrastructure |

Assessment of adaptation options

The assessment of adaptation options has been carried out in terms of costs for each of the proposed measures and is presented in the table below.

Table 11 Probability and severity of risks associated with climate variables

| No. | Climate variable | Risk | Risk score | Adaptation measures | Risc rezidual |
| --- | --- | --- | --- | --- | --- |
| 1 | Evolution of the average annual thermal regime | - Overloading of mechanical systems - Overloading of electrical power generation systems - Creating unfavorable conditions for staff - Damage to the installation and the need for more frequent maintenance work | Elevated | - Electrical units (inverters, UPS, transformers) equipped with active or passive cooling systems, designed for hot climates - Installation of permanent local weather systems to monitor temperature, humidity and heat stress accumulation - Use of light/reflective color for enclosures and technical containers (passive cooling effect) - Adaptation of maintenance work schedules to cooler hours (early morning/evening) | Reduced |
| 2 | Evolution of heat waves | - Exposure of wind turbine maintenance personnel to adverse environmental conditions - Overloading the electricity production and transmission system | Elevated | - Certified wind turbines for warm climates – selection of models tested to operate efficiently at high ambient temperatures (above 40°C) - Planning activities outside peak heating hours – maintenance work scheduled early in the morning or evening - Adaptive SCADA systems – which dynamically adjust the output power or rotation of the blades according to the anticipated thermal stress. - Creating green areas with vegetation adapted to drought – which contributes to reducing the thermal island effect. | Reduced |
| 3 | Evolution of the indicator on the abundance of precipitation | - Heavy rainfall in a short period of time can cause surface water erosion, which can erode the soil and reduce the anchoring capacity of renewable energy production systems. - Restricting access to technology platforms during periods of extreme rainfall | Elevated | - Monitoring the drainage level of the surfaces within the site - Flood risk assessment in the site area. - Monitoring the drainage level of the surfaces within the site | Reduced |
| 4 | Annual average of tornadoes | - High wind speeds can break or damage wind turbine blades - System deactivation due to exceeding the maximum speeds at which wind turbines are programmed to interrupt their activity | Elevated | - Using flexible blades that withstand high wind speeds - Building solid foundations adapted for extreme conditions - Obtaining insurance to cover the costs of damage caused by extreme weather events | Reduced |
| 5 | Wind Erosion Risk Index | - Degradation of wind turbine structures | Elevated | - Proper fastening of wind turbines - Grassing the project site | Reduced |
| 6 | Wildfire Risk Index | - Deterioration of renewable energy production infrastructure - Damage to the transport infrastructure for the energy generated by the wind farm | Elevated | - Carrying out vegetation maintenance works around the turbine site - Use of insulating materials for electricity transmission infrastructure | Reduced |

# Provisions for environmental monitoring

During the period of execution of the works, the conditions and requirements imposed by the regulatory acts obtained will be observed. We do not consider it necessary to implement a program to monitor the quality of environmental factors (analyses, measurements) at this stage.

During the execution of the project, the record of waste management, the record of environmental incidents, complaints, as well as of the measures taken to solve them will be maintained.

During the operation period, on the analyzed site, in accordance with the provisions of its own environmental management system and occupational health and safety, the following will take place:

- Periodic checks of the technical condition of the installations and the operating parameters and ensuring the permanent operation of the equipment with the role of environmental protection;

- Maintaining the record of waste management in accordance with the provisions of GD no. 856/2002 on waste management records and for the approval of the list of wastes, including hazardous wastes.

# Link with other normative acts and/or plans/programs/strategies/planning documents

Projects for the production of electricity from renewable sources are supported in the planning documents adopted at national level (e.g. the National Energy Strategy), as well as in the programs that ensure financing from European funds in the next programming period (e.g. PNRR). EU policy also includes a number of requirements regarding increasing the share of renewable energy sources in electricity production.

For the site that is subject to the construction works of the wind farm, the urban planning certificate no. 129/28.11.2022 with extended validity, by the Constanta County Council. These lands are located outside the built-up area, have the category of agricultural-arable use, pasture, communal road, exploitation roads and the destination of wind farm.

Regarding the impact on the environment, the project falls under the scope of Law no. 292/2018 on the assessment of the impact of certain public and private projects on the environment, being framed in Annex no. 2, item 3, letter i and item 10 letter e. The project does not fall under the scope of the Water Law no. 107/1996, with subsequent amendments and completions.

The proposed project falls under the scope of art. 28 of the Government Emergency Ordinance no. 57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna, with subsequent amendments and completions.

The project does not fall under the legal provisions on the control of activities that present dangers of major accidents involving dangerous substances. Also, the project does not fall under the legislation on industrial emissions.

# Works necessary for the organization of the site

  - Description of the works necessary for the organization of the site

In order to carry out the proposed investments, 3 site organizations (temporary) are needed on the plots: A251/1/31, A367/15, A250/9. The site organizations will occupy a temporary area of land of about 3,000 m2 each (total 9,000 m2).

- In order to arrange the site organization, the following works will be necessary:

- Delimitation and fencing of the site organization premises;

- Preparation of the land surface in order to place the necessary facilities;

- Arrangement of the access road;

- Arrangement and organization of the area intended for separate waste collection;

- Placement of containers for warehouses, toilets, etc.;

- Utility insurance - electricity, water, wastewater management;

- Location of the PSI picket and signaling according to the legal provisions in force;

- Ensuring the illumination of the lens;

- Installation of surveillance equipment with CCTV cameras.

  - Location of the site organization

The site organization will be located on the area related to the investment objective. The spaces intended for site organizations will be located on land areas already tabulated and fenced, during the construction works. The site organization and the storage of materials will be carried out in the project area and it will not be necessary to occupy additional land areas.

The location of the site organizations in the project area is shown in the figure below.

Figure no. 20 Location of site organizations

  - Description of the environmental impact of the site organization works

The potential impact generated by the realization and functioning of the site organizations can be manifested by:

- Temporary occupation of land areas. The impact is direct and temporary, and the temporarily occupied areas will be reduced to the minimum necessary;

- The impact on the environmental factors water, air, soil can be estimated as direct/indirect, depending on the nature of the pollutant and the local manifestation. The magnitude of the impact is low considering the scale of the works and the small number of machines involved in the execution of the project;

- Noise pollution manifests itself directly at the stage of execution of the works (see chapters 6 and 7 of the memorandum), over a limited period of time. Taking into account the small scale of the works and the short duration of the execution phase, it is estimated that the sensitive receivers will not be significantly affected by the noise produced during the construction works. During the operating period, noise pollution is limited to low values that occur during maintenance work.

Within the organization of the construction site, there will be no facilities for the preparation of construction materials, which will constitute fixed sources of atmospheric pollutants.

  - Sources of pollutants and installations for the retention, discharge and dispersion of pollutants in the environment during site organisation

During the execution of the works, the construction site is characterized by heavy traffic that causes emissions of pollutants into the atmosphere resulting either from the combustion of fuels (CO, CO2, NOx, SO2, suspended particles) or from the entrainment of dust from the roads and tire wear that generates sedimentable dust.

The activities in the organization of the site are not sources of pollution for surface and groundwater. The fecal-household wastewater generated on the site will be collected in the ecological toilets, which will be periodically emptied by authorized operators. Following the execution phase, no technological wastewater will be generated and no contaminants will be used.

Potential sources of soil and groundwater pollutants can be improper storage of waste, raw materials and materials, as well as accidental leakage of petroleum products from means of transport and machinery or wastewater leakage due to leakage.

  - Facilities and measures provided for the control of pollutant emissions into the environment

For the control of emissions into the environment, depending on the facilities that will be located in the site organization and the location and characteristics of the chosen sites, the following will be ensured:

- Ecological toilets for the management of fecal-domestic wastewater;

- The storage of materials, raw materials and waste that can lead to the appearance of pollutants for soil and groundwater will be carried out exclusively on impermeable surfaces specially designed for this purpose;

- For the organization of the construction site, waste bins and 1 fully equipped P.S.I. picket were provided.

# Site restoration works upon completion of the investment, in case of accidents and/or cessation of activity, to the extent that this information is available

  - Proposed works for the restoration of the site at the completion of the investment, in case of accidents and/or at the cessation of the activity

Upon completion of the wind farm execution works, the unoccupied land will be returned to the agricultural circuit.

Upon completion of the construction works, the contractor will ensure the restoration of the natural framework of the temporarily occupied areas. The areas affected by the construction works will be brought to a state that ensures the landscape integration of the elements subject to restoration works. These works will be carried out by sanitizing the area (completely removing the waste resulting from the specific activities of the work fronts, including household waste). The restoration works have both the purpose of ensuring the landscape restoration of the affected areas, and that of reducing the risk of entry and installation of invasive alien plant species on the affected areas, which would endanger the areas in the vicinity of the proposed project.

  - Aspects of prevention and response to accidental pollution cases (Plan of Accidental Pollution Response Measures and Provision of Necessary Means)

In case of accidental pollution, action will be taken according to the procedures established in the Plan for the prevention and combating of accidental pollution related to the construction site. The plan for preventing and combating accidental pollution on the construction site will be drawn up by the Contractor according to Order no. 278/1997 and will inventory and specify the activities, places and installations from which accidental pollution may originate. The plan will establish a set of clear measures and procedures for intervention in case of accidental pollution, as well as the attributions of the responsible persons nominated in the intervention team.

- As environmental incidents during the execution of the works, the following can be mentioned:

- Leakage or loss of fuels, oils or other hazardous substances from machinery;

- Accidental discharge of untreated wastewater from the sanitary groups within the site organization; Non-compliant collection and storage of waste containing hazardous substances.

In the event of such an incident occurring in the environment, the nature and level of the incident will be identified in order to act appropriately and limit the effects on the environment. The works will be stopped and appropriate intervention measures will be applied in order to minimize the impact. If deemed necessary, the intervention team will be mobilized, the equipment will be used, and the competent authorities will be notified, respectively the representatives of the National Administration of Romanian Waters and the Inspectorate for Emergency Situations.

  - Issues related to the closure/decommissioning/demolition of the facility

They do not impose themselves in this situation

  - Ways to restore the initial condition/rehabilitation for further use of the land

The project aims to build a wind farm for the production of electricity from renewable sources on the surface of land for agricultural use.

Of the total area for the arrangement of the wind farm, only a part will be permanently occupied by constructions, the rest of the land areas being returned to the agricultural circuit, within the limits of possibilities.

At the end of the project's lifetime, depending on the situation at that time, the wind farm will either be refurbished or the land will be reintroduced into the agricultural circuit.

ANNEX A - Documents

Annex 1. Certificate of attestation

Annex 2. Beneficiary documents

Annex 3. Property deeds-land book extracts

Anexa 4. Avize

Anexa 5. Coordonate STEREO - Shapefile

Annex 6. CSO Analysis

Annex 7. CSO Decisions

Annex 8. Related studies

ANNEX B – Plans and maps

Plate no.1. Zoning plan

Plate no.2. Situation plan

Appendices – Cartoons

# Information on protected natural areas of community interest

  - Brief description of the project and distance from the protected natural areas of community interest (ANPIC)

According to the Decision of the initial evaluation phase No. 54/07.02.2025, issued by APM Constanța, the proposed project falls under art. 28 of GEO no. 57/2007 on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna, with subsequent amendments and completions.

The coordinates in Stereo System 70 of the project site are presented in excel format and shapefile in Annex B of the presentation memorandum.

The project is not included in a plan/programme/strategy that has been subject to an environmental assessment procedure.

The details related to the types of intervention within the project and their location in relation to the protected natural areas of community interest (ANPIC) in the area of the project site can be found in the table below.

Table 12 Description of the proposed interventions and the distance from the protected natural areas of community interest (ANPIC) in the area of the site

| No. crt. | Type of intervention during the construction/operation/decommissioning period of the project Project objectives |  | Description of the main/secondary and related interventions of the project during the construction, operation and decommissioning period Description of project objectives | Location from ANPIC (distance) |
| --- | --- | --- | --- | --- |
| Construction stage |  |  |  |  |
|  | Arrangement of access roads |  | - Vegetation cleaning works; - Works to uncover the surface layer of soil; - Storage of soil from excavations; - Compaction/leveling works; - Ballast layer laying works; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works. | The proposed works do not intersect with the limit of the protected natural areas. The access roads that will be arranged are located within the perimeter studied by the project. The distance of the areas where these works are carried out from the limit of the protected areas: - zona turbinei T39 față de situl ROSAC0071 – approx. 7.47 km, ROSPA0036 – approx. 12.10 km, ROSPA0094 – approx. 23.93 km, ROSAC0157 – approx. 19.67 km, ROSPA0166 – approx. 12.90 km, BG0000569 – approx. 3.72 km, BG0000570 – approx. 8.36 km; - zona turbinei T31 față de situl ROSAC0071 – approx. 7.41 km, ROSPA0036 – approx. 10.13 km, ROSPA0094 – approx. 23.33 km, ROSAC0157 – approx. 19.54 km, ROSPA0166 – approx. 9.65 km, BG0000569 – approx. 7.57 km, BG0000570 – approx. 9.69 km; - zona turbinei T40 față de situl ROSAC0071 – approx. 6.50 km, ROSPA0036 – approx. 11.63 km, ROSPA0094 – approx. 25.25 km, ROSAC0157 – approx. 20.80 km, ROSPA0166 – approx. 14.12 km, BG0000569 – approx. 3.67 km, BG0000570 – approx. 6.89 km; - Turbine exploitation road to the ROSAC0071 site – approx. 3.22 km, ROSPA0036 - approx.8.27 km, ROSPA0094 – approx.18.79 km, ROSAC0157 – approx. 14.10 km, ROSPA0166 – approx. 7.12 km, BG0000569 – approx. 0.74 km, BG0000570 – approx. 29.42 km; |
|  | Realization of technology platforms and foundations |  | - Vegetation cleaning works; - Works to uncover the surface layer of soil; - Soil storage - Decopertat; - Compaction/leveling works; - Ballast/foundation laying works; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works. | The proposed works do not intersect with the limit of the protected natural areas. The access roads that will be arranged are located within the perimeter studied by the project. The closest areas to the boundary of the protected natural areas: - The platform of the T47 wind power plant 3.23 km from ROSAC0071 - The platform of the T23 wind power plant 6.83 km from ROSPA0166 - The platform of the T7 wind power plant 14.38 km from ROSCI0157 - T17 wind farm platform 18.93 km from ROSPA0094 - The platform of the T21 wind power plant 24.92 km from ROSPA0066 - T48 Wind Power Plant Platform at - 780 m of BG0000569 - T47 Wind Power Plant Platform at - 2.60 km from BG0000570 - The platform of the T3 wind power plant 11.6 km from BG0000130 |
|  | Installation of wind power plants |  | - Mounting the pillar, basket and blades | The proposed works do not intersect with the limit of the protected natural areas. The assembly works will be carried out within the limit of the perimeter studied by the project. The closest wind farms to the limit of the protected natural areas: - T46: ROSAC0071 at 3.34 km - T35: ROSAC0071 at 3.33 km - T47: ROSAC0071 at 3.23 km - T23: ROSPA0166 la 6.83 km - T7: ROSCI0157 at 14.34 km - T17:ROSPA0094 la 18.93 km - T21:ROSPA0066 la 25.00 km - T48:BG0000569 the 790 m - T2: BG0000570 the 1.11 km - T3: BG0000130 the 11.57 km |
|  | Making electrical connections and laying underground cables between wind generator sets and transformer stations |  | - Vegetation cleaning works; - Works to uncover the surface layer of soil; - Trench digging works; - Storage of soil from excavations; - Cable installation works; - Trench filling works; - Land leveling works; - Restoration of the vegetable carpet; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works. | The proposed works do not intersect with the limit of the protected natural areas. The works for the electrical connections will be carried out within the perimeter studied by the project, between the turbines and the transformer stations and will follow along the access roads. |
|  | Construction of transformer stations |  | - Vegetation cleaning works; - Foundation construction – excavations and concrete pouring; - Storage of soil from excavations; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works; - Installation of transformer station equipment - Fencing works. | The proposed works do not intersect with the limit of the protected natural areas. The works for the construction of the transformer stations are proposed to be carried out within the perimeter studied by the project. The closest protected natural areas to the area of the works for the transformer stations are: • SS01 - ROSAC0071 at 5.18 km - BG0000570 the 7.34 km • SS02: - ROSAC0071 at 7.82 km - BG0000569 the 3.67 km - BG0000569 the 5.66 km • SS03: - ROSAC0071 at 10.5 km - ROSCI0157 la 16,85 km • SS04: - BG0000569 3.5 km - ROSAC0071 at 11.12 km - ROSCI0157 la 16,89 km - BG0000570 11.50 km |
|  | Arrangement of the site organization |  | - Landscaping – cleaning works Vegetation; - Landscaping – cleaning works Vegetation; - Uncovering of topsoil layer; - Storage of uncovered soil Ballast layer laying works; - Works Compaction/leveling Construction site traffic of vehicles/operation of machinery necessary to carry out the works. | The proposed works do not intersect with the limit of the protected natural areas. The site organizations will be located within the perimeter studied by the project. The closest protected areas to the site area of the site organizations: • OS1: - BG0000569 3.75 km - ROSAC0071 at 7.73 km - ROSPA0036 la 12.4 km - ROSPA0166 la 13.3 km - ROSCI0157 la 19,85 km • OS2: - ROSAC0071 at 7.48 km - BG0000570 the 7.56 km - ROSPA0166 la 9.62 km - BG0000569 9.64 km - ROSPA0036 la 10.42 km - ROSCI0157 la 20,1 km • OS3: - BG0000569 3.75 km - ROSAC0071 at 6.55 km - BG0000570 the 6.84 km - ROSPA0036 la 12.4 km - ROSPA0166 la 9.73 km - ROSCI0157 la 21,21 km |
|  | Land restoration works at the completion of the construction |  | - Restoration of the site on which works were carried out and rehabilitation of the temporarily used surfaces; - Works to clear all installations, machinery and waste and to reinstall the topsoil layer on the surfaces that have been used temporarily. | The proposed works do not intersect with the limit of the protected natural areas. The restoration works of the affected lands will be carried out within the perimeter studied by the project. The areas where restoration works are needed are the areas occupied by site organizations, the cable route. The distances to the nearest protected areas are similar to those for site organizations and the route of cables/access roads. |
| Operation stage |  |  |  |  |
|  | Maintenance and milestones | - Construction site traffic of vehicles/operation of the necessary machinery; - Works, interventions and repairs of wind power plants; - Replacement of oils/lubricants; - Disposal of waste generated from repairs and maintenance works. |  | Interventions during the operation period do not intersect with the limit of protected natural areas. Because these situations are unknown at this stage of the project, the protected areas in the vicinity of the wind farm are presented: - Transformer station 1: - ROSAC0071 at 4.48 km; - ROSPA0036 la 7.90 km; - ROSPA0094 la 25.54 km; - ROSAC0157 at 22.12 km; - ROSPA0166 la 11.17 km; - BG0000569 the 8.25 km; - BG0000570 8.27 km. - Transformer station 2: - ROSAC0071 at 7.59 km; - ROSPA0036 la 12.27 km; - ROSPA0094 la 24.11 km; - ROSAC0157 at 19.75 km; - ROSPA0166 la 12.27 km; - BG0000569 3.97 km; - BG0000570 8.27 km. - Transformer station 3: - ROSAC0071 at 10.46 km; - ROSPA0036 la 13.97 km; - ROSPA0094 la 20.32 km; - ROSAC0157 at 16.31 km; - ROSPA0166 la 9.65 km; - BG0000569 la 5.77 km; - BG0000570 la 11.99 km. - Transformer station 4: - ROSAC0071 at 10.74 km; - ROSPA0036 la 15.08 km; - ROSPA0094 la 20.93 km; - ROSAC0157 at 16.38 km; - ROSPA0166 la 11.99 km; - BG0000569 la 3.38 km; - BG0000570 la 11.57 km. - T1 turbine platform: - ROSAC0071 at 12.75 km; - ROSPA0036 la 17.55 km; - ROSPA0094 la 20.70 km; - ROSAC0157 at 15.36 km; - ROSPA0166 la 14.16 km; - BG0000569 la 2.50 km; - BG0000570 la 12.72 km. - T2 turbine platform: - ROSAC0071 at 11.52 km; - ROSPA0036 la 16.53 km; - ROSPA0094 la 22.04 km; - ROSAC0157 at 16.76 km; - ROSPA0166 la 14.57 km; - BG0000569 la 1.12 km; - BG0000570 la 11.34 km. - T3 turbine platform: - ROSAC0071 at 13.35 km; - ROSPA0036 la 18.18 km; - ROSPA0094 la 20.53 km; - ROSAC0157 at 15.01 km; - ROSPA0166 la 14.53 km; - BG0000569 la 2.46 km; - BG0000570 la 13.18 km. - T4 turbine platform: - ROSAC0071 at 11.84 km; - ROSPA0036 la 16.66 km; - ROSPA0094 la 21.30 km; - ROSAC0157 at 16.12 km; - ROSPA0166 la 13.96 km; - BG0000569 la 1.86 km; - BG0000570 la 11.91 km. - T5 turbine platform: - ROSAC0071 at 8.48 km; - ROSPA0036 la 12.92 km; - ROSPA0094 la 22.88 km; - ROSAC0157 at 18.59 km; - ROSPA0166 la 12.29 km; - BG0000569 la 3.44 km; - BG0000570 la 9.36 km. - T6 turbine platform: - ROSAC0071 at 11.71 km; - ROSPA0036 la 16.19 km; - ROSPA0094 la 20.54 km; - ROSAC0157 at 15.66 km; - ROSPA0166 la 12.62 km; - BG0000569 la 2.86 km; - BG0000570 la 12.25 km. - T7 turbine platform: - ROSAC0071 at 13.01 km; - ROSPA0036 la 17.28 km; - ROSPA0094 la 19.14 km; - ROSAC0157 at 14.24 km; - ROSPA0166 la 12.11 km; - BG0000569 la 4.10 km; - BG0000570 la 13.65 km. - T8 turbine platform: - ROSAC0071 at 11.83 km; - ROSPA0036 la 16.09 km; - ROSPA0094 la 20.03 km; - ROSAC0157 at 15.33 km; - ROSPA0166 la 11.87 km; - BG0000569 la 3.57 km; - BG0000570 la 12.54 km. - T9 turbine platform: - ROSAC0071 at 12.86 km; - ROSPA0036 la 16.89 km; - ROSPA0094 la 18.82 km; - ROSAC0157 at 14.19 km; - ROSPA0166 la 11.27 km; - BG0000569 la 4.66 km; - BG0000570 la 13.70 km. - T10 turbine platform: - ROSAC0071 at 11.06 km; - ROSPA0036 la 15.28 km; - ROSPA0094 la 20.51 km; - ROSAC0157 at 16.02 km; - ROSPA0166 la 11.63 km; - BG0000569 la 3.56 km; - BG0000570 la 11.88 km. - T11 turbine platform: - ROSAC0071 at 11.99 km; - ROSPA0036 la 15.79 km; - ROSPA0094 la 19.16 km; - ROSAC0157 at 14.96 km; - ROSPA0166 la 10.35 km; - BG0000569 la 5.02 km; - BG0000570 la 13.09 km. - T12 turbine platform: - ROSAC0071 at 10.02 km; - ROSPA0036 la 14.19 km; - ROSPA0094 la 21.34 km; - ROSAC0157 at 16.99 km; - ROSPA0166 la 11.51 km; - BG0000569 la 3.71 km; - BG0000570 la 10.95 km. - T13 turbine platform: - ROSAC0071 at 11.60 km; - ROSPA0036 la 15 km; - ROSPA0094 la 19.21 km; - ROSAC0157 at 15.18 km; - ROSPA0166 la 9.19 km; - BG0000569 la 5.98 km; - BG0000570 la 12.98 km. - T14 turbine platform: - ROSAC0071 at 10.66 km; - ROSPA0036 la 14.28 km; - ROSPA0094 la 20.25 km; - ROSAC0157 at 16.19 km; - ROSPA0166 la 9.94 km; - BG0000569 la 5.22 km; - BG0000570 la 11.94 km. - T15 turbine platform: - ROSAC0071 at 9.18 km; - ROSPA0036 la 13.26 km; - ROSPA0094 la 21.93 km; - ROSAC0157 at 17.78 km; - ROSPA0166 la 11.30 km; - BG0000569 la 4.15 km; - BG0000570 la 10.27 km. - T16 turbine platform: - ROSAC0071 at 8.52 km; - ROSPA0036 la 12.69 km; - ROSPA0094 la 22.55 km; - ROSAC0157 at 18.44 km; - ROSPA0166 la 11.61 km; - BG0000569 la 4.12 km; - BG0000570 la 9.64 km. - T17 turbine platform: - ROSAC0071 at 11.05 km; - ROSPA0036 la 14.76 km; - ROSPA0094 la 18.92 km; - ROSAC0157 at 14.99 km; - ROSPA0166 la 8.17 km; - BG0000569 la 6.98 km; - BG0000570 la 13.35 km. - T18 turbine platform: - ROSAC0071 at 10.78 km; - ROSPA0036 la 14.15 km; - ROSPA0094 la 19.96 km; - ROSAC0157 at 15.97 km; - ROSPA0166 la 9.27 km; - BG0000569 la 5.90 km; - BG0000570 la 12.25 km. - T19 turbine platform: - ROSAC0071 at 10.78 km; - ROSPA0036 la 14.15 km; - ROSPA0094 la 19.96 km; - ROSAC0157 at 15.97 km; - ROSPA0166 la 9.27 km; - BG0000569 la 5.90 km; - BG0000570 la 12.25 km. - T20 turbine platform: - ROSAC0071 at 9.13 km; - ROSPA0036 la 12.94 km; - ROSPA0094 la 21.72 km; - ROSAC0157 at 17.70 km; - ROSPA0166 la 10.63 km; - BG0000569 la 4.88 km; - BG0000570 la 10.46 km. - T21 turbine platform: - ROSAC0071 at 9.96 km; - ROSPA0036 la 14.42 km; - ROSPA0094 la 18.82 km; - ROSAC0157 at 15.04 km; - ROSPA0166 la 7.09 km; - BG0000569 la 8.05 km; - BG0000570 la 13.73 km. - T22 turbine platform: - ROSAC0071 at 10.18 km; - ROSPA0036 la 13.80 km; - ROSPA0094 la 19.62 km; - ROSAC0157 at 15.79 km; - ROSPA0166 la 7.85 km; - BG0000569 la 7.37 km; - BG0000570 la 12.84 km. - T23 turbine platform: - ROSAC0071 at 8.97 km; - ROSPA0036 la 13.48 km; - ROSPA0094 la 19.56 km; - ROSAC0157 at 15.86 km; - ROSPA0166 la 6.75 km; - BG0000569 la 8.59 km; - BG0000570 la 13.31 km. - T24 turbine platform: - ROSAC0071 at 9.07 km; - ROSPA0036 la 12.81 km; - ROSPA0094 la 20.33 km; - ROSAC0157 at 16.59 km; - ROSPA0166 la 7.44 km; - BG0000569 la 8.12 km; - BG0000570 la 12.44 km. - T25 turbine platform: - ROSAC0071 at 9.68 km; - ROSPA0036 la 12.51 km; - ROSPA0094 la 20.89 km; - ROSAC0157 at 17.07 km; - ROSPA0166 la 8.32 km; - BG0000569 la 7.32 km; - BG0000570 la 11.68 km. - T26 turbine platform: - ROSAC0071 at 8.90 km; - ROSPA0036 la 11.65 km; - ROSPA0094 la 21.78 km; - ROSAC0157 at 17.97 km; - ROSPA0166 la 8.75 km; - BG0000569 la 7.30 km; - BG0000570 la 10.89 km. - T27 turbine platform: - ROSAC0071 at 8.72 km; - ROSPA0036 la 12.13 km; - ROSPA0094 la 21.95 km; - ROSAC0157 at 18.01 km; - ROSPA0166 la 9.94 km; - BG0000569 la 5.88 km; - BG0000570 la 10.38 km. - T28 turbine platform: - ROSAC0071 at 8.98 km; - ROSPA0036 la 12.13 km; - ROSPA0094 la 21.95 km; - ROSAC0157 at 18.01 km; - ROSPA0166 la 9.94 km; - BG0000569 la 5.88 km; - BG0000570 la 10.38 km. - T29 turbine platform: - ROSAC0071 at 8.80 km; - ROSPA0036 la 11.20 km; - ROSPA0094 la 22.01 km; - ROSAC0157 at 18.27 km; - ROSPA0166 la 8.38 km; - BG0000569 la 7.93 km; - BG0000570 la 10.97 km. - T30 turbine platform: - ROSAC0071 at 8.10 km; - ROSPA0036 la 10.93 km; - ROSPA0094 la 22.57 km; - ROSAC0157 at 18.75 km; - ROSPA0166 la 9.32 km; - BG0000569 la 7.20 km; - BG0000570 la 10.13 km. - T31 turbine platform: - ROSAC0071 at 7.34 km; - ROSPA0036 la 9.89 km; - ROSPA0094 la 23.47 km; - ROSAC0157 at 19.71 km; - ROSPA0166 la 9.54 km; - BG0000569 la 7.82 km; - BG0000570 la 9.59 km. - T32 turbine platform: - ROSAC0071 at 5.43 km; - ROSPA0036 la 7.48 km; - ROSPA0094 la 25.65 km; - ROSAC0157 at 21.94 km; - ROSPA0166 la 10.87 km; - BG0000569 la 9.16 km; - BG0000570 la 7.95 km. - T33 turbine platform: - ROSAC0071 at 10.79 km; - ROSPA0036 la 14.14 km; - ROSPA0094 la 19.54 km; - ROSAC0157 at 15.62 km; - ROSPA0166 la 8.34 km; - BG0000569 la 6.83 km; - BG0000570 la 12.76 km. - T34 turbine platform: - ROSAC0071 at 4.01 km; - ROSPA0036 la 8.34 km; - ROSPA0094 la 26.51 km; - ROSAC0157 at 22.57 km; - ROSPA0166 la 13.15 km; - BG0000569 la 7.10 km; - BG0000570 la 5.91 km. - T35 turbine platform: - ROSAC0071 at 3.36 km; - ROSPA0036 la 8.57 km; - ROSPA0094 la 27.64 km; - ROSAC0157 at 23.62 km; - ROSPA0166 la 14.73 km; - BG0000569 la 6.81 km; - BG0000570 la 4.58 km. - T36 turbine platform: - ROSAC0071 at 3.80 km; - ROSPA0036 la 8.72 km; - ROSPA0094 la 27.04 km; - ROSAC0157 at 23.04 km; - ROSPA0166 la 14.13 km; - BG0000569 la 6.59 km; - BG0000570 la 5.20 km. - T37 turbine platform: - ROSAC0071 at 4.98 km; - ROSPA0036 la 9.56 km; - ROSPA0094 la 25.94 km; - ROSAC0157 at 21.94 km; - ROSPA0166 la 13.24 km; - BG0000569 la 5.89 km; - BG0000570 la 6.28 km. - T38 turbine platform: - ROSAC0071 at 6.24 km; - ROSPA0036 la 10.95 km; - ROSPA0094 la 24.84 km; - ROSAC0157 at 20.69 km; - ROSPA0166 la 13.15 km; - BG0000569 la 4.57 km; - BG0000570 la 7.19 km. - T39 turbine platform: - ROSAC0071 at 7.16 km; - ROSPA0036 la 11.82 km; - ROSPA0094 la 24.05 km; - ROSAC0157 at 19.80 km; - ROSPA0166 la 12.94 km; - BG0000569 la 3.90 km; - BG0000570 la 8 km. - T40 turbine platform: - ROSAC0071 at 6.49 km; - ROSPA0036 la 11.60 km; - ROSPA0094 la 25.25 km; - ROSAC0157 at 20.82 km; - ROSPA0166 la 14.16 km; - BG0000569 la 3.75 km; - BG0000570 la 6.90 km. - T41 turbine platform: - ROSAC0071 at 6.68 km; - ROSPA0036 la 12.23 km; - ROSPA0094 la 25.76 km; - ROSAC0157 at 21.07 km; - ROSPA0166 la 15.19 km; - BG0000569 la 3.23 km; - BG0000570 la 6.73 km. - T42 turbine platform: - ROSAC0071 at 7.70 km; - ROSPA0036 la 13.44 km; - ROSPA0094 la 25.45 km; - ROSAC0157 at 20.50 km; - ROSPA0166 la 15.76 km; - BG0000569 la 2.20 km; - BG0000570 la 7.36 km. - T43 turbine platform: - ROSAC0071 at 6.50 km; - ROSPA0036 la 12.33 km; - ROSPA0094 la 26.27 km; - ROSAC0157 at 21.49 km; - ROSPA0166 la 15.81 km; - BG0000569 la 3.39 km; - BG0000570 la 6.37 km. - T44 turbine platform: - ROSAC0071 at 5.97 km; - ROSPA0036 la 12.11 km; - ROSPA0094 la 27.20 km; - ROSAC0157 at 22.41 km; - ROSPA0166 la 16.49 km; - BG0000569 la 4.17 km; - BG0000570 la 5.57 km. - T45 turbine platform: - ROSAC0071 at 4.21 km; - ROSPA0036 la 10.78 km; - ROSPA0094 la 28.75 km; - ROSAC0157 at 24.09 km; - ROSPA0166 la 17.22 km; - BG0000569 la 5.90 km; - BG0000570 la 3.88 km. - T46 turbine platform: - ROSAC0071 at 3.31 km; - ROSPA0036 la 9.91 km; - ROSPA0094 la 29.22 km; - ROSAC0157 at 24.71 km; - ROSPA0166 la 17.17 km; - BG0000569 la 6.69 km; - BG0000570 la 3.11 km. - T47 turbine platform: - ROSAC0071 at 3.19 km; - ROSPA0036 la 10.16 km; - ROSPA0094 la 30.06 km; - ROSAC0157 at 25.45 km; - ROSPA0166 la 18.09 km; - BG0000569 la 7.24 km; - BG0000570 la 2.56 km. - T48 turbine platform: - ROSAC0071 at 10.85 km; - ROSPA0036 la 15.88 km; - ROSPA0094 la 22.49 km; - ROSAC0157 at 17.32 km; - ROSPA0166 la 14.48 km; - BG0000569 la 0.72 km; - BG0000570 la 10.73 km. |
| Decommissioning stage (Interventions and activities are similar to those in the construction phase) |  |  |  |  |
|  | Construction site organization arrangement | - Landscaping – vegetation clearing works; - Landscaping – vegetation clearing works; - Uncovering of topsoil layer; - Storage of uncovered soil; - Ballast layer laying works; - Compaction/leveling works; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works. |  | Interventions during the decommissioning period do not intersect with the limit of protected natural areas. It is considered that the locations of the site organizations during the decommissioning period will be similar to those during the construction period. The closest protected areas to the site area of the site organizations: - OS1: - BG0000569 la 3.75 km; - ROSAC0071 at 7.73 km; - ROSPA0036 la 12.4 km; - ROSPA0166 la 13.3 km - ROSCI0157 la 19,85 km - OS2: - ROSAC0071 at 7.48 km - BG0000570 the 7.56 km - ROSPA0166 la 9.62 km - BG0000569 9.64 km - ROSPA0036 la 10.42 km - ROSCI0157 la 20,1 km - OS3: - BG0000569 3.75 km - ROSAC0071 at 6.55 km - BG0000570 the 6.84 km - ROSPA0036 la 12.4 km - ROSPA0166 la 9.73 km - ROSCI0157 la 21,21 km |
|  | Dismantling of wind power plants | - Dismantling the basket and pillar blades |  | The proposed works do not intersect with the limit of the protected natural areas. The assembly works will be carried out within the limit of the perimeter studied by the project. The closest wind farms to the limit of the protected natural areas: - T46:ROSAC0071 at 3.34 km; - T35:ROSAC0071 at 3.33 km; - T47:ROSAC0071 at 3.23 km - T23: ROSPA0166 la 6.83 km - T7: ROSCI0157 at 14.34 km - T17:ROSPA0094 la 18.93 km - T21:ROSPA0066 la 25.00 km - T48:BG0000569 the 790 m - T2: BG0000570 the 1.11 km - T3: BG0000130 the 11.57 km |
|  | Decommissioning of technology platforms and foundations | - Vegetation cleaning works; - Works to uncover the surface layer of soil; - Soil storage - Decopertat; - Foundation demolition works; - Compaction/leveling works; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works. |  | The proposed works do not intersect with the limit of the protected natural areas. The access roads that will be arranged are located within the perimeter studied by the project. The closest areas to the boundary of the protected natural areas: - The platform of the T47 wind power plant 3.23 km from ROSAC0071; - The platform of the T23 wind power plant 6.83 km from ROSPA0166; - The platform of the T7 wind power plant at 14.38 km from ROSCI0157; - The platform of the T17 wind power plant at 18.93 km from ROSPA0094; - The platform of the T21 wind power plant 24.92 km from ROSPA0066; - The platform of the T48 wind power plant 780 m from BG0000569; - The platform of the T47 wind power plant 60 km from BG0000570; - The platform of the T3 wind power plant 11.6 km from BG0000130. |
|  | Dismantling of electrical connections and removal of underground cables | - Vegetation cleaning works; - Works to uncover the surface layer of soil; - Digging works ditches; - Storage of soil from excavations; - Removal of electrical cables; - Trench filling and land leveling works; - Restoration of the vegetable carpet; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works. |  | The proposed works do not intersect with the limit of the protected natural areas. The works for the electrical connections will be carried out within the perimeter studied by the project, between the turbines and the transformer stations and will follow along the access roads. |
|  | Decommissioning of transformer stations | - Dismantling of transformer station equipment; - Vegetation cleaning works; - Excavation works; - Storage of soil from excavations; - Demolition of foundations; - Construction site traffic of vehicles/operation of the machinery necessary to carry out the works. |  | The proposed works do not intersect with the limit of the protected natural areas. The works for the construction of the transformer stations are proposed to be carried out within the perimeter studied by the project. The closest protected natural areas to the area of the works for the transformer stations are: - SS01 - ROSAC0071 at 5.18 km - BG0000570 7.3 - SS02: - ROSAC0071 at 7.82 km - BG0000569 the 3.67 km - BG0000569 5.66 km - SS03: - ROSAC0071 at 10.5 km - ROSCI0157 la 16,85 km • SS04: - BG0000569 3.5 km - ROSAC0071 at 11.12 km - ROSCI0157 la 16,89 km - BG0000570 11.50 km |
|  | Decommissioning of exploitation roads | - Vehicle traffic/machinery/equipment operation Necessary; - Works, interventions and repairs of wind power plants; - Current repairs, equipment replacement; - Replacement of oils/lubricants; - Disposal of waste generated from repairs and maintenance works. |  | The proposed works do not intersect with the limit of the protected natural areas. The access roads that will be arranged are located within the perimeter studied by the project. The distance of the areas where these works are carried out from the limit of the protected areas: - 3.32 km from ROSAC0071 - 7.74 km from ROSPA0166 - 15.16 km from ROSCI0157 - 18.85 km from ROSPA0094 - 24.93 km from ROSPA0066 - 630 m BG0000569 - 2.90 km from BG0000570 - 12.1 km from BG0000130 |
|  | Land restoration works at the completion of the construction | - Restoration of the site on which works were carried out and rehabilitation of the temporarily used surfaces; - Works to clear all installations, machinery and waste and to reinstall the topsoil layer on the surfaces that have been used temporarily. |  | The proposed works do not intersect with the limit of the protected natural areas. The restoration works of the affected lands will be carried out within the perimeter studied by the project. The areas where restoration works are needed are the areas occupied by site organizations, the cable route. The distances to the nearest protected areas are similar to those for site organisations, turbines and the route of cables/access roads. |

The following figure shows the location of the project in relation to the protected natural areas

Figure no. 21 Location of the project in relation to the areas of the Natura 2000 Network

  - Name and code of the protected natural area of community interest

In order to analyze the location of the project in relation to the protected natural areas and the potential for affecting them, the vector boundaries available on the website of the Ministry of Environment, Water and Forests were used.

In order to identify the Natura 2000 sites potentially affected by the implementation of the project, the criteria mentioned in the annex of the Order of the Minister of Environment, Water and Forests no. 1.682/2023 for the approval of the Methodological Guide on the adequate assessment of the potential effects of plans or projects on protected natural areas of community interest and in the annex to the Order of the Minister of Environment, Water and Forests no. 1.679/2023 for the approval of the Specific Methodological Guide on the adequate assessment of the potential effects of plans/projects in the areas of interest. These criteria are: a) intersection; b) neighborhood (area of influence); c) species mobility; d) ecological connectivity.

- Identification of Natura 2000 sites intersected by the project

The identification of the Natura 2000 sites intersected by the project was carried out through a spatial analysis that took into account all the elements of the project. They were analyzed in relation to the boundaries of the protected natural areas, and based on the overlap of the project elements with the boundaries of the Natura 2000 sites, it was found that the project intersects a site of community interest.

- Identification of Natura 2000 sites in the project's area of influence:

Area of direct influence

During the project execution period, the main effects generated are represented by noise, atmospheric emissions and the potential to favor the dispersion of invasive plant species. The area of direct influence of the project was considered the area where the effects of drilling are felt, namely, the actual execution area of the drilling.

From the analysis of the protected natural areas of community interest in relation to the location of the project, the sites ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, Kardam BG0000569 and BG0000570 Izvorovo - Kraishte were identified as being in the area of direct influence.

Indirect area of influence

The indirect area of influence represents the area where effects generated by other activities occur, modified as a result of the implementation of the analyzed project. In the case of this project, spatial extension will lead to the affectation of other Natura 2000 sites besides those already identified as a result of the indirect influence zone, these being ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii, ROSPA0036 Dumbrăveni, ROSPA0166 Plopeni-Chirnogeni, ROSPA0094 Hagieni Forest, ROSPA0151 Ciobănița-Osmancea, ROSPA0001 Aliman - Adamclisi, .

- Identification of Natura 2000 sites where highly mobile species that can reach the project area are protected

For this criterion, Natura 2000 sites located at least 4 km from the project boundary were analysed and the standard forms were checked to see if they were designated for the protection of species with high mobility, such as bird species, flying invertebrates, bats and large carnivores, which could reach and be affected by the project area. This category includes the sites: ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSCI0157/ROSAC157 Hagieni Forest, ROSPA0036 Dumbrăveni, ROSPA0166 Plopeni-Chirnogeni, ROSPA0094 Hagieni Forest,  BG0000569 Kardam and BG0000570 Izvorovo - Kraishte. Given the size of the project, a predominantly agricultural area, which has elements of no conservation value, the possible presence of species with high mobility within the listed areas is quite likely.

- Identification of Natura 2000 sites whose connectivity or ecological continuity may be affected by the implementation of the project

For this criterion, the protected natural areas of community interest whose connectivity (within the site or with the rest of the Natura 2000 network) can be interrupted by the appearance of barriers at the level of ecological corridors were analyzed. Given the nature of the proposed interventions and the specifics of the project, it is likely that the connectivity or ecological continuity of the Natura 2000 sites in the implementation area will be affected.

- Other information relevant to the influence of the proposed energy park on protected natural areas

At the end of 2012, the National Institute for Research and Development Danube Delta (INCDDD), within a contract financed by the Ministry of Environment and Forests, completed the "Study on recommendations on the areas in Dobrogea, where the location of wind farms is restricted due to the migration corridors of gliding birds (day raptors, storks, pelicans) respectively due to the wintering of geese and swans". The following areas were proposed in the study, spatially located in  the figure below:

- Exclusion zones for wind power plants and related infrastructures, where it is recommended to prohibit their construction: these areas included the Natura 2000 sites (SPAs and SCIs), the Danube Delta Biosphere Reserve, national and natural parks and nature reserves/ornithological refuges on the territory of mainland Dobrudja. The area of the proposed exclusion zones represents 50.4% of the region's area;

- High-risk areas: in these areas, a series of restrictions on the location and operation of wind farms are necessary to minimize the risk of collision (e.g. monitoring using radar technology, built-in microphones to monitor the collision rate, etc.), and long-term monitoring (minimum 10 years) of gliding bird species and/or wintering geese and swan populations is required. The area of the proposed high-risk areas represents 36.4% of the area of Dobrogea;

- Moderate-risk areas: Long-term monitoring of gliding bird species and/or wintering geese and swan populations is required in these areas. The area of the proposed moderate risk areas represents 13.2% of the area of Dobrogea.

These areas only take into account areas that are important for bird species, and other types of areas with building restrictions for wind farms are not included (e.g. areas important for bat species, localities, public road network, nesting information for bird species).

From the conclusions of the "Study on the recommendations on the areas in Dobrogea where the location of wind farms should be restricted due to the migration corridors of gliding birds (day raptors, storks, pelicans), respectively due to the wintering of geese and swans", carried out by the National Institute for Research and Development Danube Delta in 2012, it appears that the perimeter studied by the PUZ is located in an area with moderate risk in terms of the conservation of the species of and natural habitats. In moderate-risk areas. The moderate risk areas amount to approximately 206100 ha, which represents 13.2% of the area of Dobrogea. In moderate-risk areas, long-term monitoring of gliding bird species and/or wintering geese and swan populations is necessary in order to establish conditions for the construction or operation of wind farms

Figure no. 22 Map of areas where the construction of wind turbines may be allowed, but their construction or operation will have to be restricted due to the migration of gliding birds and/or wintering populations of geese and swans

The perimeter of the PUZ is located in the vicinity of the distribution areas of the bird species Red-necked goose (Branta ruficollis). In Romania, the population of the red-breasted goose is found in the lowlands of Bărăgan and Dobrogea, near the large lakes, which it uses for the night. In milder winters, several thousand geese remain to winter on the territory of Ukraine and even in the south of Russia. When it is cold, the geese move quickly south, in the Dobrogea region on the territory of Romania and Bulgaria. Currently, the most important areas for the species remain the sites in southeastern Romania, mainly the regions of Bărăgan, Danube Delta and Dobrogea declared several protection areas for the species: Balta Albă -Amara - Jirlău, Ianca - Plopu - Sărat, Strachina Lake, Iezerul Călărași, Gura Ialomiței - Berteștii de Sus, Balta Mică a Brăila Natural Park, Oltina Lake, Danube Delta and Razim-Sinoe Lagoon Complex.

The latest estimate of the national population of red-breasted goose, for the period 2013-2022, is approximately 9,915-23,641 individuals, wintering in Romania, representing about 42% of the global population. The following figure shows the distribution of the red-necked goose species (Branta ruficollis) in Romania, in the period 2000-2022 (EEA grid, 10x10 km, in ETRS89 projection system).

Figure no. 23  Distribution map of the red-necked goose (Branta ruficollis) in Romania, between 2000 and 2022

2

Currently, there is no data in Romania on the collision of red-breasted geese with power lines and wind turbines. Research in Western and Northern Europe shows that wind turbines have a negative impact on geese populations. The main effect recorded is the abandonment of feeding areas which leads to the actual loss of key habitats (Larsen & Madsen, 2000; Larsen & Clausen, 2002, Madsen & Boertmann, 2008 and etc.).

From the analysis of the map showing the correlation between the important areas for the red-necked goose (Branta ruficollis) in Dobrogea and the wind farms, it is observed that the perimeter of the analyzed energy park is located in an area of low importance (low sensitivity).

Currently, there is no data in Romania on the collision of red-breasted geese with power lines and wind turbines. Research in Western and Northern Europe shows that wind turbines have a negative impact on geese populations. The main effect recorded is the abandonment of feeding areas which leads to the actual loss of key habitats (Larsen & Madsen, 2000; Larsen & Clausen, 2002, Madsen & Boertmann, 2008 and etc.).

From the analysis of the map showing the correlation between the important areas for the red-necked goose (Branta ruficollis) in Dobrogea and the wind farms, it is observed that the perimeter of the analyzed energy park is located in an area of low importance (low sensitivity).

Figure no. 24  Correlation between important areas for red-breasted goose (Branta ruficollis) in Dobrogea and wind farms. Authorized (black) and operational (green) wind turbines in Dobrogea and Bărăgan approved in Romania, between 2000 and 2022

- Information on previous environmental procedures

The first proposal for the construction of a wind energy park in the Cerchezu area was initiated in 2011. For the construction of the energy park (composed of 3 wind farms), a PUZ was created, which proposed the location of a number of 117 wind power plants, within the radius of the Cerchezu TAU, with a total installed capacity of 351 MW. The beneficiaries of the initial project were: SC South Wind Cherchezu SRL, SC South Wind SRL, SC South Wind Viroaga SRL. For the initial project, the following procedures were followed:

- Strategic environmental assessment procedure - completed with the issuance of the adoption of the PUZ without an environmental permit (APM Decision no. 12054RP/19.12.2011)

- Adequate Assessment Procedure - For the 2011 energy park, an Adequate Assessment Study was carried out, in response to the addresses of APM Constanta no.12052/12053/120154/02.11.2011.

The results of the Adequate Assessment Study carried out in 2011 for the 3 parks around Cerchezu, from which the analyzed project was derived, were not based on the assessment in relation to the conservation objectives of the Natura 2000 sites, at that time the specific conservation objectives were not defined. The major conclusion of this Adequate Assessment Study was that the location of the wind turbines in the area of interest, an area already stored, will have an insignificant impact on the local vegetation, flora and fauna.

It should be noted that since the date of the Adequate Assessment Study, the ROSPA0166 site was not declared a protected area.

The project analyzed in this memorandum is a modification of the plan/project approved in 2011 by APM Constanța.

Figure no. 25 Location of the location of the Energy Park proposed in 2011 (Perimeter data source

CPU 2011; Cerchezu Wind Farm Adequate Assessment Study, 2011)

In the project currently under review, the number of wind farms proposed to be installed has been reduced by more than half (the 2023 version of the PUZ proposes 46 wind farms compared to 117 as proposed in 2011), some wind farm positions have been reassigned, others have been eliminated and new wind power plant positions have been introduced.

In the analysis and choice of variants, the performance of wind power plants was also taken into account, in the sense that it is proposed to place modern, high-performance equipment, with safer and quieter operation. The proposed plan increases the distance between wind farms to over 600 m compared to about 350 m in the situation approved in 2011.

At the same time, at the level of 2023, the procedure for obtaining the environmental permit was started, for this purpose, both an Environmental Report and an Adequate Assessment Study were carried out, where a series of significant impacts were identified, a series of measures were proposed (which will be implemented once the park itself is built) in order to obtain an insignificant residual impact.

Table 13 Natura 2000 sites analysed and identification of those potentially affected

| No. Crt. | Type of protected area | Protected area code | Name of protected area | Distance to project boundaries (km) | Potentially affected | Intersection | Direct influence | Indirect influence | Species mobility | Green connectivity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Site of Community Importance (SCI) | ROSCI0071/ROSAC0071 | Dumbrăveni - Urluia Valley - Vederoasa Lake | 2,76 | Yes | Not | Yes | Not | YES The site was declared for the protection of 3 species of bats, species of amphibians and reptiles and for the protection of the otter | YES The project may affect the movement of species as a result of the barrier effect |
|  | Site of Community Importance (SCI) | ROSCI0157/ROSAC157 | Hagieni Forest - Bend of the Valley | 13,81 | Yes | Not | Not | Yes | YES The site was declared for the protection of 3 species of bats, species of amphibians and reptiles and for the protection of the otter | YES The project may affect the movement of species as a result of the barrier effect |
|  | Special protection area Avifaunistică (SPA) | ROSPA0036 | Dumbrăveni | 7,71 | Yes | Not | Right away | Also | ALSO The site was declared for the projection Bird species | YES The project may affect the movement of species as a result of the barrier effect |
|  | Special protection area Avifaunistică (SPA) | ROSPA0166 | Plopeni-Chirnogeni | 6,21 | Yes | Not | Right away | Yes | YES The site was declared for the projection Bird species | YES The project may affect the movement of species as a result of the barrier effect |
|  | Special protection area Avifaunistică (SPA) | ROSPA0094 | Hagieni Forest | 18,25 | Also | Right away | Right away | Also | ALSO The site was declared for the projection Bird species | YES The project may affect the movement of species as a result of the barrier effect |
|  | Special protection area Avifaunistică (SPA) | ROSPA0151 | Ciobănița- Osmancea | 20,1 | Right away | Right away | Right away | Right away | Right away The species identified in this site will not reach the project area | Right away |
|  | Special protection area Avifaunistică (SPA) | ROSPA0001 | Aliman – Adamclisi. | 19,72 | Right away | Right away | Right away | Right away | Right away The species identified in this site will not reach the project area | Right away |
|  | Site of Community Importance (SCI) | BG0000569 | Kardam | 0,41 | Also | Right away | Also | Right away | Right away The species identified in this site will not reach the project area | Right away |
|  | Site of Community Importance (SCI) | BG0000570 | Izvorovo - Kraishte | 3,64 | Yes | Not | Yes | Not | YES The site has been declared for the protection of 12 species of bats | YES The project may affect the movement of species as a result of the barrier effect |
|  | Site of Community Importance (SCI) | BG0000130 | Kraymorska Dobrudzha | 11 | Yes | Not | Not | Yes | YES The site has been declared for the protection of 12 species of bats | YES The project may affect the movement of species as a result of the barrier effect |

Natura 2000 sites analysed and identification of those potentially affected

Table 14 Information on ANPIC potentially affected by the project

| ANPIC code and name | Intersected (Yes/ No) | Conservation objectives (Yes/No) | Management of the Plan (to/to) | ANPIC included in the Area of Influence of the plan (Yes/ No (justification)) | ANPIC is home to species of fauna that can move around the plan area (Yes/ No justification) | ANPIC ecologically connected to the plan area (Yes/No (justify)) | Restrictive measures in the PM/normative act/administrative act |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ROSAC0071 (ROSCI0071) | Dumbrăven i - Valea Urluia - Lake Vederoasa | YES ANANP Decision no.414/03.08.2022 | YES Management Plan approved by the Order of the Minister of Environment, Water and Forests no. 1557/2016 | YES Area of direct influence The site is located about 2.76 km from the limit of the perimeter studied by the PROJECT, the proposed investments may have a direct influence on bat species during the operating period. Effects generated by the PROJECT, such as noise, vibrations, air pollutants, artificial lighting, dispersal of invasive species and others | YES The site was declared for the protection of 3 species of bats, species of amphibians and reptiles and for the protection of the otter. These species have a high mobility and can move in the area of the proposed works in search of food or in transit to and from the feeding or resting areas | YES The lands in the perimeter studied by the PROJECT fall into the category of land occupied partly by agriculture, with significant areas of natural vegetation, arable land, grasslands, watercourses, irrigation canals. The area can be a migration corridor for bats. The protected area is drained by the Ceair Valley with the tributaries Magura and Cerchez. The Cerchez Creek crosses the perimeter of the site from north to south, and the Magura Creek intersects the analyzed perimeter in the western part. Also, on the analyzed lands there are disused irrigation canals. Watercourses and irrigation canals are ecological corridors and potential habitats for amphibian and reptile species | NOT The management plan, being common with other protected natural areas in the area (ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Balta Vederoasa that overlap with the boundaries ROSCI0071) establishes restrictive measures for bird species, not to place wind farms at a distance of less than 3 km from the boundary of the sites. The site ROSCI0071 not declared for the protection of bird species, |
| ROSPA0036 | Dumbrăveni | YES ANANP Decision no.414/03.08.2022 | YES Management Plan approved by the Order of the Minister of Environment, Water and Forests no. 1557/2016 | YES Area of influence Indirect The site is located about 7.71 km from the perimeter limit studied by PROIECT, the proposed investments may have a direct influence on the bird species during the operating period. The main forms of impact that may occur in ANPIC located within the indirect influence zone are represented by habitat alteration (habitat degradation), loss of feeding habitats and/or disturbance of species activity. | YES The site was declared for the projection Bird species | YES The lands in the perimeter studied by the PROJECT fall into the category of land occupied partly by agriculture, with significant areas of natural vegetation, arable land, grasslands. The area can represent a corridor to the feeding areas of bird species, between the limit of the perimeter studied by the PROJECT and the limit of the protected area there are isolated fragments of natural and semi-natural habitats that can constitute feeding areas and allow the movement and migration of species to and from the site. In areas with moderate risk for the migration of gliding birds and/or wintering populations of geese and swans it is necessary to winter. However, the aforementioned study did not take into account information on the nesting of these bird species. | YES The management plan, being common with other protected natural areas in the area (ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Balta Vederoasa that overlap with the ROSCI0071 limits) establishes restrictive measures to ensure the conservation objective for bird species. Restrictive measures are; Do not place wind farms at a distance of less than 3 km from the site line. The ROSCI0036 site is located at a distance of about 7.71 km from the limit of the perimeter studied by the PROJECT. |
| ROSPA0166 | Plopeni- Chirnogeni | YES ANANP Note no.1788/24.03.2022 | NOT | YES Area of influence Indirect The site is located at about 6.21 km from the limit of the perimeter studied by the PROJECT, the proposed investments may have a direct influence during the operation period on the bird species. represented by habitat alteration (habitat degradation) and/or disruption of species activity. | YES The site has been declared for the protection of bird species. | YES The land in the perimeter studied by the PROJECT falls into the category of land partly occupied by agriculture, with significant areas of natural vegetation, arable land, meadows. The area can represent a corridor to the feeding areas of bird species, between the limit of the perimeter studied by the PROJECT and the limit of the protected area there are isolated fragments of natural habitats and semi-natural areas that can constitute feeding areas and allow the movement and migration of species to and from the site. | NOT |
| ROSPA0094 | Hagieni Forest | YES ANANP Decision no. 426/24.09.2020 ANANP Decision no. 197/26.06.2020 ANANP Decision no.118/08.05.2020 | YES Management Plan, approved by the Order of the Minister of the Environment, Waters and Forests no 1480/2016 | YES Area of influence Indirect The site is located about 18.25 km from the limit of the perimeter studied by the PROJECT, the proposed investments may have a direct influence during the operation period on the bird species. According to the management plan, wind energy projects are considered current pressures and future threats to the bird species at the site. | YES The site has been declared for the protection of bird species | YES The lands in the perimeter studied by the PROJECT fall into the category of land occupied partly by agriculture, with significant areas of natural vegetation, non-irrigated arable land, grasslands. The area can represent a corridor to the feeding areas of bird species, between the limit of the perimeter studied by the PROJECT and the limit of the protected area there are isolated fragments of natural habitats and semi-natural areas that can constitute feeding areas and allow the movement and migration of species to and from the site. | YES Restrictive measures to reduce the disturbance of bird species on the site. For new applications for the location of buildings, the following set of restrictions will be introduced: - no constructions will be placed on the meadow, - No new constructions will be placed less than 400 m from forests or wetlands - lakes, swamps, watercourses, and the like. The constructions may be located only on arable land, in compliance with the requirements of point b). |
| ROSAC0157 (ROSCI0157) | Forest Hagieni - Bend of the Valley | YES ANANP Decision no. 426/24.09.2020 ANANP Decision no. 197/26.06.2020 ANANP Decision no.118/08.05.2020 | YES Management Plan approved by the Order of the Minister of Environment, Water and Forests no. 1480/2016 | YES Area of influence Indirect Area of direct influence The site is located about 13.81 km from the limit of the perimeter studied by the PROJECT, the proposed investments may have a direct influence on bat species during the operating period. | YES The site has been declared for the protection of 3 species of bats | YES The lands in the perimeter studied by the PROJECT fall into the category of land occupied partly by agriculture, with significant areas of natural vegetation, non-irrigated arable land, grasslands. The area can represent a corridor to the feeding areas of bird species, between the limit of the perimeter studied by the PROJECT and the limit of the protected area there are isolated fragments of natural and semi-natural habitats that can constitute feeding areas and allow the movement and migration of species to and from the site. | NOT The management plan is shared with that of the ROSPA0094 site. The plan establishes restrictions for bird species. The site was declared for the projection of bat species, invertebrate mammals and reptiles. |
| ROSPA0151 | Ciobănița- Osmancea | YES Note no.2053/04.04.2023 | NOT | NOT The analyzed perimeter was located at a distance of more than 18.3 km from the site limit. | NOT Although the site has been declared for the protection of 11 species referred to in Article 4 of Directive 2009/147/EC, species listed in Annex II Directive 92/43/EEC, the perimeter analysed is not located at a distance of more than 18 km from the site boundary, it is unlikely that the effects of the park will be felt in the area of the site | NOT Although, the lands in the perimeter studied by the PROJECT fall into the category of land partly occupied by agriculture, with significant areas of natural vegetation, arable land, grasslands, and the analyzed perimeter could constitute feeding areas for the bird species for which the site was designated, given the long distance (>18 km) to these areas makes it unlikely that the species will move from the site to the perimeter of the PROJECT. | NOT |
| ROSPA0001 | Food – Adamclisi. | YES Decision no.414/03.08.2022 | YES Approved Management Plan Approved by the Order of the Minister of Environment, Water and Forests 1557/2016 | NOT The analyzed perimeter was located at a distance of more than 19 km from the site limit. | NOT Although the site is home to 33 species of birds listed in Annex I of the 2009/147/EC and regularly migrating bird species Annex II of Directive 2009/147/EC, the analysed perimeter is not located on their migration routes. | NOT Although, the land in the perimeter studied by the PROJECT falls into the category of land partly occupied by agriculture, with significant areas of natural vegetation, arable land, grassland, and the analyzed perimeter could constitute feeding areas for the bird species for which the site was designated, given the long distance (>19 km) to these areas makes it unlikely that the species will move from the site to the perimeter of the PROJECT. | YES Implementation of restrictive measures regarding the location of wind farms within a radius of 3 km around the protected area. The limit of the perimeter studied by the PROJECT is at a distance >19 km from the site limit. |
| BG0000569 | Cardamom (Bulgaria) | NOT | NOT | NOT The perimeter of the analyzed PROJECT does not intersect with the boundaries of this site. Although the site is located about 410 m km from the limit of the perimeter studied by the PROJECT, the site has been declared for terrestrial species (mammals, reptiles) species with reduced mobility | NOT The site is not home to terrestrial species with high mobility. | NOT The boundary of the site is separated from the perimeter boundary of an exploitation road. | NOT According to the site's standard form, the use of wind energy (C03.03) is not among the current pressures for species at the site. |
| BG0000570 (Bulgaria) | Izvorovo – Kraishte | NOT | NOT | YES Indirect Area of Influence The scope of the analyzed PROJECT is not intersects with the boundaries of this site. The site is located at a distance of about 3.64 km west of the perimeter studied through the PROJECT. The site is home to several species of bats (Rhinolophus mehelyi, Pipistrellus pipistrellus, Eptesicus serotinus) | YES The site has been declared for the protection of 12 Bat species | YES The area can be a migration corridor for bats. | NOT According to the site's standard form, the use of wind energy (C03.03) is not among the current pressures for species at the site. |
| BG0000130 (Bulgaria) | Kraymorsk a Dobrudzha | NOT | NOT | YES Area of influence Indirect The site is located at a distance of about 11 km downstream of the site boundary. The proposed investments may have an influence on bat species during the period of operation (disruption of activity) | YES The site has been declared for the protection of 12 Bat species | YES The area can be a migration corridor for bats. | NOT According to the site's standard form, the use of wind energy (C03.03) is not among the current pressures for species at the site. |

  - Presence and herds/areas covered by species and habitats of community interest in the project area

The presence and areas/herds covered by habitats and species of community interest in the project area, the location, the geographical direction and the altitudinal difference from the project site, as well as the conservation status and conservation objectives according to the Specific Conservation Objectives of the analyzed sites are presented in the following tables.

Table 15 Presence and herds/areas covered by species and habitats of community interest in the project area

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| ROSAC0071 (ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake | 3150 Natural eutrophic lakes with vegetation such as Magnopotamion or Hydrocharitlon | 5.25 ha | NOT The distribution area of this habitat is located at a distance of about 27.68 km from the limit of the area studied by PROJECT | The distribution area of this habitat is located upstream, north of the Project boundary, in the lake edge area Vederoasa, near the Vlahii locality | Unfavorable | Improvement of conservation status |
|  | 40C0* Deciduous bushes Sarmatians | 6.84 ha | NOT The distribution area of this habitat is located at a distance of more than 7 km from the limit of the area studied by the PROJECT | The distribution area of this habitat is located upstream, northwest of the Project boundary. Tupo-Sarmatian deciduous bushes are found in the following areas inside the site: Şipote to the south and east of the village, south-east of Zorile, in the lower part of the Butterfly Hill inside the Dumbrăveni Forest Reserve, respectively a small fragment on the slope in the west of Lake Baciului. Outside the site of community importance, there are bushes in the Urluia areas on three cliffs in the east of the village, and northwest of Hațeg. | Unfavorable (favourable in terms of habitat structure and functions, and in terms of prospects for the future, but unfavourable — inadequate in terms of surface) | Improvement of conservation status |
|  | 62C0* Stepe ponto- Sarmatian | 86.15 ha | NOT The area studied by the PROJECT is located about 2.79 km from the border ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake. According to the Site Management Plan, the habitat was not found during the field assessment of the 2014 season. | According to the Site Management Plan, the habitat was not found during the field assessment of the 2014 season. | Unfavourable (unfavourable – inadequate in terms of area, habitat structure and functions, and prospects for the future) | Improvement of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | 6430 Border communities with high hygrophilous grasses from the plains to the mountainous level and alpine | Unknown The habitat has not been identified on the surface of the site | Not The habitat has not been identified on the surface of the site according to data presented in the Management Plan | The habitat has not been identified on the surface of the site according to the data presented in the Management Plan | Clarification of the conservation status within 3 years | Maintaining or Improving the state of conservation |
|  | 91AA Pontosarmadca forest vegetation with downy oak | 38.6 ha | NOT The distribution area of this habitat is located at distances greater than 18 km from the limit of the area studied by PROJECT | The habitat is present in the site in the form of small clumps in the upper part of the limestone cliffs, on their plateaus, in the Şipotele area, and south of the Dumbrăveni Forest Reserve. The distribution area of this habitat is located upstream of the PROJECT boundary, in the northwestern part. | Unfavorable (inadequate both from in terms of habitat structure and functions, as well as in terms of prospects in future and surface) | Improvement of conservation status |
|  | 91F0 Mixed riparian forests with Quemus robur, Ulmus laevis, Fraxinus zcelsor or Fraxinus angustifolia, along large rivers - Ulmenion minoris | The habitat has not been identified on the surface of the site | NOT According to the Site management, habitat was not found during the field assessment of the 2014 season. | According to the Site Management Plan, the habitat was not found during the field assessment of the 2014 season. | Clarification of the conservation status within 3 years | Improving the state of conserve |
|  | 91I0*Euro-Siberian forest-steppe vegetation with Quercus spp. | 1355.90 ha | NOT The distribution area of this habitat is at a distance of more than 5.8 km from the T32 turbine and at a distance of 5.74 km from the limit studied by the PROJECT. | The habitat is present on large areas inside the Dumbrăveni Forest Reserve, and in the form of small clumps in the Crângu and Șipotele areas. The distribution area is located, northwest and west of the perimeter limit studied by the PROJECT. | Unfavourable (unfavourable — inadequate both in terms of habitat structure and functions and in terms of prospects in future, and of the surface.) | Improvement of conservation status |
|  | 91M0 Balkan Sky and Sky Forests durmast oak | 1277.5 ha | NOT The habitat distribution area is located at distances greater than 13 km from the boundary of the area studied by the PROJECT. | The habitat is spread in the site, in the form of fragments of different surfaces, on gentle slopes and plateaus, from the slums near Lake Baciului, in the forests of the | Unfavorable (unfavorable — inadequate both in terms of structure and functions | Improvement of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  |  | from the areas of Floriile - Profundata, southwest of Hațeg, northeast of Pădureni, and south of Șipotele. The distribution areas of this habitat are located north of the perimeter limit studied by the PROJECT, upstream of the proposed energy park. | habitat, as well as in terms of future prospects, and area. |  |
|  | 1355 Lutra lutra (River otter) | 10-50 individuals 50-75 ha habitat area | NOT The distribution area of the species is at distances greater than 34 km from the limit of the perimeter studied by the PROJECT | The species is present in the wetlands of the site in the area of streams with depths greater than 30 cm. The distribution area of the species is located in the northern part of the PROJECT site, upstream. The species was not observed in the area of the Cerchez and Magura streams that cross the studied site. | Favourable | Maintenance of conservation status |
|  | 1310 Miniopterus schreibersii (Long-winged bat)" | The size could not be appreciated Population | YES The results of the inventories carried out between April 2022 and March 2023 in the area of the wind farm proposed by PROIECT, showed that the species is present in the area of the site, the species was more active in the area during the migration period. The evolution of ultrasonic contacts on months indicates a maximum in the autumn migration period (August), and a high level in the spring migration period. According to the information presented in the Site Management Plan, the distribution area of the species is located approximately 3.3 km from the nearest turbine. | According to the information presented in the Site Management Plan, the species' distribution area is located in the southern part of the site, approximately 3.3 km from the nearest turbine. | Unfavourable (unfavourableinadequate in terms of population size, favourable in terms of habitat area and future prospects of the species.) | Improvement of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | 1302 Rhinolophus mehelyi | Rare presence on the site, it was not identified in the field at the stage of elaboration of the management plan of Site | NOT The range of the species is not known, the presence of the species in the site is not confirmed. The results of the inventories carried out between April 2022 and March 2023 by the consultant in the area of the wind farm proposed by PROIECT, did not highlight its presence | The range of the species is not known, the presence of the species in the site is not confirmed. The results of the inventories carried out between April 2022 and March 2023 by the consultant in the area of the wind farm proposed by PROIECT, did not highlight its presence | Unknown | Maintaining or improving conservation status |
|  | 1335 Spermophilus citellus (Popândău)" | 1000-5000 Individuals Habitat with an area of approx. 300-500 ha | YES The data obtained from the monitoring carried out between April 2022 and March 2023 in the area of the wind farm proposed by the PROJECT, indicate that the species is present in the area studied by the PROJECT, especially in the north, northeast and west of the site. During the monitoring period, a number of 39 individuals were observed on the site of the wind farm, individuals were also observed in the vicinity of the wind farm. the locations of turbines T23 (approximately 310 m from the turbine platform T23), T21 (approximately 500 m from the turbine platform) and T35 (approximately 300 m from the turbine platform). According to the distribution maps in the Management Plan, the species is found in the north-eastern and northern part of the site, at distances of more than 8 km from the PROJECT limit. | According to the distribution maps in the Management Plan, the species is found in the north-eastern and northern part of the site, upstream of the Project boundary. The species is present in the well-drained grassland/islaz habitats at the site level. The species was also reported in the area of the Project's perimeter. | Favourable | Maintenance of conservation status |
|  | 2635 Vormela peregusna (Spotted ferret) | 10-15 individuals, Surface habitat 150-200 ha | NOT | The distribution area is located at distances of more than 3 km from | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | At distances of more than 3 km from the western limit of the park's site. | western limit of the park's site. |  |  |
|  | 1188 Bombina bombina (Red-bellied Pond Owl) | 10000-50000 Individuals Habitat: area of 1130 ha | NOT. The species is present in various wetland habitats in the area of Lake Vederoasa and on the Baciului Valley, at distances greater than 5 km from the north-eastern limit and 11 km from the northern limit of the Project. The species was not observed in the area of the wind farm site. | The species can be found in ponds, canals, lake edges, ditches or even ephemeral ponds around the perimeter of the site. The species is present in various wetland habitats in the area of Lake Vederoasa and on the Baciului Valley, at distances of more than 5 km from the north-eastern limit and 11 km from the northern limit of the Project. The species was not observed in the area of the wind farm site. | Favourable | Maintenance of conservation status |
|  | 1993 Triturus dobrogicus (The crested newt dobrogean) | Unknown | NOT The species is present in the wetlands of the Vederoasa Lake area, at distances of more than 20 km from the limit of the wind farm site | The species is present in the humid habitats of the lake area Vederoasa, at distances of more than 20 km, upstream of the limit of the wind farm site. | Unknown Conservation status to be clarified within 3 years | Maintaining or improving conservation status |
|  | 5194 Four-lined Elaphe (Elaphe sauromates - currently accepted scientific name) (Four-striped snake) | Unknown | NOT The presence of the species has not been reported within the boundary of the studied site or in the immediate vicinity. According to the distribution maps in the Site Management Plan, the species is present in the southern and western part of the protected area, at distances of more than 3.5 km from the limit of the wind farm site. | The species prefers natural habitats with limestone walls, rocks, bushes, and can also be found in rare forests or even near homes. The species is present at the site level in border habitats of the Tălășman, Nistradin and Dumbrăveni forests. According to the distribution maps in the Site Management Plan, the species is present in the southern and western part of the protected area, at distances of more than 3.5 km, upstream, from the limit of the wind farm site. | Unknown Conservation status to be clarified within 3 years | Maintain or improve conservation status |
|  | 1220 Emys orbicularis (European pond turtle) | 1000-5000 Individuals Habitat: area of 1920 ha | NOT The distribution areas of the species according to the Conservation Plan Management are located at | The presence of the species was identified in the wetlands in the area of Vederoasa, Baciu, Plopeni and Negrești ponds | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | north and northwest of the wind farm site limit at distances greater than 10 km. | located upstream, north and northwest of the limit of the wind farm site at distances greater than 10 km. |  |  |
|  | 1219 Testudo graeca (Dobrogean land turtle) | 5000-10000 Individuals Habitat area of 16730 ha | NOT The distribution areas are common to those of the species Bombina bombina and are located at distances greater than 5 km from the north-eastern limit and 11 km from the northern limit of the Project. The species has not been observed in the area Wind farm site | The species is present in different habitats throughout the site, especially on sunny slopes with meadows, rocks, forests, but also in orchards or vineyards, near the localities of Vlahii, Aliman, Floriile, Profondata, Urluia, Şipotele, Zorile and Plopeni | Favourable | Maintenance of conservation status |
|  | 1217 Testudo hermanni (Land Turtle) | To date, the species has only been found on the site with only one specimen believed to have been introduced and there is no resident population. Only one specimen was registered on the site in 2014, and the information in the management plan suggests that it may also be a misidentification. The species has its natural spread in the south of Banat and the south-west of Oltenia. Investigations into the presence and conservation status of the species in ROSC10071 should be continued. Based on the information available, no conservation objectives are developed at site level ROSC10071 for this species |  |  | - | - |
|  | 1130 Aspius aspius (Avat) | Habitat: area of 227 ha | NOT The species was reported in the Vederoasa Pond within the site. The distribution area of the species is located at a distance of more than 34 km from the limit of the area studied by the PROJECT. | The species was reported in the Vederoasa Pond within the site. The distribution area of the species is located upstream, at a distance of more than 34 km from the limit of the area studied by the PROJECT. | Favorable in all respects (It will be verified within 2 years, from the date of the ANANP decision, due to the lack of data) | Maintenance of conservation status |
|  | 6963 Cobitis taenia (Squirrel) | Habitat area of 227 ha | NOT In the Site Management Plan, it is mentioned that the species has not been captured, but it is estimated that the species is present in the is/may be present in the Baciului Lake according to the management plan and the lake Eye-catching. The area studied by PROJECT is located at a | The habitats favorable to the species are located in Lake Vederoasa. The area studied through PROIECT is located at a distance of more than 34 km, downstream from Lake Baciului. | Favorable in all respects (It will be verified within 2 years, from the date of the ANANP decision, due to the lack of data) | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | distance of more than 34 km from Lake Baciului. |  |  |  |
|  | 1154 Misgurnus fossilis (Țipar) | Habitat: area of 227 ha | NOT In the Site Management Plan, it is mentioned that the species has not been caught, but it is estimated that the species is present in Vederoasa Lake. The area studied through the PROJECT is located at a distance of more than 34 km from the Baciului Lake. | The favorable habitats for the species are located in the Vederoasa lake The area studied by the PROJECT is located at a distance of more than 34 km, downstream of the Baciului lake. | Favorable in all respects (It will be verified within 2 years, from the date of the ANANP decision, due to the lack of data) | Maintenance of conservation status |
|  | 2522 Pelecus cultratus (Sabiță) | Habitat: area of 227 ha | NOT In the Site Management Plan, it is mentioned that the species has not been captured, but it is estimated that the species is present in the Vedereoasa Lake. The area studied by PROIECT is located at a distance of more than 34 km from Lake Baciului. | The habitats favorable to the species are located in Lake Vederoasa. The area studied through PROIECT is located at a distance of more than 34 km, downstream from Lake Baciului. | Favorable in all respects (It will be verified within 2 years, from the date of the ANANP decision, due to the lack of data) | Maintenance of conservation status |
|  | 1134 Rhodeus sericeus amarus (Boarță) | 84074 individuals Habitat area of about 227 ha | NOT The habitats favorable to the species are located at a distance of more than 34 km from the limit of the area studied by the PROJECT. | The species is present in areas with suitable habitat, sandy substrate, submerged vegetation. The area studied by the PROJECT is located at a distance of more than 34 km, downstream of areas with habitats favorable to the species | Favorable in all respects (It will be verified within 2 years, from the date of the ANANP decision, due to the lack of data) | Maintenance of conservation status |
|  | 5329 Romanogobio vladykovi | The species was not found during the substantiation studies for the management plan. The site-level conservation objective is to maintain or improve the conservation status of the species, depending on the results of investigations aimed at clarifying the presence and conservation status of the species (to be decided within 3 years, if it is necessary to maintain or improve the conservation status). |  |  |  |  |
|  | 4028 Catopta thrips (Sfrederitorul pelinului) | The species was not found during the substantiation studies for the management plan. The site-level conservation objective is to maintain or improve the conservation status of the species, depending on the results of investigations aimed at clarifying the presence and conservation status of the species (to be decided within 3 years, if it is necessary to maintain or improve the conservation status). |  |  |  |  |
|  | 1074 Eriogaster catax (Autumn hen) | The species was not found during the substantiation studies for the management plan. The site-level conservation objective is to maintain or improve the conservation status of the species, depending on the results of investigations aimed at clarifying the presence and conservation status of the species (to be decided within 3 years, if it is necessary to maintain or improve the conservation status). |  |  |  |  |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | 1060 Lycaena dispar (Red Swamp Butterfly) | Less than 50 individuals | NOT According to the distribution maps of species in the Conservation Plan Management, the closest areas with habitats favorable to the species were identified at distances greater than 10 km from the perimeter limit studied by the PROJECT. | The extinct Lycaea species has been reported at five points on the site. The most significant population was identified in the area of Lake Vederoasa, two colonies of over 20 individuals being identified. The next largest population was found in the Plopeni area, and two other reports come from the area of canals that cross agricultural land in the area Şipotele - Tufani, respectively Aliman - Floriile The habitats favorable to the species are located in the northern part, upstream of the perimeter limit studied by the PROJECT. | Unfavourable (unfavourable – inadequate in terms of population size, habitat area and future prospects of the species) | Improvement of conservation status |
|  | 1083 Lucanus cervus (Rădaşca) | Unknown | NOT The habitat distribution area favorable to the species 91I0* is located at a distance of more than 5.8 km from the T32 turbine and at a distance of 5.74 km from the limit studied by the PROJECT, and the distribution area of the habitat 91M0 is at a distance of more than 13 km from the limit of the area studied by the PROJECT. | The potential range of the species Lucanus cervus in the ROSCI0071 Dumbrăvei - Urluia Valley - Vederoasa Lake includes all forests of habitat types 91M0 Balkan forests of sky and gorun respectively 91I0* Euro-Siberian forest-steppe vegetation with Quercus spp. The adults of the species have been reported in four areas: in the Decebal Forest between Rariștea - Ion Corvin - Crângu localities, in the area of Floriile locality, in the Dumbrăveni Forest in the area of Furnica and Tufani localities, respectively south of Şipotele. Some reports are out of the primary habitat, cvercinee forests dominated by older trees, because the adults fly out of the larval habitat in neighboring forests or even in open areas at the edge of the forests. | Unknown (clarification within 3 years) | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  |  | The abundance of identified populations is different in each area where the species has been identified. Areas with habitats favorable to the species are located in the north, north-eastern part, upstream of the limit of the Project site. |  |  |
|  | 2253 Centaurea jankae | Uncertain presence | NOT According to the data presented in the Management Plan, the species was not reported on the site ROSCI0071 Dumbrăveni - Valea Urluia - Vederoasa Lake in 2014 It was not found in the site during the systematic search, during the flowering period of the species from the field evaluation stage in the summer of 2014. Also, the presence of the species in the area of the site has not been noted in the last decade communicated. pers. M. Petrescu 2014. | The presence of the species is uncertain | Unknown Clarification within 3 years | Improvement of conservation status |
|  | 6927 Himantoglossum jankae (Ouăle popii) | 83 Habitat specimens with an area of 4.9- 5 ha | NOT The distribution area of the species, according to the data presented in the site management plan, is located at distances greater than 10.35 km from the limit of the perimeter studied by the PROJECT. | The species has been reported in the Dumbrăveni Forest reserve: on the edge of the downy oak forests, on steppe meadows, sometimes scree, on glades interspersed in forests, entering from Tufani and Furnica. The distribution areas of the species are located upstream of the limit of the perimeter studied by the PROJECT, north, northeast of its limit. | Unfavorable - inadequate (in terms of population size and prospects species in the future is unfavourable – unsuitable and in terms of potential habitat area is favourable) | Maintaining or improving conservation status |
|  | 2125 Potentilla emilii-popii (Five-toed weed) | 134000-13500 individuals Habitat: area of about 15 ha | NOT The nearest areas where the species was reported, according to the site's distribution maps, are located at distances of about 5.9 km from the limit of the perimeter studied by the PROJECT. | The species is found in the following areas: Dumbrăveni Forest Reserve and its surroundings in the northern part, entering from Tufani, on limestone scree interspersed in the forest, in the southern part, | Unfavorable (favourable in terms of population size and habitat area and unfavourable — | Maintain or improve conservation status |
| Presence and herds/areas covered by species and habitats of community interest in the Project area – ROSAC0071 |  |  |  |  |  |  |
| ANPIC code and name | Name Scientific species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  |  | entering from the quarry, on meadows and scree in the north of the thematic trail, south of the Dumbrăveni Forest Reserve, on the scree at the southern end of the site, Şipotele - on rocky meadows south of the locality, Zorile - on the rocky cliffs south-east of the locality, Profondata - on rocky meadows, rocks and scree to the north and east of the locality, east of the Baciului lake, north of the southern basin of Lake Vederoasa, and Urluia - north of the locality - outside the site. The closest areas where the species was reported, according to the site's distribution maps, are located north, northeast and east of the perimeter boundary studied by the PROJECT. | inadequate from the point of view of the future prospects of the species) |  |
|  | 4067 Echium russicum (Snake's head) | Uncertain presence | NOT From the data presented in the Site Management Plan, it follows that the presence of the species on the site is uncertain. Also, the presence of the species in the area of the site has not been noted in the last decade - comunic. pers. M. Petrescu 2014 | The presence of the species in the site is uncertain | Unknown (clarification within 3 years) | Maintaining or improving conservation status |

Table 16 Presence and herds/areas covered by species and habitats of community interest in the Project area (Part II – ROSAC0157)

| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| ROSAC0157 (ROSCI0157) Hagieni ForestBend of the Valley | 62C0* Pontosartic steppes | 1866, 6 ha | NOT The PROJECT perimeter does not intersect with this habitat. The distribution area of this habitat is located at a distance of approximately 11.22 km from the limit of the area studied by the PROJECT. | Especially in the Bend Valley area. The habitat distribution area is located east of the PROJECT boundary, at a distance of more than 11 km | UnfavourableInadequate (in terms of structure and specific functions and in terms of future prospects) | Improvement of conservation status |
|  | 40C0* Pontosarmatian deciduous bushes | 93.09 ha | NOT The distribution area of this habitat is located at a distance of approximately 20.84 km from the limit of the area studied by the PROJECT. | The distribution of habitat at the site is isolated. The distribution area is concentrated in the central part of the site, east of Albesti locality, east of the Project limit, at a distance of more than 20 km. | UnfavorableInadequate | Improvement of conservation status |
|  | 6430 "Border communities with tall hygrophilous grasses from plains to mountain and alpine level | 11.6 ha | NOT The distribution area of this habitat is located at a distance of approximately 15.73 km from the limit of the area studied by the PROJECT. | The habitat distribution area is located in the eastern part of the Project, mainly in the Bend Valley area, at a distance of more than 15 km | UnfavorableInadequate | Improvement of conservation status |
|  | 3150 "Natural eutrophic lakes with vegetation of the Magnopotamino or Hydrocharition type | 119 ha | NOT The distribution area of this habitat is located at a distance of approximately 23.74 km from the limit of the area studied by the PROJECT. | The distribution of habitat 3150 is uneven within the protected natural area, being present especially in the western area of Lake Mangalia - the tail of the lake. The distribution area is located in the eastern part of the perimeter analyzed by the PROJECT, downstream, at a distance of approx. 23.76 km | UnfavorableInadequate | Improvement of conservation status |
|  | 91M0* Balkan-Pannonian sky and gorun forests | 1.6 ha | NOT The distribution area of this habitat is located at a distance of approximately 24.26 km from the limit of the area studied by the PROJECT." | At present, the distribution of habitat 91M0 is uneven within the protected natural area, being present only on the northern branch of the reserve | UnfavorableInadequate | Improvement of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  |  | relatively near the village of Albești, where unfortunately the presence of the habitat is difficult to detect, being surrounded and invaded by forest plantations of thuja, juniperus, acacia, willow and pine. The distribution area is east of the Project boundary, at a distance of more than 24 km |  |  |
|  | 91AA Pontosarmatic forest vegetation with downy oak | 80 ha | NOT The distribution area of this habitat is located at a distance of approximately 16.31 km from the limit of the area studied by the PROJECT. | At present, the distribution of habitat 91AA is uneven within the protected natural area, being present only on the northern branch of the reserve, relatively near the village of Albești, where unfortunately the presence of the habitat is difficult to detect, being surrounded and invaded by forest plantations of thuja, juniperus, acacia, willow and pine. The distribution area is east of the Project boundary, at a distance of more than 16 km. | UnfavorableInadequate | Improvement of conservation status |
|  | 1219 Testudo graeca (Dobrogean land turtle)" | 1000-5000 individuals Habitat area 3450 ha | NOT The presence of the species was identified at approximately 6.3 km from the limit of the area studied by the PROJECT. The nearest turbine, T7, is located about 14.15 km from the limit of the favorable habitat. | The species is present on the sunny slopes of the site, in forests and meadows near Coroana, Vâlcele, Cotul Văii, Vârtop, Albești, Hagieni localities. The distribution areas are located in the eastern part of the analyzed site, at distances of more than 14 km. | Favourable | Maintaining Condition |
|  | 1220 Emys orbicularis (Broascațestoasă europeană de baltă | 1000-5000 Individuals Habitat area 400 ha | NOT The nearest turbine, T9, is located about 23.51 km from the limit of the favorable habitat. As a result of the monitoring campaigns, there were no | The species lives in various types of humid habitats, from ponds, canals, lake edges, ponds and the like, surrounded by natural habitats. The distribution areas are located in the eastern part of the perimeter PROJECT- | Favourable | Maintaining Condition |

| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | Identify individuals of this species | at distances greater than 23 km. |  |  |
|  | 1188 Bowler hat (Red-bellied pond owl) | Unknown | NOT According to the Site Management Plan ROSAC0157 Hagieni Forest - Bend of the Valley, the distribution area of this habitat is located at a distance of approximately 25.75 km from the limits of the studied area. The nearest turbine, T9, is located about 26.18 km from the limit of the favorable habitat. | The species is not picky in terms of the choice of habitats reproduction, it can be found in ponds, canals, lake edges, ditches or even ephemeral ponds in the perimeter of the site. Distribution areas are located in the eastern part, at distances greater than 25 km from the Project boundary | Unknown | Maintain or improve |
|  | 1279 Elaphe quatorlineata (Șarpe cu patru dungi) | 100-500 individuals Habitat area has not been assessed | NOT The distribution area of this habitat is located at a distance of approximately 13.82 km from the boundaries of the studied area. The nearest turbine, T7, is located about 14.18 km from the limit of the favorable habitat. As a result of the monitoring campaigns, no individuals of this species were identified | The species prefers natural habitats with limestone walls, rocks, bushes, it can also be found in rare forests or even near homes. This kind of habitats are located in the eastern part of the Project, | Favourable | Maintaining Condition |
|  | 1083 Lucanus cervus (Root) | The presence of the species is unknown, this species has not been identified in Sit Habitat area 2-4 ha | NOT The boundary of the Natura 2000 site is located about 13.82 km from the PROJECT, but the presence or distribution of habitat in the site is not known. The nearest turbine, T7, is located about 14.15 km away to the limit of the favourable habitat. | The favorable habitat is mainly confined to the edge of the gorunet and other species of trees with falling leaves - ash, hornbeam, poplar and the like, on the access roads in and inside the forest, and is located in the eastern part of the Project at a distance of more than 14 km | UnfavorableInadequate | Improvement of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | 1060 Lycaena Dispar (Red Swamp Butterfly) | The species has not been identified in and | NOT The habitat favorable to the species is located about 13.82 km from the PROJECT, but the presence or distribution of the habitat in the site is not known. The nearest turbine, T7, is located about 14.15 km from the limit of the favorable habitat. | According to the current data, the presence of the species is mentioned only from a few localities in Constanta County. From the breeding data made by Ramboll, the species was not observed in the PROJECT area. | Unfavorable | Improvement of conservation status |
|  | 1078* Callimorpha quadripunctaria (Fluture vărgat) | Unknown | NOT The boundary of the Natura 2000 site is located about 13.82 km from the PROJECT, but the presence or distribution of habitat in the site is not known. The nearest turbine, T7, is located about 14.15 km away to the limit of the favourable habitat. | Unknown | Unfavorable | Improving the state of |
|  | 4027 Aritrura musculus | This species was not identified on the surface of the site during the substantiation studies for the elaboration of the management plan. It must therefore be removed from the standard form of the site |  |  |  |  |
|  | 4033 Erannis ankeraria | This species was not identified on the surface of the site during the substantiation studies for the elaboration of the management plan. It must therefore be removed from the standard form of the site |  |  |  |  |
|  | 2327 Himantoglossum caprinum (The Eggs of the Priests)" | Population not assessed Estimated habitat area: 150200 ha | NOT The species was not identified during the intensive field campaign of 2014 on the territory of the ROSCI0157 - Hagieni Cotul Văii. | The species was not identified during the intensive field campaign in year 2014 on the territory ROSCI0157 - Hagieni Cotul Văii., no information is known regarding their distribution on the site | Unknown | Maintaining Condition |
|  | 2125 Potentilla emilii-popii (Five-toed weed)" | Population not assessed Estimated habitat area:35-50 ha | NOT The species was not identified during the intensive field campaign of 2014 on the territory of the ROSCI0157 - Hagieni Cotul Văii. | The species was not identified during the intensive field campaign in year 2014 on the territory ROSCI0157 - Hagieni Cotul Văii., no information is known regarding their distribution on the site | Unknown | Maintain or improve |

| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | 2253 Centaurea jankae (Vineţele) | Population not assessed Estimated habitat area:1866.6 ha | NOT The species was not identified during the intensive field campaign of 2014 on the territory of the ROSCI0157 - Hagieni Cotul Văii. | The species was not identified during the intensive field campaign in year 2014 on the territory ROSCI0157 - Hagieni Cotul Văii., no information is known regarding their distribution on the site | Unfavorable-- inadequate | Improvement of conservation status |
|  | 4067 Echium russicum (Snake's head) | Population not assessed Estimated habitat area: 300 ha | NOT The population of the species in the protected natural area is not known. In the 3 investigated stations, less than 12 individuals were identified. Being a biannual species, it may not have appeared in bloom during the evaluation season since 2014 | The population of the species in the protected natural area is not known. In the 3 stationary points investigated, less than 12 individuals were identified. Being a biannual species, it may not have appeared in bloom during the breeding season. 2014 Assessment | Unfavorable-- inadequate | Improvement of conservation status |
|  | 1335 Spermophilus citellus (Popândău) | 1000-5000 Individuals Estimated habitat area: 150250 ha | YES The PROJECT does not overlap with the boundary of the site, but the species with protection status at European level from the Natura 2000 network, is present in the area of the analyzed energy park. This species has also been identified outside the site. The characteristic habitat of the species includes meadows, islets, glades and the like, their presence depending on the maintenance of low vegetation (e.g. by grazing) and on the tolerance of the locals. According to the Site Management Plan ROSAC0157 Hagieni Forest - Bend of the Valley, the distribution area of this habitat is located at a distance of approximately 15.78 km from the limits of the studied area. Nearest | At the site, the favourable habitat is located in the eastern part of the PROJECT, at greater distances than 15 km | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | turbine, T9, is located approximately 16.10 km from the limit of the favorable habitat |  |  |  |
|  | 2609 Mesocricetus newtoni (Dobrogean hamster) | 500-1000 individuals Estimated habitat area: 100250 ha | NOT According to the Site Management Plan ROSAC0157 Hagieni Forest - Bend of the Valley, the distribution area of this habitat is located at a distance of approximately 26.66 km from the limits of the studied area. The nearest turbine, T9, is located about 27.07 km from the limit of the favorable habitat. | At the site, favorable habitat is located in the central north and south part. The distribution areas are located in the eastern part of the Project, at distances of about 13.82 km | Favourable | Maintenance of conservation status |
|  | 1302 Rhinolophus mehelyi (Romanian Rhinolophe) | The species was not observed at the site during the substantiation studies for the management plan |  |  | Unfavorable | Improvement of conservation status |
|  | 1310 Miniopterus schreibersii (Long-winged Bat) | 100 specimens during migration | YES According to the Site Management Plan ROSAC0157 Hagieni Forest - Bend of the Valley, the distribution area of this habitat is located at a distance of approximately 15.52 km from the limits of the studied area. The nearest wind farm, T9, is located about 15.94 km away to the limit of the favourable habitat. This species is considered a medium-distance migratory species, but occasionally it can make long dispersal or migration flights, covering hundreds of kilometers. In most cases, summer and winter shelters are located at distances of 40-100km. | In the eastern part of the perimeter studied by the PROJECT. Individuals identified in the eastern part of the energy package. | Unfavorable - inadequate | Improvement of conservation status |

| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | Although the perimeter of the Project does not intersect with the boundaries of the site, following the monitoring campaigns carried out by the consultee, a number of 85 contacts within this species were identified, arranged exclusively at the level of the natural area, near the T2 wind power plant, which could be a possible migration area. The PROJECT does not overlap with the boundary of the site, but the species with protection status at the level of the Natura 2000 network, is present in the study area |  |  |  |
|  | 1304 Rhinolophus ferrumequinum (Liliacul mare cu potcoavă) | 100 individuals Habitat area: 30 ha | NOT The species requires a mosaic of habitats with varied structure, including deciduous forests, pastures, orchards, linked together by linear structures, rows of trees, hedges. Grazing contributes significantly to increasing the amount of food sources available for the species, through the presence of beetles According to the Site Management Plan ROSAC0157 Hagieni Forest - Bend of the Valley, the distribution area of favorable habitats is located at a distance of approximately 24.88 km from the limits of the studied area. The nearest turbine, T9, is located about 25.31 km away to the limit of the favourable habitat. | In the eastern part of the perimeter studied by the PROJECT Individuals identified in the eastern part of the energy park/ | Unfavorable - inadequate | Improvement of conservation status |
| Presence and herds/areas covered by species and habitats of community interest in ROSAC0157 |  |  |  |  |  |  |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | 1316 Myotis capaccinii (Long-legged bat)" | The species was not observed at the site during the substantiation studies for the management plan | NOT According to ANANP Note no. 1788/2022, the presence of the species is not confirmed on this site. As a result of the field activities specific to the elaboration of the Site Management Plan, only one specimen was identified with the help of the ultrasonic detector | In the south-eastern part of the perimeter studied by PROIECT, near Coroana commune | Unknown | No specific conservation objectives have been formulated for this species |
|  | 2633 Mustela eversmanii (Steppe ferret) | 10-50 individuals Habitat area: 150250 ha | NOT The PROJECT does not intersect with the boundary of the site. The nearest turbine, T7, is located about 14.15 km from the limit of the habitat favorable to the species. | Characteristic habitats of the species include agricultural habitats, abandoned grasslands, and forest edges. Favorable habitats from the site are located in the part of the site studied by the PROJECT | Favourable | Maintenance of conservation status |

Table 17 Presence and herds/areas covered by species and habitats of community interest in the Project area (Part III – ROSAC0166)

| Presence and herds/areas covered by species and habitats of community interest – ROSAC0166 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| ROSPA0166 | A255 Anthus campestris (Strip of field)' | 1-15 pairs | YES The perimeter does not intersect with the boundaries of the protected areas. Considering that resident and migratory birds have a high mobility, especially during feeding or dispersal and the results of the monitoring carried out between April 2022 and March 2023, a number of 10 individuals of this species were identified, in the area of the wind farms T17, T21, T23 and T7, T8 and T9, it is considered that the distribution area of this species is intersected by the analyzed plan. Also, according to Romania's reports based on Article 12 of Directive 2009/147/EEC and the Atlas of Bird Species of Community Interest in Romania, the distribution area of this species overlaps with the analyzed perimeter. The PROJECT does not overlap with the boundary of the site, but the species with protection status at the level of the Natura 2000 network, is present in the study area. | The analyzed perimeter is located south of the site boundary, downstream. In the area of the analyzed perimeter, the results of the monitoring carried out for this project confirmed the presence of the species in the eastern part of the site. | Unknown | Preservation maintenance or improvement |

| Presence and herds/areas covered by species and habitats of community interest – ROSAC0166 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A403 Buteo rufinus (Șorecar mare) | No. of individuals during the wintering period: 1 | YES The area studied by the PROJECT does not intersect with the boundaries of the analyzed site and does not overlap with the potential habitat of the species in the site. It lives in open, low-lying areas with clumps of trees and vegetation, often near water. As the distribution within the site is not known, it is considered that the favourable habitat of the species is represented by the entire site. The boundary of the site is approximately 6.1 km from the boundary of the Project. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 19 individuals were identified in the area of the analyzed perimeter in the eastern part of the park, with a higher concentration in the area of turbines T23 and T7, T8, T9 | The analyzed perimeter is located south of the site boundary, downstream of the site. Individuals identified in the eastern part of the energy park. | Unknown | Maintaining or improving conservation status |
|  | A082 Circus cyaneus (Erete vânăt) | No. of individuals in winter: 1 Habitat area: 42.27 ha | YES The area studied by the PROJECT does not intersect with the boundaries of the analyzed site and does not overlap with the potential habitat of the species in the site. The site is located approximately 6.1 km from the Project boundary. As a result of the monitoring campaigns, a number of 7 individuals of this species were identified in the eastern part of the park, with a higher concentration in the area of the wind farms | The analyzed perimeter is located south of the site boundary, downstream of the site. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSAC0166 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | T10, T18, T19, T22, T33 |  |  |  |
|  | A429 Dendrocopos syriacus (Woodpecker gardens) | 5–10 prechi | NOT The area studied by the PROJECT does not intersect with the boundaries of the analyzed site and does not overlap with the potential habitat of the species in the site. The site is located approximately 6.1 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, no individuals of this species were identified. | The analyzed perimeter is located south of the site boundary, downstream of the site. | Unknown | Maintaining or improving conservation status |
|  | A379 Emberiza hortulana (Garden Pressura) | 5 – 10 pairs | NOT The area studied by the PROJECT does not intersect with the boundaries of the analyzed site and does not overlap with the potential habitat of the species in the site. The site is located approximately 6.1 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 3 individuals of this species were identified, in the south-western part of the park at a distance of 1.83 km from the project limit | The analyzed perimeter is located south of the site boundary, downstream of the site. | Unknown | Maintaining or improving conservation status |
|  | A098 Falco columbarius (Winter Falcon) | 1 individual Habitat area: 136.62 ha | YES The area studied by the PROJECT does not intersect with the boundaries of the analyzed site and does not overlap with the potential habitat of the species in the Sit. The site is located approximately 6.1 km from the project boundary. | The analyzed perimeter is located south of the site boundary, downstream of the site. Individuals identified in the northern part of the Project perimeter. | Unknown | Maintain or improve conservation status |
| Presence and herds/areas covered by species and habitats of community interest – ROSAC0166 |  |  |  |  |  |  |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 2 individuals of this species were identified, in the area of the T22 and T33 turbines |  |  |  |
|  | A097 Falco vespertinus (Vânturel de seară) | 30– 40 pairs Habitat area: 136.62 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 6.1 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, no individuals of this species were identified. | The analyzed perimeter is located south of the site boundary, downstream of the site. | Favorable (Good) | Maintaining the state of conservation |
|  | A339 Lanius minor (Black-fronted Shrike) | 3 – 5 pairs Habitat area: 138 ha | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 6.1 km from the project boundary. Although the PROJECT does not overlap with the boundary of the site, but the species with protection status at the level of the Natura 2000 network, is present in the study area. Following the monitoring campaigns carried out for this energy park (20222023), a number of 3 individuals of this species were identified in the eastern and south-western part of the park, with a higher density in the area of wind farms T10, T12 | The analyzed perimeter is located south of the site boundary, downstream of the site. Individuals identified in the eastern part of the energy park site. | Unknown | Maintaining or improving conservation status |

Table 18 Presence and herds/areas covered by species and habitats of community interest in the Project area (Part IV – ROSPA0036)

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| ROSPA0036 | A402 Accipiter brevipes (Short-legged hawk) | Pairs nesting: 12 - 16 Habitat area, target value: at least 1789,57 | Not The area studied by the PROJECT does not intersect with the site or with the potential habitat of the species in the site. The favorable habitat for the species is located approximately 7.11 km from the project boundary. The biodiversity monitoring studies carried out by Ramboll between April 1, 2022 and March 30, 2023 did not confirm the presence of individuals of this species in the scope of the Project. | The site is located upstream of the Project perimeter, northwest of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A255 Anthus campestris (Field strip) | No. of nesting pairs: 1600 - 2000 | YES The area studied by the PROJECT does not cross the boundary of the site. The habitat favorable to the species is located approximately 7.11 km from the project. The biodiversity monitoring studies carried out for this energy park (20222023) confirmed the presence of individuals of this species in the perimeter of the Project, a number of 10 individuals were identified in the vicinity of the wind farms T17, T21, T23 and T7, T8 and T9 | The site is located upstream of the Project perimeter, northwest of the Project boundary. Individuals identified in the east of the energy park | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A089 Aquila pomarina (Lesser Spotted Eagle) | No. of individuals in migration: 100 - 300 Habitat area, target value: at least 1789,57 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat favorable to the species is located approximately 7.11 km from the project. The biodiversity monitoring studies carried out for this energy park have confirmed the presence of individuals of this species in the perimeter of the Project, a number of 19 individuals of this species, in the area of wind farms T17, T21, T23 and T7, T8 and T9 | The site is located upstream of the Project perimeter, northwest of the Project boundary. Individuals identified in the east of the energy park | Favourable | Maintaining Condition to preserve |
|  | A215 Bubo bubo (Buhă) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The favorable habitat for the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, no individuals of this species were identified. | The site is located upstream of the Project perimeter, northwest of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A403 Buteo rufinus (Șorecar mare) | No. of nesting pairs: 9 - 10 Habitat area, target value: at least 1789,57 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The favorable habitat for the species is located approximately 7.11 km from the project boundary. The biodiversity monitoring studies carried out for this energy park (20222023) confirmed the presence of | The site is located upstream of the Project perimeter, northwest of the Project boundary. Individuals identified in the eastern part of the energy park | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | individuals of this species in the perimeter of the Project, a number of 19 individuals were identified in the eastern part of the park, with a higher concentration in the area of turbines T23 and T7, T8, T9 |  |  |  |
|  | A243 Calandrella brachydactyla (Ciocârlie de stol) | - | NOT The area studied by the PROJECT does not cross the boundary of the site The habitat favorable to the species is located approximately 7.11 km from the project boundary. The biodiversity monitoring studies carried out for this energy park (20222023) did not confirm the presence of the species in the perimeter of the energy park. | The site is located upstream of the Project perimeter, northwest of the Project boundary. | Unknown (the presence of the species in the site is uncertain) | Maintaining or improving conservation status |
|  | A244 Caprimulgus europaeus (Caprimulg) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The favorable habitat for the species is located approximately 7.11 km from the project boundary. The biodiversity monitoring studies carried out for this energy park (20222023) confirmed the presence of the species, an individual of this species was identified in the area of the T7, T8 and T9 turbines | The site is located upstream of the Project perimeter, northwest of the Project boundary. Individuals identified in the perimeter studied by the PROJECT, in the south-eastern part. | Favourable | Maintaining Condition to preserve |
|  | A082 Circus cyaneus (Erete vânăt) | No. of individuals in migration: 90 – 100 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately | The site is located upstream of the Project perimeter, northwest of the Project boundary. Individuals identified in the perimeter studied by the PROJECT, in the eastern part. | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | 7,11 Miles face of project limit. In the map of biodiversity monitoring studies carried out for this energy park (2022-2023), a number of 7 individuals were identified in the eastern part of the park with a higher concentration in the area of turbines T10, T18, T19, T22, T33 (see Figure 13(a) |  |  |  |
|  | A083 Circus macrourus (Erete alb) | No. of individuals in migration: 60 – 70 | NOT The area studied by the PROJECT does not cross the boundary of the site. The favorable habitat for the species is located approximately 7.11 km from the project boundary. Following the monitoring campaigns carried out between 2022 and 2023 for this energy park, no individuals of this species were identified | The site is located upstream of the Project perimeter, northwest of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A084 Circus pygargus (Erete sur) | No. of individuals in migration: 150 – 200 | YES The area studied by the PROJECT does not cross the boundary of the site. The favorable habitat for the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 8 individuals were identified, in the eastern part of the park with a concentration of raised in the area of the T11 and T14 turbines | The site is located upstream of the Project perimeter, northwest of the Project boundary. Individuals identified in the eastern part of the analyzed site. | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A231 Coracias garrulus (Dumbrăveancă) | No. of nesting pairs: 70– 80 | YES The area studied by the PROJECT does not cross the boundary of the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 26 individuals were identified in the passage, in the southeastern part of the park with a higher concentration in the area of turbines T1, T4 T6, T7, T8, T9, T10, T12, T15, T20 | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the southeastern part of the energy park. | Favourable | Maintaining Condition to preserve |
|  | A238 Dendrocopos medius (Oak woodpecker) | - | NOT The area studied by the PROJECT does not intersect the boundary of the site nor the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. Following the monitoring campaigns, no individuals of this species were identified. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A429 Dendrocopos syriacus (Woodpecker gardens) | No. of nesting pairs: 30– 50 | NOT The area studied by the PROJECT does not intersect the boundary of the site nor the potential habitat of the species in the site. The habitat of the species is located approximately 7.20 km from the project boundary. Following the monitoring campaigns, no individuals of this species were identified. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A379 Emberiza hortulana (Garden Press) | No. of nesting pairs: 200– 220 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 3 individuals were identified in the passage, in the southwestern part of the park, at a distance of 1.83 km from the project limit | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A103 Falco peregrinus (Peregrine Falcon) | - | NOT The area studied by the PROJECT does not intersect the boundary of the site nor the potential habitat of the species in the site. The habitat of the species is located approximately 7.20 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified for this energy park. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A321 Ficedula Apricot necks (Muscar yellowed) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, there were no | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | Identify individuals of this species. |  |  |  |
|  | A092 Hieraaetus pennatus (Acvilă mică) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 2 individuals were identified in the central area of the park | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the central part of the energy park | Favourable | Maintaining its state of conservation |
|  | A338 Lanius collurio (Reddish shrub) | No. of nesting pairs: 400 – 500 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.20 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, a number of 58 individuals were identified, in the eastern and southwestern part of the park, with a higher density in the area of the T10, T12, T14, T17, T18, T19, T33 and T35 and T36 turbines respectively | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the eastern, south-western part of the analyzed energy park. | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A339 anius minor (Black-foreheaded Shrike) | No. of nesting pairs: 180– 200 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.20 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, a number of 3 individuals were identified, in the eastern and southwestern part of the park, with a higher density in the area of the T10, T12 turbines | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the eastern and southwestern part of the analyzed energy park | Favourable | Maintaining Condition to preserve |
|  | A246 Lullula arborea (Ciocârlie de pădure) | No. of nesting pairs: 200– 250 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.20 km from the project boundary. As part of the monitoring campaigns carried out, between 2022 and 2023, for this energy park, no individuals of this species were identified. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A242 Melanocorypha calender (Ciocârlie de Bărăgan) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out, in the period 2022-2023, for this energy park, | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the eastern part of the energy park. | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | a number of 55 individuals of this species have been identified in the central-eastern part of the park, with a higher density in the area of turbines T6, T8, T13, T14, T33 |  |  |  |
|  | A073 Milvus migrans (Black Jay) | No. of individuals in migration – 2 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.20 km from the project boundary. During the monitoring campaigns (2022-2023) no individuals of this species were identified. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A533 Oenanthe pleschanka (Pietrar negru) | No. of nesting pairs: 4– 5 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), no individuals of this species were identified | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |
|  | A072 Pernis apivorus (Viespar) | No. of nesting pairs: 6 – 9 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023) a number of 2 individuals of this species were identified, in | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the eastern part of the energy park | Favourable | Maintaining Condition to preserve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | the eastern part of the park, in the area of the T10, T12, T22 and T33 turbines |  |  |  |
|  | A234 Picus canus | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), no individuals of this species were identified | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Favourable | Maintaining Condition to preserve |
| Regularly migrating species Species associated with extensively used agricultural land |  |  |  |  |  |  |
| ROSPA0036 | A247 Alauda arvensis (Ciocârlie) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), a number of 550 individuals of this species were identified, mainly in the eastern part of the park, with a higher density in the area of the T8, T26, T28, T29 (see Figure 13a) | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified, in the eastern part of the energy park. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A366 Carduelis cannabina (Cânepar) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), no individuals of this species were identified | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A364 Carduelis carduelis (Sticlete) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns (2022-2023), a number of 100 individuals of this species were identified in the eastern part of the park, in the area of turbines T10, T14, T22, T27, T33. (see Figure 13a) | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified, in the eastern part of the energy park. | Unknown | maintaining or improving the state of conservation |
|  | A113 Coturnix coturnix (Prepeliţă) | No. of nesting pairs: 150– 200 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), a number of 4 individuals of this species were identified, outside the limits of the studied area, at distances greater than 0.58 km | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified, in the vicinity of the energy park. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A382 Emberiza melanocephala (Black-headed pressura) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), no individuals of this species were identified | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A096 Falco tinnunculus (Vânturel roşu) | No. of nesting pairs: 20 – 200 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), a number of 55 individuals of this species were identified, with a greater distribution in the eastern part of the future park with a higher density in the area of the T1, T4, T6, T8 turbines | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified, in the eastern part of the energy park. | Unknown | Maintaining or improving conservation status |
|  | A360 Fringilla montifringilla (Winter Finch | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns (2022-2023), the presence of this species in the area of the analyzed energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A244 Galerida cristata (Ciocârlan) | No. of nesting pairs: 100– 150 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified during the 2022-2023 monitoring campaigns, in the eastern part of the energy park (a number of 26 individuals of this species, with a higher distribution in the area of turbines T13, T17, T19, T33) (see Figure 13b) | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified, in the eastern and northern part of the energy park. | Favourable | Maintaining the state of conservation |
|  | A299 Hippo Jaunter (Yellow foliage) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. In the 2022-2023 monitoring campaigns, they did not individuals of this species have been identified. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | maintaining or improving the state of conservation |
|  | A438 Hippolainen pale (Gray foliage) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the 2022-2023 monitoring did not have individuals of this species have been identified. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A340 Lanius excubitor (Sfrâncioc mare) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the 2022-2023 monitoring did not have individuals of this species have been identified | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A341 Lanius senator (Red-headed shroud) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns, no individuals of this species were identified | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A230 Merops apiaster (Prigorie) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the 2022-2023 monitoring campaigns, a number of 147 individuals of this species were identified, with a predominant distribution in the southern and eastern part of the future wind farm but with an increased abundance in the area of the T32 wind farm, in the north of the energy park (see Figure 13b) | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the northern part of the energy park | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A383 Miliaria calandra | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns, no individuals of this species were identified | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A262 White Motacilla | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the 2020-2023 monitoring campaigns, a number of 56 individuals of this species were identified, with a predominant distribution in the eastern part of the future park but with an increased abundance in the area of the T21 turbine, in the north of the future energy park | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the northern part of the energy park. | Unknown | Maintaining or improving conservation status |
|  | A260 Motacilla flava | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the 2022-2023 monitoring campaigns, a number of 233 individuals of this species were identified, with a predominant distribution in | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the northern part of the energy park. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | eastern part of the future park with an increased abundance in the area T22 and T23 turbines |  |  |  |
|  | A435 Oenanthe isabellina (Eastern Stone) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the 2022-2023 monitoring campaigns, no individuals of this species were identified in the area of the energy park. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |
|  | A277 Oenanthe oenanthe (Pietrar sur) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the 2022-2023 monitoring campaigns, a number of 15 individuals of this species were identified, with a predominant distribution in the eastern part of the future energy park, especially in the area of the T8, T13, T17, T18 turbines | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals of this species have been identified in the eastern part of the parcului energetic | Unknown | Maintaining or improving conservation status |
|  | A311 Saxicola torquata (Mărăcinar negru) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the area of the energy park. |  |  |  |
|  | A210 Streptopelia Turtle (Turtledove) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, in the area of the energy park, a number of 5 individuals of this species were identified, with a predominant distribution in the north-western part of the future especially in the turbine area T32 (see Figure 13b) | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the northwestern part of the energy park. | Unknown | Maintaining or improving conservation status |
|  | A353 Sturnus roseus (alternative name Pastor roseus) (Lăcustar) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the area of the energy park | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A310 Sylvia borin (Silvie de zăvoi) | - | NOT The area studied by the PROJECT does not intersect the potential habitat | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the area of the energy park |  |  | conservation status |
|  | A309 Sylvia communis (Silvie commune) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the area of the energy park | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A286 Turdus iliacus (Sturzul viilor) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the area of the energy park | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A232 Upupa epops (Pupa) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. Habitat | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | The species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 51 individuals of this species were identified in the area of the energy park, with a predominant distribution in the eastern part of the future park, with an increased abundance in the area of the T11 and T24 turbines | Individuals identified in the eastern part. |  |  |
| Species associated with forest habitats |  |  |  |  |  |  |
| ROSPA0036 | A221 Asio otus (Ciuf de pădure) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the energy park area. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A256 Anthus trivialis (Fâsă de pădure) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | species in the area of the energy park. |  |  |  |
|  | A363 Carduelis chloris (Florinte) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 46 individuals of this species were identified in the area of the energy park, with a predominant distribution in the northeastern part of the future park, with an increased abundance in the area of the T21 turbine | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the northeastern part of the future energy park. | Unknown | Maintaining or improving conservation status |
|  | A365 Carduelis spinus (Scatiu) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the energy park area. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A373 Coccothraustes coccothraustes (Botgros) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | As part of the monitoring campaigns carried out between 2022 and 2023, no individuals of this species were identified in the energy park area. |  |  |  |
|  | A208 Columba palumbus (Porumbel gulated) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. Following the monitoring campaigns, a number of 2 individuals of this species were identified, at a distance of approximately 6.22 km from the project limits. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A212 Cuculus canorus (Cuc) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, in the area of the energy park, a number of 6 individuals of this species were identified, at a distance of approximately 6.82 km from the project boundaries | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A099 Falco subbuteo (Swallow Falcon) | No. of nesting pairs: 10– 20 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, in the area of the energy park, a number of 2 individuals of this species were identified, with a predominant distribution in the eastern and northern part of the future park, in the area of the T4, T22 and T33 turbines | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the eastern and northern part of the future energy park | Favourable | Maintaining or improving conservation status |
|  | A322 Ficedula hypoleuca (Muscar negru) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the energy park area was not confirmed | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A359 Fringilla coelebs (Cinderella) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As a result of the monitoring campaigns, a | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | number of 6 individuals of this species, at a distance of approximately 6.55 km from the project limits |  |  |  |
|  | A233 Jynx torquilla (Capîntors) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the energy park area was not confirmed | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |
|  | A271 Luscinia megarhynchos (Red Nightingale) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 4 individuals of this species were observed in the energy park area, at a distance of approximately 6.25 km from the project limits | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A319 Muscicapa striata (Muscar sur) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. |  |  |  |
|  | A337 Oriolus oriolus (Grangur) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A214 Otus scops (Ciuf) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A315 Phylloscopus collybita (Pitulice mică) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out in | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | Between 2022 and 2023, the presence of the species in the Energy Park area has not been confirmed. |  |  |  |
|  | A314 Phylloscopus sibilatrix (Sizzling Dwarfs) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A316 Phylloscopus trochilus (Whistling dwarfs) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A372 Pyrrhula pyrrhula (Mugurar) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out in | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | Between 2022 and 2023, the presence of the species in the Energy Park area has not been confirmed. |  |  |  |
|  | A318 Regulus ignicapillus (Auşel sprâncenat) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A317 Regulus regulus (Yellow-headed owl) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A361 Serinus serinus (Cănăraş) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out in | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | Between 2022 and 2023, the presence of the species in the Energy Park area has not been confirmed. |  |  |  |
|  | A351 Sturnus vulgaris (Graur) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 982 individuals of this species were identified, with a predominant distribution in the eastern and northern part of the future park, in the area T14, T18, T19, T27 and T33 turbines | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals observed in the eastern and northern part of the energy park site | Unknown | Maintaining or improving conservation status |
|  | A311 Sylvia atricapilla (Black-headed Silvie) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 3 individuals of this species were identified, with a predominant distribution in the eastern and southeastern parts of the future park, in the area of the T10, T11 turbines | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals observed in the eastern and southeastern part of the energy park site | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A308 Sylvia curruca (Silvie mică) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A283 Turdus merula (Mierlă) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, a number of 18 individuals of this species were identified, at a distance of approximately 7.12 km from the project limits | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A285 Turdus philomelos (Song Thrush) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, a number of 7 individuals of this species were identified, at a distance of approximately 6.61 km from the project limits | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A284 Turdus pilaris (Cocoşar) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 38 individuals of this species were identified, with a predominant distribution in the north-western part of the future park, in the area of the T32 turbine | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals identified in the northwestern part of the energy park. | Unknown | Maintaining or improving conservation status |
|  | A282 Turdus viscivorus (Sturz de vâsc) |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
| Species associated with reedbed habitats |  |  |  |  |  |  |
| ROSPA0036 | A298 Acrocephalus arundinaceus (Lăcar mare) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A296 Acrocephalus palustris (Swamp Lake) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A 295 Acrocephalus schoenobaenus (Small Laundry) |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Necunoscută | Menținerea sau îmbunătățirea stării de conservare |
|  | A297 Acrocephalus scirpaceus (Lăcar de stuf) | - | NU Zona studiată prin PROIECT nu intersectează habitatul potențial al speciei din sit. Habitatul speciei este situat la aproximativ 7,11 km față de limita proiectului. În cadrul campaniilor de monitorizare desfășurate în perioada 2022-2023, nu s-a confirmat prezența speciei în zona parcului energetic. | Situl este localizat in amonte de perimetrul Proiectului, la nord-est de limita Proiectului. | Necunoscută | Menținerea sau îmbunătățirea stării de conservare |

|  | - 295 Acrocephalus schoenobaenus (Small Laundry) |  | NOTE The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
| --- | --- | --- | --- | --- | --- | --- |
|  | A297 Acrocephalus scirpaceus (Reed Lake) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A291 Locustella Fluviatilis (Latch Weight) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A292 Locustella luscinioides (Reed Greluşel) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
| Species associated with riparian habitats |  |  |  |  |  |  |
| ROSPA0036 | A249 Riparia riparia (Shore Lake) |  | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, an individual of this species was identified in the southeastern part of the of the future park, in the area of the T11 turbine | The site is located upstream of the Unknown Maintenance or Project perimeter, northeast of the Project boundary improvement.state of Individual identified in the south-eastern part of the conservation |  |  |
| Species associated with rocky habitats and urban areas |  |  |  |  |  |  |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| ROSPA0036 | A086 Monticola saxatilis (Stone blackbird) |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A253 Phoenicurus ochruros (Mountain Codros) |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A252 Hirundo daurica (Red Swallow) | No. of nesting pairs: 8– 12 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located upstream of the Project perimeter, northeast of the Project boundary. | Unknown | Maintaining or improving conservation status |
| Presence and herds/areas covered by species and habitats of community interest – ROSPA0036 |  |  |  |  |  |  |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A251 Hirundo rustica (Swallow) |  | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 7.11 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, a number of 219 individuals of this species were identified, with a predominant distribution in the south-eastern part of the future park, in the area of the T6, T8, T10, T11 and T12 turbines (see Figure 12b) | The site is located upstream of the Project perimeter, northeast of the Project boundary. Individuals of this species have been observed in the southeastern part of the wind farm. | Unknown | Maintaining or improving conservation status |

Table no. 3 Presence and herds/areas covered by species and habitats of community interest in the Project area (part V – ROSPA0094)

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Scientific name species/habitat | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| ROSPA0094 | A402 Accipiter brevipes (Short-legged hawk) | No. of pairs Nesting: 3– 5 No. of individuals 30 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 20.88 km from the project boundary. As part of the monitoring campaigns carried out in the period 2022-2023, there was no | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | confirmed the presence of the species in the area of the energy park. |  |  |  |
|  | A229 Alcedo atthis (Blue Seagull) | No. of nesting pairs: 4 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 25.83 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A255 Anthus campestris (Field strip) | No. of nesting pairs: 150 – 200 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 10 individuals of this species were identified, in the area of turbines T17, T21, T23 and T7, T8 and T9 | The site is located east of the Project boundary. Individuals of this species have been observed in the eastern part of the proposed energy park. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A404 Aquila heliaca (Field Eagle) | No. of individuals: 2-4 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A089 Pomegranate Eagle (Lesser Spotted Eagle) | No. of nesting pairs: 1– 2 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 19 individuals of this species were identified, in the area of turbines T17, T21, T23 and T7, T8 and T9 | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A133 Burhinus oedicnemus (Bird of the field) | No. of nesting pairs: 4 – 8 Individuals: 30 – 80 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A403 Buteo rufinus (Great Buzzard) | No. of pairs Nesting: 2– 4 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 19 individuals were identified in the eastern part of the park, with a higher concentration in the area of the T23 turbines and T7, T8, T9 | The site is located east of the Project boundary. Individuals of this species have been observed in the eastern and northern part of the energy park | Favourable | Maintenance of conservation status |
|  | - A224 Caprimulgus europaeus (Caprimulg) | No. of nesting pairs: 10– 20 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 19.17 km from the project. During the monitoring campaigns, 1 individual of this species was identified in the area of the T7, T8 and T9 turbines | The site is located east of the Project boundary. 1 individual observed in the eastern part of the Project site. | Favourable | Maintenance of conservation status |
|  | A243 Calandrella brachydactyla (Flock woodpecker) | No. of nesting pairs: 30– 40 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.55 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A027 Egretta alba (Great Egret) | No. of individuals: 10–30 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 26.12 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A138 Charadrius alexandrinus (Saltwater Squirrel) | Size Resting/feeding population: 3 (min) – 5 (max) | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 19.10 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A196 Chlidonias hybridus (Chirighiţă with white cheek) | Size Resting/feeding population: 300 (min) | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A197 Chlidonias niger (Black Chirighiţă) | Size Resting/feeding population: 30 (min) | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 26.12 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A031 Ciconia ciconia (White stork) | Number of individuals in passage: 1300 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. As part of the monitoring campaigns carried out in the period 2022-2023, identified 93 individuals of this species, in the southeastern part of the park, with a higher concentration in the area of turbines T6 and T8, T10 | The site is located east of the Project boundary. Individuals observed in the southeastern part of the park. | Favourable | Maintenance of conservation status |
|  | A080 Circaetus gallicus (Şerpar) | No. of pairs Nesting: 1– 2 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, 1 individual from the | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | of this species in the area of turbines T7, T8, T9 |  |  |  |
|  | A081 Circus aeruginosus (Reed herring) | No. of pairs Nesting: 1 – 2 Number of individuals in the landscape: 20 – 50 Number of individuals wintering: 3 – 4 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, 37 individuals were identified in the eastern part of the park, with a higher concentration in the area of turbines T6, T8, T18, T19, T33 | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A082 Circus cyaneus | No. of individuals in the landscape: 80 – 100 No. of individuals wintering: 2 – 4 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out in the period 2022-2023, identified 7 individuals of this species, in the eastern and northern part of the park with a higher concentration in the area of turbines T10, T18, T19, T22, T33 | The site is located east of the Project boundary. Individuals of the species observed in the eastern and northern part of the energy park site. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A083 Circus macrourus (White Eret) | No. of individuals in Landscape: 30 – 40 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A084 Circus pygargus (Erete sur) | No. of individuals in Landscape: 56– 100 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. As part of the monitoring campaigns carried out in the period 2022-2023, identified 8 individuals of this species, in the eastern part of the park with a concentration of raised in the area of the T11 and T14 turbines | The site is located east of the Project boundary. Individuals observed in the eastern part of the analyzed energy park. | Favourable | Maintenance of conservation status |
|  | A231 Coracias garrulus (Dumbrăveancă) | No. of nesting pairs: 10– 20 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, a number of 26 individuals of this species were identified, in | The site is located east of the Project boundary. Individuals of the species observed in the southeastern part of the perimeter studied by the PROJECT. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | the south-eastern part of the park with a higher concentration in the area of turbines T1, T4 T6, T7, T8, T9, T10, T12, T15, T20 |  |  |  |
|  | A238 Medium Dendrocopos (Oak woodpecker) | No. of nesting pairs: 20– 30 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 23.81 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A429 Dendrocopos syriacus (Woodpecker gardens) | No. of nesting pairs: 13– 20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A236 Dryocopus martius (Black-woodpecker) | No. of pairs Nesting: 4– 5 Resident population: 1– 3 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 23.81 km from the project boundary. As part of the monitoring campaigns carried out in the period 2022-2023, there was no | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | confirmed the presence of the species in the area of the energy park. |  |  |  |
|  | A379 Emberiza Hortulana (Garden Press) | No. of nesting pairs: 30– 60 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 3 individuals of this species were identified in the south-western part of the park, at a distance of 1.83 km from the project limit | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A511 Falco cherrug (Saker Falcon) | No. of individuals: 3– 4 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A098 Falco columbarius (Winter Hawk) | No. of individuals: 1 – 2 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, a number of 2 individuals of this species were identified in the area of the T22 and T33 turbines | The site is located east of the Project boundary. Individuals observed in the northern part of the proposed energy park. | Favourable | Maintenance of conservation status |
|  | A103 Falco peregrinus (Falcon) | No. of individuals: 1 – 2 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A097 Falco vespertinus (Evening Hawk) | No. of nesting pairs: 13– 21 No. of individuals in Passage: 200– 300 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A321 Ficedula Albicollis (Collared muscar) | No. of individuals: 200 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 19.18 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A320 Ficedula parva (Small Flycatcher) | No. of individuals: 100 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 19.18 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A092 Hieraaetus pennatus (Little Eagle) | No. of pairs nesting: 1– 2 No. of individuals in Passage: 15–20 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. As a result of the monitoring campaigns, no individuals of this species were identified During the monitoring campaigns carried out in the period 2022-2023, a number of 2 | The site is located east of the Project boundary. Individuals observed in the central part of the perimeter analyzed by the PROJECT. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | individuals of this species, in the central area of the park |  |  |  |
|  | A131 Himantopus himantopus (Piciorong) | No. of pairs nesting: 1–5 No. of individuals in Passage: 15–20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 26.12 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A338 Lanius collurio (Reddish shrub) | No. of nesting pairs: 30– 50 No. of individuals in passage: 15 – 20 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 58 individuals of this species were identified in the eastern and southwestern part of the park, with a higher density in the area of the T10 turbines, T12, T14, T17, T18, T19, T33 and T35 and T36 respectively | The site is located east of the Project boundary. Individuals observed in the eastern and northern part of the future energy park. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A339 Lanius minor (Black-fronted Shrike) | No. of nesting pairs: 20– 40 No. of individuals in Passage: 15–20 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. During the monitoring campaigns carried out between 2022 and 2023, a number of 3 individuals of this species were identified in the eastern and southwestern part of the park, with a higher density in the area of the T10 turbines, T12 | The site is located east of the Project boundary. Individuals identified in the eastern part of the analyzed perimeter. | Favourable | Maintenance of conservation status |
|  | A176 Larus melanocephalus (Black-headed Gull) | No. of individuals: 5500– 6000 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.50 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A177 Larus minutus (Lesser Gull) | No. of individuals: 100–200 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.50 km from the project. As part of the monitoring campaigns carried out in the period 2022-2023, there was no | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | confirmed the presence of the species in the area of the energy park. |  |  |  |
|  | A246 Lullula arborea (Woodpecker) | Number of pairs: 80– 120 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A242 Melanocorypha calandra (Woodpecker Bărăgan) | No. of nesting pairs: 40– 60 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 55 individuals of this species were identified in the central-eastern part of the park, with a higher density in the area of turbines T6, T8, T13, T14, T33 | The site is located east of the Project boundary. Individuals of the species observed in the central and eastern part of the proposed energy park. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A073 Milvus migrans (Black Jay) | No. of individuals: 5–10 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A023 Nycticorax nycticorax (Night Heron) | No. of individuals: 10 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.50 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A533 Oenanthe pleschanka (Black Stone) | No. of pairs Nesters: 4– 6 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 23.76 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A020 Pelecanus crispus (Curly Pelican) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.50 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A072 Pernis apivorus (Viespar) | No. of nesting pairs: 1– 3 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located about 18.37 km from the project boundary. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 2 individuals of this species were identified in the eastern part of the park, in the area of turbines T10, T12, T22 and T33 | The site is located east of the Project boundary. Individuals of the species have been observed in the eastern and northern parts of the proposed energy park. | Favourable | Maintenance of conservation status |
|  | A393 Phalacrocorax pygmeus (Lesser Cormorant) | No. of individuals: 20–100 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.50 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A151 Philomachus pugnax (Bully) | No. of individuals: 100 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A234 Picus canus | No. of pairs Nesters: 20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As a result of the monitoring campaigns, no individuals of this species were identified During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the energy park area was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A119 Porcelain porcelain (Variegated crest) | No. of nesting pairs: 5– 10 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 26.12 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A132 Recurvirostra avosetta (Woodpecker) | No. of individuals: 10 – 20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.50 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
|  | A397 Tadorna ferruginea (Red Caliphate) | No. of individuals: 8 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.50 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Favourable | Maintenance of conservation status |
| Regularly migrating species Species associated with open land/agricultural land |  |  |  |  |  |  |
| ROSPA0094 | A247 Alauda arvensis (Woodpecker) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 550 individuals of this species were identified, | The site is located east of the Project boundary. Individuals of the species have been observed in the eastern part of the analyzed site. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | mainly in the eastern part of the park, with a higher density in the area of the T8, T26, T28, T29 turbines |  |  |  |
|  | A256 Anthus trivialis (Forest Strip) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A088 Buteo lagopus (Booted Buzzard) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A382 Emberiza melanocephala (Black-headed pressura) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out in the period 2022-2023, there was no | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | confirmed the presence of the species in the area of the energy park. |  |  |  |
|  | A299 Hippolais jaundice (Yellow foliage) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A438 Hippolais pale (Gray foliage) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A341 Lanius senator (Red-headed shroud) |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out in the period 2022-2023, there was no | The site is located east of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | confirmed the presence of the species in the area of the energy park. |  |  |  |
|  | A230 Merops apiaster (Prigorie) |  | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 147 individuals of this species were identified, with a predominant distribution in the southern and eastern part of the future park but with an increased abundance in the area of the T32 turbine, in the north of the park | The site is located east of the Project boundary. Individuals observed in the southern and eastern part of the perimeter studied by the PROJECT. | Unknown | Maintaining or improving conservation status |
|  | A383 Miliaria calandra |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A262 White Motacilla |  | YES | The site is located east of the Project boundary. | Unknown | Maintain or improve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 56 individuals of this species were identified, with a predominant distribution in the eastern part of the future park but with an increased abundance in the area of the T21 turbine, in the north of the future park | Individuals observed in the northern part of the PROJECT site. |  | conservation status |
|  | A277 Oenanthe oenanthe (Pietrar sur) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 15 individuals of this species were identified, with a predominant distribution in the eastern part of the future park, especially in the area of the T8 turbines, T13, T17, T18 | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A435 Saxicola torquata (Black Bramble) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A353 Spartus roseus (Grasshopper) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A210 Streptopelia Turtle (Turtledove) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 5 individuals of this species were identified, with a predominant distribution in the northwestern part of the future | The site is located east of the Project boundary. Individuals of the species have been observed in the northwestern part of the Project site. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | especially in the area of the T32 turbine |  |  |  |
|  | A310 Sylvia borin (Silvie de zăvoi) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A309 Sylvia Communis (Silvie commune) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A232 Upupa epops (Pupa) |  | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 51 individuals of this species were identified, with a predominant distribution in | The site is located east of the Project boundary. Individuals observed in the northern part of the future energy park. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | the northern part of the future park, with an increased abundance in the area of the T11 and T24 turbines |  |  |  |
|  | A096 Falco tinnunculus | No. of pairs: 40 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 55 individuals of this species were identified, with a greater distribution in the eastern part of the future park with a higher density in the area of the T1, T4, T6, T8 turbines | The site is located east of the Project boundary. Individuals observed in the eastern part of the future energy park. | Unknown | Maintaining or improving conservation status |
|  | A041 Anser albifrons | No. of pairs: 3400 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A113 Coturnix coturnix (Quail) | No. of pairs: 300 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. Following the monitoring campaigns, a number of 4 individuals of this species were identified, outside the limits of the studied area, at distances greater than 0.58 km | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A244 Galerida cristata (Ciocârlan) | Number of pairs: 120– 140 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 26 individuals of this species were identified, with a greater distribution in the area of turbines T13, T17, T19, T33 | The site is located east of the Project boundary. Individuals were observed in the northern and eastern part of the analyzed placement. | Unknown | Maintaining or improving conservation status |
| Regularly migrating species Species associated with riparian and littoral habitats |  |  |  |  |  |  |
| ROSPA0094 | A168 Actitis hypoleucus (Mountain Whistle) | No. of individuals in passage: 10 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out in the period 2022-2023, there was no | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | confirmed the presence of the species in the area of the energy park. |  |  |  |
|  | A136 Charadrius dubius (Small Prundăraş) | No. of individuals in passage: 20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A165 Tringa ochropus (Green-legged whistler) | No. of individuals in winter: 30 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A153 Gallinago gallinago (Common snipe) | No. of individuals in winter: 44 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. Following the monitoring campaigns, an individual of this species was identified in the area of the T10 turbine | The site is located east of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A162 Tringa totanus (Red-legged Whistler) | No. of individuals in winter: 40 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A162 Vanellus vanellus (Nagâţ) | No. of individuals in winter: 20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A271 Luscinia megarhynchos (Red Nightingale) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As a result of the monitoring campaigns, a number of 4 individuals of this species were identified, at a distance of approximately 6.25 km from the project limits | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A249 Riparia riparia (Shore Squirrel) | - | YES | The site is located east of the Project boundary. | Unknown | Maintain or improve |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, an individual of this species was identified in the southeastern part of the future park, in the area of T11 turbines |  |  | conservation status |
| Regularly migrating species Species associated with forest habitats |  |  |  |  |  |  |
| ROSPA0094 | A221 Asio otus (Forest owl) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A373 Coccothraustes coccothraustes (Botgros) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species on the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A361 Serinus serinus (Cănăraş) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
| ROSPA0094 | A311 Sylvia atricapilla (Black-headed Silvie) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out in the period 2022-2023, 3 individuals of this species have been identified, with a predominant distribution in the southeastern part of the future park, in the area of turbines T10, T11 | The site is located east of the Project boundary. Individuals observed in the southeastern part of the wind energy park. | Unknown | Maintaining or improving conservation status |
|  | A207 Columba oenas (Hollow Pigeon) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| Regularly migrating species Species associated with urban habitats |  |  |  |  |  |  |
| ROSPA0094 | A273 Phoenicurus ochruros (Mountain Codros) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A251 Hirundo rustica (Swallow) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 219 individuals of this species were identified, with a predominant distribution in the south-eastern part of the future park, in the area of the T6, T8 turbines, T10, T11 and T12 | The site is located east of the Project boundary. Individuals observed in the southeastern part of the analyzed site. | Unknown | Maintaining or improving conservation status |
| Regularly migrating species Species associated with mixed terrestrial habitats, forests and open lands |  |  |  |  |  |  |
| ROSPA0094 | A086 Accipiter nisus (Bird Hawk) | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located at | The site is located east of the Project boundary. Individuals observed in the southeastern part of the analyzed site. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | approximately 18.37 km compared to the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 5 of this species were identified, with a predominant distribution in the southern and eastern part of the future park, in the area T22, T23, T33, T40 and T41 turbines |  |  |  |
|  | A087 Buteo buteo (Common Buzzard) | No. of individuals in passage: 200 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 162 individuals of this species were identified, with a predominant distribution in the south-eastern part of the future park, with an increased abundance in the area of the T6, T8, T9 turbines, T10 | The site is located east of the Project boundary. Individuals observed in the southeastern part of the analyzed site. | Unknown | Maintaining or improving conservation status |
|  | A212 Cuculus canorus (Cuckoo) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As a result of the monitoring campaigns, a number of 6 individuals of this species were identified. | The site is located east of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | species, at a distance of approximately 6.82 km from the project boundaries |  |  |  |
|  | A099 Subbuteo Falcon (Swallow Falcon) | No. of pairs: 25 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As a result of the monitoring campaigns, a number of 2 individuals of this species were identified, with a predominant distribution in the eastern part of the future park, in the area of the T4, T22 and T33 turbines | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A223 Jynx torquilla (Captorors) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A337 Oriolus oriolus (Grangur) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A214 Otus scops (Ciuf) |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
| ROSPA0094 | A208 Columba palumbus (Collared pigeon) |  | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As a result of the monitoring campaigns, a number of 2 individuals of this species were identified, at a distance of approximately 6.22 km from the project limits | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
| Regularly migrating species Species associated with reedbed habitats |  |  |  |  |  |  |
| ROSPA0094 | A260 Motacilla flava | - | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, a number of 233 individuals of this species were identified, with a predominant distribution in the eastern part of the future park with an increased abundance in the area of the T22 and T23 turbines. | The site is located east of the Project boundary. Individuals of this species were observed in the eastern part of the analyzed site | Unknown | Maintaining or improving conservation status |
|  | A123 Gallinula chloropus (Pond hen) | No. of individuals in passage: 44 No. of pairs: 15 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A118 Rallus aquaticus (Pond Squirrel) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. | The site is located east of the Project boundary. | Unknown | Maintain or improve conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. |  |  |  |
|  | A028 Ardea cinerea (Grey heron) | No. of individuals: 20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. |  | Unknown | Maintaining or improving conservation status |
| Regularly migrating species Species associated with open aquatic habitats |  |  |  |  |  |  |
| ROSPA0094 | A054 Anas acuta (Spear duck) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A056 Anas clypeata (Spoon duck) | No. of individuals in passage: 340 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A052 Anas crecca (Little duck) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A050 Anas penelope (Whistling Duck) | No. of individuals in winter: 200 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A053 Anas platyrhynchos (Great Duck) | No. of individuals in winter: 400 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A055 Anas querquedula (Crumbling Duck) | No. of individuals in passage: 200 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A051 Anas strepera (Variegated duck) | No. of individuals in passage: 20 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A041 Anser albifrons | No. of individuals in Winter: 3400 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A043 Anser anser (Summer Goose) | No. of individuals in winter: 80 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A059 Aythya ferina (Chestnut-headed Duck) | No. of individuals in winter: 120 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A061 Aythya fuligula (Spotted Duck) | No. of individuals in winter: 112 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A067 Bucephala clangula (St. John's wort duck) | - | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A125 Fulica atra (Lişiţă) | No. of individuals in winter: 500 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A198 Chlidonias leucopterus (White-winged Chirighiţă) | No. of individuals in winter: 30 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A036 Cygnus olor (Summer Swan) | No. of individuals in winter: 32 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A004 Tachybaptus raficollis (Little Horn) | No. of individuals in passage: 30 No. of individuals in winter: 17 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  | A048 Tadorna tadorna (White Caliper) | No. of pairs: 12 No. of individuals in passage: 66 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A459 Larus cachinnans (Mediterranean Seagull) | No. of individuals in winter: 1800 | YES The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out between 2022 and 2023, a number of 88 individuals of this species, with a predominant distribution in the eastern part of the future park with an increased abundance in the area of the T6 turbines, T8, T18, T19, T22 and T33 | The site is located east of the Project boundary. Individuals of this species have been identified in the eastern part of the analyzed site. | Unknown | Maintaining or improving conservation status |
|  | A179 Larus ridibundus (Laughing Seagull) | No. of individuals in passage: 1000 No. of individuals in winter: 2400 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. As part of the monitoring campaigns carried out in the period 2022-2023, there was no | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | confirmed the presence of the species in the area of the energy park. |  |  |  |
|  | A182 Larus canus (Grey Seagull) | No. of individuals in winter: 120 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A058 Netta rufina (Owl duck) | No. of individuals in passage: 8 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A017 Phalacrocorax carbo (Great Cormorant) | No. of individuals in winter: 83 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. | The site is located east of the Project boundary. | Unknown | Maintain or improve conservation status |
| Presence and herds/areas covered by species and habitats of community interest – ROSPA0094 |  |  |  |  |  |  |
| ANPIC code and name | Name Species/Habitat Science | Area/population | Location vs. PROJECT (intersected Yes/ No) - Distance from the PROJECT | Geographical direction and altitudinal difference | Conservation status | Conservation objectives (improvement/maintenance of conservation status) |
|  |  |  | During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. |  |  |  |
|  | A008 Podiceps nigricollis (Black-necked Corcodel) | No. of individuals in winter: 94 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |
|  | A006 Podiceps grisegena (Red-necked Woodpecker) | No. of individuals in winter: 9 | NOT The area studied by the PROJECT does not intersect the potential habitat of the species in the site. The habitat of the species is located approximately 18.37 km from the project. During the monitoring campaigns carried out between 2022 and 2023, the presence of the species in the area of the energy park was not confirmed. | The site is located east of the Project boundary. | Unknown | Maintaining or improving conservation status |

  - Clarifications regarding the direct link of the proposed project with or if necessary for the management of the conservation of the protected natural area of community interest

The project is not related to protected natural areas and is not necessary in the process of conservation of species and habitats of community interest within protected natural areas.

  - Estimation of the potential impact of the project on the species and habitats of the protected natural area of community interest

    - Identifying and estimating the impact

In order to estimate the impact, the main types of interventions, identifiable at this time, were taken into account for the categories of works involved in the quality of groundwater groundwater confined in the company's subsoil and groundwater pumping and treatment boreholes. Thus, the following were taken into account:

- Mortality risk of individuals, respectively reduction of population numbers: victims injured by machinery, destruction of nests, collision with machinery

- Risk of loss of natural habitats or habitats important for species as a result of permanent works (throughout the life of the project)

- Risk of alteration of natural habitats or habitats important for species as a result of temporary works

- The risk of disturbance of the normal activity of the species (nesting, feeding, resting) as a result of the execution of the works, especially caused by noise generation and human presence.

- Risk of habitat fragmentation.

The estimation and motivation of the potential impact of the project on the species and habitats in ANPIC was made by filling in columns 1-19 of the table in Annex no. 3C (Impact Assessment Table) of the Methodological Guide on the Appropriate Assessment of the Potential Effects of Plans or Projects on Protected Natural Areas of Community Interest approved by Order 1682/2023. That table can be found in the annexes to this presentation memorandum.

From the evaluation carried out, the following summary elements were extracted and presented below:

- identifying all the interventions of the proposed plan, the effects generated by them and the forms of impact generated on ANPIC potentially affected;

- the list of habitats, species and their parameters potentially affected by the implementation of the project/plan, including all situations in which insignificant, significant and/or uncertain adverse impacts are identified;

- description and analysis of the cumulative impact generated by the analyzed PROJECT together with other Plans/Projects that affect the parameters of the conservation objectives of the potentially affected species and habitats in ANPIC.

The following data were used to assess the potential impact of the project:

- The Management Plan of the Protected Natural Areas ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSPA0036 Dumbrăveni, ROSPA0001 Aliman - Adamclisi, ROSPA0007 Vederoasa Pond, 2,361 Dumbrăveni Forest, 2,350 Limestone Walls - Deleni Commune, 2,351 Aliman Fossil Site and IV.30 Vederoasa Lake, from 29.07.2016;

- Management Plan for the protected natural areas ROSCI0157 Hagieni Forest - Bend of the Valley, ROSPA0094 Hagieni Forest, 2,360 Hagieni Forest of 18.07.2016, approved by Order 1480/2016;

- Standard site forms ROSCI0071/ROSAC0071, ROSCI0157/ROSAC0157, ROSPA0036, ROSPA0166, ROSPA0094;

- ANANP notes and decisions for establishing the conservation objectives of species and habitats: o ANANP Decision no. 414/03.08.2022 on the approval of the Methodological Norms on the Implementation of the conservation objectives in Annex Ia Order no. 1557/2016 regarding the approval of the Management Plan and the Regulation of the protected natural areas ROSC10071 Dumbrăveni, Valea Urluia- Vederoasa Lake and ROSPA0036 Dumbrăveni;

o ANANP NOTE no.1788/24.03.2022 on the approval of the minimum set of special measures for the protection and conservation of biological diversity, as well as the conservation of natural habitats, wild flora and fauna, population safety and investments in the Plopeni-Chirnogeni ROSPA0166;

o ANANP Decision no. 197/26.06.2020 for the amendment of Annex 1 and Annex 2 to Decision no. 118 of 08.05.2020 on the approval of the Methodological Norms on the Implementation of the conservation objectives in the Annex to the Order of the Minister of Environment and Forests no. 1480/2016 on the approval of the Management Plan and the Regulation of the protected natural areas ROSC10157 Hagieni Forest-Cotul Văii, ROSPA0094 Hagieni Forest and 2360 Hagieni Forest.

- Data on the distribution of species and habitats from Romania's reports Romania's reports made on the basis of Article 12 of Directive 2009/147/EEC and on the basis of Article 17 of Directive 92/43/EEC;

- Field data collected by the consultant from the location area of the proposed energy park, period 01 April 2022 - 30 March 2023;

- Results of the appropriate evaluation carried out in 2011.

The identification of cause-effect-impact relationships is presented in the following table and in the diagram presented in the following figure

Table 19 Identification of cause-effect-impact relationships (Table No. 4 Identification of cause-effect-impact relationships, Annex No. 3A)

| Types of interventions proposed by the project in construction stages/ Operation/Decommissioning PPS objectives | Effects |  | Threshold values considered to identify the impact (where applicable) |  | Impacts | Quantifying impacts | ANPIC potentially affected |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Construction stage |  |  |  |  |  |  |  |
| - Arrangement of the site organization - Arrangement of access roads - Realization of technology platforms and foundations - Installation of wind power plants - Making electrical connections and laying underground cables between wind generator sets and transformer stations - Construction of transformer stations - Land restoration works at the completion of the construction | Removal of topsoil and vegetation |  | more than 1% of the site area or natural habitats/habitats important for species |  | Alteration of habitats in the work area/loss of habitats used for feeding | 0.9 ha temporarily occupied by site organization + area temporarily occupied by the trenches for cable installation 309.24 ha areas permanently occupied by the construction elements of the energy park | The study area does not intersect with the boundaries of the protected natural areas of the Natura 2000 Network, but species with protection status at European and national level from the neighboring protected areas were observed during field studies in the study area (e.g. birds, amphibians, reptiles, small mammals). A project must not intersect a Natura 2000 site to exert a potential impact, given that resident and migratory species have high mobility, especially during feeding or dispersal. The analyzed perimeter is a potential feeding habitat for species in protected areas. Thus, species from: - ROSPA0166 - ROSCI0071. |
|  | Temporary/permanent occupation of land areas |  |  |  |  |  |  |
|  |  |  |  |  | Habitat fragmentation | - | The analyzed perimeter does not intersect with the boundaries of protected natural areas or protected habitats. |
|  | Increased noise levels |  | ≥50 dB |  | Disturbance of species activity and removal | Not can be quantified | During the execution period, the noise level can reach up to 80 dB, in the area of the work front. In the literature (see e.g. Foreman et al., 1998) there are documented values of the noise level of where there may be a decline in grassland (>48 dB) or forest birds (>42 dB). The analyzed perimeter does not intersect with the boundaries of the protected natural areas, but species with protection status at European and National level from the neighboring protected areas were observed during the field studies in the study area (e.g. birds, amphibians, reptiles, small mammals). Thus, species from: - ROSPA0166; - ROSCI0071. |
|  | Collision of wildlife with vehicle traffic on access roads/machinery in the area of the work front |  | ≥2 individuals |  | Reduction of population numbers | Depend of species, their abundance, natural conditions |  |
|  | Pollutant emissions in atmosphere (operation of vehicles and machinery) |  | SO2 - Lower evaluation threshold: 12 μg/m3 (winter) - Upper evaluation threshold: 8 μg/m3 (winter) NO2 (annual average) - Lower evaluation threshold: 24 μg/m3 - Upper evaluation threshold 19.5 μg/m3 PM10 (annual average) - -Lower evaluation threshold: 28 μg/m3 - Upper evaluation threshold: 20 μg/m3 CO (8-hour average) - Lower endpoint: 7 mg/m3 Upper threshold of assessment: 5 mg/m3 |  | Deposition of pollutants that are transferred from the atmosphere to soil, vegetation, water – alteration of habitats | It cannot be quantified | The analyzed perimeter does not intersect with the boundaries of protected natural areas or protected natural habitats. |
|  | Accidental leakage of petroleum products from machinery-penetration of pollutants into the soil |  | more than 1% of the site area or natural habitats/habitats important for species |  | Alteration of habitats in the work area | It cannot be quantified |  |
|  | Introduction of invasive species |  | more than 1% of the site area or natural habitats/habitats important for species |  | Alteration of habitats in the work area | It cannot be quantified | It is not possible to affect the floristic composition of natural habitats by spreading invasive species; - anemocorous, because the distance between the protected natural areas and the area studied by the PROJECT is large enough, between them there are localities and agricultural lands. Also, during the execution period, access roads from hydrocorous protected areas will not be used (the only stream that crosses the site and that crosses the perimeter studied by the PROJECT is the Cerchez Creek, they are located downstream of the sites) |
| Operation Stage |  |  |  |  |  |  |  |
| - Proper operation (electricity production activity) - Current maintenance and repairs | Increased noise levels |  | ≥50 dB |  | Disturbance of species activity and removal | ≥50 dB | The analyzed perimeter does not intersect with the boundaries of the protected natural areas The estimated noise level in the immediate vicinity of the turbines is 50-55 dB, and at the limit of the perimeter of the park 40-45 dB according to the noise study carried out for this energy park. Potentially affects bat species in neighboring natural areas. Thus, species from: - ROSPA0166 - ROSCI0071 - ROSCI0157 |
|  | Night lighting |  | - |  | Disruption of species activity and removal, which can also lead to habitat loss by creating the barrier effect | It cannot be quantified | The analyzed perimeter does not intersect with the boundaries of the protected natural areas, but species with protection status at European and national level from the neighboring protected areas were observed during the field studies in the study area (e.g. birds, bats). Thus, species from: - ROSCI0071 - ROSCI0157 |
|  | Waste generation |  | - |  | Habitat damage Disruption of species activity | - | The analyzed perimeter does not intersect with the boundaries of the protected natural areas. Protected natural areas and protected species will not be affected. The activity of producing electricity does not involve waste generation. The amount of waste generated during the repair and maintenance period is reduced, all the waste generated will be eliminated from the place of generation through authorized companies. |
|  | Collision of wildlife with vehicle traffic on access roads/machinery in the area of the repair front Collision with turbine blades |  | ≥2 individuals |  | Reduction of population numbers | It depends on the species, their abundance, natural conditions | The analyzed perimeter does not intersect with the boundaries of the protected natural areas, but species with protection status at European and National level from the neighboring protected areas were observed during the field studies in the study area (e.g. birds, amphibians, reptiles, small mammals). Thus, species from: - ROSPA0166 - ROSCI0071 |
|  | Sudden change in pressure in the range of the turbine blades, but also below it (approx. 5 m) - barotrauma |  | ≥2 individuals |  | Reduction of bat populations | It cannot be quantified | The analyzed perimeter does not intersect with the boundaries of the protected natural areas. In the area of the PROJECT site, however, the field studies carried out in the period 2022-2023 confirmed the presence of bats with protection status at European and National level, species for which the neighboring protected areas were also designated. Thus, species from: - ROSCI0071 - ROSCI0157 - BG000569 - BG000570 |
|  | Introduction of invasive species |  | more than 1% of the site area or natural habitats/habitats important for species |  | Alteration of habitats in the work area | It cannot be quantified | It is not possible to affect the floristic composition of natural habitats by spreading invasive species; - anemocorous, because the distance between the protected natural areas and the area studied by the PROJECT is large enough, between them there are localities and agricultural lands. Also, during the operation period, access roads from protected areas will not be used. - by hydrochlorine (the only stream that crosses the site and that crosses the perimeter studied by the PROJECT is the Cerchez Creek, they are located downstream of the sites). |
| Decommissioning stage (Interventions and activities are similar to those in the construction phase) |  |  |  |  |  |  |  |
| - Arrangement of the site organization - Dismantling of wind power plants - Decommissioning of technology platforms and foundations Decommissioning of electrical connections and removal of underground cables - Decommissioning of transformer stations - Decommissioning of exploitation roads - Land restoration works at the completion of the construction |  | Removal of topsoil and Vegetation |  | more than 1% of the site area or natural habitats/habitats important for species | Alteration of habitats in the work area/loss of habitats | 0.9 ha temporarily occupied by site organization + area temporarily occupied by trenches for cable installation | The study area does not intersect with the boundaries of the protected natural areas of the Natura 2000 Network, but species with protection status at European and national level from the neighboring protected areas were observed during field studies in the study area (e.g. birds, amphibians, reptiles, small mammals). A project must not intersect a Natura 2000 site to exert a potential impact, given that resident and migratory species have high mobility, especially during feeding or dispersal. The analyzed perimeter is a potential feeding habitat for species in protected areas. Thus, species from: - ROSPA0166 - ROSCI0071. |
|  |  | Temporary occupation of land areas |  |  |  |  |  |
|  |  |  |  |  | Habitat fragmentation | - | The analyzed perimeter does not intersect with the boundaries of protected natural areas or protected habitats. |
|  |  | Increased noise levels |  | ≥50 dB | Disturbance of species activity and removal | It cannot be quantified | The analyzed perimeter does not intersect with the boundaries of the protected natural areas, but species with protection status at European and National level from the neighboring protected areas were observed during the field studies in the study area (e.g. birds, amphibians, reptiles, small mammals). Thus, species from: - ROSPA0166 - ROSCI0071. |
|  |  | Collision of wildlife with vehicle traffic on access roads/machinery in the area of the work front |  | ≥2 individuals | Reduction of population numbers | It depends on the species, their abundance, natural conditions | The analyzed perimeter does not intersect with the boundaries of the protected natural areas, but species with protection status at European and National level from the neighboring protected areas were observed during the field studies in the study area (e.g. birds, amphibians, reptiles, small mammals). Thus, species from: - ROSPA0166 - ROSCI0071. |
|  |  | Emissions of pollutants into the atmosphere ( vehicles and machinery) |  | SO2 - Lower evaluation threshold: 12 μg/m3 (winter) - Upper evaluation threshold: 8 μg/m3 (winter) NO2 (annual average) - Lower evaluation threshold: 24 μg/m3 | Deposition of pollutants that are transferred from the atmosphere to soil, vegetation, water – alteration of habitats | Not If maybe quantify | The analyzed perimeter does not intersect with the boundaries of protected natural areas or protected natural habitats |
|  |  |  |  | - Upper evaluation threshold 19.5 μg/m3 PM10 (annual average) - -Lower evaluation threshold: 28 μg/m3 - Upper evaluation threshold: 20 μg/m3 CO (8-hour average) - Lower endpoint: 7 mg/m3 Upper threshold of assessment: 5 mg/m3 |  |  |  |
|  |  | Accidental leakage of petroleum products from machinery-penetration of Pollutants in the soil |  | more than 1% of the site area or natural habitats/habitats important for species | Alteration of habitats in the work area | Not If maybe quantify |  |
|  |  | Introduction of invasive species |  | more than 1% of the site area or natural habitats/habitats important for species | Alteration of habitats in the work area | It cannot be quantified | It is not possible to affect the floristic composition of natural habitats by spreading invasive species; - anemocorous, because the distance between the protected natural areas and the area studied by the PROJECT is large enough, between them there are localities and agricultural lands. Also, during the period of execution of the decommissioning works, access roads from protected areas will not be used. - by hydrochlorine (the only stream that crosses the site and that crosses the perimeter studied by the PROJECT is the Cerchez Creek, they are located downstream of the sites) |

    - Results of the impact assessment based on the Specific Conservation Objectives

In order to estimate the potential impact of the project on the species in the Natura 2000 site potentially affected by the project, the requirements of the two Orders on appropriate assessment – Order 1682/2023 and Order 1679/2023 – were taken into account.

The analysis was carried out on the basis of the Site-Specific Conservation Objectives (SCOs) established by ANANP for the aforementioned potentially affected Natura 2000 site.

The analysis was carried out for each species of community interest, at the level of each of the parameters that define CSOs, taking into account the effects generated by the implementation and functioning of the project.

In the framing phase, the purpose of the analysis is to estimate the potential impact generated by the project, without taking into account the measures to avoid and reduce the impact.

The estimation of the potential impact on CSOs was carried out by following the following steps:

- Analysis of the objectives, parameters and targets set for each of the species of community interest included in the CSO;

- Case-by-case analysis for each species of the parameters that could be affected by the proposed project, carried out as follows:

- Identification of the possibility of damage to the component (species): Is the habitat of the species intersected? Is it located in the area of manifestation of an effect generated by the project? Can individuals of the species get to the project area? Can the project affect one of the ecological functions of the habitat/species?

- Identification of the possibility of affecting the parameter: is there a cause-effect relationship between the project activities and the analyzed parameter (e.g. physical or chemical interactions)?

- Analysis of the objectives, parameters and targets set for each of the species of community interest included in the CSO;

- Analysis of the objectives, parameters and targets set for each of the species of community interest included in the CSO;

- Assessing the significance of the impact, using mainly the 'significant/insignificant' classes. The significance assessment was generally carried out on the basis of the following parameters:

- Quantitative – the percentage of impairment from the target value;

- Qualitative:

- If the central or marginal area of the habitat is affected;

- Conservation status at the level of the site and at the level of the biogeographical region;

- Presence in other Natura 2000 sites;

- Species at the limit of the distribution area.

- Ecological functions:

- Maintaining/restoring ecological connectivity;

- Maintaining critical physicochemical parameters, such as water levels.

The preliminary analysis of the potential impact generated by the project on the natural elements with conservative value aimed to identify and study those forms of impact for which there is a risk of reaching significant thresholds in the absence of measures to avoid and reduce the impact. The potential impact that the project can generate on biodiversity components can manifest itself differently depending on the stage of the project.

The results indicate that the project is likely to generate impacts on the parameters of the specific conservation objectives.

Table 20 Estimation of the potential impact of the proposed plan on the species and habitats for which ANPIC has been designated

| Name of ANPIC | Species/Habitat |  | Affected parameter |  |  |  | Target Value |  | UM | Conservation status | Shape of impact |  |  | Potential impact (no measures) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROSCI0071 | Spermophilus citellus (Popândău) |  | Habitat area of the species |  |  |  | At least 325 ha |  | Ha | Favourable | Habitat loss - the occupation of land areas with favourable habitats, which can ultimately have effects on livestock population |  |  | Insignificant negative |
|  |  |  | Size Population |  |  |  | At least 3000 individuals |  | No. of individuals |  | Alteration Habitats - Spread of invasive plant species in habitats Reduction of population numbers - risk of mortality |  |  | Insignificant negative |
|  | Miniopterus schreibersii (Long-winged bat) |  | Size Population |  |  |  | At least 1500 individuals |  | No. of individuals | Unfavourable (unfavourableinadequate in terms of size) population, favorable in terms of habitat area and future prospects of the species.) | Population reduction mortality |  | Herd - risk of | Significant negative |
| ROSCI0157 | Spermophilus citellus (Popândău |  | Size Population |  |  |  | At least 3000 individuals |  | No. of individuals | Favourable | Reducing population mortality Habitat loss areas of land in breeding habitats and Alteration Spread of invasive plant species in habitats |  | Herd - risk of T - Occupancy Favorable Feeding Habitats - | Insignificant negative |
|  |  |  | Habitat area of the species |  |  |  | At least 200 ha |  | Ha |  |  |  |  | Insignificant negative |
|  | Rhinolophus ferrumequinum (Great Horseshoe Bat) |  | Size Population |  |  |  | Must be defined within a period of 3 years |  | No. of individuals | Unfavorable - inadequate | Population reduction - mortality risk |  |  | Insignificant negative |
|  | Miniopterus schreibersii (Long-winged bat) |  | Size Population |  |  |  | At least 1500 individuals |  | No. of individuals | Unfavourable (unfavourable inadequate in terms of population size, favourable in terms of habitat area and future prospects of the species). | Population reduction - mortality risk |  |  | Significant negative |
|  | Mustela eversmanii (Steppe ferret)" |  | Size Population |  |  |  | At least 30 individuals |  | No. of individuals Ha | Favourable | Population reduction - mortality risk |  |  | Insignificant negative |
|  |  |  | Habitat area of the species |  |  |  | At least 200 ha |  |  |  | Habitat loss/alteration - occupation of land areas with favorable breeding and feeding habitats Alteration Habitats - Spread of invasive plant species in habitats |  |  | Insignificant negative |
| ROSPA0166 | Anthus campestris (Field strip) |  | Population size |  |  |  | Number of pairs during the breeding period – at least 10 |  | No. of pairs Percentage change | Unknown | Population reduction - risk of mortality Disturbance of species activity and removal |  |  | Insignificant negative |
|  |  |  | Population size trend |  |  |  | Change%- Stable or Increasing |  |  |  |  |  |  | Insignificant negative |
|  | Buteo rufinus (Big mouse)" |  | Size Population |  |  |  | Number of individuals during the wintering period At least 1 |  | No. of individuals Percentage change | Unknown | Population reduction - risk of mortality |  |  | Significant negative |
|  |  |  | Population size trend |  |  |  | Change % |  |  |  |  |  |  | Significant negative |
|  | Circus cyaneus |  | Size Population |  |  |  | Number of individuals in wintering – at least 1 |  | No. of individuals Percentage change | Unknown | Population reduction - risk of mortality Disturbance of species activity and removal Alteration Habitats Spread of invasive plant species in habitats |  |  | Significant negative |
|  |  |  | Population Size Trend - Stable or Increasing |  |  |  | Change % - stable or decreasing |  |  |  |  |  |  | Significant negative |
|  | Falco columbarius (Winter Falcon) |  | Size Population |  |  |  | Number of individuals during the wintering period At least 1 |  | No. of individuals Percentage change | Unknown | Population reduction - risk of mortality Disturbance of species activity and removal |  |  | Significant negative |
|  |  |  | Population size trend |  |  |  | Change % - stable or decreasing |  |  |  |  |  |  | Significant negative |
|  | Falco vespertinus (Evening Hawk) |  | Size Population |  |  |  | No. Pairs - At least 35 |  | No. of pairs Percentage change | Favourable | Population reduction - risk of mortality Disturbance of species activity and removal |  |  | Insignificant negative |
|  |  |  | Population size trend |  |  |  | Change %- Stable or Increasing |  |  |  |  |  |  | Insignificant negative |
|  | Lanius minor (Black-fronted Shrike) |  | Size Population |  |  |  | Number of pairs - at least 3 |  | No. of pairs Percentage change | Unknown | Population reduction - risk of mortality Disturbance of species activity and removal |  |  | Insignificant negative |
|  |  |  | Population size trend |  |  |  | Change % - stable or increasing |  |  |  |  |  |  | Insignificant negative |
| ROSPA0036 |  |  |  |  | Regularly migrating species Species associated with extensively used agricultural land |  |  |  |  |  |  |  |  |  |  |
|  | Alauda arvensis (Woodpecker) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change - Long-term trend of stable or growing population |  | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha-at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in favorable habitats for feeding Habitat alteration Spread of invasive plant species in habitats |  |  | Insignificant |  |
|  | Carduelis cannabina (Hemp) |  | Population size |  |  |  | Number of nesting pairs |  | No. of pairs No. of individuals Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  |  | No individuals in migration |  |  |  |  |  |  |  |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha |  |  |  | Disturbance of species activity and removal Alteration of habitats Spread of invasive plant species in habitats Habitat loss - occupation of areas of land in habitats favourable for feeding |  |  | Insignificant |  |
|  | Carduelis carduelis (Sticlete) |  | Population size |  |  |  | Number of nesting pairs |  |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  |  | No individuals in migration Must be defined within 2 years |  | No. of pairs No. of individuals Percentage change Ha |  |  |  |  |  |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Coturnix coturnix (Quail) |  | Population size |  |  |  | Number of nesting pairs- At least 175 |  |  | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change - The long-term trend of the stable or increasing population |  | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | ha - at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Emberiza melanocephala (Black-headed pressura) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change - The long-term trend of the stable or increasing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Falco tinnunculus (Red-knuckle) |  | Population size |  |  |  | Number of nesting pairs - Long-term trend of stable or increasing population |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change - The long-term trend of the stable or increasing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Fringilla montifringilla (Winter Finch) |  | Population size |  |  |  | Number of individuals in migration |  | No. of pairs No. of individuals Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  |  | Winter number Must be defined within 2 years |  |  |  |  |  |  |  |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Galerida cristata (Woodpecker) |  | Population size |  |  |  | Number of nesting pairs-At least 125 |  |  | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Lanius excubitor (Great shrub) |  | Population size |  |  |  | Number of specimens in winter - Must be defined within 2 years |  | No. of individuals Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Lanius senator (Red-headed shroud) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha-at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Merops apiaster (Prigorie) |  | Population size |  |  |  | Number of nesting pairs |  | No. of pairs No. of individuals Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  |  | Number of individuals in migration Must be defined within 2 years |  |  |  |  |  |  |  |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Miliaria calandra (Pressura sura) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Motacilla alba (White codobate) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Motacilla flava (Yellow Squirrel) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change - Long-term trend of stable or increasing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Oenanthe isabellina (Eastern Stone) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Oenanthe oenanthe (Pietrar sur) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (agricultural land used extensively |  |  |  | Ha-at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Saxicola torquata (Black Bramble) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas |  |  | Insignificant |  |
|  | Streptopelia turtur (Turtledove) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Sturnus roseus (alternative name Pastor roseus) (Grasshopper) |  | Population size |  |  |  | Number of individuals in migration- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (agricultural land used extensively |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Turdus iliacus (Vine thrush) |  | Population size |  |  |  | Number of individuals in migration- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding |  |  | Insignificant |  |
|  | Upupa epops (Hoopoe) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- at least 4734.21 |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  |  |  |  |  | Species associated with forest habitats |  |  |  |  |  |  |  |  |  |  |
|  | Carduelis chloris (Florinte) |  | Population size |  |  |  | Number of nesting pairs Number of individuals in migration- To be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha-Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Carduelis spinus (Scatiu) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Coccothraustes coccothraustes (Botgros) |  | Population size |  |  |  | Number of nesting pairs Number of individuals in migration Must be defined within 2 years |  | No. of pairs No. of individuals Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in favourable habitats for feeding Habitat alteration |  |  | Insignificant |  |
|  | Falco subbuteo (Swallow Falcon) |  | Population size |  |  |  | Number of nesting pairs- At least 11 |  |  | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Ficedula hypoleuca (Black Flycatcher) |  | Population size |  |  |  | Number of individuals in migration |  |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than that resulting from variations |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha-Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in favourable habitats for feeding |  |  | Insignificant |  |
|  | Fringilla coelebs (Cinderella) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Reduction of population numbers - mortality risk Habitat alteration |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Jynx torquilla (Headcatchers) |  | Population size |  |  |  | Number of nesting pairs- Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensive utilised agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal |  |  | Insignificant |  |
|  | Muscicapa striata (Muscar sur) |  | Population size |  |  |  | Number of nesting pairs Number of individuals in migration- To be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Reduction of population numbers - mortality risk Habitat alteration |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Oriolus oriolus (Grangur) |  | Population size |  |  |  | Number of pairs nesting- Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Phylloscopus collybita (Little Dwarf) |  | Population size |  |  |  | Number of nesting pairs Number of individuals in migration- To be defined within 2 years |  | No. of pairs No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Phylloscopus trochilus (Whistling Dwarf) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Pyrrhula pyrrhula (Spruce) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Regulus ignicapillus (Eyebrowed Owl) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Regulus regulus (Yellow-headed Owl) |  | Population size |  |  |  | Number of individuals in migration, Must be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use, No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Serinus serinus (Cănăraş) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use-No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha-Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas |  |  | Insignificant |  |
|  | Sturnus vulgaris (Graur) |  | Population size |  |  |  | Number of individuals in migration,To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern – intensity of habitat use |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Sylvia atricapilla (Black-headed Sylvia) |  | Population size |  |  |  | Number of nesting pairs Number of individuals in migration- To be defined within 2 years |  | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Sylvia curruca (Little Silvie) |  | Population size |  |  |  | Number of nesting pairs Number of individuals in migration- To be defined within 2 years |  | No. of pairs No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Turdus merula (Blackbird) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Turdus philomelos (Song Thrush) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Turdus pilaris (Rooster) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha-Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Turdus viscivorus (Mistletoe thrush) |  | Population size |  |  |  | Number of individuals in migration- To be defined within 2 years |  | No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  |  | Species associated with reedbed habitats |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Acrocephalus arundinaceus (Great Common Grasshopper) |  | Population size |  |  |  | Number of nesting pairs - To be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  | Acrocephalus palustris (Swamp Squash) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  | Acrocephalus schonobaenus (Little Lamb) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  | Acrocephalus scirpaceus (Reed Bedbug) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  | Locustella fluviatilis (Reed Rush) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  | Locustella luscinoides (Reed Grelușel) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  | Riparia riparia (Shore Squirrel) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  | Species associated with rocky habitats and urban areas |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Monticola saxatilis (Blackbird) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  |  |  |  |  |  | Insignificant |  |
|  |  |  | Surface of open terrestrial habitats |  |  |  | Ha- Must be evaluated within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Hirundo daurica (Red Swallow) |  | Population size |  |  |  | Number of nesting pairs-At least 10 |  |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Surface of open terrestrial habitats |  |  |  | Ha- Must be evaluated within 2 years |  | No. of pairs without subtraction Ha |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Hirundo rustica (Swallow) |  | Population size |  |  |  | Number of nesting pairs-Must be defined within 2 years |  |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Distribution pattern |  |  |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal pattern or intensity of habitat use for each species other than those resulting from natural variations |  | No. of pairs without significant decrease in spatial, temporal or intensity pattern Ha |  |  |  |  | Insignificant |  |
|  |  |  | Surface of open terrestrial habitats |  |  |  | Ha- Must be evaluated within 2 years |  |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
| ROSPA0094 | Accipiter brevipes (Short-legged hawk) |  | Size of the nesting population |  |  |  | Number of nesting pairs-At least 5 |  | No. of pairs No. of individuals without significant decrease in spatial, temporal or intensity pattern Ha | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population size in the passage |  |  |  | Number of Individuals-At least 30 |  |  |  |  |  |  | Insignificant |  |
|  | Alcedo atthis (Blue Seagull) |  | Size of the nesting population |  |  |  | Number of pairs- Must be defined within 2 years |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Size of the wintering population |  |  |  | Number of individuals- At least 4 |  | No. of individuals |  |  |  |  | Insignificant |  |
|  | Anthus campestris (Field strip) |  | Size of the nesting population |  |  |  | Pairs-At least 175 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Heliacal eagle (Eagle of the field) |  | Population size in the passage |  |  |  | Individuals- At least 4 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Aquila pomarina (Lesser Spotted Eagle) |  | The mother of the nesting population |  |  |  | Number of Pairs- At least 2 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Burhinus oedicnemus (Field Bird) |  | Size of the nesting population |  |  |  | Pairs- At least 8 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Population size in the passage |  |  |  | Individuals-At least 55 |  | No. of individuals |  |  |  |  |  |  |
|  | Buteo rufinus (Great Buzzard) |  | Size of the nesting population |  |  |  | Pairs-At least 4 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  | Caprimulgus europaeus (Caprimulg) |  | Size of the nesting population |  |  |  | Number of individuals-At least 20 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Brachydactyla caterpillar (Flock woodpecker) |  | Size of the nesting population |  |  |  | Pairs-At least 35 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Ciconia ciconia (White stork) |  | Resting/feeding population size |  |  |  | Number of individuals in the passage- At least 1300 |  | No. of individuals |  |  |  |  | Insignificant |  |
|  |  |  | Population trends |  |  |  | Percentage change- The long-term trend of the stable or growing population |  | Percentage change |  |  |  |  | Insignificant |  |
|  | Circaetus gallicus (Snake) |  | Size of the nesting population |  |  |  | Pairs- At least 2 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Circus aeruginosus (Reed herd) |  | Population size |  |  |  | Number of nesting pairs- At least 2 |  |  | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  |  | Number of Individuals in Passage - At least 50 |  | No. of pairs No. of individuals |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Number of individuals wintering-At least 4 |  |  |  |  |  |  |  |  |
|  | Circus cyaneus (Eggplant heron) |  | Population size |  |  |  | Number of individuals in the passage- At least 90 |  | No. of pairs No. of individuals Percentage change | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  |  |  |  |  | Number of individuals wintering- At least 3 |  |  |  | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  | Trends in the passenger population |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Circus macrourus (White heron) |  | Population size |  |  |  | Number of individuals in the passage- At least 40 |  |  | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  | Trends in the passenger population |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  | No. of individuals Percentage change |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Circus pygargus (Erete sur) |  | Population size |  |  |  | Number of individuals in the passage- At least 175 |  |  | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  | Trends in the passenger population |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  | No. of individuals Percentage change |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Coracias garrulus (Dumbrăveancă) |  | Size of the nesting population |  |  |  | Pairs- At least 20 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Cherrug falcon (Saker Falcon) |  | Population size |  |  |  | Number of individuals- At least 4 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  | Falco columbarius (Winter Hawk) |  | Population size |  |  |  | Number of individuals- At least 2 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  | Falco peregrinus (Peregrine Falcon) |  | Population size |  |  |  | Number of individuals- At least 2 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Falco vespertinus (Evening Hawk) |  | Population size |  |  |  | Number of nesting pairs- At least 21 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  |  | Number of individuals in the passage- At least 250 |  | No. of individuals |  |  |  |  | Insignificant |  |
|  | Hieraaetus pennatus (Lesser Eagle) |  | Size of the nesting population |  |  |  | Pairs- At least 2 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  | Size of the passage population |  |  |  | Individuals- At least 20 |  | No. of individuals |  |  |  |  | Insignificant |  |
|  |  |  | Trends in the passenger population |  |  |  | Percentage change- Long-term trend of stable or growing population |  | Percentage change |  |  |  |  | Insignificant |  |
|  | Lanius collurium (Reddish shroud) |  | Size of the nesting population |  |  |  | Number of pairs- At least 50 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Lanius minor (Black-foreheaded shrike) |  | Size of the nesting population |  |  |  | Number of pairs- At least 40 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Larus melanocephalus (Black-headed Gull) |  | Size of the passage population |  |  |  | Individuals-5750 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Trends in the passenger population |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  | Percentage change |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Larus minutus (Lesser Gull) |  | Size of the passage population |  |  |  | Individuals-150 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Lullula arborea (Woodpecker) |  | Population size |  |  |  | Number of pairs- At least 100 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Melanocorypha calandra (Bărăgan woodpecker) |  | Size of the nesting population |  |  |  | Pairs-50 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  | Milvus migrans (Black Jay) |  | Size of the passage population |  |  |  | Individuals-8 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Nycticorax nycticorax (Night Heron) |  | Population size |  |  |  | Individuals- At least 10 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Oenanthe pleschanka (Blackstone) |  | Size of the nesting population |  |  |  | Pairs- At least 6 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Pelecanus crispus (Curly Pelican) |  | Population size |  |  |  | Number of individuals – to be defined within 2 years |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Trends in the passenger population |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  | Percentage change |  |  |  |  |  |  |
|  | Pernis apivorus (Viespar) |  | Size of the nesting population |  |  |  | Number of pairs- At least 3 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Significantly |  |
|  | Philomachus pugnax (Bully) |  | Population size in the passage |  |  |  | Individuals-100 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  | Trends in the passenger population |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  | Percentage change |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Porcelain porcelain/Calidris pugnax (Variegated crest) |  | Size of the nesting population |  |  |  | Number of pairs- At least 10 |  | No. of pairs | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Recurvirostra avosetta (Woodpecker) |  | Population size |  |  |  | Number of individuals in the passage- At least 20 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Tadorna ferruginea (Red Caliph) |  | Population size in the passage |  |  |  | Number of individuals-8 |  | No. of individuals | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  | Regularly migrating species Species associated with open land/agricultural land |  |  |  |  |  |  |  |  |  |  |
|  | Alauda arvensis (Woodpecker) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding |  |  | Insignificant |  |
|  | Buteo lagopus (Shoed Mouse) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Emberiza melanocephala (Black-headed pressura) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal |  |  | Insignificant |  |
|  | Senator Lynch (Red-headed shrike) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Merops apiaster (Prigorie) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Miliaria calandra (Pressura sură) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years | No. of pairs Percentage change Ha |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha-Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Motacilla alba (White codobate) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Oenanthe oenanthe (Southern Stone) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Must be defined within 2 years |  |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Streptopelia turtur (Turtledove) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Upupa epops (Hoopoe) |  |  | Population size |  |  |  | Number of pairs- Must be defined within 2 years | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Falco tinnunculus (Red-knuckle) |  |  | Population size |  |  |  | Number of pairs- At least 40 |  | Unknown | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  |  |  |  | Significantly |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Anser albifrons (Great Tern) |  |  | Population size |  |  |  | Number of pairs- At least 400 | No. of pairs Percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Significantly |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration – temporary occupation of areas |  |  | Insignificant |  |
|  | Galerida cristata (Woodpecker) |  |  | Population size |  |  |  | Number of pairs- At least 130 |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term population trend for all species stable or increasing | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensive agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal |  |  | Insignificant |  |
|  | Coturnix coturnix (Quail) |  |  | Population size |  |  |  | Number of nesting pairs- At least 300 |  | Favourable | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  | Population trends for each species |  |  |  | Percentage change- Long-term trend of stable or growing population | No. of pairs Percentage change Ha |  |  |  |  | Insignificant |  |
|  |  |  |  | Area of open terrestrial habitats (extensively used agricultural land) |  |  |  | Ha- Must be defined within 2 years |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  |  |  |  |  | Regularly migrating species Species associated with riparian and littoral habitats |  |  |  |  |  |  |  |  |  |  |
|  | Gallinago gallinago (Common Woodpecker) |  |  |  |  | Population size |  | Number of individuals in winter- At least 44 | No. of individuals Percentage change | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Insignificant |  |
|  | Hirundo rustica (Swallow) |  |  |  |  | Population size |  | Number of nesting pairs- Must be defined within 2 years | No. of pairs Spatial and temporal pattern, intensity of habitat use | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  | Distribution pattern |  | Spatial and temporal pattern, intensity of habitat use - No significant decrease in spatial, temporal or intensity of habitat use for each species other than those resulting from natural variations |  |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  | Regularly migrating species Species associated with urban habitats |  |  |  |  |  |  |  |  |  |  |
|  | Hirundo rustica (Swallow) |  |  |  |  | Population size |  | Number of pairs- Must be defined within 2 years | No. of pairs Percentage change | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Insignificant |  |
|  |  |  |  |  | Regularly migrating species Species associated with mixed terrestrial habitats, forests and open lands |  |  |  |  |  |  |  |  |  |  |
|  | Accipiter nisus (Bird Hawk) |  |  |  |  | Population size |  | Number of pairs- Must be defined within 2 years | No. of pairs percentage change Ha | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Insignificant |  |
|  |  |  |  |  |  | Area of overused grassland habitats (feeding habitat for several species in this group) |  | Ha- At least |  |  | Disturbance of species activity and removal Habitat loss - occupation of areas of land in habitats favourable for feeding Habitat alteration |  |  | Insignificant |  |
|  | Buteo buteo (Common mousetrap) |  |  |  |  | Population size |  | Number of individuals in the passage- At least 200 |  | Unknown | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  |  |  |  |  |  | Number of pairs- Must be defined within 2 years | No. of pairs Percentage change |  |  |  |  |  |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Significantly |  |
|  | Subbuteo falcon (Swallow Falcon) |  |  |  |  | Population size |  | Number of pairs- At least 25 | No. of pairs Percentage change | Unknown | Population reduction - mortality risk |  |  | Significantly |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  |  |  |  |  | Significantly |  |
|  |  |  |  |  | Regularly migrating species Species associated with reedbed habitats |  |  |  |  |  |  |  |  |  |  |
|  | Motacilla flava (Yellow codobate) |  |  |  |  | Population size |  | Number of pairs- Must be defined within 2 years | No. of pairs Percentage change | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  | Regularly migrating species Species associated with open aquatic habitats |  |  |  |  |  |  |  |  |  |  |
|  | Anser albifrons (Great Tern) |  |  |  |  | Population size |  | Number of individuals in winter- At least 80 | No. of individuals Percentage change |  |  |  |  |  |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  |  | Population reduction - mortality risk |  |  | Insignificant |  |
|  | Larus cachinnans (Mediterranean Seagull) |  |  |  |  | Population size |  | Number of individuals in winter- At least 1800 | No. of individuals Percentage change | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term population trend for all species stable or increasing |  | Unknown | Population reduction - mortality risk |  |  | Insignificant |  |
|  |  |  |  |  | Regularly migrating species Species associated with forest habitats |  |  |  |  |  |  |  |  |  |  |
|  | Sylvia atricapilla (Black-headed Silvie) |  |  |  |  | Population size |  | Number of nesting pairs Number of individuals in migration- Must be defined within 2 years | No. of pairs Percentage change | Unknown |  | Population reduction - mortality risk |  | Insignificant |  |
|  |  |  |  |  |  | Population trends for each species |  | Percentage change- Long-term trend of stable or growing population |  |  |  | Population reduction - mortality risk |  | Insignificant |  |

In order to assess the cumulative impact on protected natural areas, the information presented in the management plans of the identification areas regarding the existing and future pressures that could threaten the conservation status of species and habitats, the activities currently carried out in the area proposed for the development of the project, projects approved in the vicinity of the analyzed project were analyzed.

The following materiality thresholds were used to assess the significance of the potential impact:

- significantly

- insignificant

- uncertain.

If a habitat or species of Community interest on the territory of an ANPIC is in an unfavourable conservation status, the project alone or in combination with other plans and projects has been considered to add an additional impact on them, it may have a significant effect on the ANPIC concerned. The negative impact manifesting itself especially on these species.

The table below presents the analysis of the cumulative evaluation of the protected areas in the project's area of influence.

Table 21 Cumulative impact analysis

| Crt. no. | Name of ANPIC | Species/Habitat | Parameter affected by PP analyzed | Pressures/threats, other PPs that can generate cumulative impact on the affected parameter |  | Quantification of cumulative impact |  | Significance of cumulative impact | Justification of the significance of cumulative impact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Possible to accumulate Impact? (Yes/No) | Significance thresholds |  |  |
| 1 | ROSCI0071 | Spermophilus citellus (Popândău) | Surface habitat of the species | Existing/future pressures | A02.03 – Replacement of pasture with arable land. A04.01 Intensive grazing | Yes | <1% habitat loss without affecting the functional ecological corridor | insignificant | Although the Project does not intersect with the boundaries of the site and will not generate habitat loss in the protected area. The occupation of some land areas by the proposed investments (about 309 ha) could lead to the loss of favorable habitats for this species, areas outside the protected area |
|  |  |  | Size Population |  | D01.02 Roads, motorways E01.01Continuous urbanisation | Yes | No. of affected individuals Population reduction <1% at local level, without fragmentation or genetic isolation | insignificant | The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species was observed on the site of the future wind farm in a small number of specimens compared to the size of the population at the site. The species has a favorable conservation status. During the period of carrying out the works, there is a risk of accidental collision with road traffic on the access roads/handling of site machinery at the work front and the loss of individuals of this species. The occupation of some land areas by the proposed investments (about 309 ha) could lead to the loss of favorable habitats outside the protected area - which in the end can have effects on the population |
| 2. |  | Miniopterus schreibersii | Size Population |  | E01.01 Urbanization G01 Outdoor sports and leisure activities, recreational activities A10.01 removal of hedges and vegetation or bushes A10.02 Removal of stone walls and embankments A11 Agricultural activities not mentioned above B02 Management and use of forests and plantations B02.01 Forest replanting B02.01.01 replanting of forests (indigenous trees) B02.01.02 Forest replanting (non-indigenous trees) B02.02 Forest deforestation B02.03 removal of forest vegetation B02.04 Removal of dry and drying trees | Yes | No. of affected individuals (cannot be quantified | Insignificant | The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species was observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and loss of some individuals of this species, especially during the migration period. The results of the monitoring for the Chirnogeni Wind Farm 2014-2016 show that no mortalities were recorded among bird and bat species. |
|  |  |  |  |  | C03.03 Wind energy production (this pressure is not mentioned in the management plan for this species, it is mentioned for birds) |  |  |  |  |
|  |  |  |  |  | Operation of the Chirnogeni Wind Farm (located north of the Project site, at about 600 m) – located at a distance of more than 9 km from the site limit. |  |  |  |  |
| 3. | ROSCI0157 | Spermophilus citellus (Popândău) | Surface habitat of the species | Existing pressures | A02.03 – Replacement of pasture with arable land. A04.01 Intensive grazing | Yes | <1% habitat loss without affecting the functional ecological corridor | Insignificant | Although the Project does not intersect with the boundaries of the site and will not generate habitat loss in the protected area. The occupation of some land areas by the proposed investments (about 309 ha) could lead to the loss of favorable habitats for this species outside the protected area |
|  |  |  |  | Future pressures | E01.01 Continued urbanization | Not | - | - | Although the Project does not intersect with the boundaries of the site and will not generate habitat loss in the protected area. During the period of operation of the wind farm, no other land areas will be occupied besides. Interventions for maintenance and maintenance will not generate a cumulative impact with this pressure, the risks of alteration of existing habitats are very low. No other investments are proposed on the analyzed site and in the vicinity. |
|  |  |  | Size Population | Existing pressures | A.07 Use of Biocides, Hormones and Chemicals A10.01 Removal of hedges and groves or Bushes A.11 Agricultural activities other than those listed above E03.01 Landfill of household waste G01.03.02 Off-road driving of motor vehicles | Yes | No. of affected individuals Population reduction <1% at local level, without fragmentation or genetic isolation | Insignificant | The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species was observed on the site of the future wind farm in a small number of specimens compared to the size of the population at the site. The species has a favorable conservation status. During the period of carrying out the works, there is a risk of accidental collision with road traffic on the access roads/handling of site machinery at the work front and the loss of individuals of this species. The occupation of some land areas by the proposed investments (about 309 ha) could lead to the loss of favorable habitats outside the protected area - which in the end can have effects on the population |
|  |  |  |  | Future pressures | E01.01 Continuous urbanization | NOT | - | - | Although the Project does not intersect with the boundaries of the site and will not generate habitat loss in the protected area. During the period of operation of the wind farm, no other land areas will be occupied besides. Interventions for maintenance and maintenance will not generate a cumulative impact with this pressure, the risks of alteration of existing habitats are very low. No other investments are proposed on the analyzed site and in the vicinity. |
|  |  | - Rhinolophus ferrumequinum (The Great Horseshoe Bat) | Size Population | Existing pressures | G01 Outdoor sports and leisure activities, recreational activities A10.01 Removal of hedges and vegetation or bushes A10.02 Removal of stone walls and embankments A11 Agricultural activities not mentioned above B02 Management and use of forests and plantations B02.01.02 Forest replanting (non-indigenous trees) B02.02 Forest deforestation B02.03 Removal of forest vegetation B02.04 Removal of dry or drying trees B03 Exploitation of forests without replanting or natural regrowthA07 E01.01 Continuous urbanization | Yes | No. of affected individuals Population reduction <1% at local level, without fragmentation or genetic isolation | Insignificant | The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species was observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and loss of some individuals of this species, especially during the period of travel in search of food. The results of the monitoring for the Chirnogeni Wind Farm 2014-2016 show that no mortalities were recorded among bird and bat species. |
|  |  |  |  |  | C03.03 Wind energy production (this pressure is not mentioned in the management plan for this species, it is mentioned for birds |  |  |  |  |
|  |  |  |  |  | Operation of the Chirnogeni EP WIND PROJECT(ROM)SIX SA Wind Farm (located north of the Project site, at about 600 m) – the park is located at a distance of more than 13 km from the site limit. |  |  |  |  |
|  |  | Mustela eversmannii (Ferret steppe)" | Size Population | Existing pressures | A02.03 – Replacement of pasture with arable land. A04.01 Intensive grazing | Yes | No. of affected individuals Population reduction <1% at local level, without fragmentation or genetic isolation | Insignificant | Although the Project does not intersect with the boundaries of the site and will not generate habitat loss in the protected area, the occupation of some land areas, the proposed investments (about 309 ha) could lead to the loss of favorable habitats for this species outside the protected area |
|  |  |  | Surface habitat of the species |  | A.07 Use of Biocides, Hormones and Chemicals A10.01 Removal of hedges and groves or Bushes A.11 Agricultural activities other than those listed above E03.01 Landfill of household waste G01.03.02 Off-road driving of motor vehicles |  | <1% habitat loss without affecting the functional ecological corridor | Insignificant | Although the Project does not intersect with the boundaries of the site and will not generate habitat loss in the protected area, the occupation of some land areas, the proposed investments (about 309 ha) could lead to the loss of favorable habitats for this species outside the protected area |
|  |  | Miniopterus schreibersii | Size Population | Existing pressures | G01 Outdoor sports and leisure activities, recreational activities A10.01 removal of hedges and vegetation or bushes A10.02 Removal of stone walls and embankments A11 Agricultural activities not mentioned above B02 Management and use of forests and plantations B02.01.02 Forest replanting (non-indigenous trees) B02.02 Forest deforestation B02.03 Removal of forest vegetation B02.04 Removal of dry or drying trees B03 Exploitation of forests without replanting or natural regrowth A07 Use of biocides, hormones and substances Chemical E01.01 Continued urbanization | Yes | No. of affected individuals | Insignificant - | The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species was observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and loss of some individuals of this species, especially during the migration period. The results of the monitoring for the Chirnogeni Wind Farm 2014-2016 show that no mortalities were recorded among bird and bat species. |
|  |  |  |  |  | C03.03 Wind energy production (this pressure is not mentioned in the management plan for this species, it is mentioned for birds) |  |  |  |  |
|  |  |  |  |  | Operation of the Chirnogeni EP WIND PROJECT(ROM)SIX SA Wind Farm (located north of the Project site, at about 600 m) – the park is located at a distance of more than 13 km from the site limit. |  |  |  |  |
|  |  |  |  | Future pressures | E01.01 Continued urbanization | Not | - | - | Although the Project does not intersect with the boundaries of the site and will not generate habitat loss in the protected area. During the period of operation of the wind farm, no other land areas will be occupied besides. Interventions for maintenance and maintenance will not generate a cumulative impact with this pressure, the risks of alteration of existing habitats are very low. No other investments are proposed on the analyzed site and in the vicinity. |
| 4. | ROSPA0166 | Anthus campestris (Field Strip) Buteo rufinus (Big mouse)" Circus cyaneus Falco columbarius (Winter Falcon) Falco vespertinus (Evening Hawk) Lanius minor (Forehead Shrike black) | Size Population | Existing pressures | A 02.01 Intensive agriculture (off-site) G 01.03 .01 Ordinary driving of motor vehicles (on site) | Yes | No. of affected individuals | Insignificant - | A negative cumulative impact will be manifested especially for species that have an unfavorable conservation status The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species were observed in the implementation area of the future energy park. During the period of carrying out the works, there is a risk of accidental collision with road traffic on the access roads/handling of site machinery at the work front and the loss of individuals of this species. The occupation of some land areas by the proposed investments (about 309 ha) could lead to the loss of favorable habitats outside the protected area - which in the end can have effects on the population numbers. |
|  |  |  |  | Population size trend | Operation of the Chirnogeni Wind Farm - EP WIND PROJECT(ROM)SIX SA (located north of the project site, about 600 m). The park is located at a distance of about 3 km from the boundary of the site (downstream of the site) | Yes | No. of affected individuals | Insignificant - | The project does not overlap with the limits of the protected natural area ROSPA0166. However, the project location meets the conditions for species-friendly habitats for feeding or can be a passage lane. The species were observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and the loss of some individuals of these species. The results of the monitoring for the Chirnogeni Wind Farm 2014-2016 show that no mortalities were recorded among bird and bat species. At the same time, these monitoring results mention that the park is not a disruptive factor for the local fauna and bird species, during the monitoring no changes in their activity were observed. |
| 5.Shipping: | ROSPA036 | Accipiter brevipes (Short-legged hawk) Anthus campestris ( Field strip) Aquila pomarina (Lesser Spotted Eagle) Bubo bubo (Buha) Buteo rufinus (Big Mouse) Caprimulgus europaeus (Caprimulg) Circus cyaneus (Eggplant heron) Circus macrourus (White Erte) Circus pygargus (You were on) Coracias garrulus (Dumbrăveancă) Falco peregrinus (Peregrine Falcon) Hieraaetus pennatus (Eagle small) Lanius collurium (Reddish shroud) Lanius minor (Black-foreheaded shrike) Melanocorypha calandra (Bărăgan woodpecker) Milvus migrans (Black Jay) Pernis apivorus (Viespar) Species associated with reedbed habitats Acrocephalus arundinaceus (Great Common Grasshopper) - Acrocephalus palustris (Swamp Lake) Acrocephalus schoenobaenus (Little Grasshopper) Acrocephalus scirpaceus (Reed Bedbug) | Size Population Population size trend | Existing pressures for all bird species of interest Conservative | A02.03 Use of grassland for arable land A04.01 Intensive grazing A07 Use of Biocides, Hormones and Chemicals A08 Fertilization - with fertilizer A11Agricultural activities not mentioned above B02 Management and use of forests and plantations B02.02. Deforestation B02.01.02 Replanting Forests (non-native trees) B02.04 Removal of dry or drying trees B06 Grazing in forest areas B03 Exploitation of forests without replanting or natural regeneration C03.03 Wind energy production D02.01.01 Above-ground power lines G01 Outdoor sports and leisure activities, recreational activities E01.01 Continuous urbanization Operation of the Chirnogeni Wind Farm - EP WIND PROJECT(ROM)SIX SA (located north of the Project site, at about 600 m). The park is located at a distance of more than 10 km from the site boundary (downstream of the site) | Yes | No. of affected individuals | Insignificant | A negative cumulative impact will be manifested especially for species that have an unfavorable conservation status (see table 5) The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species were observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and loss of individuals of this species, especially during the migration period or on the routes to the feeding areas. The project does not propose overhead power lines, all electrical connections between the turbines and with the transformer station will be underground. The increase in the noise level generated during the execution and operation period as well as the presence of machinery and workers during the execution and operation period (for repairs and maintenance) can have an impact on the size of the population, lead to the disruption of the species' activity and its removal. The noise level around the wind turbines will be around 55 dB and at the limit of the 40 dB wind farm, it is not estimated that the wind farm will be a significant disruptive factor for the activity of the species. The turbines closest to the site boundary are located at distances of more than 3 km. |
|  |  | Regularly migrating species Species associated with extensively used agricultural land Alauda arvensis (Woodpecker) Carduelis cannabina (Hemp) Carduelis carduelis (Bottles) Coturnix coturnix (Quail) Emberiza melanocephala (Black-headed press) Falco tinnunculus (Red Hawk) Fringilla montifringilla (Winter Finch) Galerida cristata (Woodpecker) Lanius Excubitor (Large shroud) Senator Lynch (Red-headed shrike) Merops apiaster (Prigorie) Miliaria calandra (Sura press) Motacilla alba (White codobate) Motacilla flava (Yellow codobate) Oenanthe isabellina (Eastern Stone) Oenanthe oenanthe (Pietrar sur) Saxicola torquata (Black Bramble) Streptopelia tortur (Turtle) Sturnus roseus (alternative name Pastor roseus) (Grasshopper) Turdus iliacus (The Thrush of the Vineyards) Hoopoe Epops (Pupa) Species associated with forest habitats Carduelis chloris (Florine) Carduelis spinus (Box) Coccothraustes coccothraustes (Botgros) Falco subbuteo (Swallow Falcon) Ficedula hypoleuca (Black Muskar) Fringilla coelebs (Song) Jynx Torch (Captorors) Muscicapa striata (Muscar sur) Oriolus oriolus (Grangur) Phylloscopus collybita (Little Dwarf) Phylloscopus sibilatrix (Sizzling Dwarf) Phylloscopus trochilus (Whistling Dwarf) Pyrrhula pyrrhula (Mugur) Regulus ignicapillus (Eyebrow) Regulus regulus (Yellow-headed Owl) Serinus serinus (Cănăraş) Sturnus vulgaris (Graur) Sylvia atricapilla (Black-headed Sylvia) Sylvia curruca (Little Silvie) Turdus merula (Blackbird) Turdus philomelos (Singing thrush) Turdus pilaris (Rooster) Turdus viscivorus (Mistletoe thrush) | Size Population Population trends for each species Area of open terrestrial habitats (extensively used agricultural land) |  | A02.03 Use of grassland for arable land A04.01 Intensive grazing A07 Use of Biocides, Hormones and Chemicals A08 Fertilization - with fertilizer A11Agricultural activities not mentioned above B02 Management and use of forests and plantations B02.02. Deforestation B02.01.02 Replanting Forests (non-native trees) B02.04 Removal of dry or drying trees B06 Grazing in forest areas B03 Exploitation of forests without replanting or natural regeneration C03.03 Wind energy production D02.01.01 Above-ground power lines G01 Outdoor sports and leisure activities, recreational activities E01.01 Continuous urbanization Operation of the Chirnogeni Wind Farm - EP WIND PROJECT(ROM)SIX SA (located north of the Project site, at about 600 m). The park is located at a distance of more than 10 km from the site boundary (downstream of the site) | Yes | No. of affected individuals | Insignificant | A negative cumulative impact will be manifested especially for species that have an unfavorable conservation status (see table 5) The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species were observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and the loss of some individuals from this species, especially during the migration period or on the routes to feeding areas. The project does not propose overhead power lines, all electrical connections between the turbines and with the transformer station will be underground. The occupation of land areas with agricultural use for the location of the constructive components of the energy park leads to a reduction in the areas used for feeding, but these are insignificant compared to the areas available on the site and in the vicinity. In addition, agriculture will continue to be practiced in the area on the rest of the plots within the perimeter of the energy park. The increase in the noise level generated during the execution and operation period as well as the presence of machinery and workers during the execution and operation period (for repairs and maintenance) can have an impact on the size of the population, lead to the disruption of the species' activity and its removal. The noise level around the wind turbines will be around 55 dB and at the limit of the 40 dB wind farm, it is not estimated that the wind farm will be a significant disruptive factor for the activity of the species. The turbines closest to the site boundary are located at distances of more than 3 km. The project does not overlap with the limits of the protected natural area ROSPA0166. However, the project location meets the conditions for species-friendly habitats for feeding or can be a passage lane. The species were observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and the loss of some individuals of these species. The results of the monitoring for the Chirnogeni Wind Farm 2014-2016 show that no mortalities were recorded among bird and bat species. At the same time, these monitoring results mention that the park is not a disruptive factor for the local fauna and bird species, during the monitoring no changes in their activity were observed. |
|  |  | a. Regularly migrating species Species associated with open land/Agricultural land Alauda arvensis (Woodpecker) Buteo lagopus (Shoed Mouse) Emberiza melanocephala (Black-headed press) Senator Lynch (Red-headed shrike) Merops apiaster (Prigorie) Miliaria calandra (Sura press) Motacilla alba (White codobate) Oenanthe oenanthe (Southern Stone) Streptopelia tortur (Turtle) Hoopoe Epops (Pupa) Falco tinnunculus (Red Hawk) Anser albifrons (Great Throat) Crested Gallery (Ciocârlan) Coturnix coturnix (Quail) | Size Population Population size trend for each species Area of open terrestrial habitats (extensively used agricultural land) | Existing pressures | For these The Site Management Plan does not identify specific pressures. The general pressures identified at the site level were taken into account. A02.03 Replacement of pasture with arable land A04.01 Intensive grazing A.07 Use of biocides, hormones and chemicals A.08 Fertilization - with fertilizer A10.01 Removal of hedges and groves or bushes A.11 Agricultural activities other than those listed above B02 Management and use of the forest and plantation B02 01.02 Forest replanting - non-native trees B02.02 Clearing the forest B02.03 Removing the shoot B02.04 Removing dry trees or in the process of drying B03 Exploitation without replanting B06 Grazing in the forest/wooded area D01.02 Roads, motor vehicles D02.01.01 Power lines E01.01 Continuous urbanization E03.01 Storage household waste F02 Fisheries and harvesting of aquatic resources G.01 Outdoor sports and leisure activities, recreational activities G01.03.02 Leadership in Outside of the road of vehicles Motorized H01 Water pollution I01 Non-native/allogeneic invasive species K03.01 Competition - invasive species Operation of the Chirnogeni Wind Farm - EP WIND PROJECT(ROM)SIX SA (located north of the Project site, at about 600 m). The park is located at a distance of more than 10 km from the site boundary (downstream of the site) | Yes | No. of affected individuals Decrease of <1% of the estimated local population | Uncertain Insignificant | A negative cumulative impact will be manifested especially for species that have an unfavorable conservation status (see table 5) The project does not overlap with the limits of the protected area. However, the project's location meets the conditions for habitats favorable to the species. The species were observed in the implementation area of the future energy park. During the period of operation of the works there is a risk of collision and loss of individuals of this species, especially during the migration period or on the routes to the feeding areas. The project does not propose overhead power lines, all electrical connections between the turbines and with the transformer station will be underground. The occupation of land areas with agricultural use for the location of the constructive components of the energy park leads to a reduction in the areas used for feeding, but these are insignificant compared to the areas available on the site and in the vicinity. In addition, agriculture will continue to be practiced in the area on the rest of the plots within the perimeter of the energy park. The increase in the noise level generated during the execution and operation period as well as the presence of machinery and workers during the execution and operation period (for repairs and maintenance) can have an impact on the size of the population, lead to the disruption of the species' activity and its removal. The noise level around the wind turbines will be around 55 dB and at the limit of the 40 dB wind farm, it is not estimated that the wind farm will be a significant disruptive factor for the activity of the species. The cumulative impact is considered uncertain, given the long distance (18.39 km) of the Project site from the site boundary. The impact depends on the management of the site and the application of the measures set out for the conservation objectives through the site management plan. A negative cumulative impact will be manifested especially for species that have an unfavorable conservation status (see table 5) The project does not overlap with the limits of the protected natural area ROSPA0166. However, the project location meets the conditions for species-friendly habitats for feeding or can be a passage lane. The species were observed in the implementation area of the future energy park. Some of these species are also present in the site ROSPA0036 which is at a shorter distance from the limit of the Project. During the period of operation of the works there is a risk of collision and the loss of some individuals of these species. The results of the monitoring for the Chirnogeni Wind Farm 2014-2016 show that no mortalities were recorded among bird and bat species. At the same time, these monitoring results mention that the park does not constitute a disturbing factor for the local fauna and bird species, during the monitoring no changes in their activity are observed. During the period of operation of the wind farm, no other land areas will be occupied besides. Interventions for maintenance and maintenance will not generate a cumulative impact with this pressure on the species, the risks of alteration of the existing habitats in the park area are very low. No other investments are proposed on the analyzed site and in the vicinity. |
|  |  | b. Regularly migrating species Species associated with riparian and littoral habitats Gallinago gallinago (Common snipe) | Size Population Population size trend for each species |  |  |  |  |  |  |
|  |  | c. Regularly migrating species Species associated with urban habitats Hirundo rustica (Swallow) Hirundo rustica (Swallow) | Size Population Population size trend for each species |  |  |  |  |  |  |
|  |  | d. Regularly migrating species Species associated with mixed terrestrial habitats, forests and open lands Accipiter Nisus (Bird Hawk) Buteo buteo (Common mousetrap) Subbuteo falcon (The Swallow Falcon | Size Population Population size trend for each species |  |  |  |  |  |  |
|  |  | is. Regularly migrating species Species associated with reedbed habitats Anser albifrons (Great Throat) Cygnus smell (Summer Swan) Anser anser (Summer Goose) Larus cachinnans (Mediterranean Seagull | Size Population Population size trend for each species |  |  |  |  |  |  |
|  |  | g. Regularly migrating species Species associated with forest habitats Sylvia atricapilla (Black-headed Silvie) | Size Population Population size trend for each species |  |  |  |  |  |  |

Note: Pressures are taken from Site Management Plans

    - Identifying uncertainties

In preparing this Memorandum, a number of uncertainties were identified at the time of its submission in May 2025, but given that the project was subsequently subjected to a strategic environmental assessment at the PUZ (Zonal Urban Plan) stage completed by Decision No. 1428/08.10.2025 of the Constanta County Environmental Directorate, with the Environmental Opinion to be issued, and that the elements of the project remained unchanged, the possible uncertainties identified at the time of drafting the presentation memorandum, prepared prior to the completion of the strategic environmental assessment at the PUZ stage, were eliminated.

The table below presents the list of identified uncertainties and how they were clarified.

Table 22 Analysis of identified uncertainties

| Component | Uncertainties identified |
| --- | --- |
| Project description | The working periods, duration and effort for some of the interventions proposed in the project are not established. The types of works are treated in general, without being able, at this time, to estimate the quantities of excavated, stored materials, the necessary materials, etc. Initial uncertainties regarding the duration and effort of the works were clarified by supplementing the documentation (related to the strategic environmental assessment) with data on the phasing and technological description of the activities. The updates confirm that the works are temporary and controlled in nature, without modifying the project elements and without generating significant effects on the environment. |
| Other plans and projects | For a cumulative impact assessment, the possibility of an impact together with existing works and investments on Natura 2000 sites was considered. No spatial information was available on projects/plans developed on the territory of Bulgaria in the vicinity of the analysed site, and on projects developed in the vicinity of the analysed project. For the cumulative impact assessment, possible interactions between the proposed project and existing works or investments in the area, including those in the vicinity of Natura 2000 sites, have already been analyzed. The analysis was based on public data and field observations. The results indicated that there are no significant cumulative effects on the environment or on the integrity of protected natural areas in the area. |
| Pressures and threats identified for ANPIC | The exact spatial location of the pressures and threats identified in the Standard Forms and Management Plans is not known. The assessment was carried out by correlating general information from management documents with field observations and updated GIS data, resulting in the conclusion that the identified pressures do not significantly interfere with the locations of the analyzed project. |
| Location of habitats and species in relation to the project | The exact distribution of species and habitats of Community interest within protected natural areas is not known. Flight routes are not known for all species on the site. Data on the distribution of habitats and species are not available in vector format and do not allow an overlap with the project boundaries. Thus, it was considered conventional that any species or habitat can have distribution over the entire surface of the protected natural area (the worst possible case). At the SEA procedure level, the assessment was carried out in the most unfavorable scenario, considering their potential presence across the entire area of the analyzed sites, the approach being agreed upon by the authorities of the neighboring state as wel |
| Information on the current value of the parameters of conservation objectives | Quantitative data on habitat area and species population size are not available, and the conservation objectives of adjacent Bulgarian sites have not been officially communicated. The assessment was based on public information, fieldwork, and correspondence with the Bulgarian authorities, applying the worst-case scenario, which confirms the absence of a significant impact on the Natura 2000 sites. |
| Conservation status | The conservation status of some habitats and species is not assessed or updated in official documents. At the SEA procedure level, the assessment was carried out based on field observations and the precautionary principle, considering the worst-case scenario, concluding that the potential impact remains insignificant on the Natura 2000 sites analyzed. |
| Parameter target value | For some of the habitats and species of Community interest, no quantifiable conservation target values have been set. In the absence of such values, the assessment at plan level was carried out through a qualitative and precautionary analysis, based on the current status of habitats and species, concluding that the project does not affect the integrity of Natura 2000 sites. |
| Possibility that the dimension may be affected | - |
| Quantifying the impact | At the SEA procedure level, the potential mortality for bat species is estimated at less than 1 individual/turbine/year (≈ 20–25 individuals/year for the entire park), and the degree of behavioral disturbance is less than 5%, values considered insignificant and without effect on the conservation status of species of Community interest. |

  - Conclusions on the description and quantification of impacts and why it is or is not necessary to continue the procedure with the appropriate assessment study stage

Following the analysis of the potential impact on protected natural areas generated by the construction of the wind farm, it was found that it is necessary to continue the evaluation within an appropriate evaluation study taking into account the following:

| - Loss directly by reducing the area covered by the habitat as a result of its physical destruction: | The proposed energy park does not intersect with the boundary of Natura 2000 sites. The proposed energy park does not overlap with the boundaries of protected natural habitats in Natura 2000 sites. In the vicinity of the Project, at a distance of less than 5 km, a number of 3 natural areas of community interest have been identified. Analyzing the distance and the species/habitats for which these areas were designated, it was identified that a number of 5 protected natural areas on the territory of Romania can be potentially directly or indirectly affected by the project: - Area in the area of direct influence: - ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake at a distance of about 2.79 km from the Project limit. However, the nearest turbines are located at distances of more than 3 km from this area: - T46 - 3.34 km from ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake; - T47 - 3.23 km from ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake; - T35 - 3.33 km from ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake. - 4 protected natural areas in the area of indirect influence: - ROSPA0166 Plopeni-Chirnogeni at a distance of 6.21 km from the Project limit; - ROSPA0036 Dumbrăveni located at a distance of 7.07 km from the Project limit; - ROSPA0094 Hagieni Forest at a distance of 18.39 km from the Project; - ROSAC0157 Hagieni Forest - Bend of the Valley at a distance of 13.81 km from the Project limit. |
| --- | --- |
| - Loss of breeding, feeding, resting habitat of species: | The proposed energy park does not intersect with the boundary of Natura 2000 sites. The land in the perimeter studied by the project falls into the category of land partly occupied by agriculture, with significant areas of natural vegetation, arable land, meadows, watercourses, irrigation canals. Sites ROSCI0071/ROSAC0071,ROSCI0157/ROSAC0157 ROSPA0036, ROSPA0166, ROSPA0094 have been declared protected for bird and bat species. In the area of the wind farm site during the field studies, the presence of bats and birds with protection status at European and national level from Natura 2000 sites was confirmed. These species have a high mobility and can move in the area of the proposed works in search of food or in transit to and from the feeding or resting areas. The project location area can represent a corridor to the feeding areas of bird species, between the limit of the perimeter studied by the project and the limit of the protected area there are agricultural lands and pastures that can constitute feeding areas and allow the movement and migration of species to and from the site. The construction of the energy park will lead to the permanent occupation of land areas outside the protected natural areas, which can represent feeding habitats: - 283.34 ha for the location of wind power plants and complementary constructions and transformer stations; - 26 ha permanently occupied by the arranged exploitation roads. |
| - Alteration/degradation by deterioration of habitat quality, leading to a reduced abundance of characteristic species or to changes in the structure of the biocenosis (species component): | It is not possible to affect the floristic composition of natural habitats by spreading invasive species; - anemocorous path, because the distance between the protected natural areas and the area studied by the project is large enough, between them there are localities and agricultural land. Also, during the execution period, access roads from protected areas will not be used; - by hydrochlorine (the only stream that crosses the site and that crosses the perimeter studied by the project is the Cerchez Creek, they are located downstream of the sites). |
| - Alteration/degradation by deterioration of breeding, feeding, resting habitats of species: | The alteration of habitats, as well as the loss of habitats, can be determined by the permanent or temporary occupation of some areas of land by the proposed works. - 3.61 ha temporarily occupied by site organization + area temporarily occupied by trenches for cable installation; - 114.71 ha of areas permanently occupied by all the constructive elements of the energy park (roads, transformer station, technological platforms and the foundation of the wind farms). A risk of alteration of the habitats of species such as Spermophilus citellus (Popândău), Mustela eversmanii (Steppe ferret) was identified during the excavation and excavation works. |
| - Disturbance by changing existing environmental conditions: displacement of specimens of species, behavioral changes of species: | During the execution period, the disruption of the activity of fauna species is determined by the human presence and road traffic/machinery operation (during the construction period, decommissioning) but also by the actual operation of the wind farm. Noise, vibrations and artificial lighting are the main cause that leads to the disturbance of the activity of fauna species and behavioral changes. Disturbance affects not only nesting, but also specific inter and intra communications, reproduction or feeding of wild animals. Artificial lighting is also a cause for the increase in mortality due to the collision of individuals of bird and bat species with wind farms, as a result of the attractiveness of lighting sources first of all for invertebrate species and then for those that feed on them. At the same time, an effect recorded by the disturbance of species is also the abandonment of feeding areas that can lead to the effective loss of key habitats. In the area of the wind farm site during the field studies, the presence of bat and bird species with protection status at European and national level from Natura 2000 sites was confirmed. The most sensitive bat species identified, but also with a high risk of impact in terms of wind energy is Miniopterus schreibersii (present in ROSCI0071 and ROSSCI0157). |
| - Fragmentation by creating physical or behavioural barriers in physically or functionally connected habitats or by dividing them into smaller and more isolated fragments: | The project does not propose elements that could lead to the creation of physical barriers with consequences on the connectivity of the species' habitats. The distance between the turbines is large enough. However, the cumulative effects of a large number of turbines can be important if birds are excluded from some favorable habitats or if the deviation of travel routes leads to significant increases in energy consumption, given that in the vicinity of the analyzed wind farm there is another wind farm in operation (Chirnogeni Wind Farm). |
| - Reduction of population numbers as a result of direct mortality generated by the project or as a result of other forms of impact: | The study area is not located in protected natural areas of the Natura 2000 Network, but the species with protection status at European and National level from this network are present in the study area (species of bats, mammals, birds). The construction and operation of the park can lead to accidental collisions, increasing mortality among species. The most exposed species are predatory birds that have a gliding flight and bats. From the conclusions of the "Study on the recommendations on the areas in Dobrogea where the location of wind farms should be restricted due to the migration corridors of gliding birds (day raptors, storks, pelicans), respectively due to the wintering of geese and swans", carried out by the National Institute for Research and Development Danube Delta in 2012, it appears that the perimeter studied by the project is located in an area with moderate risk from the point of view of species conservation and natural habitats. In moderate-risk areas. The moderate risk areas amount to approximately 206100 ha, which represents 13.2% of the area of Dobrogea. In moderate-risk areas), long-term monitoring of gliding bird species and/or wintering geese and swan populations is necessary in order to establish conditions for the construction or operation of wind farms. Species for which a significant negative has been identified are: - ROSCI0071: Miniopterus schreibersii (Long-winged Bat) - ROSCI00157: Miniopterus schreibersii (Long-winged Bat) - ROSPA0166: Buteo rufinus (Great Buzzard), Circus cyaneus, Falco columbarius (Winter Falcon) - ROSPA0036: Buteo rufinus (Great Buzzard), Circus cyaneus (Hunted Heron), Circus pygargus (Southern Heron), Falco subbuteo, (Swallow Falcon) Jynx torquilla (Skipjack) - ROSPA0094: Buteo rufinus, (Great Buzzard), Circus cyaneus (Eurasian Buzzard), Circus macrourus (White Buzzard), Circus pygargus (Southern Buzzard), Hieraaetus pennatus (Little Eagle), Pernis apivorus (Wasp), Falco tinnunculus (Red Hawk), Anser albifrons (Great Buzzard), Buteo buteo (Common Buzzard) Falco subbuteo (Swallow Falcon), Anser albifrons. For all species identified in the area of the analyzed site, it is necessary to analyze the risk of collision. |
| - Other indirect impacts through indirect changes in environmental quality: | No other forms of impact could be identified at this time |
| - Uncertainties identified: | The uncertainties identified in the initial analysis stage were clarified through the documentation submitted at the SEA phase of the PUZ procedure. The stages and duration of the works were defined, the exact location of the interventions was specified, the activities were described in technological terms, and the cumulative effects with other projects in the area were assessed. In the absence of complete spatial and quantitative information on habitats, species, and conservation objectives, a conservative approach was applied, considering the most unfavorable scenario. The integrated analysis confirms that the potential impact of the project remains low and insignificant, without affecting the conservation status of habitats, species, or the integrity of Natura 2000 sites. |

XIII.7.    Conclusions of the strategic environmental assessment for the Zonal Urban Plan related to the proposed project

The proposed project was subject to a strategic environmental assessment for the Zonal Urban Plan, necessary for its development.

The assessment of potential significant environmental impacts (taking into account: cumulative impacts, evaluation of alternatives, sustainable development aspects, the impact of related activities in a cross-border context with the Republic of Bulgaria for all environmental factors, including the cumulative impact with neighboring wind farms located on Bulgarian territory, impact on public health) was detailed and analysed in the Appropriate Assessment Study and its annexes, in the Environmental Report and its annexes, and in the Collision Study carried out for bird species.

Thus, a series of mitigation measures were proposed, such as:

•	Conducting regular training for all personnel involved in construction/decommissioning works on general environmental issues, habitats and protected species, and measures to avoid and minimize impact. Particular attention will be paid to issues related to the prohibition of collecting plants and animals or deliberately injuring/killing protected species.

• Construction and installation works shall be planned so as to take place outside the breeding and rearing seasons of species of Community interest, regardless of the presence or absence of nests on site. The planning of activities shall take into account the biological calendar of the species concerned for.

• Regularly trim vegetation around turbines to maintain low abundance of insect species that are a food source for both bats and birds

•    To reduce the risk of mortality, it is proposed to limit the turbines (T47, T46, T35, T36, T34) to a wind speed of 6.5 m/s during the sensitive period (migration), from half an hour before sunset to sunrise and bat protection systems that emit ultrasonic deterrent signals to keep bats away from the rotor sweep area. The measure will apply from the first year of operation of the wind farm.

•	Installation of radar systems and day and night video cameras (on turbines T8, T18, T29, T32, and T37) to detect and prevent the risk of collision of bats with wind farm structures.

•    The diversion signaling system must be installed from the first year of operation of the wind farm.

Furthermore, given that the environmental procedure was cross-border, involving the Ministry of Environment and Water of the Republic of Bulgaria, the latter requested, in accordance with letter no. 99.00. 268-40/17.09.2025, a series of mandatory conditions to be included in the normative act to be issued by the competent authorities in Romania for the investment in question:

•    "Wind farm 48 (46) wind turbines, transformer stations, electrical connection networks, construction and modernization of communication and access roads" – will be implemented in compliance with all measures provided for in the Environmental Report submitted, in order to prevent, reduce, and eliminate negative effects on the environment, the living environment, and the health of the population as a result of the project's implementation;

•    Given that the energy park is located in the immediate vicinity of the border with the Republic of Bulgaria, an assessment of the impact of physical factors in a cross-border context, in particular light effects, will be carried out during the design phase. The assessment will be sent to the Republic of Bulgaria;

•	The paint used for wind turbines (tower and rotor) must be of the "absorbent" type, so as not to create conditions for the reflection of incident light;

After the implementation of the investment proposal, controlled monitoring of noise levels, non-ionizing radiation, and light effects at the border with the affected Bulgarian localities will be ensured. The results of the monitoring will be communicated to the Republic of Bulgaria.

•    After implementation of the investment proposal, controlled monitoring of noise levels, non-ionizing radiation, and light effects will be ensured at the border with the affected Bulgarian localities. The monitoring results will be transmitted to the Republic of Bulgaria.

Considering the above and the fact that the project elements have remained unchanged from the documentation submitted for the Environmental Permit, within the strategic environmental assessment procedure completed by DJM Constanța Decision No. 1428/08.10.2025, it is considered that the environmental impact assessment procedure can be completed by issuing the scoping decision, without the need to prepare a new appropriate assessment study.

- Additional data regarding the classification of the project under the provisions of art. 48 and art. 54 of the Water Law 107/1996

  - Localization of the project in relation to water bodies

    - River basin

The project is located in the Danube-Litoral River Basin, administered by the Dobrogea-Litoral Water Basin Administration (ABADL) and covers an area of approximately 16,501 km², including the entire counties of Constanta and Tulcea. The total hydrographic network managed by ABADL totals 1,623 km of inland watercourses and 341.5 km of the Danube River, as well as natural lakes with a total area of 15,500 ha.

    - Surface watercourses

The project intersects 2 surface water courses (at two points each), namely Cerchez cadastral code XIV_1.40.1.1. and Magura cadastral code XIV_1.40.1.1.

    - Surface water bodies

The project elements do not intersect surface water bodies. The closest bodies of surface water identified in the project area on the territory of Romania are located at distances of more than 28 km (Mangalia - code ROCT01_B2, at a distance of about 29 km, east of the site limit, Eforie Nord-Vama Veche - ROCT02_B2, , northeast of the site limit, at a distance of about 35 km there is also Tatlageac Lake - ROLW15-1_B9,  respectively Sweet Techirghiol Lake - ROLW15.1_B1).

The nearest surface water bodies identified in the project area on the territory of Bulgaria are located south of the site boundary: BG1DJ900R1015 at a distance of about 1 km south of the project site, BG2DO700L017 at a distance of about 34 km, respectively BG2BS000C001 at a distance of about 36 km, south-east of the site boundary.

The following figure shows the location of the project in relation to the surface water bodies.

Figure no. 26 Surface water bodies in the project area

    - Groundwater bodies

The project implementation area overlaps with 3 bodies of groundwater in Romania and is located in the vicinity of two bodies of groundwater in Bulgaria:

- 2 transboundary deep groundwater bodies (RODL04 Cobadin-Mangalia, RODL06 - Wallachian Platform);

- 1 body of groundwater (RODL10 South Dobrogea);

- 2 bodies of groundwater located on the territory of Bulgaria (BG1G000000N049 and BG1G0000J3K051).

Groundwater Body RODL04 – Cobadin – Mangalia (Transboundary Water Body)

The deep groundwater body is accumulated in oolitic limestone deposits and Sarmatian (Kersonian) limestone located in the SE extremity of Dobrogea. The Sarmatian limestone deposits consist of a slab with a thickness of 10-150 m slightly inclined to the east, which counts free-level waters that represent the main source of supply of the coastline south of Eforie Nord.

At the base of the Sarmatian limestone is a bundle of Senonian chalks that represent the impermeable bed of the aquifer.

At the upper part, the Sarmatian aquifer complex is generally covered by Pleistocene permeable loessoid deposits (middle and upper), but locally there are also impermeable clay strata of lower Pleistocene age.

The majority of the groundwater body (85%) is covered with arable land.

Groundwater Body RODL06 - Wallachian Platform (Transboundary Water Body)

This deep body of groundwater has a great extension, partially covering the Wallachian Platform, and is described below in two areas, which show different degrees of knowledge and exploitation:

- the area with development in Southern Dobrogea: The deep aquifer – but partially and with free level (sector adjacent to the Danube) – is confined in Jurassic and Barremian limestone and dolomitic formations, sometimes fractured and karstified, with regional extension (approx. 4500 km2) throughout Southern Dobrudja. The Barremian-Jurassic aquifer, also called the lower aquifer, is confined in the fissured and karstified carbonate formations of Barremian and Upper Jurassic age (Tithonian, Kimmeridgian, Oxfordian). The formations of Jurassic and Barremian age are characterized by a hydraulic communication by means of an aquatard.

- Giurgiu – Calarasi area: A series of study boreholes that were executed in the area bordering the Danube, between Zimnicea - to the west and Feteşti - to the east, intercepted a series of carbonate rocks (compact or cracked limestone, whitish or gray with frequent flint lenses) of Lower Cretaceous and Jurassic age located between depths of 200 – 400 meters. This deep aquifer is strongly ascending, the piesometric level being located at depths between 4 and 12 m. The flow rates obtained by pumping are between 20 – 60 l/s, the specific flows being in the order of 10 – 25 l/s/m. In the Giurgiu area, under the gravels and boulders of the low terrace of the Danube, under the depths between 15-25 m, there are a series of deposits consisting of gray, white or greenish marls and sandstones, attributed to the Upper Cretaceous (Turonian - Senonian).

The RODL06 groundwater body has the surface covered by agricultural land and if chemical fertilizers are applied to them, it is possible to have a negative impact on the chemical status of the groundwater body.

RODL10 Groundwater Body - South Dobrogea (Transboundary Water Body)

The body of groundwater is phreatic, porous-permeable or fissural type, being located in current and subcurrent alluvium (attributed to the Holocene), in loessoid deposits (Upper Pleistocene-Holocene), in loess (Middle Pleistocene-Upper Pleistocene), as well as at the boundary between loess/loessoids/red clays (the latter being attributed to the Lower Pleistocene) and the terminal part of the Sarmatian deposits (Cotu Văii Formation),  Badenian-Upper (Seimeni Formation) or Cretaceous-Lower Cretaceous. Due to the lithological constitution, geomorphological characteristics and structural-tectonic conditions, the body presents great quantitative and qualitative variations, both horizontally and vertically.

Most of the surface of the water body is covered by agricultural land (83%).

Figure no. 27 Groundwater bodies in the project area

  - Indication of the ecological status/ecological potential and the chemical status of the surface water body; for the groundwater body, the quantitative status and chemical status of the water body shall be indicated

The following table shows the ecological status, respectively the chemical status of the surface water bodies in the area where the peoiect is located, based on the data provided by the updated Management Plan of the Danube River, Danube Delta, Dobrogea Hydrographic Area and Coastal Waters (2022-2027):

Table 23 Ecological and chemical status of surface water bodies in the project area

| Name of water body | Water body category | Typology of the water body | Surface water body code | State/ Potential (S/P) | Ecological status/ecological potential | Chemical Status |
| --- | --- | --- | --- | --- | --- | --- |
| Mangalia | CW | ROCT01CAPM | ROCT01_B2 | P | 5 | 2 |
| Eforie Nord-Vama Veche | CW | ROCT02 | ROCT02_B2 | S | 5 | 2 |
| Lake Bugeac | LW | ROLN02 | ROLW14-1-37_B1 | S | 2 | 2 |
| Lake Iortmac | LW | ROLN01 | ROLW14-1-39_B1 | S | 2 | 2 |
| Oltina Lake | LW | ROLN02 | ROLW14-1-39-1_B1 | S | 2 | 2 |
| Danube Lake | LW | ROLN01 | ROLW14-1-39A_B1 | S | 2 | 2 |
| Vederoasa Lake | LW | ROLN02 | ROLW14-1-40_B1 | S | 2 | 2 |
| Sweet Lake Techirghiol | LW | ROLN05 | ROLW15.1_B1 | S | 2 | 2 |
| Salty Techirghiol Lake | HMWB - LW | ROLNPM01T | ROLW15-1_B2 | - | - | 2 |
| Tatlageac Lake | LW | ROLN02 | ROLW15-1_B9 | S | 2 | 2 |

Column "Water body category": LW – lake, RW natural river/ CAMP river/ artificial river

"Status/Potential" column: S – ecological status; P – ecological potential

Column "Ecological status/ecological potential": 1- very good ecological status; 2- good ecological status/maximum and good potential; 3- moderate ecological status/moderate potential, 4- poor ecological status/weak potential; 5- Poor environmental status/potentially bad

The current quantitative and chemical status19F  of the groundwater bodies intersected by the project can be found in the following table:

Table 24 Current quantitative and chemical status of groundwater bodies in the project area

| Groundwater body name | Body code groundwater | Environmental objective |  | Current quantitative status | Current chemical status |
| --- | --- | --- | --- | --- | --- |
|  |  | Quantitative status | Chemical status |  |  |
| Cobadin -Mangalia | RODL04 | B | B | B | S |
| Wallachian Platform | RODL06 | B | B | B | B |
| Southern Dobrogea | RODL10 | B | B | B | S |

  - Indication of the environmental objective(s) for each identified body of water, specifying the exceptions applied and the related deadlines, as applicable

The environmental objectives established within the updated Management Plan of the Dobrogea Hydrographic Area, Danube Delta and Coastal Waters (Cycle III), for the surface water bodies identified in the project area are presented in the following table. For these, exceptions have been established to extend the deadlines for achieving environmental objectives with a time horizon of 2022-2027.

Table 25 Environmental objectives set on surface water bodies identified in the project area

| Name of water body | Water body category | Typology of the water body | Surface water body code | Environmental objective |  | Deadline for achieving the environmental objective |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ecological status | Chemical status | Ecological status | Chemical status |
| Mangalia | CW | ROCT01CAPM | ROCT01_B2 | B | B | 2027 | 2020 |
| Eforie Nord-Vama Veche | CW | ROCT02 | ROCT02_B2 | B | B | 2027 | 2020 |
| Lake Bugeac | LW | ROLN02 | ROLW14-1-37_B1 | B | B | 2020 | 2020 |
| Lake Iortmac | LW | ROLN01 | ROLW14-1-39_B1 | B | B | 2020 | 2020 |
| Oltina Lake | LW | ROLN02 | ROLW14-1-39-1_B1 | B | B | 2020 | 2020 |
| Danube Lake | LW | ROLN01 | ROLW14-1-39A_B1 | B | B | 2020 | 2020 |
| Vederoasa Lake | LW | ROLN02 | ROLW14-1-40_B1 | B | B | 2020 | 2020 |
| Sweet Lake Techirghiol | LW | ROLN05 | ROLW15.1_B1 | B | B | 2020 | 2020 |
| Salty Techirghiol Lake | HMWB - LW | ROLNPM01T | ROLW15-1_B2 | B | B | 2020 | 2020 |
| Tatlageac Lake | LW | ROLN02 | ROLW15-1_B9 | B | B | 2020 | 2020 |

Table 26 Environmental objectives set on groundwater bodies identified in the project area

| Groundwater body name | Body code groundwater | Environmental objective |  | Deadline for reaching environmental objective |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Quantitative status | Chemical status | Quantitative status | Chemical status |
| Cobadin -Mangalia | RODL04 | B | B | 2020 | 2027 |
| Wallachian Platform | RODL06 | B | B | 2020 | 2020 |
| Southern Dobrogea | RODL10 | B | B | 2020 | 2027 |

- Additional data regarding the classification of the project under the provisions of art. 48 and art. 54 of the Water Law 107/1996

  - Features of the project

- Size and design of the entire project

The present project involves the construction of a wind farm (wind turbines), foundations, technological platforms, access roads, internal electricity grid, 33-110 kV transformer stations and site organization on an area of 3027.77 ha. The land on which the investment is to be made is located in Cerchezu commune. The location of the wind farm will be arranged exclusively on the surface of land with the category of agricultural-arable use, pasture, unproductive land, exploitation roads.

- Cumulation with other existing and/or approved projects

The most relevant projects proposed to be carried out in the vicinity of the present analyzed project, according to the information available in various online sources and which could contribute to a cumulative impact are presented in chap. III.6.18. The environmental aspects on which a cumulative impact could be generated are represented by biodiversity, population and human health, noise, air and soil.

- Use of natural resources, in particular soil, land, water and biodiversity

The main natural resources used for the implementation of the project are water and natural aggregates (ballast, broken stone and sand).

In the operation phase of the objective, the technological process of producing electricity in the wind power plant does not involve the use of natural resources.

- Quantity and types of waste resulting

The main types of waste, the estimated quantities generated, as well as how they are managed, are presented in Section VI.1.8.1.

- Pollution and other negative effects

Most of the effects will be generated during the execution works, but they will be temporary and reversible. During this period, emissions of atmospheric pollutants (mainly particles) and noise from machinery and means of transport and from construction activity will be generated.

- Risks of major accidents and/or disasters relevant to the project concerned, including those caused by climate change, according to scientific information

No risks of major accidents or disasters relevant to the analyzed project were identified.

- Risks to human health

Given the nature of the project, a significant negative impact cannot be generated during the execution phase. Discomfort may be generated for the population as a result of the transport of turbine components, but also of the raw materials necessary for the implementation of the project, but this activity is not able to generate a significant negative impact.

In the operation phase of the project, in terms of the maintenance works that will be necessary at the wind farm, a significant negative impact cannot be generated, taking into account the short execution time but also the small number of machines required.

From the point of view of the operation of the wind farm, discomfort could be generated on the population of the nearest localities towards the turbines (up to 1 km), as a result of the appearance of the shading phenomenon, especially for the nearest homes, thus generating a significant negative impact.

  - Location of the project

The ecological sensitivity of geographical areas likely to be affected by projects must be taken into account, in particular with regard to:

- Current and approved land use

The project is carried out on an area of 114.71 ha. The land on which the investment is to be made is located in Cerchezu commune. The location of the wind farm will be arranged exclusively on the surface of land with the category of agricultural-arable use, pasture, non-productive land, exploitation roads.

- The relative richness, availability, quality and regenerative capacity of natural resources, including soil, land, water and biodiversity, in the area and its underground

From the point of view of the location of the project, it does not overlap with sensitive areas/areas or areas of importance for biodiversity or rich in resources. It is specified that the project does not intersect Natura 2000 sites, but is in their vicinity.

- Absorption capacity of the natural environment, paying special attention to the following areas

- Wetlands, riparian areas, river mouths

The project is not located near wetlands or river mouths, but is located in the vicinity of surface watercourses.

- Coastal areas and the marine environment

This is not the case. The project is located at a great distance from the coastal and marine areas of Romania and is not able to affect them.

- Mountain and forest areas

The project is not located in a mountainous area with forest habitats.

- Protected natural areas of national, community, international interest

From the point of view of protected natural areas, a number of 3 natural areas of community interest have been identified: one area located on the territory of Romania ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake at a distance of about 2.79 km from the project limit and two areas located on the territory of Bulgaria - BG0000569 Kardam (Bulgaria) located at a distance of about 410 m from the project limit and BG0000570 Izvorovo - Kraishte located at a distance of about 3.64 km of the project limit).

The nearest protected natural area of international interest is the RAMSAR site – RORMS0017 Ostroavele Danube - Bucgeac - Iortmac, located 3.67 km from the project.

Details of the protected natural areas are set out in section VI.1.6 of the presentation memorandum.

- Areas classified or protected according to the legislation in force: Natura 2000 sites designated in accordance with the legislation on the regime of protected natural areas, conservation of natural habitats, wild flora and fauna; the areas provided by the legislation on the approval of the National Spatial Planning Plan - Section III - protected areas, the protection areas established according to the provisions of the legislation in the field of waters, as well as the one on the character and size of the sanitary and hydrogeological protection areas

The project is not able to affect the protection areas established according to the provisions of the water legislation.

- Areas where there have already been cases of non-compliance with environmental quality standards laid down by national and European Union legislation and relevant to the project or where such cases are considered to exist

This is not the case.

- Areas with a high population density

The wind farm is proposed to be built in the outskirts of Cerchezu commune, the nearest localities being: T32 at 1.1 km from Viroaga, T5 at 1 km from Cerchezu, T35 at 1 km from Magura, T3 at 1.56 km from the Bulgarian locality of Iovkov, T47 at 2.1 km from the Bulgarian locality Kraiste, T9 at 3.98 km from Negru Vodă.

These localities do not fall into the category of areas with a high population density.

- Historically, culturally or archaeologically important landscapes and sites

The project does not intersect any historical monument, nor the 500 m protection area (according to Article 59 of Law no. 422/2001 on the protection of historical monuments). The nearest historical monument, the Căscioarele Movila Bacu Tumulus is located at a distance of about 636 m from T27 and about 1 km from T30. The legionary camp is located about 742.26 m from the T27 turbine.

  - Types and characteristics of potential impact

- Nature of the impact

The implementation of the project will generate an impact on environmental, human health and biodiversity factors.

- Cross-border nature of the impact

The project will generate a cross-border impact, being located at a sufficiently small distance from the country's borders (approximately 400 m from the border with Bulgaria).

- The intensity and complexity of the impact

The intensity of the impact will be significant for the environmental aspect of human health as a result of the appearance of the shading phenomenon on the closest inhabited areas to the location of the turbines (up to approx. 1 km).

In the case of biodiversity, the significance of the impact is uncertain.

- Likelihood of impact

Very likely.

- Expected onset, duration, frequency and reversibility of impact

The impact on human health will start with the operation phase (during the period of operation of the turbines and depending on the weather conditions). The impact on biodiversity will be present both in the construction phase and in the operation phase.

The rest of the forms of impact associated with the project are reversible, except for the occupation of small areas of the land (occupation until the end of the operating period, after which the land will be returned to the initial circuit).

- Cumulation of impact with the impact of other existing and/or approved projects

The impact of the project can be cumulated with other projects in the vicinity of the site that involve construction works, only if they will be carried out simultaneously. Analyzing the types of projects planned in the area (chapter no. III.6.18) it is considered that even if these projects will be carried out simultaneously, no potential significant negative impacts are expected.

- Possibility of effective impact reduction

Where potential significant negative impacts on the environmental components as a result of the implementation of the project were estimated, different sets of measures to avoid and reduce the impact were analyzed, the main ones being presented in Chapter 6 for each environmental component.

ANNEX A - Documents

Annex 1. Certificate of attestation

Annex 2. Beneficiary documents

Annex 3. Property deeds - land registry extracts

Annex 4. Opinions

Annex 5. STEREO coordinates - Shapefile

Annex 6. OSC analysis

Annex 7. OSC decisions

Annex 8. Related studies

ANNEX B – Plans and maps

Plate no. 1. Area classification plan

Plate no. 2. Site plan

DRAWINGS and APPENDICES

www.ramboll.com

www.ramboll.com

https://www.uvp-verbund.de/documents-ige-ng/igc_ni/F825CD8D-6E07-4EF4-A5FF-C34F2FCC6D1D/6.9_Flugbefeuerungssysteme1.pdf

https://www.uvp-verbund.de/documents-ige-ng/igc_mv/d941ac0f-cb63-4acf-9b92-47aa66d6ba21/3%20Anlage%20und%20Betrieb/16%20Anlagenspez.%20Unterlagen/16.1.3%20Sicherheitseinrichtungen/16.1.3.2-2%20Arbeitsanweisung%20Erdungssystem.pdf

https://www.uvp-verbund.de/documents-ige-ng/igc_mv/d941ac0f-cb63-4acf-9b92-47aa66d6ba21/3%20Anlage%20und%20Betrieb/16%20Anlagenspez.%20Unterlagen/16.1.3%20Sicherheitseinrichtungen/16.1.3.2-2%20Arbeitsanweisung%20Erdungssystem.pdf

https://docvia.regione.liguria.it/via/V320/011_Studio%20di%20impatto%20ambientale/SI_011_C_Studio%20di%20impatto%20ambientale.pdf

https://eea.government.bg/bg/soer/2023

https://eea.government.bg/bg/soer/2023

” Studiu privind recomandările asupra zonelor din Dobrogea unde amplasarea centralelor eoliene să fie restricționată din cauza coridoarelor de migrație a păsărilor cu zbor planat (răpitoare de zi, berze, pelicani), respectiv din cauza iernării gâștelor și lebedelor”, realizat de Institutul Național de Cercetare-Dezvoltare Delta Dunării, 2012

Anexa la Ordinul ministrului mediului, apelor și pădurilor nr. 2.480/2022 privind aprobarea Planului național de acțiune pentru conservarea și managementul populației de gâscă cu gât roșu - Branta ruficollis, în perioada

Notă: nu are în vedere doar zona de influență a proiect, ci toate ANPIC posibil afectate de implementarea proeictului.

Notă: nu are în vedere doar zona de influență a proiect, ci toate ANPIC posibil afectate de implementarea proiectului.

Notă: nu are în vedere doar zona de influență a proiectului, ci toate ANPIC posibil afectate de implementarea proiectului.

Note: it does not only take into account the area of influence of the project, but all ANPICs possibly affected by the implementation of the project.

https://apmct.anpm.ro/biodiversitate

https://apmct.anpm.ro/biodiversitate

https://apmct.anpm.ro/biodiversitate

https://apmct.anpm.ro/biodiversitate

Updated Management Plan of the Danube River, Danube Delta, Dobrogea Hydrographic Space and Coastal Waters, 2022-2027