Beneficiary SC South Wind SRL Document type Environment Report Date August 2025

Environmental Report Zonal Urban Plan Zonal Urban Plan Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads loc. Cerchezu, Constanta county

| Document type: | Environment Report |
| --- | --- |
| Beneficiaries: | South Wind SRL |
| Elaborated : Field activity and special contributions: | Ruxandra Florentina Buga Oana Falup Dan George Constantin Vlad Dinu |
| Verified: | Gabriela Mussat |
| Approved | Ileana Fălcescu |
| Project name: | Zonal Urban Plan Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads |
| Date | Revision August 2025 |

CONTENTS

Environmental Report

Zonal Urban Plan Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads

1.	Presentation of the contents and main objectives of the Zonal Urban Plan (PUZ) and its relationship with other relevant plans and programs	5

1.1.	Presentation of the contents and main objectives of the PUZ	5

1.1.1.	Content of the PUZ	5

1.1.2.	Objectives of the PUZ	8

1.2.	Relationship with other relevant plans and programs	11

2.	Relevant aspects of the current state of the environment and its likely evolution in the case of non-implementation of the proposed PUZ	29

2.1.	Relevant aspects of the current state of the environment	29

2.1.1.	Water	29

2.1.2.	Air	34

2.1.3.	Soil/Subsoil	40

2.1.4.	Use of natural resources	44

2.1.5.	Waste management	45

2.1.6.	Noise and vibration	46

2.1.7.	Biodiversity	47

2.1.8.	Population and human health	60

2.1.9.	Landscape	67

2.1.10.	Climate change	72

2.1.11.	Cultural heritage	84

2.1.	Evolution of the state of the environment in case of non-implementation of the proposed PUZ	86

3.	Environmental characteristics of the area likely to be significantly affected	88

4.	The environmental protection objectives, set at national, Community or international level, which are relevant to the PUZ and how these objectives and any other environmental considerations have been taken into account during the preparation of the PUZ	90

4.1.	Environmental protection objectives set at national, Community or international level which are relevant to the plan or program	90

4.2.	How these objectives and any other environmental considerations have been taken into account during the preparation of the PUZ	92

5.	Existing environmental issues that are relevant to the PUZ, including in particular those related to any areas of particular environmental importance, such as special bird protection areas or special conservation areas	98

6.	Potential significant environmental effects	100

6.2.	Evaluation methodology	100

6.3.	Potential significant effects of the alternative without the implementation of the PUZ (Alternative "zero")	106

6.4.	Potential significant impacts alternative to the implementation of the PUZ - selected option	108

7.	Possible significant environmental, including health, effects in a transboundary context	125

8.	Proposed measures to prevent, reduce and compensate as fully as possible any adverse environmental effects of the implementation of the PUZ	149

9.	A statement of the reasons which led to the selection of the variants chosen and a description of how the evaluation was carried out, including any difficulties (such as technical deficiencies or lack of know-how) encountered in processing the required information	155

10.	Description of the measures envisaged to monitor the significant effects of the implementation of the PUZ	159

11.	Non-technical summary	162

Introduction

This Environmental Report describes and evaluates the possible significant environmental effects of the " Plan Urbanistic Zonal Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads".

The Zonal Urban Development Plan was initiated in order to establish the conditions of location, sizing, technological compliance and urban planning regulation for the "Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads".

Holder of the plan "Plan Urbanistic Zonal Wind energy park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads" is SOUTH WIND S.R.L., with registered office in Bucharest, Sector 1, Str. Turda, Nr. 122, Bl. 39, Sc. B, Et. 2, Ap. 54.

General Designer ARHICO CONSULTING S.R.L., with registered office in Constanța, Bld. Mamaia, Nr.68, Bl.Bi4, Sc. B, Et. 2, Ap. 35, Constanța County.

"Wind energy park 48 (46) wind power plants approx. 316,8MW (303,6MW), transformer stations, electrical connection networks, construction and modernization of communication and access roads" will be located in the extravilan, in the western and eastern part of the administrative territory of Cerchezu commune, at the border of Chirnogeni, Independența, Dumbrăveni and Negru Voda communes.

APM Constanta decided by the initial classification Decision no. 32/12.04.2024, that for this plan it is necessary to go through the environmental assessment procedure, the appropriate assessment procedure and the environmental assessment procedure in a transboundary context, taking into account the classification of the PUZ in the provisions of art.5, paragraph 2 of HG 1076/2004, the results of the assessment of the criteria set out in Annex 1 of HG 1076/2004 and the possible effects that it could have on protected natural areas. Thus, the PUZ will be submitted to the adoption procedure with environmental opinion.

In order to go through the environmental procedure it was necessary to prepare the Environmental Report and the Adequate Assessment Study.

The environmental assessment is an integral part of the procedure for adopting plans and programs that may have significant effects on the environment, the procedure for its realization being regulated by HG 1076/2004 on establishing the procedure for carrying out the environmental assessment for plans and programs. This decision transposes into national law the provisions of Directive 2001/42/EC of the European Parliament and of the Council of 27.06.2001 on the assessment of the effects of certain plans and programs on the environment (SEA Directive).

The environmental report identifies, describes and evaluates the potential significant environmental effects of the implementation of the Zonal Urban Plan, as well as its alternatives, taking into account the objectives and the geographical area of coverage.

The Environmental Report was prepared in accordance with the provisions of Annex 2 to GD 1076/2004, the recommendations of the Generic Guide for the Strategic Environmental Assessment (SEA) procedure and the Manual for the application of the procedure for carrying out the environmental assessment of plans and programs, prepared by the Ministry of Environment and Water Management in collaboration with the National Agency for Environmental Protection.

For the realization of the Adequate Assessment Study, the provisions of the Methodological Guide on the adequate assessment of the potential effects of plans or projects on protected natural areas of community interest approved by Order 1682/2023 were respected. The conclusions of the Adequate Assessment Study are presented in the Environmental Report.

The Environmental Report and the Adequate Assessment Study have been prepared by SC Ramboll South East Europe SRL, a company certified by the Ministry of Environment, registered in the Register of certified experts for the preparation of environmental studies, for the following areas and studies: Environmental Impact Assessment Report (RIM-2, RIM-3, RIM-7, RIM-11a, RIM-11b, RIM-11c); Site Report (RA-7; RM-3, RM-11a), Environmental Report (RM-11b, RM-11c, RM-13b); Safety Report (RS-2, RS-11c); Environmental Balance (BM-2, BM-11b, BM-11c); Adequacy Assessment Study (EA); Climate Change Study (EGSC) - Certificate no. RGX 333/1.08.2022 (see Annex 1).

# Presentation of the content and main objectives of the Zonal Urban Plan (PUZ) and its relationship with other relevant plans and programs

  - Presentation of the content and main objectives of the PUZ

The Zonal Urban Plan was initiated in order to establish the conditions of location, sizing, technological compliance and urban planning regulation for the "Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads".

    - Content of the PUZ

The commercial company SOUTH WIND SRL intends to develop on the territory of Cerchezu municipality a wind energy park consisting of 48 (46) wind power plants with a total capacity of approx. 316,8 (303,6) MW.

In the process of realizing the PUZ 4 alternatives were studied:

- Alternative "zero" - no PUZ implementation

- Alternative I - was initiated in 2011 (consisting of 3 wind farms) - 117 wind power plants with a total installed capacity of 351 MW

- Alternative II - started in 2022 - 48 wind power plants with a total installed capacity of 316.8 MW

- Alternative III - was initiated in 2023 after obtaining the TRANSGAZ SA approval no. 34618/948/10.05.2023, as a result of which 2 power plants T17 and T23 were abandoned, thus the new wind farm will comprise 46 wind power plants with a total installed capacity of 303.6 MW

The content description of the PUZ includes the optimal alternative selected for implementation, namely Alternative III.

The proposed power plant will be located in the extravilan, in the western and eastern part of the administrative territory of Cerchezu commune, on the border with the communes of Chirnogeni, Independența, Dumbrăveni and Negru Voda.

The location of the energy park is in the immediate vicinity of the border line with Bulgaria.

In the area, in the vicinity of wind power plants T1, T4, T12, T26, T27 and T28 there are irrigation canals, which are not operational.

The area studied for the implementation of the objective includes agricultural land used as arable land and land with special destination - exploitation roads in the public domain of the territorial administrative unit of Cerchezu commune, managed by the Local Council of Cerchezu commune and the public domain of county interest. The beneficiary of the investment SOUTH WIND S.R.L. has concluded contracts of cession and superficies for the land on which the investment proposed by the PUZ is to be realized.

The total area of the proposed plan is 3,027.77 ha.

On the basis of TRANSGAZ SA's authorization no. 34618/948/10.05.2023, two sites, T17 and T23, which did not comply with the legal distances from the main pipelines identified in the authorization, were abandoned. The resulting wind energy park will consist of 46 wind power plants of 6.6 MW each, totaling 303.6 MW.

The plots of land on which the investment to be developed are represented by the parcels shown in the following table (see Situation Plan, Annex 3):

| Investments | Go to |  |
| --- | --- | --- |
| Wind turbines | - T1 - A 439/2/15 - T15 - A 208/1/20 - T2 - A 411/1/10 - T16 - A 208/1/13 - central T3 - A 439/1/16 - T17 - A 78/22+A 78/23 - canceled - central T4 - A 439/3/3 - T18 - A 177/23 - T5 - A 208/2/3 - T19 - A 177/10 - T6 - A 430/2/15 - T20 - A 169/27 - T7 central - A 98/2/4/2 - T21 - A 359/12 - T8 - A 430/1/17 - T22 - A 359/3 - T9 - A 98/1/8 - T23 - A 359/25 - canceled - T10 - A 207/7 - T24 - A 363/2/9 - T11 - A 195/25 - T25 - A 363/1/23 - T12 - A 208/2/21 - T26 - A 72/4 - T13 - A 179/10/1 - T27 - A 90/9 | - T14 - A 174/11 - T28 - A 363/2/30; - T29 - A 367/49; - T30 - A 72/34; - T31 - A 367/12/2 - T32 central - A 9/12 lot1,A9/12 lot2,A9/12 lot3 - T33 - A 78/10; - T34 central - A 1/20/1, A 1/20/2 - T35 - A 277/2/11 - T43 - A 338/2/6 - T36 - A 277/2/3 - T44 - A 338/3/5; - T37 - A 124/11 - T45 - A 335/1/3; - T38 - A 250/26 - T46 - A 314/1/18/1, A 314/1/18/2; - T39 - A 251/28/1 - T47 - A 314/1/12 - T40 - A 250/10 - T48 - A 411/2/4; - T41 - A 338/1/21 - T42 - A 379/7/5 |
| Transformer stations | - SS01 - A5/28 - SS02 - A251/1/26 - canceled - SS03 - A177/16 - newly proposed (near turbine T19) | - -SS03 - A367/49 will be subject to another authorization procedure - -SS04 - A207/11 |
| Site organization (temporary) | - OS1 - A251/1/31 - OS2 - A367/15 - OS3 - A250/9 |  |
| Landscaped access roads on plots | - A163/18/2 - A277/4/7 - A251/31 |  |

Access to the site is from the existing national road (DN), communal roads (DC) and exploitation roads (De) listed below:

| Road type | Site access |
| --- | --- |
| National road | DN38 Constanta - Negru Voda |
| County Road | DJ391 Mangalia - Negru Voda - Cobadin - Tuzla |
| Existing municipal roads | DC13, DC16, DC17, DC18, DC19 |
| Existing logging roads | From 357, From 64, From 68, From 77, From 75/20, From 78/3, From 87, From 78/1, From 94/1, From 163/1,From 181, From 176, From 168, From 173, From169/1, From 172, From 208/1/1, From 198, From 195/16, From 98/1/1, From 98/2/1, From 430/1/1, From 430/2/1, From 432, From 440, From 439/2/1, From 411/1/1, From 439/1/1, From 224/4, From 251/1, From 130/2, From 130/1, From 250/1, From 130/1, From 129, From 128, From 125, From 271, From 258/10, From 379/2/1, From 361, From 356/1, From 356, From 356/2, From 339/1, From 339, From 337, From 336, From 314/1/1, From 315, From 277/8, From 277/7. |
| Landscaped access roads on plots | A163/18/2, A277/4/7 and A251/31. |

The county road Dj 391 is a paved road, the communal roads are partially paved and all other roads are dirt roads.

As the access roads are not designed for heavy traffic, it is necessary to make them suitable in order to provide the necessary turning radius for the trailers used to transport the components of the wind power plants. Access and connecting roads between the wind turbines and the assembly and maintenance platforms will be built on the sites.

Roads and platforms shall be made of bituminized stone laid on a sand bed. Their structure must be able to withstand a maximum load of 12 tons per axle and a pressure of 18,5 tons/m2 . The width of the enclosure roads shall be 4.50 m with a radius of curvature of at least 50.00 m in the access area from the service roads.

Slopes of min. 3% shall be provided for rainfall runoff and radii of curvature of min. 50 m at site access. Each plot shall have direct access to a landscaped farm road. The radii of curvature will be provided by temporary lease of land. The farm access roads to the sites will be created by laying a bituminized stone surface on a sand bed. The access roads, which are in the local public domain, will be built by the beneficiary of the investment on the basis of a contract concluded with Cerchezu Town Hall.

For the realization of the investment it is necessary to cross/sub-cross the main pipelines belonging to Transgaz Media SA only on the route of the De 75/20 operating road.

The wind power plant has the following main components shown in the following figure and table:

Figure 1. 1 Components of a wind turbine

| Nacela Pale Pylon Foundation/Platform | The nacelle is mounted on the upper section of the mast. Inside the nacelle is the generator. The generator is equipped with a rotor and a control system. The rotor has 3 blades, 170 m in diameter. The pillar supporting the nacelle is made of metal and will be 155 m high. The purpose of the foundation is to support the pylon, the rotor, the blades and the nacelle with all the equipment, as well as to transmit the specific loads mentioned to the ground, without producing deformations that compromise the safe operation of the work (without exceeding the bearing capacity of the foundation ground). The foundation of the wind generator set shall be an insulated, concrete slab type, weight-bearing foundation made of monolithic cast reinforced concrete. It is proposed to build technological platforms for the period of installation of the wind generator sets. The platforms will be located adjacent to the road, in the immediate vicinity of each wind generator foundation. The technological platforms will be kept, they will be used in case of possible interventions on the wind generator sets for land leveling, followed by covering with ballast. The maximum height of the site platform will be max. 0,95 m. |
| --- | --- |

Network: For the operation of the wind power plants, no other types of utilities are required. The existing electricity network and cables in the area will be protected both during construction and operation.

The wind farms are connected to each other by an underground cable system (LES) to the 33 kV/110 kV transformer stations located within the study area In order to connect the wind farm to the National Energy System (SEN), an overhead/underground electrical connection will be made from the 33kV/110kV transformer stations to the 110 kV LEA in the area. The electrical connection between the power plants to the 33kv - 110kv transformer stations located on plots A 5/28 and A 177/16 will be buried at a depth of 1.00 - 1.50 m, and will be located in the developed/undeveloped farm roads in the study area.

Site organization: For the realization of the proposed investments 3 site organizations (of temporary character) are necessary on the plots: A251/1/31, A367/15, A250/9. The site organizations will occupy a temporary land area of about 3,000 m2 each.

In order to set up the site organization, the topsoil will be stripped and a drained layer (sand and gravel) will be installed to prevent soil contamination due to accidental run-off. The topsoil will be stored in the vicinity of the site organization in order to reuse it for the restoration of the site after the dismantling of the site organization.

    - Objectives of the PUZ

The objectives of the analyzed PUZ refer to the study of the area and the promotion of an alternative in the use of certain areas of land in the Cerchezu TAU, Constanta County, which will lead to the economic development of the county in order to increase the living standards of the population by attracting significant sustainable investments.

The only development potential with significant economic impact for the area is the almost permanent presence of winds. In addition to this potential, the area can be used for agriculture.

The main objective of the plan is the realization of a wind energy park with the following construction elements:

- 48 (46) SIEMENS GAMESA 6.6 MW SIEMENS GAMESA wind power plants, with a total capacity of approx. 316.8MW (303.6 MW), which will occupy an area of approx. 254,17 - ha.

- 4 (3) 33 kV/ 110 kV transformer substations, which will occupy a total area of approx. 1,70 ha

- access roads to the turbines, area of approx. 36,65 ha permanently occupied (10,65 ha of exploitation roads and 26 ha of landscaped roads).

The implementation of the plan capitalizes both the wind potential of the area by using it as an alternative source of electricity generation and the infrastructure - electricity grids and the readiness of the local community and investors.

In addition to the main objective, there are other objectives of major importance:

- reducing greenhouse gas and air emissions from the power generation sector;

- reducing pollutant emissions to air, water and soil from the energy sector;

- Romania's commitments under the Paris Agreement and the subsequent policy created to fulfill the targets contained in the agreement;

- Developing low GHG electricity generation capacity;

- new electricity generation capacity, under the constraint of achieving the objectives of energy security, competitiveness and decarbonization of the energy sector;

- diversification and flexibility of electricity generation capacities according to the national mix;

- Sustainable exploitation of all types of the country's primary energy resources:

- promoting the transition to a circular and resource-efficient economy.

The general objective of the Zonal Urban Plan consists in:

- OP1 to study the area and promote an alternative in the use of certain areas of land in the Cerchezu TAU for the construction of a wind farm;

In addition to this objective, the following environmental protection objectives were taken into account in the realization of the PUZ

- OP2 Reduce the amount of greenhouse gas emissions from the energy sector;

- OP3 Promote the transition to a circular and resource-efficient economy;

- OP4 Enhance the natural setting and harmonize it with the proposed site;

- OP5 Wind farm development with consideration of environmental impacts and protection of human health;

- OP6 Economic and social growth of the locality;

- OP7 Reduce impacts on biodiversity by ensuring biodiversity protection and conservation measures.

The functional zoning of the study site resulted in the following regulations:

- respecting the boundaries of the plots according to the parcel plans approved by the OCPI;

- siting of wind power plants in compliance with the provisions of the Local Urban Planning Regulation related to the PUZ;

- Strict adherence to the routes of the existing farm roads and modernization of access roads to the site

On the basis of the proposals made by the PUZ resulted functional unit ZA.

The functional sub-areas were established taking into account the following criteria:

- dominant function;

- cadastral boundaries of the related land.

The resulting functional sub-units are:

- ce - sub-area for wind turbines + complementary constructions;

- a - sub-area related to agricultural - arable land;

- st - sub-area of the transformer substation;

- cn - irrigation canal sub-area;

- ps - pasture sub-area;

- np - non-reducing land sub-area

- yes - sub-area related to the developed logging roads;

- cm - major circulation subarea Dj+Dc;

- de - the sub-area related to the working roads.

Table 1- 1 Territorial balance of the PUZ

| Functional zone ZA | Existing |  | Proposed |  |
| --- | --- | --- | --- | --- |
|  | ha | % | ha | % |
| Area wind power plants | - | - | 254,17 | 8,40 |
| Agricultural land | 2.908,24 | 96,05 | 2652,37 | 87,60 |
| Transformer station area | - | - | 1,70 | 0,05 |
| Pasture area | 64,00 | 2,11 | 64,00 | 2,11 |
| Area of non-productive land | 1,87 | 0,06 | 1,87 | 0,06 |
| Irrigation canal area | 11,77 | 0,40 | 11,77 | 0,40 |
| Developed roads | - | - | 26,00 | 0,86 |
| Major traffic zone DJ+DC | 5,24 | 0,17 | 5,24 | 0,17 |
| Area exploitation roads | 36,65 | 1,21 | 10,65 | 0,35 |
| Total study area | 3.027,77 | 100,00 | 3.027,77 | 100,00 |

Physical changes resulting from the implementation of the zoning plan:

The realization of the energy park will lead to the permanent occupation of land:

- 255.87 ha for the siting of wind power plants and ancillary buildings and transformer stations

- 36.65 ha permanently occupied by farm roads (10.65 ha) and landscaped roads (26 ha).

For each plot on which the wind power plants will be installed, the DTAC phase will require the areas occupied by the base of the pillar, the access roads to the plants and the maintenance platforms to be permanently taken out of the agricultural circuit. The energy park will lead to the permanent removal from agriculture of an area of approx. 30 ha and the temporary set-aside of approx. 70 ha.

Each wind farm operates independently of each other, depending on the wind speed range for which it was designed. Wind power plants are designed to operate from wind speeds of 3-4 m/s, for a nominal wind speed of 15 m/s, and is interrupted in operation for wind speeds above 25 m/s.

For the wind energy park there are 3 stages of implementation:

- execution period;

- operating period;

- decommissioning period (closure, demolition)

Main interventions that will result from the wind farm:

| Stage | Intervention type |
| --- | --- |
| Construction | Construction of access roads |
|  | Building technological platforms and foundations |
|  | Installing wind power plants |
|  | Making electrical connections and laying underground cables between the wind generator sets |
|  | Building transformer substations |
|  | Site organization |
|  | Land reclamation work at the end of construction |
| OPERATE | Maintenance and repairs |
| DEFEAT Interventions and activities are similar to those in the construction phase | Site organization |
|  | Dismantling wind farms |
|  | Decommissioning of technological platforms and foundations |
|  | Dismantling electrical connections and removing underground cables |
|  | Decommissioning of transformer stations |
|  | Decommissioning of logging roads |
|  | Land reclamation work on completion of construction |

  - Relationship with other relevant plans and programs

The specific objective of this PUZ is to produce energy from non-polluting sources, thus increasing the overall air quality by reducing pollution from power plants.

From the multitude of national / European plans, strategies and programs that are related to the energy and climate sector, the most important plans, programs, strategies, policies, conventions existing at both national and European level were analyzed in order to identify the aspects and themes common with those of the PUZ subject to the strategic environmental assessment.

The most important relevant documents analyzed are presented in the following table:

Table 1-2: Relationship of the Zonal Urban Plan with other plans and programs

| Strategy / Plan / Program relevant for the PUZ | Summary of the document | Reference period | Link to PUZ/Relevation for PUZ |
| --- | --- | --- | --- |
| European policies: |  |  |  |
| Energy Union Package (2015) A framework strategy for a resilient Energy Union with a forward-looking climate change policy | Energy policy aims to make the EU the world leader in renewable energy The Energy Union strategy has five interrelated and mutually reinforcing dimensions designed to contribute to enhancing sustainability, competitiveness and energy security: - energy security, solidarity and trust; - a fully integrated European energy market; - energy efficiency to support demand moderation; - decarbonizing the economy; and - research, innovation and competitiveness. As part of the decarbonizing dimension of the economy, the EU has set a target of at least 27% for the share of renewable energy consumed in the EU in 2030, ensuring The EU's domestic energy production (in particular renewable energy sources needed for decarbonization) contributes to reducing Europe's dependence on energy imports. | 2015-2030 The European Union is determined to become the world leader in renewable energy, setting a target of at least 27% for the share of renewable energy consumed in the EU in 2030. | The analyzed UZP contributes to the target of at least 27% for the share of renewable energy consumed in the EU in 2030 by increasing the contribution of renewable energy sources. |
| Paris Agreement, 2015 | Through the Paris Agreement, all countries of the world agreed on a collective effort to: - keeping global average temperature increase well below 2°C above pre-industrial levels; - Increasing the capacity to adapt to the adverse effects of climate change and fostering climate resilience and low greenhouse gas emitting development in a way that does not jeopardize food production; - taking the necessary steps to ensure that financial flows are consistent with a move towards low greenhouse gas emission and climate resilient development. Under the agreement, signatories are obliged to draw up national emission reduction plans every five years, setting more ambitious targets with each new plan. | 2015-2050 | The analyzed PUZ contributes to the objective of limiting global warming by creating sources of electricity without CO2 , the main vector of global warming and which can replace fossil fuel sources of electricity. |
| European Green Deal (Green Deal ), 2019 | An ambitious EU package of measures to achieve climate neutrality by 2050. The European Green Deal will transform the EU into a modern, competitive and resource-efficient economy where: - zero net greenhouse gas emissions by 2050, - decoupling economic growth from resource use; - no person and no place left behind. This growth strategy aims to transform the EU into a fair and prosperous society with a modern, competitive and resource-efficient modern economy, with no net greenhouse gas emissions in 2050 and with economic growth decoupled from resource use. A more ambitious target for the EU to reduce its greenhouse gas emissions in a responsible way, i.e. a reduction of at least 50% by 2030 compared to 1990 levels and moving towards 55%. In this context, the sustained decarbonization of the energy system is crucial to achieve the 2030 and 2050 climate targets, and renewable energy sources have a key role to play. | 2020-2050 | In the context of climate neutrality targets, the analysed PUZ contributes to the objective of decarbonization of the energy system by increasing the share of renewable energy sources in the country's energy mix, as well as to the objective of zero net greenhouse gas emissions by 2050 |
| Climate and Energy Card 2030, 2020 Setting a more ambitious climate target for Europe towards 2030 Investing in a climate-neutral future for the benefit of citizens | It includes policy targets and objectives for the period 2021-2030 in a wide range of sectors (energy production, industry, transport, buildings, agriculture, waste, land use, etc.). Decarbonizing both energy supply and demand is key to achieving climate neutrality. Achieving the 55% greenhouse gas reduction target will lead to a new and greener energy mix. By 2030, coal consumption would fall by more than 70% compared to 2015, oil by more than 30% and gas by more than 25%, while the share of renewables would rise. By 2030 it would reach 38-40% of gross final consumption. Overall, this would lead to a balanced trajectory towards climate neutrality by 2050. | 2021-2030 | The analyzed PUZ contributes to the objective of decarbonization of the energy system by increasing the share of renewable energy sources in the energy mix of the country. |
| European climate law, 2021 | It turns the European Green Pact's political commitment to EU climate neutrality by 2050 into an obligation, giving European citizens and businesses the legal certainty and predictability they need to plan for this transition. After 2050, the EU will aim to achieve negative emissions. The law increases the EU's target for reducing greenhouse gas (GHG) emissions by 2030 from 40% to at least 55% compared to 1990 levels. | 2021-2050 | The analyzed PUZ contributes to the objective of reducing greenhouse gas (GHG) emissions by 2030 from 40% to at least 55% compared to 1990 levels by promoting climate neutral energy sources. |
| Ready for 55 legislative package, 2022 | The legislative package includes the revision of 13 interconnected pieces of legislation and 6 new legislative proposals on climate and energy. It addresses the energy aspects of the EU's climate transition through two legislative proposals: - Renewable Energy Directive and - Energy Efficiency Directive. The new binding EU-wide target of 42.5% renewables in the overall energy mix by 2030, up from at least 32% previously. The Council has set the following indicative objectives relevant to the proposed PUZ: - average annual increase of 1.1% t in the use of renewable energy for industry. - at least 49% share of renewable energy in buildings in 2030 The Council included accelerated permitting procedures for renewable energy projects in line with the priorities of the RePowerEU plan. | 2022-2030 | The analyzed PUZ contributes to the objective of achieving a 42.5% share of renewable energy in the overall EU energy mix. |
| REPowerEU Plan , 2022 | The aim is to rapidly reduce Europe's dependence on fossil fuels from Russia by accelerating the transition to clean energy and working together to build a more resilient energy system and a true energy union. It introduces an additional set of actions aimed at: - save energy; - diversify supply; - to rapidly replace fossil fuels by accelerating Europe's transition to clean energy; - intelligently combine investment and reforms. The plan proposes to increase the Renewable Energy Directive target for 2030 to 45%, up from 40% in last year's proposal | Achieving climate neutrality by 2050 | The analyzed PUZ contributes to the goal of massively accelerating and expanding the use of renewable energy for electricity production which will hasten the phasing out of fossil fuel imports from Russia. The analyzed PUZ contributes to the proposed objective of increasing the percentage of energy from renewable sources. |
| MAP - The Union's 2030 Environment Action Program : 8th EAP, 2022 | The new program supports and builds on the environmental and climate objectives of the European Green Pact and six priority objectives: - irreversibly and progressively reducing greenhouse gas emissions and enhancing removals by natural and other sinks in the Union to achieve the 2030 greenhouse gas emission reduction target and climate neutrality by 2050; - enhancing adaptive capacity, building resilience and reducing vulnerability to climate change - promoting a regenerative growth model, decoupling economic growth from resource use and environmental degradation and accelerating the transition to a circular economy; - aiming for zero pollution, including for air, water and soil and protecting the health and quality of life of Europeans; - protecting, conserving and restoring biodiversity and enhancing natural capital, in particular air, water and soil, forest, freshwater, wetland, wetland and marine ecosystems; - Promote environmental sustainability and reduce key environmental and climate pressures related to production and consumption, in particular in the areas of energy, industrial development, buildings and infrastructure, mobility and the food system. | Achieving the 2030 target to reduce greenhouse gas emissions and achieve climate neutrality by 2050. | The analyzed PUZ contributes to the overall objective: a greener, low-carbon Europe by irreversibly and progressively reducing greenhouse gas emissions and aiming for zero pollution |
| Communication from the Commission A Green Deal Industrial Plan for the Net Zero Emissions Era , 2023 | The Green Deal Industrial Plan will capitalize on its strengths: openness, innovation, inclusiveness and sustainability. Under the right conditions, Europe's net zero emission industry will play a vital role in transforming the continent into a green economy. The industrial plan of the Green Deal is based on four pillars: - a predictable and simplified regulatory environment; - faster access to sufficient funding; - skills and - Open trade for resilient supply chains. | 2050 | The analyzed PUZ contributes to the overall objective of transforming the current energy systems by introducing clean technologies to ensure the transition to the zero emission era while ensuring the security of electricity supply to the European Union. |
| EU Biodiversity Strategy to 2030 , 2020 | It aims to put Europe's biodiversity on a path to recovery by 2030 for the benefit of people, the climate and the planet. The overarching ambition is to ensure that by 2050 all the world's ecosystems are restored, resilient and adequately protected. The strategy contains specific commitments and actions to be achieved by 2030: - Creating a larger EU-wide network of protected areas on land and at sea; - Launch an EU nature recovery plan; - Introduce measures to enable the necessary transformational change; - Introduce measures to tackle the global biodiversity challenge. | 2020-2030 | The analyzed PUZ must meet the challenge of ensuring biodiversity protection in operation |
| European Directives / Regulations / Laws |  |  |  |
| European climate law EU Regulation 2021/1119 | Achieving climate neutrality in the Union by 2050. A domestic reduction of net GHG emissions by at least 55% by 2030 compared to 1990 levels | 2020 - 2050 | The analyzed PUZ contributes to the objective of reducing GHG emissions by at least 55% by 2030 |
| Directive on the promotion of the use of energy from renewable sources EU Directive 2018/2001 with subsequent amendments and additions (Reg. EU 2022/759, Dir. EU 2023/2413) | Sets a binding Union target for the overall share of energy from renewable sources in the Union's gross final consumption of energy in 2030 Member States shall collectively ensure that the share of energy from renewable sources in the Union's gross final consumption of energy in 2030 is at least 42.5%. Member States are working collectively to increase the share of renewable energy in the Union's gross final energy consumption to 45% in 2030. Member States set an indicative target for innovative renewable energy technologies of at least 5% of new installed renewable energy capacity by 2030. | 2023-2030 | The analyzed PUZ contributes to achieving the share of renewable energy in the Union's gross final energy consumption in 2030 of at least 42.5%, with a perspective of 45%. |
| Regulation (EU) 2022/869 on guidelines for trans-European energy infrastructure | It establishes guidelines for the timely development and interoperability of trans-European energy infrastructure priority corridors and priority areas that contribute to ensuring climate change mitigation, in particular the achievement of the Union's 2030 energy and climate targets and the objective of climate neutrality by 2050 at the latest, as well as to ensuring interconnections, energy security, market and system integration, competition for the benefit of all Member States and affordable energy prices. It facilitates the timely implementation of projects on the Union list by streamlining, coordinating more closely and speed up permitting procedures and by improving transparency and public participation | 2022-2050 | The analyzed PUZ contributes to the diversification of the national energy infrastructure contributing to climate change mitigation. |
| Regulation (EU) 2022/2577 establishing a framework for the accelerated deployment of renewable energy | Establishes temporary rules of an urgent nature aimed at speeding up the procedure for granting authorizations for renewable energy production. The planning, construction and operation of plants and installations for the production of energy from renewable sources and their connection to the grid, the grid network itself and storage assets are considered to be of overriding public interest and to serve public health and safety | 2022-2030 | The analyzed PUZ can benefit from the provisions of this regulation given the importance of diversification of renewable energy sources. The planning, construction and operation of renewable energy plants, their connection to the grid and the grid itself, as well as storage assets are considered by the European Council to be of overriding public interest. |
| Nature Restoration Law, Proposed Regulation on Nature Restoration, 2022 | It establishes a framework for Member States to implement, without delay, effective, area-based restoration measures that together cover at least 20% of the Union's terrestrial and marine areas by 2030 and all ecosystems in need of restoration by 2050. It aims, among others, to restore habitats and species protected by EU nature legislation and enhanced farmland biodiversity, e.g. butterflies, farmland birds, highly diverse landscape features. It will require Member States to develop national recovery plans that take account of national circumstances. Target all natural habitats, not just those protected under the Birds and Habitats Directives or Natura 2000 | 2022-2050 | The analyzed PUZ must respond to the challenge of ensuring coherence between the objectives of promoting renewable sources and nature restoration. |
| National Policies/ Strategies/ Plans |  |  |  |
| Renewable energy strategy (HG 1535/2003) | The general objectives of the Renewable Energy Strategy are: - integrating renewable energy sources into the national energy system; - reducing the technical-functional and psycho-social barriers in the process of harnessing renewable energy sources, while identifying the cost and economic efficiency elements; - promoting private investment and creating conditions to facilitate access of foreign capital to the renewable energy market; - ensuring the independence of energy consumption of the national economy; - ensuring, where appropriate, energy supply to remote communities by harnessing the potential of local renewable sources; creating the conditions for Romania's participation in the European "Green Certificates" market for renewable energy. | 2003-2010 | The analyzed PUZ contributes to all the general objectives of the Strategy for the use of renewable energy sources |
| Romanian Energy Strategy 2007-2020, (HG 1069/2007) | The general objective is to meet the energy needs both at present and in the medium and long term, at the lowest possible price, appropriate to a modern market economy and a civilized standard of living, in conditions of quality, security of supply, respecting the principles of sustainable development. The strategic objective Energy Security envisages increasing energy security by securing energy resource needs and limiting dependence on imported energy resources. The Sustainable Development strategic objective is to promote the production of energy from renewable resources. Romania's theoretical annual wind energy potential is estimated at 1.2 TWh. The Dobrogea area is framed in solar and wind energy potential | 2007-2020 | The analyzed PUZ is in line with the direction of action of the energy strategy on the promotion of the use of renewable energy resources, in accordance with the European Union practices |
| Romania's Energy Strategy 2022-2030, with a 2050 perspective , in approval | The Energy Strategy has eight fundamental strategic objectives that structure the entire analysis and planning approach for the period 2022-2030 and the time horizon 2050. The objectives of the strategy support the achievement of the national 2030 targets: - 43.9% reduction in emissions from ETS sectors compared to 2005, and a 2% reduction in emissions from non-ETS sectors compared to 2005; - 30.7% share of renewable energy in gross final energy consumption; - 40.4% reduction in final energy consumption compared to the PRIMES 2007 projection. | 2022-2050 | The analyzed PUZ contributes to the fulfillment of the strategic targets assumed at national and European level by increasing the installed capacity of electricity production from wind sources |
| Integrated National Energy and Climate Change Plan 2021-2030 (HG 1076/2021) | The level of ambition for the share of renewable energy of 30.7% in 2030, implying additional RES-E capacity of about 6.9 GW compared to 2015. Measures to develop new SRE capacity and integration with other markets in the region. Decarbonizing the energy sector, by achieving the targets for renewable energy in gross final consumption of electricity as follows: - 564.6 ktoe in 2020; - 828.8 ktoe in 2025; - 1004.9 ktoe in 2030. Projections to 2030 forecast an increase in wind capacity to 5,255 MW. The net power generation capacities from RES to be installed are: - + 822 MW additional installed capacity in 2022 compared to 2020; - + 559 MW additional installed capacity in 2025 compared to 2022; - + 556 MW additional installed capacity in 2027 compared to 2025; - + 365 MW additional installed capacity in 2030 compared to 2027. | 2020-2030 | The analyzed PUZ contributes to the replacement of the existing conventional power generation capacities with low-carbon ones, reaching the net production projected for the period 2022-2030. |
| National Integrated Energy and Climate Change Plan 2021-2030 , draft update 2023 | Romania's target is to reach a share of at least 36.2% of gross final consumption of RES in gross final consumption by 2030, projected to reach 32.3% by 2025. The share of RES in the electricity sector will increase to 55.8% in 2030 as a result of the construction and commissioning of new RES (mainly wind and solar) electricity generation capacity. | 2021-2030 | The analyzed PUZ contributes to u achieving the objective on the share of RES in the electricity sector |
| Law 220/2008 establishing the system for the promotion of energy production from renewable energy sources | It creates the legal framework to expand the use of renewable energy sources. The scheme for the promotion of electricity produced from renewable energy sources shall apply to electricity delivered into the electricity grid and/or to consumers produced from [...] wind energy [...]. The national targets for the share of electricity produced from renewable energy sources in gross final consumption of electricity for the years 2010, 2015 and 2020 are 33%, 35% and 38% respectively. The mandatory annual shares of electricity produced from renewable energy sources benefiting from the promotion scheme through green certificates for the period 2010-2013 are as follows: 2010 - 8.3%; 2011 - 10%; 2012 - 12%; 2013 - 14% | 2008-2020 | The analyzed PUZ contributes to the stimulation of sustainable development at local and regional level and the creation of new jobs related to the processes of utilization of renewable energy sources and reduction of environmental pollution by reducing the production of pollutant emissions and GHG |
| Romania's National Strategy for Sustainable Development 2030 and Action Plan , HG 877/2018, amended by HG 754/2022 | Overall Strategy Objective 7: Clean and affordable energy The energy sector has a central role to play in mitigating global warming, both by gradually shifting from fossil fuels to low GHG-emitting sources, mainly solar and wind, and by increasing the energy efficiency of buildings and cars and switching to electric transport. Target 2030: Strategically support the share of electricity in total household, industrial and transport consumption by setting performance standards for installations and appliances Overall Strategy Objective 11: Sustainable cities and communities Target 2030: Reduce the effects of air pollution on human health and the environment by paying special attention to air quality Objective 13: Action on climate change 2030 Targets: To intensify Romania's efforts to make the transition to a green, low-carbon, climate resilient economy and to mainstream climate change adaptation measures in vulnerable economic, social and environmental sectors Objective 15: Terrestrial life - Protect, restore and promote the sustainable use of terrestrial ecosystems, sustainably manage forests, combat desertification, halt and reverse land degradation and halt biodiversity loss Preserving Romania's position as the most biogeographically diverse country in Europe by further integrating environmental policies into all relevant national and sectoral strategies and by improving environmental infrastructure | 2020-2030 | The analyzed PUZ contributes to the strategic objectives on GHG reduction, clean energy and sustainable communities. |
| National strategy on climate change and low-carbon growth for 2016-2020 National Action Plan for the implementation of the National Strategy on Climate Change and Low Carbon Growth 2016-2020 HG 739/2016 with amendments HG 1215/2023 | The main objective is to reduce GHG emissions from economic activities in line with EU targets and to adapt to the impacts of climate change, both current and future. The SC Strategy adopts quantifiable targets in line with the EU 2030 aspirations: a 40% reduction in total GHG emissions compared to 1990 levels | 2016-2050 | The analyzed PUZ contributes to the objective of reducing GHG emissions by creating an alternative for electricity production to the traditional production from solid fuels In addition, the PUZ contributes to the adaptation of the energy sector by diversifying energy production and, in particular, by including renewable energy sources |
| National Strategy on Adaptation to Climate Change for the period 2022-2030 with a 2050 outlook (NACS) National Action Plan for the implementation of the National Strategy on Adaptation to Climate Change (NSCAP), 2022 | The overall objective of the NCACS is to improve the adaptive capacity and increase the resilience of socio-economic and natural systems to the impacts of climate change, across different areas and timeframes. The NCACS ensures continuity and consistency with the adaptation component of the "National Strategy on Climate Change and Low Carbon Growth for 2016-2030". At the same time, the NACDS aims at sectoral development in line with the principles of the EU's New Adaptation Strategy for Adapting to Climate Change, namely smart, rapid, systemic and connected adaptation at the global scale of action. | 2022-2030 | The analyzed PUZ contributes to the achievement of the specific objective Changes in management or behavior of tourism staff and tourists, measure Use of renewable energy by replacing fossil fuels with energy sources that emit less or no greenhouse gases, such as wind and solar energy |
| Romania's long-term strategy to reduce GHGs Romania Neutral in 2050 (HG 1215/2023) | Romania aims to become climate neutral in 2050, achieving a 99% reduction in net GHG emissions by 2050 compared to 1990 levels. The sectoral targets for reducing net GHG emissions by 2050 under the Neutral Romania scenario include: - Energy sector: According to the Neutral Romania scenario, Romania needs to reduce its net emissions by 78% in 2030 and its non-LULUCF emissions by 67% compared to 1990 in order to be on track to reach climate neutrality in 2050. According to the REF scenario, net emissions will be reduced by 85% in 2050 compared to 1990. In 2019, 69% of the 2050 emission reduction target has already been achieved. By 2035, 98% of the target will have been achieved. - Transport: Cut sectoral emissions by 82% in 2050 compared to 1990 levels; - Buildings sector: Emissions in 2050 78% below 1990 levels; - Industry Sector: 2050 emissions 89% below 1990 levels; - Agriculture: Emissions in 2050 49% below 1990 levels; - LULUCF sector: increase absorption by 14% in 2050 compared to 1990, similar to the level already reached in 2005; - Waste Sector: Emissions in 2050 73% below 1990 levels; The share of RES in gross final energy consumption will be 86.1% in 2050 and 36.2% in 2030 for the Neutral Romania scenario | 2023-2050 | The analysed PUZ contributes to all the objectives of the Strategy on decarbonization of the energy sector |
| National Strategy and Action Plan for Biodiversity Conservation 2014-2020 (HG 1081/2013) | The strategy is an essential reference point for the country's sustainable development. General medium-term action directions 2014-2020: - Action Line 1: Halting the decline of biological diversity represented by genetic resources, species, ecosystems and landscapes and restoring degraded systems by 2020. - Action Line 2: Mainstream biodiversity conservation policies into all sectoral policies by 2020. - Action Line 3: Promote traditional knowledge, practices and innovative methods and clean technologies as supportive measures for biodiversity conservation in support of sustainable development by 2020. - Action Line 4: Improving biodiversity communication and education by 2020. | 2014-2020 | The analyzed PUZ contributes to the rational correlation of development objectives, including investment programs, with the potential and capacity to support biodiversity |
| National circular economy strategy (HG 1172/2022) | It aims to develop strong and sustainable local economies in a globalized world. To support the development of local and regional circular economy strategies linking local and global economies. Encourage the diversification of local economies and efforts to strengthen innovation capacities in all regions. | 2022-2050 | In the realization and operation of the investments related to the PUZ, the principles of circular economy will be applied by using recyclable and sustainable materials for turbine components, designing turbines with modular components that can be easily replaced or upgraded, extending the lifetime of the turbine, minimizing waste, optimizing the use of resources and creating a sustainable value over the life cycle of the wind farm. |
| Regional/county/local Policies/ Strategies/ Plans |  |  |  |
| Sustainable Development Strategy of Constanța County for 2021 - 2027 | In terms of natural and built heritage, Constanța County is a territory endowed with a wide range of natural resources, archaeological remains and climatic conditions favorable to the use of renewable energy systems, and from this point of view there is a trend of increasing investment in the development of natural resources. In addition to concerns for the sustainable exploitation of existing resources, investments are needed and should be prioritized that solve both housing problems or lead to increased quality of life but at the same time accelerate the reduction of non-renewable energy consumption and greenhouse gas emissions. Investments in renewable energy and energy efficiency systems | 2021-2027 | The analyzed PUZ contributes to the achievement of the strategic objective of harnessing the local energy potential by stimulating and supporting economic activities for the production and distribution of energy from renewable sources |
| Spatial Development Plan - Constanta County | The main objective is to create equal development opportunities for the whole territory in the medium term and the prerequisites for long-term sustainable development. Territorial disparities and development gaps mean that population, industrial activities and services are concentrated in 70-80% of the 11 urban localities, especially within a radius of 25-30 km around Constanța. The major problems of territorial development are: - Concentration of population and activities in a small area; - The accentuation of the peripheralization phenomenon at the extremities of the county - Pollution and degradation of environmental factors in the area of the Danube-Black Sea canal and the coastline between Capul Midia and Năvodari; - The low level of development of territorial infrastructure, equipment and endowment of rural areas, the majority in the county; - Degradation of the natural and built heritage due to the lack of administrative measures and regulations and their lack of appropriate use; - Lack of qualified staff in local development management issues. UTS 10, in which Cerchezu commune is located, is isolated from the central area, with dominant agricultural functions (cereal and vegetable production), with a low development of settlements and territory. Wind energy needs to be used to a greater extent, especially in rural areas and isolated households or farms. | 1991-present | The analyzed PUZ contributes to the development of the locality by attracting sustainable investments. |
| Dobrogea Sud-Vest LAG Strategy 2014-2020 , modified 2023 | Rural development objectives: Achieving a balanced territorial development of rural economies and communities, including job creation and maintenance(P1-P6); Ensuring sustainable management of natural resources and combating climate change(P5); Encouraging local development in rural areas; Facilitating the supply and use of renewable energy sources, by-products, waste, residues, residues and other non-food raw materials for biomass. | 2014-2023 | The analyzed PUZ contributes to facilitate the supply and use of renewable energy sources. |
| Local Environmental Action Plan , 2019 | The strategic objective is to improve the quality of life by ensuring a clean environment that contributes to raising the standard of living of the population, improving the quality of the environment, preserving and improving the state of the natural heritage that the county benefits from | 2014-2020 | The analyzed PUZ contributes to the general objective of promoting clean energy and energy efficiency through the use of renewable resources. |

# Relevant aspects of the current state of the environment and its likely evolution in the case of non-implementation of the proposed PUZ

  - Relevant aspects of the current state of the environment

In order to analyze the current state of the environment for this PUZ, the existing regional, county and local information available for consultation at the time of the Environmental Report was used as input data.

The environmental components considered to be relevant for the PUZ are the following: water, air, soil/soil, use of natural resources, waste management, noise and vibrations, biodiversity, population and human health, landscape, climate change and cultural heritage.

The purpose of this chapter is to understand the existing environmental situation and its evolution in the area of implementation of the PUZ and to identify how the PUZ can generate significant effects on the environment.

The assessment of the state of the environment under the conditions of non-implementation of the plan is a requirement of the SEA Directive 2001/42/EC (art. 5, Annex I-b) transposed into national legislation by HG no. 1076/2004 (art.15).

"Wind energy park 48 (46) wind power plants approx. 316.8MW (303.6MW), transformer stations, electrical connection networks, construction and modernization of communication and access roads" will be located in the extravilan, in the western and eastern part of the administrative territory of Cerchezu commune, at the border of Chirnogeni, Independența, Dumbrăveni and Negru Voda communes.

The location of the energy park is in the immediate vicinity of the border line with Bulgaria.

    - Water

From the hydrographical point of view, the studied site is located in the Danube-Littoral Hydrographic Basin. The nearest surface water bodies identified in the area studied by the PUZ on the Romanian territory are located at distances greater than 28 km (Mangalia - code ROCT01_B2, at a distance of about 29 km, east of the site boundary, Eforie Nord-Vama Veche - ROCT02_B2, north - east of the site boundary, at a distance of about 35 km are also located Tatlageac Lake - ROLW15-1_B9, respectively Techirghiol Sweet Lake - ROLW15.1_B1). v.Figure 2.1.

The closest surface water bodies identified in the area studied by the PUZ on the territory of Bulgaria are located south of the site boundary: BG1DJ900R1015 at a distance of about 1 km south of the PUZ site, BG2DO700L017 at a distance of about 34 km, respectively BG2BS000C001 at a distance of about 36 km, south-east of the site boundary.

The following table presents the ecological and chemical status of the surface water bodies in the area of the PUZ, based on the data provided by the updated Management Plan of the Danube River, Danube Delta, Dobrogea Hydrographic Area and Coastal Waters (2022-2027):

Table 2 - 1: Ecological status of surface water bodies in the site area

| Name of water body | Water body category | Water body typology | Surface water body code | Status/ Potential (S/P) | Ecological status/ ecological potential | Chemical status |
| --- | --- | --- | --- | --- | --- | --- |
| Mangalia | CW | ROCT01CAPM | ROCT01_B2 | P | 5 | 2 |
| Eforie Nord-Vama Veche | CW | ROCT02 | ROCT02_B2 | S | 5 | 2 |
| Lake Bugeac | LW | ROLN02 | ROLW14-1-37_B1 | S | 2 | 2 |
| Lake Iortmac | LW | ROLN01 | ROLW14-1-39_B1 | S | 2 | 2 |
| Lake Oltina | LW | ROLN02 | ROLW14-1-39-1_B1 | S | 2 | 2 |
| Lake Danube | LW | ROLN01 | ROLW14-1-39A_B1 | S | 2 | 2 |
| Lake Vederoasa | LW | ROLN02 | ROLW14-1-40_B1 | S | 2 | 2 |
| Sweet Lake Techirghiol | LW | ROLN05 | ROLW15.1_B1 | S | 2 | 2 |
| Salty Lake Techirghiol | HMWB - LW | ROLNPM01T | ROLW15-1_B2 | - | - | 2 |
| Tatlageac Lake | LW | ROLN02 | ROLW15-1_B9 | S | 2 | 2 |

Water body category column: RW - natural river/ CAMP river/ artificial river, LW - natural lake/ reservoir/ heavily modified natural lake/ artificial lake, CW - natural/ heavily modified coastal waters, HMWB = heavily modified water body

Status/ Potential column: S - ecological status; P - ecological potential

Water body typology column: ROCT02 - natural coastal waters,

Column "Ecological status / Ecological potential": 1- very good ecological status; 2- good ecological status/maximum and good potential; 3- moderate ecological status/moderate potential, 4- poor ecological status/poor potential; 5- poor ecological status/poor potential

Chemical status column: 2 = good

Figure 2. 1 Location of the study perimeter in relation to surface water bodies

According to the Management Plan of the Dan Danube Littoral River Basin, at the level of the river basin, no interdependencies have been identified between the watercourses (no surface water bodies have been delimited on these watercourses) that are found in the area analyzed by the PUZ.

In Figure 2.2, it can be seen that the area studied by the PUZ overlaps with 3 groundwater bodies in Romania and is in the vicinity of two groundwater bodies in Bulgaria:

- 2 transboundary deep groundwater bodies (RODL04 Cobadin-Mangalia, RODL06 - Valaka Platform) and

- 1 groundwater groundwater body (RODL10 South Dobrogea)

- 2 groundwater bodies located on the territory of Bulgaria (BG1G000000N049 and BG1G00000000J3K051)

Figure 2. 2 Location of the studied perimeter in relation to groundwater bodies

Groundwater body RODL04 - Cobadin - Mangalia (transboundary water body)

The body of deep groundwater is accumulated in deposits of oolitic and lumaceous Sarmatian (Kersonian) oolitic and lumaceous limestones located in the extreme SE of Dobrogea. The Sarmatian calcareous deposits form a 10-150 m thick plate gently sloping to the east that contains free-flowing waters that are the main source of supply to the coastline south of Eforie Nord.

At the base of the Sarmatian limestones is a package of Senonian chalk that forms the impermeable bed of the aquifer.

In the upper part, the Sarmatian aquifer complex is generally covered by Pleistocene (middle and upper) permeable loessososososic deposits, but locally there are also impermeable clay strata of Lower Pleistocene age.

The majority of the groundwater body (85%) is covered by arable land.

Groundwater body RODL06 - Wallachian Platform (transboundary water body)

This deep groundwater body has a large extent, covering part of the Wallachian Platform, and is described below in two zones, which present different degrees of knowledge and exploitation:

- area with development in South Dobrogea: The deep aquifer - but partially also with free level (sector adjacent to the Danube) - is confined in Jurassic and Barremian limestone and dolomitic formations, sometimes fractured and karstified, with regional extension (approx. 4500 km2 ) throughout the whole South Dobrogea. The Barremian - Jurassic aquifer, also called the Lower Barremian - Jurassic aquifer, is confined within the fractured and karstified carbonate formations of Barremian and Upper Jurassic age (Tithonian, Kimmeridgian, Oxfordian). Formations of Jurassic and Barremian age are characterized by hydraulic communication through an aquitard.

- Giurgiu - Călărași area: A series of study boreholes that were drilled in the area bordering the Danube, between Zimnicea - to the west and Fetești - to the east, intercepted a series of carbonate rocks (compact or fissured, whitish or grayish limestones with frequent lenses of flint) of Lower Cretaceous and Jurassic age lying between depths of 200 - 400 meters. This deep-seated aquifer is strongly uplipositional, with piezometric levels at depths between 4 and 12 meters. Pumping rates are in the range of 20 - 60 l/s, with specific flow rates in the order of 10 - 25 l/s /m. In the Giurgiu area, under the gravels and boulders of the Danube low terrace, at depths between 15 and 25 m, a series of deposits of marls and sandstones of grayish, white or greenish color, attributed to the Upper Cretaceous (Turonian - Senonian).

The groundwater body RODL06 is covered by agricultural land and if chemical fertilizers are applied to the agricultural land it is likely to have a negative impact on the chemical status of the groundwater body.

Groundwater body RODL10 - South Dobrogea (transboundary water body)

The groundwater body is phreatic, porous-permeable or fissural, localized in current and sub-current alluvium (attributed to the Holocene), in loess deposits (upper Pleistocene-Holocene), in loess (middle Pleistocene-upper Pleistocene), as well as at the boundary between loess/loessoids/red clays (the latter being attributed to the Lower Pleistocene) and the terminal part of Sarmatian (Cotu Văii Formation), Upper Badenian (Seimeni Formation) or Lower Cretaceous deposits. Due to its lithologic constitution, geomorphologic characteristics and structural-tectonic conditions, the body shows great variations in quantity and quality, both horizontally and vertically.

Most of the water body is covered by agricultural land (83%).

The environmental objectives of the groundwater bodies, their current quantitative and chemical status are given in the following table:

Table 2 - 2 Environmental objectives of groundwater bodies, current quantitative and chemical status

| Name of groundwater body | Body code groundwater | Environmental objective |  | Current quantitative situation | Current chemical status | Deadline for reaching environmental objective |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Quantitative status | Chemical status |  |  | Quantitative status | Chemical status |
| Cobadin -Mangalia | RODL04 | B | B | B | S | 2020 | 2027 |
| Wallachian Platform | RODL06 | B | B | B | B | 2020 | 2020 |
| South Dobrogea | RODL10 | B | B | B | S | 2020 | 2027 |

Legend: B - good condition S - poor condition

Three watercourses have been identified in the study area, see Figure 2.3:

Figure 2. 3 Location of the studied perimeter in relation to the watercourses

For the implementation of the PUZ it is necessary to undercross the Cerchez (2) and Măgura (2) watercourses as shown in Figure 3. The works will be carried out by horizontal drilling technology at a depth of 1 m between the upper generator of the protection pipeline and the watercourse slope. On completion of the works, the affected areas will be restored to their original condition.

Figure 2. 4 Location of watercourse undercrossing works

In the area, in the vicinity of the wind power plants T1, T4, T12, T26, T27 and T28 there are irrigation canals, which are not operational.

Figure 2. 5 Location of the studied perimeter in relation to irrigation canals

    - Air

      - Sources of local air pollution

The main sources of air pollution in Cerchezu commune are represented by transportation activities, construction/ demolition activities, diffuse combustion sources, agricultural activities, industrial activities.

Transport activities in Cerchezu commune include only road traffic, there are no rail transport lines in the area. Road traffic is mainly on:

- County road DJ391 Mangalia - Negru Voda - Negru Voda - Cobadin - Tuzla, a road that crosses the Cerchezu commune and provides the connection with the other localities at county level;

- communal roads DC19 Măgura - Cerchezu, DC18 Viroaga - Măgura, DC17 Chirnogeni-Viroaga, DC16 DN38-Chimogeni-Plopeni-Negrești, DC13 DN38-Pelinu;

- on the internal streets of the commune connecting to the main roads.

Road traffic as a mobile source of pollution has the largest share in air pollution in the area of Cerchezu Commune, with the highest contribution of tanzit traffic.

The main specific pollutants resulting from road transport activity are: nitrogen oxides (NOx ), sulphur dioxide (SO2 ), carbon oxides (COx ), particulate matter (PM10 , PM2,5 ), volatile organic compounds (VOC).

According to the latest traffic census carried out in 2022 by CESTRIN - Center for Technical Road Studies and Informatics of CNAIR (National Company for Road Infrastructure Administration), on a section of road DN38 (closest to the locality Cerchezu) with a length of 53.817 km, the average daily average of vehicles is around 3750 vehicles per day, of which: 2740 light vehicles (passenger cars), 433 truck-type vehicles, 218 trucks and vans, 98 buses/coaches, 98 minibuses, 24 trucks and derivatives, 18 tractors, 29 bicycles and 14 motorcycles.

Areas along the road and at intersections, particularly where there are overlaps of heavy vehicle traffic, are most exposed to pollution. Other exposed areas are centers of general interest - transit points.

The level of air pollution generated by road transport is variable in time and space. The level of pollution depends on the type of vehicle, the age of the vehicle, the volume of traffic, the type of fuel used, the technical condition of the road and the vehicle. The greatest impact is particularly noticeable in built-up areas and congested traffic arteries. Concentrations of air pollutants are higher in areas with traffic arteries lined with tall buildings (blocks of flats) in a compact form, which prevent the dispersion of pollutants into the atmosphere - a situation which is not specific to the municipality of Cerchezu.

Construction/decommissioning activities in Cerchezu commune can be sources of air pollution by:

- handling activities of earth masses (excavation of fertile soil, digging, filling, grading, loading - unloading, transport), some construction materials and construction waste - undirected stationary sources. The resulting specific pollutants are: dust;

- vehicles and machinery involved in landscaping and transportation of materials and equipment needed for construction works (excavators, compactors, cranes, electric generators) - non-road mobile sources. The main specific pollutants resulting are: NOx , SOx , CO, dust.

According to the data of the local administration , in the period 2019-2023 , the following categories of works are in progress in the area of Cerchezu commune:

- Road infrastructure rehabilitation and modernization works;

- Works for the modernization of the local farm roads;

- School rehabilitation works;

- Works to set up natural gas distribution systems;

- Works to develop sports infrastructure;

- Work to build a kindergarten.

The impact generated by construction/demolition activities is usually temporary and local. The most affected areas are in the vicinity of work fronts and site organizations.

Diffuse combustion sources

Residential and institutional heating represented by individual heating sources (central heating plants, stoves) that use solid fuels (wood) and do not have appropriate flue gas stacks to ensure the dispersion of pollutants into the air can be a source of air pollution.

To these can be added the unauthorized open-air burning of household waste, used tires, plastics, street waste in the urban perimeter, failure to maintain public sanitation, improper disposal of household waste. These sources constitute, by accumulation, a permanent global source of pollution with material dusts, various gaseous pollutants, unpleasant odors, especially noticeable in adverse weather conditions (fog, atmospheric calm, thermal inversion).

The main pollutants from these sources are: CO, SO2 , NO2 ,O3 , PM10 , PM2,5 , NO2 .

Agriculture

Emissions to air associated with agriculture consist mainly of NH3 , particulate matter, NOx , CH4 from animal husbandry (manure management, enteric fermentation) and plant cultivation.

In Constanța County, according to the Annual State of the Environment Report 2022, there is no information available on the quantification of the contribution of agriculture to air emissions.

Commune Cerchezu is part of the Local Action Group Dobrogea Sud-Vest (LAG) along with 9 other communes. Agriculture is one of the main fields of activity in the LAG area, with cereals and technical crops, fruit and vegetables, vineyards, bee-keeping and animal husbandry. Agricultural activities are carried out either a) on large holdings, made up of large properties, but also of small plots of land, which are combined by renting, and consist of cereal cultivation, or b) at semi-subsistence and subsistence level, on relatively small areas, up to about 10 ha. At this smaller scale, agricultural production can be more varied, including vegetables, in garden plots, production of seedlings for the local market and for own consumption.

The lack of irrigation infrastructure limits production to crops that cope with the low rainfall in the area.

No data on agricultural activities were available for Cerchezu commune.

Industrial activities

According to data from the Local Development Strategy 2014-2020 , amended in 2023, many economic enterprises in the area face a lack of capital for investment. Because of this, innovation - often associated with the risk of loss - is slow to penetrate, whether it is new growing technologies or methods of processing, promoting or selling products.

In Cerchezu municipality, the registered companies active in the field of Growing of cereals (excluding rice), leguminous plants and oilseed producing plants, in Retail trade in non-specialized stores, with predominant sale of food, beverages and tobacco, in Tourism and Sea fishing are not included in the category of those leading to significant air pollution.

There are currently no industrial objectives in Cerchezu that are subject to the Industrial Emissions Directive (IED Directive).

      - Local air quality

Air quality monitoring in Constanța County is performed through the automatic air quality monitoring system included in the RNMCA, consisting of 8 stations located according to the specific criteria currently provided by Law 104/2011 on air quality. The structure of this network is:

- station CT-1- traffic station, located in Constanta, Casa de cultură, the respective location being representative in terms of traffic (radius of representativity area max. 100m); the monitored pollutants are SO2 , NO2 , NO , NOx , PM10 Pb, C H66 , CO, Ni, Cd;

- station CT-2- urban background station located in the Fantazio - City Hall Park area, less exposed to traffic and industry; monitored pollutants SO2 , NO2 , NOx , CO, PM2,5 , O3 , benzene toluene, o,m,pxylen, ethyl benzene, Meteorological parameters;

- station CT-3- suburban background station, located in the suburban environment, in the Năvodari Tabăra Victoria urban area (area of representativeness 25-150 km), for the assessment of average pollution levels within a large urban area, due to phenomena produced inside the city, with possible significant contributions due to transport phenomena originating outside the city; The pollutants monitored are SO2 , NO2 , NOx , CO, O3 , Ni, Cd, benzene toluene, toluene benzene, o,m,pxylen, ethyl benzene, Meteorological parameters;

- station CT-4- traffic station, located in Mangalia, in an urban environment (area of representativeness 10-100 km); the monitored pollutants are SO2 , NO2 , NOx , CO, PM10 Pb, C H66 ;

- station CT-5- industrial type station, located in Constanta, in urban environment in Prelungirea Liliacului street (representativeness area 1 km); monitored pollutants are SO2 , NO2 , NOx , PM10 Pb, C H66 , CO, O3 , Ni, Cd, Meteorological parameters;

- station CT-6- industrial type station, located in Năvodari town, in an urban environment, in the area of Lazăr Edeleanu high school (representativeness area 1 km); monitored pollutants are SO2 , NO2 , NOx , CO, PM10 , O3 , benzene, Meteorological parameters;

- station CT-7- industrial type station, located in an urban environment in the area of Medgidia city hall (area of representativeness 1 km); monitored pollutants are SO2 , NO2 , NOx , PM10 Pb, CO, O3 , Meteorological parameters;

- station CT-8- urban background station, located in the urban environment in Mangalia municipality, Crișanei street (area of representativeness 1 km); monitored pollutants are NO2 , NOx , PM10 Meteorological parameters;

The commune of Cerchezu is not within the radius of influence of these monitoring stations, and there are no real-time air quality measurement stations (road transportation) in its area.

At the time of preparation of this Environmental Report, no information on air quality determinations in Cerchezu commune was available.

In order to establish a current air quality situation in the area of the proposed plan, an analysis of air quality maps for the indicators PM10 , PM2,5 , NO2 and O3 , available at European level, was carried out.

Source: processing after European Environment Agency

Figure 2. 6 Annual average for PM10

Source: processing after European Environment Agency

Figure 2. 7 Annual average for PM2,5

Source: processing after European Environment Agency

Figure 2. 8 Annual average for NO2

Source: processing after European Environment Agency

Figure 2. 9 Annual average for NOx

As can be seen in the above maps, no exceedances of the limit values set by the legislation in force (Law no. 104/2011 on ambient air quality) have been found in the project implementation area.

In the area of Cerchezu commune, taking into account the activities currently carried out, air quality can be considered to be generally good, being an open rural area uncrowded by urban constructions and without intense industrial activities.

    - Soil/Subsoil

Soil characteristics

From the geological point of view, the land in the area of the PUZ belongs to the South Dobrogea Platform, between the Central Dobrogea Massif, the Wallachian Platform and the state border with Bulgaria.

The Dobrogea plateau is a relatively rigid plateau, formed on ancient rocks (green shales, granites), Mesozoic and Neozoic sedimentary deposits, strongly eroded by the long action of exogenous factors, with a gentle, gently undulating relief and relatively low altitudes (200-300m).

Critical areas for soil deterioration

According to publicly available sources, there are no historically contaminated sites and no potentially contaminated sites on the administrative territory of Cerchezu commune.

Currently on the administrative territory of Cerchezu the soil quality is not affected by industrial activities. The areas with potential risk of soil pollution are agricultural activities. Inappropriate use of fertilizers can lead to excess of nitrates and phosphates, with effects on soil microflora and their accumulation in vegetation.

According to the County Report on the state of the environment, year 2022 of Constanta County, out of the total area of 707129 ha, registered in the statistical land register according to the data submitted by DAJ Constanta, almost 80% are agricultural land (558204 ha), the remaining 20% are non-agricultural land.

Land use

The land on which it is intended to realize the investment proposed by PUZ are the private property of individuals / legal entities with which the beneficiary of the investment SOUTH WIND SRL, has concluded contracts of assignment and surface.

The total area of the proposed plan is 3,027.77 ha.

The area studied for the implementation of the objective includes agricultural land for arable land and land with special destination - exploitation roads in the public domain of the territorial administrative unit of Cerchezu commune, managed by the Local Council of Cerchezu commune and the public domain of county interest.

The main function of the studied area is agriculture, the agricultural land with current arable function has an area of 2,908.24 ha, representing 96.05% of the total area studied. The land use categories are shown in the following table and figure.

|  | Use categories; ha. Pasture; 64. Irrigation canal; 11,77. Major traffic (DN+Dc); 5,24. Roads (De); 36,65. Unproductive land; 1,87. Arable land; 2908,24. Total; 3027,77. |
| --- | --- |

Figure 2. 10 Land use categories

For each plot of land on which the wind turbines will be installed, the DTAC phase will require the definitive removal from the agricultural circuit of the areas occupied by the base of the pillar, the access roads to the power plants and the maintenance platforms. The energy park will lead to the definitive removal from agriculture of an area of approx. 30 ha.

For the areas occupied by the component parts storage and crane assembly platforms, temporary set-aside will be required. The energy park will lead to the temporary set-aside of approx. 70 ha.

The quality classes and goodness scores of agricultural land are closely linked by assessing soil fertility and productivity. Thus, agricultural land is grouped into 5 quality classes, differentiated by the average bonanza score (class I: 81-100 points, class V: 1-20 points).

The land on which the investments are to be developed on the extravilan territory of Cerchezu commune is of class III quality (on a scale from I to V), i.e. a medium fertile soil, moderately affected by degradation.

Soil categories in the study area

The formation and geographic distribution of soils in the territory of Cerchezu commune were influenced by physical-natural factors, relief, lithology and climate.

The many soil classes and types are distinctly different in their properties, productive capacity and measures to maintain and enhance fertility.

The following figure shows the soil map of the proposed project area:

Figure 2. 11 Map of soils in the PUZ coverage area

Analyzing the soil map, it can be seen that the predominant soil class in the project implementation area is that of cernisols (cernoziomols), about 90% of the total area of the PUZ.

The map with the distribution of land use categories in the PUZ coverage area is represented in the figure below.

Source: CORINE Land Cover, 2018, Version 2020_20u1mDate of publication: Jun 14, 2019 Revision date: May 13, 2020

Figure 2. 12 Distribution of land use categories in the PUZ coverage area

Concerning the distribution of land use categories, the study area represents an area of arable land (majority - 90%), secondary grassland, predominantly agricultural land mixed with natural vegetation, discontinuous urban space and industrial or commercial units.

The Convention on Biological Diversity (CBD) has defined soil biodiversity as "the variation in soil life, from genes to communities and the ecological complexes of which they are part". In other words, soil biodiversity is the diversity of underground life.

A paper recently published by the Joint Research Centre (JRC) maps the potential threats to three categories of soil biodiversity (micro-organisms, fauna and soil biological functions) in the EU and provides guidelines for identifying soils that may be at risk. The JRC concludes that intensive human use/exploitation poses the greatest potential threat, while genetically modified agricultural organisms (GMOs) pose the least threat.

The maps show that most European countries have high-risk soils, with the risk being higher in agricultural areas and the Danube floodplains than in other forested areas.

The biological potential of soil was assessed and mapped, using critical thresholds of certain soil influencing factors such as climate change, land use change, habitat fragmentation, intensive anthropogenic exploitation, soil organic matter depletion, industrial pollution, soil compaction, soil erosion, soil erosion, soil sealing, soil salinization, GMO use in agriculture and invasive species. These can affect soil biological status and indirectly also soil biodiversity levels.

Analyzing the data presented in the JRC study on the potential risks to soil biological functions, it could be observed that in the project implementation area the majority of soils present a high risk in Romania, decreasing to a moderate risk in Bulgaria, thus biological functions are likely to be affected by soil degradation processes.

Source: Joint Research Centre-European Soil Data Centre- Soil Biodiversity; A knowledge-based approach to estimating the magnitude and spatial patterns of potential threats to soil biodiversity

Figure 2. 13 Map of potential risks threatening soil biological functions in the area of implementation of the PUZ

    - Use of natural resources

According to HG 1535/2003 on the approval of the Strategy for the Valorization of Renewable Energy Sources, five distinct wind energy zones have been identified in Romania, according to the existing energy potential, environmental and topographical conditions. Within these regions, the favorable sites for the siting of wind turbines are those which aim at "exploiting the energy from the flow effect over the hilltop or the effect of channelling air currents".

The Dobrogean plateau which is the beneficiary of a "mild" climate is part of one of the wind areas with high energy potential.

|  | Legend. - Danube Delta; Solar energy. - Dobrogea; Solar and wind energy. - Moldova (plain and plains); Micro-hydro, wind, biomass. - Carpathian Mountains; Biomass, Microhydro. - Transylvanian Plateau; Microhydro. - Western Plain; Geothermal energy. - Subcarpathians; Biomass, Microhydro. - South Plain; Biomass, geothermal and solar energy. |
| --- | --- |

Source: Decision No 1069/2007 approving Romania's Energy Strategy for 2007-2020

Figure 2. 14 Map of available renewable resources by region

Cerchezu Commune is located in a region with wind potential, which led to the proposal to develop "Wind Energy Park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads". The development of the project will take into account the challenge of sustainable management of natural resources, thus ensuring a balance between economic development and environmental protection. The implementation of the proposed PUZ offers the opportunity to capitalize on renewable energy resources, contributing to reducing dependence on fossil fuels and reducing greenhouse gas emissions. At the same time, it is essential to adopt appropriate measures to conserve local biodiversity and minimize impacts on natural habitats.

There is no exploitation of natural resources such as sand or stone on the land of the PUZ, which ensures the preservation of mineral resources in the area.

    - Waste management

The impact on the environment of the landfilling of household (municipal, town, city, municipality) or industrial waste is significant. The most affected parts of the environment are soil, air and water (surface and groundwater). These problems are created by poor waste management.

The project "Integrated Waste Management System in Constanta County" has been finalized at the level of Constanta County, with the following objectives:

- Increase the number of population benefiting from quality and affordable waste collection and management services;

- Reduce the amount of landfilled waste;

- Increase the amount of recyclable and recoverable waste in Constanta County;

- Establishing an efficient waste management structure.

According to the County Waste Management Plan Constanta, the index of household waste generation in the planning period is:

Table  2-1 : Evolution of household waste generation rates

| Residence environment | Generation rate (kg/inhabitant x day) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 |
| Urban | 0.64 | 0.64 | 0.63 | 0.62 | 0.61 | 0.60 |
| Rural | 0.29 | 0.29 | 0.28 | 0.28 | 0.28 | 0.27 |

Source: County Waste Management Plan - for Constanta County

In the municipality of Cerchezu sanitation is provided by Iridex Grup Salubrizare S.R.L. .

Sanitation consists of:

- Pre-collection, collection and transportation of municipal waste from household waste in the abovementioned localities

- Separate collection and transportation of municipal waste and similar commercial waste from industry and institutions, including separately collected fractions.

- Collection and transportation of household waste arising from indoor and/or outdoor refurbishment and indoor and/or outdoor activities

- Collection and transportation for recovery of all types of recyclable industrial waste (cardboard and paper, plastics, glass, etc.)

- Sweeping (manual and mechanized), washing (manual and mechanized), sprinkling and maintenance of public roads, scraping and cleaning gutters, cleaning and scraping sidewalks and curbs

Snow clearing and ice control, clearing and transporting snow from public roads and keeping them in operation during icy and freezing weather.
The waste is transported to the Costinești Integrated Waste Management Center which has the following facilities:

- Landfill Total storage capacity: 1 200 000 m3

- The plant for mechanical tarring and sorting of dry fraction of municipal waste has a capacity of 50 000 t/year

- The mechano-biological tartare station has a capacity of about 70 000 tons of waste/year

- Mobile crushing plant with a capacity of 18000 tons/year

Waste collection frequency is: once a week in rural areas.

    - Noise and vibration

The main sources of noise in Cerchezu commune are road traffic on the county road DJ391 and on the municipal roads DC13, DC16, DC17, DC18 and DC19, as well as construction / demolition activities through the operation of machinery used in the execution of landscaping / erection / concrete pouring, vehicles transporting materials.

Noise pollution has a whole range of effects on the human body, from mild auditory fatigue to severe neurotic states and even trauma to the auditory organ. Higher-frequency sounds are more dangerous than low-frequency sounds.

The legal framework for the avoidance, prevention or reduction of harmful effects, including discomfort, caused by the exposure of the population to ambient noise is represented by Law no. 121/2019 on the assessment and management of ambient noise with subsequent amendments and additions, which provides for the noise mapping to determine the exposure to ambient noise and the adoption of action plans for the prevention and reduction of ambient noise, where appropriate, and for maintaining ambient noise levels below the limit values in the situation where they are not exceeded.

At Constanța County level, noise mapping and the elaboration of strategic noise maps and action plans were carried out in the period 2016-2024 as follows:

- The City Hall of Constanta prepared the strategic noise maps for Constanta in 2016, they were redrafted in 2018 for Constanta, Palazu Mare district and Mamaia resort, and in 2024 they were updated for Constanta;

- The National Company "Administrația Porturilor Maritime" S.A. Constanța prepared the strategic noise maps for the Port of Constanța during 2019. The strategic noise maps were approved by Order of the Minister of Transportation no. 1245/05.09.2019. The Noise Action Plan was approved by Order of the Minister of Transport no. 1246/05.09.2019;

- The Ministry of Transport - National Railway Company "CFR" S.A., as administrator of the Palas - Saligny - 49 km railway section, has prepared the strategic noise maps for 2016;

- In 2017, the National Road Infrastructure Management Company prepared strategic noise maps for the main roads with more than 6,000,000 vehicle passages per year: highways, national roads and bypasses.

The coverage area of the strategic noise maps realized at county level does not include the area of Cerchezu commune.

In view of the above and taking into account that the area of Cerchezu commune is an open rural area, uncrowded by urban constructions, without industrial activities and without intense traffic on the internal roads and roads connecting with other localities, it can be concluded that the noise level in the commune is low.

    - Biodiversity

The study area is located in south-eastern Romania on the administrative territory of Constanța County, in the western part of the county, in the immediate vicinity of the border with Bulgaria. The study area is delimited by the localities: Negru Voda (in the east), Căsoaiele and Viroaga (in the north) and Magura (in the east), in the south and upper-western part, the study area is delimited by the state border. The study area is a predominantly agricultural area with natural pasture pastures used for grazing domestic animals. The study area does not intersect with any protected natural areas, the closest of which are shown below.

A number of 3 natural areas of community interest have been identified in the vicinity of the PUZ at a distance of less than 5 km:

- 1 area situated on the territory of Romania ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake at a distance of about 2,79 km from the boundary of the PUZ-

- 2 areas on the territory of Bulgaria - BG0000569 Kardam (Bulgaria) at a distance of about 410 m from the boundary of the PUP and BG0000570 Izvorovo - Kraishte at a distance of about 3.64 km from the boundary of the PUP).

There are 5 natural protected areas within the Plan implementation area at a distance of 5-20 km:

- 5 protected natural areas for birdlife protection:

  - ROSPA0166 Plopeni-Chirnogeni at a distance of 6.21 km from the PUZ boundary

  - ROSPA0036 Dumbrăveni located at a distance of 7.07 km from the PUZ boundary

  - ROSPA0094 	Hagieni Forest at a distance of 18,39 km from the PUZ boundary

  - ROSPA0151 	Ciobănița-Osmancea at a distance of 18,55 km from the PUZ boundary

  - ROSPA0001 	Aliman - Adamclisi. at a distance of 19.12 km from the PUZ boundary

- 2 protected natural areas of Community interest:

  - ROSAC0157 Hagieni Forest - Cotul Văii at a distance of 13.81 km from the PUZ boundary,

  - BG0000130 Kraymorska Dobrudzha (Bulgaria) at a distance of 11 km from the PUZ boundary

ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Vederoasa Lake was declared a site of Community importance by the Order MMDD no. 1964/2007 on the establishment of the protected natural area regime of sites of Community importance, as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

The area has been designated as a Site of Community Importance because it hosts at least 8 types of habitats protected at European level, of which 3 are priority, i.e. strictly protected: pont-sarmatic steppes - about 160 ha in the site, scrublands of pont-sarmatic deciduous trees present in coastal areas, on stony substrate, with limestone on the surface, with a total area of about 200 ha in the site and East European forests of downy oak, whose total area has been estimated at about 900 ha. Within these habitats 58 nationally protected rare plant species have been identified. Four of these are also protected at European level: Centaurea jankae, Echium russicum, Himantoglossum caprinum and Potentilla emilli-popii.

ROSCI0071 Dumbrăveni - Urluia Valley - Lake Vederoasa has an area of 17.971 ha and overlaps ROSPA0036 Dumbrăveni (2056 ha), Dumbrăveni Forest (2361 ha), ROSPA0001 Aliman - Adamclisi (19.468 ha), Aliman Fossiliferous Site (2351 ha), ROSPA0007 Vederoasa Pond (2144 ha), Vederoasa Lake (IV.30), ROSPA0039 Danube - Ostroave (16.224 ha), Bratca Forest (IV.26), Limestone Walls of Petroșani (2350 ha). ROSAC0071(ROSCI0071) Dumbrăveni - Urluia Valley - Lake Vederoasa is located in the territory of the following localities: Vlahii, Aliman, Floriile, Adâncata, Crângu, Adamclisi.

Located in southern Dobrogea, the ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake site presents a rich biodiversity, which is due to a mosaic of different habitats developed on a varied geomorphology: here we find limestone cliffs with caves, rocky hills and loess, valleys with temporary waters, permanent watercourses and lakes. Interspersed with forests we find scrublands and ponto-sarmatic grasslands.

Habitat types present on the site are as follows:

- 62C0* Point-sarmatic stepping stones

- 91I0 * Eurosiberian woodland vegetation with Quercus spp.

- 3150 Natural eutrophic lakes with Magnopotamion or Hydrocharition vegetation

- 40C0* Deciduous broad-leaved deciduous ponto-sarmatitic coppices

- 6430 Lowland, upland to montane and alpine, tall-grass, hygrophilous upland communities

- 91F0 Mixed riparian forests with Quercus robur, Ulmus laevis, Fraxinus excelsior or Fraxinus angustifolia, along large rivers - Ulmenion minoris - -

- 91AA Forest vegetation punctuate-sarmatic with downy oak

- 91M0 Balkan-Pannonian sky and gorun forests

In terms of their importance for the conservation of endangered habitats, the site's forest-scrub formations are of the highest priority. Two plant types have been identified:

- Danubian-west-pontic mixed forests of the western-pontic forests of the red oak Quercus penduculiflora and silver lime Tilia tomentosa with topora Viola jordanii. This type of habitat has a small area of only about 3000 ha nationally and is of very high conservation value. Within the study area they represent 2% of the total area. It is characterized by the plant association Querco penduculiflorae-Tilietum tomentosae. The phytocenoses are edified by sub-Mediterranean and continental European and Caucasian species. The tree layer is composed of the European oak Quercus penduculiflora, silver lime Tilia tomentosa, these two species are edifying for the type of habitat, the European maple Acer tataricum, the European maple Fraxinus ornus, the European ash Fraxinus excelsior, the European ash Sorbus torminalis, the elm Ulmus procera.

- Pontic sylvatic vegetation of scrub oak and Scot's oak and Scot's maple. In the country, it is reported on relatively small areas totaling 16,000 ha in southern and south-eastern Romania. In the rest of Europe this habitat has not been reported so far. The characteristic association observed is Quercetum penduculiflorae-cerris. Edi edifying species observed: prickly maple Acer tataricum, Turkey maple Quercus tataricum, Turkey oak Quercus cerris, European oak Quercus penduculiflora. Threatened plant species in these forests: wood alder Asphodeline lutea, common ash Crocus pallisii, Mercurialis ovata, Myrrhoides nodosa, Pipitatherum virescens.

European protected plant species - Annex II of the Habitats Directive - are present on the site: Centaurea jankae, Himantoglossum caprinum, five-toed weed Potentilla emilii-popii, snake's head Echium russicum.

Of the invertebrate species the order Lepidoptera butterflies is best represented. Of these 6 butterflies are protected at European level under the Habitats Directive: Lycaena dispar, a vulnerable species that prefers habitats with swampy or damp oak forests, rich in Polygonum bistorta roach, which is a plant that is the insect's staple food; Lucanus cervus, whose presence is determined by the old trees in the Dumbrăveni forest; Eriogaster catax moth, a very rarely reported species in Romania; two other species are Catopta thrips and Asthena anseraria.

Among the species of reptiles and amphibians present in the site are the Dobrogean crested newt Triturus dobrogicus, the red-bellied puff-bellied opossum Bombina bombina, the water turtle Emys orbicularis and others. One of the most important species is the Dobrogean tortoise Testudo graeca, declared a natural monument and strictly protected by law. The population of this species is currently in drastic decline.

This site hosts a rich avifauna, with over 100 bird species identified. Some bird species have important flocks such as raptors, larks, woodpeckers, woodpeckers, blackcap and others. which is why it has been designated as a Special Avifauna Protection Area.

The mammalian fauna includes a significant number of species including the otter Lutra lutra lutra, the gopher Spermophilus citellus and many bat species.

ROSPA0166 Plopeni - Chirnogeni. The natural avifaunistic protection area was declared in 2016 by Government Decision no. 663/2016 on the establishment of the protected natural area regime and the declaration of special avifaunistic protection areas as an integral part of the Natura 2000 ecological network in Romania. The site has an area of 137.20 ha being designated for 9 bird species namely: Anthus campestris, Buteo rufinus, Circus cyaneus, Dendrocopos syriacus, Emberiza hortulana, Falco columbarius, Falco vespertinus, Lanius collurio, Lanius minor. The site is aligned from north to south, between the localities of Plopeni and Chirnogeni (Constanța County), along the road linking the two localities. Evening Buzzards (Falco vespertinus) nest in the nests of the seed crow in the walnut trees lining the road. More or less degraded grasslands are found along the sides of the road, also on the site. Also within the site there are significant stands of acacia plantations which may in the future harbor other colonies of red grouse. Surrounding the site are agricultural areas which are the hunting grounds for the evening red grouse.

ROSPA0036 Dumbrăveni was established as a special avifaunistic protection area by GD no. 1284/2007 on the declaration of special avifaunistic protection areas as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

ROSPA0036 Dumbrăveni is of particular importance for its 22 species of birds of European conservation interest, being important because two globally threatened species are present here, the common moorhen Coracias garrulus, which nests here, and the white ib ibis, Circus macrourus.

The site is also important because it is an important nesting area for the EU threatened species Dendrocopos syriacus, Lanius minor, Hieraaetus pennatus, Emberiza hortulana, Lullula arborea, Accipiter brevipes, Milvus migrans, Buteo rufinus. The site is particularly important during migration for raptor species.

ROSPA0036 Dumbrăveni with an area of 2056 ha overlaps with ROSCI0071 Dumbraveni-Valea Urluia-Lacul Vederoasa and Dumbrăveni Forest and is found in the territory of the following localities: Șipotele, Tufani, Furnica, Dumbrăveni, Olteni.

From the avifaunistic point of view, the Dumbrăveni Forest - together with its surroundings - hosts important flocks of protected bird species. The following categories have been identified within the site: 23 species listed in Annex 1 of the Birds Directive; 66 migratory species, listed in the Annexes of the Bonn Convention on Migratory Species; 2 globally endangered species.

The protected area is an important nesting area for numerous species such as the garden woodpecker Dendrocopos syriacus, the garden presura Emberiza hortulana, the black-browed spruce grouse Lanius minor, short-legged hawk Accipiter brevipes, short-tailed tit Anthus campestris, black-headed tit Circus cyaneus, little eagle Hieraaetus pennatus, white-tailed tit Circus macrourus, and the field martin Coracias garrulus, the last two species being globally threatened.

The site is crossed during migrations by numerous species of birds of prey, including the southern heron Circus pygargus, the red heron Circus cyaneus, the white heron Circus macrourus, the peregrine falcon Falco peregrinus, the black jay Milvus migrans.

The 22 species of birds listed in Annex I of Council Directive 2009/147/EC, comprise: A402 Accipiter brevipes, A403 Buteo rufinus, A092 Hieraaetus pennatus, A231 Coracias garrulus, A246 Lullula arborea, A339 Lanius minor, A379 Emberiza hortulana, A084 Circus pygargus, A082 Circus cyaneus, A083 Circus macrourus, A103 Falco peregrinus, A338 Lanius collurio, A242 Melanocorypha calandra, A533 Oenanthe pleschanka, A255 Anthus campestris, A089 Aquila pomarina, A243 Calandrella brachydactyla, A224 Caprimulgus europaeus, A321 Ficedula albicollis, A429 Dendrocopos syriacus, A072 Pernis apivorus, A073 Milvus migrans.

ROSPA0094 Hagieni Forest was established as a special avifauna protection area by Government Decision no. 1284/2007 on the declaration of special avifauna protection areas as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

The site R0SPA0094 Hagieni Forest hosts important populations of protected bird species, i.e. 48 species of Community importance for conservation.

ROSPA 0094 Hagieni Forest has an area of 1374 ha and is located within ROSCI 0157 Hagieni-Cotul Văii Forest, partially overlapping with the Hagieni Forest Reserve (code 2360).

This site hosts important flocks of protected bird species: 48 species listed in Annex 1 of Directive 2009/147/EC of the European Parliament and of the Council of November 30, 2009 on the conservation of wild birds; 38 migratory species, listed in the annexes of Law no. 13/1998 for Romania's accession to the Convention on the Conservation of Migratory Species of Wild Animals, adopted in Bonn on June 23, 1979: 5 globally endangered species.

The site is important for nesting populations of the following species: Falco cherrug, Falco vespertinus, Coracias garrulus, Aquila heliaca, Accipiter brevipes, Antus campestris, Calandrella brachydactyla, Lanius minor, Milvus migrans, Pernis apivorus, Picus canus, Lullula arborea, Melanocorypha calandra.

The site is important during migration for the species Larus melanocephalus, Larus minutus, Pelecanus crispus, Phalacrocorax pygmaeus.

Site designated by the Romanian Ornithological Society as IBA - Important Birds Area, according to the following criteria elaborated by BirdLife International: C1, C2, C3, C6.

The 48 bird species listed in Annex I of Directive 2009/147/EC include: Accipiter brevipes, Alcedo atthis, Anthus campestris, Aquila heliaca, Burhinus oedicnemus, Buteo rufinus, Calandrella brachydactyla, Caprimulgus europaeus, Ciconia ciconia, Charadrius alexandrinus, Chlidonias hybridus, Chlidonias niger, Circaetus gallicus, Circus aeruginosus, Circus cyaneus, Circus macrourus, Circus pygargus, Coracias garrulous, Dendrocopos medius, Dendrocopos syriacus, Dryocopus martius, Egretta alba, Emberiza hortulana, Falco cherrug, Falco columbarius, Falco peregrinus, Falco vespertinus, Ficedula albicollis, Ficedula parva, Himantopus himantopus, Hieraaetus pennatus, Lanius minor, Lanius collurio, Larus melanocephalus, Larus minutus, Lullula arborea, Melanocorypha calandra, Milvus migrans, Nycticorax nycticorax, Oenanthe pleschanka, Pelecanus crispus, Pernis apivorus, Picus canus, Philomachus pugnax, Phalacrocorax pygmeus, Porzana porzana, Recurvirostra avosetta, Tadorna ferruginea.

ROSPA0001 Aliman - Adamclisi was established as a special avifaunistic protection area by GD no. 1284/2007 on the declaration of special avifaunistic protection areas as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

ROSPA0001 Aliman - Adamclisi is of particular importance for its 33 bird species of European conservation interest

ROSPA0001 Aliman - Adamclisi has an area of 19468 ha and overlaps with ROSCI0071 Dumbrăveni-Valea Urluia-Lacul Vederoasa and Dumbrăveni Forest.

The Aliman-Adamclisi area, in addition to having a remarkable historical value, also presents a high diversity of fauna and flora, thus having a high conservation value. This has led to the designation of the area as a Special Avifauna Protection Area.

This site is home to important flocks of birds protected at European level, of which the most characteristic are the field hen, the common skylark, the common skylark, the red-crested shrike, the black-fronted shrike, the black-crested shrike, the blackcap, the common sandpiper, the common poch moorhen, the common grouse, and others. Their high numbers are favored by the large areas of agricultural areas and steppe meadows.

During the spring and fall migrations the area is crossed by a large number of birds, especially birds of prey, such as the waspsparrow, the great tit, the great tit, the eagle eagle, the red kite, the evening tit, the red kite, the Danube falcon, and others. The Molciova Forest is an important landmark and resting area for migrating birds.

Also the Molciova and Abrud forests, located in Abrud and Hațeg, provide optimal nesting places for passerines such as flycatchers, woodpeckers and wood larks.

This site is essential for the conservation of priority habitats which are home to a number of rare plants protected at national and European level.

The local flora includes Eastern European floristic elements as well as Mediterranean and Balkan species. The plant species characteristic of the steppe have adapted to climatic conditions of low humidity xerophytic species.

The significant habitats are the Ponto-Sarmatian steppe grasslands, which now occupy restricted areas on the coasts. They are threatened with extinction through conversion to agricultural land or occupation by secondary grasslands created by overgrazing. In this habitat type 14 plant associations have been identified, including rare and some endemic plant species. From the floristic composition point of view, the most interesting grassland areas are some spots on the Văleanu and Talașman hills located N-NNW of Hațeg.

The edifying species for the steppe meadows that develop on specific substrate and by their configuration determine the landscape structure so characteristic of Dobrogea are: steppe fescue Festuca valesiaca, sadin grass Chrysopogon gryllus, the fir Poa angustifolia, the niger Stipa capillata, the white fescue Teucrium polium subsp. capitatum, the dandelion Taraxacum serotinum, the thyme Satureja caerulea. On the territory of this site, 17 rare plant species have been identified on the meadows of the Ponto-Sarmatian steppe: the grass grass-moss Astragalus hamosus, Centaurea napulifera subsp. thirkei, the carnation Dianthus leptopetalus, the tumbleweed Echinops ritro subsp. ruthenicus, Dobrogean fescue Festuca callieri, bulb-tree flicker Gagea bulbifera, hyacinth Hyacinthella leucophaea, lentil Lathyrus cicera, whorl Medicago orbicularis, Onion-cherry Muscari neglectum, wolf's-eye Nonea pallens, graceful nuthatch Onobrychis gracilis, poroin Orchis morio, Rochelia dysperma, charlock Salvia aethiopis, Scandix australis, vetch Vicia peregrina.

Another habitat specific to the region is the 40C0* ponto-sarmatic shrublands, which occur in coastal areas, but also on stony substrate with limestone on the surface. It occurs in dense, almost impenetrable plant cover, the bushes being protected from herbivores by thorns and thorn thorns. Characteristic species, illustrative for this type of habitat identified were: ha hawthorn Crataegus monogyna, redthorn Prunus spinosa, jasmine Jasminum fruticans, Christ's thorn Paliurus spina-christi, dwarf caragana Caragana frutex, dogwood Cornus mas, dwarf almond Prunus tenella, rosehip Rosa canina, dogwood Ligustrum vulgare, hairy woodthorn Pyrus pyraster, yarrow Teucrium chamaedrys, obsigera Bromus inermis, blushing Orlaya grandiflora.

Potentilla emilii - five-toed weed Potentilla emilii-popii, a strictly protected species of Community interest, was noted in the scrubby scrublands of Paliurus spina-christi.

Among the amphibian and reptile species are the crested newt Triturus dobrogicus, the red-bellied puff-bellied opossum Bombina bombina, the water turtle Emys orbicularis and others. One of the most important species is the Dobrogea turtle, declared a natural monument and strictly protected by law. The population of this species is currently in drastic decline.

This site is home to a rich avifauna, with over 100 bird species identified. Some bird species have important flocks such as raptors, larks, woodpeckers, woodpeckers, blackcap and others. which is why it has been designated as a Special Avifauna Protection Area.

The mammal fauna comprises a significant number of species including the otter Lutra lutra lutra, the gopher Spermophilus citellus and many bat species. Of the mammals, bats are the best represented group. Although there are few studies on the distribution of bats in Dobrogea, recent studies have identified 20 bat species in the Aliman-Adamclisi area. The presence of different bat species is favored by forests, human constructions or natural structures that provide roosting, feeding and roosting habitats. The cave of Cișmeluța, Deleni commune, although strongly influenced by human activities, in winter is a hibernation site for 5 species of bats such as the common bat, the gray-eared bat, the great horseshoe bat, the little horseshoe bat, Mehely's horseshoe bat.

The bird species listed in Annex I of Council Directive 2009/147/EC, comprise: A402 Accipiter brevipes, A255 Anthus campestris, A133 Burhinus oedicnemus, A243 Calandrella brachydactyla, A082 Circus cyaneus, A084 Circus pygargus, A097 Falco vespertinus, A242 Melanocorypha calandra, A533 Oenanthe pleschanka, A224 Caprimulgus europaeus, A083 Circus macrourus, A231 Coracias garrulous, A238 Dendrocopos medius, A511 Falco cherrug, A321 Ficedula albicollis, A320 Ficedula parva, A092 Hieraaetus pennatus, A338 Lanius collurio, A339 Lanius minor, A246 Lullula arborea, A072 Pernis apivorus, A234 Picus canus, A080 Circaetus gallicus, A103 Falco peregrinus, A215 Bubo bubo, A403 Buteo rufinus, A404 Aquila heliacal, A089 Aquila pomarina, A081 Circus aeruginosus, A429 Dendrocopos syriacus, A073 Milvus migrans, A379 Emberiza hortulana, A307 Sylvia nisoria.

ROSAC0157 Hagieni - Cotul Văii Forest was established by the Order of the Minister of Environment and Sustainable Development no. 1964/2007 on the establishment of the protected natural area regime of sites of Community importance, as an integral part of the Natura 2000 European ecological network in Romania, with subsequent amendments and additions.

ROSCI0157 The Hagieni-Cotul Văii Forest is of great scientific importance because of the 6 habitats and 18 species of Community interest for which it has been designated.

R0SCI0157 Hagieni-Cotul Văii Forest has an area of 3618 ha and overlaps with R0SPA0094 Hagieni Forest and Hagieni Forest Reserve-cod 2.360.

Habitat types present on the site - 6 habitat types:

- 3150 Natural eutrophic lakes with Magnopotamion or Hydrocharition vegetation

- 6430 Lowland, upland to montane and alpine, tall-grass, hygrophilous upland communities

- 91AA Forest vegetation punctuate-sarmatic with downy oak

- 40C0* Deciduous broad-leaved deciduous ponto-sarmatitic forests

- 62C0* Point-sarmatic stepping stones

- 91M0 Balkan-Pannonian sky and gorun forests

The site hosts:

- 7 species of mammals listed in Annex II to Directive 92/43/EEC Spermophilus citellus, Mesocricetus newtoni, Rhinolophus mehelyi, Miniopterus schreibersii, Rhinolophus ferrumequinum, Myotis capaccinii, Mustela eversmannii;

- 2 species of amphibians and reptiles listed in Annex II of Directive 92/43/EEC: Testudo graeca, Emys orbicularis;

- 4 plant species listed in Annex II of Directive 92/43/EEC: Himantoglossum caprinum, Potentilla emilii-popii, Centaurea jankae, Echium russicum:

- 42 Other important flora and fauna : Anacamptis pyramidalis, Beta trigyna, Centaurea napulifera ssp. thirkei, Centaurea rutifolia ssp. jurineifolia, Centaurea varnensis, Chamaecytisus jankae, Colchicum fominii Convolvulus lineatus, Crocus chrysanthus, Dictamnus albus, Galanthus elwesii, Heliotropium dolosum, Hottonia palustris, Minuartia bilykiana, Myrrhoides nodosa, Nepeta parviflora, Ononis pusilla, Onosma taurica, Opopanax bulgaricus, Orchis purpurea, Orchis simian, Ornithogalum comosum, Ornithogalum oreoides, Paeonia peregrine, Paeonia tenuifolia, Parietaria lusitanica ssp. serbica, Phleum subulatum, Plumbago europaea, Prunus tenella, Rindera umbellate, Salvia ringens, Salvinia natans, Scabiosa micrantha, Scorzonera mollis, Seseli tortuosum, Silene dichotoma, Stachys oblique, Sternbergia colchiciflora, Stipa ucrainica, Trapa natans, Trigonella gladiate, Trigonella monspeliaca and animals: Vipera ammodytes, Lacerta viridis, Lacerta taurica, Coluber jugularis

ROSPA0151 Ciobănița-Osmancea was established as a special avifauna protection area by GD no. 663/2016 on the establishment of the protected natural area regime and the declaration of special avifauna protection areas as an integral part of the Natura 2000 European ecological network in Romania.

ROSPA0151 Ciobănița-Osmancea has an area of 211.30 ha and is located between the localities of Osmancea, Ciobănița, Casicea and Credința. The site is crossed by three valleys: V Osmancea, V.Agemiei and V. Ciair. The site has no management plan.

According to the ANANP Note on the site-specific conservation objectives, the following habitat classes are found on the site area: improved grassland - 144.38 ha, cereal crops - 35.43 ha, transitional forests - 27.13 ha, anthropized habitats - 4.33 ha.

Inside the site, near Shepherd's Shearwater there is an active quarry and between Osmancea and Shepherd's Shearwater there is a large plantation of Gladdike (Gleditsia triacanthos) with a colony of Evening Finch (Falco vespertinus) and Common Raven (Corvus frugilegus). Between Shepherd's Shearwater and Casicea there is a small plantation of Acacia

The bird species listed in Annex II of Dir. 92/43/EEC include: A255 Anthus campestris, A403 Buteo rufinus, A243 Calandrella brachydactyla, A082 Circus cyaneus, A379 Emberiza hortulana, A098 Falco columbarius, A097 Falco vespertinus, A338 Lanius collurio, A339 Lanius minor, A242 Melanocorypha calandra.

BG0000569 Kardam was established as a Site of Community Importance in 2008 and covers an area of 918.92 ha. The site is important for the existing invertebrate fauna. Small isolated patches of steppe and dry karst valleys and canyons are important for small mammals and rare steppe bats.

The site has been designated for the protection of 8 fauna species: 4 reptiles (Testudo hermanni, Testudo graeca, Emys orbicularis, Elaphe sauromates) and 4 mammals (Vormela peregusna, Spermophilus citellus, Mustela eversmanii, Mesocricetus newtoni) and two habitats (6110 Rock meadows with pioneer species and 6210 Steppe meadows).

BG0000570 Izvorovo - Kraishte was established as a Site of Community Importance in 2008 and has an area of 1081,52 ha. The site is important for the existing invertebrate fauna. Small isolated patches of steppe and dry karst valleys and canyons are important for small mammals and rare steppe bats.

The site has been designated for the protection of 11 fauna species: 4 reptiles (Testudo hermanni, Testudo graeca, Emys orbicularis, Elaphe sauromates), 6 mammals (Canis lupus, Vormela peregusna, Spermophilus citellus, Rhinolophus mehelyi, Mustela eversmanii, Mesocricetus newtoni) and one invertebrate (Lycaena dispar) and two habitats (6110 Rock meadows with pioneer species and 62C0 Ponto-Sarmatian Steppe).

BG0000130 Kraymorska Dobrudzha was established as a Site of Community Importance in 2008 and covers an area of 6657,70 ha. The site is important for the existing invertebrate fauna. A narrow strip of semi-steppe grassland communities on either side of a long wooded ravine (wide-lived tree species) in the middle of a large area of arable land. Apart from habitat importance, it is a potentially important 'corridor' for migratory birds (NE-SW direction).

The site has been designated for the protection of 19 wildlife species:1 flora (Potentilla emilii-popii), 2 reptiles (Testudo graeca, Elaphe sauromates), 12 mammals (Barbastella barbastellus, Mesocricetus newtoni, Miniopterus schreibersii, Mustela eversmanii, Myotis capaccinii, Myotis emarginatus, Rhinolophus blasii, Rhinolophus euryale, Rhinolophus mehelyi, Spermophilus citellus, Rhinolophus ferrumequinum, Vormela peregusna) and 4 invertebrates (Lycaena dispar, Cerambyx cerdo, Lucanus cervus, Morimus funereus) and two habitats (6110 Rock meadows with pioneer species, 8310 Caves closed to public access, 91G0 Pannonian oak forests, 91H0 Pannonian forest vegetation with Quercus pubescens., 40A0 Peripannonian sub-continental Peripannonian forests, 91M0 Balkan-Pannonian forests of oak and gorun and 62C0 Ponto-Sarmatian steppes).

The following figure shows the location of the project in relation to protected natural areas.

Figure 2. 15 Location of PUZ in relation to the boundaries of protected natural areas

According to the Final Biodiversity Monitoring Report of the future DUMBRĂVENI-CERCHEZU wind farm, in order to analyze the habitats in the study area, a land use categories analysis was initially conducted to identify the areas of land with natural or semi-natural land use regimes. For this analysis, the Corine Land Cover 2018 database, available at www.copernicus.eu, was used. The study area in terms of habitats was established by intersecting the third level administrative-territorial units (communes) with the total proposed wind turbines. It was found that all the wind turbines will be located in the administrative territory of Cerchezu Commune, in areas in agricultural use, without intersecting grassland areas.

The area of land south of Cerchezu is characterized by predominantly herbaceous vegetation, with sparse scrubland, visibly affected by grazing, as it is surrounded by agricultural land. Due to this limitation the vegetation is overgrazed, with less than 100% ground cover. The vegetation consists of predominantly herbaceous, native species, with Asteraceae and Poaceae being the families with the most representatives.

During the field activities, in areas with natural or semi-natural vegetation, i.e. grasslands and lands mostly occupied by agriculture, with significant areas of natural vegetation, the presence of some forms of activities that represent pressures on plant communities, i.e. grazing (see Figure no. 2.16) and the presence of roads that favor the spread of non-native species, was reported.

| Natural vegetation between Cerchezu and Viroaga | Area with natural vegetation used for grazing domestic animals |
| --- | --- |

Figure 2. 16 Photos relevant to habitat pressures

During the monitoring period, in the studied area (by which we mean both the project site and the southern extremity of the Natura2000 site ROSCI0071 Dumbrăveni - Urluia Valley - Vederoasa Lake), the presence of 14 species of mammals, two species of amphibians, four species of reptiles and 63 species of birds (totaling almost 3800 individuals) was established.

In order to observe chiropteran activity on the site, best chiropteran practices were implemented using state-of-the-art equipment.

The study area is part of the southern sector of Dobrogea, a migration area for chiroptera.

The most sensitive species identified at the site, but also with a high risk of impact from wind energy is Miniopterus schreibersii. This species is regionally migratory and migrations have been documented between southern Dobrogea and central Bulgaria. In Romania the species migrates to form maternity colonies, but there are also cases of hibernation colonies. The migration corridors are unknown, but in southern Dobrogea they are most probably closely correlated with the canals (dry canyon valleys). In the south-western and western area of the proposed project there are such valleys (Măgura Valley - Olteni and Căciulați Valley, Aron's Valley - between Chercezu and Căscioarele localities) which can be used by all chiroptera species.

Other sensitive chiroptera species identified are all species of the genus Nyctalus sp, Pipistrellus sp, Vespertilio murinus and Eptesicus serotinus. These species may have a high risk of impact during operation (accidental kills). The rest of the identified species fly much lower to feed or migrate and the risk of impact is low (but not absent). For these species, the park may exert a greater impact through a barrier effect, but also through the alteration of habitats (roads, LES), even ruderal vegetation, used by all species that hunt in close proximity to the ground.

Eleven species of chiroptera have been identified, 8 of which are at high risk of impact from wind energy.

A breakdown of the monitoring results realized for an entire life cycle are detailed in Chapter 3. Presentation of the results of the field activities of the Adequate Assessment Study.

The plan area does not cross any Natura 2000 sites, but it borders 5 Natura 2000 sites in Romania that have the potential to be influenced by its construction and operation: ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii, ROSPA0036 Dumbrăveni, ROSPA0166 Plopeni-Chirnogeni, ROSPA0094 Hagieni Forest - Cotul Văii and with 2 Natura 2000 sites in Bulgaria: BG0000569 Kardam, BG0000570 Izvorovo - Kraishte.

These sites have been analyzed in the appropriate assessment study in terms of the impact of the project on their integrity.

The assessment of the impact of the project on the Natura 2000 sites likely to be affected has been carried out on the basis of the Specific Conservation Objectives set by ANANP for the period 2021-2023 for all the sites considered in the assessment. The assessment has considered the potential cumulative impact, generated together with the proposed and existing wind farms both on Romanian and Bulgarian territory.

The most important potential form of impact associated with the project is the reduction in wildlife populations, which may occur during the construction and operation phases as a result of construction works, collision with construction traffic and mainly collisions with wind turbines (especially for bird and chiropteran species). The reduction in population numbers is likely to have a significant impact on wildlife populations and affect the population size parameters of the specific conservation objectives set for the species.

The measures proposed in the Appropriate Impact Avoidance and Impact Mitigation Assessment cover all forms of impacts identified, which may affect the parameters of the conservation objectives set for the habitats and species of Community interest in the potentially affected Natura 2000 sites.

The Appropriate Assessment Study has identified the need to implement measures that can ensure that the residual impact remains insignificant. In order to validate the effectiveness of the avoidance and mitigation measures, a monitoring program has been proposed which includes provisions for both the construction and operational periods. The implementation of the monitoring program is essential in order to be able to ensure the correct implementation and functionality of the avoidance and mitigation measures.

The conclusions of the Detailed Appropriate Assessment Study for each affected ANPIC and the conclusions of the Gliding Bird Collision Risk Study are included as an appendix to this study.

    - Population and human health

According to the last census conducted in 2021, the total population by residence of Cerchezu commune on July 1 was 1360 people, of which 645 were female and 711 were male.

The population structure by age and sex at the last census is shown in the table below:

Table 2 - 3: Population structure at the last census (2021)

|  |  | AGE GROUPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | sub 5 | 5-9. | 10-14. | 15-19 | 20-24 | 25-29 | 30-34 | 35-39 | 40-44 | 45-49 | 50-54 | 55-59 | 60-64 | 65-69 | 70-74 | 75-79 | 80-84 | over 85 |
| Total | 1349 | 71 | 74 | 80 | 80 | 87 | 84 | 105 | 79 | 91 | 94 | 112 | 72 | 72 | 78 | 83 | 32 | 31 | 24 |
| Male | 711 | 45 | 33 | 44 | 39 | 54 | 46 | 62 | 43 | 44 | 56 | 65 | 41 | 38 | 30 | 37 | 15 | 9 | 10 |
| Female | 638 | 26 | 41 | 36 | 41 | 33 | 38 | 43 | 36 | 47 | 38 | 47 | 31 | 34 | 48 | 46 | 17 | 22 | 14 |

Source: processing after National Institute of Statistics

The population density of Cerchezu commune registered the value of 16,25 places/km2 in 2021, being lower than the county (108,6 places/km2 ), regional (71,2 places/km2 ) and national (84,4 places/km2 ) densities.

Figure 2. 17 Population density in Cerchezu commune

Population change

According to the data provided by the National Institute of Statistics, the evolution of the population of Cerchezu commune shows a downward trend in the period 2011-2024, both due to natural movement (births and deaths) and migration. Compared to 2011, the population of Cerchezu commune decreased by 15% in 2024.

Source: processing after NSI

Figure 2. 18 Population evolution in Cerchezu commune 2011-2024

Regarding the distribution of the population of the commune by sex in the period 2011-2024 the male population is more numerous than the female population. Compared to 2011, in 2024 the female population of Cerchezu decreased by about 19% and male population decreased by 12%.

Population movement

The main demographic indicators that characterize population movement are: the natural increase and the migration increase. In the period 2012-2022, the natural increase at the county level recorded increasingly negative values with a peak in 2021, while the increase at the rural level was -696 persons in 2022. The rate of the increase maintains the negative trend and at the rural level is lower than at the regional, macro-regional and national level.

The low values of the natural increase explain the ageing population in rural areas, as a result of the migration of young able-bodied people from the component localities to urban and industrial centers in Constanța County or outside it.

The migration rate is negative. Permanent migration, expressed as the difference between the number of persons settling and the number of persons leaving with residence, has been negative since 2005, and at the 2022 level it is -10 persons.

Ethnicity

Only one ethnic group is registered in Cerchezu: Romanian .

Religion

From a confessional point of view, the majority of the inhabitants of Cerchezu commune are Orthodox (90.84%), while for 8.52% their confessional affiliation is unknown.

Workforce

According to the National Institute of Statistics, in Cerchezu commune, the average number of employees shows a downward trend in the period 2018-2022, the lowest value was recorded in 2022 (77 employees) and the highest in 2021 (86 employees).

Thus, the number of employees in 2022 was 3.75% lower than in 2018, and 10.46% lower than in 2021.

As regards the employment rate, no data are available at local level, but in Constanța County it is relatively constant between 2018 and 2022, reaching 66.8 in 2022, which is higher than the national level.

According to the data provided by the National Institute of Statistics, in Cerchezu commune, the fewest unemployed were registered in November 2014 (8 unemployed), and the most were registered in June and July 2021 (67 unemployed). In December 2022, the number of unemployed in Cerchezu commune was 44 people.

According to the data recorded by INS, the number of unemployed people in Cerchezu municipality in the period 2010-2023 had an upward trend, in 2023, the number of unemployed people registered a value of 44 unemployed people, being 54% higher than in 2013.

Living standards

An overview of the standard of living in the study area can be expressed by indicators such as the relative poverty rate , which in the period 2012-2022 has recorded a downward trend for the South-East Development Region, which includes Constanța County and implicitly the commune of Cerchezu. However, correlating the data at county level with those at commune level and taking into account the economic profile of the area, it can be concluded that the relative poverty rate in Cerchezu is higher than that at county level

Source: processing after NSI

Figure 2. 19 Relative poverty rate

Population health

The National Center for Monitoring Risks in the Community Environment, established by Order no. 2846/2002, has as its object of activity, among others, the monitoring of health in relation to the environment. By monitoring and assessing the health status of the population and the prevalence of risk factors, it participates in prioritizing strategic public health objectives.

The National Programme for Monitoring Determinants of the Living and Working Environment (hereinafter - NP V) is an organized set of activities and services, established by law in order to protect public health by preventing diseases associated with determinants of the living and working environment.

Thus, for the Field on health protection and prevention of diseases associated with risk factors in the living environment are defined 6 methodologies applied by the National Institute of Public Health and the territorial Public Health Directorates, of which relevant for establishing the current state of the population and human health related to this PUZ are:

- Protection of public health in relation to water hygiene through "Surveillance of centralized drinking water supply in large supply areas" aiming to protect public health by preventing diseases associated with risk determinants in the living and working environment;

- Protection of public health in relation to air hygiene by "Health impact assessment of ambient air pollutants in urban environment" which for the protection of health and prevention of diseases associated with ambient air pollution by specific air pollutants monitoring the health status of the population in relation to air pollution.

Human health can also be affected by poor environmental quality. In terms of the effects of environmental pollution on the health status of the population, the following are mentioned:

- Effects of air quality on human health

Atmospheric air quality can influence specific health indicators. Tracking the evolution of these indicators can provide insight into the impact of air pollution on the health of the population.

Air pollution, in the form of fine particulate matter (PM2.5 ) and ozone exposure, contributed an estimated 7% of all deaths in 2019 (more than 17 000 deaths) ( ) - much higher than the EU average of 4%. In most cases, air pollution contributes to deaths from cardiovascular and respiratory diseases and certain types of cancer.

Some of the most harmful air pollutants for human health are PM10 and PM2.5 .

National emissions of PM10 and PM2,5 , reported at the European level, showed a decreasing trend from 2012 to 2017, after which an increasing trend followed. According to European-wide data, the annual average population exposure to air pollution by PM10 and PM2,5  in the period 202012-2019 showed a downward trend for PM10 emissions and a slight upward trend for PM2,5 emissions:

Source: processed from ECHI Data Tool (https://webgate.ec.europa.eu/dyna/echi/?indlist=10a)

Figure 2. 20 Nationwide trend in annual average exposure to PM pollution10 and PM2,5  in the period 2012-2021

Results from air quality monitoring stations nationwide have shown that air pollution is higher in urban areas than in rural areas. People living in the areas of large urban agglomerations and near roads and industrial centers are more exposed to pollution.

In Constanta County only one station monitors the PM2,5 fraction of suspended particulate matter, CT2 - urban background station located in the city hall park area. The recordings made according to the regulations in force showed an upward trend in the period 2017-2019, followed by a decrease until 2022.The recorded values were around 12 µg/m3 being lower than the limit value, in the period of analysis no exceedances were recorded.

In Cerchezu commune, the most important source of air pollution is currently the transportation activity (road traffic), but even this is insignificant considering that the commune's roads are represented by one county road and three municipal roads. The main pollutants from transportation activity that affect human health are: particulate matter (PM10 and PM2,5 ), COx , NO , NOx , SOx , volatile organic compounds, heavy metals, benzene and other non-burned hydrocarbons.

In Romania, there are no statistics on the number of cases of respiratory or cardiovascular diseases due to the transport sector or air pollution.

Respiratory and cardiovascular mortality account for a significant share of overall national mortality, with a higher share in urban than in rural areas, partly due to the fact that urban areas are more polluted.

The situation of deaths associated with air pollution at county level is as follows:

Source: processed from NSI data

Figure 2. 21 Air pollution-related deaths

- Effects of noise pollution on human health

Another stress factor that can aggravate certain conditions (hearing loss, mental illness, cardio-vascular diseases, endocrine diseases) is noise, which is one of the most widespread threats to the health of the population in industrialized countries.

At the level of Cerchezu commune the traffic sources do not represent a significant source of noise taking into account that the roads of the commune are local roads (county and municipal).

Wind turbines can generate constant and characteristic noise, which can affect sleep quality and cause acoustic discomfort. An important reference value is the noise level outside dwellings, which should not exceed the limit imposed by national and international regulations.

Persistent noise, including during the night, can affect the quality of sleep and can lead to insomnia, cognitive disturbances and anxiety or irritability.

Some studies suggest that long-term exposure to wind turbine noise can have a negative impact on the cardiovascular system, increasing the risk of high blood pressure, heart disease and stroke.

Individual reactions to noise may vary depending on personal, contextual and individual sensitivity factors. Some people may be more sensitive to specific wind turbine noise and may experience more pronounced adverse effects.

- Effects of water quality on human health

The occurrence of diseases related to the quality of water intended for human consumption is caused by the excess or deficiency of certain chemical elements in water, but also by the presence of substances foreign to the normal composition of water. Water has a varied chemical composition with a large number of dissolved chemical elements. As a result of pollution, other chemical substances can be added to the normal composition, which can have a variety of effects on the human body.

Water can be a transmission route for many toxic chemicals. These can enter the body through water drunk every day in small quantities but over a long period of time, favoring chronic diseases. There are situations when toxic substances are present in large quantities and can cause acute poisoning.

In Constanta County, the drinking water quality is monitored by the Constanta Public Health Department. Periodic determinations are carried out for chemical parameters: nitrites, nitrates, pesticides, cadmium, lead and indicator parameters: oxidizability, turbidity, ammonium, aluminum, iron, manganese.

The drinking water distribution network in Cerchezu commune is owned by the operator S.C. RAJA S.A. Constanța, has a length of 12 km, serves a population of 563 inhabitants and provides a volume of water of 19 m3 /day. ZAP Cerchezu has a sanitary operating authorization issued in 2014 and having visa 2023 .

The quality of the drinking water distributed to the population of Cerchezu commune is satisfactory, there are no parameters for which the system is declared non-compliant, the microbiological parameters and physico-chemical indicators of the water are within the permissible values according to the analysis bulletins .

In Cerchezu there are no public wells and public toilets.

- Green space and its effects on health and quality of life

Green spaces represent a functional category within or adjacent to the localities, whose specificity is determined, first of all, by the vegetation and, secondly, by the built environment, including facilities and equipment for cultural-educational, sports or recreational activities of the population.

Depending on the nature of ownership, green spaces can be public (parks, squares, sports and recreation space, planted space for protection and protection and forests of various types and areas of spontaneous vegetation that are in the public domain) or private (green spaces in private ownership that are not used in the public interest).

From an ecological perspective, urban green spaces are a true moderator of the impact of human activities on the environment. They make an important contribution to the chemical purification of the atmosphere, which maintains the day-night balance in favor of oxygen production, but also to its physical purification by trapping dust and particles. For example, a leaf area of 25 m2 provides the oxygen required by a person, a grass lawn retains 3-6 times more dust than a bare surface, and a mature tree retains 10 times more impurities than a lawn the size of the projection of its crown on the ground.

At county level, the surface area of green spaces had a negative evolution in the period 2010-2019, followed by an increasing trend that reached 1080 ha in 2022, which, if compared to a resident population of 656,631 inhabitants, means a green space area of 16.45 m2 / inhabitant, well below the average required by the European norm of 26 m2 green space / inhabitant.

- Climate change and its effects on health and quality of life

The main impacts of climate change on health are related to extreme weather events, changes in the distribution of climate-influenced diseases, and changes in environmental and social conditions. Over the last decade, flooding from rivers and coastal flooding has affected millions of people in Europe. Health effects include injuries, infections, exposure to chemical hazards and mental health consequences. Heat waves have become more frequent and intense, causing tens of thousands of premature deaths in Europe. This trend is expected to grow and intensify unless appropriate adaptation measures are taken. The spread of tick species, Asian tiger mosquitoes and other disease carriers increases the risk of diseases such as Lyme disease, tick-borne encephalitis, West Nile fever, dengue fever, Chikungunya and leishmaniasis .

Low temperatures in winter and other average maximum and minimum temperatures in summer can contribute to increased mortality from cerebrovascular disease and myocardial infarction, and high/low relative humidity in the same month can also be a risk factor for cerebrovascular disease and myocardial infarction.

    - Landscape

The European Landscape Convention adopted in Florence on October 20, 2000 (ratified in Romania by Law 451/2002) defines landscape as the part of the territory perceived as such by the population, whose character is the result of the action and interaction of natural and/or human factors. The main features that give landscape value are:

- Aesthetic value (particularity, diversity, cohesion of landscape elements);

- Traditional value (natural endemics, cultural distinctiveness).

The main aim is to provide elements to encourage public authorities to adopt policies and measures for local, regional and transboundary landscape protection, management and planning so as to maintain and improve landscape quality and to bring the public, institutions and local and regional authorities to recognize the value and importance of landscape and to participate in related public decisions. The integration of the landscape dimension in the preparation of spatial management policies, both general and sectoral, will lead to better protection and management of natural heritage in the cross-border area.

The study area is a predominantly agricultural area, with natural pasture meadows used for grazing domestic animals.

The landscape is typically Dobrogean, emphasizing the limestone deposits of the canara type, at the level of which can also be found rare species, typically Dobrogean, calciphilous. The meadow is also bordered and partly covered by a plantation of acacia (Robinia pseudoacacia, an invasive allogenic species).

The distribution of vegetation is heterogeneous, alternating areas with abundant vegetation with specimens in good state of conservation with areas with sparse vegetation, with reduced size and cover and affected by the presence of ruderal and some segetal species.

Landscape fragmentation is assessed using the indicator "effective mesh size" (meff, km2 ), which suggests the probability that two random points in an area are connected without obstacles ("Landscape fragmentation in Europe"). This indicator is used in some European Union countries and Switzerland to assess the state of the environment, more specifically to understand ecological processes at the landscape level. The following figure shows the variability of landform fragmentation at the level of Cerchezu municipality, using EEA data. The lower the effective mesh size, the more fragmented the landscape is and the lower the connectivity.

The indicator underlying the map considers "medium and major anthropogenic fragmentation" (roads, railroads, built-up areas, etc.) and excludes natural barriers. In the figure below it can be seen how the highest values of landscape fragmentation are found in the north-west of the site in the T32 turbine area. Based on the landscape fragmentation analysis it can be deduced that the project implementation area (Cerchezu commune) is associated with a very low fragmentation value.

Source: European Environment Agency

Figure 2. 22 Degree of landscape fragmentation in the PUZ implementation area

According to a study on the effect of drought on the physical characteristics of the landscape , changes in temperature have an indirect effect on changes in landforms and landscape. Temperature increases directly alter components of water balance, soil properties and vegetation. The main impact on the landscape, will be related to the effect of changes in precipitation amounts, which can lead to drought and then soil erosion.

The existing European LANMAP3 database was used to identify landscape types in the site area. Landscape types are established on the basis of criteria considering the following elements:

- Climate type of the area;

- Terrain topography;

- The parent material of the rock;

- Land use.

For our country there is a variety of landscapes, about 45 landscapes with mountains with gorges and canyons, hills and plateaus, plains and river plains, rivers and lakes.

According to the regional differentiation of Bulgarian landscapes (Velchev, Todorov, Penin, 2003), the territory of Bulgaria falls into three major landscape provinces of Eurasia: Alpine, East Mediterranean and Pontic.

The following figure shows the spatial distribution of landscape types in the analyzed area.

Figure 2. 23 Landscape types characteristic of the area where the PUZ is proposed

It can be seen from the above figure that the general aspect of the area is dominated by the following landscape types: continental - hilly - hills - sediments - arable land and steppe - hilly - sediments - arable land.

In conclusion, it can be observed that the Cerchezu commune is dominated by a steppe-type landscape with a predominantly hilly relief, with arable areas. No areas of high landscape sensitivity have been identified in the whole plan area.

The areas of moderate and high sensitivity identified in the study area are the natural areas associated with protected natural areas in the vicinity of the project.

The landscape of the area can be seen from the photos taken during the monitoring campaign of species of flora and fauna of community importance carried out by Ramboll in the perimeter of the proposed energy park (April 01, 2022 - March 30, 2023).

Figure 2. 24 Grassland vegetation aspects of the analyzed areas

The distribution of vegetation is heterogeneous, alternating between areas with abundant vegetation and areas with sparse vegetation, with reduced size and cover.

    - Climate change

The climate change assessment has two components: the project's impact on climate change (contribution to greenhouse gas (GHG) emissions) and adaptation to climate change.

Climate change is one of the greatest threats to the environment, social and economic framework. These changes are mostly due to greenhouse gas emissions resulting from anthropogenic activities, represented by: CO2 , CH4 , N2 O, HFCs, PFCs, SF6 .

The main sources of greenhouse gases produced by humans are: burning fossil fuels for electricity generation, transportation, industry and households; changes in agriculture and land use (deforestation), landfilling, and the use of fluorinated industrial gases.

Climate change brings two major challenges: drastically reducing greenhouse gas emissions and adapting through sustainable development, moving to a carbon-decoupled economy that at the same time ensures a good quality of life for citizens and effective protection of their lives and property from new vulnerabilities and risks of natural disasters.

With regard to the GHG emission reduction component, as an EU Member State (MS), Romania is committed to the key principles of the European Green Deal (EGP): (i) to reduce net GHG emissions by at least 55% by 2030 compared to 1990 ("Fit for 55") and (ii) to achieve zero net GHG emissions by 2050 ("net zero").

Meeting the Net0@2050 target will bring substantial challenges, requiring political and public acceptance. Substantial emission reductions in sectors that are difficult to switch and an additional 96% emission reduction even if the "Fit for 55" target is achieved are needed to meet the ambitious targets.

At the end of 2023 by HG 1215/2023 was adopted Romania's Long-Term Strategy for Reducing Greenhouse Gas Emissions - Neutral Romania in 2050 (STL 2050). The main target of the strategy is Romania's climate neutrality in 2050, reaching a 99% reduction of net emissions in 2050 compared to 1990.

In terms of decarbonization of the energy sector in 2019, 69% of the 2050 GHG reduction target has already been achieved. By 2035, 98% of the target will have been achieved.

Target for the Renewable Energy Sources (RES) sector Under the selected RO Neutral scenario, the share of RES in gross final energy consumption will be 86.1% in 2050 and 36.2% in 2030.

The major increase in wind and solar electricity generation, as well as green hydrogen, contributes to increasing the share of SRE in the electricity generation sector (SRE-E) to about 80% in 2050 in the Neutral RO scenario.

The wind capacity targets for the period 2030-2050 in the Neutral RO scenario include new wind capacity - 550 MW installed each year from 2023-2025, 600 MW each year from 2026-2030, 675 MW each year from 2031-2040 and 750 MW each year from 2041 to 2050.

As regards the adaptation component, the National Strategy on Adaptation to Climate Change for the period 2023-2030 with a 2050 perspective (N.S.A.C.C.S.) aims to improve adaptive capacity and increase the resilience of socio-economic and natural systems to the impacts of climate variability and change over different areas and timeframes. Specifically, for 2030, the strategy aims to

- Strengthening Romania's resilience and capacity to adapt to the risks associated with climate change and natural disasters;

- Increased ability to react quickly to unexpected extreme weather events;

- Improving education, awareness and human and institutional capacity to mitigate, adapt to and reduce the impacts of climate change, as well as early warning systems;

- stepping up national efforts to make the transition to a green economy.

According to the guide on adaptation to climate change approved by Ord. 1170/2008, for the specific objective Energy, the Action Plan provides for the following adaptation measures relevant to the proposed PUZ:

- urgently develop studies on the risk assessment of climate change impacts for the energy sector in general and, in particular, on the risk assessment for the hydropower sector, and take these risks into account for investment projects planned to be built;

- promoting renewable energy production;

- developing local public administration authorities' own strategies for the use of energy sources that comply with European environmental and efficiency standards for the production of electricity and heat in centralized systems.

80% of Constanta County belongs to the continental climate sector (lowland climate zone and in a limited area hilly climate zone) and about 20% to the coastal maritime climate sector (lowland climate zone).

The general climatic regime is characterized, in the continental part of the county, by hot summers with low precipitation and not too cold winters, sometimes with strong blizzards, but also with frequent warming intervals that make the snow cover episodic, and in the maritime part, by summers whose heat is attenuated by the cool sea breeze and mild winters, marked by strong winds and wet from the sea.

Mean annual temperatures are above the national average - 11.20 C at Mangalia and 11.20 C at Murfatlar - and in the central-northern half of the territory the values do not fall below 10 C.0

The absolute minimum temperatures recorded in Constanța county were -250C at Constanța on February 10, 1929, -33,10 C at Basarabi (Murfatlar) on January 25, 1954 and - 25,20 C at Mangalia on January 25, 1942. The absolute maximum temperatures recorded were +430 C at Cernavodă on July 31, 1985, +410 C at Basarabi on August 20, 1945, +38,50 C at Constanța on August 10, 1927 and +360 C at Mangalia on May 25, 1950.

Annual precipitation ranges between 350 mm and 475 mm, placing Constanta among the driest regions in the country. The low values are due to the continentalization of the air masses as they move from west to east and to particular local conditions (low altitudes, Black Sea basin).

The ratio of precipitation to temperature indicates dry, dry and wet periods. Dry periods are longer in the south of the county. Droughts frequently occur during barometric highs with light winds and high temperatures.

The commune of Cerchezu has a temperate-continental climate with an average annual temperature of about 11°C. The absolute maximum temperature recorded was 38.5°C on July 25, 2007 and the absolute minimum temperature was -25°C. The average annual precipitation is 396 mm.

The average frost-free period is about 200 days a year, with the first frost usually occurring after October 25.

Changes in the general circulation of the atmosphere from one season to the next are reflected by changes in the frequency of winds in certain directions. The prevailing winds in Cerchezu are usually from the northeast and east during winter and spring, which contributes to wet, cold winters and hot, dry summers.

Climate hazard identification and area sensitivity assessment

The relevant climatic parameters for Cerchezu commune are:

Extreme temperatures

Extreme temperatures are exceptionally high or low temperature values in a particular region or place at a particular time. These temperatures significantly exceed normal or usual temperature limits for that area or time period.

Temperature increase phenomena intensified after 2000, the winter of 2006-2007 being considered the warmest since instrumental measurements have been available in Romania. In that year, pronounced deviations of the maximum/minimum temperature from the multiannual average regime persisted for long periods of time.

In terms of extreme weather trends at national level, the results recorded at meteorological stations showed a decrease in the number of freezing days in some areas in the south of the country and an increase in heat waves in most parts of the country.

According to the global climate data for the period 1970-2000 available on WorldClim, the mean annual maximum temperature in the area of influence of the PUZ, at the level of Cerchezu commune, is between 5.82 ÷ 6.390 C.

Source: Adapted from www.worldclim.org

Figure 2. 25 Historical mean annual maximum temperature values in the area of impact of the PUZ

With regard to the evolution of the mean annual maximum temperature in the area of influence of the proposed PUZ, in 2050 the temperature is expected to increase by about 130 C, with values in the area ranging from 19.01÷19.360 C.

Source: Adapted from www.worldclim.org

Figure 2. 26 Maximum temperature values in the impact zone of the PUZ in 2050

According to the global climate data for the period 1970-2000 available on WorldClim, the average annual minimum temperature in the area of influence of the PUZ in the municipality of Cerchezu is between 0,68÷0,330 C.

Source: Adapted from www.worldclim.org

Figure 2. 27 Historical average minimum temperature values in the impact area of the PUZ

As regards the evolution of the mean annual minimum temperature in the area of influence of the proposed PUZ, in the municipality of Cerchezu, in 2050 it is expected an increase in temperature by about 90 C, in the area recorded values ranging from 9,49÷9,840 C.

Source: Adapted from www.worldclim.org

Figure 2. 28 Annual mean minimum temperature values in the impact zone of the PUZ in 2050

Analyzing the data presented above, it can be seen that by 2050 there will be an increase in temperature, both in terms of maximum and minimum temperature, but the exposure to extreme temperature risk is reduced for wind farms.

Precipitation

Precipitation can take various forms such as liquid water droplets, snow crystals or ice pellets. Extreme precipitation can have significant impacts on water supply and sewage infrastructure by increasing the turbidity of surface water supplies.

According to the global climate data for the period 1970-2000 available on WorldClim, the annual precipitation in the area of influence of the PUZ in the commune of Cerchezu varies between 460÷476 mm.

Source: Adapted from www.worldclim.org

Figure 2. 29 Historical rainfall values in the impact area of the PUZ

In terms of the annual precipitation in the area of influence of the proposed PUZ, in 2050 it is expected a decrease in the amount of precipitation in the municipality of Cerchezu by about 10 mm, in the area recorded values ranging from 450÷475 mm.

Source: Adapted from www.worldclim.org

Figure 2. 30 Rainfall values in the impact zone of the PUZ in 2050

Analyzing the data presented above, it can be seen that by 2050 there will be a decrease in the amount of precipitation, but the exposure to precipitation risk is reduced for wind farms.

Floods

Floods are events in which the ground is covered with excess water, usually due to heavy rainfall, rapid snowmelt or overflowing streams.

Exposure to flood risk, already considerable in most parts of the country, is expected to increase with climate change. According to the data available in the Flood Hazard and Flood Risk Maps - Cycle 2, the Impact Zone of the proposed PUZ is not subject to flood risk, the distance to water bodies at risk of flooding being at least 15 km

Source: processed by inindații.ro

Figure 2. 31 Map of areas at significant flood risk

The watercourses in the site area (Măgura, Cerchez, Ceair) are not included in the Dobrogea Litoral Watershed Management Plan and do not appear in the hazard and flood risk maps - cycle 2. However, according to the Law 575/2001 on the approval of the National Spatial Plan - Section V - Natural risk areas, Cerchezu commune is exposed to the risk of flooding from torrential floods.

In conclusion, it can be assessed that the area of implementation of the PUZ is not exposed to flood risk.

Drought

Drought can be defined as "a shortage of precipitation over a long period of time (usually a season or more), leading to a shortage of water".

Several types of drought are categorized as follows:

- meteorological drought which refers to an extended period of time when there is little or no rain;

- Agricultural droughts occur when soil water levels are low and water shortages damage or destroy agricultural crops;

- Hydrological drought which refers to reduced water supply to rivers, lakes, aquifers and other reservoirs and usually follows meteorological drought;

- Ecological drought is an episodic shortage of water availability that drives ecosystems beyond vulnerability thresholds and triggers responses in natural and/or human systems and sets in when widespread ecological damage is caused by lack of soil moisture;

- Socio-economic drought occurs when water supply is reduced or stopped due to lack of water resources.

According to the data available at the European Drought Observatory, the complex drought indicator for the project implementation area in Cerchezu commune shows mainly agricultural drought (soil moisture deficit).

Source: European Drought Observatory

Figure 2. 32 Distribution map of drought types

Fires

Wildfires are fires that spread through vegetated areas, such as forests, grasslands or bushes. These fires can have multiple causes, including careless human activity, electrical discharges or natural phenomena such as drought.

In recent years, as the climate has warmed globally, there has been an increase in the duration and magnitude of weather conditions favorable for forest fires (Davis et al. 2018).

In the period 1995-2018, the number of forest fires and the area affected by forest fires showed significant annual variations, with the highest values in 2000, 2007 and 2012. This result can be explained by the fact that these years were considered to be the warmest since the reference year 1951, when the maximum temperature record in our country was recorded (Busuioc et al 2007). Of the total number of fires affecting forests in Romania in the period 1995-2018, most occurred in March, August and November.

In the area of implementation of the PUZ, an area without forests, it can be estimated that there is no probability of fires.

Figure 2. 33 Forest fire probability map

Earthquakes

Earthquakes are caused by the release of stress, generated by forces related to plate tectonics or by anthropogenic activities such as the creation of reservoirs, deposits, ponds, mining and/or the injection of fluids into underground formations.

Earthquakes can cause cracks, cracks or damage to water and sewage systems.

From the seismic point of view, in accordance with the Standard P 100-1/2013 (Seismic Design Code), Part 1 - Design provisions for buildings, the area of location of the investments provided for by the PUZ is characterized, for seismic events with the average recurrence interval (of magnitude) IMR = 225 years, as follows:

- the acceleration of the design terrain ofg = 0.10 g/ 0.15 g;

- control period ( corner ) Tc = 0.7 sec.

Figure 2. 34 Map of seismic zonation in terms of peak values of ground acceleration (a )g

Figure 2. 35 Seismic zonation map in terms of control period (T )c

In view of the above, the exposure to the risk of earthquakes in the area of implementation of the PUZ is very low.

Landslides

Landslides are driven by the forces of gravity, but can be triggered by a variety of processes. Some of the most common triggers include earthquakes and periods of prolonged and/or intense rainfall. Deforestation can also increase the likelihood of landslides.

Figure 2. 36 Landslide hazard zonation map

In Cerchezu commune, the risk associated with landslides is very low, concluding that the exposure to the risk of landslides in the area of implementation of the PUZ is very low.

    - Cultural heritage

According to the List of Historical Monuments 2015 in Constanta County approved by Order no. 2361/2010, the following historical monuments have been identified in the area of the proposed Plan.

Table 2 - 4: Objectives of interest in the area of the PUZ site

| LMI Code | Locality | Name | Date |
| --- | --- | --- | --- |
| CT-IV-m-B-B-02967 | village VIROAGA; commune SEARCH | Obelisk in memory of the heroes of the First World War | 1936 |

Also, in the area of the PUZ, there are precisely localized archaeological sites that are not included in the List of Historic Monuments, but have been identified with the help of the National Archaeological Repertory. These are:

- Villa rustica de la Viroaga - village Viroaga, commune CERCHEZU

- Tumulul de la Căscioarele - Movila Bacu - village Căscioarele, commune CERCHEZU

- Legionary Castrum from Căscioarele - village Căscioarele, commune CERCHEZU

Figure 2. 37 Map of the location of objectives of historical interest (National Archaeological Repertory (cIMeC) and National Heritage Institute - eGISpat Romania)

Interventions on historical monuments are regulated by the provisions of art. 23 of Law no. 422/2001, republished, on the protection of historical monuments. In the area of protection of historical monuments (200,00m in the urban area, respectively 500,00m in the rural area) for the authorization of new constructions or interventions on existing constructions, the opinion of the Ministry of Culture and National Identity, respectively of the County Directorate for Culture and National Heritage Constanta, will be required.

The legal regime of historical monument buildings is regulated by the provisions of Law no. 422 / 2001 published in the Official Journal of Romania no. 938 of 20.11.2006, republished under Article V of Law no. 259/2006 for amending and supplementing Law no. 422 / 2001 on the protection of historical monuments, published in the Official Journal of Romania, Part I, no. 573 of 03.07.2006, giving the texts a new numbering.

  - The evolution of the state of the environment in the case of non-implementation of the proposed PUZ

The analysis of the state of the environment under the conditions of non-implementation of the plan is a requirement of both Directive 2001/42/EC Directive on Strategic Environmental Assessment (SEA1 - art. 5 and Annex I-b) and the Government Decision no. 1076/8 July 2004 on establishing the procedure for carrying out the environmental assessment of plans and programs (art.15).

The purpose of this analysis is to assess how the "48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer stations, Electrical connection networks, construction and modernization of communication and access roads" meets the needs and requirements of the environmental state of the analyzed territory and its development trends.

The project implementation area is located in the southern part of Constanța County, an area with a continuously decreasing population, being among the poorest in the region: the local human development index (IDUL 2011) has values below 55. The population in the area is faced with a shortage of jobs, lack of capital for investment and lack of businesses as an alternative to farming.

The analysis of the "zero" alternative (the alternative that takes into account the situation of not implementing the plan) is based on the current state of knowledge and summarizes the environmental effects that will occur if the PUZ is not implemented.

The non-implementation of the PUZ is in fact maintaining the current state of the environment will reduce Romania's ability to realize its commitments undertaken at European level, will reduce the local potential for diversification of economic activities, use of the wind potential of the area, increase the standard of living of the population by attracting sustainable investment.

The following table shows the possible evolution of the state of the environment if the PUZ is not implemented ("Alternative 0")

Table 2 - 5: Evolution of the state of the environment if the PUZ is not implemented

| Environmental aspect | Possible evolution of the state of the environment if the PUZ is not implemented ("Alternative 0") |
| --- | --- |
| Water | The quality of surface and groundwater bodies in the PUZ area is generally good (except for the surface water bodies ROCT02_B2 Eforie Nord - Vama veche and ROCT01CAPM Mangalia, with poor ecological status and the groundwater bodies RODL10 Dobrogea de Sud and RODL04 Cobadin Mangalia with poor chemical status). The non-implementation of the analyzed PUZ will not lead to a degradation of the quality of surface and deep waters. By its specifics the Plan has no significant impact on the quality of water bodies in the implementation area. Taking into account that the environmental objectives in 2027 for the water bodies are good chemical status, the likely evolution of the quality of the water bodies in the implementation area in the absence of the PUZ is improving. |
| Air | The main sources of air pollution in the PUZ implementation area are traffic and agriculture. As the implementation area is rural, the level of air pollution is low. The emission concentration values of PM10 , PM2,5 , NO2 , NOx are lower than the annual emission limit value and in the case of dust they are above the lower assessment threshold. At county level, the Air Quality Maintenance Plan for Constanța County is being realized, and the identification of air quality sensitive areas will allow taking measures to reduce air pollution. The overall assessment of the likely evolution of air quality in the coverage area of the proposed PUZ is that the current state will be maintained even if the PUZ is not implemented. The non-implementation of the analyzed PUZ will not directly lead to a degradation of the air quality in the area of the site, but given the objective of the Plan to create a renewable source of electricity to replace the traditional sources of electricity generation it can be concluded that the non-implementation of the PUZ indirectly leads to degradation of air quality. |
| Soil/subsoil | The area is relatively anthropized, at present the land of the site has the current use - arable according to the Certificate of Urbanism no. 129 of 28.11.2022. In terms of soil quality, the trend is to maintain the current characteristics. If this plan is not implemented, it is expected that no changes in soil quality will occur. |
| Use of natural resources | The lack of policies to save and conserve energy/use renewable resources will continue the current trend of increasing consumption of natural resources. |
| Waste management | In Cerchezu, the sanitation activity is provided by Iridex Grup Salubrizare S.R.L. which provides the infrastructure and equipment necessary for separate waste collection (bins/containers/bags) and for preventing improper disposal of waste. In the absence of the PUZ, no significant changes in waste management are anticipated, thus maintaining the current situation. |
| Noise | In Cerchezu commune, noise pollution is insignificant, the main source of pollution being road traffic. Failure to implement the Plan will maintain the status quo. |
| Biodiversity | The non-implementation of the Plan will not influence local biodiversity in the area. However, given the continuous urbanization and the need to implement new infrastructure elements, and taking into account the negative effects of climate change evolution, the flora and fauna at the project may be negatively impacted, leading to the loss or eventual disappearance of some natural elements. However, this hypothesis needs to be confirmed over time through rigorous monitoring of elements potentially sensitive to these factors. |
| Population and human health | The non-implementation of the analyzed PUZ will maintain the current state of lack of jobs, continuous depopulation and reduced attractiveness of the area for sustainable investments. The health status of the population will not be influenced by the non-implementation of the PUZ. |
| Landscape | The lack of a sustainable vision for the development of Cerchezu will contribute to the continued degradation of the natural landscape and the expansion of urbanization. |
| Climate change | The non-implementation of the analyzed PUZ will maintain the current mode of electricity production from fossil fuels, which will increase GHG emissions leading to changes in the current climatic conditions, globally and implicitly, in the Cerchezu commune. |
| Cultural heritage | The absence of policies and measures oriented towards the protection, conservation and valorization of cultural heritage will lead to its degradation/deterioration over time. |

# Environmental characteristics of the area likely to be significantly affected

The PUZ area is located in southeastern Romania on the administrative territory of Constanta County, in the western part of the county, in the immediate vicinity of the border with Bulgaria, being delimited by the localities: Negru Voda (in the east), Căsoaiele and Viroaga (in the north) and Magura (in the east), in the south and south-west, the study area is delimited by the state border. The study area belongs to the administrative unit (UAT) Cerchezu and is a predominantly agricultural area, with natural pasture pastures used for grazing domestic animals.

Given that the area studied in the PUZ is 3027.77 ha, it is estimated that the environmental impact resulting from the implementation of the development plan will be felt only locally and in its immediate vicinity, both due to the specific construction works to be carried out, which involve the arrangement of site organizations, excavation of material and works for the actual installation of turbines, as well as the works for the construction / modernization of related infrastructure.

The area studied for the implementation of the objective includes agricultural land for arable land and land for special use - farm roads in the public domain of the municipality of Cerchezu, managed by the Local Council of Cerchezu and public domain of county interest. The proposed investments will be located in the extravilan of the locality.

The realization of new building elements will result in changes to the physical and aesthetic characteristics of the natural landscape locally.

The effects on the environment of the implementation works of the PUZ investments will be felt locally (in the area of the working fronts) and will be temporary. The quantification of the effects will depend on the type and volume of works carried out, the site conditions and the construction method adopted.

Air emissions

Air quality may be affected only during the construction and decommissioning phases by emissions of air pollutants (such as nitrogen oxides, sulphur oxides, carbon oxides, volatile organic compounds, suspended particulate matter) from the means of transport and machinery used to implement the investments provided for by the PUZ. To these are added the dust from the organization of the construction site, earthworks, excavations for foundations, platforms and connections, modernization of the exploitation roads, from the handling of earth masses and construction materials.

However, depending on climatic conditions and the activities carried out at the same time as the construction activities, there may be an increase in dust levels which may reach the local settlements in the area and the natural areas in Bulgaria.

The ADMS 6 software application was used to forecast air pollution as a result of the works related to the execution phase of the project. It uses a model based on a Gaussian pollutant plume equation for continuous emission sources. The model calculates dispersed pollutant concentrations in the environment based on a set of input data including:

- Characterization of meteorological conditions (main wind direction, wind speed, turbulence, turbulence, air temperature, rural/urban environment);

- Source characterization (source type, mass flow rate, height, gas velocity, gas temperature);

- Spatial characterization of the investigated area (with the possibility of graphical representation of concentration isolines on maps or images).

For the modeling, a number of three turbines were chosen that are found in all three alternatives for which the impact on environmental factors was analyzed, the impact of the turbines was evaluated at different stages of execution in order to capture the worst case scenario of the execution (excavation execution, turbine erection, final erection). The modeling results cover, by extrapolation, all three alternatives analyzed in chapter 6.3.

Regarding the modeling results, the following observations should be made:

- The dispersion of pollutants will be predominantly N-NE (in the predominant wind direction);

- In the case of the development phase of the turbine platforms (handling of earth masses), the intervention threshold and alert threshold values for the PM10 indicator (with an estimated maximum concentration of 60.66 μg/m3) and the hourly average values for the NO2 indicator (with an estimated maximum concentration of 239 μg/m3) will be exceeded, but these are estimated to occur only near the working fronts of the turbines. The concentration of PM10 (0.5 μg/m3) respectively NO2 (<30 μg/m3) estimated at the nearest sensitive receptor falls below the intervention and alert threshold values at this stage.

The maximum concentrations will be recorded at the level of the closest households located to the turbine platforms, i.e. households in Viroaga, but they are considerably lower than the maximum permissible concentrations according to Law 104/2011.

It was thus found that for all atmospheric pollutants emitted into the environment during the execution phase, at sensitive receptors in the area, the concentrations are estimated to be below the limits set by the legislation indicating no potential impact on ambient air quality.

We emphasize that there are no significant sources of air pollution in the analyzed area, so that the emissions generated from the analyzed activity could have a cumulative effect that could lead to exceeding the limits set by Law no. 104/2011. However, it can be assessed that the only cumulative effect with an impact on air quality in the area could be generated if the works for the development of the turbine platforms (stripping, earth moving) will overlap with the agricultural works carried out in the area. This effect will be temporary, short-term and reversible.

Table 3- 1 Maximum concentrations over different averaging ranges

| Source | Pollutant | Mediation interval | Maximum concentration |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cmax [μg/m ]3 | Alert threshold [μg/m ]3 | Limit value= intervention threshold [μg/m ]3 |  |
| Earth handling | PM10 | Daily average | 60,66 | 35 | 50 | <PA; <VL |
| Operating machinery and equipment | SO2 | Hourly maximum | 1,3 | 245 | 350 | <PA; <VL |
|  | NO2 | Hourly average | 239 | 140 | 200 | >AP; >VL |
|  | PM10 | Annual average | 1,62 | 35 | 40 | <PA; <VL |
|  | CO | Maximum 8-hour averages | 0,0334 | 7000 | 10000 | <PA; <VL |

Table 3- 2 Comparison between maximum concentrations and limit values

| Source | Distance between the point of maximum concentration and the receiver (km) | Concentration/ concentration range at sensitive receptors ( g/m )3 | Limit value ( g/m )3 | Upper assessment threshold for vegetation protection (µg/m )3 | Lower assessment threshold for vegetation protection (µg/m )3 | Upper assessment threshold for health protection (µg/m )3 | Lower assessment threshold for health protection (µg/m )3 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Earth handling PM10 - daily average | 1,5 | Within the 0,5 | 50 | 28 | 20 | 35 | 25 | < limit |
| Operation of machinery and equipment - SO2 | 1,45 | Outside the 0,2 | 350 | 12 | 8 | 75 | 50 | < limit |
| Operation of machinery and equipment - NO2 | 1,56 | Outside the 30 | 200 | 24 | 19,5 | 140 | 100 | > limit |
| Operation of machinery and equipment - PM10 | 1,2 | Within the range of 0.03 | 50 | 28 | 20 | 35 | 25 | < limit |
| Operation of machinery and equipment - CO | 1,32 | Within the 0,005 | 10000 | - | - | 7000 | 5000 | < limit |

Emissions into water

Sources of construction pollution that can affect water resources include:

- improper management and storage of domestic waste water from the green toilets used on site;

- improper disposal of construction waste;

- excavation works, which can pollute surface water with small particles;

- improper handling or storage of construction materials (concrete, earth, sand, etc.), which may end up in surface water carried by stormwater;

- Accidental spills of fuels and lubricants from construction vehicles and machinery.

Manipulation of land masses (digging, grading, compaction) could have a minor impact on surface water quality through the deposition of dust sediments.

The works necessary to implement the plan are not significant sources of impact on groundwater and surface water quality.

Emissions to soil/soil

The main potential sources of soil and subsoil contamination or degradation are:

- accidental loss of petroleum products from construction machinery or motor vehicles transporting raw materials and materials.

- uncontrolled storage of raw materials or construction waste directly on the ground.

Soil and subsoil contamination from construction works could only occur in accidental situations. To manage such situations, it is recommended to provide absorbent materials.

Noise and vibration

During the construction phase, the main form of physical pollution associated with the analyzed plan will be the noise and vibration generated by the operation of certain installations, equipment and vehicles during the construction period.

Noise level regulated by SR 10009:2017/C91:2020, "Acoustics. Permissible limits of environmental noise level" is 65 dB(A) at the site boundary. According to the Order of the Ministry of Health No. 119/ 2014 for the approval of the Hygiene and Public Health Norms on the living environment of the population, the continuous equivalent sound level measured outside the dwelling according to SR ISO 1996-2:2018, at 1.5 m above the ground, shall not exceed 55 dB(A) and the noise curve Cz 50. During the night (23:00 - 7:00), the continuous equivalent sound level shall not exceed 45 dB(A) and the noise curve Cz 40.

In terms of the effects of noise on wildlife, currently there is still a low level of knowledge. Barber et al. (2010) indicate that an increase in noise (relative to natural background noise) of 3 to 10 dB(A) can result in a reduction in wildlife alerting distances of 30 to 90%. However, most of the evidence to support this claim comes from the investigation of road noise. Less is known about the effects of intermittent noise over short periods of time. However, it is known that higher noise levels generally lead to the removal of wildlife species from the area, thus Foreman et al. (1998) indicate that a decline in grassland birds occurs at noise equivalent levels above 48 dB(A) and in woodland birds at levels above 42 dB(A).

The noise limit values according to current legislation and standards have been summarized in the following table.

Table 3- 2 Reference values for noise level - db(A)

| Legislation | LAeq 31.5Hz | LAeq 63Hz | LAeq 125Hz | LAeq 250Hz | LAeq 500Hz | LAeq 1kHz | LAeq 2kHz | LAeq 4kHz | LAeq 8kHz | LAeq |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order 119/2014 - for residential areas - day | - | - | - | - | - | - | - | - | - | 55 |
| Order 119/2014 - for residential areas - night | - | - | - | - | - | - | - | - | - | 45 |
| SR 6156:2020 - for the hygiene and public health of the living environment of the population* | 89,4 | 75 | 65,2 | 58,5 | 53,5 | 50 | 47,2 | 45,2 | 43,5 | - |
| SR 10009:2017/C91:2020- for outdoor markets, commercial premises and catering/industrial premises | - | - | - | - | - | - | - | - | - | 65 |
| SR 6156:2020 - for markets, commercial and restaurant/industrial premises** | 96,3 | 82,9 | 74,2 | 67,8 | 63,2 | 60 | 57,4 | 55,4 | 53,8 | - |

The main sources of noise and vibration on site during the works will be:

- the operation of earth-moving machinery used to lay turbine platforms and foundations;

- handling of turbine components.

Noise sources will be temporary. In order to assess the noise level generated by the execution of the project, a worst-case situation has been considered, i.e. the operation of all equipment and machinery involved in the construction activities in three different work fronts: one for the excavation and preparation of the ground, one for the execution of the foundations and platforms of the turbines and one for the actual assembly of the installations.

According to data and information from the literature and similar projects, the machinery involved in the construction of wind infrastructure and the associated noise levels are represented by:

- Excavators - 2 pcs. (117 dB);

- Bulldozers - 2 pcs. (115 dB);

- Autogreder - 1 pc. (112 dB);

- Compaction cylinder - 4 pcs. (105 dB);

- Dumpers - 7 pcs. (107 dB);

- Truck crane - 1 pc. (96 dB);

- Automacara - 1 pc. (83 dB);

- Truck mixer - 2 pcs. (107 dB);

- Hydraulic pile drill - 2 pcs. (120 dB);

- Autotrailer - 3 pcs. (90 dB);

- Excavator - 2 pcs. (116 dB);

- Ground stabilizing equipment - 1 pc. (105 dB).

In order to evaluate the noise level generated in the above scenario, noise source modeling was performed using the software application SoundPLANnoise 9.0. The input data used were represented by:

- digital terrain model of the analyzed area - ASCII format;

- position of the detailed scenario noise sources (STEREO 70 projection coordinates);

- position of the sensitive receivers relative to the noise sources (coordinates in STEREO 70 projection);

- information from literature on the noise level of each type of equipment and machinery that are sources of noise.

Normally construction work will take place during the day between 08:00 and 18:00. There are, however, operations that need to be carried out continuously, such as pouring concrete for foundations, which may also require night work.

The results of the modeling carried out with the SoundPLANnoise software show that, during the construction phase, the noise level generated by the project will not generate a significant impact on the quality of living in the neighboring villages, at the level of the nearest receptors, the operation of the equipment used in the modeling generating a maximum noise level of about 36 dB. The noise generated by the construction activities is not able to change the current noise level induced mainly by the motor traffic on the DJ391.

Also in the most unfavorable situation (illustrated in the previous figure) it is highlighted that the area of significant impact where exceedances of the daytime limit values of 55 dB may occur (work being carried out exclusively during the day), has a maximum area of approximately 4.92 ha located strictly at the working front, at a maximum distance of 300 m from the working front.

No studies have been carried out in the analyzed area to determine the actual noise level. According to the literature a rural background pollution value can be considered as 40 dB during the day and 30 dB at night (Ir. F.W.J. van Deventer, 2014, Handboek Basiskennis Vliegtuiggeluid).

Taking into account the background noise level in the project area (rural area without significant noise sources) of 40 dB(A), the noise level resulting at sensitive receptors from modeling and applying the calculation formula for summing the noise levels.

L=10×log10(10L110+10L110)  dB

It results that the project will not have a major contribution on the current noise level at the identified sensitive receptors in the area, the calculation indicating very small changes, i.e.: between 40.008 dB to 40.016 dB.

Also, for the nearest identified protected natural areas in the project area (BG0000569 Kardam) the noise level estimated from modeling is 27 dB(A) which corresponds to a change on the current noise level of 0.2 dB (40.2 dB).

The modeling results indicate that the project will not have a significant adverse impact on sensitive receptors identified in the study area. In the case of both the social and faunal components, it should be noted that the noise sources that have been estimated using the SoundPLANnoise program are temporary in nature, being present on the site only during the project execution period (about 30 days).

The noise sources presented above may have a potential negative impact on the staff directly involved in the specific project activities. For them, noise discomfort can be minimized by complying with occupational health and safety rules, i.e. by using personal noise protection equipment.

By building the wind farm, the electricity produced by the wind farm will replace an equal amount of fossil fuel energy, which means a reduction in the amount of GHGs associated with non-renewable energy production, thus contributing to climate change mitigation.

Construction works and transportation traffic will cause an increase in noise and vibration levels, but these will be local, in the area of the work fronts and in their vicinity.

Soil pollution and changes in soil quality, i.e. pollution of surface water and groundwater with changes in physical, chemical and biological qualities could be caused by accidental/uncontrolled spills of pollutants directly on the soil or in water, deposition of potentially contaminated dust on the ground with other air pollutants from excavation, improper waste disposal, improper wastewater discharge.

The following quantities of resources will be used for the realization of the wind farm: topsoil from stripping about 108600 m3 , natural earth from excavation 24350 m3 , crushed stone fill 114250 m3 , to these quantities are added insignificant quantities of water for various activities on site (sprinkling to reduce dust pollution when necessary, spraying of foundations during the drying period, etc.). Water will be brought to the site as needed.

During the operation of the wind farm, the only natural resource used is wind energy, which is inexhaustible.

The park is located in the Cerchezu local administrative unit, in its extravilanan, the nearest localities being: T32 at 1.1 km from Viroaga, T5 at 1 km from Cerchezu, T35 at 1 km from Măgura, T3 at 1.56 km from the Bulgarian locality Iovkov, T47 at 2.1 km from the Bulgarian locality Kraiste, T9 at 3.98 km from Negru Voda.

A total of 10 protected natural areas have been identified in the area of the wind energy park, on a distance of about 20 km from the PUZ boundary:

- 1 protected natural area identified in the area of direct influence (ROSAC0071)

- 6 protected natural areas identified in the area of indirect influence (ROSPA0036, ROSPA0166, ROSPA0094, ROSAC0157(ROSCI0157), BG0000570(Bulgaria), BG0000130 (Bulgaria))

- 3 protected natural areas which are not within the direct or indirect influence of the PUZ ( BG0000569 (Bulgaria), ROSPA0151, ROSPA0001)

- 3 of these nature protected areas are found in Bulgaria's administrative territory.

Analyzing the distance and the species/habitats for which these areas have been designated, it has been identified that a number of 5 natural protected areas on the territory of Romania may be potentially affected directly/indirectly by the PUZ.

The assessment of the impact of the plan on the Natura 2000 sites likely to be affected has been carried out on the basis of the Specific Conservation Objectives set by ANANP for the period 2021-2023 for all the sites considered in the assessment. The assessment took into account the potential cumulative impact potentially generated together with the proposed and existing wind farms both on Romanian and Bulgarian territory.

The most important potential form of impact associated with the plan is the reduction in wildlife populations, which may occur during the construction and operational phases as a result of construction works, collisions with construction traffic and mainly collisions with wind turbines (especially for bird and chiropteran species). The reduction in population numbers is likely to have a significant impact on wildlife populations and affect the population size parameters of the specific conservation objectives set for the species.

In order to assess the potential impact of the realization of the PUZ on wildlife species, the mortality risk for chiroptera and the mortality risk for avifauna were analyzed in the Appropriate Assessment Study.

Mortality risk - chiroptera

As regards the analysis of mortality risk for chiroptera species, this was carried out using NatureScot's Bats and onshore wind turbines - survey, assessment and mitigation, which takes into account both the risk of collision of bat species and the risk of barotrauma due to pressure differentials generated by wind turbine operation, the vulnerability of the species, their activity in the project area (assessed based on field observations), the location of the project and the types of interventions proposed.

Based on the methodology described, the plan site presents a moderate-value habitat, i.e., an area with buildings, trees or other structures with moderate-high potential to serve as roosts, which may be widely used by foraging bats. In terms of the size of the project it fits in with larger projects (> 40 turbines) with other wind energy projects under development within a 5km radius of it, with turbines greater than 100m in height. Analyzing the initial risk assessment matrix the project site falls into the high risk category.

Taking into account the information listed above as well as the monitoring data and using the general risk assessment matrix, the plan site has a high risk of collision and therefore mortality for chiroptera.

However, existing methodologies do not allow the calculation of a potential number of casualties for bat species, as is done for bird species.

Bird mortality risk

The collision risk of the observed species is calculated according to the Scottish National Heritage guide "Wind farms and Birds: Calculating a theoretical collision risk assuming no avoiding action".

For this plan was prepared the "Study on the risk of collision of gliding birds in the future Cerchezu Wind Farm, Constanta County", document, Annex to the Adequate Assessment Study.

The findings of the Gliding Bird Collision Risk Study showed that:

- Of the 18 species for which the risk of collision with wind turbines has been assessed, 12 species show a positive final collision risk reported over one year, namely for the species: Accipiter nisus (0.014 individuals/yr), Aquila pomarina (0.066 individuals/yr), Buteo buteo (0.605 individuals/yr), Buteo rufinus (0.031 individuals/yr), Ciconia ciconia (0.998 individuals/yr), Circaetus gallicus (0.009 individuals/yr), Circus aeruginosus (0,025 individuals/year), Circus cyaneus (0,008 individuals/year), Corvus corax (0,007 individuals/year), Falco subbuteo (0,006 individuals/year), Falco tinnunculus (0,201 individuals/year), Pernis apivorus (0,008 individuals/year). Of these, none of them show annual supra-unit values.

- all 12 species with a positive final collision risk reported on a yearly basis are bird species that, according to the Red List of European Bird Species (Birdlife International, 2015), are classified as species of Least Concern. At Romanian level, according to the Red List of bird species in Romania, three species are classified as Near Threatened, one is assessed as Vulnerable and two have not been assessed (Not Evaluated).

- to determine the significance of the impact of the Cerchezu wind farm operation on the species of raptors and gliding birds, the population size of the species in the study area was used (where possible) as a reference, wishing to compare the victims of the collision with the relevant population of the species, either the nesting or wintering population, or the population in passage. Thus, the population values of the species in the neighboring Natura 2000 sites ROSPA0036 Dumbraveni and ROSPA0166 Plopeni-Chirnogeni were consulted, considering that a 1% reduction of the minimum population within the site represents a significant impact on the species, and may prevent the achievement of the site's conservation objectives. However, there have been situations where within the conservation objectives of the sites ROSPA0036 Dumbraveni and ROSPA0166 Plopeni-Chirnogeni did not show species population values or did not show relevant population values (such as the case of the species Accipiter gentilis, Accipiter nisus, Buteo buteo, Buteo rufinus, Ciconia ciconia, Ciconia nigra, Circaetus gallicus, Circus aeruginosus, Corvus corax, Corvus frugilegus, Falco columbarius, Falco subbuteo, Falco tinnunculus, Pernis apivorus). The national population sizes of the species as reported by Romania in accordance with Art. 12 of the Birds Directive (available at https://nature-art12.eionet.europa.eu/article12/summary) were consulted in this situation, considering that a 0.05% reduction of the national minimum population is considered a significant impact.

The conclusions of the Appropriate Assessment Study detailed for each affected ANPIC can be found as Annex to this study.

The details of the assessment of the cumulative impact of the plan proposed by the PUZ with the relevant projects with which it may have an impact are presented in the Appropriate Assessment Study, chapter 4. 4. Analysis of pressures and threats.

Details of the potential impact of the PUZ on species and habitats in protected natural areas are extensively presented in the Appropriate Assessment Study.

# The environmental protection objectives, established at national, Community or international level, which are relevant to the PUZ and how these objectives and any other environmental considerations have been taken into account during the preparation of the PUZ

  - Environmental protection objectives, set at national, Community or international level, which are relevant to the plan or program

In order to create the framework for the assessment of the environmental effects generated by the implementation of the PUZ subject of this Environmental Report, in the scoping stage of the SEA procedure, several relevant environmental objectives, directly related to:

- Environmental aspects indicated in Annex 2 of HG 1076/2004;

- The environmental issues relevant to the PUZ presented in chapter 5 of this Environmental Report, identified on the basis of the analysis of the current state of the environment in the study area.

- Correlation of the analyzed PUZ with relevant plans/programs/strategies/guidelines:

  - Energy Union Package (2015) A framework strategy for a resilient Energy Union with a forward-looking climate change policy;

  - Paris Agreement;

  - European Green Pact

  - Climate and Energy Card 2030 - Setting a more ambitious climate target for Europe towards 2030 Investing in a climate-neutral future for the benefit of citizens:

  - The "Ready for 55" legislative package;

  - REPowerEU Plan;

  - The Union's Environment Action Program to 2030, 8th EAP, 2022;

  - A Green Deal Industrial Plan for the Net Zero Emissions Era;

  - EU Biodiversity Strategy to 2030.

  - European directives, regulations and laws:

    - European Climate Act (EU Regulation 2021/1119);

    - Directive on the promotion of the use of energy from renewable sources (EU Directive 2018/2001 with subsequent amendments and additions (Reg. EU 2022/759, Dir. EU 2023/2413));

    - Regulation (EU) 2022/869 on guidelines for trans-European energy infrastructure

    - Regulation (EU) 2022/2577 establishing a framework for accelerating the deployment of renewable energy;

    - Nature Restoration Law, Proposal for a Regulation on nature restoration

  - National Strategies and Plans:

    - Renewable Energy Strategy;

    - Romania's Energy Strategy 2007-2020;

    - Romania's Energy Strategy 2022-2030, with a 2050 perspective;

    - Integrated National Energy and Climate Change Plan 2021-2030;

    - National Integrated Energy and Climate Change Plan 2021-2030;

    - Romania's National Strategy for Sustainable Development 2030 and Action Plan;

    - National Strategy on Climate Change and Low Carbon Growth for 2016-2020 and National Action Plan;

    - National Strategy on Adaptation to Climate Change for the period 2022-2030 with a 2050 perspective and National Action Plan;

    - Romania's long-term strategy to reduce GHGs Romania Neutral in 2050;

    - National Strategy and Action Plan for Biodiversity Conservation 2014-2020;

    - National circular economy strategy

    - Updated Management Plan of the Danube-Littoral River Basin.

The correlation of the analyzed PUZ with the mentioned relevant plans/strategies/guidelines aim at achieving the following environmental objectives, relevant to the analyzed plan, for each environmental aspect and are presented in the following table:

Table 4. 1 Objectives relevant for the PUZ

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Targets |
| --- | --- | --- |
| Water | - Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | Maintain/improve by 2027 the environmental objectives for water bodies with poor ecological and chemical status/potential. |
| Air | - Maintaining air quality by reducing emissions from energy sector activities | Maintaining the level of air pollutants within the limits imposed by Law 104/2011 on ambient air quality. |
| Soil/subsoil | - Limiting soil pollution and land degradation. | Prevention of deterioration and impairment of soil and subsoil quality. Limitation of occupied areas. |
| Use of natural resources | - Reducing the exploitation of exhaustible resources and facilitating the exploitation of renewable ones | Producing around 303 MW of renewable capacity |
|  | - Promoting the transition to a circular and resource-efficient economy | Waste recovery |
| Waste management | - Reducing the amounts of waste generated and disposed of through landfilling (compliant landfills) | Achieving a level for reuse, recycling and other material recovery operations of at least 70% by mass of non-hazardous waste from construction and demolition activities |
| Noise and vibration | - Limitation, at source, of noise pollution in areas with noise sensitive receptors. Limitation of vibration levels | Compliance with the maximum permissible limits for noise and vibration set by Order 119/2014 on Hygiene and Public Health Norms regarding the living environment of the population: - 65 dB for the industrial enclosure boundary; - 55 dB outside during the day and 45 dB at night; - 50 dB (A) during the day and 40 dB (A) at night in areas in the vicinity of a protected territory. |
| Biodiversity | - Conservation of natural habitats and species of flora and fauna of Community importance | Maintaining/improving the conservation status of natural habitats and species of flora and fauna of Community importance |
|  | - Maintaining the Natura 2000 network of protected areas. | Avoiding/minimizing occupied areas |
| Population and human health | - Maintaining the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health | Keeping the level of air pollutants within the limits imposed by L104/2011 on air quality Compliance with the maximum permissible limits for noise and vibration set by Order 119/2014 on Hygiene and Public Health Norms regarding the living environment of the population: - 65 dB for the industrial enclosure boundary; - 55 dB outside during the day and 45 dB at night; - 50 dB (A) during the day and 40 dB (A) at night in areas in the vicinity of a protected territory |
|  | - Improving living standards and social conditions of the population | Economic development of the area |
| Landscape | - Ensuring protection of the natural landscape and reducing the impact on the natural landscape | Regulating zones and the way in which they are built, in relation to their functions, in order to ensure the most aesthetically pleasing landscape |
| Climate change | - Reducing GHG emissions from the energy sector | The share of RES in gross final energy consumption of 86.1% in 2050 and 36.2% in 2030 for the Neutral EN scenario |
|  | - Minimal risks of climate change (adaptation) | Climate resilient energy infrastructure design |
| Cultural heritage | - Protection and conservation of cultural heritage | Ensure the protection and conservation of all archaeological sites identified during the construction phase |

  - The manner in which these objectives and any other environmental considerations have been taken into account during the preparation of the PUZ

In December 2015, for the first time, all the world's countries agreed on a collective effort to keep global warming well below 20 C, trying to limit it to 1.50 C and combat the effects of climate change under the Paris Agreement.

The Paris Agreement aims to strengthen the global response to the threat of climate change, in the context of sustainable development and poverty eradication efforts, including through:

- keeping global average temperature increase well below 2°C above pre-industrial levels;

- Increasing the capacity to adapt to the adverse effects of climate change and fostering climate resilience and low greenhouse gas emitting development in a way that does not jeopardize food production;

- taking the necessary steps to ensure that financial flows are consistent with a move towards low greenhouse gas emission and climate resilient development.

At European level, 2019 saw the adoption of the European Green Deal, an ambitious initiative in the form of a package of measures, which encourages and incentivizes all stakeholders to be an integral part of the transition to a green economy that supports the achievement of the goal of climate neutrality.

It sets out a new growth strategy that aims to transform the EU into a fair and prosperous society with a modern, competitive and resource-efficient modern economy, with zero net greenhouse gas emissions in 2050 and economic growth decoupled from resource use. To this end, financing the transition to a fair and inclusive green economy will be provided through:

- Greening EU countries' national budgets, i.e. better integration of environmental aspects into the financial accounting system, with reference to revenues and expenditures for environmental protection, tax reform programs to eliminate fossil fuel subsidies, encouraging environmental and energy state aid priorities and initiatives;

- investments in modernization and upgrading of equipment;

- supporting research and innovation;

- encouraging citizens across the European Union to use private and public infrastructure in hygienic, accessible and healthy conditions;

- decarbonizing the energy sector;

- improving and improving the energy efficiency of buildings;

- exchange best practices and international collaboration on climate change.

Transforming the EU economy for a sustainable future requires a profoundly transformative set of policies for clean energy supply in all economic and industrial sectors, across the production and consumption chain, for large-scale infrastructure projects, in transport, food and agriculture, construction, taxation and social benefits.

The Green Deal thus encourages the EU to take on a more ambitious target to responsibly reduce its GHG emissions, i.e. a reduction of at least 50% by 2030 compared to 1990 levels and moving towards 55%.

The provision of clean, secure and affordable energy through the sustained decarbonization of the energy system, a crucial measure for achieving the climate goals in 2030 and 2050, is relevant to the main objective of the PUZ: the creation of a renewable source of electricity generation. This will develop an electricity sector relying heavily on renewables, to be complemented by the rapid phase-out of coal and decarbonization of gas.

Renewables will play a key role in the transition to clean energy. Smart integration of renewables, energy efficiency and other sustainable solutions across sectors will help achieve decarbonization at the lowest possible cost.

2030 Climate and Energy Framework - Setting a more ambitious climate target for Europe towards 2030 Investing in a climate-neutral future in the interest of citizens aims to change the emissions reduction trajectory to achieve climate neutrality by 2050.

It includes policy targets and objectives for the period 2021-2030 in a wide range of sectors (energy production, industry, transport, buildings, agriculture, waste, land use, etc.).

Decarbonizing both energy supply and demand is essential to achieve climate neutrality. Achieving the 55% greenhouse gas reduction target will lead to a new and greener energy mix. By 2030, coal consumption would fall by more than 70% compared to 2015, oil by more than 30% and gas by more than 25%, while the share of renewable energy would increase. By 2030 it would reach 38-40% of gross final consumption. Overall, this would lead to a balanced trajectory towards climate neutrality by 2050.

The "Ready for 55" legislative package includes the revision of 13 interlinked pieces of legislation and 6 new legislative proposals on climate and energy. It tackles the energy aspects of the EU's climate transition through two legislative proposals covering:

- Renewable Energy Directive and

- Energy Efficiency Directive.

The new binding EU-wide target of 42.5% renewables in the overall energy mix by 2030, up from at least 32% previously.

The Council has set the following indicative objectives relevant to the proposed PUZ:

- 1.1% average annual increase in the use of renewable energy for industry.

- at least 49% share of renewable energy in buildings in 2030

The Council included accelerated permitting procedures for renewable energy projects in line with the priorities of the RePowerEU plan.

The REPowerEU plan aims to rapidly reduce Europe's dependence on fossil fuels from Russia by accelerating the transition to clean energy and working together to build a more resilient energy system and a true energy union. It introduces an additional set of actions aimed at:

- save energy;

- diversify supply;

- to rapidly replace fossil fuels by accelerating Europe's transition to clean energy;

- intelligently combine investment and reforms.

The plan proposes to increase the Renewable Energy Directive target for 2030 to 45%.

As a member of the European Union, Romania is aligning itself with the common effort for climate neutrality by integrating the targets set at European level into national legislation.

The first steps have been taken through Romania's Energy Strategy 2007-2020, the general objective of which is to meet the energy needs both currently and in the medium and long term, at the lowest possible price, appropriate for a modern market economy and a civilized standard of living, in conditions of quality, security of supply, respecting the principles of sustainable development.

The strategic objective Energy Security is to increase energy security by ensuring energy supply and limiting dependence on imported energy resources, while the strategic objective Sustainable Development is to promote the production of energy from renewable resources.

According to the studies on which the strategy was based, Romania's theoretical annual wind energy potential is estimated at 1.2 TWh, and the Dobrogea area is classified as having solar and wind energy potential.

Romania's Energy Strategy 2022-2030 is currently in the process of being adopted, with the perspective of 2050, which has eight fundamental strategic objectives that structure the entire analysis and planning approach for the period 2022-2030 and the time horizon of 2050.

The objectives of the strategy support the achievement of the national 2030 targets:

- 43.9% reduction in emissions from ETS sectors compared to 2005, and a 2% reduction in emissions from non-ETS sectors compared to 2005;

- 30.7% share of renewable energy in gross final energy consumption;

- 40.4% reduction in final energy consumption compared to the PRIMES 2007 projection.

As an obligation of the Energy Union Strategy, in 2021, Romania adopted the National Integrated Energy and Climate Change Plan 2021-2030, which set out the national objectives and contributions to achieving the EU climate change objectives.

Thus, the level of ambition for the share of renewable energy is 30.7% in 2030, which implies additional RES capacities of about 6.9 GW compared to 2015, and measures are foreseen for the development of new RES capacities and integration with other markets in the region.

Decarbonizing the energy sector, by achieving the targets for renewable energy in gross final consumption of electricity as follows:

- 564.6 ktoe in 2020;

- 828.8 ktoe in 2025;

- 1004.9 ktoe in 2030.

Projections to the year 2030 project an increase in wind capacity to 5,255 MW. The net SRE generation capacities required to be installed are:

- + 822 MW additional installed capacity in 2022 compared to 2020;

- + 559 MW additional installed capacity in 2025 compared to 2022;

- + 556 MW additional installed capacity in 2027 compared to 2025;

- + 365 MW additional installed capacity in 2030 compared to 2027.

The Plan is currently being updated. Romania's target for the share of RES in gross final consumption of energy in 2030 is 36.2%, which is higher than the European Commission's recommended target of 34%. The share of RES in the electricity sector will increase to 55.8% in 2030, as a result of the construction and commissioning of new RES (mainly wind and solar) electricity generation capacity. On the other hand, due to the declining use of biomass, especially in rural areas, which will be replaced by cleaner technologies, the share of RES in the heating and cooling sector will increase slightly by the end of the decade, reaching 36.3% in 2030.

Wind energy will have the largest share, about 37%, followed by solar with about 24% of gross final energy consumption. These targets will be achieved through the construction and commissioning by 2050 of a total installed capacity of solar power plants of about 30.5 GW (both ground-mounted and rooftop) and a total installed capacity of wind power plants of about 16 GW.

The National Strategy on Climate Change and Low Carbon Growth for 2016-2020 and the National Action Plan have as their main objective to reduce GHG emissions from economic activities in line with EU targets and to adapt to the impacts of climate change, both current and future.

The SC Strategy adopts quantifiable targets in line with the EU 2030 aspirations, including a 40% reduction in total GHG emissions from 1990 levels. Energy sector adaptation measures of relevance to the proposed PUZ include diversification of energy production and in particular through the inclusion of renewable sources.

Romania's long-term strategy for GHG reduction Romania Neutral to 2050 aims to reduce net GHG emissions by 99% by 2050, compared to 1990 levels.

The studied land is located in a favorable area for the development of the wind energy electricity generation function, both in terms of the presence of wind potential, topography and the presence of infrastructure - electricity grids, as well as in terms of the availability of the local community and investors.

The sectoral targets for reducing net GHG emissions by 2050 under the Neutral Romania scenario include:

- Energy sector: According to the Neutral Romania scenario, Romania needs to reduce its net emissions by 78% in 2030 and its non-LULUCF emissions by 67% compared to 1990 in order to be on track to reach climate neutrality in 2050. According to the REF scenario, net emissions will be reduced by 85% in 2050 compared to 1990. In 2019, 69% of the 2050 emission reduction target has already been achieved. By 2035, 98% of the target will have been achieved.

- Transport: Cut sectoral emissions by 82% in 2050 compared to 1990 levels;

- Buildings sector: Emissions in 2050 78% below 1990 levels;

- Industry Sector: 2050 emissions 89% below 1990 levels;

- Agriculture: Emissions in 2050 49% below 1990 levels;

- LULUCF sector: increase absorption by 14% in 2050 compared to 1990, similar to the level already reached in 2005;

- Waste Sector: Emissions in 2050 73% below 1990 levels;

For the Neutral Romania scenario, the share of RES in gross final energy consumption will be 86.1% in 2050 and 36.2% in 2030.

For the development area of the PUZ the development potential with significant economic impact is the almost permanent presence of winds. Together with this potential the area can be utilized for agriculture.

According to HG 1535/2003 on the approval of the Strategy for the Valorization of Renewable Energy Sources, five distinct wind energy zones have been identified in Romania, according to the existing energy potential, environmental and topographical conditions. Within these regions, favorable locations for the siting of wind turbines are those aiming at "energy exploitation of the flow over the hilltop effect or the effect of channelling air currents".

The Dobrogean plateau which is the beneficiary of a "mild" climate is part of one of the wind areas with high energy potential.

The proposed plan to capitalize on the natural potential of the area is an opportunity that must be approached with the utmost professional exigency, with critical discernment, so that the present effects of such an objective do not produce urban dysfunctions, irreparable degradation of environmental conditions, changes to ecosystems present in the area.

# Existing environmental problems which are relevant to the PUZ, including in particular those related to any area of particular environmental importance, such as special bird protection areas or special conservation areas

Based on the analysis of the available data on the current state of the environment (presented in chapter 2 of the Environmental Report) the following environmental issues relevant for the PUZ have been identified:

| Relevant environmental aspects | Environmental problems identified | Associated causes |
| --- | --- | --- |
| Water | Risk of affecting groundwater and surface water quality | The surface water bodies ROCT02_B2 Eforie Nord - Vama veche and ROCT01CAPM Mangalia located in the area of implementation of the PUZ have bad ecological status, and the groundwater bodies RODL10 Dobrogea de Sud and RODL04 Cobadin Mangalia have bad chemical status. In Cerchezu commune, the water operator RAJA S.A. Constanta provides the water supply, but the commune does not have a centralized sewerage system, which affects the water table and contributes to soil and groundwater pollution. |
| Air | Risk of local exceedances of local limit values for certain pollutants emitted to air (NOx , SOx , PM10 , PM2,5 , COx NMCOV) | From the analysis of regional and local level data, the main sources of air pollution are transportation, construction works in progress in the TAU and the machinery involved in the works, as well as agricultural activities. The lack of air quality data at local level and the data available at European level allow a rough estimation of the air quality status in the analyzed area. Thus, the concentration value of PM emissions10 is lower than the annual limit value and the upper assessment threshold, but higher than the lower assessment threshold. Emissions of PM2,5 are below the annual limit values and the upper assessment threshold but above the lower assessment threshold. NO emissions2 and NO emissionsx are below the annual limit value, the critical level for the protection of vegetation and the upper and lower assessment thresholds. |
| Ground / Basement | Deterioration of soil characteristics and functions, i.e. changes in their bioproductive capacity | On the administrative territory of Cerchezu commune the soil quality is not affected by industrial activities. The areas with a potential risk of soil pollution are represented by agricultural activities, namely the improper use of fertilizers that can lead to the excess of nitrates and phosphates, with effects on soil microflora and their accumulation in vegetation. On the administrative territory of Cerchezu there are no historically contaminated sites and no potentially contaminated sites. No areas affected by impoverishment or acidification have been identified in the administrative territory. Uncontrolled or improper disposal of waste. |
| Use of natural resources | Consumption of natural resources | The use of renewable resources in Cerchezu is low. The construction of the wind farm will facilitate the use of the wind, a renewable resource characteristic of the development area of the PUZ. |
| Waste management | Pollution of the environment through improper waste management. | In the municipality of Cerchezu the sanitation activity is provided by Iridex Grup Salubrizare S.R.L. which collects and transports separately municipal and similar waste, waste from houses from interior/exterior refurbishment/ rehabilitation activities from all generators. It also collects and transports all types of recyclable industrial waste (cardboard and paper, plastics, glass, etc.) for recovery. Wind farm construction and maintenance generates waste. Improper management of this waste can lead to environmental pollution. |
| Noise and vibration | Noise pollution and vibrations affecting local wildlife and residents. | In the Cerchezu commune are the road traffic on the county road DJ391 and on the municipal roads DC13, DC16, DC17, DC18 and DC19, as well as the construction/demolition activities through the operation of machinery used in the execution of landscaping works / erection / pouring of concrete, vehicles transporting materials. Being an open rural area uncrowded by urban constructions, without industrial activities and without heavy traffic on the internal roads and connecting roads with other localities, the noise level in Cerchezu Commune is low. Wind turbines generate noise and vibrations, which can negatively affect local wildlife and the comfort of nearby residents. |
| Biodiversity | Disturbance of natural habitats and local species. | The lands in the perimeter studied by the PUZ fall into the category of land partly occupied by agriculture, with significant areas of natural vegetation, arable land, meadows, meadows, watercourses, irrigation canals. A number of 3 natural areas of Community interest have been identified in the vicinity of the PUZ at a distance of less than 5 km (one on the territory of Romania and 2 on the territory of Bulgaria). Implementation of the plan may lead to negative biodiversity impacts on both potential (feeding/nesting/breeding) and wildlife habitats. The construction and operation of the wind farm presents risks for some mammals identified in the area of implementation but also for avifauna as a result of collisions with turbines and for chiroptera (bats) due to the phenomenon of barotrauma (sudden change of pressure in the range of action of turbine blades). |
| Population and human health | Discomfort due to noise and intermittent shadowing effect | Studies of the perception of noise and vibration generated by wind turbines indicate that the level of discomfort or annoyance can vary according to people's individual attitudes and sensitivities. In general, modern wind turbines have been optimized to reduce noise and vibration and minimize the impact on the community. The shadowing effect of moving turbine rotors can affect the illumination and visual comfort of people in the surrounding area. |
| Landscape | Neglect, distance from the natural environment | Aesthetic integration of the wind farm into the natural landscape of the area needs to be carefully designed to preserve the beauty and natural features of the area. Excavations and constructions that disregard heritage sites. |
| Climate change | Sources of GHG emissions contributing to climate change. | In the municipality of Cerchezu, a rural area with no developed industry, GHG emissions are insignificant, being represented by residential heating and transportation. The realization of the investments foreseen by the PUZ may contribute to the increase in GHG emissions associated with the manufacture, transportation and installation of the turbines, but the contribution is insignificant and for a limited period (the lifetime of the wind farm). The implementation of the investments envisaged by the PUZ contributes to the reduction of CO2 emissions by replacing a quantity of electricity produced, in the absence of the wind farm, from fossil fuels. |
| Cultural heritage | Possible damage to archaeological sites or other cultural heritage. | Excavations and constructions that disregard heritage sites. According to the opinion issued by the County Directorate for Culture Constanta, the site is located in an area of archaeological interest, prior to the approval of the documentation in the DTAC phase it is necessary to carry out an archaeological diagnosis with a specialized institution. |

# Potential significant environmental effects

  - Evaluation Methodology

The finalization of the Zonal Urban Plan, the establishment of the scope and level of detail of the information to be included in the environmental report, as well as the analysis of the significant effects of the plan on the environment were carried out in working groups, according to Article 14 of HG 1076/2004 on establishing the procedure for carrying out the environmental assessment for plans and programs.

In determining the potential significant effects on the environment for the PUZ, the criteria set out in Annex no. 1 of HG 1076/2004 were taken into account and whether the plan proposals provide a framework for meeting the relevant environmental objectives set for each environmental factor (presented in the previous section of the report).

The quantification of the effects of the proposals in the PUZ (selected optimal alternative) was carried out using an evaluation matrix based on a numerical scoring system and a color coding system - presented in the following table.

The following elements were taken into account to determine the magnitude of potential effects:

- Types of interventions arising from the realization of the investment proposed by the plan

- Magnitude of the changes brought about by the investment proposed by the PUZ - "Wind energy park 48 (46) wind power plants approx. 316,8MW (303,6MW), Transformer Stations, Electrical connection networks, construction and modernization of communication and access roads" (construction period, operation period and decommissioning period)

- Environmental impact assessment (quantitative and qualitative changes generated by the investment proposed by the PUZ)

- The quantification of the effects of the proposals in the PUZ (selected optimal alternative) was carried out using an evaluation matrix based on a numerical scoring system and a color coding system - presented in the following table.

Table  6-1 Proposed scoring system for the assessment of potential environmental effects

| Notation/ color code | The scale of the impact | Description |
| --- | --- | --- |
| +2 | Significant positive impact | It refers to major (significant) effects of a positive nature, which are long-term or permanent, wide-ranging and contribute to the achievement of the environmental objective. |
| +1 | No significant positive impact | It refers to minor (low) direct or indirect positive effects of a minor (low) nature, which are felt locally and which may partially contribute to the achievement of the environmental objective. |
| 0 | No effect/effect cannot be determined | No effects, extremely low effects or effects for which no accurate predictions can be made, further details from the field about the characteristics of the proposed projects and their size are needed. |
| -1 | Negligible negative impact | It refers to minor (low) direct or indirect negative effects of a direct or indirect negative nature that are felt locally and make it difficult to achieve the environmental objective. |
| -2 | Significant negative impact | It refers to major (significant) adverse effects that are of a long-term or permanent nature, have a wide scale of coverage and do not achieve the environmental objective. |

For the quantification of the effects for each environmental aspect identified, the notes are accompanied by justifications.

In particular, the medium and long term effects of the PUZ alternatives on the relevant environmental objectives were taken into account in the scoring. At the same time, for the individual assessment of each provision of the Plan as well as for the cumulative assessment, situations were taken into account in which there would be the possibility of interaction between two or more environmental factors as a result of the implementation of the PUZ provisions:

- Air-biodiversity;

- Air-population and human health;

- Climate change - biodiversity;

- Climate change - population and human health;

- Waste management - soil - water;

- Waste management - biodiversity;

- Waste management - population and human health;

- Air - soil - water;

- Soil - water - biodiversity;

- Soil - water - population and human health;

- Noise - biodiversity - population and human health;

- Noise - biodiversity;

- Noise - cultural heritage and historical monuments;

- Air - cultural heritage and historical monuments;

This assessment methodology allows on the one hand the rapid visualization of the effects on environmental objectives and creates the possibility to understand the type, nature and duration of the effect, and on the other hand it allows the rapid identification of environmental objectives that will not be achieved without the application of appropriate measures to minimize/reduce the negative effects.

Also, taking into account the provisions of the "Generic Guide on Environmental Assessment for Plans and Programs", an assessment of the compatibility of the objectives/provisions of the PUZ with the relevant environmental objectives was also carried out.

In the process of realizing the PUZ 4 alternatives were studied:

- Alternative without PUZ implementation ("Zero" Alternative)

- Alternative with implementation of PUZ investments initiated in 2011 (composed of 3 wind farms) - 117 wind power plants with a total installed capacity of 351 MW (Alternative "I")

- Alternative with implementation of PUZ investments initiated in 2022 - 48 wind power plants with a total installed capacity of 316.8 MW (Alternative "II")

- Alternative with the implementation of the PUZ investments after obtaining the TRANSGAZ SA approval no. 34618/948/10.05.2023, as a result of which 2 T17 and T23 plants were abandoned, thus the new wind farm will comprise 46 wind power plants with a total installed capacity of 303.6 MW - (Alternative "III")

Table  6-2 PUZ characteristic elements in the three project alternatives

| Nr. crt | Features of the PUZ | Alternative I (2011) | Alternative II (2022) | Alternative III (2023) |
| --- | --- | --- | --- | --- |
| 1. | Area surveyed (ha) | 476.99 ha | 3027,77 | 3027,77 |
| 2. | Number of wind turbines | 117 | 48 | 46 |
| 3. | Turbine characteristics - Post height: - Blade diameter: - Total height: - Power | 119 m 56 m 175 m 3.0 MW | 155 m 170 m 240 m 6.6 MW | 155 m 170 m 240 m 6.6 MW |
| 4. | Installed capacity MW | 351 MW | 316.8 MW | 303.6 MW |

    - Evaluation of PUZ objectives

The purpose of this assessment is to identify the synergies between the two types of objectives (the objectives of the PUZ and the relevant environmental objectives) as well as possible incompatibilities between them.

As presented in chapter 1.1.2. of this environmental report, the PUZ includes the following objectives:

- OP1 to study the area and promote an alternative in the use of certain areas of land in the Cerchezu TAU for the construction of a wind farm;

- OP2 Reduce the amount of greenhouse gas emissions from the energy sector;

- OP3 Promote the transition to a circular and resource-efficient economy;

- OP4 Enhance the natural setting and harmonize it with the proposed site;

- OP5 Wind farm development with consideration of environmental impacts and protection of human health;

- OP6 Economic and social growth of the locality;

- OP7 Reduce impacts on biodiversity by ensuring biodiversity protection and conservation measures.

The relevant environmental objectives identified and proposed for assessment have been presented in Chapter 4 of this environmental report.

The evaluation of the objectives of the proposed PUZ was carried out using a matrix evaluation system.

Within this matrix the following notations have been used:

- "+" if the objectives are compatible,

- "x" if the objectives are not compatible,

- "-" for the situation where no link has been identified between the two categories of objectives assessed.

- "?" where compatibility depends on certain presumptions

- "=" if the objectives are identical

Table  6-3 Compatibility between the objectives of the PUZ

| PUG Objectives | OP1 |
| --- | --- |
| OP2 | + | OP2 |
| OP3 | + | + | OP3 |
| OP4 | + | ? | + | OP4 |
| OP5 | + | + | + | + | OP5 |
| OP6 | + | + | + | ? | + | OP6 |
| OP7 | + | ? | ? | + | ? | ? | OP7 |

The assessment of the compatibility between the objectives resulted in the following:

- 71% of the PUZ objectives are compatible with each other.

- There were objectives between which direct links could not be identified, the link between them depending on a number of assumptions - 29%.

- For the PUZ no objectives have been formulated that are identical or incompatible with each other.

Table  6-4 Compatibilities between PUZ objectives and relevant environmental objectives

| Objectives PUz | Relevant environmental objectives |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Water | Air | Soil/Subsoil | Use of natural resources |  | Waste management | Noise and vibration | Biodiversity |  | Population and human health |  | Landscape | Climate change |  | Cultural heritage |
|  | OM1 | OM2 | OM3 | OM4 | OM5 | OM6 | OM7 | OM8 | OM9 | OM10 | OM11 | OM12 | OM13 | OM14 | OM15 |
| OP1 | ? | + | + | + | - | ? | ? | + | + | + | + | ? | + | ? | - |
| OP2 | - | + | - | + | - | - | ? | - | - | + | + | ? | + | + | ? |
| OP3 | ? | ? | ? | + | = | + | - | ? | - | + | + | ? | - | - | - |
| OP4 | ? | ? | ? | ? | ? | - | ? | + | + | + | + | + | - | + | + |
| OP5 | ? | + | + | + | - | ? | ? | + | + | + | + | ? | + | ? | - |
| OP6 | - | - | - | - | - | - | - | - | - | + | = | - | - | + | + |
| OP7 | ? | ? | ? | ? | ? | ? | ? | = | + | + | + | + | - | - | - |

As regards the assessment of the compatibility between the objectives of the PUZ and the environmental objectives, the following conclusions can be drawn:

- For OP1 there is compatibility with the environmental objectives set for air, soil, use of natural resources (exploitation of renewable resources), biodiversity, population and human health, climate change (reduction of GHG emissions), there is compatibility depending on certain assumptions with the environmental objective set for water, waste management, noise and vibration, landscape and climate change.

- OP2 is compatible with the environmental objective set for air, use of natural resources (exploitation of renewable resources), population and human health and climate change, compatibility with the environmental objectives set for noise, landscape and cultural heritage depends on certain assumptions.

- For OP3 there is compatibility with the environmental objective set for the use of natural resources (exploitation of renewable resources), waste management, population and human health, and there is compatibility dependent on certain assumptions with the environmental objective set for water, air, soil, biodiversity (habitat conservation) and landscape. OP3 and OM5 are identical.

- OP4 is compatible with the environmental objective set for biodiversity, human population and health, landscape, climate change (adaptation) and cultural heritage, and compatibility with the environmental objectives set for water, air, soil, natural resource use, noise and vibration depends on certain assumptions.

- For OP5, there is compatibility with the environmental objective set for air, soil, use of natural resources (exploitation of renewable resources), biodiversity, population and human health and climate change (reduction of GHG emissions) and there is compatibility dependent on certain assumptions with the environmental objective set for water, waste management, noise and vibration, landscape and climate change (adaptation).

- OP6 is compatible with the environmental objective set for human population and health, climate change (adaptation) and cultural heritage. OP6 is identical to OM11.

- OP7 is compatible with the environmental objective set for biodiversity, population and human health and landscape and there is compatibility dependent on certain assumptions with the environmental objective for water, air, soil, use of natural resources, waste management and noise and vibration. OP7 is identical to OM8.

The identification of potential environmental effects took into account:

- all activities necessary for the realization of the project;

- all the components of the receiving environment, i.e. the environment that will bear the effects of the project;

- the current state of the receiving environmental factor (environmental factor quality);

- all possible interactions between activities and environmental components.

The assessment was carried out for each studied alternative, taking into account all project-specific phases (construction, operation and decommissioning phases). The decommissioning phase was considered to include works similar to the construction phase.

In the assessment of potential environmental impacts, only investments that create the framework for projects that are listed in Annex no. 2 of Law 292/2018 on the assessment of the environmental impact of certain public and private projects on the environment were considered.

The following table shows the main interventions that will result from the realization of the wind energy park:

Table  6-5 Main interventions resulting from the implementation of the PUZ

| Stage | Intervention type | Activities included |
| --- | --- | --- |
| Construction | Construction of access roads | - Clearing vegetation - Topsoil stripping works - Depositing uncovered soil - Compaction/leveling works - Ballast layer laying works - Site traffic of vehicles/operation of necessary machinery |
|  | Building technological platforms and foundations | - Clearing vegetation - Topsoil stripping works - Depositing uncovered soil - Compaction/leveling works - Ballast/foundation works - Site traffic of vehicles/operation of necessary machinery |
|  | Installing wind power plants | - Fitting the pillar, nacelle and blades - Running the necessary machinery |
|  | Making electrical connections and laying underground cables between the wind generator sets | - Clearing vegetation - Topsoil stripping works - Trench digging works - Disposal of excavated soil - Cable installation work - Filling works - Levelling works - Site traffic of vehicles/operation of necessary machinery |
|  | Building transformer stations | - Clearing vegetation - Topsoil stripping works - Foundation - excavation and concrete pouring - Disposal of excavated soil - Site traffic of vehicles/operation of necessary machinery - Installation of transformer station equipment - Fencing works |
|  | Site organization | - Landscaping - clearing vegetation - Topsoil stripping - Storage of uncovered soil - Ballast layer laying works - Compaction/leveling works - Site traffic of vehicles/operation of necessary machinery |
|  | Land reclamation work at the end of construction | - Restoration of the site where - works and rehabilitation of temporarily used areas. - Work to remove all plant, machinery and waste and to reinstate topsoil on areas that have been temporarily used |
| OPERATE | Maintenance and repairs | - Vehicle traffic/operation of necessary machinery/equipment - Intervention and repair works wind power plants - Current repairs, equipment replacement - Replacing oils/lubricants - Disposal of repair and maintenance waste |
| DEFEAT Interventions and activities are similar to those in the build | Site organization | - Landscaping - clearing vegetation - Landscaping - clearing vegetation - Topsoil stripping - Storage of uncovered soil - Ballast layer laying works - Compaction/leveling works - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Dismantling wind farms | - Removing the blades, nacelle and pillar |
|  | Decommissioning of technological platforms and foundations | - Clearing vegetation - Topsoil stripping works - Depositing uncovered soil - Foundation demolition works - Compaction/leveling works - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Dismantling electrical connections and removing underground cables | - Clearing vegetation - Topsoil stripping works - Trench digging works - Disposal of excavated soil - Removing electrical cables - Trench filling and land levelling works - Restoring the green carpet. - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Decommissioning of transformer stations | - Dismantling transformer substation equipment - Clearing vegetation - Excavation work - Disposal of excavated soil - Demolition of foundations - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Decommissioning of logging roads | - Topsoil stripping works - Disposal of excavated soil - Ballast layer removal works - Compaction/leveling works - Site traffic of vehicles/operation of machinery needed to carry out the works |
|  | Site restoration work on completion of construction | - Restoration of the site where - works and rehabilitation of temporarily used areas. - Work to remove all plant, machinery and waste and to reinstate topsoil on areas that have been temporarily used |

  - Potential significant effects of the alternative without the implementation of the PUZ (Alternative "zero")

The following table assesses the effects of non-implementation of the PUZ on the environmental objectives set out in the previous section:

Table  6-6 Assessment of the effects of not implementing the PUZ

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Impact evaluation | Justification |
| --- | --- | --- | --- |
| Water | - Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | 0 | The non-implementation of the analyzed PUZ will not lead to a degradation of the quality of surface and deep waters. By its specificity the Plan has no impact on the quality of water bodies in the implementation area. Given that the environmental objectives in 2027 for the water bodies are good chemical status, the likely evolution of the quality of the water bodies in the implementation area in the absence of the PUZ is that of improvement. |
| Air | - Maintaining air quality by reducing emissions from energy sector activities | -1 | The non-implementation of the analyzed PUZ will not directly lead to a degradation of the air quality in the area of the site, but given the objective of the Plan to create a renewable source of electricity to replace the traditional sources of electricity generation it can be concluded that the non-implementation of the PUZ indirectly leads to degradation of the air quality. |
| Soil/subsoil | - Limiting soil pollution and land degradation. | 0 | In terms of soil quality, the trend is to maintain the current characteristics. The soil in the area of implementation of the PUZ is classified in quality class III (on a scale from I to V), i.e. medium fertile soil, moderately affected by degradation If this plan is not implemented, it is expected that no changes in soil quality will occur. |
| Use of natural resources | - Reducing the exploitation of exhaustible resources and facilitating the exploitation of renewable ones | -2 | The non-implementation of the PUZ implies the continued exploitation of exhaustible resources (coal, natural gas) for the production of electricity with effects on pollutant emissions to air, water, soil and reduces the potential for capitalizing on the use of renewable resources (wind) |
|  | - Promoting the transition to a circular and resource-efficient economy | -1 | Without the implementation of the PUZ there is no impetus for the transition to a circular economy and resource efficiency. |
| Waste management | - Reducing the amount of waste generated and disposed of by landfilling Eva (compliant landfills) | 0 | No significant changes in waste management are anticipated without the implementation of the PUZ, thus maintaining the current situation. |
| Noise and vibration | - Limitation, at source, of noise pollution in areas with noise sensitive receptors. Limitation of vibration levels | 0 | Without the implementation of the PUZ, there will be no additional sources of noise and vibration, nor will the existing ones be eliminated. |
| Biodiversity | - Conservation of natural habitats and species of flora and fauna of Community importance | 0 | The status quo will be maintained, with no additional disturbance to habitats, but no new measures will be taken to conserve biodiversity. |
|  | - Maintaining the Natura 2000 network of protected areas. | 0 | The non-implementation of the Plan will not influence local biodiversity in the area. However, given the continuous urbanization and the need to implement new infrastructure elements, and taking into account the negative effects of climate change evolution, the flora and fauna at the project may be negatively impacted, leading to the loss or eventual disappearance of some natural elements. However, this hypothesis needs to be confirmed over time through rigorous monitoring of elements potentially sensitive to these factors. |
| Population and human health | - Maintaining the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health | 0 | The health status of the population will not be influenced by the non-implementation of the PUZ. |
|  | - Improving living standards and social conditions of the population | -1 | The non-implementation of the analyzed PUZ will maintain the current state of lack of jobs, continuous depopulation and reduced attractiveness of the area for sustainable investments. |
| Landscape | - Ensuring protection of the natural landscape and reducing the impact on the natural landscape | 0 | The lack of a sustainable vision for the development of Cerchezu will contribute to the continued degradation of the natural landscape and the expansion of urbanization. |
| Climate change | - Reducing GHG emissions from the energy sector | -2 | By not implementing the PUZ, the current structure of fossil fuel power generation will be maintained, which has the direct effect of generating GHG emissions. With an increase in electricity demand, if the current way of electricity generation is maintained, GHG emissions will increase, which will exacerbate climate change phenomena. |
|  | - Minimal risks of climate change (adaptation) | -1 | Without the implementation of the PUZ, it will not contribute to climate change mitigation, maintaining the current risks. |
| Cultural heritage | - Protection and conservation of cultural heritage | 0 | No significant impact on cultural heritage is anticipated without the implementation of the PUZ. |

  - Potential significant effects of the alternatives with the implementation of the PUZ

The purpose of this step is to identify and describe the effects of the PUZ alternatives on environmental objectives.

Identify and describe in the environmental report the potential environmental effects of each of the anticipated outcomes for each alternative.

After identifying the expected outcomes for each development alternative, their potential environmental effects must be predicted and described. On the basis of the assessment and the alternatives, the environmentally selected alternative will be chosen.

Table  6-7 Assessment of the effects of Alternative I PUZ initiated in 2011 in the absence of mitigation measures

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Implementation period |  |  |  |  |  |  |  |  | Operating period |  |  | Decommissioning period |  |  |  |  |  |  |  |  | Justification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Construction of access roads | Building technological platforms and foundations | Installing wind power plants | Making electrical connections and laying underground cables between the wind generator sets | Building transformer substations | Site organization | Land reclamation work at the end of construction | Final evaluation | Maintenance and repairs |  | Final evaluation | Site organization |  | Dismantling wind farms | Decommissioning of technological platforms and foundations | Dismantling electrical connections and removing underground cables | Decommissioning of transformer stations | Decommissioning of logging roads | Site restoration work on completion of construction | Final evaluation |  |  |
| Water | OM1. Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | -2 | -2 | -1 | -1 | -1 | -1 | 0 | -1 | 0 |  | 0 | -1 |  | -1 | -1 | -1 | -2 | -2 | 1 | -1 | No watercourse undercrossing works were foreseen for this alternative at the construction stage at the time of preparation. The main sources of impact are: possible leakage of used construction materials or fuels/lubricants from the machinery involved in the works for the construction of site organization, access roads, technological platforms, electrical connections and cable laying, turbine erection and land reclamation at the end of the works. Erosion caused by excavation works is another factor that can alter the quality of surface water bodies in the implementation area by increasing turbidity. The PUZ does not foresee the withdrawal of groundwater or surface water from the site area, so there will be no effects on the hydrology of the area, nor will other activities dependent on this resource be affected in secondary. The potential impact is felt locally, generally limited to the perimeter of the turbine sites and working faces. In general, such works are carried out in stages, which will imply periodic displacement of the working fronts and of the areas of negative impact. The impact will be temporary (for the duration of the works). The construction works will not have a significant long-term impact on the quality of water bodies. No wastewater is generated during the operation phase of the wind farm. The maintenance and repair works will be carried out according to the maintenance and repair programs and plan, respectively, without affecting environmental factors. In the decommissioning phase of the investments foreseen by the PUZ, the estimated impact is similar to the construction phase. |  |
| Air | OM2. Maintaining air quality by reducing emissions from energy sector activities | -2 | -2 | -1 | -2 | -2 | -1 | 0 | -2 | 2 |  | 2 | -1 |  | -1 | -2 | -2 | -2 | -2 | -1 | -2 | In the construction phase, the sources of emissions are dust emissions from excavation, vehicular and construction material handling, as well as emissions of pollutants specific to fossil fuel combustion associated with construction machinery/equipment and transportation. Erosion caused by excavation works is another factor that may lead to air quality alteration in the implementation area. The level of emissions of dust and pollutants is dependent on the level of daily activity, the specifics of the works, varying substantially from day to day, from one phase of the construction process to another, and on weather conditions. Given the large number of turbines foreseen by the PUZ (117 wind turbines are planned to be built), even in the conditions of phased execution (road laying, earthworks, foundations, turbine assembly) it can be estimated that this environmental factor will be significantly negatively affected, but the potential impact is temporary, felt locally, generally limited to the perimeter of the sites and work fronts. In the operational phase, the wind turbines contribute to reducing both GHG emissions and pollutant emissions associated with electricity generation by replacing an amount of electricity equal to the capacity of the wind farm produced from fossil fuels, contributing to climate change mitigation and having a significant positive impact on air quality. In the decommissioning phase of the investments foreseen by the PUZ the estimated impact is similar to the construction phase. |  |
| Soil/subsoil | OM3. Limit soil pollution and land degradation. | -2 | -2 | -1 | -2 | -2 | -1 | 0 | -2 | 0 |  | 0 | -1 |  | -1 | -1 | -1 | -1 | -1 | 1 | -1 | During the execution phase, the main forms of impact on the soil are the temporary and permanent occupation of certain areas and the change of their use. In the case of alternative I the total area occupied by the project is 476.99 ha of which 473.090 ha is temporarily occupied and 3.8998 ha is permanently occupied by the footprint of the turbines and transformer stations. Of this area 5 ha will be taken out of agricultural use. The soil on the perimeters where the construction and assembly of the wind turbines, grid connections and transformer stations will be carried out will be affected by: § Loss of topsoil; §modification of soil structure; §modification of the infiltration regime of precipitation water, in particular in the perimeters of foundations, as an effect of the construction of concrete structures. Once the soil is stripped and deposited, a quantity of nutrients is taken out of the natural circuit, which will be reintegrated into the circuit as the deposited topsoil will be used for the ecological restoration of the territory. These changes to the soil are reversible in the case of temporarily occupied areas, and the soil can be returned to its original state after completion. A potential source of soil and subsoil pollution is the activity of vehicles and machinery in the working fronts. Particulate pollutants from emissions end up deposited on the ground and can be carried into the subsoil by stormwater infiltration. Soil and subsoil can also be polluted by accidental losses/leaks of hazardous substances (oils, fuels, lubricants) and/or wastewater, as well as by improper storage of raw materials, materials and waste. In the operation phase of the wind farm, the possible impact on the soil is the change of land use of the land currently used for other purposes, plus maintenance and repair works. In the decommissioning phase of the investments foreseen by the PUZ, the estimated impact is insignificant, with the specification that the land restoration works will restore the site to its original state, returning it to its current functionality (arable land). |  |
| Use of natural resources | Reduce the exploitation of exhaustible resources and facilitate the exploitation of renewable ones | -2 | -2 | 0 | -2 | -1 | -1 | -1 | -2 | 2 |  | 2 | -1 |  | -1 | -1 | -1 | -1 | -1 | -2 | -1 | During the construction of the 117 turbines and the infrastructure that serves them, the use of natural resources (sand, stone, earth) for construction is unavoidable, but the impact, although limited, is significant given the large number of turbines. In the operational phase, the wind farm promotes the use of renewable resources, having a positive impact. In addition, by realizing the wind farm, an amount of electricity corresponding to the capacity of the wind farm will replace the same amount of electricity produced from non-renewable sources (fossil fuels), thus reducing their exploitation. |  |
|  | OM5. Promoting the transition to a circular and resource-efficient economy | 0 | 0 | 1 | -1 | -1 | -1 | 1 | -1 | 2 |  | 2 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Reusing and recycling products will slow down the depletion of natural resources, reduce the impact on landscapes and natural habitats. The circular economy leads to reduced total annual greenhouse gas emissions. The wind turbines and transformer stations that will equip the wind farm will correspond to the principles of the circular economy, reducing the impact on resource consumption. |  |
| Waste management | OM6. Reducing the amounts of waste generated and disposed of by landfilling Eva (compliant landfills) | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 0 |  | 0 | -1 |  | -2 | -2 | -2 | -2 | -1 | 0 | -2 | At the execution stage it is difficult to estimate the amount of waste generated, this amount depends on the technologies adopted which determine the nature of the waste and the amount of waste. The collection and temporary storage of waste resulting from wind farm construction and erection activities will be carried out under controlled conditions, in specially designated areas, for disposal/recovery by authorized operators. Inert waste resulting during the execution period (e.g. earth from excavations, mixtures of earth and stones, etc.) will be disposed of at an inert waste landfill within the county of Constanța or reused as fill in the works foreseen by the PUZ (e.g. for new access roads as fill material, in foundations, etc.). Waste quantities at this stage are reduced. During the operational period the resulting waste comes from maintenance and servicing activities of wind turbines and transformer stations. During the decommissioning period the waste generated requires appropriate management to minimize the negative impact, the impact would be significant without it. In addition, for this alternative the amount of waste is significant given the large number of turbines. |  |
| Noise and vibration | OM7. Limiting noise pollution at sources in areas with noise sensitive receptors. Limitation of vibration levels | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | -1 |  | -1 | -1 |  | -1 | -1 | -1 | -1 | 0 | 0 | -1 | During the execution phase, the means of transportation and machinery used, as well as the activities at the work front are a source of noise and vibration, so that during this period there will be an increase in the noise level resulting from the sustained transportation activity and the operation of machinery involved in the works. For this alternative the noise is estimated to be higher than in the others taking into account the significant number of turbines which result in a proportionate number of plant/machinery/equipment. Thus, discomfort will be felt mainly due to the noise and vibrations produced by motor vehicles, the machinery used and the presence of workers, but given that the area is crossed by local roads, farm roads and on the neighboring land is carried out seasonally agricultural works with various machines, the noise will not create a significant impact, will be local and temporary. During operation, the noise produced by the wind turbine increases with wind speed and rotational speed. Modern turbine design incorporates nacelle insulation to prevent mechanical noise from being transmitted into the air. The nacelle is also insulated to prevent vibrations from moving parts (blades, hub, gearbox) from being transmitted to the tower and foundation. The aerodynamic noise of large turbines is quite dominant compared to mechanical noise and is dependent on the rotational speed of the blades (wind speed). Although, the capacity of the 3 MW turbines makes the noise produced by an individual turbine less, the large number of turbines envisaged by the PUZ increases the likelihood of higher cumulative noise. In the decommissioning phase, the impact of noise and vibration is similar to the construction phase. |  |
| Biodiversity | OM8. Conservation of natural habitats and species of flora and fauna of Community importance | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 |  | -1 | -1 |  | -1 | -1 | -1 | -1 | -1 | 1 | -1 | The proposed plan does not cross any Natura 2000 protected areas on the national territory, the distance to the nearest protected area is about 3.97 km, and on the Bulgarian territory, the nearest Natura 2000 protected area is about 410 m from the nearest wind turbine. The area studied by the PUZ is located on land used for agricultural purposes, but which may constitute possible favorable habitats (feeding/breeding/nesting) for species of community interest for which the sites have been designated. Confirmation of this hypothesis will be made by further field monitoring to determine exactly which species use the future park area. During the construction phase, pollution with suspended solids due to construction works may adversely affect vegetation by deposition on the foliar apparatus, causing partial or total closure of stomata and disruption of physiological and biochemical processes of plants. Other forms of impacts from the implementation phase include disruption of species activity and possible reduction in population numbers due to behavioral barriers for wildlife. The construction and decommissioning works may temporarily affect biodiversity, but the restoration works will lead to an improvement in the condition of the sensitive elements affected. In the operational phase, given the number of turbines proposed by the PUZ and the number and species of birds and chiroptera identified on the site, which are sensitive in terms of wind energy projects, the impact can be considered negative on them even if they are properly located. In the decommissioning phase, the impact on biodiversity is similar to that of the construction phase. |  |
|  | OM9. Maintain the Natura 2000 network of protected areas. | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -1 |  | -1 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | The proposed plan is not close to any protected area of the Natura 2000 Network on the national territory, the distance to the nearest protected area is about 3.97 km, but on the territory of Bulgaria, the nearest protected area of the Natura 2000 Network is about 390 m from the nearest turbine. |  |
| Population and human health | OM10. Maintaining the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |  | -1 | -1 |  | -1 | -1 | -1 | -1 | 0 | 0 | -1 | During the execution phase, the population in the area of implementation of the PUZ may be affected by increased traffic on local roads and access roads of the locality, by the emissions of dust and pollutants generated by the works of the plan, by the noise and vibrations caused by machinery and equipment involved in the works. Their impact is local, temporary and low in significance, given that the area of implementation of the PUZ is in the outskirts of the locality, that it is a rural area, not crowded with construction and with low traffic. During the operational phase of the wind farm, the impact on the population and human health is the noise produced by the turbines in operation, the shading effect and the change in landscape. The capacity of 3 MW turbines means that the noise produced by an individual turbine is lower, but the large number of turbines increases the likelihood of higher cumulative noise that would have negative effects on the population. Given the large number of turbines proposed by the project (117) which generates a higher turbine density within the Cerchezu TAU boundary, a negative impact on the resident population can be estimated in terms of overshadowing effect. From the point of view of the visual impact on the population, it differs from one person to another due to the difference in perception. However, the large number of turbines significantly changes the local landscape. During the decommissioning phase, the impact on the population in the project implementation area is similar to that during the execution phase. |  |
|  | OM11. Improving living standards and social conditions of the population | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |  | 2 | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | In the implementation/ decommissioning phase, the creation of temporary jobs can have a positive impact on the standard of living of the local population. In the operation phase, the project contributes to improved living standards through the provision of a clean and stable energy source, which can reduce energy costs in the long term, and through contributions to the local budget related to surface/lease/use rights. |  |
| Landscape | OM12. Ensuring the protection of the natural landscape and reducing impacts on the natural landscape | -1 | -1 | 0 | -1 | -1 | -1 | 0 | -1 | 0 |  | 0 | -1 |  | 2 | -1 | 2 | 2 | -1 | 1 | 2 | The main landscape and visual impact of the implementation of the plan is the modification of the rural landscape of the area characterized only by agricultural land use. The construction of the wind turbines and associated infrastructure will introduce built features that are visible, some of which will only be visible from the immediate vicinity, such as landscaped roads and transformer station enclosures, and others, such as the silhouettes of the turbine poles and rotors, visible at a larger but still local scale. Wind turbines can have a long-term visual impact on the landscape, but this impact is relatively minor compared to other forms of energy generation. Decommissioning and restoring the land to its original state helps to restore the natural landscape. |  |
| Climate change | OM13. Reducing GHG emissions from the energy sector | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 2 |  | 2 | -1 |  | -1 | -1 | -1 | -1 | -1 | 0 | -1 | During the execution phase of the PUZ, GHG emissions are generated that could contribute to the global warming phenomenon, but given the small quantities, the influence is insignificant. In the operational phase, the wind farm contributes significantly to the reduction of GHG emissions by replacing an equivalent amount of electricity produced from fossil fuels, contributing to the decarbonization of the energy sector. In the decommissioning phase, the impact is similar to the construction phase. |  |
|  | OM14. Minimizing risks to climate change (adaptation) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |  | 2 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | It is considered that the PUZ proposal will not increase the vulnerability of the area to climate change. Works will be carried out in accordance with specific design regulations and site surveys. The wind farm supports climate change adaptation of the energy sector by producing renewable energy, reducing dependence on fossil fuels. |  |
| Cultural heritage | OM15. Protection and conservation of cultural heritage | -1 | -1 | 0 | -1 | 0 | 0 | 0 | -1 | 0 |  | 0 | 0 |  | 0 | -1 | -1 | 0 | -1 | 0 | -1 | In the execution and decommissioning phase of the works foreseen by the PUZ, the impact on the cultural heritage sites and those in the Archaeological Repertory is estimated as insignificant negative, given that the site of the turbines is located in the extravilan localities in the area of implementation of the PUZ, and the nearest site is the Tumulul de la Căscioarele Movila Bacu at a distance of about 226 m from the 2 turbines foreseen by the PUZ. No effects on cultural heritage sites are expected during the operation phase of the wind farms. |  |

Table  6-8 Assessment of the effects of Alternative II PUZ initiated in 2022 with 48 wind turbines without mitigation measures

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Implementation period |  |  |  |  |  |  |  |  | Operating period |  |  | Decommissioning period |  |  |  |  |  |  |  | Justification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Construction of access roads | Building technological platforms and foundations | Installing wind power plants | Making electrical connections and laying underground cables between the wind generator sets | Building transformer substations | Site organization | Land reclamation work at the end of construction | Final evaluation | Maintenance and repairs |  | Final evaluation | Site organization |  | Dismantling wind farms | Decommissioning of technological platforms and foundations | Dismantling electrical connections and removing underground cables | Decommissioning of transformer stations | Decommissioning of logging roads | Land restoration work on completion of decommissioning | Final evaluation |  |
| Water | OM1. Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | -1 | -1 | 0 | -2 | -1 | -1 | -1 | -1 | 0 |  | 0 | -1 |  | 0 | -1 | -2 | -1 | -1 | 1 | -1 | In the construction phase, the present PUZ foresees the undercrossing of the Cerchez (2 times) and Măgura (2 times) watercourses, which will be carried out by horizontal drilling at a depth of 1 m between the upper generator of the protection pipeline and the watercourse embankment. On completion, the affected areas will be restored to their original condition. In addition to these works, there may be leaks of used construction materials or fuels/lubricants from the machines involved in the work to create the site organization, access roads, technological platforms, electrical connections and cable laying, turbine erection and land reclamation at the end of the works. Erosion caused by the excavation works is another factor that may alter the quality of surface water bodies in the implementation area by increasing turbidity. The PUZ does not foresee the abstraction of ground or surface source water from the site area, so there will be no effect on the hydrology of the area, nor will other activities dependent on this resource be affected secondarily. The potential impact is felt locally, generally confined to the perimeter of the sites and working frontages. In general, such works are carried out in phases, which will involve periodic displacement of the working fronts and areas of negative impact. The impact will be temporary (for the duration of the works). The execution works of the PUZ will not have a significant long-term impact on water quality. No wastewater is generated during the operation phase of the wind farm. The maintenance and repair works will be carried out according to the programs, respectively the maintenance and repair plan, without affecting environmental factors. In the decommissioning phase of the investments foreseen by the PUZ the estimated impact is similar to the construction phase. |
| Air | OM2. Maintaining air quality by reducing emissions from energy sector activities | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 2 |  | 2 | -1 |  | -1 | -1 | -1 | -1 | -1 | 1 | -1 | In the construction phase, the sources of emissions are dust emissions from excavation, vehicular and construction material handling, as well as emissions of pollutants specific to fossil fuel combustion associated with construction machinery/equipment and transportation. Erosion caused by excavation works is another factor that may lead to air quality alteration in the implementation area. The level of emissions of dust and pollutants is dependent on the level of daily activity, the specifics of the works, varying substantially from day to day, from one phase of the construction process to another, and on weather conditions. Taking into account that the works are being carried out in stages (road laying, earthworks, execution of foundations, turbine erection), it can be estimated that this environmental factor will not be significantly negatively affected, the potential impact being temporary, felt locally, generally limited to the perimeter of the sites and work faces. In the operational phase, the wind turbines contribute to GHG emission reduction by replacing an amount of electricity equal to the capacity of the wind farm produced from fossil fuels, contributing to climate change mitigation and having a significant positive impact on air quality. In the decommissioning phase of the investments foreseen by the PUZ, the estimated impact is similar to the construction phase, with the specification that the land reclamation works at the end of the phase will contribute positively to air quality. |
| Soil/subsoil | OM3. Limit soil pollution and land degradation. | -2 | -2 | 0 | -1 | -1 | -1 | 1 | -2 | 0 |  | 0 | -1 |  | -1 | -1 | -1 | -1 | -1 | 1 | -1 | At the execution stage, the main forms of impact on the soil are temporary and permanent occupation of areas and changes of use. For Alternative II the total area occupied by the project is 3,027.77 ha of which 2,718.43 ha is temporarily occupied and 309.34 ha is permanently occupied by the footprint of the turbines and transformer stations. Of this 309.34 ha will be taken out of agricultural use. The soil on the perimeters where the construction and erection of the wind turbines, grid connections and transformer stations will be carried out will be affected by: § loss of topsoil; § modification of soil structure; § modification of the infiltration regime of rainwater, in particular in the perimeters of the foundations, as a result of the construction of concrete structures. Once the soil is stripped and deposited, a quantity of nutrients is removed from the natural cycle, which will be reintegrated into the cycle as the deposited topsoil is used to restore the ecological balance of the land. These changes to the soil are reversible in the case of temporarily occupied areas, and the soil can be returned to its original state after completion. A potential source of soil and subsoil pollution is the activity of vehicles and machinery at the working faces. Particulate pollutants from emissions end up deposited on the ground and can be carried into the subsoil by stormwater infiltration. Soil and subsoil can also be polluted by accidental spills/leaks of hazardous substances (oils, fuels, lubricants) and/or wastewater, as well as by improper storage of raw materials, materials and waste. In the operation phase of the wind farm, the possible impact on the soil is the change of land use of the land currently used for other purposes, plus maintenance and repair works. In the decommissioning phase of the investments foreseen by the PUZ, the estimated impact is insignificant, with the specification that the land restoration works will restore the site to its original state, returning it to its current functionality (arable land). |
| Use of natural resources | Reduce the exploitation of exhaustible resources and facilitate the exploitation of renewable ones | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 2 |  | 2 | -1 |  | 0 | -1 | -1 | -2 | -1 | -1 | -1 | During the construction period, the use of natural resources (sand, stone, earth) for the construction of the wind farm is unavoidable, but the impact is limited. In the operation phase, the wind farm promotes the use of renewable resources and has a positive impact. In addition, by realizing the wind farm, an amount of electricity corresponding to the capacity of the wind farm will replace the same amount of electricity produced from non-renewable sources (fossil fuels), thus reducing their exploitation. |
|  | OM5. Promoting the transition to a circular and resource-efficient economy | 0 | 0 | 1 | -1 | -1 | -1 | 1 | -1 | 2 |  | 2 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Reusing and recycling products will slow down the depletion of natural resources, reduce the impact on landscapes and natural habitats. The circular economy leads to reduced total annual greenhouse gas emissions. The wind turbines and transformer stations that will equip the wind farm will correspond to the principles of the circular economy, reducing the impact on resource consumption. |
| Waste management | OM6. Reducing the amounts of waste generated and disposed of by landfilling Eva (compliant landfills) | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 0 |  | 0 | -1 |  | -2 | -2 | -2 | -2 | -1 | 0 | -2 | At the execution stage it is difficult to estimate the amount of waste generated, this amount depends on the technologies adopted which determine the nature of the waste and the amount of waste. The collection and temporary storage of waste resulting from the construction and erection activities of the wind farm will be carried out in a controlled manner, in specially designated areas, for disposal/ recovery by authorized operators. Inert waste resulting during the execution period (e.g. earth from excavations, mixtures of earth and stones, etc.) will be disposed of at an inert waste landfill within the county of Constanța or reused as fill in the works foreseen by the PUZ (e.g. for new access roads as fill material, in foundations, etc.). Waste quantities at this stage are reduced. During the operation period the resulting waste comes from maintenance activities (turbine oil) and wind turbine maintenance. In the decommissioning period the waste generated requires appropriate management to minimize the negative impact, the impact being significant in the absence of such management. |
| Noise and vibration | OM7. Limiting noise pollution at sources in areas with noise sensitive receptors. Limitation of vibration levels | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 0 |  | 0 | -1 |  | -1 | -1 | -1 | -1 | 0 | 0 | -1 | During the execution phase, the means of transportation and machinery used, as well as the activities in the work front are a source of noise and vibration, so that during this period there will be an increase in the noise level resulting from the sustained activity of transportation and the operation of machinery involved in the works. Thus, there will be discomfort due mainly to the noise and vibration produced by vehicles, the machinery used and the presence of workers, but given that the area is crossed by local roads, farm roads and on the land in the vicinity there is seasonal agricultural work with various machines, the noise will not create a significant impact, will be local and temporary. During operation, the noise produced by the wind turbine increases with wind speed and rotation speed. Modern turbine design incorporates nacelle insulation to prevent mechanical noise from being transmitted into the air. The nacelle is also insulated to prevent vibrations from moving parts (blades, hub, gearbox) from being transmitted into the tower and foundation. The aerodynamic noise of large turbines is quite dominant compared to mechanical noise and is dependent on the rotational speed of the blades (wind speed). In general the noise level of a turbine varies between 92 - 107.7 dB. For the 6.6 MW turbine at 95% load the maximum noise level is 106.6 dB (level according to the noise study). From the noise modeling, in the receptor area (settlements in the PUZ implementation area) the noise level is less than 40 dB. In the decommissioning phase, the noise and vibration impact is similar to the construction phase. |
| Biodiversity | OM8. Conservation of natural habitats and species of flora and fauna of Community importance | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | 0 |  | 0 | -1 |  | -1 | -1 | -1 | -1 | -1 | 1 | -1 | The proposed plan does not cross any Natura 2000 protected areas on the national territory, the distance to the nearest protected area is about 3.97 km, and on the Bulgarian territory, the nearest Natura 2000 protected area is about 410 m from the nearest wind turbine. The area studied by the PUZ is located on land used for agricultural purposes, but which may constitute possible favorable habitats (feeding/breeding/nesting) for species of community interest for which the sites have been designated. Confirmation of this hypothesis will be made by further field monitoring to determine exactly which species use the future park area. During the construction phase, pollution with suspended solids due to construction works may adversely affect vegetation by deposition on the foliar apparatus, causing partial or total closure of stomata and disruption of physiological and biochemical processes of plants. Other forms of impacts from the implementation phase include disruption of species activity and possible reduction in population numbers due to behavioral barriers for wildlife. The construction and decommissioning works may temporarily affect biodiversity, but the restoration works will lead to an improvement in the condition of the sensitive elements affected. In the operational phase, given the number of turbines proposed by the PUZ and the number and species of birds and chiroptera identified on the site, which are sensitive in terms of wind energy projects, the impact can be considered negative on them even if they are properly located. In the decommissioning phase, the impact on biodiversity is similar to that of the construction phase. |
|  | OM9. Maintain the Natura 2000 network of protected areas. | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -1 |  | -1 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | The proposed plan is not close to any protected area of the Natura 2000 Network on the national territory, the distance to the nearest protected area is about 2.79 km, but on the Bulgarian territory, the nearest protected area of the Natura 2000 Network is about 410 m from the boundary of the PUZ. |
| Population and human health | OM10. Maintaining the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 0 |  | 0 | -1 |  | -1 | -1 | -1 | -1 | 0 | 0 | -1 | During the execution phase, the population in the area of implementation of the PUZ may be affected by increased traffic on local roads and access roads of the locality, by the emissions of dust and pollutants generated by the works of the plan, by the noise and vibrations caused by machinery and equipment involved in the works. Their impact is local, temporary and low in significance, given that the area of implementation of the PUZ is in the outskirts of the locality, that it is a rural area, not crowded with construction and with low traffic. During the operation phase of the wind farm, the impact on the population and human health is constituted by the noise produced by the turbines in operation, the shading effect and the modification of the landscape. From the noise modeling carried out for the PUZ, the noise level in the receptor area (settlements in the area of implementation of the PUZ) is less than 40 dB, in accordance with the limits imposed by the legislation in force for protected areas. Regarding the effect of shading, according to the modeling carried out for the proposed PUZ in the area of the settlements in the PUZ implementation area, shading has predominant values between 0.1 and 8 h/year, the regulatory limit in the USA is 30 hours. From the point of view of the visual impact on the population it differs from one person to another by the difference in perception. In the decommissioning phase, the impact felt by the population in the project implementation area is similar to that in the execution phase. |
|  | OM11. Improving living standards and social conditions of the population | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |  | 2 | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | In the implementation/ decommissioning phase, the creation of temporary jobs can have a positive impact on the standard of living of the local population. In the operation phase, the project contributes to improved living standards through the provision of a clean and stable energy source, which can reduce energy costs in the long term, and through contributions to the local budget related to surface/lease/use rights. |
| Landscape | OM12. Ensuring the protection of the natural landscape and reducing impacts on the natural landscape | -1 | -1 | 0 | -1 | -1 | -1 | 0 | -1 | 0 |  | 0 | -1 |  | 2 | -1 | -1 | 2 | -1 | 1 | 1 | The main landscape and visual impact of the implementation of the plan is the modification of the rural landscape of the area characterized only by agricultural land use. The construction of the wind turbines and associated infrastructure will introduce built features that are visible, some of which will only be visible from the immediate vicinity, such as landscaped roads and transformer station enclosures, and others, such as the silhouettes of the turbine poles and rotors, visible at a larger but still local scale. Wind turbines can have a long-term visual impact on the landscape, but this impact is relatively minor compared to other forms of energy generation. Decommissioning and restoring the land to its original state helps to restore the natural landscape. |
| Climate change | OM13. Reducing GHG emissions from the energy sector | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 2 |  | 2 | -1 |  | -1 | -1 | -1 | -1 | -1 | 0 | -1 | During the execution phase of the PUZ, GHG emissions are generated that could contribute to the global warming phenomenon, but given the small quantities, the influence is insignificant. In the operation phase, the wind farm contributes significantly to the reduction of GHG emissions by replacing an equivalent amount of electricity produced from fossil fuels, contributing to the decarbonization of the energy sector. In the decommissioning phase, the impact is similar to the construction phase. |
|  | OM14. Minimizing risks to climate change (adaptation) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |  | 2 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | It is considered that the PUZ proposal will not increase the vulnerability of the area to climate change. Works will be carried out in accordance with specific design regulations and site surveys. The wind farm supports climate change adaptation of the energy sector by producing renewable energy, reducing dependence on fossil fuels. |
| Cultural heritage | OM15. Protection and conservation of cultural heritage | -1 | 0 | 0 | -1 | 0 | 0 | 0 | -1 | 0 |  | 0 | 0 |  | 0 | 0 | -1 | 0 | -1 | 0 | -1 | In the execution and decommissioning phase of the works foreseen by the PUZ, the impact on the cultural heritage sites and those in the Archaeological Repertory is estimated as insignificant negative, given that the site of the turbines is located in the outskirts of the localities in the area of implementation of the PUZ, and the nearest site is the Tumulul de la Căscioarele Movila Bacu, at a distance of about 636 m from T27 and about 1 km from T30. The Legionary Castrum is about 742.26 m from turbine T27. No effects on cultural heritage sites are expected during the operation phase of the wind farms. |

Table 6-9 Assessment of the effects of Alternative III PUZ initiated in 2022 with 46 wind turbines in the absence of mitigation measures

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Implementation period |  |  |  |  |  |  |  | Operating period |  | Decommissioning period |  |  |  |  |  |  |  |  | Justification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Construction of access roads | Building technological platforms and foundations | Installing wind power plants | Making electrical connections and laying underground cables between the wind generator sets | Building transformer substations | Site organization | Land reclamation work at the end of construction | Final evaluation | Maintenance and repairs | Final evaluation | Site organization | Dismantling wind farms | Decommissioning of technological platforms and foundations | Dismantling electrical connections and removing underground cables | Decommissioning of transformer stations | Decommissioning of logging roads | Land restoration work on completion of decommissioning | Final evaluation |  |  |
| Water | OM1. Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | -1 | -1 | 0 | -2 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | 0 | -1 | -2 | -1 | -1 | 1 | -1 | In the construction phase, the present PUZ foresees the undercrossing of the Cerchez (2 times) and Măgura (2 times) watercourses, which will be carried out by horizontal drilling at a depth of 1 m between the upper generator of the protection pipeline and the watercourse embankment. On completion, the affected areas will be restored to their original condition. In addition to these works, there may be leaks of used construction materials or fuels/lubricants from the machines involved in the work to create the site organization, access roads, technological platforms, electrical connections and cable laying, turbine erection and land reclamation at the end of the works. Erosion caused by the excavation works is another factor that may alter the quality of surface water bodies in the implementation area by increasing turbidity. The PUZ does not foresee the abstraction of ground or surface source water from the site area, so there will be no effect on the hydrology of the area, nor will other activities dependent on this resource be affected secondarily. The potential impact is felt locally, generally confined to the perimeter of the sites and working frontages. In general, such works are carried out in phases, which will involve periodic displacement of the working fronts and areas of negative impact. The impact will be temporary (for the duration of the works). The execution works of the PUZ will not have a significant long-term impact on water quality. No wastewater is generated during the operation phase of the wind farm. The maintenance and repair works will be carried out according to the programs, respectively the maintenance and repair plan, without affecting environmental factors. In the decommissioning phase of the investments foreseen by the PUZ the estimated impact is similar to the construction phase. |  |
| Air | OM2. Maintaining air quality by reducing emissions from energy sector activities | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 2 | 2 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | In the construction phase, the sources of emissions are dust emissions from excavation, vehicular and construction material handling, as well as emissions of pollutants specific to fossil fuel combustion associated with construction machinery/equipment and transportation. Erosion caused by excavation works is another factor that may lead to air quality alteration in the implementation area. The level of emissions of dust and pollutants is dependent on the level of daily activity, the specifics of the works, varying substantially from day to day, from one phase of the construction process to another, and on weather conditions. Taking into account that the works are being carried out in stages (road laying, earthworks, execution of foundations, turbine erection), it can be estimated that this environmental factor will not be significantly negatively affected, the potential impact being temporary, felt locally, generally limited to the perimeter of the sites and work faces. In the operational phase, the wind turbines contribute to GHG emission reduction by replacing an amount of electricity equal to the capacity of the wind farm produced from fossil fuels, contributing to climate change mitigation and having a significant positive impact on air quality. In the decommissioning phase of the investments foreseen by the PUZ, the estimated impact is similar to the construction phase, with the specification that the land reclamation works at the end of the phase will contribute positively to air quality. |  |
| Soil/subsoil | OM3. Limit soil pollution and land degradation. | -2 | -2 | 0 | -1 | -1 | -1 | 1 | -2 | 0 | 0 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | During the execution phase, the main forms of impact on the soil are the temporary and permanent occupation of certain areas and the change of their use. In the case of alternative III the total area occupied by the project is 3,027.77 ha of which 2,735.25 ha is temporarily occupied and 292.52 ha is permanently occupied by the footprint of the turbines and transformer stations and the roads and roads for development and operation. Of this area 100 ha will be taken out of agricultural use. The soil in the perimeters where the construction and assembly of wind turbines, grid connections and transformer stations will be carried out will be affected by: § loss of topsoil; § modification of soil structure; § modification of the infiltration regime of rainwater, in particular in the perimeters of foundations, as a result of the construction of concrete structures. Once the soil is stripped and deposited, a quantity of nutrients is removed from the natural cycle, which will be reintegrated into the cycle as the deposited topsoil is used to restore the ecological balance of the land. These changes to the soil are reversible in the case of temporarily occupied areas, and the soil can be returned to its original state after completion. A potential source of soil and subsoil pollution is the activity of vehicles and machinery at the working faces. Particulate pollutants from emissions end up deposited on the ground and can be carried into the subsoil by stormwater infiltration. Soil and subsoil can also be polluted by accidental spills/leaks of hazardous substances (oils, fuels, lubricants) and/or wastewater, as well as by improper storage of raw materials, materials and waste. In the operation phase of the wind farm, the possible impact on the soil is the change of land use of the land currently used for other purposes, plus maintenance and repair works. In the decommissioning phase of the investments foreseen by the PUZ, the estimated impact is insignificant, with the specification that the land restoration works will restore the site to its original state, returning it to its current functionality (arable land). |  |
| Use of natural resources | Reduce the exploitation of exhaustible resources and facilitate the exploitation of renewable ones | -1 | -1 | 1 | -1 | -1 | -1 | 1 | -1 | 2 | 2 | -1 | 0 | -1 | -1 | -2 | -1 | -1 | -1 | During the construction period, the use of natural resources (sand, stone, earth) for the construction of the wind farm is unavoidable, but the impact is limited. In the operational phase, the wind farm promotes the use of renewable resources and has a positive impact. In addition, by realizing the wind farm, an amount of electricity corresponding to the capacity of the wind farm will replace the same amount of electricity produced from non-renewable sources (fossil fuels), thus reducing their exploitation. |  |
|  | OM5. Promoting the transition to a circular and resource-efficient economy | 0 | 0 | 1 | -1 | -1 | -1 | 1 | -1 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Reusing and recycling products will slow down the depletion of natural resources, reduce the impact on landscapes and natural habitats. The circular economy leads to reduced total annual greenhouse gas emissions. The wind turbines and transformer stations that will equip the wind farm will correspond to the principles of the circular economy, reducing the impact on resource consumption. |  |
| Waste management | OM6. Reducing the amounts of waste generated and disposed of by landfilling Eva (compliant landfills) | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 0 | 0 | -1 | -2 | -2 | -2 | -2 | -1 | 0 | -2 | At the execution stage it is difficult to estimate the amount of waste generated, this amount depends on the technologies adopted which determine the nature of the waste and the amount of waste. The collection and temporary storage of waste resulting from the construction and erection activities of the wind farm will be carried out in a controlled manner, in specially designated areas, for disposal/ recovery by authorized operators. Inert waste resulting during the execution period (e.g. earth from excavations, mixtures of earth and stones, etc.) will be disposed of at an inert waste landfill within the county of Constanța or reused as fill in the works foreseen by the PUZ (e.g. for new access roads as fill material, in foundations, etc.). Waste quantities at this stage are reduced. During the operation period the resulting waste comes from maintenance activities (turbine oil) and wind turbine maintenance. In the decommissioning period the waste generated requires appropriate management to minimize the negative impact, the impact being significant in the absence of such management. |  |
| Noise and vibration | OM7. Limiting noise pollution at sources in areas with noise sensitive receptors. Limitation of vibration levels | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 0 | 0 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | During the execution phase, the means of transportation and machinery used, as well as the activities in the work front are a source of noise and vibration, so that during this period there will be an increase in the noise level resulting from the sustained activity of transportation and the operation of machinery involved in the works. Thus, there will be discomfort due mainly to the noise and vibration produced by vehicles, the machinery used and the presence of workers, but given that the area is crossed by local roads, farm roads and on the neighboring land where agricultural work is carried out seasonally with various machines, the noise will not create a significant impact, will be local and temporary. During operation, the noise produced by the wind turbine increases with wind speed and rotation speed. Modern turbine design incorporates nacelle insulation to prevent mechanical noise from being transmitted into the air. The nacelle is also insulated to prevent vibrations from moving parts (blades, hub, gearbox) from being transmitted into the tower and foundation. The aerodynamic noise of large turbines is quite dominant compared to mechanical noise and is dependent on the rotational speed of the blades (wind speed). In general the noise level of a turbine varies between 92 - 107.7 dB. For the 6.6 MW turbine at 95% load the maximum noise level is 106.6 dB (level according to the noise study). From the noise modeling, in the receptor area (settlements in the PUZ implementation area) the noise level is less than 40 dB. In the decommissioning phase, the noise and vibration impact is similar to the construction phase. |  |
| Biodiversity | OM8. Conservation of natural habitats and species of flora and fauna of Community importance | -1 | -1 | 0 | -1 | -1 | -1 | 1 | -1 | 0 | 0 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | -1 | The proposed plan does not cross any Natura 2000 protected areas on the national territory, the distance to the nearest protected area is about 3.97 km, and on the Bulgarian territory, the nearest Natura 2000 protected area is about 410 m from the nearest wind turbine. The area studied by the PUZ is located on land used for agricultural purposes, but which may constitute possible favorable habitats (feeding/breeding/nesting) for species of community interest for which the sites have been designated. Confirmation of this hypothesis will be made by further field monitoring to determine exactly which species use the future park area. During the construction phase, pollution with suspended solids due to construction works may adversely affect vegetation by deposition on the foliar apparatus, causing partial or total closure of stomata and disruption of physiological and biochemical processes of plants. Other forms of impacts from the implementation phase include disruption of species activity and possible reduction in population numbers due to behavioral barriers for wildlife. The construction and decommissioning works may temporarily affect biodiversity, but the restoration works will lead to an improvement in the condition of the sensitive elements affected. In the operational phase, given the number of turbines proposed by the PUZ and the number and species of birds and chiroptera identified on the site, which are sensitive in terms of wind energy projects, the impact can be considered negative on them even if they are properly located. In the decommissioning phase, the impact on biodiversity is similar to that of the construction phase. |  |
|  | OM9. Maintain the Natura 2000 network of protected areas. | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | The proposed plan is not close to any protected area of the Natura 2000 Network on the national territory, the distance to the nearest protected area is about 2.79 km, but on the Bulgarian territory, the nearest protected area of the Natura 2000 Network is about 410 m from the boundary of the PUZ. |  |
| Population and human health | OM10. Maintain the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | During the execution phase, the population in the area of implementation of the PUZ may be affected by increased traffic on local roads and access roads of the locality, by the dust and pollutants generated by the works of the plan, by the noise and vibrations caused by machinery and equipment involved in the works. Their impact is local, temporary and low in significance, given that the area of implementation of the PUZ is in the outskirts of the locality, that it is a rural area, not congested with construction and with low traffic. During the operation phase of the wind farm, the impact on the population and human health is constituted by the noise produced by the turbines in operation, the shading effect and the modification of the landscape. From the noise modeling carried out for the PUZ, the noise level in the receptor area (settlements in the area of implementation of the PUZ) is less than 40 dB, in accordance with the limits imposed by the legislation in force for protected areas. Regarding the effect of shading, according to the modeling carried out for the proposed PUZ in the area of the settlements in the PUZ implementation area, shading has predominant values between 0.1 and 8 h/year, the regulatory limit in the USA is 30 hours. From the point of view of the visual impact on the population it differs from one person to another by the difference in perception. In the decommissioning phase, the impact felt by the population in the project implementation area is similar to that in the execution phase. |  |
|  | OM11. Improving living standards and social conditions of the population | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | In the implementation/ decommissioning phase, the creation of temporary jobs can have a positive impact on the standard of living of the local population. In the operation phase, the project contributes to improved living standards through the provision of a clean and stable energy source, which can reduce energy costs in the long term, and through contributions to the local budget related to surface/lease/use rights. |  |
| Landscape | OM12. Ensuring the protection of the natural landscape and reducing impacts on the natural landscape | -1 | -1 | 0 | -1 | -1 | -1 | 0 | -1 | 0 | 0 | -1 | 2 | -1 | -1 | 2 | -1 | 1 | 1 | The main landscape and visual impact of the implementation of the plan is the modification of the rural landscape of the area characterized only by agricultural land use. The construction of the wind turbines and associated infrastructure will introduce built features that are visible, some of which will only be visible from the immediate vicinity, such as landscaped roads and transformer station enclosures, and others, such as the silhouettes of the turbine poles and rotors, visible at a larger but still local scale. Wind turbines can have a long-term visual impact on the landscape, but this impact is relatively minor compared to other forms of energy generation. Decommissioning and restoring the land to its original state helps to restore the natural landscape. |  |
| Climate change | OM13. Reducing GHG emissions from the energy sector | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | 2 | 2 | -1 | -1 | -1 | -1 | -1 | -1 | 0 | -1 | During the execution phase of the PUZ, GHG emissions are generated that could contribute to the global warming phenomenon, but given the small quantities, the influence is insignificant. In the operation phase, the wind farm contributes significantly to the reduction of GHG emissions by replacing an equivalent amount of electricity produced from fossil fuels, contributing to the decarbonization of the energy sector. In the decommissioning phase, the impact is similar to the construction phase. |  |
|  | OM14. Minimizing risks to climate change (adaptation) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | It is considered that the PUZ proposal will not increase the vulnerability of the area to climate change. Works will be carried out in accordance with specific design regulations and site surveys. The wind farm supports climate change adaptation of the energy sector by producing renewable energy, reducing dependence on fossil fuels. |  |
| Cultural heritage | OM15. Protection and conservation of cultural heritage | -1 | 0 | 0 | -1 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | -1 | 0 | -1 | In the execution and decommissioning phase of the works foreseen by the PUZ, the impact on the cultural heritage sites and those in the Archaeological Repertory is estimated as insignificant negative, given that the site of the turbines is located in the outskirts of the localities in the area of implementation of the PUZ, and the nearest site is the Tumulul de la Căscioarele Movila Bacu, at a distance of about 636 m from T27 and about 1 km from T30. The Legionary Castrum is about 742.26 m from turbine T27. No effects on cultural heritage sites are expected during the operation phase of the wind farms. |  |

  - Assessment of the cumulative impact of the proposed PUZ with other existing or proposed PPs

Regarding the relation with other plans and programs both on the territory of Romania and Bulgaria, according to the available information, in the vicinity of the proposed wind farm there are the following existing/proposed wind farms (see Figure :

- Existing:

  - "WIND ENERGY PARK 32 WIND POWER PLANTS, TOTAL POWER 80 MW, TRANSFORMATION STATION, ELECTRIC CONNECTION NETWORKS, CONSTRUCTION AND MODERNIZATION OF COMMUNICATION AND ACCESS ROADS", located in the municipalities of Chirnogeni and Independența. The distance between the proposed wind farm and the existing wind farm is approximately 1,5 km (distance between the proposed T24 turbine and the existing T1).

  - Wind farm on the territory of Bulgaria, located 5.6 km away from the proposed project (distance between the proposed T3 turbine and the existing T6).

- Proposed:

  - Wind energy wind farm 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântâna Mare, Independența commune, jud. Constanța,

  - Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - Independența and Cerchezu communes, jud. Constanța,

  - Comana Wind Farm - Pecineaga - MV underground electric cables and fiber optic network,

  - Construction of wind power plant including: wind turbines, access roads and assembly/maintenance platforms, electrical transformer station (own), medium and high voltage electrical network, telecommunication network, Independența commune, jud. Constanța,

  - Demolition of existing buildings and construction of photovoltaic power plant 3,212 MW, connection to SEN, PC and PTAB location, internal road and access, fencing and site organization located in Independența,

  - Construction of underground fiber optic network for electronic communication services in Constanta County, Independența commune, Dumbrăveni,

  - Construction of underground fiber optic communications network for electronic services, Negru Voda,

  - Electricity supply to water household, Furnica village, Dumbrăveni commune, jud. Constanța,

  - General Urban Plan of Independența commune,

  - Establishment of a smart gas distribution network in Independența commune, Constanta county,

  - Construction of a bridge in Movila Verde, Church Street, Movila Verde, Independența commune,

  - Asphalting streets Comana, Tătaru and Pelinu, stage I, Comana commune, Constanta County

  - Construction of bridges, culverts and paving of rainwater drainage channels in Credința village, Chirnogeni commune,

  - Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county.

Figure 6 - 1 Map showing the proposed plan in relation to other plans and programs

For the already existing projects (Chirnogeni-Independence Wind Farm, Bulgaria Wind Farm) the cumulative impact of the operation of the wind farms with the construction period of the wind farm proposed by the PUZ and the cumulative impact of the operation of the existing wind farms with the operation period of the wind farm proposed by the PUZ were estimated.

Taking into account the significant distance of the wind farm in Bulgaria from the proposed wind farm, no cumulative impacts are estimated in any of the development phases of the PUZ (construction, operation).

Taking into account that in operation wind farms have no impact on environmental aspects: air, water, soil/soil, resource utilization, waste management, climate change and cultural heritage, the cumulative impact of the operation of the Chirnogeni-Independența wind farm with the construction phase for the wind farm proposed by the PUZ, is the same as the one estimated in the construction phase (negative insignificant, except the impact on soil/soil which is negative significant).

In terms of noise, during the construction period of the park proposed by the PUZ, the noise generated by the works will be added to the noise of the park in operation, but given the topography of the area, the fact that the works will be carried out in stages and the absence of other sources of noise, the cumulative impact is not expected to be greater than that estimated in the construction phase, i.e. insignificant negative.

Regarding the landscape, the construction of the proposed wind farm will not bring a significant change to the existing landscape, it will harmoniously integrate into the existing environment, given the presence in the area of the Chirnogeni-Independence wind farm already operating since 2014.

With regard to biodiversity, from the monitoring reports on the operation of wind turbines in the Chirnogeni-Independența park it was concluded that it does not constitute a significant disturbance factor for the local fauna and especially for bird species, as no clear avoidance behavior of the areas where they are located was observed, the structure and composition of avifauna being similar to similar periods in the years prior to the implementation phase of the project. In addition, no major changes in the flora were observed, the structure and composition of the vegetation developed on the site and in its vicinity being similar to previous monitoring periods.

Therefore, the cumulative impact of the operation of the Chirnogeni-Independența wind farm with the construction phase of the wind farm proposed by the PUZ is estimated to be insignificant.

As regards the cumulative impact of the operation of the Chirnogeni-Independența wind farm with the period of operation of the wind farm proposed by the PUZ it is estimated as follows:

- Noise impact: Wind turbines generate both mechanical and aerodynamic noise. The accumulation of several wind farms may increase the noise level in the area, affecting the quality of life of nearby residents, but considering the noise study carried out for the wind farm proposed by the PUZ, its position in relation to the Chirnogeni-Independence wind farm about 1.5 km away and the open topography of the area, the cumulative noise effect associated with the simultaneous operation of wind farms can be estimated to be insignificant negative.

- Landscape Impact: Wind turbines are structures visible from long distances and can significantly change the local landscape. The accumulation of several wind farms may lead to a crowded visual perception and impact on the natural and cultural landscape, but given the small number of wind turbines proposed in the PUZ and the fact that the Chirnogeni-Independence wind farm has been operating since 2014 and is integrated into the landscape, the cumulative impact of the simultaneous operation of the wind farms on the landscape is estimated to be insignificant negative, with the new turbines providing visual continuity and harmoniously integrating into the landscape.

- Impact on human health: In addition to noise, wind turbines can produce flicker effects (intermittent shadows caused by rotating blades). From the shading study carried out for the project proposed by the PUZ, it can be estimated that in the contact area of the two parks the shading effects of the two parks may overlap, but this area is in the outskirts of the localities, 1.2 km away from inhabited areas, so that the cumulative impact of the simultaneous operation of the park proposed by the PUZ with the Chirnogeni-Independența park is estimated as insignificant negative.

- Impact on the electricity grid: The addition of a new wind farm in an area with several existing and proposed wind farms may affect the stability and balance of the electricity grid. Through the projects of connection to the National Energy System (SEN) the capacity of the grid to manage the energy produced and to ensure an efficient integration is assessed in detail, so a cumulative impact of the simultaneous operation of the park proposed by the PUZ with the Chirnogeni-Independența park is not estimated

- Impact on biodiversity In order to assess the impact on species and habitats of the Natura 2000 sites considered in the assessment, the cumulative impact of the project on them was analyzed. For this purpose, the presence of pressures and threats in the Management Plans and Standard Forms of the sites, as well as other projects to be carried out or currently in the process of being carried out that have the potential to affect habitats and species of community interest, were analyzed.

The management plans of the potentially affected Natura 2000 sites indicate several pressures and threats that may affect habitats and species of Community interest, among the most common being urbanization and the expansion of land areas. Road or highway construction or road traffic are also mentioned as pressures/threats for some of the sites included in the assessment.

Gathering information on proposed plans and projects in the area was also carried out through consultation with local stakeholders, in particular the municipalities of the TAUs in the proposed plan area.

Table  6-10 Characteristics of other PPs (under implementation, approved or under evaluation) that may have a cumulative impact on ANPIC with the evaluated PP

| No. ctr. | Name PP | Location in relation to ANPIC (distance) | Effects generated |  |  |  |  |  |  |  |  | Impacts |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Changing air quality | Increased noise and vibration | Increased brightness | Increased concentration of pollutants in soil/pollution | Land occupation | Changes in vegetation | Introduction/ spread of invasive species | Wildlife mortality | Emergence of behavioral barriers for wildlife | PH | AH | FH | PAS | REP |
| ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 5.1 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 0.25 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 0.095 km | X | X | - | - | - | - | X | X | X | - | X | X | X | X |
| 4 | Comana - Pecineaga Wind Farm, Tătaru and Comana (proposed) | 0.59 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 6.61 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6 | Karnobat wind farm (existing) | 16.89 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSCI0157/ROSAC157 Hagieni - Cotul Văii Forest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 13.57 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 20, 50 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 19.05 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Comana - Pecineaga Wind Farm, Tătaru and Comana (proposed) | 1.66 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 10.34 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6 | Karnobat wind farm (existing) | 13.62 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSPA0036 Dumbrăveni |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 10.66 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 3.34 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 4.67 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Comana - Pecineaga Wind Farm, Tătaru and Comana (proposed) | 12.75 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 17.42 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6 | Karnobat wind farm (existing) | 23.05 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSPA0166 Plopeni-Chirnogeni |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 3.18 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 4.37 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 7.68 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Comana - Pecineaga Wind Farm, Tătaru and Comana (proposed) | 1.84 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | Intersected | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6 | Karnobat wind farm (existing) | 18.91 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| ROSPA0094 Hagieni-Cotul Văii Forest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 17.05 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 23.36 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 23.04 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4 | Comana - Pecineaga Wind Farm, Tătaru and Comana (proposed) | 4.26 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 14.26 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6 | Karnobat wind farm (existing) | 20 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| BG0000569 Kardam |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 9.65 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 13.05 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 7.38 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 4 | Comana - Pecineaga Wind Farm, Tătaru and Comana (proposed) | 8.86 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 5 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 13.62 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6 | Karnobat wind farm (existing) | 0.54 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| BG0000570 Izvorovo - Kraishte |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chirnogeni wind farm (existing) | 12.70 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 2 | Wind energy park 14 CE-92,4 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan Fântana Mare, com. Independența, jud. Constanța (proposed) | 8.51 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 3 | Wind energy park 17 CE-112,2 MW, transformer station, electrical connection networks, construction and modernization of communication and access roads - extravilan com. Independența and com. Cerchezu jud. Constanța (proposed) | 1.63 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 4 | Comana - Pecineaga Wind Farm, Tătaru and Comana (proposed) | 14.77 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |
| 5 | Modernization and rehabilitation of streets in Chirnogeni commune, Constanta county (proposed) | 18.92 km | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| 6 | Karnobat wind farm (existing) | 12.79 km | - | - | - | - | - | - | - | X | X | - | X | X | X | X |

| Legend | PH | habitat loss |
| --- | --- | --- |
|  | AH | altering habitat conditions |
|  | FH | habitat fragmentation |
|  | PAS | species disturbance |
|  | REP | Reducing stock numbers |

As regards the cumulative impact of the project proposed by the PUZ with the other wind farms proposed to be developed in the area, it is difficult to estimate in the absence of concrete data on the status of the projects and the estimated period of commencement of the implementation.

Applying the precautionary principle, for the estimation of the cumulative impact, a worst-case scenario will be assessed in which all projects are considered to be implemented during the same period. Taking into account the positioning of the proposed wind farms, the distances between the sites, the number of wind turbines proposed (about 123 wind turbines), it can be estimated that the cumulative impact during the execution period is significant negative for the environmental aspects biodiversity air, soil/soil and use of natural resources justified by the number of turbines, the areas affected by the works and the need for resources for the foundations, platforms, connections and roads (new or rehabilitated), and insignificant negative for the environmental aspects: water, waste management, noise and vibration, population and human health, climate change landscape and cultural heritage.

In the simultaneous operation phase of the wind farms, no impact on environmental aspects: water, soil/soil, waste management, landscape and cultural heritage is estimated. However, a potential negative impact on the biodiversity component is expected, particularly on bird and chiropteran species, which are prone to negative impacts due to the wind infrastructure.

The simultaneous operation of the wind farms will have a significant positive cumulative impact on air quality and climate change (reduction of GHG emissions) by replacing the amount of electricity produced from fossil fuels in the absence of the wind farms. Another significant positive impact is the improvement of the standard of living and social conditions of the population in the project areas.

During operation, particular attention will be paid to noise, shading and biodiversity monitoring to minimize cumulative impacts.

# Possible significant environmental, including health, effects in a transboundary context

In this chapter, we intend to analyze the possible significant environmental impacts, including health impacts, in a transboundary context, generated by the construction and operation of the proposed energy complex to be realized in the Cerchezu municipality, in the immediate vicinity of the border with Bulgaria.

In order to provide an overview of the potential impacts, we will further examine the current status in Bulgaria of the environmental factors presented in chapter 2.1, thus identifying potential risks and impacts associated with the project.

Water

The nearest surface water bodies identified in the area studied by PUZ on the territory of Bulgaria are located south of the site boundary: BG1DJ900R1015 at a distance of about 1 km, BG2DO700L017 at a distance of about 34 km, respectively BG2BS000C001 at a distance of about 36 km, south-east of the site boundary.

The PUZ study area is located in the vicinity of two groundwater bodies in Bulgaria (BG1G000000N049 and BG1G00000000J3K051).

The PUZ does not foresee the withdrawal of groundwater or surface water from the site area, so there will be no effects on the hydrology of the area, nor will other activities dependent on this resource be affected in secondary.

The construction of the wind farm in the Cerchezu municipality will not have a significant negative impact on the quality of surface or groundwater bodies in Bulgaria.

Air

According to the requirements of national and European legislation, the territory of Bulgaria is divided into six regions for ambient air quality assessment and management (ROCAV) - agglomeration Sofia, agglomeration Plovdiv, agglomeration Varna, North/Danube, South-West and South-East. The analysis of ambient air quality (AQA) data is carried out by region, taking into account the specifics of each settlement where the monitoring is carried out. In the year 2020, the National Ambient Air Quality Monitoring System (NMSCAV) operated a total of 48 stationary points - 34 automatic measuring stations (AIS), 9 points with manual sampling (RP) and subsequent laboratory analysis, 5 DOAS (on the principle of optical differential atomic absorption spectrophotometry) systems located in the cities of Svishtov, Nikopol, Silistra, Burgas and St. Zagora (village of Rajena) .

Figure 7. 1 Map of air quality assessment and management zones in Bulgaria

According to the map the project borders the North Zone (gray portion of the map) in terms of the air quality assessment and management area.

The main sources of air pollution in Bulgaria are similar to those in Romania, being represented by transportation activities, construction/dismantling activities, diffuse combustion sources, agricultural activities, industrial activities.

Transportation activities include only road traffic, there are no rail lines in the area.

The main specific pollutants resulting from road transport activity are: nitrogen oxides (NOx ), sulphur dioxide (SO2 ), carbon oxides (COx ), particulate matter (PM10 , PM2,5 ), volatile organic compounds (VOC).

According to the information published for the fourth quarter of 2022 - North region the following issues were highlighted:

- Sulphur dioxide (SO2 ): According to reports for the fourth quarter of 2022, no exceedance of SO2 .

- Nitrogen dioxide (NO2 ): The levels of NO2 were within the permissible limits and showed no significant exceedances.

- Fine particulate matter (PM10 and PM2.5 ): Problems with PM10 pollution have continued, with occasional exceedances of the standards, especially during the cold season.

As can be seen from the above, no significant exceedances of the limit values set by the Bulgarian legislation in force in the project area were found.

Taking into account current activities, the air quality can be considered to be generally good, as it is an open rural area uncrowded by urban developments and without intense industrial activities.

The construction of the wind farm in the Cerchezu municipality will not have a significant negative impact on air quality in Bulgaria. During the construction phase, dust emissions from excavation works, vehicles and construction materials, as well as emissions of pollutants specific to fossil fuel combustion associated with machinery and transportation means are the main sources of emissions. Erosion from excavation can also alter local air quality.

The level of emissions of dust and pollutants varies significantly depending on the daily activity, the specific nature of the works and the meteorological conditions, and is dependent on the phases of the construction process (road laying, earthworks, foundations, turbine erection). However, these emissions are temporary and localized, their impact being felt only in the perimeter of the sites and work fronts.

When in operation, wind turbines help reduce greenhouse gas (GHG) emissions by displacing fossil fuel electricity, thus contributing to climate change mitigation and having a significant positive impact on air quality.

Soil/subsoil

Northern Bulgaria, part of the Danube Plain, is characterized by a variety of geological formations. It includes recent alluvial sediments deposited by rivers flowing into the Danube, as well as older sedimentary rock formations from the Neogene and Paleogene periods . The Danube Plain is characterized by flat and gently undulating terrain, ideal for agriculture.

In Bulgaria, over the period 2011-2021, there was a constant trend of increasing or maintaining the area occupied by arable land and decreasing the area occupied by non-arable land. Compared to 2020, the increase in arable land is insignificant (by 0.27%) . In 2021, the utilized agricultural area was 5 046 597 ha and occupies 45.5% of the country's territory.

According to the map of soils in the coverage area of the PUZ presented in subchapter 2.1.3. it can be observed that in the south of the proposed project in the territory of Bulgaria chestnut soils (xerophytic forests and sibleacs) with interlayers of chernozem soils predominate (xerophytic forests and sibleacs) with interlayers of chernozem soils.

Chernozem soils are very fertile, humus-rich soils, ideal for agriculture, while the forest ash soils, although less fertile, are also used in agriculture and forestry. This region, part of the Danube Plain, is predominantly agricultural due to the fertile soils.

Concerning the distribution of land use categories (see Figure 2. 12 Distribution of land use categories in the coverage area of the PUZ), the Bulgarian territory in the vicinity of the proposed project shows an area of arable land (majority), with small interspersions of deciduous forests, predominantly agricultural land mixed with natural vegetation, discontinuous urban space and industrial or commercial units.

In conclusion, the northern part of Bulgaria, bordering Romania, is relatively anthropized and has predominantly fertile soils.

In terms of impact on soil and subsoil, there is no significant impact on soil and subsoil in the neighboring territory of Bulgaria. During the execution phase, the impact on soil is temporary and reversible, limited to the perimeter of the construction works. Changes in soil structure and water infiltration regime are managed through ecological restoration measures. Possible sources of pollution from vehicle and machinery activities are monitored and controlled to prevent contamination. In addition, wind turbines have a smaller footprint on the ground than other forms of energy, which limits soil erosion and compaction. During operation, the wind farm does not generate pollutants that affect the quality of the soil or subsoil, thus ensuring long-term environmental protection.

Use of natural resources

In chapter 2.1.4, it was described that the implementation of the wind farm project in Cerchezu commune, Romania, offers an opportunity to capitalize on renewable energy resources. The following quantities of resources will be used for the realization of the wind farm: topsoil from stripping (about 108600 m³), natural earth from excavation (24350 m³), crushed stone fill (114250 m³), and insignificant quantities of water for various activities on the site.

The construction of the wind farm will not use any natural resources on the territory of Bulgaria, nor will it affect Bulgaria's natural resources. All resources needed for the project will be extracted and managed on Romanian territory. Thus, Bulgarian soils and subsoils will remain unaffected by the activities related to the construction and operation of the wind farm in the Cerchezu municipality.

Waste management

During the implementation of the wind farm project in Cerchezu commune, Romania, proper waste management will be ensured through proper waste collection, storage and disposal. Waste generated will include construction materials, packaging and household waste, all managed according to Romanian legislation. No waste will be transported or managed on Bulgarian territory, thus ensuring that Bulgarian resources and environment will not be affected by the activities related to the construction and operation of the wind farm.

Noise and vibration

During the execution phase, the means of transportation and machinery used, as well as the activities at the work front, will generate noise and vibrations. However, as the area is crossed by local and farm roads, the impact will be local, temporary and insignificant, with no impact on the territory of the neighboring State of Bulgaria.

During operation, the noise produced by wind turbines will increase with wind speed and blade rotation speed. Modern turbine design includes nacelle isolation to prevent the transmission of mechanical noise and vibration. Noise levels vary between 92 and 107.7 dB, and noise modeling shows that in the receiving areas the level will be below 40 dB.

Thus, the construction and operation of the wind farm will not generate significant noise and vibration affecting the territory of Bulgaria. Mitigation measures, such as strategic siting of the turbines and the use of modern technologies, will reduce noise and vibration emissions. This will not affect communities and the environment in Bulgaria.

Biodiversity

In the vicinity of the PUZ at a distance of less than 5 km a number of 2 natural areas of Community interest in Bulgaria have been identified: BG0000569 Kardam located at a distance of about 410 m from the boundary of the PUZ and BG0000570 Izvorovo - Kraishte located at a distance of about 3.64 km from the boundary of the PUZ.

As detailed in the Adequate Assessment Study, it is expected that there will be a semi-significant negative impact on some species within the protected natural areas: ROSCI0071/ROSAC0071 Dumbrăveni - Urluia Valley - Vederoasa Lake, ROSCI0157/ROSAC157 Hagieni Forest - Cotul Văii, ROSPA0036 Dumbrăveni, ROSPA0166 Plopeni-Chirnogeni, ROSPA0094 Hagieni Forest - Cotul Văii, BG0000569 Kardam, BG0000570 Izvorovo - Kraishte. The pontential groups significantly affected by the implementation and operation of the wind energy project will be terrestrial mammals, chiroptera and avifauna. For these groups a number of both general and specific measures have been proposed that will result in insignificant residual impacts after implementation. In addition, in order to have as accurate a picture as possible of the abundance and number of species using the proposed site as breeding/feeding/nesting habitat.

Population and human health

The park is being developed in the Cerchezu local administrative unit, in its extravilan area, the nearest Bulgarian localities being Iovkov, 1.56 km from turbine T3 and Kraiste, 2.1 km from turbine T47.

Iovkovo and Kraiste are two villages in the commune of Gheneral-Toșevo, Dobrici region, South Dobrogea, Bulgaria .

As for the total population of the commune of Gheneral-Toșevo as of 31.12.2023 it is 5 429, out of which the male population is 2 634 and the female population is 2795. Compared to 2011, in 2023 the female population of Gheneral-Toșevo has decreased by about 21% and male population has decreased by 22% .

The structure of the population by age and sex on 31.12.2023 in the Dobrici region is shown in the table below:

Table 7 - 1 Population structure by age and sex

|  |  | AGE GROUPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | sub 5 | 5-9. | 10-14. | 15-19 | 20-24 | 25-29 | 30-34 | 35-39 | 40-44 | 45-49 | 50-54 | 55-59 | 60-64 | 65-69 | 70-74 | 75-79 | 80-84 | over 85 |
| Total | 2714 | 8 | - | 1 | 3 | 4 | 1 | 9 | 14 | 33 | 54 | 84 | 137 | 224 | 297 | 417 | 433 | 415 | 580 |
| Male | 1433 | 3 | - | - | 1 | 4 | 1 | 7 | 11 | 20 | 42 | 60 | 91 | 149 | 205 | 252 | 210 | 175 | 202 |
| Female | 1281 | 5 | - | 1 | 2 | - | - | 2 | 3 | 13 | 12 | 24 | 46 | 75 | 92 | 165 | 223 | 240 | 378 |

Source: NSI Bulgaria ( https://www.nsi.bg/en/content/2981/population-towns-and-sex)

According to the data provided by the National Institute of Statistics, the evolution of the population in Gheneral-Toșevo commune shows a downward trend in the period 2010-2023 as shown in the graph below:

Figure 7. 2 Total population change 2010-2023 in Bulgaria

The construction and operation of the wind farm in the Cerchezu municipality will not affect the population on the territory of Bulgaria. As presented in the previous chapters, during the construction phase, noise, vibration and dust emissions will be local and temporary, with low impact. In the operation phase, the noise produced by the turbines will be below 40 dB in the reception areas, respecting the legal limits. The overshadowing effect is insignificant and the visual impact is subjective. In the decommissioning phase, the impact will be similar to the construction phase, being local and temporary.

Landscape

According to the map with the landscape types characteristic of the area where the PUZ is proposed, it can be seen that the landscape types in the project area are similar for both countries, as follows:

- Continental landscape - this area represents hills and arable land with sedimentary soils. It is predominant in most of the PUZ coverage area.

- Stepic landscape - this area is a steppe landscape with hills and sedimentary soils.

In Figure 2. 22 Degree of landscape fragmentation in the PUZ implementation area, the map provides a detailed analysis of the landform energy and degree of landscape fragmentation. Land fragmentation and relief energy are essential for understanding potential environmental impacts and for planning infrastructure projects.

The map shows that on the territory of Bulgaria, in the vicinity of the perimeter of the PUZ, very low fragmentation prevails. The relief is moderately fragmented, suggesting the presence of low hills and valleys. This type of landscape can support diversified agricultural activities and has moderate biodiversity.

The construction and operation of the wind farm in the Cerchezu municipality will not affect the landscape of Bulgaria.

Climate change

Bulgaria's national climate policy is determined, on the one hand, by the country's international commitments stemming from the United Nations Framework Convention on Climate Change (UNFCCC), the Kyoto Protocol (KP) and the Paris Agreement, which provide the general framework for international efforts to address the challenges of climate change, and, on the other hand, by the obligations stemming from the country's EU membership and from the current and recently adopted European legislation in this field.

As an EU Member State, Bulgaria is committed to reducing greenhouse gas (GHG) emissions by 55% by 2030 compared to 1990 levels, as part of the EU's objective to achieve climate neutrality by 2050 .

According to the National Report on the State and Protection of the Environment in Bulgaria major GHG emissions - for the period 1988-2021 are on a downward trend. In 2021, total GHG emissions were generated - 53917,27 Gg CO2-eq . This means that GHG emissions have decreased by 52.55% compared to the emissions in the base year 1988.

The following three graphs show the quantities of the main GHGs emitted by the different sectors in Bulgaria.

Source: National GHG Inventory Report 2021

Figure 7. 3 Share of main sources of GHG emissions in 2021, %

The National Report on the State and Protection of the Environment in Bulgaria presents the following aspects of climate factors:

- average annual temperature - in the period 1988-2021, the average annual air temperature for the lower part of the country fluctuates between 10.6°C and 13.3° C as shown in the graph below

Source: NIMH

Figure 7. 4 Annual mean air temperature fluctuations (oC) 1988-2021

In 2021, the average annual temperature for the lowlands was 12.3°C, which is 0.3°C above normal. This is the thirteenth warmest year from 1988-2021, and January is the fifth warmest since 1930 - an average of 2.5°C above the monthly norm (from +1.2°C in the village of Kostenets to +3.7°C in Ruse).

Source: NIMH

Figure 7. 5 Departure of annual mean air temperature (°C) in 2021 from the climate norm for the period 1991-2020

- average annual precipitation - in the period 1988-2021 for regions with altitudes up to 800 m above sea level ranges from 377 mm to 1013 mm, while maintaining the positive trend of this indicator (+4.2 mm/year). In 2021, the average annual precipitation was 741 mm, which is about 20% above the norm for the 1991-2020 period.

Source: NIMH

Figure 7. 6 Fluctuations in mean annual precipitation (mm) for regions up to 800 m elevation in the period 1988-2021

- snow - in the period 1988-2021 there was no significant decreasing trend in the fluctuations of the mean maximum mean snow cover height in the altitude zones between 800-1800 m. The value for 2021 is 43 cm - below the average for the period 1991-2020.

- climatic phenomena - during 8-12.01.2021, torrential and fluvial floods were recorded, mainly in western and south-eastern Bulgaria, due to heavy precipitation, combined in some places with snow melting. In the districts of Sofia, Blagoevgrad, Yambol and Burgas, the amount of precipitation for 72 hours exceeded three times the monthly norm. State of disaster due to floods, destroyed bridges and broken roads was declared in 8 districts. Heavy snow in north-west Bulgaria led to power cuts in many places.

Cultural heritage

No data sources have been found that completely list all the historical monuments in Bulgaria by region, as they exist in Romania. However, the construction of the proposed park will not affect the territory of Bulgaria, thus ensuring the preservation of the country's cultural and historical heritage.

In accordance with Law 22/2001 on the ratification of the Convention on Access to Information, Public Participation in Decision-making and Access to Justice in Environmental Matters (Aarhus Convention), it is essential to ensure full transparency and involvement of the local and transboundary community at all stages of the project. This includes access to relevant information on environmental and health impacts and active public participation in decision-making. Compliance with this law ensures that all interested parties are informed and have the opportunity to contribute to decision-making, thus ensuring effective and equitable management of environmental impacts.

The complete analysis of the construction, operation and decommissioning phases shows that, at the border line, noise, air quality, water and other environmental components remain permanently below the limit levels allowed by European standards, so that the project does not generate significant risks for the population or ecosystems of the neighboring state.

| Environmental factor | Border Impact Summary (Romania / Bulgaria) | Pages in the Republic of Moldova |
| --- | --- | --- |
| Water | The undercrossings of the Cerchez and Magura rivers are carried out by horizontal drilling; The risk of sediments or accidental leaks remains local and temporary. No water is captured from the RODL04 and RODL06 transboundary bodies; The chemical quality is not affected across the border. | 31-34, 98, 179 |
| Air | ADMS modelling shows that the dust and gases in the construction phase are diluted below the limits before the border; In operation, the turbines do not emit pollutants. | 90-92 |
| Ground / Basement | Any oil or fuel leaks can contaminate punctiformly, but it is controlled by absorbent materials; There is no overseas transport vector. | 98-99 |
| Natural resources | No aggregates or water from Bulgaria are exploited; The materials come from local chains, so the consumption of resources remains internal. | 204 (OM4) |
| Waste | Only inert waste and packaging appear in the construction, selectively collected and transported to the county landfill; in operation – small quantities of turbine oil, handed over to authorized operators. No flow crosses the border. | 117, 128-129 |
| Noise and vibration | SoundPLANnoise modelling indicates ≤ 36 dB in villages and 27 dB in the Natura 2000 area "Kardam", below the WHO thresholds; in operation, the cumulative level remains < 40 dB at the border. | 99-100, 138-139, 178-180 |
| Population and health | Calculated noise level at homes < 40 dB; the modeling shows increases of only 0.016 dB compared to the background. Air emissions and vibrations do not reach risk values for Bulgarian communities. | 66-68, 178-180 |
| Climate | Wind farm (303 MW) will avoid CO₂ emissions by replacing fossil fuel-based energy; The positive contribution is common to both states. | 118, 180 |
| Cultural heritage and landscape | The archaeological diagnosis required prevents any damage; the visual profiles show landscape continuity, and the objectives in Bulgaria do not enter the visibility cone of the turbines. | 116, 140 |

In order to clarify to what extent the operation, construction and decommissioning of the wind farm can influence the health of the population of the neighboring state, the following paragraphs summarize the results of the noise, air, water and vibration modeling, in relation to the safety thresholds established at international level and European legislation, as well as the possible associated climate benefits.

- Noise and vibration

Construction / decommissioning. The SoundPLANnoise modeling shows that, at the most exposed point of the Romanian localities, the equivalent continuous level reaches ≈ 36 dB(A) – below the limit of 55 dB(A) day and over 1 km from the border; the area where it could exceed 55 dB(A) is restricted to < 5 ha, directly on the work front and at a maximum of 300 m from it.

Operation. After commissioning, the cumulative noise of the 46 turbines is modeled at < 40 dB(A) in the villages and at 27 dB(A) at the level of the Natura 2000 site BG0000569 "Kardam" in Bulgaria, i.e. +0.2 dB compared to the natural background and well below the WHO guideline of 40 dB(A) at night. The increase calculated in the nearest households is only 0.008-0.016 dB – clinically imperceptible.

Thus, the potential cumulative impact of the plan with the activities in the area (agriculture around the plan, transport infrastructure Road II-29 Dobrich – border – 1.5 km and energy infrastructure – Karnobat wind farm – 5.6 km) will be small, insignificant.

Health implications. The literature cited in the report mentions cardiovascular effects at chronic exposures > 45 dB(A); the frontier value (27 dB(A)) remains ~18 dB below this threshold, resulting in a virtually zero risk to the Bulgarian population.

- Air quality

Construction activities produce dust and exhaust gases, but ADMS modelling indicates a sub-detectable contribution to the boundary line; locally the PM₁₀ background of the commune is ~12 μg/m³, i.e. below half of the European limit of 40 μg/m³.

In operation, the turbines do not emit pollutants, and the energy produced replaces gas or coal generation, reducing regional emissions.

Thus, the potential cumulative impact of the plan with the activities in the area (agriculture around the plan, transport infrastructure Road II-29 Dobrich – border – 1.5 km and energy infrastructure – Karnobat wind farm – 5.6 km) will be small, insignificant.

Health implications. Additional exposure to particulate matter and NO₂ is virtually nil.

- Drinking water and surface waters

The construction activities do not include technological discharges; rainwater is collected in the gutters and infiltrates the site. There is no connection with the transboundary water bodies, and the monitoring of the Constanta Public Health Department shows compliant drinking parameters throughout Cerchezu commune.

During the operation phase, maintenance works are carried out on platforms equipped with retention trays sized to fully collect any oil leaks, thus eliminating any discharge of liquid substances into the environment.

Therefore, the cumulative impact with the activities in the area (agriculture around the plan, transport infrastructure Road II-29 Dobrich–border – 1.5 km and energy infrastructure – Karnobat wind farm – 5.6 km) is insignificant, as there is no vector for the transport of contaminants to Bulgaria.

Health implications. The safety of drinking water sources in the neighboring state remains unaffected.

- Soil and seismic vibrations

In the construction phase , small excavations and isolated foundations are carried out; the excavated earth remains on site, and the vibrations of the machines are temporally limited.

In operation , there is no more ground work, and the vibrations generated by the turbines quickly dissipate in the geological layer, falling below the threshold of human perception up to the border.

Cumulative potential. The sum of all sources does not exceed 0.2 mm/s at the border, so the effect remains insignificant.

- Shadow-flicker and visual effect

The modelling indicates 0.1–8 h/year of intermittent shadows in the nearest Romanian households; At the border, the duration decreases below the detection limit. The Karnobat wind farm adds an apparent angle of only 0.3–0.4°, insufficient for shadow overlap.

Cumulative potential: even if both parks were to generate shadows simultaneously, the total would remain < 10 h/year, well below the international threshold of 30 h/year, resulting  in insignificant cumulative visual impact.

- Traffic and road pollution

The increase in traffic on DJ 391 in the construction phase adds < 3 μg/m³ NO₂ and < 2 μg/m³ PM₁₀, values that dissipate before reaching the neighboring country limit.

In operation, the service visits amount to only a few vans per day, and at decommissioning the flow of trucks is lower than in construction and limited in time.

Health implications. The concentrations of pollutants remain below the limit values set at European level; Thus, there are no exceedances of the existing values at the level of the Bulgarian communities.

- The climate benefit, with an indirect effect on health

The installed capacity (≈ 303 MW) will avoid ~0.55 Mt CO₂ regionally over its lifetime, reducing the risk factors associated with fine particulate mortality and heat waves

In construction and decommissioning, CO₂ emissions from machinery diesel are punctual and low compared to the total carbon savings.

Health implications. By contributing to the decarbonization of the regional energy mix, the project reduces the population's exposure to fossil-based air pollutants and limits the intensity of extreme heat episodes; These factors, epidemiologically correlated with the incidence of cardiovascular and respiratory diseases, are attenuating, which supports the increase in life expectancy and the decrease in public health costs on both sides of the border.

The comparative evaluation of the three stages shows that:

- The construction brings local-temporary disturbances (dust, noise, inert waste), but these dissipate before the border line; there are no exceedances of international standards for air or noise, and the water works are controlled by horizontal drilling and emergency plans.

- The operation has the lowest degree of pressure: the turbines do not emit liquid or gaseous pollutants, the noise calculated at the border is 18 dB below the maximum permissible threshold, and the project generates a net climate gain by avoiding fossil fuels.

- Decommissioning reiterates the disruptions in the construction, but in a shorter period and with the possibility of recycling more than 90% of the dismantled materials; the transport and treatment of waste are managed exclusively on the national territory, eliminating the cross-border vector.

Therefore, no combined life-cycle environmental factor exceeds the maximum permissible values at European level. The effects remain local, temporary and of low intensity, and in terms of climate-energy, the balance is positive for both states. Thus, the project sums up an insignificant cross-border impact  throughout its operational path.

| Project phase | Activities in the vicinity of the plan | Cumulative cross-border impact |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Water | Air | Ground/Basement | Natural resources | Waste | Noise and vibration | Population and human health | Climate | Cultural heritage |
| Construction/decommissioning | Agricultural activities – immediate vicinity | Rainwater infiltrates the soil and does not reach the transboundary rivers No wastewater is discharged into surface water bodies crossing the border, and the regional groundwater has no significant transboundary flow. | The relevant emissions are dust and exhaust gases from machinery; dissipates quickly, and the distance to the border (> 1.5 km) and the short duration do not allow the limit concentrations to be exceeded on the territory of the neighboring state | The excavations only produce temporary storage of earth and gravel on site; There is no mechanism for the effect to cross the border. | The materials (ballast, cement, steel) come from the local market of the host country; their extraction/reduction of availability does not affect resources across the border. | Plant and construction debris are collected and disposed of at nearby licensed facilities. No waste is transported or deposited across the border. | Site noise levels (70-75 dB(A) at source) fall below 35-40 dB(A) at 1 km; at the border are below the WHO materiality thresholds, so there is no cross-border health impact. | Direct exposure is limited to nearby rural areas; The population across the border is not exposed to any additional risk factors. | GHG emissions from construction (diesel machinery) are negligible compared to the national inventory and irrelevant on a transboundary scale. | There are no common archaeological sites on the border that are influenced by the works. |
|  | Power generation activities (Karnobat Wind Farm) – 5.6 km | Bulgarian turbines do not discharge water; The direction of the underground flow is opposite to the border | The air is not loaded with pollutants, and there are no odor sources. | The foundations are stable; no particles are transported to Romania, the cross-border impact being zero | In the process of operating the park, no natural resources related to the Romanian state are used | The waste is not transported across borders, so the impact is zero | Given the distance from the border and the fact that the disturbance caused by the noise produced by a wind farm is felt at a maximum of 500 m, the transboundary impact is zero | The localities in the immediate vicinity, given the distance from the wind infrastructure, are not affected | The operation of the park does not bring a GHG contribution to the local level | Turbines do not influence Romanian historical sites |
|  | Transport infrastructure Road II – 29 Dobrich – Border with Romania – 1.5 km | Road gutters prevent cumulative cross-border impacts | Possible temporary impact due to site traffic and construction work | Existing infrastructure has no potential to have a cross-border impact | The existing infrastructure does not use natural resources other than local ones | No cross-border waste is conveyed | Even if the noise level will increase locally, it does not cumulate with the existing infrastructure | The Romanian population will not perceive changes, the exposure remaining unchanged | The climate impact will be insignificant | The existing infrastructure does not affect common cultural areas or historical landscapes |
| Operation | Agricultural activities – immediate vicinity | Rainwater penetrates the ground and does not reach the border rivers. | The dust produced dissipates quickly and does not increase pollution at the border. | Light tillage does not create erosion that influences transboundary soiling. | Fertilizers and fuel come from the local market; no resources from the neighboring country are used. | The straw remains in the field and is briquetted or incorporated; No waste is transported across the border. | The sound of the machines fades before the border and remains below the level allowed in rural areas. | The distance to the villages in the neighboring country is too long for risks or discomfort to occur. | Emissions from agriculture are very low and do not influence the climate abroad. | The agricultural landscape does not contain common cultural objectives that could be affected. |
|  | Power generation activities (Karnobat Wind Farm) – 5.6 km | Turbines do not generate wastewater; Any hydraulic oil is in closed circuits, with retention systems, so the risk of cross-border contamination is non-existent. | The operation does not produce atmospheric emissions; in fact, it replaces fossil sources and reduces regional pollution. | Minimal contact with the ground after the completion of the foundations; there are no erosion or contamination processes that cross the border. | The consumption of resources is limited to spare parts; their extraction is not related to territories in the neighboring state. | Maintenance waste (filters, oils) is small in volume and is managed by authorized operators in the host country. | Acoustic modelling for turbines shows < 35 dB(A) at distances > 1 km; The border is far enough, so the reception limits in the neighboring state are not exceeded. | Possible visual discomfort for the inhabitants of the vicinity (< 2 km), but the settlements across the border are further away; There are no proven medical or psychological effects at that distance. | The impact is positive (reduction of CO₂ emissions); The benefit is global, so it does not generate negative cross-border "impacts". | No cross-border sight-line heritage sites were identified that would be affected by the presence of turbines. |
|  | Transport infrastructure Road II – 29 Dobrich – Border with Romania – 1.5 km | The water from the road drains into the gutters next to the road and does not cross the border. | The normal level of traffic does not produce an increase in air pollution beyond background values. | The infrastructure does not bring a contribution of dangerous substances to the Romanian soil. | Only local resources are used | The resulting waste is disposed of locally through authorised operators | The existing noise will not be cumulated with the noise produced by the operation of the park | The level of comfort and health for the inhabitants of the immediate vicinity will not change | The operation of the existing infrastructure will not lead to an influence on the transboundary climate balance | Common cultural objectives will not be affected |

# Proposed measures to prevent, reduce and compensate as fully as possible any adverse environmental effects of the implementation of the PUZ

Government Decision 1076/2004 requires the establishment of measures to prevent, reduce and compensate for significant environmental effects resulting from the implementation of the plan subject to environmental assessment.

Although, from the analysis of the evaluation of the objectives of the Zonal Urban Plan, it appears that the environmental objectives will be achieved, it is necessary to establish preventive measures to offset any negative effects and to strengthen the positive effects. As the impact analysis of the measures proposed by the analyzed plan shows, some of them will have a negative influence on environmental factors. Prevention and minimization of adverse environmental effects can only be achieved through environmental assessment at all stages of project preparation and implementation.

The following table summarizes the environmental protection measures recommended for the implementation of the PUZ. In establishing these measures, account was taken of the legislative provisions, as well as of the measures laid down in national/regional strategies and local development strategies.

The measures envisaged are based on the code of good practice specific to construction works/site organization and have taken into account the environmental legislation in force at the time of the study, local conditions and the environmental sensitivity of the area.

Table 8 - 1 Proposed measures to prevent, reduce and compensate as fully as possible any adverse environmental effects of the implementation of the PUZ

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Proposed measures | Responsible |
| --- | --- | --- | --- |
| Water | - Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | Execution stage: - the disposal of domestic fecaloid wastewater will be done in portable ecological toilets, will be emptied and transported to the nearest treatment plant. - carrying out the works by providing drainage slopes for rainwater. - Drinking water supply during the site organization period will be provided from external sources: bottled water. - construction materials shall not be stored in the vicinity of watercourses, in order to prevent deterioration of the status of water bodies (ecological status/ecological potential and chemical status for surface water bodies, respectively quantitative status and chemical status for groundwater bodies). - The waste will be selectively collected and handed over to specialized companies in order to prevent spills. - all works shall be supervised in such a way as to prevent possible contamination of the area due to accidental spillage of fuels or lubricants from equipment/utilities used in the works. In the event of accidental pollution, immediate action will be taken with absorbents/neutralizers, and breakdowns of means of transport and/or machinery will be rectified in specialized service units. - the works of undercrossing the watercourses (Cerchez and Magura rivers) will be carried out by horizontal drilling at a depth of 1 m between the upper generator of the protection pipeline and the watercourse slope Operating stage: - the technologies used during the operation of the wind farm do not have any impact on the environmental factor water and no measures are needed to prevent, reduce and compensate for adverse environmental effects. Decommissioning phase: - the decommissioning work will be carried out in compliance with all precautionary measures to eliminate accidental spills of petroleum products and the collection of all waste resulting from this work. In the event of accidental spills of petroleum products on the ground, they will be collected by means of absorbent materials to be provided on site and by removing/cleaning the affected soil layer. | The contractor responsible for the wind farm |
| Air | - Maintaining air quality by reducing emissions from energy sector activities | Execution stage: - the use of water spraying of the working face to reduce dust, if the resulting dust is visible after the project works. - as regards emissions from transport vehicles, they must comply with the technical conditions laid down in the roadworthiness tests that are carried out periodically throughout the period of use of all motor vehicles registered in the country. - using machinery/means of transport with high-performance engines (EURO 4 or EURO 5) and driving at low speed (maximum 30 km/h), especially on dirt or gravel roads (in very dry periods, it is recommended to spray them with water). - tipper trucks must be fitted with tarpaulins during transportation. - the topsoil on the site of projects on which foundation works are being carried out will be stored and reused to restore the land to its original state after the works have been completed. - dust-intensive technological processes will be reduced during periods of high winds, or more intense wetting of surfaces under the action of working machinery or access roads, especially unpaved ones, will be pursued. - the application of paints on building elements shall be done by using minimum quantities of paint, primer and thinners and applying with special devices that ensure minimum VOC emissions to the atmosphere. Operating stage: - The operation of the wind farm does not have a negative impact on the air and no measures are required to prevent, reduce and offset adverse environmental effects. Decommissioning phase: - during the decommissioning period the impact is similar to the execution period, therefore the same measures will be taken with regard to work fronts, decommissioning and transportation equipment. | The contractor responsible for the wind farm |
| Soil/subsoil | - Limiting soil pollution and land degradation. | Execution stage: - delimitation of work areas before the start of construction works, so as to indicate the boundaries between which construction - erection activities will take place, and minimization of affected areas. - Controlled storage of construction materials and waste generated during the execution and decommissioning phase will be ensured in specially designated areas on site. - avoid the storage on the ground of materials which, exposed to precipitation, may cause seepage into the soil and groundwater (impermeable storage areas). - deposits of fertile soil and earth resulting from excavations carried out for the foundations of buildings shall be deposited in the immediate vicinity of the working areas from which they originate, without affecting adjacent land. - waste generated during construction activities will be managed in accordance with the relevant legal provisions (selective collection, reuse and final disposal). - the refueling of means of transport will be carried out in authorized service stations. - maintenance, repair and washing of the machines and means of transport used will be carried out by authorized companies. - restoration of vegetation by ecological reconstruction in the area of the foundation platforms and technological platforms by covering with topsoil and restoring vegetation specific to the habitats in the area. - the access roads on the analyzed site will be arranged in order to improve the roadway parts, as well as to restore the infrastructure, so that it will be possible to access the machinery involved in the construction, as well as to provide easy maintenance for the access of the inspection staff throughout the entire duration of operation; Operating stage: - The operation of the wind farm does not have a negative impact on the soil and subsoil and no measures are necessary to prevent, reduce and compensate for adverse environmental effects. Decommissioning phase: - during the decommissioning period the impact is similar to the execution period, therefore the same measures will be taken with regard to work fronts, decommissioning and transportation equipment. | The contractor responsible for the wind farm |
| Use of natural resources | - Reducing the exploitation of exhaustible resources and facilitating the exploitation of renewable ones | - using energy efficient construction equipment to reduce fossil fuel consumption. - In the operational phase, the wind farm promotes the use of renewable resources, having a positive impact. In addition, by realizing the wind farm, an amount of electricity corresponding to the capacity of the wind farm will replace the same amount of electricity produced from non-renewable sources (fossil fuels), thus reducing their exploitation. | The contractor responsible for the wind farm |
|  | - Promoting the transition to a circular and resource-efficient economy | - Using more reliable products that can be reused, upgraded and repaired reduces waste. - The wind turbines and transformer stations that will equip the wind farm will correspond to the principles of the circular economy, reducing the impact on resource consumption. | The contractor responsible for the wind farm |
| Waste management | - Reducing the amount of waste generated and disposed of through landfilling (compliant landfills). | - implementing a waste management plan including recycling and safe disposal of construction materials. - minimizing waste generation through resource efficiency and reuse of materials. | The contractor responsible for the wind farm |
| Noise and vibration | - Limiting noise pollution at sources in areas with noise sensitive receptors. Limitation of vibration levels | Execution stage: - carrying out the works phased in time and space, according to the works schedule, so that the noise level is below the maximum permissible limits. - planning the transportation of materials in such a way that vehicle movements are limited to the minimum necessary to carry out the works in order to reduce the inconvenience to the local population. - use of appropriate noise attenuation systems at sources (engines, machinery, etc.); - erecting mobile billboards in the immediate vicinity of the noise-generating activity in order to protect inhabited areas - setting and imposing speed limits for the circulation of means of transport in localities and on technological roads; - daytime activities (between 7:00 and 23:00), while respecting the rest period of riverside residents or modifying traffic routes accordingly. - monitoring of noise emissions to check compliance with the limits imposed by the applicable legislation depending on the situation. - As far as vibrations are concerned, they are generally low-frequency sounds and cannot adversely affect human health or the environment. Operating stage: - the noise level in the vicinity of the areas where protective measures have been taken shall be monitored; if it is found that they are not effective, additional measures shall be taken. | The contractor responsible for the wind farm Wind farm operator |
| Biodiversity | - Conservation of natural habitats and species of flora and fauna of Community importance | The biodiversity protection measures proposed in the Appropriate Assessment Study are as follows: - General measures - Conduct regular training for all staff involved in construction / decommissioning works on general environmental issues, protected habitats and species and measures to avoid and minimize impacts. Particular attention will be given to issues relating to the prohibition of the collection of plants and animals or the deliberate injuring/killing of protected species. - implementation of low-attractive lighting systems with ground-only dispersion (excluding incandescent heat-generating luminaires). - compliance with the proposed schedule of works, as well as with the period proposed by this plan. - respect of the perimeter of the site organization proposed to be located in the immediate vicinity of the work area. - to carry out activities within the perimeter on strictly necessary areas. - the storage of construction materials will be done only in the areas foreseen by the plan within the site organization and work points, without affecting the adjacent areas. - avoid any spillage of liquid fuels, oils, paints, etc. onto the ground. In case of accidental pollution, they will be removed by applying absorbent materials and removed from the site by contracting companies specialized in the management of these types of hazardous waste. - ensuring proper waste management with regular waste disposal without the use of intermediate and non-compliant landfills. It is forbidden to abandon waste in the immediate vicinity of the site organization and beyond. - access to the work points will be via existing access roads so as not to affect additional areas of land. - the use of machinery and equipment for carrying out the works that produce a minimum level of noise and vibration, that are efficient, low-polluting and quiet, so that wildlife species are not affected. - Topsoil or topsoil resulting from excavation will be properly deposited and then reused. Revegetation of temporarily occupied areas. - Specific measures - in the case of nest identification, postpone activities in the areas concerned beyond the April-May period. - Turbines should be signaled at night with flashing lights, with long intervals between two consecutive lights. These turbines are more easily recognized by birds if flashing rather than continuous light is used. - regular mowing of vegetation around turbines to maintain a low abundance of insect species that are a food source for both chiropteran and bird species. - Installation of an Aircraft Detection Lighting System (ADLS) on turbines T47, T46, T35, T36, T34 to reduce the risk of mortality of species sensitive to artificial lighting (birds and bats). The measure will apply from the first year of operation of the project. - To reduce the risk of mortality it is proposed to limit the turbines (T47, T46, T35, T36, T34) to a wind speed of 6.5 m/s during the sensitive period (migration), starting half an hour before sunset until sunrise and bat protection systems that emit acoustic ultrasonic deterrent signals to remove bats from the swept rotor area. The measure will apply from the wind farm's first year of operation. - The installation of collision risk prevention systems will be implemented from the first year of operation of the wind farm. An automatic shutdown/power-down control system (e.g. DTBird, IdentiFlight, Robin Radar Max, STRIX BirdTrack). The system will target bird species for which a potentially significant impact has been identified and will be implemented from the first year of operation. - Installation of video bird detection system on turbines T8, T18, T29. T32 and T37. High-resolution - camera systems shall be installed.The system to emit deflection signals shall be installed from the first year of operation of the wind farm. The camera system detects birds from a distance of up to 600 m and emits chasing sounds. If the bird continues to approach the turbine, after 300 meters the blades start to slow down to a speed of 3 rpm, which eliminates the risk of collision. - Installation of day and night radar and video camera systems (on turbines T8, T18, T29. T32 and T37) that detect and prevent the risk of chiroptera collision with wind farm structures. The deflection beacon system shall be installed from the first year of operation of the wind farm. |  |
|  | - Maintaining the Natura 2000 network of protected areas. |  |  |
| Population and human health | - Maintaining the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health | - using state-of-the-art equipment and technologies that generate low noise levels. - during construction to prevent dust emissions, particularly during periods of high winds, more intensive wetting of surfaces under the action of working machinery or access roads, especially unpaved ones, shall be pursued. - Implement green building practices that reduce the impact on soil, such as proper waste management and the use of sustainable building materials. | The contractor responsible for the wind farm |
|  | - Improving living standards and social conditions of the population | - Employing local labor to build, maintain and operate the wind farm. - the development of local infrastructure (roads) necessary for the construction of the wind farm which will remain for the benefit of the community. - providing financial compensation or direct benefits to landowners affected by the construction and operation of the wind farm. | The contractor responsible for the wind farm. Wind farm operator |
| Landscape | - Ensuring protection of the natural landscape and reducing the impact on the natural landscape | - using colors that reduce the contrast between turbine structures and the landscape. - use matt finishing paints to reduce sunlight reflection. - restoration of damaged land areas - maintenance of vegetated areas and access roads on the site. - design and construction of substations in correlation with the site area. | The contractor responsible for the wind farm |
| Climate change | - Reducing GHG emissions from the energy sector | - The actual construction of the wind farm is an effective measure to reduce greenhouse gas (GHG) emissions by replacing fossil energy sources with clean and sustainable renewable energy. | The contractor responsible for the wind farm |
|  | - Minimal risks of climate change (adaptation) | - using turbines designed to withstand high winds and storms. - using turbines made of materials and components that can withstand wide temperature variations and extreme weather conditions. - designing foundations to resist erosion and soil changes - integrating advanced weather forecasts to anticipate and manage the impact of extreme weather conditions in order to establish the appropriate operating regime for the conditions. | Wind farm operator |
| Cultural heritage | - Protection and conservation of cultural heritage | - monitoring construction activities to ensure that they do not adversely affect cultural heritage sites. | The contractor responsible for the wind farm |

# A statement of the reasons which led to the selection of the chosen options and a description of how the evaluation was carried out, including any difficulties (such as technical deficiencies or lack of know-how) encountered in processing the required information

  - Analysis of alternatives

In the process of realizing the PUZ 4 alternatives were studied:

- Alternative "zero" - no PUZ implementation

- Alternative I - was initiated in 2011 (consisting of 3 wind farms) - 117 wind power plants with a total installed capacity of 351 MW

- Alternative II - started in 2022 - 48 wind power plants with a total installed capacity of 316.8 MW

- Alternative III - was initiated in 2023 after obtaining the TRANSGAZ SA approval no. 34618/948/10.05.2023, as a result of which 2 power plants T17 and T23 were abandoned, thus the new wind farm will comprise 46 wind power plants with a total installed capacity of 303.6 MW

In Chapters 6.2 and 6.3 the effects of non-implementation and implementation of the PUZ in the three alternatives on the environmental objectives have been assessed, respectively, and the table below presents the final assessment of the significance of the impacts for each alternative analyzed.

Table 9- 1 Impact assessment of the analyzed alternatives

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Impact assessment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ZERO Alternative | Execution |  |  | Operate |  |  | Decommissioning |  |  |
|  |  |  | Alternative I | Alternative II | Alternative III | Alternative I | Alternative II | Alternative III | Alternative I | Alternative II | Alternative III |
| Water | OM1. Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | 0 | -1 | -1 | -1 | 0 | 0 | 0 | -1 | -1 | -1 |
| Air | OM2. Maintaining air quality by reducing emissions from energy sector activities | -1 | -2 | -1 | -1 | 2 | 2 | 2 | -2 | -1 | -1 |
| Soil/subsoil | OM3. Limit soil pollution and land degradation. | 0 | -2 | -2 | -2 | 0 | 0 | 0 | -1 | -1 | -1 |
| Use of natural resources | Reduce the exploitation of exhaustible resources and facilitate the exploitation of renewable ones | -2 | -2 | -1 | -1 | 2 | 2 | 2 | -1 | -1 | -1 |
|  | OM5. Promoting the transition to a circular and resource-efficient economy | -1 | -1 | -1 | -1 | 2 | 2 | 2 | 0 | 0 | 0 |
| Waste management | OM6. Reducing the quantities of waste generated and disposed of through landfilling (compliant landfills) | 0 | -1 | -1 | -1 | 0 | 0 | 0 | -2 | -2 | -2 |
| Noise and vibration | OM7. Limiting noise pollution at sources in areas with noise sensitive receptors. Limitation of vibration levels | 0 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | -1 | -1 |
| Biodiversity | OM8. Conservation of natural habitats and species of flora and fauna of Community importance | 0 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | -1 | -1 |
|  | OM9. Maintain the Natura 2000 network of protected areas. | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Population and human health | OM10. Maintaining the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health | 0 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | -1 | -1 |
|  | OM11. Improving living standards and social conditions of the population | -1 | 1 | 1 | 1 | 2 | 2 | 2 | 1 | 1 | 1 |
| Landscape | OM12. Ensuring the protection of the natural landscape and reducing impacts on the natural landscape | 0 | -1 | -1 | -1 | 0 | 0 | 0 | 2 | 1 | 1 |
| Climate change | OM13. Reducing GHG emissions from the energy sector | -2 | -1 | -1 | -1 | 2 | 2 | 2 | -1 | -1 | -1 |
|  | OM14. Minimizing risks to climate change (adaptation) | -1 | 0 | 0 | 0 | 2 | 2 | 2 | 0 | 0 | 0 |
| Cultural heritage | OM15. Protection and conservation of cultural heritage | 0 | -1 | -1 | -1 | 0 | 0 | 0 | -1 | -1 | -1 |
| TOTAL |  | -8 | -14 | -12 | -12 | 9 | 12 | 12 | -9 | -9 | -9 |

From the evaluation of the three alternatives in relation to the environmental objectives established for the impact assessment of the implementation of the PUZ it can be concluded that alternatives II and III have the lowest impact.

The advantages and disadvantages of each alternative and the rationale for choosing the optimal alternative are presented below.

The "zero" alternative has been considered as a reference element against which to compare the other alternatives for the different elements of the analyzed PUZ subject of the analyzed zonal urban plan. The main forms of impact associated with the adoption of the "zero" alternative are:

- loss of the opportunity to capitalize on the high wind potential in this area;

- Loss of job opportunities (estimated at 30 ÷ 75 direct jobs in the pre-construction and construction phase);

- the loss of economic opportunities that would support the local community in the socio-economic development that would help solve environmental problems;

- loss of private investors' interest in future industrial development projects in the region and in Romania;

- the loss of support for the development of a reliable alternative electricity generation in line with policies developed at European and national level;

- Failure to meet the nationally agreed objectives on decarbonization of the energy sector, promotion of renewable energy production resources for a GHG neutral Romania by 2050.

Alternative I consists in the realization of three wind farms consisting of 117 wind turbines of 3 MW/turbine, with a total capacity of 351 MW and 3 transformer stations.

For this variant, a total land area of 473.09 ha is required (according to the decisions of the classification stage issued in 2013: DEI 14866/29.04.2013, DEI 14867/22.04.2013, DEI 14868/22.04.2013) of which 3.90 ha permanently occupied by construction and 469.19 ha temporarily occupied by works. In this variant, the land with the current agricultural function - arable land requires the conversion of the category of use to "technical and infrastructural facilities", which implies the removal of 5 ha of arable land from the agricultural use.

The advantage of this alternative is the total power of 351 MW of the three wind farms, which will replace an equivalent amount of electricity produced from fossil fuels, thus contributing to the national and European commitments to decarbonize the energy system and diversify energy production sources, ensuring energy independence.

The disadvantage of Alternative I is represented by the large number of wind turbines foreseen by the PUZ which implies a high density of turbines with effects on inhabited areas through an increase in noise level, an intensification of shading, an agglomeration of the natural landscape.

Plus the land affected by new roads, foundations and platforms for the 117 wind turbines and 3 transformer stations.

Furthermore, in 2011, the date of the Adequate Assessment Study for the 3 parks around Cerchezu, from which the analyzed PUZ was derived, the ROSPA0166 site was not declared as a protected area and the Adequate Assessment Study was not based on the assessment in relation to the conservation objectives of Natura 2000 sites, since at that time the specific conservation objectives were not defined. The major conclusion of this Adequate Assessment Study was that the siting of the wind turbines in the area of interest, which is already anthropized, will have a negligible impact on the local vegetation, flora and fauna.

Alternative II consists in the realization of a wind farm consisting of 48 wind turbines with a power of 6.6 MW/turbine, the total power of the wind farm being 316.8 MW and 4 transformer stations.

For this variant, a total land area of 3,027.77 ha is required, of which 309.34 ha is permanently occupied by buildings and 2,718.43 ha is temporarily occupied by works. In this variant, the land currently used for agriculture - arable land needs to be converted from agricultural use to "technical and infrastructure facilities", which implies taking 309.34 ha of arable land out of agricultural use.

The advantage of this alternative is the significantly smaller number of turbines, which minimizes the disadvantages of the first alternative, and the location of the turbines at greater distances from each other.

The reduction in the number of turbines foreseen by the PUZ increases the distance between the wind farms to more than 600 m, compared to about 350 m in the situation approved in 2011, and the lower density results in a more balanced use of natural resources and reduces the pressure on biodiversity both during the implementation period of the project and during the operation of the wind farm.

The disadvantage of Alternative II is the lower power output of the power park and the incompatibility of the T17 and T23 turbine sites with the gas transmission main, which led to the emergence of Alternative III.

Alternative III consists in the realization of a wind farm consisting of 46 wind turbines with a power of 6.6 MW/turbine, the total power of the wind farm being 303.6 MW and 2 transformer stations.

For this variant, a total land area of 3,027.77 ha is required, of which 292.52 ha are permanently occupied by buildings and 2,735.25 ha are temporarily occupied by works. In this variant, the land currently used for agriculture and arable land would need to be converted from agricultural to "technical and infrastructure facilities", which would involve taking 100 ha of arable land out of agricultural use.

The advantage of this alternative is the compatibility with the natural gas transmission grid plus the advantages of Alternative II related to the smaller number of wind turbines and lower density.

The disadvantage of Alternative III is the lower power output.

Following the analysis of the alternatives, alternative III is considered optimal for the following reasons:

- minimizing negative environmental impacts by reducing the number of wind turbines;

- more efficient use of air currents by optimally positioning the turbines closer to each other and at greater distances, which leads to the best possible operation of the turbines and increases the efficiency of the wind farm.

  - Difficulties encountered in processing the requested information

During the realization of the Environmental Report for the analyzed P.U.Z., the difficulties encountered consisted in the lack of local data on air quality, economic activities carried out at the level of the UATs, the weight of accessing data from Bulgaria.

# Description of the measures envisaged for monitoring the significant effects of the implementation of the PUZ

According to the provisions of HG no. 1076/2004, the monitoring of the implementation of the plan, based on the program proposed by the owner, aims to identify from the outset its significant effects on the environment, as well as unforeseen adverse effects, in order to be able to take appropriate remedial action. The implementation and realization of the environmental effects monitoring program is the responsibility of the holder of the PUZ.

As shown in the assessment of potential environmental effects, the provisions of the PUZ will not generate significant negative effects on the environment (see chapter 7). At the same time, the implementation of the PUZ may bring changes of economic or social nature in the Cerchezu commune.

The monitoring of the effects of the implementation of the plan will be carried out according to the provisions of art. 27 of the H.G. 1076/2004, referring to the significant effects on the environment, i.e. all types of effects, positive, adverse, foreseen or unforeseen. Not only direct, but also indirect, synergistic and cumulative effects should be monitored. The monitoring of other effects (not assessed as significant), may be justified and useful if the quantification of the overall effects of the implementation of the PUZ is considered.

Tracking the quality of environmental factors over time should be carried out in collaboration with the authorities involved in the PUZ approval process. The plan holder's monitoring will refer only to those activities that can be quantifiable in terms of values, quantities and time of realization, will include the period of execution of construction works and the period of operation.

The competent institutions providing support in the implementation and compliance with the Monitoring Program are:

- Local Public Administration (AL) (Cerchezu Municipality);

- Constanta County Council

- Environmental Protection Agency (APM Constanta)

- National Agency for Protected Natural Areas

- Environment Guard (Constanta Police Station)

- Directorate for Public Health (DSP Constanta)

- Dobrogea Litoral Water Basin Administration

- Forest Guard Bucharest

- Ministry of Internal Affairs

- National Natural Gas Transportation Company "TRANSGAZ SA"

- National Electricity Transmission Company - TRANSELECTRICA SA

- County Directorate for Culture Constanta

- Constanta County Agricultural Directorate

- Forestry Directorate

- Other entities: Regional Roads Directorate/CNAIR, Sanitation Operator, Economic operators that propose projects in the area/carry out activities in the area.

The table below presents the proposals for monitoring the effects of the implementation of the analyzed plan on the environmental aspects assessed for this PUZ.

Table 10- 1 Proposals for monitoring the effects of the implementation of the PUZ

| Relevant environmental aspects | Environmental objectives relevant to the PUZ | Indicators | Responsible |
| --- | --- | --- | --- |
| Water | - Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies | Periodic visual assessment of surface water quality in the vicinity of the site (Magura and Cerchez watercourse). Visual inspection to identify potential leaks of oils or other chemicals from equipment and record inspections. | The contractor responsible for the wind farm. Wind farm operator. |
| Air | - Maintaining air quality by reducing emissions from energy sector activities | The level of air pollutant emissions (CO, NO2, SO2, PM10, PM2.5, VOC) resulting during the construction period are associated with the machinery/machinery/equipment involved in the works, but are low. During the operation of the wind farm there are no emissions of pollutants except for the machines involved in maintenance and intervention activities, but the associated emissions are insignificant and cannot affect the environmental factor air. | The contractor responsible for the wind farm. |
| Soil/subsoil | - Limiting soil pollution and land degradation. | Monitoring waste and hazardous materials management to prevent soil contamination. | The contractor responsible for the wind farm. |
| Use of natural resources | - Reducing the exploitation of exhaustible resources and facilitating the exploitation of renewable ones | Monitoring the consumption of exhaustible resources during the PUZ implementation phase Monitoring renewable electricity production. | The contractor responsible for the wind farm. |
|  | - Promoting the transition to a circular and resource-efficient economy | Monitoring the use of renewable materials in the project. | The contractor responsible for the wind farm. |
| Waste management | - Reducing the amount of waste generated and disposed of through landfilling (compliant landfills). | Monitoring the amounts of waste generated during construction and operation. Implementation and regular evaluation of a waste management plan. | The contractor responsible for the wind farm. |
| Noise and vibration | - Limitation, at source, of noise pollution in areas with noise sensitive receptors. Limitation of vibration levels | Monitoring noise and vibration levels near residential areas and other sensitive receptors. | Wind farm operator |
| Biodiversity | - Conservation of natural habitats and species of flora and fauna of Community importance | Impact assessment on natural habitats and local species. Implement biodiversity conservation and monitoring measures. | Wind farm operator |
|  | - Maintaining the Natura 2000 network of protected areas. | Monitoring the impact of the project on Natura 2000 protected areas | Wind farm operator |
| Population and human health | - Maintaining the quality of environmental factors within the limits of the legal provisions for the protection of the environment and public health | Monitor air, water and soil quality in accordance with legal standards. | Wind farm operator |
|  | - Improving living standards and social conditions of the population | Assessment of community satisfaction with the project and social benefits. | Wind farm operator |
| Landscape | - Ensuring protection of the natural landscape and reducing the impact on the natural landscape | Implementation of landscape integration measures for wind structures. | Wind farm operator |
| Climate change | - Reducing GHG emissions from the energy sector | Monitoring of electricity production and assessment of avoided GHG emissions due to wind farm operation. | Wind farm operator |
|  | - Minimal risks of climate change (adaptation) | Using wind turbines designed to withstand extreme weather conditions such as high winds, storms and extreme temperatures. These can include tougher materials and self-regulating speed technologies. | Beneficiary/Contractor responsible for the execution of the wind farm. |
| Cultural heritage | - Protection and conservation of cultural heritage | Monitoring the impact of constructions and activities on cultural heritage sites. Implement measures to protect and preserve cultural heritage. | Wind farm operator |

# Non-technical summary

This report presents the Environmental Report for the Strategic Environmental Assessment of the Zonal Urban Development Plan (PUZ) for the development of a wind energy park in Cerchezu commune, Constanța county. The plan is realized in the context of the need to assess the environmental impact and to respond to national and international objectives on renewable energy and environmental protection.

The Zoning Urban Plan analyzes the development of a wind farm consisting of 48 (46) wind farms with a total capacity of approximately 316.8 (303.6) MW. The project also includes transformer stations, electrical connection networks and the construction and modernization of the necessary access roads.

The Environmental Report was prepared in accordance with the provisions of Annex 2 to GD 1076/2004, the recommendations of the Generic Guide for the Strategic Environmental Assessment (SEA) procedure and the Manual for the application of the procedure for carrying out the environmental assessment of plans and programs, prepared by the Ministry of Environment and Water Management in collaboration with the National Agency for Environmental Protection.

For the realization of the Adequate Assessment Study, the provisions of the Methodological Guide on the adequate assessment of the potential effects of plans or projects on protected natural areas of community interest approved by Order 1682/2023 were respected. The Environmental Report has identified, described and assessed the potential significant environmental effects of the implementation of the PUZ, as well as its alternatives, taking into account the objectives and geographical area of the plan.

The overall objective of the PUZ is to contribute to the economic development of Constanța County by attracting sustainable investments and utilizing the wind potential of the area. It also aims to reduce emissions of greenhouse gases and other pollutants, in line with Romania's commitments under the Paris Agreement and other national and European strategies and plans.

The wind farm will be located in the Cerchezu municipality, bordering the municipalities of Chirnogeni, Independența, Dumbrăveni and Negru Voda. The project is in the immediate vicinity of the border line with Bulgaria. Four alternatives were studied in the evaluation of the PUZ, including the zero variant (without implementation), and three development variants with different configurations and wind energy capacities.

In order to create the framework for the assessment of the environmental effects generated by the implementation of the PUZ subject of this Environmental Report, the following relevant environmental objectives were analyzed and selected during the scoping stage of the SEA procedure:

- Maintaining and improving the ecological status/potential and chemical status of surface and groundwater bodies

- Maintaining air quality by reducing emissions from energy sector activities

- Limiting soil pollution and land degradation.

- Reducing the exploitation of exhaustible resources and facilitating the exploitation of renewable ones

- Promoting the transition to a circular and resource-efficient economy

- Reducing the amounts of waste generated and disposed of through landfilling (compliant landfills)

- Limitation, at source, of noise pollution in areas with noise sensitive receptors. Limitation of vibration levels

- Conservation of natural habitats and species of flora and fauna of Community importance

- Maintaining the Natura 2000 network of protected areas.

- Maintaining the quality of environmental factors within the limits of legal provisions for the protection of the environment and public health

- Improving living standards and social conditions of the population

- Ensuring protection of the natural landscape and reducing the impact on the natural landscape

- Reducing GHG emissions from the energy sector

- Minimal risks of climate change (adaptation)

- Protection and conservation of cultural heritage

For the strategic environmental assessment of the proposed project the following steps have been completed:

- analyzing the current state of the environment and identifying environmental problems

- Relevant environmental objectives (objectives leading to the solution of these identified environmental problems) have been formulated on the basis of the environmental problems. 15 environmental objectives have been formulated covering the following environmental issues: water, air, soil/soil, use of natural resources, waste management, noise and vibration, biodiversity, population and human health, landscape and cultural heritage.

- A specific scoring system has been used to assess the effects of the proposed PUZ in relation to the degree of contribution to the achievement of the relevant environmental objectives, with a score ranging from -2 to +2 for each objective. This system has allowed a ranking of the positive and negative effects that the proposed investment has on the achievement of the relevant environmental objectives. This assessment has also identified situations where the plan does not contribute unless certain measures are proposed, or contributes little to the achievement of some of the environmental objectives mentioned above.

Environmental Impact:

- Water: The project does not anticipate significant long-term impacts to surface and groundwater quality. The work will be temporary and will include measures to prevent runoff of construction materials and erosion. Protective measures will be provided to prevent water contamination during construction and operation of the wind farm.

- Air: The implementation of the PUZ will contribute to the reduction of greenhouse gas emissions by using renewable energy instead of fossil fuels. This will improve air quality in the area and contribute to national and international air pollution reduction targets.

- Soil and Subsoil: Measures to minimize soil pollution will be implemented, including controlled storage of materials and revegetation after construction completion. Foundations and access roads will be designed to minimize impact on soil structure and prevent erosion.

- Waste Management: The project includes plans for the proper management of waste generated during construction and operation. Measures for recycling and safe disposal of waste will be implemented, minimizing environmental impacts.

- Noise and Vibration: During construction and operation, measures will be taken to minimize noise and vibration. Wind turbines will be designed to operate at acceptable noise levels, and construction work will comply with noise regulations to minimize impacts on local communities.

- Biodiversity: The project will include detailed assessments to ensure that impacts on local flora and fauna are minimized. Measures will be implemented to protect natural habitats and avoid disturbance of protected species.

- Population and Human Health: The implementation of the wind farm will bring benefits to the local community, including job creation and improved infrastructure.

- Landscape: The visual impact of wind turbines will be reduced by using colors and materials that blend into the existing landscape.

- Climate Change: The project will contribute to climate change mitigation through the use of wind energy, reducing dependence on fossil fuels and greenhouse gas emissions.

Given the proximity to Bulgaria, the report assessed potential transboundary impacts. The implementation of the PUZ is not anticipated to generate significant negative effects on the environment in Bulgaria. However, monitoring and communication measures with the Bulgarian authorities have been considered to ensure transparency and appropriate management of any transboundary concerns.

To prevent and reduce potential negative environmental impacts, the PUZ includes a set of specific measures. Also, in order to monitor the environmental effects of the implementation of the proposed Zonal Urban Plan, a set of indicators has been established in the Environmental Report to allow a simple and efficient assessment of how the wind farm will achieve the environmental objectives set, as well as to evaluate the effectiveness of the proposed measures.

As a general conclusion, the implementation of the Zonal Urban Plan for the wind energy park can make a significant positive contribution to the environmental objectives and to the fulfillment of Romania's obligations regarding renewable energy, climate change mitigation, decarbonization of the energy system and environmental protection. No significant negative environmental effects have been identified and the proposed measures will ensure that the impact on the environment and the local community will be minimized.

Bibliography:

- HG 1076/2004 on establishing the procedure for carrying out the environmental assessment of plans and programs

- Generic guidance on environmental assessment for plans and programs

- Manual for the application of the procedure for carrying out environmental assessment of plans and programs

- Guidelines on environmental assessment for development plans and programs in the energy sector

- Guidelines on environmental assessment for spatial and urban planning plans and programs

- Framework Strategy for a resilient Energy Union with a forward-looking climate change policy (https://eur-lex.europa.eu/resource.html?uri=cellar:1bd46c90-bdd4-11e4-bbe1-01aa75ed71a1.0012.03/DOC_1&format=PDF )

- Paris Agreement, 2015 (https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:22016A1019(01)&from=EN )

- European Green Deal (https://eur-lex.europa.eu/resource.html?uri=cellar:b828d165-1c22-11ea-8c1f-01aa75ed71a1.0020.02/DOC_1&format=PDF)

- Climate and Energy Card 2030 (https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:52020DC0562 )

- European climate law (https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:32021R1119 )

- Ready for 55 legislative package

- REPowerEU Plan REPowerEU Plan (https://eur-lex.europa.eu/resource.html?uri=cellar:fc930f14-d7ae-11ec-a95f-01aa75ed71a1.0014.02/DOC_1&format=PDF )

- Communication from the Commission A Green Deal Industrial Plan for the Net Zero Emissions Era (https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:52023DC0062 )

- EU Biodiversity Strategy to 2030 (https://eur-lex.europa.eu/resource.html?uri=cellar:a3c806a6-9ab3-11ea-9d2d-01aa75ed71a1.0023.02/DOC_1&format=PDF )

- Renewable energy strategy

- Romania's Energy Strategy 2007-2020

- Romania's Energy Strategy 2022-2030, with a 2050 perspective

- Integrated National Energy and Climate Change Plan 2021-2030, including 2023 update

- Romania's National Strategy for Sustainable Development 2030 and Action Plan

- National Strategy on Climate Change and Low Carbon Growth for 2016-2020

- National Strategy on Adaptation to Climate Change for the period 2022-2030 with a 2050 perspective

- National Action Plan for the implementation of the National Strategy on Adaptation to Climate Change

- Romania's long-term strategy to reduce GHGs Romania Neutral in 2050

- National circular economy strategy

- Sustainable Development Strategy of Constanța County for 2021 - 2027

- Spatial Development Plan - Constanta County

- South-West Dobrogea South-West LAG Strategy 2014-2020, modified in 2023 (https://galdobrogeasv.ro/modificari-ale-strategiei-de-dezvoltare-locala/ )

- Local Environmental Action Plan (http://apmct.anpm.ro/-/planul-local-de-actiune-pentru-mediu-constanta )

- Danube Littoral River Basin Management Plan 2022-2027

- CESTRIN 2022 traffic census

- County report on the state of the environment Constanța county, year 2022

- County Waste Management Plan - for Constanta County

- Final biodiversity monitoring report on the future Dumbrăveni-Cerchezu wind farm

- National Institute of Statistics, population statistics

- State of Health in the EU: Romania Health Country Profile 2021

- Landscape fragmentation Effective Mesh Density time-series: major and medium anthropogenic fragmenting elements (FGA2-S) - European Environment Agency (europa.eu)

- East, A., E.,Sankey, J. B. (2020), How is modern climate change affecting landscape processes?

www.ramboll.com

ANNEXES

- Annex 1 - Certificate of registration in the Register of certified experts for the elaboration of environmental studies, for the following fields and studies: RIM-2, RIM-3, RIM-7, RIM-11a, RIM-11b, RIM-11c; RA-7; RM-3, RM-11a, RM-11b, RM-11c, RM-13b; RS-2, RS-11c; BM-2, BM-11b, BM-11c; EA; EGSC - Certificate RGX Series no.333/1.08.2022

- Annex 2 - Town Planning Certificate nr.129/28.11.2028

- Annex 3 - Classification plan

Situation       plan

www.ramboll.com

https://eur-lex.europa.eu/resource.html?uri=cellar:1bd46c90-bdd4-11e4-bbe1-01aa75ed71a1.0012.03/DOC_1&format=PDF

https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:22016A1019(01)&from=EN

https://eur-lex.europa.eu/resource.html?uri=cellar:b828d165-1c22-11ea-8c1f-01aa75ed71a1.0020.02/DOC_1&format=PDF

https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:52020DC0562

https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:32021R1119

https://eur-lex.europa.eu/resource.html?uri=cellar:fc930f14-d7ae-11ec-a95f-01aa75ed71a1.0014.02/DOC_1&format=PDF

https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:32022D0591

https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:52023DC0062

https://eur-lex.europa.eu/resource.html?uri=cellar:a3c806a6-9ab3-11ea-9d2d-01aa75ed71a1.0023.02/DOC_1&format=PDF

https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:02018L2001-20231120

https://eur-lex.europa.eu/legal-content/RO/TXT/PDF/?uri=CELEX:32022R0869

https://eur-lex.europa.eu/resource.html?uri=cellar:f5586441-f5e1-11ec-b976-01aa75ed71a1.0009.02/DOC_1&format=PDF

https://energie.gov.ro/wp-content/uploads/2022/08/Strategia-2030_DGJRI_AM_12.08.2022_MU_Clean_25.08.2022-1.pdf

https://energie.gov.ro/wp-content/uploads/2023/12/NECP_Romania_first-draft-version-21.12.2023_RO.pdf

https://www.edu.ro/sites/default/files/Strategia-nationala-pentru-dezvoltarea-durabila-a-Rom%C3%A2niei-2030.pdf

https://www.mmediu.ro/app/webroot/uploads/files/SNASC_SEA_2022.pdf

http://www.cjc.ro/sectiune.php?s=247

https://galdobrogeasv.ro/modificari-ale-strategiei-de-dezvoltare-locala/

http://apmct.anpm.ro/-/planul-local-de-actiune-pentru-mediu-constanta

Updated Management Plan for the Danube River, Danube Delta, Dobrogea Hydrographic Area and Coastal Waters, 2022-2027

https://www.cestrin.ro/assets/pdf/recensamant%202022.pdf

https://www.graiuldobrogei.ro/page/1/?s=cerchezu

https://galdobrogeasv.ro/modificari-ale-strategiei-de-dezvoltare-locala/

https://galdobrogeasv.ro/modificari-ale-strategiei-de-dezvoltare-locala/

COUNTY WASTE MANAGEMENT PLAN CONSTANTA COUNTY 2020 - 2025

https://galdobrogeasv.ro/legaturi-utile/

The relative poverty rate is defined as the share of the poor (by the relative method of estimation) in the total population. Persons in households with a disposable income per adult-equivalent (including or excluding consumption from own resources) below the poverty line are considered poor.

State of Health in the EU: Romania Health Country Profile 2021

https://dspct.ro/wp-content/uploads/2024/04/RAPORT-JUDETEAN-APA-POTABILA-2023.pdf

County State of the Environment Report 2022

Climate change is a growing risk factor for ecosystems, human health and the economy in Europe, European Environment Agency, 2017

Landscape fragmentation Effective Mesh Density time-series: major and medium anthropogenic fragmenting elements (FGA2-S) - European Environment Agency (europa.eu)

East, A., E.,Sankey, J. B. (2020), How is modern climate change affecting landscape processes?

Dobrogea South-West LAG Local Development Strategy, modified 2023

Ambient air quality - EEA (government.bg)

https://eea.government.bg/bg/dokladi/threemonth/threemonth_04_2022/air/listset

https://en.wikipedia.org/wiki/Geography_of_Bulgaria#Soils

National Report on the State and Protection of the Environment in the Republic of Bulgaria - EEA (government.bg)

https://ro.wikipedia.org/wiki/Krai%C8%99te,_Dobrici

Deaths by districts, municipalities and sex | National statistical institute (nsi.bg)

https://ec.europa.eu/regional_policy/whats-new/newsroom/21-12-2023-eu-supports-just-climate-transition-in-bulgaria-with-a-budget-of-eur1-2-billion_en

https://eea.government.bg/bg/soer/2023

https://eea.government.bg/bg/soer/2023