Answers to the Bulgarian notice  No. 49/12.10.2019.

- Estimated quantities of all types of waste that will be generated by the plant:

The types of waste to be generated by the plant, with the corresponding waste codes according to Government Decision no. 856/2002, will be:

  - mixed municipal waste - code 20 03 01 – 2.22 cbm/day

  - paper and cardboard packaging - code 15 01 01- 2 kg/day

  - plastic packaging - code 15 01 02 – 1 kg/day

  - hydraulic oils, containing PCBs – code 13 01 01* - 0.2 kg/day

  - other hydraulic oils – code 13 01 13* - 0.5 kg/day

  - other engine, gear and lubricating oils – code 13 02 08* - 2 kg/day

  - other insulating and heat transmission oils – code 13 03 10* - 0.4 kg/day

  - sludges from oil/water separators – code 13 05 02* - 4 kg/day

  - oily water from oil/water separators – code 13 05 07* - 15 kg/day

  - packaging containing residues of or contaminated by hazardous substances – code 15 01 10* - 0.3 kg/day

  - spent catalysts contaminated with hazardous substances – code 16 08 07* - 0.2 kg/day

  - solid wastes from gas treatment – code 19 01 07* – 6.5 t/day

- Noise level generated by the plant affecting Tutrakan town

A “Noise Level Assessment and Noise Propagation Maps” was prepared by Enviro Consult to determine the noise level generated by the plant.

The levels of the acoustic pressure generated by the equipment in the “Oltenita Used Oil Refining Plant” were assessed. The results obtained were processed and entered in the Predictor-LimA software version 2020. The sources of noise in the plant were identified and a current noise distribution map was prepared.

The study took into consideration the most unfavourable situations, when the atmospheric conditions influence the propagation of noise to the receptors, and the noise generated by the equipment reaches the maximum admissible design value.

The site under assessment is similar to an industrial area as regards the legislation in force and the Oltenita General Plan.

Calculations of the noise level were made on the property line and at the nearest sensitive receptors, taking into account the normal operation of the plant. The distance between the plant’s property line and receptor L1 (the nearest residential buildings in Oltenita) is over 770 m, and the distance between the plant’s property line and receptor T1 (the nearest residential buildings in Turtukan) is 1.27 km.

The conclusions of the assessment were that under normal operating conditions of the “Oltenita Used Oil Refining Plant”, the nearest receptors are exposed to noise levels of 34.15 dBA in Oltenita and 24.87 dBA in Turtukan. Under normal operating conditions of the the “Oltenita Used Oil Refining Plant”, the industry maximum admissible level was not found to be exceed on the property line.

We enclose herewith the “Noise Level Assessment and Noise Propagation Maps” prepared by Enviro Consult, where the noise level charts can be seen.

- Table of hazardous substances and mixtures present on site, plus the hazard categories and precautionary statements according to the CLP Regulation:

| Pos. | Hazardous Product / Substances Stored | CAS no. | CLP Hazard Categories | CLP Precautionary Statements | Classification (1272/2008/CE) | Storage Space | Storage Capacity (t) | Physical State | Handling / Storage Method | Storage Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Used Lube Oil | 4978-12-04 | None Category 1 Category 1A & 1B None Category 2 Category 3 | None P301+P310; P405, P501 P201, P308+P313, P405, P501 None P264, P302+P352 P273, P501 | H227, H304, H350, H336, H315, H412 | metallic storage tank | 5849 | liquid | tank truck | 4 metallic tanks, capacity 1 x 107 cbm and 3 x 1914 m3 |
| 2 | Diesel/Light Oil | 64741-77-1 | Category 1 | P301+P310, P405, P501 | H304 | metallic storage tank | 718 | liquid | tank truck | aboveground, double wall metallic tank, 1 x 473 cbm and 1 x 245 cbm |
| 3 | Middle Distillate | 64742-54-7 | Category 1 | P301+P310, P405, P501 | H304 | condensed metallic storage tank | 1864 | liquid | tank truck | two aboveground metallic tanks 1 x 930 cbm and 1 x 934 cbm |
| 4 | Heavy Distillate | 6474176-0 | Category 1 |  | H304 | condensed metallic storage tank | 1879 | liquid | tank truck | two aboveground metallic tanks 1 x 945 cbm and 1 x 934 cbm |
| 5 | Heavy Lubricating | 8052-42-4 | Not classified | Not classified | Not classified | tank | 488 | liquid | tank truck | 2 tanks x 244 cbm |
| 6 | Caustic Soda | 1310-73-2 | Subcategories 1A, 1B & 1C, and category 1 | P260, P301+P330+P33 1, P405, P501 | H314 | anti-corrosion tank | 26.35 | Liquid 32% conc. | tank truck | 1 x 54.89 cbm and 1 x 11 cbm |
| 7 | Hydrogen | 1333-74-0 |  | P210; P337; P381; P403; | H220 H280 | hydrotreating unit | 0.43 | gas | Pressure vessels, pipes | Pressure vessel |

After having submitted the Seveso Notification to the Environmental Protection Agency, the facility received in answer notice no. 7162/07.08.2018 issued by the Calarasi Environmental Protection Agency, the Calarasi Environmental Guard, and the Calarasi Inspectorate for Emergency Situations, stating that following an analysis of the site and the types and quantities of substances that will be present on site, the facility does not fall under Law no. 59/2016 on the control of major accident hazards involving hazardous substances.

The information regarding the classification of hazardous substances was taken from the products’ technical data sheets.